3GPP TSG RAN Meeting #109						RP-252641

Beijing, China, September 15-18, 2025

Agenda Item: 8.2.1.4

Source: Google

Title: On the support of ISAC in 6G

Document for: Discussion and Decision

# Introduction

This document outlines the perspective of Google on the core elements and the key aspects of Integrated Sensing and Communication (ISAC) for 6G.

We are building on the groundwork of 5G-Advanced and in line with the goals of the new 6G Radio study to propose a framework that is straightforward, efficient, and extensible, with a strong emphasis on practical deployment, market needs and commercial viability.

# Discussion

## Key Use cases and prioritisation

ISAC is predicted to be a cornerstone of 6G, enabling a new range of services beyond traditional communication. To facilitate focused and productive work, we propose prioritizing a subset of use cases for initial studies.

- UAVs: The detection and tracking of UAVs are a high priority, supporting applications like low-altitude economy, traffic surveillance, and intrusion detection. Our evaluation framework should accommodate intra- and inter-TRP mobility tracking for sensing objects.

- Human/Pedestrian: Sensing of humans is critical for public safety, smart homes, and industrial automation. This includes intrusion detection, gesture recognition, and health monitoring.

- Automotive/Vehicles: ISAC can significantly enhance smart transportation by complementing onboard sensors with global contextual data. Key applications include early accident indication, pedestrian detection on highways, and assisting AGVs.

- Digital twinning:  ISAC for environment awareness can assist communication and augment applications like Google Maps. By using sensing to acquire information about the physical world, a digital twin can be created to reflect real-time conditions. This digital representation of the environment can then be used to enhance communication services, for example, by providing data for beam management, channel estimation, and power saving. For applications such as Google Maps, a digital twin could provide augmented information, offering a more dynamic and accurate representation of the environment.

- Focus the initial 6G sensing study on the most commercially viable and technically feasible use cases for object detection and tracking, specifically prioritizing UAVs, humans, and automotive-related applications.

- RAN should identify the communication aspects which can be assisted/enhanced by sensing, including beam management, channel estimation, and power saving .

- Focus on digital twin to reflect real-time conditions of the physical world to augment applications like Google Maps

## Sensing Modes and Configuration

The diversity of sensing use cases necessitates a flexible system capable of supporting various sensing modes. We propose a framework that accounts for all six sensing modes but prioritizes the study of those involving UE:

- TRP-to-UE and UE-to-TRP Bistatic Sensing: These modes are a priority for evaluation due to their ability to improve sensing performance. UE involvement can overcome coverage issues related to TRP antenna downtilt and NLOS conditions, particularly in urban and indoor environments.

- UE Monostatic/UE-to-UE Bistatic Sensing: These modes allow for direct device-to-device sensing and are particularly suited for short-range applications like gesture recognition and indoor localization. UE Monostatic will be very useful for XR and immersive use cases.

- TRP Monostatic/TRP-to-TRP Bistatic Sensing: These are valuable for large-scale sensing, outdoor sensing and continuous area monitoring.

- Prioritize the evaluation and study of sensing modes involving UEs (TRP-to-UE, UE-to-TRP, UE-to-UE, and UE monostatic).

- Mechanisms for dynamic switching between sensing modes based on specific criteria (e.g., distance, velocity, SINR) should also be investigated.

## Sensing Procedure and Mobility

A key challenge for ISAC is maintaining uninterrupted sensing performance for mobile targets as they move across cell boundaries or out of range of the sensing node. We propose a set of procedures to ensure seamless sensing handovers.

- Sensing Handovers: When a sensing target moves from one cell to another, the first TRP should trigger sensing in the second TRP. This triggering should include assistance information about the target, such as its last-known position, velocity, and type, to facilitate seamless tracking.

- Signaling and Coordination: We need to define new signaling procedures and Information Elements over interfaces like Xn and F1 to enable the exchange of sensing configuration information, capabilities, and measurement reports between TRPs.

- Centralized Control: The Sensing Function can be responsible for orchestrating these handovers and coordinating sensing activities across multiple devices and TRPs. This entity can collect and combine measurements from multiple sources to improve accuracy and reliability.

- Study procedures for managing the mobility of sensing targets, including proactive triggering and seamless handovers between sensing nodes. This should be supported by standardized signaling over 6G interfaces for the exchange of sensing configuration, capabilities, and target-specific assistance information.

## Sensing Metrics and Evaluation

Evaluation of ISAC performance should be based on a comprehensive set of metrics that go beyond traditional communication KPIs. The following metrics should be adopted for 6G sensing evaluation:

- Detection Performance: Missed detection probability and False alarm probability.

- Accuracy and Resolution: Positioning accuracy (horizontal/vertical), velocity accuracy, and sensing resolution.

- Service Metrics: Maximum sensing latency and refreshing rate.

- System Capacity: Measuring the maximum number of targets a system can sense simultaneously while maintaining a specific Quality of Service (QoS). This is critical for real-world deployments with multiple targets.

- Efficiency: Evaluate the efficiency of resource allocation based on configured sensing parameters such as periodicity, start offset, and duration. Coexistence mechanisms between sensing and communication should also be assessed

- Evaluate 6G ISAC performance using a comprehensive set of metrics, including detection performance, accuracy, resolution, and service-level KPIs. Sensing Capacity can be defined and evaluated to reflect the ability to handle multiple targets simultaneously.

# Conclusion

In this contribution, we made the following proposals for the support of ISAC in 6G:

- Focus the initial 6G sensing study on the most commercially viable and technically feasible use cases for object detection and tracking, specifically prioritizing UAVs, humans, and automotive-related applications.

- RAN should identify the communication aspects which can be assisted/enhanced by sensing, including beam management, channel estimation, and power saving .

- Focus on digital twin to reflect real-time conditions of the physical world to augment applications like Google Maps

- Prioritize the evaluation and study of sensing modes involving UEs (TRP-to-UE, UE-to-TRP, UE-to-UE, and UE monostatic).

- Mechanisms for dynamic switching between sensing modes based on specific criteria (e.g., distance, velocity, SINR) should also be investigated.

- Study procedures for managing the mobility of sensing targets, including proactive triggering and seamless handovers between sensing nodes. This should be supported by standardized signaling over 6G interfaces for the exchange of sensing configuration, capabilities, and target-specific assistance information.

- Evaluate 6G ISAC performance using a comprehensive set of metrics, including detection performance, accuracy, resolution, and service-level KPIs. Sensing Capacity can be defined and evaluated to reflect the ability to handle multiple targets simultaneously.

# References

- “Feasibility Study on Integrated Sensing and Communication”, 3GPP TSG SA, TR 22.837.

- “Service requirements for Integrated Sensing and Communication; Stage 1”, 3GPP TSG SA, TS 22.137.