3GPP TSG RAN WG1 #123	        		   			R1-2509713

# Dallas, USA, November 17th – 21st, 2025

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | 0746 | rev |  | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on Non-Terrestrial Networks (NTN) for NR Phase 3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_Ph3-Core, NR_IoT_NTN_req_test_enh |  |  |  |  |  | Date: |  |  | 2025-12-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - Missing reference to 3MHz channel bandwidth in Clause 13. - Incorrect RRC parameter names in Clause 17.2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add reference to 3MHz channel bandwidth in Clause 13. - Correct RRC parameter names in Clause 17.2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete/ambiguous specifications. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 13, 17.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** Unchanged parts are omitted ***

# 13	UE procedure for monitoring Type0-PDCCH CSS sets

If during cell search a UE determines from MIB that a CORESET for Type0-PDCCH CSS set is present, as described in clause 4.1, the UE determines a number of consecutive resource blocks and a number of consecutive symbols for the CORESET of the Type0-PDCCH CSS set from controlResourceSetZero in pdcch-ConfigSIB1, as described in Tables 13-0 through 13-10, for operation without shared spectrum channel access in FR1, FR2-1 and FR2-NTN, or as described in Tables 13-1A and 13-4A for operation with shared spectrum channel access in FR1, or as described in Table 13-10A for FR2-2, and determines PDCCH monitoring occasions from searchSpaceZero in pdcch-ConfigSIB1, included in MIB, as described in Tables 13-11 through 13-15A. SFNc and nc are the SFN and slot index within a frame of the CORESET based on SCS of the CORESET and SFNSSB,i and nSSB,i are the SFN and slot index based on SCS of the CORESET, respectively, where the SS/PBCH block with index i overlaps in time with system frame SFNSSB,i and slot nSSB,i. The symbols of the CORESET associated with pdcch-ConfigSIB1 in MIB or with searchSpaceSIB1 in PDCCH-ConfigCommon have normal cyclic prefix. In Table 13-0, configurations with index 0 to 9 are applicable when an associated SS/PBCH block is located according to Table 5.4.3.3-2 in [8-1, TS 38.101-1] or Table 5.4.3.3-1a in [8-5, 3GPP TS 38.101-5], configurations with index 10 to 11 are applicable when an associated SS/PBCH block is located according to NOTE 12 of Table 5.4.3.3-1 in [8-1, TS 38.101-1], and non-interleaved CCE-to-REG mapping applies for configurations with index 6 to 9. In Table 13-1, the associated SS/PBCH block is not located according to NOTE 12 of Table 5.4.3.3-1 in [8-1, TS 38.101-1].

*** Unchanged parts are omitted ***

## 17.2	Half-Duplex UE in paired spectrum

A half-duplex UE (HD-UE) in paired spectrum is not capable of simultaneous transmissions and receptions on a serving cell with paired spectrum. This clause is applicable for communication of a HD-UE on a serving cell with paired spectrum. Procedures for a HD-UE are same as described for a UE in all other clauses of this document unless stated otherwise.

A HD-UE that operates on a non-NTN serving cell, or a HD-UE that operates on an NTN serving cell and does not indicate CollisionHandlingOfHDFDDOperation, does not expect to detect a DCI format scheduling a reception in a set of symbols and detect a DCI format scheduling a transmission in any symbol from the set of symbols. A HD-UE that operates on an NTN serving cell in the RRC_CONNECTED state, indicates CollisionHandlingOfHDFDDOperation, is scheduled to receive a PDSCH or CSI-RS in a set of symbols based on an indication by a first DCI format, and is scheduled to transmit a PUSCH, PUCCH, or PRACH that overlap with any symbol from the set of symbols based on an indication by a second DCI format, determines

-	to either receive the PDSCH or the CSI-RS or transmit the PRACH based on the HD-UE implementation;

-	to either receive the PDSCH or transmit the PUSCH or PUCCH based on the HD-UE implementation, if the first DCI format is provided by a PDCCH the HD-UE received according to a Type0/0A/1/2-PDCCH CSS set;

-	to receive the PDSCH or the CSI-RS if the first DCI format is not provided by a PDCCH the HD-UE received according to a Type0/0A/1/2-PDCCH CSS set, and the HD-UE is not provided ntn-RedcapPrioritizeUL-Dynamic, and the transmission of the PUCCH or the PUSCH would not start before Tproc,2 after the last symbol of the PDCCH reception providing the first DCI format;

-	to transmit the PUSCH or PUCCH, if the first DCI format is not provided by a PDCCH the HD-UE received according to a Type0/0A/1/2-PDCCH CSS set, and

-	the HD-UE is provided ntn-RedcapPrioritizeUL-Dynamic, or

-	the HD-UE is not provided ntn-RedcapPrioritizeUL-Dynamic and the transmission of the PUCCH or the PUSCH would start before Tproc,2 after the last symbol of the PDCCH reception providing the first DCI format.

A HD-UE that operates on an NTN serving cell in the RRC_CONNECTED state, indicates CollisionHandlingOfHDFDDOperation, is scheduled to receive a PDSCH or CSI-RS based on an indication by a first DCI format, and is scheduled to transmit an SRS based on an indication by a second DCI format and the transmission of the SRS in a set of symbols overlaps with the PDSCH reception or CSI-RS reception, determines

-	to either receive the PDSCH or transmit the SRS based on the HD-UE implementation, if the first DCI format is provided by a PDCCH the HD-UE received according to a Type0/0A/1/2-PDCCH CSS set;

-	to receive the PDSCH or the CSI-RS if the first DCI format is not provided by a PDCCH the HD-UE received according to a Type0/0A/1/2-PDCCH CSS set, and the HD-UE is not provided ntn-RedcapPrioritizeUL-Dynamic, and the first symbol from the set of symbols is not before Tproc,2 after the last symbol of the PDCCH reception providing the first DCI format;

-	to transmit the SRS if the first DCI format is not provided by a PDCCH the HD-UE received according to a Type0/0A/1/2-PDCCH CSS set, and

-	the HD-UE is provided ntn-RedcapPrioritizeUL-Dynamic, or

-	the HD-UE is not provided ntn-RedcapPrioritizeUL-Dynamic and any symbol from the set of symbols is before Tproc,2 after the last symbol of the PDCCH reception providing the first DCI format

When a PDCCH reception by a HD-UE includes two PDCCH candidates from corresponding search space sets, as described in clause 10.1, the end of the PDCCH reception is the end of the PDCCH candidate that ends later.

*** Unchanged parts are omitted ***

A HD-UE that operates on an NTN serving cell in the RRC_CONNECTED state, indicates CollisionHandlingOfHDFDDOperation, would receive a PDCCH or would receive a PDSCH or CSI-RS or PRS based on a configuration by higher layers in a set of symbols, and would transmit a PUSCH, PUCCH, or SRS based on a configuration by higher layers that overlap with any symbol from the set of symbols, determines

-	to either receive the PDCCH or transmit the PUSCH/PUCCH/SRS based on the HD-UE implementation, if the PDCCH reception is according to a Type0/0A/1/2-PDCCH CSS set

-	to receive the PDCCH, if the PDCCH reception is not according to a Type0/0A/1/2-PDCCH CSS set, or the PDSCH, or the CSI-RS, or the PRS if the HD-UE is not provided ntn-RedcapPrioritizeUL-Semistatic

-	to transmit the PUSCH, the PUCCH, or the SRS, if the PDCCH reception is not according to a Type0/0A/1/2-PDCCH CSS set, and the HD-UE is provided ntn-RedcapPrioritizeUL-Semistatic

A HD-UE that operates on an NTN serving cell in the RRC_INACTIVE state, would receive a PDCCH in a set of symbols and would transmit a PUSCH or SRS that overlap with any symbol from the set of symbols, determines to either receive the PDCCH or transmit the PUSCH or SRS based on the HD-UE implementation.

*** Unchanged parts are omitted ***