3GPP TSG RAN Meeting #110				                  RP-253064

Baltimore, USA, December 8-11, 2025

Source:	AT&T

Title:	Requirements for New and Existing 6G Services

Agenda Item:	8.2.1.4

Document for: 	Discussion/Decision

- Introduction

At the TSG RAN #109 meeting in Beijing, China a revised SID was approved to study 6G scenarios and requirements [1]. The following was agreed to be captured in Section 5.4 of TR38.914 on requirements of New and Existing services.

5.4 Requirements of New and Existing Services ### 5.4.0 General This section describes the requirements of new and existing services.\ ### 5.4.1 Mobile Broadband ### 5.4.2 Immersive Communication ### 5.4.3 Massive Communication (IoT) The 6GR and 6G RAN architecture shall support the following minimum requirements for Massive Communication (IoT): - 6G Massive Communication (IoT) shall be supported for FR1. - 6GR should have a common/scalable design that supports the above usage scenario in addition to eMBB - Prioritize 6GR design for eMBB - The above usage scenario should not overlap with Ambient IoT and NB-IoT - [PHY or MAC] [minimum] peak data rate is [TBD] Mbps in DL and [TBD] Mbps in UL for lowest-tier device. Editor note: “6G should support coexistence with NB-IoT (all deployment modes) and eMTC via semi-static configuration” is moved to 5.2 (migration and architecture) ### 5.4.4 Sensing Editor note: More sensing use cases can be included depending on further discussion. The 6GR and 6G RAN architecture shall at least support use cases of detection and/or tracking of passive objects, at least including UAVs, human, vehicles and AGVs. ### 5.4.5 AI ### 5.4.6 Voice ### 5.4.7 Regulatory Services

In this document, we provide additional views on requirements for new and existing services in 6G. Text proposals are provided for inclusion into 3GPP TR 38.914, “Study on 6G Scenarios and Requirements”.

- Requirements for Existing 6G Services

  - Massive IoT

3GPP has been successfully addressing the Internet-of-Things (IoT) segment since LTE Release 13 with the introduction of eMTC and NB-IoT. These technologies have subsequently been evolved as evident in Table 1. Additionally, each release has also improved latency, coverage, device complexity, power consumption, mobility, and spectral efficiency.

Table 1: Evolution of eMTC and NB-IoT in 3GPP

Since the introduction of eMTC and NB-IoT, 3GPP has introduced additional IoT technologies as shown in Figure 1. IoT-NTN comprises the non-terrestrial network (NTN) complements of eMTC and NB-IoT and was introduced in Rel. 17.  Unlike eMTC and NB-IoT, which are low power wide area technologies, Ambient IoT was specified in Rel. 19 as low power short-range 3GPP technology. And lastly, RedCap introduced reduced capability devices with reduced UE complexity and capability constraints in Rel. 17. All of these are 5G technologies, however, as depicted in Figure 1, some are 5G NR technologies (38 series) while others are 5G LTE technologies (36 series), a consequence of the TSG RAN #79 agreement  that “no NR based solution is specified for Low-Power Wide-Area (LPWA) use cases.”

In this context, it is worth pointing out that many carriers no longer maintain 2G or 3G services to support the IoT segment meaning absent a 5G NR solution, the massive IoT segment is entirely served by LTE based technologies. Any strategic planning for LTE maintenance and sunset considerations is thus intimately intertwined with the technological evolution of massive IoT in 3GPP.

Figure 1: Overview of 3GPP IoT Technologies

Beyond 3GPP, there are additional considerations of utmost importance to mobile network operators. Absent a 5G NR based solution, the longevity of eMTC/NB-IoT impacts the strategic planning of LTE maintenance and sunset. Concurrently, massive IoT design-ins can span 10+ years meaning 2030 and beyond is well within today’s time horizon of day-to-day business decisions. In fact, publicly available RFPs have benefit periods of more than 20 years. The lack of a 5G NR based solution and the multi-decade longevity of massive IoT devices mean massive IoT solutions for 2030 and beyond significantly impact business decisions TODAY!

The above informs the following 6G massive IoT “must haves”:

- Long-term visibility is imperative TODAY both from a technological (e.g., LTE sunset) and business (e.g., 10+ years of benefit period) perspective

- Unlike in 4G and 5G, massive IoT requires a single global solution provided by a single 6G RAT

- Because crucial design questions can most efficiently be decided in the first release of 6G, such as the minimum bandwidth for a common design framework for all verticals, 6G massive IoT must be specified in the first release of 6G

- Lastly, solutions are needed that can last across technology generations in the core network, the RAN, and in overlapping spectrum with significantly improved efficiency compared to DSS between 5G and eMTC

Areas where massive IoT can be improved include, but are not limited to:

- Enhanced accessibility with larger number of simultaneous active connections (number of RRC connected users) compared to eMTC

- Significantly better latency in addition to higher throughputs

- Power efficiency enhancements with support for advanced discontinuous reception (DRX) cycles (e.g., eDRX > 20 minutes) and power saving mode (PSM) extensions > 1 week

- Extended coverage enhancement mechanisms beyond MaxCL of [154] dB

- Improved mobility handling with lightweight handover or RACH-less mobility support for massive IoT UEs moving slowly (e.g., <10 km/h)

- Support of optional multicast or group messaging capability for firmware updates and public safety

- Security optimizations

- UE RF requirements without a need for band specific SAW filters enabling global and low cost IoT solutions

  - Text Proposal for Massive IoT

The following text is proposed for inclusion into 3GPP TR 38.914, “Study on 6G Scenarios and Requirements”.

### 5.4.3 Massive Communication (IoT) The 6GR and 6G RAN architecture shall support the following minimum requirements for Massive Communication (IoT): - 6G Massive Communication (IoT) shall be supported for FR1. - 6GR should have a common/scalable design that supports the above usage scenario in addition to eMBB - Prioritize 6GR design for eMBB - The above usage scenario should not overlap with Ambient IoT and NB-IoT - [PHY or MAC] [minimum] peak data rate is [TBD] Mbps in DL and [TBD] Mbps in UL for lowest-tier device. - Enhanced accessibility with larger number of simultaneous active connections (number of RRC connected users) compared to eMTC - Significantly better latency in addition to higher throughputs - Power efficiency enhancements with support for advanced discontinuous reception (DRX) cycles (e.g., eDRX > 20 minutes) and power saving mode (PSM) extensions > 1 week - Extended coverage enhancement mechanisms beyond MaxCL of [154] dB - Improved mobility handling with lightweight handover or RACH-less mobility support for massive IoT UEs moving slowly (e.g., <10 km/h) - Support of optional multicast or group messaging capability for firmware updates and public safety - Security optimizations - UE RF requirements without a need for band specific SAW filters enabling global and low cost IoT solutions Editor note: “6G should support coexistence with NB-IoT (all deployment modes) and eMTC via semi-static configuration” is moved to 5.2 (migration and architecture)

- Requirements for New 6G Services

  - Sensing

Sensing use cases were introduced in SA1 in 5G-A, and the subsequent focus of sensing use cases in RAN was on object detection and tracking. In particular, the SI/WI for Rel. 20 is to study and potentially specify UAV detection and tracking with monostatic sensing.

The Rel. 19 study item on channel modeling for integrated sensing and communication for NR developed a channel model supporting object detection and/or tracking including UAVs, humans indoors and outdoors, automotive vehicles, automated guided vehicles and objects creating hazards on roads/railways. The developed channel model supports all sensing modes, including TRP-TRP bistatic and monostatic sensing, TRP-UE bistatic, UE-TRP bistatic, UE-UE monostatic and bistatic sensing.  The Rel. 19 item, however, did not consider channel modelling aspects for non-object detection and tracking use cases.

In Rel. 20, the study item currently under way in 5G-A for UAV TRP-monostatic sensing considers multiple levels of data for use in object detection evaluations. Various levels of data have various requirements in terms of data rates, required bandwidth, required latency, and required storage.  Various levels of data are needed for the same use case, depending on the location and the needs of the sensing functions. And different use cases might consider different types/levels of data to be collected in the RAN and consumed by the RAN and or the sensing function outside of the RAN.

Additional sensing use cases have been identified for 6G in TS22.870. In addition to object detection and tracking use cases, including smart transportation, UAV supervision, public safety applications, non-object detection and tracking use cases such as infrastructure collapse monitoring and other environmental monitoring use cases have been identified.

3GPP RAN#109 agreed to include at least object detection and tracking use cases for ISAC in 6G [1]:

Editor note: More sensing use cases can be included depending on further discussion. The 6GR and 6G RAN architecture shall at least support use cases of detection and/or tracking of passive objects, at least including UAVs, human, vehicles and AGVs.

Other use cases, such as infrastructure collapse monitoring and other environmental monitoring are also of interest, and can be considered, with an extension of channel modeling to evaluate these scenarios.

Proposal 1: For ISAC use cases in 6G, consider environmental monitoring use cases such as infrastructure collapse monitoring, after corresponding channel modelling and evaluation methodology have been identified.

Considering the requirements for sensing for these use cases, including probability of false alarm, probably of miss detection, and positioning and velocity accuracy, TRP-TRP bistatic/multi-static and monostatic sensing have a clear advantage of achieving the sensing KPIs for most of these use cases,  and should be at least considered for ISAC in 6G. UE-TRP bistatic sensing can also be considered in some use cases, for detection of non-aerial targets, or aerial targets within the clutter, when needed.

Proposal 2: For ISAC use cases in 6G, at least TRP-TRP monostatic and bi/multi-static sensing are considered as sensing modes.  Other UE assisted and UE based sensing modes are considered as needed.

Furthermore,  depending on the sensing use case, sensing algorithms for tracking, association and classification may be available in different parts of the network, and different sensing use cases have different requirements on levels of data needed for collection. The sensing data collection framework, including the sensing measurement data should thus be flexible enough to accommodate the sensing function availability in the RAN as well as outside of the RAN, at the OAM or the CN. Multiple levels of data can potentially be supported to accommodate various use cases as needed for object detection and tracking or otherwise.

Proposal 3: For ISAC use cases in 6G, a flexible data collection framework, integrated with other services, such as AI/ML, is supported to accommodate sensing functions in the RAN, OAM and in the CN.

  - Text Proposal for Sensing

The following text is proposed for inclusion into 3GPP TR 38.914, “Study on 6G Scenarios and Requirements”.

# 5.4.4​ Sensing

The 6GR and 6G RAN architecture shall support at least TRP-TRP monostatic and TRP-TRP bi/multi-static sensing modes for object detection and tracking, using  a flexible data collection/reporting framework.

  - AI

When 5G was specified in Rel. 15, AI/ML use cases and their related lifecycle management and corresponding data collection framework were not considered as part of the 5G design. AI/ML models and use cases are however continuously emerging across multiple layers of the network architecture, across multiple domains, including the UE, RAN, CN. and yet their specification in 5G-advanced is being retrofitted into the 5G architecture and design.

This contribution emphasizes the necessity of an integrated framework for AI/ML models data collection, data transfer and delivery, and performance monitoring from the onset of 6G. Such a framework is essential for improving network efficiency, interoperability, vendor uniformity, and scalability while addressing critical security and privacy concerns.

An integrated AI/ML framework enables MNOs to manage network capacity effectively, fostering innovation, and unlocking new value opportunities. A unified approach to training and deploying AI/ML models supports continuous innovation in model development, ensuring the network evolves dynamically. Furthermore, a uniform life cycle management (LCM) framework is crucial for running multiple models simultaneously across different layers, meeting diverse service requirements end-to-end.

To realize this vision, the following core principles should be included for AI/ML framework design in 6G:

- A unified flexible LCM framework for model management, model transfer, model training, and model testing

- A unified data collection framework to enhance management efficiency

- Network visibility to drive innovation while proactively addressing security and privacy concerns

- Network control over data collection to ensure network performance is not impacted while providing potential new value opportunities via hosting/routing/augmenting the data

- Scalability to accommodate various emerging and future use cases.

The design requirements for AI/ML data traffic management needed to support these principles are as follows:

- MNO visibility and control of AI/ML based data traffic (model transfer/delivery, data collection, model monitoring and management)

- Universal framework to avoid fragmentation and per-use case specifications and ensure scalability to new AI/ML use cases

- Support multiple termination points for AI/ML data within the network with MNO visibility

- Differentiate AI/ML data traffic from user plane traffic and control plane traffic

- Ensure data traffic security and privacy

Towards these design requirements, 6G RAN should study various options to realize a consistent data management framework in 6G.

  - Text Proposal for AI

We propose the following text to the 6G Scenarios and Requirements TR 38.314.

5.4.5 AI

- Define an integrated framework for AI/ML data management in 6G, based on the following design principles:

  - MNO visibility and control of AI/ML based data traffic (model transfer/delivery, data collection, model monitoring and management)

  - Universal framework to avoid fragmentation and per-use case specifications and ensure scalability to new AI/ML use cases

  - Support multiple termination points for AI/ML data within the network with MNO visibility

  - Differentiate AI/ML data traffic from user plane traffic and control plane traffic

  - Ensure data traffic security and privacy

- Define a unified and flexible LCM framework for model management, model transfer, model training, and model testing

- Conclusion

In this document, additional views were provided on requirements for new and existing services in 6G. Text proposals are provided for inclusion into 3GPP TR 38.914, “Study on 6G Scenarios and Requirements”. Additionally, the following is proposed.

Proposal 1: For ISAC use cases in 6G, consider environmental monitoring use cases such as infrastructure collapse monitoring, after corresponding channel modelling and evaluation methodology have been identified.

Proposal 2: For ISAC use cases in 6G, at least TRP-TRP monostatic and bi/multi-static sensing are considered as sensing modes.  Other UE assisted and UE based sensing modes are considered as needed.

Proposal 3: For ISAC use cases in 6G, a flexible data collection framework, integrated with other services, such as AI/ML, is supported to accommodate sensing functions in the RAN, OAM and in the CN.

# References

- RP-250810, Revised SID: Study on 6G Scenarios and requirements, CMCC, Verizon, NTT DOCOMO, Deutsche Telekom