3GPP TSG RAN Meeting #110					RP-253093

Baltimore, USA, Dec 8-11, 2025

Agenda Item:	8.2.1.4

Source:	Ericsson, Airbus, AT&T, BDBOS, BMW, Deutsche Telekom, FirstNet, Erillisverkot, KT Corp., Motorola Solutions, Nokia, NTT DOCOMO, Softil, SyncTechno Inc, T-Mobile USA, Telstra

Title:	Requirements for regulatory services and positioning

Document for:	Approval

# 1	Introduction

In this contribution, we discuss the requirements for supporting regulatory services and positioning. It is key to have these supported in 6G RAN and Radio in the initial release.

# 2	Discussion

## 2.1	Emergency communications

Emergency communication is a basic feature that needs to be part of the Day-1 deployment to allow standalone deployment. It needs to support E911, 112 calling and other similar regulations. This includes supporting emergency voice and all the associated capabilities needed by regulation in order for a 6G system to be deployed without having callers required to fallback to previous generations of technology for an emergency voice call.

## 2.2	Public warning/emergency alert systems

Like earlier generations, 6G needs to support Public Warning System (PWS). Examples of such systems to be supported are Earthquake and Tsunami Warning System (ETWS), EU-Alert, Korean Public Alarm System (KPAS), and Commercial Mobile Alert System (CMAS)/Wireless Emergency Alerts (WEA). It is expected that 6G RAN and 6G Radio support PWS in the initial release.

## 2.3	Location services

There exist different regulations that require location services to be available for emergency purposes (e.g., E911 services in the US). These requirements will have to be met by the 6G System.

An example of one such regulation is E911. According to E911 requirements, service providers must supply either a dispatchable location or horizontal coordinates that meet specific accuracy benchmarks (50 meters horizontal radius from the caller’s true horizontal position), along with vertical (z-axis) data that identifies the caller’s floor level in indoor settings. The FCC requires z-axis accuracy within ±3 meters for 80% of E911 calls from z-axis–capable devices [1].

Although the IMT-2030 discussions are not yet finalized, there will be a TPR on positioning. The 3GPP submission towards ITU-R WP5D will have to fulfil this requirement.

When it comes to positioning, 3GPP has studied and specified many techniques over the last two decades (for 5G NR, see [2]). To ensure that positioning is available in Rel-21 it is important to minimize the UE impact while providing sufficient performance in both indoor and outdoor environment. The positioning techniques A-GNSS (Assisted GNSS) and E-CID (Enhanced Cell ID) are good candidates for meeting the positioning requirements set by emergency location service and IMT-2030 [2]. E-CID is based on reporting cell ID and reuses the existing RRM measurements for positioning or mobility measurements, together with network measurements such as Timing Advance and Angle of Arrival. Both further have the key benefits of minimizing UE impact. In addition, it seems feasible to natively support both techniques with the 6G RAN architecture.

Given this and the amount of previous work done in 3GPP, we do not see any need to study which positioning method to support within Rel-20. However, during Rel-21 timeframe there will be need to evaluate the performance as part of the IMT-2030 evaluation processes.

## 2.4	Support for eCall

In Europe and other regions, the support for eCall functionality in cars is mandatorily required. From 1.1.2026 onwards each new car sold in Europe needs to support NextGeneration eCall packet switch based [3]. In order to be prepare in 6G from the outset, the support of 6G-eCall is expected in 6G RAN and 6G Radio in the initial release.

# 3	Text proposal

Based on the discussion above we have several text proposals for 38.914.

## 5.4.6 Positioning The 6GR and 6G RAN shall support positioning. The positioning methods A-GNSS and E-CID shall be supported by the 6GR and the 6G RAN. ### 5.4.7 Regulatory Services The 6GR and the 6G RAN shall provide mechanisms to enable emergency calls , emergency location service, Public Warning System (PWS), 6G-eCall (for automotive) and Lawful Interception (LI).

# 4	Conclusion

In this contribution we propose to add the text proposals in section 3 to TR 38.914.

# References

- Federal Communications Commission. (2021). Fourth Report and Order, Wireless E911 Location Accuracy Requirements https://www.911.gov/assets/Wireless-E911-Location-Accuracy-Requirements-1638567121.pdf

- TS 38.305 	NG Radio Access Network (NG-RAN); Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN V19.0.0

- COMMISSION DELEGATED REGULATION (EU) 2025/1871 
https://eur-lex.europa.eu/legal-content/EN/TXT/HTML/?uri=OJ:L_202501871