3GPP TSG RAN Meeting #110	                                               	RP-253110

Baltimore, USA, December 8-11, 2025

Agenda Item:	8.2.1.4

Source:	OPPO

Title:	Discussion on 6GR for AI service

Document for:	Discussion

Introduction

Constructing a next-generation wireless communication system to support the rapidly growing of AI services is one of the critical challenges as well as opportunities in 6GR development. This topic has triggered extensive discussions among multiple 3GPP working groups in the past few meetings, including both RAN-side and SA-side studies.

In this contribution, we aim to discuss the key transformations brought by AI to wireless systems beyond the scope of AI4Net, the typical use cases of new AI services, the understanding on AI traffic model and the interdependencies across different WGs. The potential specification impacts in RAN1 and RAN2 are analyzed as well.

Discussion

Key transformations brought by AI to 6GR

The evolution from 1G to 5G has involved not only providing a stable and reliable "bit transmission pipeline" but also introducing growing capability to identify diverse services and their specific requirements, to create a better system.

In 5G, QoS mechanisms are used to differentiate between typical services such as video and voice, each with distinct transmission demands. Besides, network slicing is also proposed and allows transmission solutions to be flexibly combined on demand. These technologies have significantly enhanced the flexibility and adaptability of system architecture and transmission scheme design.

However, with the continuous emergence of new services anticipated for 6G, particularly those driven by AI, current methods of service identification and classification may be hard to keep pace. Challenges such as overly broad categorization, as well as limited adaptability to novel services and requirements (e.g., to various downstream tasks) are becoming apparent.

In 6G era, from our understanding, following transformations may occur and provide potential for the evolution of the new communication paradigm:

- Intelligent Pre-processing that extracts fine-grained source features. For example, pre-processing techniques like AI codec or AI tokenization may parse the internal structure, feature and importance of different sources.

- Powerful Post-processing that handles transmission errors and adapts to various downstream tasks. For example, advanced receiver algorithms, represented by Large Models, can effectively recover key/semantic information from impaired or incomplete data to serve downstream applications.

These technological advancements allow us to potentially relax the requirement that the transmission pipeline "must guarantee 100% lossless bit-level transmission", thereby pursuing higher efficiency. Accordingly, the core design of 6G may shift from "bit quality centric" to "service quality centric", including:

- Shift in Objectives: The goal of communication evolves from ensuring bit-level accuracy to guaranteeing semantic-level effectiveness.

- Relaxation of Constraints: Leverage source pre-processing and post-processing to relax constraints on transmission design.

Observation1: 6GR design may shift from "bit quality centric" to "service quality centric", including:

- Shift in Objectives: The goal of communication evolves from ensuring bit-level accuracy to guaranteeing semantic-level effectiveness.

- Relaxation of Constraints: Leverage source pre-processing and post-processing to relax constraints on transmission design.

New AI traffic in 6GR

- Communication system design with service-oriented AI pre/post processing

In the 6G era, with the proliferation of diverse AI services, enhanced AI capabilities, and deep understanding of service requirement, integrating the impact of service-oriented AI pre/post-processing into communication system design could be a new communication paradigm, e.g., it could be featured by a kind of token communication, as shown in Figure 1.

In 3GPP studies, token could be simply and broadly treated as the minimum unit of data generated/needed in different AI services, e.g., the minimum unit for model inputs and outputs, or the minimum unit for AI encoding and decoding, and be transmitted over the air.

Observation 2: In 3GPP studies, token could be simply and broadly treated as the minimum unit of data generated/needed by different AI services, and be transmitted over the air.

- An example for token communication

Figure 2 illustrates a typical token communication flow:

- UE tokenizes the source data (e.g., an image) and then transmits the tokens over the air.

- The receiver de-tokenizes the tokens to recover the information, which is then fed into an AI service (e.g., a classification task).

- For AI services, the essential requirement is service performance (e.g., classification accuracy), not 100% correct data transmission. Note that: Non-100% data correctness ≠ Failure to meet service requirements.

In this section, simulations on token communication are shown, based on the ImageNet dataset (with 3000 images, belonging to 1000 categories) and a classification task. Each image is converted into 256 tokens (each token is 12 bits), and the impact of token errors or token loss on the classification accuracy is evaluated, e.g., if the Top-1/Top-5 predicted categories for an image contain the true category of the image, it is determined that the image has been classified correctly.

Characteristic 1: Error-Tolerant

As shown in Figure 3, the classification accuracy shows significant tolerance to token errors. When errors are randomly distributed across all 256 tokens, different Token Error Rates (TER) significantly impact classification accuracy. For instance, if the requirement is only to ensure Top-5 accuracy > 90%, the system can tolerate a TER as high as 20% (TER=0.2). Correspondingly, identifying and leveraging such error tolerance characteristics enables the design of more efficient transmission schemes tailored to service needs, avoiding unnecessary redundant transmission protection mechanisms.

- The relationship between token error rate and classification accuracy

Characteristic 2: Token importance

Some advanced Tokenization schemes can generate token representations differentiated by importance. For example, the 256 tokens can be numbered from 0 to 255 in descending order of importance; a lower number indicates higher criticality for information expression, while a higher number corresponds to detail information. In this case, as shown in Figure 4, if the goal is merely to ensure Top-5 accuracy > 90%, transmitting only the most critical first 25% of tokens can achieve it. This selection based on token importance, which involves discarding non-critical tokens as needed, also helps build efficient transmission schemes, reducing the amount of data transmitted and improving efficiency.

- The relationship between token transmission rate and classification accuracy

Observation 3: For AI traffic, e.g., represented by token communication, following characteristics should be further studied:

- Tolerance to transmission errors.

- Tolerance to only select and transmit partial data(e.g., partial tokens) with high importance.

Potentially 6GR enhancement for AI services

To efficiently support such new services, studies are needed in RAN aspect. The core challenges and potential impacts may include:

Traffic type and traffic model for AI services

Regarding AI traffic types, a basic classification could consider the transmission of text, audio, image, video and XR involved in different AI services. This includes the transmission of the original source data, as well as the transmission of such data after AI processing (e.g., via AI codec or AI tokenizer).

Besides, in addition to distinguishing the type of transmitted source data, it is also necessary to consider the transmission requirements imposed by the different AI services themselves. For a specific source data (such as image, or video), different downstream tasks may introduce varying transmission requirements, which in the end lead to different transmission schemes.

When defining a traffic model for AI services, the factors above should be considered. Unlike existed traffic models, the traffic model for AI service may be specifically tailored to different AI traffic types. Characteristics such as packet size, packet arrival, and packet importance may vary significantly across different AI traffic types. Additionally, the traffic model should incorporate transmission requirements decided by different AI service themselves. For example, definitions of packet delay budget and success rate could be specified using a set of typical values to accommodate potentially diverse AI service requirements.

Observation 4: When defining a traffic model for AI services,

- Characteristics such as packet size, packet arrival, and packet importance may vary significantly across different AI traffic types.

- Traffic model should incorporate transmission requirements decided by different AI services.

Discussions on AI services, particularly regarding the traffic model and characteristic, are ongoing across RAN1, RAN2, SA1, SA2, and SA4. The relationship between RAN1/RAN2 studies and SA4 contributions has been a key topic in recent meetings.

Based on the LS reply and related discussions in SA4#134[2], SA4 suggests RAN1 to take their current information and any future information into account when defining detailed realistic traffic models. This information will undoubtedly benefit the research in RAN1 and RAN2, when available. However, this does not imply that RAN side studies should halt. Our view is a wait in RAN is unnecessary.

As an important and new area, in parallel with SA-side study, RAN-side could also begin related studies, e.g., by establishing a general framework for various AI traffic models. The framework could highlight key AI traffic characteristics identified through RAN research. At this stage, there is no immediate need to determine or down-select whether we need a new traffic model, an enhanced XR traffic model, or an enhanced FTP model. The purpose of the framework is to set aside such nominal distinctions and focus research on the AI traffic characteristics identified by companies. For such a framework, not every AI service necessarily requires all the features within it, e.g., some traffic may not need to consider the PDB, some may not need to consider the jitter. Within the identified general framework, companies could continue evaluating 6GR RAN designs for AI services using their own, temporarily non-standardized, traffic model first. When SA4 inputs available, the detailed values for different traffic types and traffic model parameters could be updated accordingly.

Besides, according to our understanding, another key consideration is that AI services are often characterized by non-standardization solutions and the presence of proprietary algorithms and codecs. This contrasts with traditional services and codecs (e.g., only a few standardized codec for video or image). Defining a set of typical candidate values to cover diverse AI service requirements is likely more practical than defining a specific traffic model solely based one or a few standardized AI codec.

Proposal 1: When defining a traffic model for AI services,

- A general framework for various AI traffic models should be considered

- A set of typical candidates to cover diverse AI service requirements should be considered

Service awareness in RAN

SA1 has begun discussing the service requirements for different services in 3GPP TR 22.870, e.g., for error tolerant communication. In TR 22.870, some use cases (e.g., in clause 6.7, 6.28, 9.11, 9.14) have been agreed on how to release QoS (e.g., reliability) while maintaining deterministic QoE for some real time service, e.g., potentially falling under the 99.99% to 50% reliability range proposed in SA1 TR 22.870 clause 9.11. Furthermore, SA2 is discussing whether and how the adaptive QoS mechanisms for AI characteristics traffic e.g. under work task 3.2. The corresponding impacts in RAN may also need to be evaluated.

As discussed in RAN3, service awareness refers to enabling the RAN to acquire and use relevant information to appropriately handle a service. For various AI services, RAN may need more precise awareness of service characteristics compared to the past (e.g., more precise classification information, priority information, importance information).

Specification impact on UP

RAN2 has discussed the service/application awareness in the user plane design. The intended design target for 6G user plane protocol stacks is to have a more generic framework for the service/application awareness in L2, so that the UP architecture is able to adapt to diverse service/application requirements. Considering that the AI services, e.g., token-based communication, may have the traffic transmission characteristics of error-tolerance for different tokens, the L2 needs to have differentiated handling for different tokens for a specific service/application so as to achieve different reliability requirements for different tokens. Given that different tokens have different importance, the L2 transmission should also prioritize the transmission of some tokens which are more important than other tokens. Additionally, as the tokens are generated from different types of source files (e.g. (real-time) video/audio), the delay budget or priority of the source file should also be considered for the token transmission.

Specification impact on scheduling and HARQ

Given the inherent error tolerance potentially related to AI services, a key consideration in PHY is whether and how to tolerant transmission errors, without impacting the user experience.

Taking token-based communication as an example, identifying token errors is a new challenge. Existing systems use TB or CBG as the granularity for ACK/NACK feedback. A single TB or CBG may contain a large number of tokens (e.g., 256 tokens for one image). Even an error in just one token, even within a tolerated token error rate of a target service, can trigger the retransmission of the entire TB or CBG, which is inefficient.

Figure 5 shows the mapping of different token error rates to the corresponding BLER under different transmission conditions[1]. It can be observed that originally acceptable token error rates can lead to high or even completely erroneous TB error rates. It is necessary to consider introducing new mechanisms at the physical layer, such as direct identity or indirect mapping of token errors, to effectively identify errors at the token granularity and design matched and effective scheduling/retransmission schemes.

For example, if we set a 2% BER(corresponds to about 20% TER in our simulation) as an error tolerance threshold, where TBs with BER less than or equal to this threshold are not retransmitted, while those exceeding it are retransmitted, this simple approach shows the potential to achieve approximately 8% system throughput gain. Furthermore, a higher error tolerance threshold, combined with more effective scheduling/HARQ strategies, may lead to more TP gains.

(a) MCS=3                                                                                            (b) MCS=8

(c) MCS=13                                                            	                         (d) MCS=25

- Mapping of different Token Error Rates (TER) to the corresponding BLER and BER under different transmission conditions

Observation 5: To efficiently support AI services, studies are needed in RAN1 and RAN2. The core challenges and potential impacts may include:

- Service awareness in RAN

- Specification impact on UP, scheduling and HARQ

Conclusion

In this contribution, we have discussed the transformations brought by AI to 6G, as well as the understanding on AI traffic model and potential specification impacts in RAN, and got the following proposals/observations:

Observation1: 6GR design may shift from "bit quality centric" to "service quality centric", including:

- Shift in Objectives: The goal of communication evolves from ensuring bit-level accuracy to guaranteeing semantic-level effectiveness.

- Relaxation of Constraints: Leverage source pre-processing and post-processing to relax constraints on transmission design.

Observation 2: In 3GPP studies, tokens could be simply and broadly treated as the minimum unit of data generated/needed by different AI services, and be transmitted over the air.

Observation 3: For AI traffic, e.g., represented by token communication, following characteristics should be further studied:

- Tolerance to transmission errors.

- Tolerance to only select and transmit partial data(e.g., partial tokens) with high importance.

Observation 4: When defining a traffic model for AI services,

- Characteristics such as packet size, packet arrival, and packet importance may vary significantly across different AI traffic types.

- Traffic model should incorporate transmission requirements decided by different AI services.

Observation 5: To efficiently support AI services, studies are needed in RAN1. The core challenges and potential impacts may include:

- Service awareness in RAN

- Specification impact on UP, scheduling and HARQ

Proposal 1: When defining a traffic model for AI services,

- A general framework for various AI traffic models should be considered

- A set of typical candidates to cover diverse AI service requirements should be considered

References

- R1-2508732, AI/ML use cases for 6GR air interface

- S4-252135, Reply LS on traffic model study in RAN1