3GPP TSG RAN Meeting #110		RP-253118

Baltimore, USA, December 8-11, 2025

Source: 	CMCC

Title:	Discussion on 3GPP Coverage targets and evaluation methodologies

Agenda item:	8.2.1.1

Document for:	Discussion & Decision

Introduction

In the last RAN plenary and RAN1 meeting, the following agreements have been achieved [1].

Agreement in RAN#109 - Following points were endorsed in RP-252883 in RAN#109 - For 3GPP internal evaluation, link budget is used as the evaluation methodology for coverage when applicable - For 3GPP internal evaluation, the target for coverage is to be determined by RAN - FFS: Exact coverage target value(s) - FFS: Additional details considering control/data channel

In the recent RAN1 meetings, the following agreements have been achieved [2][3][4].

Agreement in RAN1#122 - On enhanced overall coverage, identify coverage target(s) considering diverse use cases and device types Agreement in RAN1#122bis (the complete agreement can be found in the Annex) For link budget template, consider the following candidates: - Candidate 1: Reusing the link budget template from TR38.830, i.e., the following table with notes as follows: - The values of the parameters are TBD. - MCL in row (22bis) is TBD. - FFS: whether/how/why to update ---------The table from TR38.830 is omitted and can be found in Annex------ - Candidate 2: Template as Table 7.10.1-1 from TR38.913. - FFS: whether/how/why to update. ---------The table from TR38.913 is omitted and can be found in Annex------ Agreement in RAN1#122b - RAN1 provides methodology and corresponding initial analysis of potentially achievable coverage to RAN#110 to determine the coverage target(s) Agreement in RAN1#123 - For the RAN1 study of “Re-use of existing 5G mid-band (~3.5GHz) site grid for 6G deployments in at least around 7 GHz and targeting comparable coverage to 5G mid-band”, - The link budget template candidates 1 and 2 are used to calculate the metric(s) as starting point to compare existing 5G mid-band and 6G deployments in at least around 7 GHz, with potential future update. - During initial access/random access - Coverage target is referring the bottleneck channel (i.e. Rel-15 NR Msg3) during initial access/random access for existing 5G mid-band - FFS target value(s) of data rate for data channels relative to 5G mid-band - Following deployment scenarios are considered - Urban macro (both O2I and outdoor) - Sub-urban macro (both O2I and outdoor) - Following carrier frequencies are considered to calculate the metric(s) - [4 GHz] as the existing 5G mid-band - 7 GHz as 6G deployment - Template in R1-2509615 is to be used for collecting inputs on the values from companies.

In this contribution, we provide our views on coverage targets and methodologies for 3GPP internal evaluations.

Discussion

Discussion on the coverage targets

According to the SID, there are two objectives related to coverage:

- Enhanced overall coverage, focus on cell-edge performance and UL coverage

- Re-use of existing 5G mid-band (~3.5GHz) site grid for 6G deployments in at least around 7 GHz and targeting comparable coverage to 5G mid-band

For the enhanced overall coverage, in TR 38.913 MaxCL, is used to define the coverage target. MaxCL is independent of the carrier frequency, base station implementation (e.g. antenna configuration). Using MaxCL as metric can also compare the coverage target between 5G and 6G. So We propose to use MaxCL as metric to define the overall coverage target.

Proposal 1: Use MaxCL as metric to define the overall coverage target in TR38.914.

-	UL MaxCL = UL Max Tx power - eNB Sensitivity

-	DL MaxCL = DL Max Tx power - UE Sensitivity

Regarding the target value, 6G target should be not lower than 5G. So we propose 143dB MaxCL with at leat [1]Mbps for DL and [30kbps] for UL for the basic coverage. And for the lowest-tier device, i.e. IOT device, 10dB coverage extension should be supported compared to MBB.

Proposal 2:

- For a basic coverage characterized by a downlink data rate of at least [1Mbps] and an uplink data rate of at least [30kbps], the target on maximum coupling loss is MaxCL = 143dB.

- For extended coverage characterized by a downlink data rate of at least [100kbps] and an uplink data rate of at least [3kbps], the target on maximum coupling loss is MaxCL = 153 dB.

For the objective of “Re-use of existing 5G mid-band (~3.5GHz) site grid for 6G deployments in at least around 7 GHz and targeting comparable coverage to 5G mid-band”, RAN1#123 discussed the methodology to evaluate coverage, and made following agreements:

Agreement - For the RAN1 study of “Re-use of existing 5G mid-band (~3.5GHz) site grid for 6G deployments in at least around 7 GHz and targeting comparable coverage to 5G mid-band”, - The link budget template candidates 1 and 2 are used to calculate the metric(s) as starting point to compare existing 5G mid-band and 6G deployments in at least around 7 GHz, with potential future update. - During initial access/random access - Coverage target is referring the bottleneck channel (i.e. Rel-15 NR Msg3) during initial access/random access for existing 5G mid-band - FFS target value(s) of data rate for data channels relative to 5G mid-band - Following deployment scenarios are considered - Urban macro (both O2I and outdoor) - Sub-urban macro (both O2I and outdoor) - Following carrier frequencies are considered to calculate the metric(s) - [4 GHz] as the existing 5G mid-band - 7 GHz as 6G deployment - Template in R1-2509615 is to be used for collecting inputs on the values from companies.

Following the SID and the conclusions from RAN1, trying to reuse the 5G mid-band site grid for 6G deployments, the carrier frequency used in 5G network should be identified. In China Mobile’s network, 2.6GHz is widely deployed as the 5G mid-band with over 600000 base stations. And in the current commercial deployment of 5G NR, though the carrier frequencies are different, the products of 2.6GHz and 3.5GHz shares almost same design of base station products, including transmit power, TxRU numbers, antenna element deployment and numbers, the mapping between antenna elements and TxRU. Considering the amount of 5G NR commercial deployments, both 2.6GHz and 3.5GHz should be considered as the baseline for overage comparison. In Annex, we provide initial coverage gap analysis between 2.6GHz/3.5GHz and 7GHz.

Proposal 3: Re-use of existing 5G mid-band (e.g. 2.6GHz, 3.5GHz) site grid for 6G deployments in at least around 7 GHz and targeting comparable coverage to 5G mid-band. Comparable coverage should take both initial access procedure and target data rate into account.

Text Proposal

======================Start of Text Proposal========================

## 5.1.20	Coverage

6GR aims to support enhanced overall coverage, focus on cell-edge performance and UL coverage; and re-use of existing 5G mid-band (e.g. 2.6GHz, 3.5GHz) site grid for 6G deployments in at least around 7 GHz and targeting comparable coverage to 5G mid-band

Link budget and/or link level analysis are used as the evaluation methodology.

### 5.1.20.1	Overall coverage

The coupling loss is defined as the total long-term channel loss over the link between the UE antenna ports and the eNode B antenna ports, and includes in practice antenna gains, path loss, shadowing, body loss, etc. The MaxCL is the limit value of the coupling loss at which the service can be delivered, and therefore defines the coverage of the service. The MaxCL is independent of the carrier frequency. It is defined in the UL and DL as:

-	UL MaxCL = UL Max Tx power - eNB Sensitivity

-	DL MaxCL = DL Max Tx power - UE Sensitivity

The MaxCL is evaluated via link budget analysis (supported by link level simulations). The proposed MaxCL calculation template is given in following table 5.1.20-1:

Table 5.1.20-1: MaxCL calculation template

| Item | Value |
| --- | --- |
| - Transmitter |  |
| - (1) Tx power (dBm) |  |
| - Receiver |  |
| - (2) Thermal noise density (dBm/Hz) |  |
| - (3) Receiver noise figure (dB) |  |
| - (4) Interference margin (dB) |  |
| - (5) Occupied channel bandwidth (Hz) |  |
| - (6) Effective noise power - = (2) + (3) + (4) + 10 log(5) (dBm) |  |
| - (7) Required SINR (dB) |  |
| - (8) Receiver sensitivity - = (6) + (7) (dBm) |  |
| - (9) MaxCL - = (1) - (8) (dB) |  |

The following assumptions are used.

Table 7.10.1-2: MaxCL calculation assumptions

| - UE Tx power | - 23dBm |
| --- | --- |
| - DL Tx power | - 46dBm |
| - eNB receiver noise figure | - 5dB |
| - UE receiver noise figure | - 9dB |
| - Interference margin | - 0dB |

For a basic coverage characterized by a downlink datarate of at least [1Mbps] and an uplink datarate of at least [30kbps] for stationary users, the target on maximum coupling loss is 143dB. At this coupling loss relevant downlink and uplink control channels should also perform adequately.

For an extended coverage characterized by a downlink datarate of at least [100kbps] and an uplink datarate of at least [3kbps] for stationary users, the target on maximum coupling loss is 153dB. At this coupling loss relevant downlink and uplink control channels should also perform adequately.

======================End of Text Proposal========================

- Reference

- MeetingReport_RAN_109, 3GPP TSG RAN Meeting #109, Beijing, China, September 15-18, 2025

- Chair notes RAN1#122, 3GPP TSG RAN WG1 #122, Bengaluru, India, Aug 25th – 29th, 2025

- Chair notes RAN1#122bis, 3GPP TSG RAN WG1 #122bis, Prague, Czech, Oct 13th – 17th, 2025

- Chair notes RAN1#123, 3GPP TSG RAN WG1 #123, Dallas, USA, Nov 17th – 21st, 2025

- R1-2007501, Final Report of 3GPP TSG RAN WG1 #102-e v1.0.0, 3GPP TSG RAN WG1 Meeting #103-e, e-Meeting, October 26th – November 13th, 2020

- Annex

6G BS working at 7GHz will experience higher propagation loss and penetration loss compared with 5G at 2.6GHz and 3.5GHz. And since almost 80% traffic happens in the indoor scenario and served by outdoor Macro cells, the O2I penetration loss should be considered for the coverage enhancements. Based on TR38.901, we compared the additional loss induced by the higher operation frequency as below.

Table 8.2-1 Additional propagation loss at 7GHz compared with 2.6GHz and 3.5GHz

|  | Additional loss at 7GHz |  |
| --- | --- | --- |
| Scenarios | 2.6GHz | 3.5GHz |
| LOS/NLOS pathloss (dB) | 8.602 | 6.02 |
| pathloss+ penetration loss low loss model (dB) | 9.964 | 6.94 |
| pathloss+ penetration loss high loss model (dB) | 15.598 | 10.528 |

Larger number of the antenna elements can compensate parts of the additional loss. Both traffic channel and common control channels should be considered for the coverage evaluations. The additional gain provided by the larger antenna array is listed below. Traffic channels can fully use the antenna array gains. But for the common control channels, 3dB and 6dB beamforming loss are assumed, since they are required to serve more UEs.

Table 8.2-2 Additional gain brought by higher antenna element number

|  |  | 2.6GHz | 3.5GHz | 7GHz | 7GHz |
| --- | --- | --- | --- | --- | --- |
| BS antenna elements |  | 192 | 192 | 1024 | 2048 |
| Antenna array gain for traffic channel |  | 22.83 | 22.83 | 30.10 | 33.11 |
| Additional gain from BS antenna array for traffic channel |  |  |  | 7.27 | 10.28 |
| Additional gain from BS antenna array for common channel | Option 1: 3dB loss compared with traffic channel |  |  | 4.27 | 7.28 |
|  | Option 2: 6dB loss compared with traffic channel |  |  | 1.27 | 4.28 |

Table 8.2-3 Preliminary coverage gap for 7GHz with 2048 antenna elements

| 7GHz-2048 antenna element-coverage gap |  |  |  |
| --- | --- | --- | --- |
| Coverage gap compared with 2.6GHz |  |  |  |
|  | O2O | O2I-low loss | O2I-high loss |
| Traffic channel | -1.68 | -0.32 | 5.32 |
| Common control channel with option 1 | 1.32 | 2.68 | 8.32 |
| Common control channel with option 2 | 4.32 | 5.68 | 11.32 |
| Coverage gap compared with 3.5GHz |  |  |  |
|  | O2O | O2I-low loss | O2I-high loss |
| Traffic channel | -4.26 | -3.34 | 0.25 |
| Common control channel with option 1 | -1.26 | -0.34 | 3.25 |
| Common control channel with option 2 | 1.74 | 2.66 | 6.25 |

* No additional gain from UE side is considered including higher number of antenna element or higher transmit power.

As illustrated in the above table, 2048 antenna elements are assumed at gNB side, parts of the additional propagation loss can be compensated. But it may not be sufficient.

If the 2.6GHz NR site grids are reused, the traffic channel of 7GHz in O2O coverage and O2I with low penetration loss model may still have a similar coverage performance as NR. But for the traffic channel in O2I high penetration loss, 5.32dB coverage gap would be observed. For the control channel, if 2.6GHz NR site grid are reused, all the control channels for both O2O and O2I scenario, coverage gaps from 1.32dB to 11.32 dB can be observed.

If the 3.5GHz NR site grids are reused, the traffic channel in O2O and O2I with low penetration loss would still have the coverage margin in 7GHz. 0.25dB coverage gap can be observed for traffic channel in the O2I scenario with high penetration loss. With the assumption of 3db antenna gain loss in option 1, O2O and O2I with low penetration loss has a slight margin for coverage. But with high penetration loss, the coverage gap for control channel can be observed when reusing the 3.5GHz NR site grids. With the assumption of 6dB antenna loss in option 2, the coverage gap would be observed for all the O2O and O2I scenarios.