3GPP TSG-RAN WG Meeting #110       	                       			RP-253127

Baltimore, USA, December 8-11, 2025

Agenda item:	8.2.1.3.3

Source:	CMCC

Title:	Requirements of 6G Self-Organizing Networks

WID/SID:	FS_6G_RAN_Scen_Req

Document for:	Discussion

Background

The design of 6G scenarios and requirements is expected to support ease of operation and self-organization [1]. This document discusses the principles and proposed supported features in 6G Day 1.

Discussion

Principles

To simplify the network operation, self-organization network (SON) was introduced first in LTE to meet the requirements on network management/operation simplicity and achieve cost efficiency, and it had verified its value in the initial network deployment and configuration of LTE/5G. For 6G, SON also seems to be one of the important functions to achieve operation cost efficiency, and enable real-time adaptability to the evolving wireless channel environment. Therefore, it is necessary to support SON function in 6G Day 1.

Observation 1: SON function is necessary to be supported in 6G Day 1 to simplify network operation and achieve network deployment cost efficiency.

In 5G, the following requirements are captured in TR 38.913 for self-organization.

The RAN design for the Next Generation Radio Access Technologies shall be designed to fulfill the following requirements: - RAN shall support the deployment of RAN SON functions in a hybrid manner (distributed and centralized). - Collaboration and coordination among RAN SON functions need to be addressed. - User / application level QoS and QoE monitoring capability by UEs and network elements shall be supported.

In 6G, at least the above requirements shall be supported.

Proposal 1: Self-organizing function shall be designed to at least fulfill the following requirements:

- RAN shall support the deployment of RAN SON functions in a hybrid manner (distributed and centralized).

- Collaboration and coordination among RAN SON functions need to be addressed.

- User/application level QoS and QoE monitoring capability by UEs and network elements shall be supported.

Supported features in 6G Day 1

Automatic Neighbor Relation (ANR) function is designed to relieve the operator from the burden of manually managing NCRs. It resides in the base station and manages the Neighbour Cell Relation Table (NCRT). Two functions are supported in ANR: Neighbour Detection Function and Neighbour Removal Function. Neighbour Detection Function finds new neighbours and adds them to the NCRT. Neighbour Removal Function removes outdated NCRs.

By automatically adding or removing neighbor cells, ANR can significantly reduce the burden on operators manually manage neighbor relations, thereby enhancing the efficiency and flexibility of network operation and deployment.

Observation 2: In the initial 6G network deployment phase, ANR can be utilized to achieve automated neighbor cells addition/removal, which can enhance the efficiency and flexibility of network operation and deployment.

In 5G, one of the first introduced SON solutions in standards was the ANR functionality, which enables the network automatically generate relations between base stations, and the value of ANR was proven in the 5G network deployment phase.

Observation 3: ANR was one of the first SON solutions in 5G, it assisted the network automatically generated relations between base stations and simplify the network management/operation.

In addition to ANR, we believe that in the initial phase of 6G network deployment, there is another function that needs to be considered, i.e. minimization of Drive Tests (MDT). MDT function enables network configure MDT measurements to UEs to collect measurements and report it to assist network optimization. There are two modes for the MDT measurements: Logged MDT and Immediate MDT. Logged MDT collects the measurements in IDLE and INACTIVE state. Immediate MDT collects the measurements in CONNECTED state. Compared with traditional drive tests, MDT can reduce the operation workload, and lower the network operation costs as well as improve network performance. In 5G, MDT was not used in the real network, even though it’s a really helpful function for network operation. This was primarily due to its absence in the first NR release and lack of UE vendor supports.

Considering that MDT plays a crucial role in identifying coverage issues and optimizing network deployment by enabling the network to configure measurement tasks for UEs, it’s desirable to be supported and utilized in initial 6G network deployment phase.

Observation 4: MDT is beneficial for network to identify coverage issue and optimize network deployment as well as reduce the network operation cost compared with traditional drive tests.

Based on the above analysis, it’s observed that ANR and MDT are both beneficial for the 6G initial network deployment, i.e. ANR is beneficial for simplify network operation and MDT is beneficial for identifying coverage issue and optimize network deployment. Therefore, it’s proposed to support ANR and MDT in 6G Day 1.

Proposal 2: It’s proposed to support ANR and MDT for network operation and self-organization in 6G Day 1.

Conclusions

In this contribution, the following proposals are provided.

Observation 1: SON function is necessary to be supported in 6G Day 1 to simplify network operation and achieve network deployment cost efficiency.

Proposal 1: Self-organizing function shall be designed to at least fulfill the following requirements:

- RAN shall support the deployment of RAN SON functions in a hybrid manner (distributed and centralized).

- Collaboration and coordination among RAN SON functions need to be addressed.

- User/application level QoS and QoE monitoring capability by UEs and network elements shall be supported.

Observation 2: In the initial 6G network deployment phase, ANR can be utilized to achieve automated neighbor cells addition/removal, which can enhance the efficiency and flexibility of network operation and deployment.

Observation 3: ANR was one of the first SON solutions in 5G, it assisted the network automatically generated relations between base stations and simplify the network management/operation.

Observation 4: MDT is beneficial for network to identify coverage issue and optimize network deployment as well as reduce the network operation cost compared with traditional drive tests.

Proposal 2: It’s proposed to support ANR and MDT for network operation and self-organization in 6G Day 1.

References

- 3GPP TR 38.914, Study on 6G Scenarios and Requirements

Annex: TP to 38.914

### 5.3.4	Ease of operation and Self Organization

Self-organizing function shall be designed to fulfill the following requirements:

- RAN shall support the deployment of RAN SON functions in a hybrid manner (distributed and centralized).

- Collaboration and coordination among RAN SON functions need to be addressed.

- User/application level QoS and QoE monitoring capability by UEs and network elements shall be supported.

6G Day 1 supports ANR and MDT functions for network operation and self-organization.