3GPP TSG RAN Meeting #110						 RP-253187

Baltimore, USA, December 8-11, 2025

Agenda Item:		8.2.1.3

Source:	Nokia, T-Mobile USA, Deutsche Telekom, AT&T, BT, Verizon, SK Telecom, China Unicom, CHTTL

Title:				UE and RAN requirements for Service awareness in 6G RAN

Document for:		Approval

# 1	Introduction

In this contribution we discuss UE and RAN requirements for service awareness in 6G radio access network for addressing new and evolving services effectively while providing enhanced means to improve user experience.

We also make the corresponding text proposals to capture the requirements in TR 38.914.

# 2	Discussion

3GPP wireless technology advancement has been the catalyst for the emerging of new verticals. From limited voice and SMS (2G), and early broadband data with optimized voice (3G), to high-speed broadband data, video and unlimited voice/SMS (4G), and unlimited data, IoT, AR and slicing (5G), the industry is at the stage of massive introduction of new use cases. In a conventional network where eMBB services are the predominant traffic carried by the network, maximizing bit/Hz and using it as the main driver for spectrum efficiency makes sense. However, with more diverse services, such as IoT, XR and AI, being introduced into the wireless network, it is becoming more and more challenging for the network to manage the radio resources which are shared by different services. Fixed Wireless Access (FWA) has also experienced significant growth in recent years, driven by technological advancements. It shouldn't come as a surprise considering that the current 5G specification is designed for eMBB, i.e. smartphone use cases which primarily are voice, SMS and best effort data applications (video streaming, Web browsing, mapping etc) and radio protocol and procedures are heavily designed to ensure robust mobility performance. While eMBB continues to be an important service provided by operators, new applications/services, such as FWA, XR or AI traffic, have significantly different traffic profiles and performance criteria, and need specific consideration for optimisation when the specification are defined for 6G. As the existing and new services will evolve further and new technologies such as AI and sensing will set new requirements for the network, 6G RAN needs to be even more capable of supporting diversified services and applications effectively managing a balance between efficient radio resource usage while optimising user experience.

6G is expected to natively integrate AI into the mobile network fabric, enabling new AI applications that are real-time, distributed and mobile native – transforming society through new forms of intelligent interaction, automation and augmentation, all done wirelessly and contextually. However, in the existing 5G system, RAN does not have direct exposure to the application performance, and lacks a mechanism that can influence radio resource management  to meet the specific real-time application QoE requirement.  For AI applications, in addition to  the regular communication resources, AI computation resources may need to be identified and allocated also. To solve these problems, changes are needed to make RAN to be more adaptive to the applications.

In the current 5G system, 5QI is used to define the specific quality profile for the service as set by the core network. Different 5QI values correspondent to a set of service level thresholds such as throughput, latency, error rate etc. 5QI index is passed to the RAN in which the 5QI value, along with other RAN domain parameters, is used by the MAC scheduler to calculate the MAC scheduling priority for each data flow. Such mechanism works for a system carrying data applications with less stringent throughput and latency requirement, such as web browsing and non-real time video streaming, however, RAN will struggle to support applications, such as XR, which require both stringent throughput and latency adjustments in real time unless the RAN is over dimensioned to ensure the quality of service for such applications. The lack of UE support of features to optimise service flows with multiple QoS requirements makes it very challenge for operators to launch such new services without compromising the overall system spectrum efficiency and legacy eMBB service performance.

3GPP made the efforts to address this issue by introducing RvQoE which allows the application attributes such as video buffer level to be transferred to the AS layer through RRC containers, however, in commercial network, operators ran into challenges,as  device ecosystem players have been hesistantto  pass the application information/APP IDs towards the lower layers for various reasons. Operators have very limited visibility and control on the massive and diversified applications that are going through the network and are not able to effectively meet all applications’ performance targets while maintaining optimal network spectrum and operation efficiency. In addition, the idea of using RRC containers to transfer that information between device and RAN introduces additional signalling overhead which could be non-trivial for a loaded network. To solve this problem, changes are needed to make RAN to be more aware and adaptive to the services and applications. Service Aware RAN for 6G aims to address the issue from the basic protocol levels.

It should also be studied in how far indications from the RAN to the UE could help to optimise the application and service experience of customers. Examples are helpful indications to the UE to be considered by the device OS e.g. for avoiding background synchronisations (e.g. to the cloud storage) in times of higher radio network utilisation while majority of traffic in a cell should be treated with higher prioroity (e.g. responsive applications like chats or AI requests). Real life examples are UL congestions in football stadiums before the game, during the break or right after the game, where device activity (e.g. taking pictures and sending chat messages) also triggers the OS to start cloud synchronisation of photos taken.

3GPP needs to understand

- The benefits of the current 5G QoS and slicing concept are not widely commercialized due to lacking support for appropriate QoS class requests coming from CN to RAN based upon information provided by the devices and their OSs. All ecosystem player should benefit from the improved QoE realizable by making better use of the already-define QoS mechanisms.

- Current 5G system, especially RAN doesn’t have the visibility of real-time application level performance attributes and therefore cannot effectively allocate the radio resources to meet the applications minimum requirements while maintaining optimal overall system spectrum efficiency.  6G RAN needs to have the visibility of real time AI application performance requirements (in each direction uplink and downlink separately as each direction performance could be different due to limitations, e.g. latency) once the application is active, especially when the performance gets worse.

- Current 5G QoS framework is too slow to allow RAN dynamically allocate radio resource efficiently in real time based on the application layer performance.

- The impact of end-to-end encryption on the application layer needs to be evaluated while studying solutions for enhanced service awareness.

- Currently, the 5G RAN does not have visibility of application-level characteristics (requirements, performance, etc.) for addressing the type of requirements that are expected for new service types. Thus, the RAN cannot efficiently utilize RAN resources and features to address other objectives (e.g., to provide resilience to users with critical services, etc.).

- While eMBB continues to be an important service provided by operators, new applications/services, such as XR or AI traffic, have significantly different traffic profiles and performance criteria, and need specific consideration for optimisation as services when the specification are defined for 6G.

- How indications from the RAN to the UE could help to optimise the application and service experience of customers while maintaining overall system efficiency in terms of radio resource utilisation and energy consumption.

# 3 	Text proposal

Based on the discussion we make the following text proposals for TR 38.914.

************ start of changes****************

### 5.3.5	Network Resilience

### 5.3.6	Service awareness in RAN

6G radio access network (6G RAN) shall have means to address evolving and new services like immersive communication (XR) and AI effectively and become more adaptive to the application traffic. 6G RAN will need to obtain more visibility to application-level attributes (e.g., performance requirements, status, etc.) to effectively allocate radio resources and utilize the different RAN resources to meet the applications minimum requirements while maintaining optimal overall system efficiency and efficient utilization of RAN resources. It shall also be possible to provide related information from the 6G RAN to the 6G UE.

6G shall fulfil the following requirements:

- 6G RAN shall be able to understand and be capable of adapting to traffic demands of applications while maintaining overall system efficiency in terms of radio resource utilisation and energy consumption.

- 6G UE shall provide assistance information related to traffic and application-level attributes to the 6G RAN.

- 6G RAN shall provide configuration to the 6G UE based on network load, scheduling efficiency and application characteristics.

************ end of changes****************

# 4	Conclusion

In this contribution we have discussed UE and RAN requirements for service awareness in 6G radio access network (RAN) to address new and evolving services effectively. In section 3 we make a text proposal for capturing 6G UE and 6G RAN related service awareness requirements in TR 38.914.