3GPP TSG RAN Meeting #110									RP-253248

Baltimore, USA, December 8-11, 2025

Agenda Item:	8.2.1.1

Source:	Xiaomi

Title:	Discussion on 6G Key performance indicators

Document for:	Decision

- Introduction

In RAN#109, the study on 6G minimum TPR for ITU-R submission has completed, and the LS on minimum requirements for IMT-2030 has been sent [1]. However, the discussion on technical performance indicators for 3GPP internal study will continue.

In this contribution, the remaining issues on KPIs for 3GPP internal study are discussed.

- Coverage requirement

In RAN#109, the following agreements on 6G coverage requirement were made [2]:

Proposal 4: For 3GPP internal study, link budget is used as the evaluation methodology for coverage when applicable Proposal 5: For 3GPP internal study, the target for coverage is to be determined by RAN. - FFS: Exact coverage target value(s). - FFS: Additional details considering control/data channel

Multiple performance metrics have been proposed for link budget evaluation, e.g., MCL, MPL, MIL, etc. The difference is on whether to take into account of some antenna gains, SLS factors, etc. As there would be different implementations of antenna gains and different values of SLS factors in different frequency band, to enable a coverage requirement independent of frequency bands, Maximum Coupling Loss (MCL) would be preferred. The MCL value(s) defined in NR requirements can be considered as the starting point, subject to requirements from operators.

On the other hand, in RAN1#122 meeting, the following agreement was made:

Agreement On enhanced overall coverage, identify coverage target(s) considering diverse use cases and device types

Therefore, different values as coverage requirement may be needed considering different 6G use cases and device types. Since the discussion on 6G device types is ongoing, including the necessity and motivation of device types, the necessity to define extended coverage requirements for specific 6G use cases or device types (e.g. IOT, NTN, etc.) can be further discussed.

Proposal 1: MCL (Maximum Coupling Loss) is used as the metric for 6G coverage requirement

- the value of NR coverage requirement is considered as the starting point for normal 6G coverage requirement, subject to requirements from operators

- FFS extended coverage requirement(s)

- Energy efficiency

Better energy efficiency compared with 5G NR is the key to a sustainable and successful generation for 6GR. Energy efficiency can be denoted as the power consumption per transmitted bit, in terms of Joule.

In RAN#109 meeting, the requirement in terms of energy efficiency is defined as the relative energy [savings/consumption] (in terms of percentage) for the selected load case(s) relatively to a fully loaded reference case [4]. In the latest TR 38.914, the following two tables are captured for the minimum requirements for Network Energy efficiency and Device Energy efficiency, respectively.

Table 5.1.15-1: The minimum requirements for Network Energy efficiency

| [Test environment(s)]** | Selected load case(s)* | The reference case | Relative energy [saving/consumption] (%) |
| --- | --- | --- | --- |
| N. A | empty load | Fully loaded case* | [X1%] |
| [Dense urban-IC]** | [A%] |  | [X2%] |
| [Rural-IC]** | [B%] |  | [X3%] |
| * empty load: L=0, low load: 0 < L≤15, light load: 15 < L≤30, medium load: 30 < L≤50, fully loaded case: L=100 ** needed only in case of simulation |  |  |  |

Table 5.1.15-2: The minimum requirements for Device Energy efficiency

| [Test environment(s)]** | Selected load case(s)* | The reference case | Relative energy [saving/consumption] (%) |
| --- | --- | --- | --- |
| N. A | empty load | Fully loaded case | [Y1%] |
| [Dense urban-IC]** | [C%] |  | [Y2%] |
| * empty load: L=0, low load: 0 < L≤15, light load: 15 < L≤30, medium load: 30 < L≤50, fully loaded case: L=100 ** needed only in case of simulation |  |  |  |

However, the 3GPP internal evaluation metrics need to be discussed which should be captured depending on pending discussion. Considering the nature of power consumption and comprehensive assessment on energy efficiency, the following EE definition can be used as metric to calculate energy efficiency for UE and base station respectively:

EE=vE；E=i=0nEiTtotal=（Eon+Eoff+Etransition）Ttotal

v is the data rate, and v=R(Ttotal)/Ttotal , R(Ttotal) denotes the number of correctly transmitted or received bits during the total observation/evaluation time, i.e., Ttotal. E denotes the average energy consumption during the Ttotal . Ei  is the energy computation state, and three states, i.e., on, off, and transition, can be considered as starting point.  Eon is the energy consumption for on state when there is transmission and/or reception.  Eoff is the energy consumption for off state when there is no transmission and/or reception. Etransition is the energy consumption during transition between on state and off state.

For unloaded cases, as it does not involve the transmission and reception of user data, energy efficiency analysis can be reflected through average energy consumption, e.g., E.

Furthermore, it is fundamental for 6GR to guarantee that the base station and UE can simultaneously benefit from high energy efficiency, which is one of the targets for 6GR and captured in [3]:

The detailed objectives of the study are: - Single technology framework based on a stand-alone architecture (Note1) to support the agreed existing and new services, and to satisfy the usage scenarios, requirements, deployment scenarios and design principles with acceptable performance/complexity trade-off, as determined by the RAN requirements in [RP-250810] and [TR38.914], including: [RAN1], [RAN2], [RAN3], [RAN4] - Ensuring appropriate set of functionalities, minimize the adoption of multiple options for the same functionality, avoid excessive configurations, excessive UE capabilities and UE capabilities reporting. - Energy efficiency and energy saving: both for network and device.

The joint metric to evaluate overall energy efficiency between base station and UE should take both sides into consideration. For example, the impacts on the other side should be analyzed when evaluating the energy efficiency for one side. No matter which way to go, a comprehensive assessment on energy efficiency for both base station and UE should be achieved.

Proposal 2:  Energy efficiency can be evaluated in terms of power consumption per transmitted bit, which can be defined by the following formula for base station and UE respectively:

EE=vE；E=i=0nEiTtotal=（Eon+Eoff+Etransition）Ttotal

- Sensing

Various use cases are identified in SA1 TR 22.837 and TR 22.870. In RAN #109, it is agreed that the 6GR and 6G RAN architecture shall at least support use cases for detection and/or tracking of passive objects, at least including UAVs, human, vehicles and AGVs. Further, the 6GR study should contain a wide set of  sensing modes, including TRP monostatic, TRP-TRP bistatic, TRP-UE DL, UE-TRP UL, UE-UE bistatic and UE monostatic. As Rel-20 is first release for 3GPP to introduce ISAC, it is preferred to focus on the most urgent commercial needs taking to account what RAN already has developed, i.e., the ISAC channel model in Rel-19 and the outcomes from NR ISAC in Rel-20. Therefore, the introduction of more advanced use cases should be well justified.

According to the TPR discussions, the following performance metrics can be considered for sensing.

- Missed detection probability and False alarm probability

Missed detection probability is defined as the probability of not detecting the presence of a target object when the target object is present, and False alarm probability is defined as the probability of falsely detecting the presence of a target object when the target object is not present.

- Position accuracy

The accuracy on position is defined as the difference of the sensed position of the target object to its true position value. The position accuracy shall be obtained based on the X percentile point of the cumulative distribution function (CDF) of the position estimation errors. Sensing accuracy on position can be further derived into a horizontal accuracy and a vertical accuracy.

- Velocity accuracy

The accuracy on velocity is defined as the difference of the sensed velocity of the target object to its true velocity value. The velocity accuracy shall be obtained based on the X percentile point of the cumulative distribution function (CDF) of the velocity estimation errors. Sensing accuracy on velocity may be further derived into a horizontal accuracy and a vertical accuracy too.

For an evaluation work in RAN, the following metrics which are used in SA1 TS/TR may not be considered.

- The latency and refreshing rate defined in SA1 TS/TR is the E2E requirement. RAN only contributes a small portion of the overall performance.

- Sensing resolution may be directly obtained from a formula using the system parameters. For example, for monostatic sensing, range resolution for monostatic is defined as DR=c2B, and velocity resolution for monostatic is defined as ∆v=λ2TCPI, where B is the bandwidth, TCPI is the coherent processing interval, c is the speed of light. Such formulas are well known and justified in literature.

The requirements on the performance metrics are dependent on the use cases for sensing. Even for the same target type, the performance requirements also vary a lot in different use cases. In the first release for an extensive study on ISAC, it is preferred to start with relaxed requirements. Taking TS 22.137 as reference, and possible improvements compared to NR ISAC in Rel-20, we propose the KPI values in Table 1 for sensing use cases.

Table 1:	Performance requirements for 6G sensing

| Metrics | KPI values |
| --- | --- |
| Position accuracy with confidence level 90% | Horizontal: [5]m Vertical: [5]m |
| Velocity accuracy with confidence level 90% | Horizontal: [2]m/s Vertical: [2]m/s |
| Missed detection probability / False alarm probability | [5]% / [5]% |

Proposal 3:  Adopt the following TP for section 5.17 in TR 38.914

5.1.17	Sensing

The following performance metrics can be considered for sensing:

- Missed detection probability and False alarm probability

Missed detection probability is defined as the probability of not detecting the presence of a target object when the target object is present, and False alarm probability is defined as the probability of falsely detecting the presence of a target object when the target object is not present.

- Position accuracy

The accuracy on position is defined as the difference of the sensed position of the target object to its true position value. The position accuracy shall be obtained based on the X percentile point of the cumulative distribution function (CDF) of the position estimation errors. Sensing accuracy on position can be further derived into a horizontal accuracy and a vertical accuracy.

- Velocity accuracy

The accuracy on velocity is defined as the difference of the sensed velocity of the target object to its true velocity value. The velocity accuracy shall be obtained based on the X percentile point of the cumulative distribution function (CDF) of the velocity estimation errors. Sensing accuracy on velocity may be further derived into a horizontal accuracy and a vertical accuracy too.

The performance requirements for 6G sensing are shown as in Table 5.1.17-1

Table 5.1.17-1:	Performance requirements for 6G sensing

| Metrics | KPI values |
| --- | --- |
| Position accuracy with confidence level 90% | Horizontal: [5]m Vertical: [5]m |
| Velocity accuracy with confidence level 90% | Horizontal: [2]m/s Vertical: [2]m/s |
| Missed detection probability / False alarm probability | [5]% / [5]% |

- Conclusion

In this contribution, the remaining details on legacy TPR values, performance indicators for coverage, energy efficiency and sensing are discussed, and the following proposals are made:

Proposal 1: MCL (Maximum Coupling Loss) is used as the metric for 6G coverage requirement

- the value of NR coverage requirement is considered as the starting point for normal 6G coverage requirement, subject to requirements from operators

- FFS extended coverage requirement(s)

Proposal 2:  Energy efficiency can be evaluated in terms of power consumption per transmitted bit, which can be defined by the following formula for base station and UE respectively:

EE=vE；E=i=0nEiTtotal=（Eon+Eoff+Etransition）Ttotal

Proposal 3:  Adopt the following TP for section 5.17 in TR 38.914

5.1.17	Sensing

The following performance metrics can be considered for sensing:

- Missed detection probability and False alarm probability

Missed detection probability is defined as the probability of not detecting the presence of a target object when the target object is present, and False alarm probability is defined as the probability of falsely detecting the presence of a target object when the target object is not present.

- Position accuracy

The accuracy on position is defined as the difference of the sensed position of the target object to its true position value. The position accuracy shall be obtained based on the X percentile point of the cumulative distribution function (CDF) of the position estimation errors. Sensing accuracy on position can be further derived into a horizontal accuracy and a vertical accuracy.

- Velocity accuracy

The accuracy on velocity is defined as the difference of the sensed velocity of the target object to its true velocity value. The velocity accuracy shall be obtained based on the X percentile point of the cumulative distribution function (CDF) of the velocity estimation errors. Sensing accuracy on velocity may be further derived into a horizontal accuracy and a vertical accuracy too.

The performance requirements for 6G sensing are shown as in Table 5.1.17-1

Table 5.1.17-1:	Performance requirements for 6G sensing

| Metrics | KPI values |
| --- | --- |
| Position accuracy with confidence level 90% | Horizontal: [5]m Vertical: [5]m |
| Velocity accuracy with confidence level 90% | Horizontal: [2]m/s Vertical: [2]m/s |
| Missed detection probability / False alarm probability | [5]% / [5]% |

- References

- RP-252946, Draft Reply LS to ITU-R_WP5D_TEMP_167  = RP-243202 on Minimum requirements related to technical performance for IMT-2030 radio interface(s), RAN#109, Beijing, Jun, 2025.

- RP-252883, Moderator's summary for RAN led 6G SI: Key performance indicators, RAN#109, Beijing, Jun. 2025.

- 3GPP TR 38.914 v 0.1.0, Study on 6G Scenarios and requirements, Jun. 2025

- RP-252882, 3GPP TR 38.914 V0.2.0 (2025-09), Study on 6G Scenarios and Requirements, Sep. 2025