3GPP TSG RAN Meeting #110                                                       RP-253252

Baltimore, USA, December 8-11, 2025

Source:	vivo

Title:	Discussion on 6G key performance indicators

Agenda Item:	8.2.1.1

Document for:	Discussion and Decision

Introduction

In RAN#109, TR 38.914 V0.2.0 [1] was endorsed which captures the agreed definition and requirement values of key performance indicators. In this contribution, for energy efficiency and sensing, we provide separate definitions or additional performance metrics for ITU IMT-2030 and 3GPP internal usage. For coverage, we provide our consideration for 3GPP internal usage.

Discussion

For the traditional KPIs and the composite requirements KPI, we suggest same definition and target values for ITU IMT2030 and 3GPP internal usage. Regarding sensing, considering diverse sensing use cases, we suggest to consider more performance metric(s) for 3GPP internal usage. Regarding energy efficiency, we suggest to define the evaluation metric(s) to jointly consider both power/energy consumption and communication related performance metric. Regarding coverage, we suggest to consider MCL as link budget metric and provide potential coverage target with initial analysis.

Sensing

In ITU-R WP5D #50 meeting, the following Sensing-related requirements are agreed:

-	Detection Probability and False Alarm Probability: Detection probability is the probability of correctly detecting the presence of the sensing target when it is present, and false alarm probability is the [associated] probability that a sensing target is detected when no sensing target is present.

-	Horizontal/Vertical Localization Accuracy: It is defined as the absolute difference between the estimated horizontal/vertical location and the actual horizontal/vertical location of the sensing target. The value of localization accuracy shall be obtained assuming 90% confidence level, which is the 90th percentile point of the cumulative distribution function (CDF) of the location estimation error. Both the horizontal and vertical localization accuracy are in Cartesian coordinates.

-	Velocity Accuracy: Velocity accuracy is defined as the absolute difference between the estimated velocity and the actual velocity of the sensing target. The value of velocity accuracy shall be obtained assuming 90% confidence level, which is the 90th percentile point of the cumulative distribution function (CDF) of the velocity estimation error.

It is observed that these requirements are the primary performance metrics for sensing use cases of detection and tracking. Other use cases, such as motion detection and vital sign monitoring, and sensing-assisted communication should also be considered in 6G sensing study.

Regarding the use case of motion detection and vital sign monitoring, such as UAV identification, bird identification, human respiratory/sleep monitoring, sports monitoring, etc., the movement characteristics of targets need to be identified and measured in order to support these sensing services, and the velocity accuracy which is more applicable to linear motion which cannot characterize these types of motion. Therefore, we propose the following sensing-related metrics:

Micro-Doppler accuracy - It is defined as the difference between the estimated doppler and the actual doppler due to the sensing object motion.

Regarding the use case of sensing-assisted communication, the sensing results, such as the environmental information and UE state (e.g., position and height of UE) can be used to assist communication (e.g., beam tracking and mobility) and enhance communication performance, some traditional communication performance metrics can be considered, such as spectrum efficiency, user perceived throughput (UPT), HOF, RLF etc.

- For sensing, consider following key performance metrics:

  - Detection Probability and False Alarm Probability

  - Horizontal/Vertical Localization Accuracy

  - Velocity Accuracy

  - Micro-doppler accuracy

The performance requirements, which are highly related to the use cases and deployment scenarios, can refer to the discussion in our companion contribution [2].

Energy efficiency

The 3GPP internal definition of energy efficiency should jointly consider both power/energy consumption and communication related performance metric, such as data rate, and latency for control signaling/paging or data.

For unloaded case, the energy efficiency could be defined taking both the impact on paging/control signaling latency and energy consumption into account, two options can be considered.

- Option 1, define EE=VP, where v is the delivery/reception rate of paging, v = sum(1/ti)/N, ti is the time interval between the arrival of paging and its successful delivery to the receiver.

- Option 2, define EE as the relative power consumption with tolerable/less than [TBD] latency deterioration.

For loaded case, the energy efficiency could be defined by taking both the data rate and power consumption into account, two options can be considered.

- Option 1, define EE=VP  or EE=PV ,  where P is the power consumption, v is the user perceived throughput (UPT), which is widely used in 3GPP evaluations in all SI/WIs and reflects the user's experience data rate, and P=EC/T, where EC is the energy consumption during an evaluation time T, EC= Pon ×Ton + Psleep ×Tsleep+ Ptransition ×Ttransition .

- Option 2, define EE as the relative power consumption with tolerable/less than [TBD] data rate deterioration.

- Both unloaded case and loaded case should be considered for energy efficiency.

- The definition of energy efficiency should jointly consider both power/energy consumption and communication related performance metric, such as data rate, and latency for control signaling/paging or data.

- For unloaded case, two options can be considered:

- - Option 1:  EE=VP  , where P is the power consumption, v is the delivery/reception rate of paging, v = sum(1/ti)/N, ti is the time interval between the arrival of paging and its successful delivery to the receiver.

- -  Option 2:  EE is the relative power consumption with tolerable/less than [TBD] latency deterioration.

- For loaded case, two options can be considered:

- - Option 1:  EE=VP  or E=PV , where P is the power consumption, v is the user perceived throughput (UPT).

- - Option 2:  EE is the relative power consumption with tolerable/less than [TBD] data rate deterioration.

Coverage

Following objectives relevant to coverage are captured in 6GR SID [3]:

- Enhanced overall coverage, focus on cell-edge performance and UL coverage. - Re-use of existing 5G mid-band (~3.5GHz) site grid for 6G deployments in at least around 7 GHz and targeting comparable coverage to 5G mid-band

Following agreement was reached in RAN#109 on 6GR coverage.

Agreement in RAN#109 Proposal 4: For 3GPP internal study, link budget is used as the evaluation methodology for coverage when applicable Proposal 5: For 3GPP internal study, the target for coverage is to be determined by RAN. - FFS: Exact coverage target value(s). - FFS: Additional details considering control/data channel

Further agreements on detailed performance metric for the link budget were made in RAN#122bis and RAN1#123 as below.

Agreement in RAN1#122bis RAN1 provides methodology and corresponding initial analysis of potentially achievable coverage to RAN#110 to determine the coverage target(s) Agreement in RAN1#123 - For the RAN1 study of “Re-use of existing 5G mid-band (~3.5GHz) site grid for 6G deployments in at least around 7 GHz and targeting comparable coverage to 5G mid-band”, - The link budget template candidates 1 and 2 are used to calculate the metric(s) as starting point to compare existing 5G mid-band and 6G deployments in at least around 7 GHz, with potential future update. - During initial access/random access - Coverage target is referring the bottleneck channel (i.e. Rel-15 NR Msg3) during initial access/random access for existing 5G mid-band - FFS target value(s) of data rate for data channels relative to 5G mid-band - Following deployment scenarios are considered - Urban macro (both O2I and outdoor) - Sub-urban macro (both O2I and outdoor) - Following carrier frequencies are considered to calculate the metric(s) - [4 GHz] as the existing 5G mid-band - 7 GHz as 6G deployment Template in R1-2509615 is to be used for collecting inputs on the values from companies.

RAN1 discussed 4 different metrics (MaxCL, MCL, MIL, MPL), and was not able to down-select. To compare existing 5G mid-band and 6G deployments in at least around 7 GHz where MCL/MIL/MPL belongs to candidate 1 and MaxCL belongs to candidate 2.  However, for RAN-level consideration of coverage target, it is preferable to use one common metric, based on following criteria:

- The defined metric should be a common metric which can be applied to all channels/signals, rather than channel/signal specific.

- The defined metric should be simple and robust enough for link budget evaluation.

- The defined metric and related coverage target should be able to show the coverage difference between 3.5GHz and 7GHz, so that the coverage of co-site deployment of NR and 6GR can be evaluated

Metric in TR 38.913 - MaxCL = Total transmit power – Receiver sensitivity Metric in TR 38.830 - MCL = Total transmit power – Receiver sensitivity + gNB antenna gain (component 2) - MIL = Total transmit power – Receiver sensitivity – Tx loss – Rx loss + gNB antenna gain (component 2 + 3 + 4) + UE antenna gain - MPL = MIL – Shadow fading margin + BS selection/macro-diversity gain – Penetration margin + Other gain

MaxCL fulfils the first two criteria, but it does not provide information on whether co-site deployment of 3.5GHz and 7GHz can be achieved, because it provides a single value independent of carrier frequency.

MCL fulfils the first two criteria, and it can exemplify the objective of equivalent coverage for co-site deployment of 3.5GHz and 7GHz, by adding an offset value, which takes potential pathloss including penetration loss difference and antenna gain difference into account. For example, for MCL, the coverage target for 3.5GHz can be defined as X dB at a given rate, whereas the target of 7GHz can be defined as (X+Y) dB, where Y is the offset which depends on the pathloss loss difference and BS antenna array difference between 3.5GHz and 7GHz.

MPL and MIL are more complex than MCL and MaxCL as they depend on too many parameters, and some of them are not common for all channels/signals, which contradicts with the first and second criteria. While MPL may seem more intuitive for reflecting co-site deployment objective, due to the challenge of converging on so many parameters required for MPL calculation, the utility of MPL as a target metric for guiding 6GR system design is ultimately rendered.

Based on the analysis above, MCL should be used as metric for coverage performance.

Regarding exact coverage target, following aspects are considered

- For enhanced overall coverage, focus on cell-edge performance and UL coverage.

- For eMBB, 6GR should achieve at least same coverage as NR.  The NR coverage can take MCL results in TR 38.830 as reference rather than the value in TR 38.913 which is defined based on MaxCL.

Meanwhile, to deliver a better user experience, higher UL and DL data rate than 5G NR shall be supported across the whole cell coverage for the same spectrum deployment. Table 1 provides an example of achievable UL data rate in NR system under urban deployment at 4 GHz across different coupling loss values. It can be expected that, with the techniques mentioned above, 6GR systems can achieve approximately 2x improvement in UL data rates at given coupling loss values.

Table 1 UL Coverage Improvement Target for 6GR system

|  | MCL (dB)note 2 | UL Data rate (NR) | Data rate (6GR) |
| --- | --- | --- | --- |
| Urban (4GHz TDD) note 1 | 131 | 1Mbps | [2x] |
| Urban (4GHz TDD) | 144 | 50Kbps | [2x] |

- For IoT, wider coverage than eMBB service is expected, however, 6GR should consider a more practical coverage that suffices for viable IoT services while minimizing impact on eMBB. 10dB extension rather than 20dB (164dB MCL in TR 38.913) would be a reasonable choice.

Since some common channels are shared by all device types, e.g., SSB commonly received by IoT and eMBB device, the actual achievable coverage for these channels would not differentiate device type, while target coverage of PDSCH/PUSCH for data transmission varies with device types, i.e., roughly 1/10 of eMBB data rate can be achieved by IoT device with 10dB coverage extension.

- For re-use of existing 5G mid-band (~3.5GHz) site grid for 6G deployments in at least around 7 GHz and targeting comparable coverage to 5G mid-band

- As discussed above, the target coverage for around 7GHz is (X+Y) dB, where X is target MCL for NR @3.5GHz, and Y is the difference between pathloss and gNB antenna gain for 3.5GHz and 7GHz. According to channel model in TR 38.901, the pathloss gap including penetration loss is around 7 ~ 10.5dB for O2I low or high loss. Regarding gNB antenna gain, assuming BS antenna modelling combination 1 with 192 antenna elements and 64 TXRU for 3.5GHz, which is aligned with assumption for Rel-17 CE in TR 38.830, and BS antenna modelling combination 4 or 5 with 2048 elements and 256 or 512 TXRU for 7GHz, the additional gain provided by larger antenna array in 7GHz can be around 10.3dB. Consequently, Y can be up to ~0.2dB (i.e., 10.5dB-10.3dB). To leave some margin, Y=1 can be considered as starting point. It can be updated after RAN1 progress on modeling and evaluation, if needed.

- For 6GR coverage, MCL (Maximum Coupling Loss) is used as the link budget metric for coverage performance evaluation

- For the same spectrum deployment, a [10] dB MCL improvement over Reference NR coverage should be achieved.

    - Target of the [10] dB MCL improvement over NR for common channels applicable to all device types.

    - For data channel

      - For eMBB, at least same data rate as NR shall be achieved at same MCL of NR eMBB. Furthermore, approximately 2x improvement of data rates at given coupling loss values can be considered.

      - For IoT, Target of the [10] dB MCL improvement over NR is achieved with roughly 1/10 of eMBB data rate.

- For reusing existing 5G mid-band (~3.5GHz) site grid for 6G deployments in at least around 7 GHz, comparable coverage with same data rate shall be achieved at MCL=X dB for 3.5GHz and MCL= (X+Y) dB for 7GHz, where Y=1 can be the starting point.

- Capture coverage metric MCL definition and MCL target with corresponding data rate in TR 38.914 after RAN decides the exact target.

Note: Achievable coverage in TR 38.830 is used as Reference NR coverage as starting point.

Conclusion

In this contribution, we provide our considerations on 6G KPIs and the proposals are as following:

- For sensing, consider following key performance metrics:

  - Detection Probability and False Alarm Probability

  - Horizontal/Vertical Localization Accuracy

  - Velocity Accuracy

  - Micro-doppler accuracy

- Both unloaded case and loaded case should be considered for energy efficiency.

- The definition of energy efficiency should jointly consider both power/energy consumption and communication related performance metric, such as data rate, and latency for control signaling/paging or data.

- For unloaded case, two options can be considered:

- - Option 1:  EE=VP  , where P is the power consumption, v is the delivery/reception rate of paging, v = sum(1/ti)/N, ti is the time interval between the arrival of paging and its successful delivery to the receiver.

- -  Option 2:  EE is the relative power consumption with tolerable/less than [TBD] latency deterioration.

- For loaded case, two options can be considered:

- - Option 1:  EE=VP  or E=PV , where P is the power consumption, v is the user perceived throughput (UPT).

- - Option 2:  EE is the relative power consumption with tolerable/less than [TBD] data rate deterioration.

- For 6GR coverage, MCL (Maximum Coupling Loss) is used as the link budget metric for coverage performance

- For the same spectrum deployment, a [10] dB MCL improvement over Reference NR coverage should be achieved.

    - Target of the [10] dB MCL improvement over NR for common channels applicable to all device types.

    - For data channel

      - For eMBB, at least same data rate as NR should be achieved at same MCL of NR eMBB. Furthermore, approximately 2x improvement of data rates at given coupling loss values can be considered.

      - For IoT, Target of the [10] dB MCL improvement over NR is achieved with roughly 1/10 of eMBB data rate.

- For reusing existing 5G mid-band (~3.5GHz) site grid for 6G deployments in at least around 7 GHz, comparable coverage with same data rate shall be achieved at MCL=X dB for 3.5GHz and MCL= (X+Y) dB for 7GHz, where Y=1 can be the starting point.

Note: Achievable coverage in TR 38.830 is used as Reference NR coverage.

- Based on discussion in section 2, it is proposed to agree on the TPs below.

TPs for 38.914

=====================Start of the changes======================

5.1.15	Energy efficiency

5.1.15.1	Energy efficiency for ITU IMT-2030

Note: update the text according to the output of ITU-R WP5D #50 meeting.

Energy efficiency is an important metric of sustainability. Energy efficiency of the candidate RIT/SRIT can be characterized by evaluating network and device energy efficiency.

Energy efficiency can be estimated as the relative energy consumption (in terms of percentage) for the selected load case(s) relative to a fully loaded reference case.

The RIT/SRIT shall have the capability to support low relative energy consumption in the selected load cases, considering at least the unloaded (L= 0%) and loaded scenario of [0% < L ≤ 15%] / [15% < L ≤ 30%] / [30% < L ≤ 50%] / [0% < L ≤ 30%], where L is load level, i.e., network resource utilization ratio.

Proponents should report the evaluated energy efficiency values and the power model used for the evaluation, as well as impacts on communication performance associated with the evaluated energy saving technologies.

Proponents are encouraged to describe mechanisms of the RIT/SRIT that improve the support of low energy consumption operation for both network and device.

This requirement is defined for the purpose of evaluation in the Immersive communication usage scenario.

Conditions for evaluation are included in Report ITU-R M.[IMT-2030.EVAL].

5.1.15.2	Energy efficiency for 3GPP internal

Energy efficiency is an important metric of sustainability. This requirement of the candidate RIT/SRIT is characterized by evaluating network and device energy efficiency. Network energy efficiency is the capability of a RIT/SRIT to support radio access network energy saving in relation to the traffic capacity provided. Device energy efficiency is the capability of the RIT/SRIT to support device power saving in relation to the traffic characteristics.

For unloaded case, the energy efficiency could be defined taking both the impact on paging/control signaling latency and energy consumption into account, two options can be considered.

- Option 1, define EE=VP, where v is the delivery/reception rate of paging, v = sum(1/ti)/N, ti is the time interval between the arrival of paging and its successful delivery to the receiver.

- Option 2, define EE as the relative power consumption with tolerable/less than [TBD] latency deterioration.

For loaded case, the energy efficiency could be defined by taking both the data rate and power consumption into account, two options can be considered.

- Option 1, define EE=VP  or EE=PV ,  where P is the power consumption, v is the user perceived throughput (UPT), which is widely used in 3GPP evaluations in all SI/WIs and reflects the user's experience data rate, and P=EC/T, where EC is the energy consumption during an evaluation time T, EC= Pon ×Ton + Psleep ×Tsleep+ Ptransition ×Ttransition .

- Option 2, define EE as the relative power consumption with tolerable/less than [TBD] data rate deterioration.

This requirement is defined for the purpose of evaluation in the Immersive communication usage scenario. This requirement is evaluated by analytical and simulation approach.

5.1.17	Sensing

5.1.17.1	Sensing for ITU IMT-2030

Note: update the text according to the output of ITU-R WP5D #50 meeting.

Sensing-related capabilities are measured in terms of the following technical performance requirements:

- Detection Probability and False Alarm Probability: Detection probability is the probability of correctly detecting the presence of the sensing target when it is present, and false alarm probability is the [associated] probability that a sensing target is detected when no sensing target is present.

- Horizontal/Vertical Localization Accuracy: It is defined as the absolute difference between the estimated horizontal/vertical location and the actual horizontal/vertical location of the sensing target. The value of localization accuracy shall be obtained assuming 90% confidence level, which is the 90th percentile point of the cumulative distribution function (CDF) of the location estimation error. Both the horizontal and vertical localization accuracy are in Cartesian coordinates.

- Velocity Accuracy: Velocity accuracy is defined as the absolute difference between the estimated velocity and the actual velocity of the sensing target. The value of velocity accuracy shall be obtained assuming 90% confidence level, which is the 90th percentile point of the cumulative distribution function (CDF) of the velocity estimation error.

In the above technical performance requirements, the sensing target is a passive object, which is not connected to the radio network for communication.

This requirement is defined for the purpose of evaluation in the Integrated Sensing and Communication usage scenario.

The minimum requirements for Sensing-related requirements are summarized in Table x.

Table x

Sensing-related requirements

| Test environment | Indoor Factory ‒ ISAC | Urban Macro – ISAC |
| --- | --- | --- |
| Detection Probability |  | [95%] |
| False Alarm Probability |  | [5%] |
| Horizontal Localization Accuracy |  | [5m or 10m] |
| Vertical Localization Accuracy |  |  |
| Velocity Accuracy |  |  |

5.1.17.2	Sensing for 3GPP internal

Sensing-related capabilities are measured in terms of the following technical performance requirements:

-	Detection Probability and False Alarm Probability: Detection probability is the probability of correctly detecting the presence of the sensing target when it is present, and false alarm probability is the associated probability that a sensing target is detected when no sensing target is present.

-	Horizontal/Vertical Localization Accuracy: It is defined as the absolute difference between the estimated horizontal/vertical location and the actual horizontal/vertical location of the sensing target. The value of localization accuracy shall be obtained assuming 90% confidence level, which is the 90th percentile point of the cumulative distribution function (CDF) of the location estimation error. Both the horizontal and vertical localization accuracy are in Cartesian coordinates.

-	Velocity Accuracy: Velocity accuracy is defined as the absolute difference between the estimated velocity and the actual velocity of the sensing target. The value of velocity accuracy shall be obtained assuming 90% confidence level, which is the 90th percentile point of the cumulative distribution function (CDF) of the velocity estimation error.

-	Micro-Doppler accuracy: It is defined as the difference between the estimated doppler and the actual doppler due to the sensing object motion. The required value of micro-doppler accuracy shall be obtained assuming 90% confidence level, which is the 90th percentile point of the cumulative distribution function (CDF) of the micro-doppler estimation error.

This requirement is defined for the purpose of evaluation in the Integrated Sensing and Communication usage scenario. This requirement is evaluated by simulation approach.

5.1.18	AI

Applicable AI-related capabilities refer to the ability to provide certain functionalities throughout IMT-2030 to support AI enabled applications. These capabilities include:

-	Data collection

-	Data/Model exchange

-	AI model training

-	AI model inference

-	AI model monitoring

-	AI computing resource management

This requirement is defined for the purpose of evaluation in the Artificial Intelligence and Communication usage scenario. This requirement is evaluated by inspection approach.

======================End of the changes======================

Reference

[1]   RP-252882-3GPP TR 38.914 V0.2.0.

[2]	RP-253256, “Discussion on 6G requirements for ISAC”, vivo.

[3]	RP-251395, “Revised SID: Study on 6G Scenarios and requirements”; CMCC, Verizon, NTT DOCOMO, INC., Deutsche Telekom.