3GPP TSG RAN Meeting #110	RP-253285

Baltimore, USA, December 8-11, 2025

Agenda Item:	8.2.1.3.2

Source:	Huawei, HiSilicon

Title:	Consideration on Device Types in 6GR

Document for:	Discussion and Decision

# Introduction

In the RAN#109 meeting, the following proposals related to device types were approved [1]:

Proposal 1: Endorse the following two RAN1 agreements (with the clarification that the 2nd agreement is applicable to FR1). Companies are invited to bring contributions regarding the minimum spectrum allocation in RAN#110, while RAN1 is requested to continue the study on both the minimum spectrum allocation and the smallest maximum UE bandwidth from the 6GR design perspective. Revisit in RAN#110. Proposal 2: 6GR aims to specify a set of mandatory baseline functionalities for all UEs, with details to be further studied, by taking into account diverse device types for scalable and forward compatible design, and the requirements, as necessary, as in the 6G SID. Further study whether there is a need to mandate base stations for certain requirements/functionalities. Proposal 3: To investigate further: - Motivations/justifications behind the proposed diverse device types, which should be a limited set - Whether/how to have one or more device types for eMBB or 6G IoT - Whether/how to have other device types for, e.g., XR/immersive experiences, FWA, VUE, wearables/RedCap, sensing, NTN-specific, AI agents, collaborative robots, etc. - Whether/how to explicitly standardize device types - Ensuring forward compatibility - Minimizing/avoiding potential market fragmentation Note: the terminology “device type” is subject to further discussion and possible refinement. Proposal 4: In terms of diverse device types, study further: - Possible parameters/factors, e.g.: - Number of Tx antennas/chains - Number of Rx antennas/chains - Power classes - Maximum UE bandwidth (DL/UL) - Peak data rate (DL/UL) - Maximum MIMO layers (DL/UL) - Duplex mode - Max modulation order (DL/UL) - CA/spectrum aggregation (DL/UL) - UE processing capabilities - Coverage - Energy efficiency - Mobility/speed - Sensing - AI Note: some of the above parameters/factors may be related with form factor Note: aim to have a focused/limited set of parameters/factors for a device type - The value(s) for the identified parameters for a device type

In this contribution, the considerations and views on whether, when, and how to handle 6GR device type discussion are provided.

# Discussion

## Background

Both 5G NR and LTE support a wide range of device types, from high-performance eMBB to mid-performance reduced-complexity devices, to low-power IoT devices. Examples include NR Rel-15/16 eMBB, NR Rel-17/18 RedCap, as well as LTE Cat-1, Cat-1 bis, Cat-4, NB-IoT, and Cat-M. These device categories differ in bandwidth capability, peak data rate, antenna configuration, coverage requirements, cost, battery life, and other factors, thereby meeting different application scenarios. The landscape of diverse 5G and LTE device types can be found in Figure 1.

Figure 1. The landscape of diverse 5G and LTE device types

## Consideration on factors defining device types

At RAN#109, aspects proposed by companies related to device type discussion are as follows:

  - Number of Tx antennas/chains

  - Number of Rx antennas/chains

  - Power classes

  - Maximum UE bandwidth (DL/UL)

  - Peak data rate (DL/UL)

  - Maximum MIMO layers (DL/UL)

  - Duplex mode

  - Max modulation order (DL/UL)

  - CA/spectrum aggregation (DL/UL)

  - UE processing capabilities

  - Coverage

  - Energy efficiency

  - Mobility/speed

  - Sensing

  - AI

It is not realistic to consider such a long list for device type discussion. Each aspect can consist of different values, and this will result in RAN and eventually RAN WGs attempting to consider an unmanageable number of cases. On the other hand, it is also difficult at this stage to justify whether and how the device type will be defined, as the fundamental requirements for performance and services, as well as corresponding functions have not yet made sufficient progress. Once the requirements at RAN-level and functions/techniques at WG-level discussion have been thoroughly studied, it is easier to identify which aspects are the most essential factors for potential device type discussion.

In general, attempting to handle all the aspects of 6G devices and requirements simultaneously in recent RAN meetings has proven challenging, and a managed order of discussions would, we believe, prove more fruitful. The list of proposed aspects in RAN#109 is difficult to map to device types before sufficient understanding exists of how the aspects may be described, quantified where applicable, potentially depend on each other, and/or depend on specification features which have not been created yet. Attempting to continue the discussion at this time looks premature.

Proposal 1: RAN#110 shall prioritize requirements discussion relevant to performance targets and new services.

Proposal 2: The need and potential categorization of device types shall be discussed in RAN at a time aligned approximately with the second half of the RAN WGs study, and only once the study of RAN-level requirements and WG functions has made sufficient progress.

Once the conditions above are fulfilled, and RAN should resume consideration of how to structure the concept of diverse device types, some rationalization of the list provided in RAN#109 is evidently necessary. At present, it seems the list combines things which do relate to device types, such as Tx/Rx ‘antenna chains’, power classes, and maximum supported data rates, with things that to-date have not been clearly expressed for how they relate to device types rather than requirements, such as mobility/speed, coverage. Other items in the list might or might not be within the remit of ‘device types’.

Thus, we think a block-wise approach to the agreed list will be more useful, when the time is right per Proposal 2. For example, we think that essentially the following three items from the RAN#109 proposals are likely considered first, since they have relevance to coverage, spectral efficiency, etc.:

- Smallest maximum channel bandwidth for lowest-tier devices: This factor sets an upper limit on the initial access signal design.

- Number of Tx/Rx antennas: The number of antennas will have a fundamental impact on coverage and spectral efficiency.

- Transmit power: The UE transmit power will have a fundamental impact on coverage performance.

Observation 1: Of the aspects proposed by companies in RAN#109, the maximum UE bandwidth, number of Tx/Rx antennas, and UE transmit power (i.e. power classes) seem to have the most impact on the fundamental design and performance targets.

# Conclusion

According to the above discussions, we have the following proposals:

Proposal 1: RAN#110 shall prioritize requirements discussion relevant to performance targets and new services.

Proposal 2: The need and potential categorization of device types shall be discussed at the second half of WG study, once the study of RANP requirements and WG functions make sufficient progress.

Observation 1: Of the aspects proposed by companies in RAN#109, the maximum UE bandwidth, number of Tx/Rx antennas, and UE transmit power (i.e. power classes) seem to have the most impact on the fundamental design and performance targets.

# References

RP-252873, “Summary of 6G Diverse Device Types (including Minimum UE Bandwidth)”, RAN#109, Beijing, China, September 15-19, 2025.

RP-251881, “New SID: Study on 6G Radio”, RAN#108, Prague, Czech Republic, June 9-13, 2025.