3GPP TSG RAN Meeting #110												RP-253301

Baltimore, USA, December 8-11, 2025

Agenda Item:	8.2.1.0  Deployment scenarios

Source:	Volkswagen AG, BMW AG, Toyota ITC, Robert Bosch GmbH, AUMOVIO, Fraunhofer IIS

Title:	Discussion on deployment scenarios for Vehicular Use Cases

Document for:	Discussion

# Introduction

This contribution discusses proposals for TR 38.914 on deployment scenarios for Vehicular Use Cases. According to the Moderator’s summary for AI 8.2.1.1 “Deployment Scenarios” in RP-252914 [1] from RAN#109 the not agreed parts of TR 38.914 are captured in a pCR in RP-252915 [2], which should be used as a starting point for further discussion in RAN#110.

Following the guidance by the moderator in RP-252914 [1] and RP-252915 [2] and considering the automotive perspective, in this document consolidated versions of scenarios “4.11 Urban grid” and “4.12 Highway Scenario” are proposed for further discussion in RAN#110.

# Text Proposal

======================Start of the changes========================

# 4.	Deployment scenarios

## 4.11 Urban grid

The urban macro deployment scenario focuses on scenario of highly densely deployed vehicles moving in urban area. It could cover a scenario where freeways lead through an urban grid. The main KPIs evaluated under this scenario are availability, reliability, latency and data rate in high network load and high UE density scenarios.

Some of its attributes are listed in Table 4.11.

Table 4.11: Attributes for Urban grid

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency NOTE1 | Around 2 GHz Around 4 GHz for macro and micro Around 7 GHz [NOTE: no preferences on any combination of frequencies] [NOTE: 2GHz have been added on top RAN+109 conclusion] |
| Aggregated system bandwidth NOTE1,2 | Around 2 GHz: Up to [200] MHz (DL+UL) Around 4 GHz: Up to [200] MHz (DL+UL) Around 7 GHz: Up to [400] MHz (DL+UL) [NOTE: no preferences on bandwidth] [NOTE: 2GHz have been added on top RAN+109 conclusion] |
| Layout | Single layer: - Hex. Grid Baseline: Macro only (with the road configuration and BS placement as depicted in Figure 4.11 taken from [4]) |
| ISD | ISD 1: 500 m ISD 2: 250 m |
| BS height | [Macro BS: antenna height 25m Micro BS: antenna height 5m] [NOTE: no preferences on BS height] |
| Building size | 413m x 230m x 20m |
| BS antenna elements | Up to [x] for Tx and Rx antenna elements 2GHz Up to [512] for Tx and Rx antenna elements 4GHz Up to [2048] Tx and Rx antenna elements for 7GHz [NOTE: no preferences on amount of BS antenna elements] |
| UE antenna elements | Tx: Min 1 Tx, Up to [8] Tx antenna elements Rx: Min 2 Rx, Up to [8] Rx antenna elements |
| User distribution and UE speed | Urban grid model (car lanes and pedestrian/bicycle sidewalks are placed around a road block. 2 lanes (3.5m) in each direction, 4 lanes in total, 1 sidewalk (3m), one block size: 433m x 250m) 100% outdoor 20% Vehicular UE in the lanes with up to 120km/h 80% Vehicular UE in the lanes with up to 50km/h Pedestrian UE in the sidewalks with 3km/h UE changes its direction at the intersection as follows: - Go straight with probability 0.5 - Turn left with probability 0.25 - Turn right with probability 0.25 The distance between the rear bumper of a vehicle and the front bumper of the following vehicle in the same lane is max {2 meter, an exponential random variable with the average of the speed * 2 sec}. - All the vehicles in the same lane have the same speed. - Vehicle type distribution is not dependent of the lane. |
| Service profile | NOTE: Whether to use full buffer traffic or non-full-buffer traffic depends on the evaluation methodology adopted for each KPI. |

NOTE1:	The options noted here are for evaluation purpose, and do not mandate the deployment of these options or preclude the study of other spectrum options.

NOTE2:	The aggregated system bandwidth is the total bandwidth typically assumed to derive the values for some KPIs such as area traffic capacity and user experienced data rate. It is allowed to simulate a smaller bandwidth than the aggregated system bandwidth and transform the results to a larger bandwidth. The transformation method should then be described, including the modelling of power limitations.

An illustrative diagram of an urban grid layout can be found in Figure 4.11, which was taken from [4].

Figure 4.11: Exemplary depiction of urban grid and road configuration and BS locations (500m ISD) taken from [4]

## 4.12	Highway Scenario

The highway deployment scenario focuses on scenario of vehicles placed in highways with high speeds. The main KPIs evaluated under this scenario would be reliability/availability/sensing performance under high speeds/mobility (and thus frequent handover operations).

Some of its attributes are listed in Table 4.12.

Table 4.12 Attributes for highway scenario

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency NOTE1 | Macro only: Around 700 MHz Around 2 GHz Around 4 GHz Around 7 GHz Around 7GHz +Around 4GHz Around 2GHz+ Around 700 MHz [Around 7GHz +Around 4GHz + Around 2GHz+Around 700MHz] |
| System bandwidth NOTE1,2 | Around 700 MHz: Up to [60] MHz (DL+UL) Around 2GHz: Up to [120] MHz (DL+UL) Around 4 GHz: Up to [200] MHz (DL+UL) Around 7 GHz: Up to [400]MHz (DL+UL) [NOTE: no preferences on bandwidth] |
| Layout | Single macro layer: - Hex. Grid (only around the highway sides) Baseline: Macro only (straight line BS placement as in Figure 2.2-1 from [4] and with Road configuration in Figure 6.1.8-1 [3]) |
| ISD | ISD 1: 1732m (for around 4 and around 7 GHz) - 35m BS height for ISD 1732m ISD 2: 5000m (Optional ISD 3): 500m - 25m BS height for ISD 500m [NOTE: no preferences on BS height] |
| BS antenna elements | Up to [x] for Tx and Rx antenna elements 2GHz Up to [512] for Tx and Rx antenna elements 4GHz Up to [2048] Tx and Rx antenna elements for 7GHz [NOTE: no preferences on amount of BS antenna elements] |
| UE antenna elements | Tx: Min 1 Tx, Up to [8] Tx antenna elements Rx: Min 2 Rx, Up to [8] Rx antenna elements |
| User distribution and UE speed | Option 1: 85% outdoor vehicles (130km/h), 15% outdoor vehicles (200 km/h) Option 2: 100% outdoor vehicle, Speed is 120km/h Randomly distributed in the middle of each lane. - Average inter-vehicle distance (between two vehicles’ center) in the same lane is 2sec * average vehicle speed (average speed: 100-200km/h) - All the vehicles in the same lane have the same speed. - Vehicle type distribution is not dependent of the lane. |
| Service profile | NOTE: Whether to use full buffer traffic or non-full-buffer traffic depends on the evaluation methodology adopted for each KPI. |

NOTE1:	The options noted here are for evaluation purpose, and do not mandate the deployment of these options or preclude the study of other spectrum options.

NOTE2:	The aggregated system bandwidth is the total bandwidth typically assumed to derive the values for some KPIs such as area traffic capacity and user experienced data rate. For UL User Experienced Data rate it is not allowed to simulate a smaller bandwidth than the aggregated system bandwidth and transform the results to a larger bandwidth.

Considering the text proposals above, the following proposal is presented:

Proposal: Consider the inclusion of the text in section 2 of this TDoc into TR38.914, Section 4 “Deployment Scenarios”.

# Conclusion

In this document consolidated versions of the deployment scenarios Urban grid and Highway scenario were introduced.

The following proposal is presented:

Proposal: Consider the inclusion of the text in section 2 of this TDoc into TR38.914, Section 4 “Deployment Scenarios”.

# References

[1] RP-252914, Deutsche Telekom AG (Moderator), Moderator's summary AI 8.2.1.1 “Deployment  Scenarios”, Beijing, China, September 2025

[2] RP-252915, Deutsche Telekom AG (Moderator), pCR on Deployment scenarios for TR 38.914 (e.g for RAN#110 discussion continuation), Beijing, China, September 2025

[3]	3GPP TR 38.913: “Study on Scenarios and Requirements for Next Generation Access Technologies”

[4]	 RP-252127: “Discussion on 6G ISAC deployment scenarios”