3GPP TSG RAN meeting #110								                      RP-253509

Baltimore, US, December 08-11, 2025

Status Report to TSG

Agenda item: 9.3.4.1

| WI / SI Name | UE RF enhancements for NR FR1, Phase 5 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NR_UE_RF_Ph5 |  |  |  |  |
| Unique ID | 1090065 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-252952 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: | Core part: Mar. 2027 | Performance part: Sep. 2027 | Testing part: |  |
| Overall Completion level | Study Item: | Core part: 20% | Performance Part: 0% | Testing part: |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN4 |
| --- | --- | --- |
| Rapporteur | Name | Ye Liu Bill Shvodian |
|  | Company | Huawei T-Mobile USA |
|  | Email | leo.liuye@huawei.com bill.shvodian@T-Mobile.com |

1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

2.1	RAN1

2.1.1	Agreements

2.1.2	Remaining Open issues

2.2	RAN2

2.2.1	Agreements

2.2.2	Remaining Open issues

2.3	RAN3

2.3.1	Agreements

2.3.2	Remaining Open issues

2.4	RAN4

2.4.1	Agreements

RAN4#117

UE RF Core part:

- High power UE (HPUE) for NR single carrier operation (WF R4-2522975)

- For 4Tx with up to 32 dBm MOP

- Maximum output power is discussed together with power class

  - Maximum output power is discussed together with power class

    - Companies report needed back off in relation to 32 dBm for different modulations and Inner/Outer/edge allocations

    - FFS how to handle the backoff and power class in specification

- Uplink full power transmission Mode-1 and Mode-2 feasibility

  - Requirements shall be defined with Mode-1. Mode-2 can be discussed later

- Uplink full power transmission Mode-full power feasibility

  - RAN4 agree that Mode-full power is not feasible under the objectives of this WI.

- REFSENS requirements

  - RAN4 agree to reuse the current requirements.

- UE ACS requirements

  - RAN4 agree to reuse the current requirements.

- Other Rx requirements

  - RAN4 agree that no additional receiver requirements needs to be defined.

- Mode-full power feasibility for PC2

  - RAN4 agree that Mode-full power for PC2 is not under the scope of this WI.

- For PC1.5 with 2Tx for FDD bands

  - Reusing MPR for 2Tx PC1.5 from TDD bands

    - Companies are encouraged to check if greater than 10 dB isolation is possible for low FDD bands.

    - Aim to conclude in RAN4#118

  - NS to be evaluated

    - Agree to also evaluate PC1.5 A-MPR for NS_05U, and NS_100.

    - De prioritize NS_03U

    - FFS: UTRA ACLR to be used

- For method to predict PC1.5 2TX RSD for FDD bands (WF R4-2523041)

- Firstly, investigate if it is possible to have a unified approach to predict 2TX PC1.5 RSD from the 1TX/2TX PC2 RSD in the specification or directly from PC3. The approach could be either a procedure or a simple delta value that can be applied to other FDD bands. Determine the feasibility of applying a method or applying a simple delta value.

Consider the following starting point assumptions:

  - Baseline PC3 REFSENS scalable to the desired BW assuming no TX OOB and TX blocking effect.

  - Existing 1TX/2TX PC2 RSD agreed vales in the specification.

  - Existing RFFE architecture used for PC2 1TX/2TX RSD. (ie. Existing filter isolations still apply)

  - Interference Imbalance [Consider 6dB, 8dB as starting values].

- Secondly, determine if a generalized LUT approach is possible to predict the 1TX PC2, 2TX PC2, and 2TX PC1.5 RSD. There could be deviations from the already agreed values in the specification for 1TX PC2 and 2TX PC2, and it is up to the group to decide whether the deviation is acceptable for the sake of simplicity.

  - Baseline PC3 REFSENS scalable to the desired BW assuming no TX OOB and TX blocking effect.

  - Existing RFFE architecture used for PC2 1TX/2TX RSD. (ie. Existing filter isolations still apply)

  - Interference Imbalance [Consider 6dB, 8dB as starting values].

- Thirdly, consider the following aspects:

  - Evaluation criteria for the method: Check that the derived PC1.5 2Tx RSD is larger than the PC2 1Tx and 2Tx RSD.

  - When UL interference cannot be extracted from the PC3 REFSENS but a PC2 RSD is specified, consider using PC2 as a starting point as an exception.

  - Whether the method can support future PC2 1Tx and 2Tx and PC1.5 1Tx RSD evaluation.

- HPUE co-existence study (WF R4-2522974)

- WF captures agreed updates on simulation assumptions for adjacent channel co-existence study for 4Tx HPUE with up to 32dBm max output power, including

- Simulation scenarios

- Network layout model

- Propagation model

- Antenna characteristics

- Uplink transmission power control model

- Other simulation parameters

- Performance metric

- 6MHz channel bandwidth (WF R4-2522973)

- UE RF

  - NS_06

  - For PC3 NS_06, no A-MPR is applicable for 6MHz CBW.

  - For PC2 NS_06, have the same A-MPR for 6 MHz CBW as for other CBWs as specified in Table 6.2.3.32-1 in TS 38.101-1.

  - Above two agreements lead to no spec change.

  - NS_15

    - Evaluation A-MPR for NS_15 and 6 MHz channel bandwidth assuming deployment within 814-845 MHz or within 841.5-849 MHz.

    - To enhance the similarity of company proposals, use the regions defined for 7 MHz channel bandwidth (Table 1 and Table 2) and modify them to accommodate the 6 MHz channel. Further evaluate/align the A-MPR regions and values in next meeting.

Table 1: A-MPR regions for NS_15 and PC3 Channel BW; Carrier Frequency, Fc, MHz; RBend*12*SCS (MHz); LCRB*12*SCS (MHz); A-MPR. 7MHz; 841.5 < Fc ≤ 845.5; ≥ 4.5; > 0; A7. ≥ 2.16, < 4.5; ≥ 1.8; A2. ≤ 1.44; ≤ 0.36; A8. 838 < Fc ≤ 841.5; > 0; > 2.7; A9. ≥ 5.94; ≤ 2.7; A10. Table 2: A-MPR regions for NS_15 and PC2 Channel BW; Carrier Frequency, Fc, MHz; RBend*12*SCS (MHz); LCRB*12*SCS (MHz); A-MPR. 7MHz; 841.5 < Fc ≤ 845.5; ≥ 4.5; > 0; A8. ≥ 2.16, < 4.5; ≥ 1.8; A9. ≥ 2.16, < 4.5; ≥ 1.44, < 1.8; A10. < 1.44; ≤ 0.36; A11. 837 < Fc ≤ 841.5; ≥ 0; > 2.7; A12. ≥ 5.94; ≤ 2.7; A13.

- BS RF

  - ACS

    - Specify the ACS interferer frequency offset with ±2.5125 MHz value.

  - Narrow band blocking

    - Specify the interfering RB centre frequency offset to the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap with value ± (360+m*180) KHz, with m=0, 1, 2, 3, 4, 9, 14, 19, 24.

  - Intermodulation

    - Specify the interfering signals for the intermodulation requirement according to the following table:

Table: Interfering signals for intermodulation requirement BS channel bandwidth of the lowest/highest carrier received (MHz); Interfering signal centre frequency offset from the lower/upper Base Station RF Bandwidth edge (MHz); Type of interfering signal (Note 3). 6; ±7.475; CW. ±17.5; 5 MHz DFT-s-OFDM NR signal (Note 1).

  - Narrow band blocking

    - Specify the narrowband intermodulation requirement according to the following table:

Table: Interfering signals for narrowband intermodulation requirement in FR1 BS channel bandwidth of the lowest/highest carrier received (MHz); Interfering RB centre frequency offset from the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap (kHz) (Note 3); Type of interfering signal. 6; ±425; CW. ±1240; 5 MHz DFT-s-OFDM NR signal, 1 RB (Note 1).

  - In-Channel selectivity

    - Specify the In-channel selectivity requirements according to the following table:

Table: Wide Area BS in-channel selectivity BS channel bandwidth (MHz); Subcarrier spacing (kHz); Reference measurement channel; Wanted signal mean power (dBm); Interfering signal mean power (dBm); Type of interfering signal. 6; 15; G-FR1-A1-7; -100.6; -79.6; DFT-s-OFDM NR signal, 15 kHz SCS, 15 RBs.

Performance part:

N/A

RAN4#116bis

UE RF Core part:

- High power UE (HPUE) for NR single carrier operation (Workplan R4-2515086, WF R4-2515087)

- For 4Tx with up to 32 dBm MOP

- MPR/A-MPR assumptions

    - Evaluate assuming following antenna isolation assumptions

      - For CPE, assuming both 10dB and 20dB,

      - For FWA, assuming 20dB.

Come back once MPR results are available, to evaluate if it is possible to select a single CPE antenna isolation assumption based on the MPR results.

- NS to be evaluated

    - Evaluate A-MPR for NS_04 and NS_47 for n41

- For PC1.5 with 2Tx for FDD bands

- Maximum output power

    - 29 dBm is the maximum output power.

- ACLR assumption

    - 31 dB is the agreed ACLR value

- NS to be evaluate

    - Agree to evaluate PC1.5 A-MPR for NS_03, NS_05, NS_48, and NS_49.

    - Encourage companies to check if NS_03U, NS_05U, and NS_100 are needed.

      - If needed, how to define UTRA ACLR for PC1.5

- REFSENS degradation

    - Encourage companies to provide the general methodology for deriving the REFSENS degradation (RSD) in next meeting.

- HPUE co-existence study (WF R4-2515085)

- Preliminary simulation assumptions were captured in the WF, some of them are to be further determined, e.g. study scenarios, UE antenna patterns, UL power control model as well as performance metric.

- 6MHz channel bandwidth (WF R4-2515084)

- System parameter

  - Spectrum utilization

  - For 6MHz (SCS=15KHz), the maximum transmission bandwidth configuration (NRB) is 30RB and guard band is 292.5KHz.

  - Channel spacing, channel raster and sync raster

  - Channel spacing, channel raster and sync raster are not affected by the introduction of 6 MHz channel bandwidth.

  - Optional CBW

    - Support of 6 MHz is optional in Rel-20 and in the previous releases. Update TS38.101-1 Table 5.3.5-1 as shown below.

    - For single CC operation, 6 MHz is release independent from Rel-15.

SCS (kHz); UE Channel bandwidth (MHz). 3; 5; 6; 10; 15; 20; 25; 30; 35; 40; 45; 50; 60; 70; 80; 90; 100. n5; 15; 34; 5; 64; 10; 15; 20; 253. 30; 10; 15; 20; 253. 60. n8; 15; 5; 64; 10; 15; 20; 253; 303; 353. 30; 10; 15; 20; 253; 303; 353. 60. n12; 15; 34; 5; 64; 10; 15. 30; 10; 15. 60. n29; 15; 5; 64; 10. 30; 10. 60. n85; 15; 34; 5; 64; 10; 15. 30; 10; 15. 60. NOTE 4: This UE channel bandwidth is optional in this release of the specification..

  - Minimum CBW

    - 6 MHz can not be the minimum channel BW in a NR band.

- UE RF

  - SEM

    - SEM requirement for 6MHz channel BW is the same with 7 and 10 MHz channel BW.

  - MPR

    - The MPR requirements are not impacted by the introduction of 6MHz.

  - NS_01 and NS_100

    - NS_01: 6 MHz shall be added to the list of considered channel BW for the NS.

    - NS_100: No update needed.

  - NS_06, NS_14, NS_15, NS_43/43U

    - Conduct A-MPR evaluation (if needed) for

      - NS_06 for n12 (PC3)

        - For information: Current spec states “For power class 3 operation on bands n12, n13, n14, n85 and n110, no A-MPR is applicable.

      - NS_06 for n85 (PC3 and PC2)

        - For information: Current spec states “For power class 3 operation on bands n12, n13, n14, n85 and n110, no A-MPR is applicable.

      - NS_14 for n5 (PC3 and PC2): A-MPR evaluation is not needed for 6MHz leveraging the experience of 7 MHz

      - NS_15 for n5 (PC3 and PC2)

      - NS_43/43U for n8: A-MPR evaluation is not needed as 6MHz is not demanded in Japan as confirmed by SoftBank

    - Additional requirements for NS_06 for 6MHz is defined as

ΔfOOB (MHz); Channel bandwidth (MHz) / Spectrum emission limit (dBm); Measurement bandwidth. 3; 5; 6; 10; 15. ± 0 – 0.1; -13; -15; -15; -18; -20; 30 kHz. ± 0.1 – 1; -13; -13; -13; -13; -13; 100 kHz. ± 1 – 5; -13; -13; -13; -13; -13; 1 MHz. ± 5 – 6; -25; -13; -13; -13; -13; 1 MHz. ± 6 – 10; -25; -25. ± 10 – 11; -25. ± 11 – 15. ± 15 – 20; -25.

      - FFS whether 6 MHz channel bandwidth needs to be added in the Additional requirements for “NS_15” table

        - More information from Operators are needed. This aspect can be further checked offline before Nov meeting.

  - Minimum Output power, Transmit Off power, Occupied bandwidth, ACLR, general spurious

    - The requirement for 6 MHz is the same requirement with the requirement of 5/7/10MHz. 6 MHz shall be added to:

      - Table 6.3.1-1 (minimum output power).

      - Table 6.3.2-1(Transmit Off power).

      - Table 6.5.1-1 (occupied bandwidth).

      - Table 6.5.2.4.1-1 (ACLR).

      - Table 6.5.3.1-1 (general spurious).

    - Other Tx requirements are not impacted by the introduction of 6 MHz channel BW.

  - REFSENS and UL configuration for FDD bands

Operating Band; SCS kHz; 3 MHz (dBm); 5 MHz (dBm); 6 MHz (dBm); 7 MHz (dBm); 10 MHz (dBm); 15 MHz (dBm); 20 MHz (dBm); 25 MHz (dBm); 30 MHz (dBm); 35 MHz (dBm). n5; 15; -100.2; -98.0; -97.2; -96.5; -94.8; -93.0; -86.8; -84.8. 30; -95.1; -93.1; -88.6; -84.9. n8; 15; -97.0; -96.2; -93.8; -91.4; -85.8; -83.6; -81.3; -78.4. 30; -94.1; -91.7; -87.2; -84.7; -81.4; -78.5. n12; 15; -99.2; -97.0; -96.2; -93.8; -84.0. 30; -94.1; -84.1. n85; 15; -99.2; -97.0; -96.2; -93.8; -84.0. 30; -94.1; -84.1. Operating Band; SCS; 3; 5; 6; 7; 10; 15; 20; 25; 30; 35; Duplex Mode. n5; 15; 15; 25; 251; 251; 251; 201; 201; Note 5; FDD. 30; 121; 101; 101; Note 5. n8; 15; 25; 251; 251; 201; 201; Note 5; Note 5; Note 5; FDD. 30; 121; 101; 101; Note 5; Note 5; Note 5. n12; 15; 15; 201; 201; 201; 201; FDD. 30; 101; 101. n85; 15; 15; 201; 201; 201; 201; FDD. 30; 101; 101.

  - REFSENS for SDL band n29

    - Add 6MHz for n29 in Table 7.3.2-1b

  - Maximum input level

    - The requirement for 6 MHz is the same requirement than the requirement for 3-20 MHz. 6 MHz shall be added to Table7.4-1.

  - ACS, In-band blocking, Out of band blocking, Narrow Band blocking, Intermodulation, Spurious response

    - The requirement for 6 MHz is the same requirement with the requirement for 5,7 and10 MHz. 6 MHz shall be added to:

      - Tables 7.5-1, 7.5-3 and 7.5-4 (ACS).

      - Table 7.6.2-1 (In-band blocking).

      - Table 7.6.3-1, and power in transmission bandwidth configuration as REFSENS + 6.0 (Out of band blocking).

      - Table 7.6.4-1, and Pw =15 (Narrow band blocking).

      - Table 7.7-1(Spurious response).

      - Table 7.8.2-1(Intermodulation).

    - Other Rx requirements are not impacted.

- General for BS

  - NB-IoT support

    - NB-IoT operating in NR in-band is supported for 6 MHz channel bandwidth.

  - Multi-carrier and CA

    - Muti-carrier (including MSR) and CA related requirements are considered for new added 6MHz WID but no new BCS is specified within this WI.

  - IAB and NCR

    - 6 MHz should not be supported by IAB and NCR

- BS RF

  - ACLR

    - Add 6 MHz to the list of NR channel bandwidths for ACLR/CACLR.

  - Total power dynamic range

    - The total power dynamic range is 14.7dB for core requirement

  - EVM

    - Annex B.5.3 should be updated to include EVM window length for 6 MHz channel BW, which is same with 5 and 7MHz channel BW.

  - OBW test requirement

    - For the Occupied bandwidth test requirement, define Span as 12 MHz, with Minimum number of measurement points as 400.

  - FRC for Rx dynamic range

    - For RF Rx dynamic range requirement, reuse FRC G-FR1-A2-1

  - REFSENS

    - REFSENS for 6 MHz channel BW is same with 5, 7, 10, 15MHz, assuming FRC G-FR1-A1-1

  - Dynamic range (conducted core)

    - Dynamic Range limit and Interfering signal level for 6 MHz channel BW.

      - Wanted signal is:

        - WA BS: -70.7 dB

        - MR BS: -65.7 dB

        - LA BS: -62.7 dB

      - Interfering signal is:

        - WA BS: -81.7 dB

        - MR BS: -76.7 dB

        - LA BS: -73.7 dB

  - Dynamic range for NB-IoT operation in NR in-band

    - Add 6MHz CBW to Table 6.2.4.2-1, reuse legacy NB-IoT RB power dynamic range for 3, 5, 7, 10MHz CBW.

  - Other impacted Rx requirements

    - ACS

    - General blocking

    - Narrow band blocking

    - Receiver intermodulation

    - In-Channel selectivity

Performance part:

N/A

2.4.2	Remaining Open issues

High power UE (HPUE) for NR single carrier operation

- Specify the UE RF requirements for single carrier TDD to support 4Tx for CPE/FWA UE with 4x26dBm PA configurations

  - Co-existence study for ACLR, specify ACLR requirements if needed

  - Specify the UE RF requirements including maximum output power (MOP), MPR requirements, etc.

    - Supporting 4x4 UL MIMO and TxD

  - Specify the band specific requirements for exemplary bands

    - Example bands: n41, n77/n78, n79, n104

  - Maximum output power is up to 32 dBm

    - RAN4 to determine whether to reuse PC1 or introduce new power class

- PC1.5 with 2Tx for FDD bands for handheld and FWA UE

  - Specify the UE RF requirements including maximum output power (MOP), MPR requirements, etc.

    - Supporting 2x2 UL MIMO and TxD

    - Baseline PA configuration: 2 x 26 dBm

  - Specify the band specific requirements for exemplary bands

  - Example bands: n1, n25

- PC1.5 1Tx for TDD bands for FWA UE

  - Study the possibility of applying PC1.5 with 2Tx MPR/A-MPR requirements to PC1.5 with 1Tx, if feasible, specify the corresponding UE RF requirements, including maximum output power (MOP), MPR/A-MPR requirements, etc.

    - Baseline PA configuration: 1 x 29 dBm

    - Example bands: n41, n77/n78, n79

  - This work is to start in 2026 Q4

- 2Tx for TDD bands for FWA UE

  - Study the possibility of applying PC1 with 4Tx A-MPR/MPR requirements to PC1 with 2Tx, if feasible, specify the corresponding UE RF requirements, including maximum output power (MOP), MPR/A-MPR requirements, etc.

    - Baseline PA configuration: 2 x 29 dBm

    - Example bands: n41, n77/n78, n79, n104

  - Maximum output power is up to 32 dBm

    - RAN4 to determine whether to reuse PC1 or introduce new power class

  - This work is to start in 2026 Q4

6MHz channel bandwidth

- Specify the generic core requirements to support 6 MHz NR FR 1 channel bandwidth [RAN 4]

  - Only include 15 kHz SCS

  - Specify the spectrum utilization

  - Specify UE RF requirements for UL and DL

  - Specify BS RF requirements (OTA and conducted) for UL and DL

- Specify the band specific requirements to introduce 6 MHz for NR band n5, n8, n12, n85, n29 [RAN 4]

  - BS and UE Channel bandwidth per operating band

  - UE reference sensitivity

  - When needed

    - Additional Maximum Power Reduction (A-MPR)

- Once RAN 4 has introduced the new channel bandwidth, RAN 2 will introduce the necessary signalling for this channel bandwidth [RAN 2]

  - Note This should be similar to the signalling for 7 MHz channel bandwidth

- Once RAN 4 has introduced the new channel bandwidth, RAN 3 will introduce the necessary signalling for this channel bandwidth [RAN 3]

  - Note This should be similar to the signalling for 7 MHz channel bandwidth

NOTE: Once this WI is completed, the 6 MHz bandwidth can be introduced also for other bands (not addressed in this WI) via the appropriate basket WI. The support of 6MHz bandwidth is release independent.

General for all the areas

- Specify release independence requirements for the core part in TS 38.307, if needed and feasible.

Performance part:

6MHz channel bandwidth

- Specify, if necessary, UE demodulation performance and CSI reporting requirements for the 6 MHz channel bandwidth

- Specify, if necessary, BS demodulation performance requirements

- Specify necessary BS conformance tests

2.5	RAN5

2.5.1	Agreements

2.5.2	Remaining Open issues

2.5.3	Remaining Open issues with cross-WG dependencies

2.6	RAN6

2.6.1	Agreements

2.6.2	Remaining Open issues

3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

3.1	SAx/CTs

3.1.1	Agreements with cross-TSG impacts

3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

RAN4#117

- R4-2522226	Topic summary for [117][316] NR_UE_RF_Ph5	Moderator (Nokia)

- R4-2522227	Topic summary for [117][329] NR_UE_RF_Ph5_Part2	Moderator (Samsung)

- R4-2522973	Way Forward for [117][329] NR_UE_RF_Ph5_Part2: 6 MHz	Samsung

- R4-2522974	Way Forward for [117][329] NR_UE_RF_Ph5_Part2: HPUE coexistence	Huawei

- R4-2522975	Way Forward for [117][316] NR_UE_RF_Ph5	Nokia, Murata, Skyworks, MediaTek

- R4-2523041	Way Forward for [117][316] PC1.5 2Tx RSD methodology	Murata, Skyworks, MediaTek, Qualcomm, Apple, Nokia

- R4-2520090	Discussion on R20 HPUE	CATT

- R4-2520224	Discussion on general methodology for FDD PC1.5 2Tx RSD	Murata Manufacturing Co Ltd.

- R4-2520263	On HPUE RF requirements for Rel-20	Qualcomm Incorporated

- R4-2520290	Influence of antenna coupling and ACLR on 4x26dBm and 2x26dBm HPUE back-off	Skyworks Solutions Inc.

- R4-2520580	On the introduction of PC1.5 with 2Tx for FDD bands	Apple

- R4-2520732	Discussion on HPUE for NR single carrier operation	vivo

- R4-2520770	Discussion on RF requirements for Rel-20 HPUE	ZTE Corporation, Sanechips

- R4-2520823	TN HP related RF requirements for both HHUE and FWA	MediaTek Korea Inc.

- R4-2521142	Views on R20 4Tx HPUE requirements	China Telecom

- R4-2521305	PC1.5 2Tx A-MPR simulations	Nokia

- R4-2521403	On High power UE (HPUE) for NR single carrier operation	Nokia

- R4-2521517	Discussion for Rel-20 HPUE RF requirement	LG Electronics France

- R4-2522135	Discussion on HPUE RF requirements	Huawei, HiSilicon

- R4-2522170	On HPUE RF requirements for 4Tx with up to 32 dBm and 2Tx PC1.5 FDD	Ericsson

- R4-2520188	Further discussion on BS RF requirements for 6MHz channel bandwidth	CATT

- R4-2520189	Further discussion on UE RF requirements for 6MHz channel bandwidth	CATT

- R4-2520286	On BS specification impact due to introduction of 6MHz channel bandwidth	Nokia

- R4-2520546	6 MHz A-MPR simulations and proposals	Nokia

- R4-2520579	On the introduction of 6MHz CBW	Apple

- R4-2520771	Discussion on BS RF requirements for 6MHz CBW	ZTE Corporation, Sanechips

- R4-2521428	6 MHz channel BW - BS RF impacts	Ericsson

- R4-2521429	6 MHz channel BW - system and UE RF impacts	Ericsson

- R4-2522133	PC3 A-MPR requirements for 6MHz channel bandwidth	Huawei Technologies France

- R4-2522187	Further discussion on the BS RF requirements for introduction of the 6MHz channel bandwidth	Huawei, HiSilicon

- R4-2522243	6 MHz Channel Bandwidth A-MPR Analysis	Qualcomm Incorporated

- R4-2520089	Discussion on R20 HPUE coexistence study	CATT

- R4-2520279	Simulation results for HPUE coexistence study	Nokia

- R4-2520280	Proposals on simulation assumptions for HPUE coexistence study	Nokia

- R4-2521261	Co-existence study for HPUE with maximum output power up to 32dBm	Huawei, HiSilicon

- R4-2521896	Views on the coexistence study for High Power UE	Qualcomm Germany

- R4-2522171	On HPUE 4Tx PC1 initial coexistence study results	Ericsson

- R4-2522323	Initial coexistence evaluation results for TN FWA 4x26dBm device in Rel-20	MediaTek

- R4-2522972	Discussion on co-existence for HPUE	ZTE Corporation, Sanechips

RAN4#116bis

- R4-2514439	Topic summary for [116bis][316] NR_UE_RF_Ph5	Moderator (Nokia)

- R4-2514536	Topic summary for [116bis][329] NR_UE_RF_Ph5_Part2	Moderator (Samsung)

- R4-2515084	Way Forward for [116bis][329] NR_UE_RF_Ph5_Part2 -- 6 MHz channel bandwidth	Samsung, Nokia, Ericsson, Skyworks, Apple, Huawei, CATT, ZTE, Qualcomm

- R4-2515085	Way Forward for [116bis][329] NR_UE_RF_Ph5_Part2 - HPUE coexistence	Huawei

- R4-2515087	Way Forward for [116bis][316] NR_UE_RF_Ph5	Nokia, Murata

- R4-2513832	6 MHz channel BW - work plan	Ericsson, T-Mobile USA

- R4-2513916	Work plan for UE RF enhancements for NR FR1, Phase 5 —— HPUE	Huawei, HiSilicon

- R4-2513925	Discussion on general aspects for 4Tx HPUE	ZTE Corporation, Sanechips

- R4-2514385	Workplan for Rel20 UE RF enhancements Ph5	Nokia

- R4-2515086	Workplan for Rel20 UE RF enhancements Ph5	Nokia, Huawei, T-Mobile, Ericsson, Samsung

- R4-2513174	Discussion on R20 HPUE	CATT

- R4-2513567	Initial views on HPUE for NR single carrier operation	vivo

- R4-2513734	Discussion on UE RF enhancements for NR FR1, Phase 5	LG Electronics France

- R4-2513760	Initial view for TN HP related topics for both HHUE and FWA	MediaTek Korea Inc.

- R4-2513781	Inputs to 4Tx based on 26dBm PAs and PC1.5 for FDD	Skyworks Solutions Inc.

- R4-2513849	On the introduction of PC1.5 with 2Tx for FDD bands	Apple

- R4-2513927	Discussion on RF requirements for HPUE	ZTE Corporation, Sanechips

- R4-2513990	Discussion on R20 4Tx and 2Tx HPUE	OPPO

- R4-2513991	Discussion on R20 PC1.5 with 2Tx for FDD HPUE	OPPO

- R4-2514073	On High power UE (HPUE) for NR single carrier operation	Nokia

- R4-2514212	Discussion on HPUE requirements for 4TX	Spreadtrum, UNISOC

- R4-2514249	Discussion on Rel-20 HPUE	Samsung

- R4-2514355	Views on PC1.5 with 2 Tx for FDD bands for handheld and FWA UE	Qualcomm Incorporated

- R4-2514418	Discussion on HPUE RF requirements for NR single carrier operation	Huawei, HiSilicon

- R4-2514420	UE RF requirement set for 4x26 dBm PC1	Qualcomm Incorporated

- R4-2514486	Initial views on Rel-20 HPUE requirements	NTT DOCOMO INC.

- R4-2514500	On HPUE RF requirements for 4Tx PC1 and 2Tx PC1.5 FDD	Ericsson

- R4-2513067	On BS specification impact due to introduction of 6MHz channel bandwidth	Nokia

- R4-2513236	Discussion on system parameters and UE RF requirements for 6MHz channel bandwidth	CATT

- R4-2513237	Discussion on BS RF requirements for 6MHz channel bandwidth	CATT

- R4-2513792	Inputs to 6MHz channel bandwidth	Skyworks Solutions Inc.

- R4-2513793	on 6MHz	OPPO

- R4-2513833	6 MHz channel BW - BS RF impacts	Ericsson

- R4-2513834	6 MHz channel BW - system and UE RF impacts	Ericsson

- R4-2513848	On the introduction of 6MHz CBW	Apple

- R4-2513928	Discussion on 6MHz CBW BS RF	ZTE Corporation, Sanechips

- R4-2513929	Discussion on 6MHz CBW UE RF	ZTE Corporation, Sanechips

- R4-2514045	Discussion on system parameter and UE RF aspects of 6 MHz channel bandwidth	Nokia

- R4-2514289	Introduction of 6MHz channel bandwidth: System parameters and RF requirements	Huawei, HiSilicon

- R4-2514356	6 MHz Channel Bandwidth for UE RF	Qualcomm Incorporated

- R4-2514470	Introduction of 6MHz channel bandwidth: BS RF aspects	Huawei, HiSilicon

- R4-2513173	Discussion on R20 HPUE coexistence study	CATT

- R4-2513566	Discussion on HPUE co-existence study	vivo

- R4-2513759	Coexistence evaluation and detail parameters on TN FWA 4x26dBm device in Rel-20	MediaTek Korea Inc.

- R4-2513917	Co-existence study for HPUE with maximum output power up to 32dBm	Huawei, HiSilicon

- R4-2513926	Discussion on co-existence for HPUE	ZTE Corporation, Sanechips

- R4-2513996	Coexistence considerations for High Power UEs	Qualcomm Germany

- R4-2514501	On HPUE 4Tx PC1 coexistence study aspects	Ericsson

01.08.2022		minor adaptations for RAN #97e

21.05.2022		minor adaptations for RAN #96

10.01.2022		minor adaptations for RAN #95e

04.10.2021		minor adaptations for RAN #94e

08.08.2021		minor adaptations for RAN #93e

17.05.2021		minor adaptations for RAN #92e

28.01.2021		minor adaptations for RAN #91e

09.11.2020		minor adaptations for RAN #90e

31.08.2020		minor adaptations for RAN #89e

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template