3GPP TSG-RAN WG5 Meeting # 109	R5-256323

# Dallas, US, November 17 – 21, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-2 | CR | 0899 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of UL MIMO band n100 for ICS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_bands_xFeature_R19-UEConTest |  |  |  |  |  | Date: |  |  | 2025-10-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The baseline implementation capabilities for ULMIMO band n100 is missing and should be added. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add UL MIMO band n100 info into: - Table A.4.3.9-12: NR FR1 UL MIMO Capabilities. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The RF baseline implementation capabiltiers for UL MIMO band n100 will be missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.4.3.9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Start of changes >>

A.4.3.9	Additional capabilities for UE declared capability

Table A.4.3.9-1: UE declared capabilities

| Item | UE declared capabilities | Ref. | Release | Mnemonic | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | Enhanced Type 1 Receiver for NR | 38.101-4, 5 | Rel-15 | pc_nr_enh_type1_receiver | Support for Enhanced Type 1 Receiver (SU-MIMO Interference Mitigation advanced receiver) |
| 2 | Vehicular UE | 38.101-1, 3 | Rel-15 | pc_nr_vehicular_ue |  |
| 3 | MMSE-IRC (Minimum Mean Square Error - Interference Rejection Combining) receiver | 38.101-4, 5 | Rel-15 | pc_nr_mmse_irc_receiver | Support of MMSE-IRC processing for scenarios with inter-cell and intra-cell inter-user interference. |
| 4 | Baseline SU-MIMO 8Rx receiver | 38.306, 5.2 | Rel-17 | pc_baseline_su-mimo_8rx_receiver | Support of 8Rx receivers for SU-MIMO transmissions with support of up to 8 layers with joint 8Rx MIMO detector in FR1 |
| 5 | Simplified SU-MIMO 8Rx receiver | 38.306, 5.2 | Rel-17 | pc_simplified_su-mimo_8rx_receiver | Support of 8Rx receivers for SU-MIMO transmissions with support of up to 4 layers with two joint 4Rx MIMO detectors in FR1 |
| 6 | PDSCH absolute physical layer throughput requirements with link adaptation | 38.101-4, 5.6 38.307, 5.4 | Rel-17 | pc_pdsch_phy_link_adaptation | Support of PDSCH absolute physical layer throughput requirements with link adaptation (Optional for Rel-17 UEs) |
| 7 | Enhanced Type 2 Receiver Type 2-2a | 38.101-4, 3 38.306, 5.2 | Rel-18 | pc_nr_enh_type2_2a_receiver | Support of R-ML (reduced complexity ML) receivers with enhanced inter-user interference suppression for MU-MIMO for 2 layers across target and co-scheduled UEs with 2RX and 4RX in FR1. |
| 8 | Enhanced Type 2 Receiver Type 2-2b | 38.101-4, 3 38.306, 5.2 | Rel-18 | pc_nr_enh_type2_2b_receiver | Support of R-ML receivers with enhanced inter-user interference suppression for MU-MIMO for 2 layers across target and co-scheduled UEs with 2RX and maxNumberMIMO-LayersPDSCH layers across target and co-scheduled UEs with 4RX in FR1. |

-------- Skipped unchanged texts --------

Table A.4.3.9-12: NR FR1 UL MIMO Capabilities

| Item | RF Baseline Implementation Capabilities | Ref. | 2Tx (Note 3, 4) | 4Tx (Note 3, 4) | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | NR Frequency band: 1920-1980 MHz, 2110-2170 MHz | 38.101-1, 5.2D |  |  | NR FDD FR1 Band 1 |
| 2 | NR Frequency band: 1850-1910 MHz, 1930-1990 MHz | 38.101-1, 5.2D |  |  | NR FDD FR1 Band 2 |
| 3 | NR Frequency band: 1710-1785 MHz, 1805-1880 MHz | 38.101-1, 5.2D |  |  | NR FDD FR1 Band 3 |
| ... |  |  |  |  |  |
| 5 | NR Frequency band: 824-849 MHz , 869-894 MHz | 38.101-1, 5.2D |  |  | NR FDD FR1 Band 5 |
| ... |  |  |  |  |  |
| 7 | NR Frequency band: 832-862 MHz, 791-821 MHz | 38.101-1, 5.2D |  |  | NR FDD FR1 Band 7 |
| 8 | NR Frequency band: 880-915 MHz, 925-960 MHz | 38.101-1, 5.2D |  |  | NR FDD FR1 Band 8 |
| ... |  |  |  |  |  |
| 13 | NR Frequency band: 777-787 MHz, 746-756 MHz | 38.101-1, 5.2D |  |  | NR FDD FR1 Band 13 |
| ... |  |  |  |  |  |
| 24 | NR Frequency band: 1626.5-1660.5 MHz, 1525-1559 MHz | 38.101-1, 5.2D |  |  | NR FDD FR1 Band 24 |
| 25 | NR Frequency band: 1850-1915 MHz, 1930-1995 MHz | 38.101-1, 5.2D |  |  | NR FDD FR1 Band 25 |
| 26 | NR Frequency band: 814-849 MHz, 859-894 MHz | 38.101-1, 5.2D |  |  | NR FDD FR1 Band 26 |
| ... |  |  |  |  |  |
| 30 | NR Frequency band: 2305-2315 MHz, 2350-2360 MHz | 38.101-1, 5.2D |  |  | NR FDD FR1 Band 30, (see NOTE 1) |
| ... |  |  |  |  |  |
| 34 | NR Frequency band: 2010-2025 MHz | 38.101-1, 5.2D |  |  | NR TDD FR1 Band 34 |
| ... |  |  |  |  |  |
| 38 | NR Frequency band: 2570-2620 MHz | 38.101-1, 5.2D |  |  | NR TDD FR1 Band 38 |
| 39 | NR Frequency band: 1880-1920 MHz | 38.101-1, 5.2D |  |  | NR TDD FR1 Band 39 |
| 40 | NR Frequency band: 2300-2400 MHz | 38.101-1, 5.2D |  |  | NR TDD FR1 Band 40 |
| 41 | NR Frequency band: 2496-2690 MHz | 38.101-1, 5.2D |  |  | NR TDD FR1 Band 41 |
| ... |  |  |  |  |  |
| 46 | NR Frequency band: 5150-5925 MHz | 38.101-1, 5.2D |  |  | NR TDD FR1 Band 46 |
| ... |  |  |  |  |  |
| 48 | NR Frequency band: 3550-3700 MHz | 38.101-1, 5.2D |  |  | NR TDD FR1 Band 48 |
| ... |  |  |  |  |  |
| 66 | NR Frequency band: 1710-1780 MHz, 2110-2200 MHz | 38.101-1, 5.2D |  |  | NR FDD FR1 Band 66 |
| ... |  |  |  |  |  |
| 70 | NR Frequency band: 1695-1710 MHz, 1995-2020 MHz | 38.101-1, 5.2D |  |  | NR FDD FR1 Band 70 |
| 71 | NR Frequency band: 663-698 MHz, 617-652 MHz | 38.101-1, 5.2D |  |  | NR FDD FR1 Band 71, (see NOTE 2) |
| ... |  |  |  |  |  |
| 77 | NR Frequency band: 3300–4200 MHz | 38.101-1, 5.2D |  |  | NR TDD FR1 Band 77 |
| 78 | NR Frequency band: 3300–3800 MHz | 38.101-1, 5.2D |  |  | NR TDD FR1 Band 78 |
| 79 | NR Frequency band: 4400–5000 MHz | 38.101-1, 5.2D |  |  | NR TDD FR1 Band 79 |
| 80 | NR Frequency band: 1710–1785 MHz | 38.101-1, 5.2D |  |  | NR SUL FR1 Band 80 |
| 81 | NR Frequency band: 880 MHz – 915 MHz MHz | 38.101-1, 5.2D |  |  | NR SUL FR1 Band 81 |
| 84 | NR Frequency band: 1920–1980 MHz | 38.101-1, 5.2D |  |  | NR SUL FR1 Band 84 |
| 85 | NR Frequency band: 698-716 MHz, 728-746 MHz | 38.101-1, 5.2D |  |  | NR FDD FR1 Band 85 |
| ... |  |  |  |  |  |
| 95 | NR Frequency band: 2010–2025 MHz | 38.101-1, 5.2D |  |  | NR SUL FR1 Band 95 |
| 97 | NR Frequency band: 2300–2400 MHz | 38.101-1, 5.2D |  |  | NR SUL FR1 Band 97 |
| 98 | NR Frequency band: 1880–1920 MHz | 38.101-1, 5.2D |  |  | NR SUL FR1 Band 98 |
| 99 | NR Frequency band: 1626.5–1660.5 MHz | 38.101-1, 5.2D |  |  | NR SUL FR1 Band 99 |
| 100 | NR Frequency band: 874.4 – 880 MHz, 919.4 – 925 MHz | 38.101-1, 5.2D |  |  | NR FDD FR1 Band 100 |
| ... |  |  |  |  |  |
| 104 | NR Frequency band: 6425–7125 MHz | 38.101-1, 5.2D |  |  | NR TDD FR1 Band 104 |
| 105 | NR Frequency band: 663-703 MHz, 612-652 MHz | 38.101-1, 5.2D |  |  | NR FDD FR1 Band 105 |
| NOTE 1: Uplink transmission is not allowed at this band for UE with external vehicle-mounted antennas. NOTE 2: UL MIMO is targeted for FWA form factor. NOTE 3: The UE supplier shall indicate the maximum supported UL-MIMO capability is 2Tx (two transmit antenna connectors), or 4Tx (four transmit antenna connectors). NOTE 4: See 2Tx(table_index) in Note 2 of Table 4.0-2 and 4Tx(table_index) in Note 3 of Table 4.0-2 in TS 38.522 [9]. |  |  |  |  |  |

==============================================================

<< End of changes >>