3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250409

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4932 | rev | 1 | Current version: | 18.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to test case 13.3.1.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI8_Test |  |  |  |  |  | Date: |  |  | 2025-08-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | As per TS 36.523-1 Table 13.3.1.1.3.2-1 the Power level before step-1 for shall be acording to T0 which should be according to Power level from 36.508 clause 6.2.2.1. Pdefault as serving cell which is -85. In the current TTCN implimentation it is being set as -97 which is incorrect, also the refrence of the power level table is not correct. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correcting the power level and the reference |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 13.3.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required.	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2024-06_D25wk24 related to the title of this CR.

Contact:	Mohit Kanchan
	mohit.kanchan@keysight.com

# Corrections required.

## Change 1

| Function name | function f_TC_13_3_1_1_EUTRA() |
| --- | --- |
| Reason for change | As per TS 36.523-1 Table 13.3.1.1.3.2-1 the Power level before step-1 for shall be acording to T0 which should be according to Power level from 36.508 clause 6.2.2.1. Pdefault as serving cell which is -85. In the current TTCN implimentation it is being set as -97 which is incorrect, also the refrence of the power level table is not correct. |
| Summary of change | Correcting the power level and the reference |
| TTCN module | MultiLayer_Procedures.ttcn |
| MCC160 Comment |  |

Before Change:

function f_TC_13_3_1_1_EUTRA() runs on EUTRA_PTC { /* Intra System Connection Reestablishment / Radio Link Recovery while T310 is running */ var SubFrameTiming_Type v_TimingInfo; var PDCP_SDUList_Type v_PDCP_SDUList_DRB1 := { f_IPv4IPv6_IcmpEchoReply(f_LoopbackModeB_IP_Address_UE()) }; /* @sic R5s120470 sic@ */ //Init cell array and security f_EUTRA_Init(c1); //Set Sys info for cell 1 to be used at creation f_EUTRA_CellInfo_SetSysInfo_T310(eutra_Cell1, ms2000); //Create and configure cell f_EUTRA_CellConfig_Def(eutra_Cell1); //The UE is in state Loopback Activated (state 4) on Cell 1 according to [18] using the UE TEST LOOP MODE B. f_EUTRA_Preamble(eutra_Cell1, STATE2A_TESTLOOP_ModeB); f_EUTRA_GenericRbEst_TAInfinity(eutra_Cell1); //@sic R5s100720 sic@ f_EUTRA_CloseUE_TestLoopModeB(eutra_Cell1, '05'O); //* @desc Set cell power levels according to T0 in Table 13.4.1.2.3.2-1 f_EUTRA_SetCellPower(eutra_Cell1, -97); f_EUTRA_TestBody_Set(true, layer2); //SS wait for the 660ms to ensure that DL Timing Advance is sent at least once and UE apply the //value of timeAlignmentTimerDedicated which is updated during the preamble. v_TimingInfo := f_EUTRA_GetNextSendOccasion(eutra_Cell1, 660); //@siclog "Step 1" siclog@ /* The SS transmits one IP packet to the UE on the DRB associated with the default EPS bearer context on Cell 1. */ DRB.send(cas_DRB_COMMON_REQ_PDCP_SDU(eutra_Cell1, tsc_RbId_DRB1, cs_TimingInfo_EUTRA_NB(v_TimingInfo), v_PDCP_SDUList_DRB1)); /* @sic R5s120470 sic@ */ //@siclog "Step 2" siclog@ /* 100 ms after step 1 the SS reduce DL RS TX power level of Cell 1 from "T0" to "T1" in table 13.3.1.1.3.2-1. Note: 100 ms delay after step 1 is to ensure that UE has time to transmit STATUS PDU for data received in step 1 */ v_TimingInfo := f_EUTRA_TimingInfoAdd(v_TimingInfo, 100); //@sic R5s110051 sic@ f_EUTRA_SetCellPower(eutra_Cell1, tsc_NonSuitableOffCellRS_EPRE, cs_TimingInfo_EUTRA_NB(v_TimingInfo)); //@sic R5s110051 sic@ //@siclog "Step 3" siclog@ /* The SS waits for 1.5s. T310 is 2s. */ v_TimingInfo := f_EUTRA_TimingInfoAdd(v_TimingInfo, 1500); //@siclog "Step 4" siclog@ /* The SS recovers DL RS TX power level of Cell 1 from "T1" to "T2" in table 13.3.1.1.3.2-1. */ f_EUTRA_SetCellPower(eutra_Cell1, tsc_ServingCellRS_EPRE, cs_TimingInfo_EUTRA_NB(v_TimingInfo)); **************************************************************************************************************

After Change:

function f_TC_13_3_1_1_EUTRA() runs on EUTRA_PTC { /* Intra System Connection Reestablishment / Radio Link Recovery while T310 is running */ var SubFrameTiming_Type v_TimingInfo; var PDCP_SDUList_Type v_PDCP_SDUList_DRB1 := { f_IPv4IPv6_IcmpEchoReply(f_LoopbackModeB_IP_Address_UE()) }; /* @sic R5s120470 sic@ */ //Init cell array and security f_EUTRA_Init(c1); //Set Sys info for cell 1 to be used at creation f_EUTRA_CellInfo_SetSysInfo_T310(eutra_Cell1, ms2000); //Create and configure cell f_EUTRA_CellConfig_Def(eutra_Cell1); //The UE is in state Loopback Activated (state 4) on Cell 1 according to [18] using the UE TEST LOOP MODE B. f_EUTRA_Preamble(eutra_Cell1, STATE2A_TESTLOOP_ModeB); f_EUTRA_GenericRbEst_TAInfinity(eutra_Cell1); //@sic R5s100720 sic@ f_EUTRA_CloseUE_TestLoopModeB(eutra_Cell1, '05'O); //* @desc Set cell power levels according to T0 in Table 13.3.1.1.3.2-1 f_EUTRA_SetCellPower(eutra_Cell1, -85); f_EUTRA_TestBody_Set(true, layer2); //SS wait for the 660ms to ensure that DL Timing Advance is sent at least once and UE apply the //value of timeAlignmentTimerDedicated which is updated during the preamble. v_TimingInfo := f_EUTRA_GetNextSendOccasion(eutra_Cell1, 660); //@siclog "Step 1" siclog@ /* The SS transmits one IP packet to the UE on the DRB associated with the default EPS bearer context on Cell 1. */ DRB.send(cas_DRB_COMMON_REQ_PDCP_SDU(eutra_Cell1, tsc_RbId_DRB1, cs_TimingInfo_EUTRA_NB(v_TimingInfo), v_PDCP_SDUList_DRB1)); /* @sic R5s120470 sic@ */ //@siclog "Step 2" siclog@ /* 100 ms after step 1 the SS reduce DL RS TX power level of Cell 1 from "T0" to "T1" in table 13.3.1.1.3.2-1. Note: 100 ms delay after step 1 is to ensure that UE has time to transmit STATUS PDU for data received in step 1 */ v_TimingInfo := f_EUTRA_TimingInfoAdd(v_TimingInfo, 100); //@sic R5s110051 sic@ f_EUTRA_SetCellPower(eutra_Cell1, tsc_NonSuitableOffCellRS_EPRE, cs_TimingInfo_EUTRA_NB(v_TimingInfo)); //@sic R5s110051 sic@ //@siclog "Step 3" siclog@ /* The SS waits for 1.5s. T310 is 2s. */ v_TimingInfo := f_EUTRA_TimingInfoAdd(v_TimingInfo, 1500); //@siclog "Step 4" siclog@ /* The SS recovers DL RS TX power level of Cell 1 from "T1" to "T2" in table 13.3.1.1.3.2-1. */ f_EUTRA_SetCellPower(eutra_Cell1, tsc_ServingCellRS_EPRE, cs_TimingInfo_EUTRA_NB(v_TimingInfo)); **************************************************************************************************************