3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250477

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4947 | rev | - | Current version: | 18.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to LTE RRC test case 8.6.4.4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI10_Test |  |  |  |  |  | Date: |  |  | 2025-11-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | At Step5, the timer t_T304_Max is not started immediately after transmitting RRCConnectionReconfiguration message on Cell 1, this may cause TTCN to not handle the RRCConnectionReestablishmentRequest message received at Step7, even though UE transmitted after timer T304 expiry on UE side. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is proposed to start the timer t_T304_Max as soon as the RRCConnectionReconfiguration message on Cell 1 is transmitted.. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conformant UE may fail the testcase. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.6.4.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

- Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2024-06_D25wk38 related to the title of this CR.

Contact:	Narendra Kalahasti
                         Narendra.kalahasti@anritsu.com

- Corrections required

  - Change 1

| Function name | f_TC_8_6_4_4_EUTRA |
| --- | --- |
| Reason for change | At Step5, the timer t_T304_Max is not started immediately after transmitting RRCConnectionReconfiguration message on Cell 1, this may cause TTCN to not handle the RRCConnectionReestablishmentRequest message received at Step7, even though UE transmitted after timer T304 expiry on UE side. |
| Summary of change | It is proposed to start the timer t_T304_Max as soon as the RRCConnectionReconfiguration message on Cell 1 is transmitted. |
| TTCN module | LTE_A_R10_R11\8_6\RRC_LoggedRLF.ttcn |

Before Change:

function f_TC_8_6_4_4_EUTRA() runs on EUTRA_PTC { // Radio Link Failure logging / Reporting at E-UTRA handover // SKIPPED CODE //@siclog "Step 5" siclog@ //The SS transmits an RRCConnectionReconfiguration message on Cell 1 to order the UE to perform inter frequency handover to Cell 12. v_MobilityControlInfo := f_Generate_cs_MobilityControlInfo_HO(eutra_Cell12, //@sic R5s140147 sic@ cs_CarrierFreqEUTRA_v9e0 (v_Frequency_IE_Cell12.UL_DL_Earfcn.dl_CarrierFreq_r9, omit), //@sic R5-171144, R5s140147 sic@ -, -, -, -, -, v_RACH_ConfigDedicated); SRB.send(cas_SRB1_RrcPdu_REQ(eutra_Cell1, cs_TimingInfo_Now, cs_RRCConnectionReconfiguration_Common(v_RRC_TI, omit, v_MobilityControlInfo, f_Generate_cs_508_RadioResourceConfigDedicated_HO (eutra_Cell12, n4), //@sic R5s140147 sic@ cs_508SecurityConfigHO_IntraLTE))); //@sic R5s140147 sic@ f_EUTRA_SS_CommonCellConfig(eutra_Cell1, cas_PUCCH_Synch_Config_REQ(eutra_Cell1, cs_TimingInfo_Now, cs_UplinkTimeAlignment_Synch_None)); //EXCEPTION: In parallel to the events described in step 6 the steps specified in Table 8.6.4.4.3.2-3 should take place. //EXCEPTION: The steps 1 and 2 below are repeated for the duration of T304. //1. The UE attempts to perform the intra frequency handover using MAC Random Access Preamble on Cell 2. //2. The SS does not respond. t_T304Max.start(v_T304); f_EUTRA_SS_SRBs_DRBs_Reset(eutra_Cell1, -, 100); //@sic R5s150402, R5s140216, R5s140230 sic@ v_ShortMAC_I := f_EUTRA_Calculate_ShortMAC(eutra_Cell1, eutra_Cell1); //@sic R5s140147 sic@ //@siclog "Step 6" siclog@ //The SS changes Cell 12 power level according to the row "T2" in Table 8.6.4.4.3.2-1. f_EUTRA_SetCellPowerList(v_CellPowerList_AtT2); t_T304Max.timeout; //Configure Cell12 to normal PRACH operation mode // f_EUTRA_SS_ConfigRachProcedure_Def(eutra_Cell12, tsc_C_RNTI_Def); //@sic R5s140147 sic@ //@siclog "Step 7" siclog@ //The UE transmits an RRCConnectionReestablishmentRequest message on Cell 1 SRB.receive(car_SRB0_RrcPdu_IND(eutra_Cell1, cr_508_RRCConnectionReestablishmentRequest(tsc_C_RNTI_Def, v_PhysCellId_Cell1, cr_ReestablishmentCause_HandoverFailure, v_ShortMAC_I))); //@sic R5s140147 sic@ // SKIPPED CODE } // end of f_TC_8_6_4_4_EUTRA

After Change:

function f_TC_8_6_4_4_EUTRA() runs on EUTRA_PTC { // Radio Link Failure logging / Reporting at E-UTRA handover // SKIPPED CODE //@siclog "Step 5" siclog@ //The SS transmits an RRCConnectionReconfiguration message on Cell 1 to order the UE to perform inter frequency handover to Cell 12. v_MobilityControlInfo := f_Generate_cs_MobilityControlInfo_HO(eutra_Cell12, //@sic R5s140147 sic@ cs_CarrierFreqEUTRA_v9e0 (v_Frequency_IE_Cell12.UL_DL_Earfcn.dl_CarrierFreq_r9, omit), //@sic R5-171144, R5s140147 sic@ -, -, -, -, -, v_RACH_ConfigDedicated); SRB.send(cas_SRB1_RrcPdu_REQ(eutra_Cell1, cs_TimingInfo_Now, cs_RRCConnectionReconfiguration_Common(v_RRC_TI, omit, v_MobilityControlInfo, f_Generate_cs_508_RadioResourceConfigDedicated_HO (eutra_Cell12, n4), //@sic R5s140147 sic@ cs_508SecurityConfigHO_IntraLTE))); //@sic R5s140147 sic@ t_T304Max.start(v_T304); f_EUTRA_SS_CommonCellConfig(eutra_Cell1, cas_PUCCH_Synch_Config_REQ(eutra_Cell1, cs_TimingInfo_Now, cs_UplinkTimeAlignment_Synch_None)); //EXCEPTION: In parallel to the events described in step 6 the steps specified in Table 8.6.4.4.3.2-3 should take place. //EXCEPTION: The steps 1 and 2 below are repeated for the duration of T304. //1. The UE attempts to perform the intra frequency handover using MAC Random Access Preamble on Cell 2. //2. The SS does not respond. f_EUTRA_SS_SRBs_DRBs_Reset(eutra_Cell1, -, 100); //@sic R5s150402, R5s140216, R5s140230 sic@ v_ShortMAC_I := f_EUTRA_Calculate_ShortMAC(eutra_Cell1, eutra_Cell1); //@sic R5s140147 sic@ //@siclog "Step 6" siclog@ //The SS changes Cell 12 power level according to the row "T2" in Table 8.6.4.4.3.2-1. f_EUTRA_SetCellPowerList(v_CellPowerList_AtT2); t_T304Max.timeout; //Configure Cell12 to normal PRACH operation mode // f_EUTRA_SS_ConfigRachProcedure_Def(eutra_Cell12, tsc_C_RNTI_Def); //@sic R5s140147 sic@ //@siclog "Step 7" siclog@ //The UE transmits an RRCConnectionReestablishmentRequest message on Cell 1 SRB.receive(car_SRB0_RrcPdu_IND(eutra_Cell1, cr_508_RRCConnectionReestablishmentRequest(tsc_C_RNTI_Def, v_PhysCellId_Cell1, cr_ReestablishmentCause_HandoverFailure, v_ShortMAC_I))); //@sic R5s140147 sic@ // SKIPPED CODE } // end of f_TC_8_6_4_4_EUTRA