3GPP TSG-RAN WG4 Meeting #117	R4-2521819

Dallas, USA, Nov. 17 – 21, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 6210 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft big CR on core maintenance for Netw_Energy_NR_enh_RRM |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | Netw_Energy_NR_enh-Core |  |  |  |  |  | Date: |  |  | 2025-10-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Draft Big CR on core maintenance for Netw_Energy_NR_enh_RRM. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Editorial changes The following endorsed CRs are consolidated in this big CR. R4-2514831 Draft CR to TS 38.133 on R19 NES core requirements R4-2514832 Corrections on NES requirements R4-2514833 DraftCR on core requirements maintenance for Rel-19 NES R4-2514834 draftCR for interruptions due to OD-SSB SCell operation R4-2514835 draft CR for network energy savings enhancements R4-2514836 Draft CR on OD-SSB of R19 NES R4-2514491 Draft CR38.133 Miscellaneous corrections to on-demand SIB1 naming R4-2514844 Draft CR for OD-SSB based cell identification requirement for deactivated SCell |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | RRM requirements for Enhancements of network energy savings would be incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.3, 4.2.2.6, 7.1.2, 8.2.1.2.21,8.2.1.2.3, 8.2.1.2.4, 8.2.2.2.22, 8.2.2.2.1, 8.2.2.2.2, 8.2.2.2.3 8.3.19, 8.3.20, 8.5.5.2, 8.5.12, 9.5.4.1, 9.8.4.2, 9.8.7, 9.17.1, 9.17.5.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS38.533 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

-------------------------- CHANGE #1 ------------------------------

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [11] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [11].

AoA	Angle of Arrival

AoD	Angle of Departure

ATG	Air to Ground

AWGN	Additive White Gaussian Noise

BFD	Beam Failure Detection

BFD-RS	BFD Reference Signal

BLER	Block Error Rate

BM-RS	Beam Management Reference Signal

BW	Bandwidth

BWP	Bandwidth Part

CA	Carrier Aggregation

CBD	Candidate Beam Detection

CBW	Channel Bandwidth

CC	Component Carrier

CCA	Clear Channel Assessment

CCE	Control Channel Element

CG-SDT	Configured Grant Small Data Transmission

CHO	Conditional Handover

CLI	Cross Link Interference

CLTM	Conditional L1/L2 triggered mobility

CMR	Channel Measurement Resource

CN	Core Network

CORESET	Control Resource Set

CP	Cyclic Prefix

CPC	Conditional PSCell Change

CSI	Channel-State Information

CSI-RS	CSI Reference Signal

CSI-RSRP	CSI Reference Signal based Reference Signal Received Power

CSI-RSRQ	CSI Reference Signal based Reference Signal Received Quality

CSI-SINR	CSI Reference Signal based Signal to Noise and Interference Ratio

CSI_RP	Received (linear) average power of the resource elements that carry NR CSI-RS signals and channels, measured at the UE antenna connector

DAPS	Dual Active Protocol Stack

DBT	Discovery Burst Transmission

DC	Dual Connectivity

DCI	Downlink Control Information

DL	Downlink

DL-AoD	Downlink Angle-of-Departure

DL-TDOA	Downlink Time Difference Of Arrival

DMRS	Demodulation Reference Signal

DPC	Delta Power Class

DRX	Discontinuous Reception

E-CID	Enhanced Cell ID

eDRX	Extended DRX

E-UTRA	Evolved UTRA

E-UTRAN	Evolved UTRAN

EMP	Effective measurement periodicity

EMR	Early measurement reporting

EMW	Effective measurement window

EMWRP	Effective measurement window repetition period

EN-DC	E-UTRA-NR Dual Connectivity

FDD	Frequency Division Duplex

FH	Frequency Hopping

FR	Frequency Range

FMW	Fast measurement window

GEO	Geostationary Earth Orbit

GNSS	Global Navigation Satellite System

GSO	Geosynchronous Orbit

HARQ	Hybrid Automatic Repeat Request

HO	Handover

HST	High Speed Train

GAP	Refers to any of Measurement Gap, activated Pre-MG and NCSG

IMR	Interference Measurement Resource

kHz	Kilo Hertz

L1-RSRP	Layer 1 RSRP

L1 SL-RSRP	Layer 1 Sidelink RSRP which corresponds to PSCCH-RSRP and/or PSSCH-RSRP

LEO	Low Earth Orbit

LMF	Location Management Function

LO 	LP-WUS Occasion

LPP	LTE Positioning Protocol

LP-SS 	Low Power-Synchronization Signal

LP-WUR 	Low Power-Wake up Receiver

LP-WUS 	Low Power-Wake up Signal

LR 	Low Power-Wake up Receiver

LTM	L1/L2 triggered mobility

MAC	Medium Access Control

MCG	Master Cell Group

MDT	Minimization of Drive Tests

MG	Measurement Gap

MGL	Measurement Gap Length

MGRP	Measurement Gap Repetition Period

MHz	Mega Hertz

MIB	Master Information Block

ML	Measurement Length

MN	Master Node

MR-DC	Multi-Radio Dual Connectivity

MR 	Main Receiver

MUSIM	Multi-Universal Subscriber Identity Module

NCSG	Network Controlled Small Gap

NE-DC	NR-E-UTRA Dual Connectivity

NGEN-DC	NG-RAN E-UTRA-NR Dual Connectivity

NGSO	Non-Geosynchronous Orbit

NR	New Radio

NR-DC	NR-NR Dual Connectivity

NTN	Non-Terrestrial Network

OCNG	OFDMA Channel Noise Generator

OD-SIB1	On-demand SIB1

OD-SSB	On-demand SSB

OFDM	Orthogonal Frequency Division Multiplexing

OFDMA	Orthogonal Frequency Division Multiple Access

OOK 	On-Off Keying

OTDOA	Observed Time Difference Of Arrival

PBCH	Physical Broadcast Channel

PCC	Primary Component Carrier

PCell	Primary Cell

PCI	Physical Cell Identity

PDCCH	Physical Downlink Control Channel

PDSCH	Physical Downlink Shared Channel

PLMN	Public Land Mobile Network

PRACH	Physical RACH

Pre-MG	Pre-configured Measurement Gap

ProSe	Proximity-based Service

PRB	Physical Resource Block

PRP	PRS Received Power

PRS	Positioning Reference Signal

PRS-RSRP	Positioning Reference Signal based Reference Signal Received Power

PPW	PRS Processing Window

PPWL	PRS Processing Window Length

PPWRP	PRS Processing Window Repetition Period

PSBCH	Physical Sidelink Broadcast Channel

PSBCH-RSRP	Physical Sidelink Broadcast Channel DMRS based Reference Signal Received Power

PSCCH	Physical Sidelink Control Channel

PSCCH-RSRP	Physical Sidelink Control Channel DMRS based Reference Signal Received Power

PSCell	Primary SCell

PSS	Primary Synchronization Signal

PSSCH	Physical Sidelink Shared Channel

PSSCH-RSRP	Physical Sidelink Shared Channel DMRS based Reference Signal Received Power

pTAG	Primary Timing Advance Group

PTW	Paging Time Window

PUCCH	Physical Uplink Control Channel

PUSCH	Physical Uplink Shared Channel

QCL	Quasi Co-Location

RACH	Random Access Channel

RAN	Radio Access Network

RAT	Radio Access Technology

RF	Radio Frequency

RLM	Radio Link Monitoring

RLM-RS	Reference Signal for RLM

RMC	Reference Measurement Channel

RMSI	Remaining Minimum System Information

RRC	Radio Resource Control

RRH	Remote Radio Head

RRM	Radio Resource Management

RRT	RF Retuning Time

RS	Reference Signal

RSCP	Reference Signal Carrier Phase

RSCPD	Reference Signal Carrier Phase Difference

RSSI	Received Signal Strength Indicator

RSRP	Reference Signal Received Power

RSRPP	Reference Signal Received Path Power

RSRQ	Reference Signal Received Quality

RSTD	Reference Signal Time Difference

RTD	Receive Timing Difference

RTOA	Relative Time Of Arrival

RTT	Round Trip Time

S-SSB	Sidelink Synchronization Signal Block

SSB	Synchronization Signal Block

SSB_RP	Received (linear) average power of the resource elements that carry NR SSB signals and channels, measured at the UE antenna connector or radiated interface boundary.

SA	Standalone operation mode

SAB	Satellite access band

SAN	Satellite Access Node

SBFD	Subband non-overlapping Full Duplex

SCC	Secondary Component Carrier

SCCH	Sidelink Control Channel

SCell	Secondary Cell

SCG	Secondary Cell Group

SCH	Synchronization Channel

SCS	Subcarrier Spacing

SCSSSB	SSB subcarrier spacing

SDL	Supplementary Downlink

SDT	Small Data Transmission

SFN	System Frame Number

SFTD	SFN and Frame Timing Difference

SI	System Information

SIB	System Information Block

SL	Sidelink

SL AoA	Sidelink AoA

SL PRS-RSRP	Sidelink PRS-based RSRP

SL PRS-RSRPP	Sidelink PRS-based RSRPP

SL RSTD	Sidelink RSTD

SL RTOA	Sidelink RTOA

SL Rx-Tx	Sidelink Receive-Transmit time difference

SL-PRP	SL-PRS Received Power

SL-PRS	Sidelink PRS

SL-RSSI	Sidelink Received Signal Strength Indicator

SLPP	Sidelink Positioning Protocol

SLSS	Sidelink Synchronization Signal

SMTC	SSB-based Measurement Timing configuration

SpCell	Special Cell

SRS	Sounding Reference Signal

SRS-RSRP	Sounding Reference Signal based Reference Signal Received Power

SS-RSRP	Synchronization Signal based Reference Signal Received Power

SS-RSRQ	Synchronization Signal based Reference Signal Received Quality

SS-SINR	Synchronization Signal based Signal to Noise and Interference Ratio

SSB	Synchronization Signal Block

SSB_RP	Received (linear) average power of the resource elements that carry NR SSB signals and channels, measured at the UE antenna connector.

SSS	Secondary Synchronization Signal

sTAG	Secondary Timing Advance Group

SUL	Supplementary Uplink

TA	Timing Advance

TAG	Timing Advance Group

TCI	Transmission Configuration Indicator

TDCP	Time Domain Channel Properties

TDD	Time Division Duplex

TDOA	Time Difference Of Arrival

TE	Test Equipment

TN	Terrestrial Network

TRP	Transmission-Reception Point

TRS	Tracking Reference Signal

TTI	Transmission Time Interval

U2N	UE-to-Network

U2U	UE-to-UE

UE	User Equipment

UL	Uplink

UL-WUS	Uplink Wake-up Signal

V2X	Vehicle-to-Everything service

VIL	Visible Interruption Length

VIRP	Visible Interruption Repetition Period

VSAT	Very Small Aperture Terminal

-------------------- END OF CHANGES--------------------------

-------------------------- CHANGE #2 ------------------------------

#### 4.2.2.6	Maximum interruption in paging reception

UE shall perform the cell re-selection with minimum interruption in monitoring downlink channels for paging reception. In addition, when the UE is configured with eDRX_IDLE cycle, the UE shall not miss any paging in a PTW provided the paging is sent in at least 2 DRX cycles before the end of that PTW.

At intra-frequency and inter-frequency cell re-selection, the UE shall monitor the downlink of serving cell for paging reception until the UE is capable to start monitoring downlink channels of the target intra-frequency and inter-frequency cell for paging reception. The interruption time shall not exceed TSI-NR + 2*Ttarget_cell_SMTC_period ms. Ttarget_cell_SMTC_period is the periodicity of the SMTC occasions configured for the target NR cell. If the target cell is in the PCI list of smtc2-LP, the SMTC periodicity follows smtc2-LP; otherwise, the SMTC periodicity follows smtc. The interruption time could be longer due to on demand SIB1 acquisition if the target NR cell is an NES cell and UE is capable of on-demand SIB1 acquisition.

At inter-RAT cell re-selection, the UE shall monitor the downlink of serving cell for paging reception until the UE is capable to start monitoring downlink channels for paging reception of the target inter-RAT cell. For NR to E-UTRAN cell re-selection the interruption time must not exceed TSI-EUTRA + 55 ms.

TSI-NR is the time required for receiving all the relevant system information data according to the reception procedure and the RRC procedure delay of system information blocks defined in TS 38.331 [2] for an NR cell. If the target NR cell is an NES cell and UE is capable of on-demand SIB1 acquisition, TSI-NR could be longer due to UL WUS transmission and RAR reception for on-demand SIB1 acquisition of NES cell.

TSI-EUTRA is the time required for receiving all the relevant system information data according to the reception procedure and the RRC procedure delay of system information blocks defined in TS 36.331 [16] for an E-UTRAN cell. These requirements assume sufficient radio conditions, so that decoding of system information can be made without errors and does not take into account cell re-selection failure.

-------------------- END OF CHANGES--------------------------

-------------------------- CHANGE #3 ------------------------------

### 7.1.2	Requirements

The UE initial transmission timing error shall be less than or equal to Te where the timing error limit value Te is specified in table 7.1.2-1. This requirement applies:

-	when it is the first transmission in a DRX cycle for PUCCH, PUSCH and SRS, or it is the PRACH transmission, or it is the msgA transmission, or it is the first transmission sent on the PSCell for activating the deactivated SCG without RACH, or it is the UL-WUS transmission for OD-SIB1.

-	when it is the transmission for PUSCH on CG resources for SDT in RRC_INACTIVE.

-	when it is the first transmission on target cell after UE receives LTM cell switch command.

-	when it is the first transmission among SCell(s) in an sTAG after a dormant BWP switching delay as defined in 8.6.2 or 8.6.2A, if the dormant BWP switch includes BWP switch of an SCell configured with UL in the sTAG.

When the UL SCS is 120 kHz or smaller, the UE shall meet the Te requirement for an initial transmission provided that at least one SSB is available at the UE during the last 160 ms. When the UL SCS is 480 kHz the UE shall meet the Te requirement for an initial transmission provided that at least one SSB is available in the last 80 ms. When the UL SCS is 960 kHz the UE shall meet the Te requirement for an initial transmission provided that at least one SSB is available in the last 40 ms.

The reference point for the UE initial transmit timing control requirement shall be the downlink timing of the reference cell minus NTA+NTA offset×Tc. The downlink timing is defined as the time when the first path (in time) of the corresponding downlink frame used by the UE to determine downlink timing is received from the reference cell at the UE antenna. NTA for PRACH and UL-WUS for OD-SIB1 is defined as 0.

NTA+NTA offset×Tc (in Tc units) for other channels is the difference between UE transmission timing and the downlink timing immediately after when the last timing advance in clause 7.3 was applied. NTA for other channels is not changed until next timing advance is received. The value of NTA offset depends on the duplex mode of the cell in which the uplink transmission takes place and the frequency range (FR). NTA offset is defined in table 7.1.2-2.

For multi-DCI based multi-TRP operation with two TAs and for asymmetric DL sTRP/UL mTRP deployment with two TAs for first transmission in a DRX cycle for PUCCH, PUSCH, SRS and PRACH transmission triggered by PDCCH order, UE initial transmission timing error requirements specified in this clause is applicable for each TAG and shall be met for each TAG separately. The reference point for each TAG for the PUCCH/PUSCH/SRS and PRACH transmission triggered by PDCCH order is defined in clause 7.1.1.

Table 7.1.2-1: Te Timing Error Limit

| Frequency Range | SCS of SSB signals (kHz) | SCS of uplink signals (kHz) | Te |
| --- | --- | --- | --- |
| 1 | 15 | 15 | 12*64*Tc |
|  |  | 30 | 10*64*Tc |
|  |  | 60 | 10*64*Tc |
|  | 30 | 15 | 8*64*Tc |
|  |  | 30 | 8*64*Tc |
|  |  | 60 | 7*64*Tc |
| 2-1 | 120 | 60 | 3.5*64*Tc |
|  |  | 120 | 3.5*64*Tc |
|  | 240 | 60 | 3*64*Tc |
|  |  | 120 | 3*64*Tc |
| 2-2 | 120 | 120 | 3.5*64*Tc |
|  |  | 480 | 1.58*64*Tc |
|  | 480 | 120 | 2.86*64*Tc |
|  |  | 480 | 1.35*64*Tc |
|  |  | 960 | 0.90*64*Tc |
|  | 960 | 120 | 2.80*64*Tc |
|  |  | 480 | 1.13*64*Tc |
|  |  | 960 | 0.86*64*Tc |
| NOTE 1: Tc is the basic timing unit defined in TS 38.211 [6] |  |  |  |  |

Table 7.1.2-2: The Value of NTA offset

| Frequency range and band of cell used for uplink transmission | NTA offset (Unit: TC) |
| --- | --- |
| FR1 FDD or TDD band with neither E-UTRA–NR nor NB-IoT–NR coexistence case | 25600 (Note 1) |
| FR1 FDD band with E-UTRA–NR and/or NB-IoT–NR coexistence case | 0 (Note 1) |
| FR1 TDD band with E-UTRA–NR and/or NB-IoT–NR coexistence case | 39936 (Note 1) |
| FR2 | 13792 |
| Note 1: The UE identifies NTA offset based on the information n-TimingAdvanceOffset as specified in TS 38.331 [2]. For UL-WUS transmission for OD-SIB1, UE identifies NTA offset based on the information n-TimingAdvanceOffset from UL-WUS configuration as specified in TS 38.331 [2]. If UE is not provided with the information n-TimingAdvanceOffset, the default value of NTA offset is set as 25600 for FR1 band. In case of multiple UL carriers in the same TAG, UE expects that the same value of n-TimingAdvanceOffset is provided for all the UL carriers according to clause 4.2 in TS 38.213 [3] and the value 39936 of NTA offset can also be provided for an FDD serving cell. Note 2: Void |  |

When it is not the first transmission in a DRX cycle or there is no DRX cycle, and when it is the transmission for PUCCH, PUSCH and SRS transmission, the UE shall be capable of changing the transmission timing according to the received downlink frame of the reference cell except when the timing advance in clause 7.3 is applied.

Table 7.1.2-3: void

If the UE uses a reference cell on a carrier frequency subject to CCA for deriving the UE transmit timing, then the UE shall meet all the transmit timing requirements defined in clause 7.1.2 provided that the reference cell is available at the UE. If the reference cell is not available at the UE on a carrier frequency subject to CCA, then the UE is allowed to transmit in the uplink provided that the UE meets all the transmit timing requirements defined in clause 7.1.2; otherwise the UE shall not transmit any uplink signal.

If a reference cell on a carrier frequency belonging to the pTAG, which is subject to CCA, is not available at the UE then the UE is allowed to use any of available activated SCell(s) at the UE in pTAG as a new reference cell. If the SCell used as reference cell is deactivated, or becomes not available, the UE is allowed to use another active serving cell in pTAG as new reference cell.

If a reference cell on a carrier frequency belonging to the sTAG, which is subject to CCA is not available at the UE then the UE is allowed to use any of available activated SCell(s) at the UE in sTAG as a new reference cell.

-------------------- END OF CHANGES--------------------------

-------------------------- CHANGE #4 ------------------------------

##### 8.2.1.2.21	Interruptions at OD-SSB activation/deactivation

The requirements in this clause shall apply for the UE configured with PSCell and at least one deactivated OD-SSB SCell. If more than one deactivated SCells are configured with OD-SSB transmission, the requirements in this clause shall apply provided that OD-SSB is activated or deactivated on only one of the deactivated OD-SSB SCells.

When OD-SSB is activated or deactivated on one deactivated SCell in SCG by a MAC CE, the UE is allowed an interruption on any active serving cell in SCG, provided that the OD-SSB activation/deactivation command and SCell activation/deactivation command are not received at the same time:

-	of up to X2 slots, if the active serving cell and the SCell on which the OD-SSB is being activated or deactivated are in a FR1 band pair or in a FR1+FR2 band pair.

-	of up to X2 slots, if the active serving cells and the SCells on which the OD-SSB is being activated or deactivated are in a FR2 band pair and UE is capable of independent beam management on this FR2 band pair.

-	of up to X2 slots, if the active serving cell is non-contiguous to the SCell on which the OD-SSB is being activated or deactivated in the same FR1 band and UE is capable of intraBandNR-CA-non-collocated-r18 and nonCollocatedTypeNR-CA-r18 is not provided.

or

-	of up to Y2 slots, if the active serving cells are contiguous to the SCell on which the OD-SSB is being activated or deactivated in the same FR1 band, or if the active serving cells are non-contiguous to the SCell on which the OD-SSB is being activated or deactivated in the same FR1 band and UE is not capable of intraBandNR-CA-non-collocated-r18 or UE is capable of intraBandNR-CA-non-collocated-r18 and nonCollocatedTypeNR-CA-r18 is provided, provided the cell specific reference signals from the active serving cells and the SCells on which the OD-SSB is being activated or deactivated are available in the same slot.

-	of up to Y2 slots if the active serving cells are in the same FR2 band as the SCell on which the OD-SSB is being activated or deactivated, provided the cell specific reference signals from the active serving cells and the SCells on which the OD-SSB is being activated or deactivated are available in the same slot.

Where X2 and Y2 are specified in table 8.2.1.2.21-1.

Table 8.2.1.2.21-1: Interruption length X2 and Y2 at OD-SSB activation/deactivation

|  | NR Slot length (ms) of victim cell | Interruption length X2 (slots) |  | Interruption length Y2 (slots) |
| --- | --- | --- | --- | --- |
| 0 | 1 | 1 |  | 1 |
| 1 | 0.5 | 1 |  | 1 |
| 2 | 0.25 | Both aggressor cell and victim cell are on FR2 | 2 | 2 |
|  |  | Either aggressor cell or victim cell is on FR1 | 3 |  |
| 3 | 0.125 | Aggressor cell is on FR2 | 4 | 4 |
|  |  | Aggressor cell is on FR1 | 5 |  |

-------------------- END OF CHANGES--------------------------

-------------------------- CHANGE #4 ------------------------------

##### 8.2.1.2.3	Interruptions at SCell addition/release

The requirements in this clause shall apply for the UE configured with PSCell.

When one E-UTRA SCell in MCG is added or released, the UE is allowed an interruption on any active serving cell in SCG:

-	of up to X1 slots, if the active serving cell is not in the same band as any of the E-UTRA SCells being added or released, or

-	of up to X1 slots, if the active serving cell is in the band overlapping or partially overlapping with the E-UTRA SCell being added or released and UE not supporting requirementTypeIndication-r18 indicates it is capable of interBandMRDC-WithOverlapDL-Bands-r16 on this band pair or UE supporting requirementTypeIndication-r18 is capable of interBandMRDC-WithOverlapDL-Bands-r16 on this band pair and is not provided with nonCollocatedTypeMRDC-r18, or

-	of up to max{Y1 slots + TSMTC_duration, 5ms}, if the active serving cells are in the same band or in the band overlapping or partially overlapping with the E-UTRA SCells being added or released, and UE does not indicates it is capable of interBandMRDC-WithOverlapDL-Bands-r16 on this band pair or UE capable of requirementTypeIndication-r18 indicates that it is capable of interBandMRDC-WithOverlapDL-Bands-r16 on this band pair and provided with nonCollocatedTypeMRDC-r18, provided the cell specific reference signals from the active serving cells and the E-UTRA SCells being added or released are available in the same slot, where TSMTC_duration is the longest SMTC duration among all above active serving cells in SCG;

-	if the active serving cell is in the band overlapping or partially overlapping with the E-UTRA SCell being added or released,

1>	when UE is configured with maxMIMO-Layers with value equal to 4 for all corresponding serving cells, in case of TDD-TDD inter-band (NG) EN-DC with overlapping or partially overlapping bands

2> if UE supporting requirementTypeIndication-r18 is capable of interBandMRDC-WithOverlapDL-Bands-r19 on this band pair

3> if nonCollocatedTypeMRDC-v19xy is presented with “type 4”

4>	of up to X1 slots

3>else if nonCollocatedTypeMRDC-v19xy is presented with “type 1”

4> of up to max{Y1 slots + TSMTC_duration, 5ms}

3>else if nonCollocatedTypeMRDC-v19xy is not provided

4> of up to max{Y1 slots + TSMTC_duration, 5ms}

2> else if UE not supporting requirementTypeIndication-r18 indicates it is capable of interBandMRDC-WithOverlapDL-Bands-r19 on this band pair

3> of up to X1 slots

1>	when UE is configured with maxMIMO-Layers with value equal to 2 for all corresponding serving cells, in case of TDD-TDD inter-band (NG) EN-DC with overlapping or partially overlapping bands

2> if nonCollocatedTypeMRDC-r18 is not provided

3>	of up to X1 slots

2> else

3> of up to max{Y1 slots + TSMTC_duration, 5ms},

Where X1 and Y1 are specified in table 8.2.1.2.3-1.

When one SCell in SCG is added or released, the UE is allowed an interruption on any active serving cell in SCG:

-	of up to X1 slot, if the active serving cell and the SCell being added or released are in a FR1 band pair or in a FR1+FR2 band pair.

-	of up to X1 slots, if the active serving cell and the SCell being added or released are in a FR2 band pair and UE is capable of independent beam management on this FR2 band pair

-	of up to X1 slots, if the active serving cell is non-contiguous to the SCell being added or released in the same FR1 band and UE is capable of intraBandNR-CA-non-collocated-r18 and nonCollocatedTypeNR-CA-r18 is not provided.

or

-	of up to Y1 slots + TSMTC_duration, if the active serving cells are contiguous to any of the SCells being added or released in the same FR1 band, or if the active serving cells are non-contiguous to any of the SCells being added or released in the same FR1 band and UE is not capable of intraBandNR-CA-non-collocated-r18 or UE is capable of intraBandNR-CA-non-collocated-r18 and nonCollocatedTypeNR-CA-r18 is provided, provided the cell specific reference signals from the active serving cells and the SCells being added or released are available in the same slot,

-	if the active serving cell is non-contiguous to the SCell being added or released in the same FR1 band, and UE is capable of intraBandNR-CA-non-collocated-r19,

1>	when UE is configured with maxMIMO-Layers with value equal to 4 for all corresponding serving cells, in case of TDD-TDD intra-band NR-CA

2> if nonCollocatedTypeNR-CA-v19xy is presented with  “type 4”

3>	of up to X1 slots

2> else if nonCollocatedTypeNR-CA-v19xy is presented with “type 1”

3>	of up to Y1 slots + TSMTC_duration

2> else if nonCollocatedTypeNR-CA-v19xy is not provided

3> of up to Y1 slots + TSMTC_duration

1>	when UE is configured with maxMIMO-Layers with value equal to 2 for the configured FR1 intra-band non-contiguous CA for all corresponding serving cells, in case of TDD-TDD intra-band NR-CA

2> if nonCollocatedTypeNR-CA-r18 is not provided

3>	of up to X1 slots

2> else

3> of up to Y1 slots + TSMTC_duration

where, TSMTC_duration is

-	the longest SMTC duration among all above active serving cells in SCG and the SCell being added when one SCell is added. If SSB configuration (absoluteFrequencySSB) but no SMTC configuration is provided for the SCell being added, the SSB transmission periodicity is assumed to be 5ms and TSMTC duration for the SCell being added is x ms, where x = the number of consecutive subframes containing all SSBs in one SSB burst transmitted by the SCell being added. If neither SSB configuration (absoluteFrequencySSB) nor SMTC configuration is provided for the SCell being added, TSMTC duration for the SCell being added is 0 ms;

-	the longest SMTC duration among all above active serving cells in SCG when one SCell is released.

-	of up to Y1 slots + TSMTC_duration if the active serving cells are in the same FR2 band as any of the SCells being added or released, provided the cell specific reference signals from the active serving cells and the SCells being added or released are available in the same slot, where, TSMTC_duration is

-	the longest SMTC duration among all above active serving cells in SCG and the SCell being added when one SCell is added. If SSB configuration (absoluteFrequencySSB) but no SMTC configuration is provided for the SCell being added, the SSB transmission periodicity is assumed to be 5ms and TSMTC duration for the SCell being added is x ms, where x = the number of consecutive subframes containing all SSBs in one SSB burst transmitted by the SCell being added. If neither SSB configuration (absoluteFrequencySSB) nor SMTC configuration is provided for the SCell being added, TSMTC duration for the SCell being added is 0 ms;

-	the longest SMTC duration among all above active serving cells in SCG when one SCell is released.

Where X1 and Y1 are specified in table 8.2.1.2.3-2.

For UE supporting On-demand SSB operation, when one OD-SSB SCell in SCG is added or released, no interruption is allowed on any active serving cell in SCG before OD-SSB is activated on the SCell wherein there is only OD-SSB transmission without the first SSB transmission.

Table 8.2.1.2.3-1: Interruption length X1 and Y1 at E-UTRA SCell addition/Release

|  | NR Slot length | Interruption length X1 (slots) |  | Interruption length Y1 (slots) |  |
| --- | --- | --- | --- | --- | --- |
|  | (ms) | Sync | Async | Sync | Async |
| 0 | 1 | 1 | 2 | 1 | 2 |
| 1 | 0.5 | 2 | 3 | 2 | 3 |
| 2 | 0.25 | 5 |  | 4 | 5 |
| 3 | 0.125 | 9 |  | N/A | - N/A |

Table 8.2.1.2.3-2: Interruption length X1 and Y1 at SCell addition/Release

|  | NR Slot length (ms) of victim cell | Interruption length X1 (slots) |  | Interruption length Y1 (slots) |
| --- | --- | --- | --- | --- |
| 0 | 1 | 1 |  | 1 |
| 1 | 0.5 | 2 |  | 2 |
| 2 | 0.25 | Both aggressor cell and victim cell are on FR2 | 4 | 4 |
|  |  | Either aggressor cell or victim cell is on FR1 | 5 |  |
| 3 | 0.125 | Aggressor cell is on FR2 | 8 | 8 |
|  |  | Aggressor cell is on FR1 | 9 |  |

-------------------- END OF CHANGES--------------------------

-------------------------- CHANGE #4 ------------------------------

##### 8.2.1.2.4	Interruptions at SCell activation/deactivation

The requirements in this clause shall apply for the UE configured with PSCell and one SCell.

When one E-UTRA SCell in MCG is activated from deactivated or dormant state, or deactivated from activated or dormant state, the UE is allowed an interruption on any active serving cell in SCG:

-	of up to X2 slots, if the active serving cell is not in the same band as any of the E-UTRA SCells being activated or deactivated, or

-	of up to X2 slots, if the active serving cell is in the band overlapping or partially overlapping with the E-UTRA SCell being activated or deactivated and UE not supporting requirementTypeIndication-r18 indicates it is capable of interBandMRDC-WithOverlapDL-Bands-r16 on this band pair or UE supporting requirementTypeIndication-r18 indicates that it is capable of interBandMRDC-WithOverlapDL-Bands-r16 on this band pair and not provided with nonCollocatedTypeMRDC-r18, or

-	of up to max{Y2 slots + TSMTC_duration, 5ms}, if the active serving cells are in the same band or in the band overlapping or partially overlapping with the E-UTRA SCells being activated or deactivated, and UE does not indicate it is capable of interBandMRDC-WithOverlapDL-Bands-r16 on this band pair or UE supporting requirementTypeIndication-r18 indicates that it is capable of interBandMRDC-WithOverlapDL-Bands-r16 on this band pair and is provided with nonCollocatedTypeMRDC-r18, provided the cell specific reference signals from the active serving cells and the E-UTRA SCells being activated or deactivated are available in the same slot, where TSMTC_duration is the longest SMTC duration among all above active serving cells in SCG.

-	if the active serving cell is in the band overlapping or partially overlapping with the E-UTRA SCell being added or released,

1>	when UE is configured with maxMIMO-Layers with value equal to 4 for all corresponding serving cells, in case of TDD-TDD inter-band (NG) EN-DC with overlapping or partially overlapping bands

2> if UE supporting requirementTypeIndication-r18 is capable of interBandMRDC-WithOverlapDL-Bands-r19on this band pair

3> if nonCollocatedTypeMRDC-v19xy is presented with “type 4”

4>	of up to X2 slots

3> else if nonCollocatedTypeMRDC-v19xy is presented with “type 1”

4> of up to max{Y2 slots + TSMTC_duration, 5ms}

3> else if nonCollocatedTypeMRDC-v19xy is not provided

4> of up to max{Y2 slots + TSMTC_duration, 5ms}

2> else if UE not supporting requirementTypeIndication-r18 indicates it is capable of interBandMRDC-WithOverlapDL-Bands-r19 on this band pair

3> of up to X2 slots

1>	when UE is configured with maxMIMO-Layers with value equal to 2 for all corresponding serving cells, in case of TDD-TDD inter-band (NG) EN-DC with overlapping or partially overlapping bands

2> if nonCollocatedTypeMRDC-r18 is not provided

3>	of up to X2 slots

2> else

3> of up to max{Y2 slots + TSMTC_duration, 5ms}Where X2 and Y2 are specified in table 8.2.1.2.4-1.

When one SCell in SCG is activated or deactivated, the UE is allowed an interruption on any active serving cell in SCG:

-	of up to X2 slots, if the active serving cell and the SCell being activated or deactivated are in a FR1 band pair or in a FR1+FR2 band pair.

-	of up to X2 slots, if the active serving cells and the SCells being activated or deactivated are in a FR2 band pair and UE is capable of independent beam management on this FR2 band pair.

-	of up to X1 slots, if the active serving cell is non-contiguous to the SCell being activated or deactivated in the same FR1 band and UE is capable of intraBandNR-CA-non-collocated-r18 and nonCollocatedTypeNR-CA-r18 is not provided.

or

-	of up to Y1 slots + TSMTC_duration, if the active serving cells are contiguous to any of the SCells being activated or deactivated in the same FR1 band, or if the active serving cells are non-contiguous to any of the SCells being activated or deactivated in the same FR1 band and UE is not capable of intraBandNR-CA-non-collocated-r18 or UE is capable of intraBandNR-CA-non-collocated-r18 and nonCollocatedTypeNR-CA-r18 is provided, provided the cell specific reference signals from the active serving cells and the SCells being activated or deactivated are available in the same slot,

-	if the active serving cell is non-contiguous to the SCell being activated or deactivated in the same FR1 band, and UE is capable of intraBandNR-CA-non-collocated-r19,

1>	when UE is configured with maxMIMO-Layers with value equal to 4 for all corresponding serving cells, in case of TDD-TDD intra-band NR-CA

2> if nonCollocatedTypeMRDC-v19xy is presented with “type 4”

3>	of up to X1 slots

2> else if nonCollocatedTypeMRDC-v19xy is presented with “type 1”

3>	of up to Y1 slots + TSMTC_duration

2> else if nonCollocatedTypeMRDC-v19xy is not provided

3> of up to Y1 slots + TSMTC_duration

1>	when UE is configured with maxMIMO-Layers with value equal to 2 for the configured FR1 intra-band non-contiguous CA for all corresponding serving cells, in case of TDD-TDD intra-band NR-CA

2> if nonCollocatedTypeNR-CA-r18 is not provided

3>	of up to X1 slots

2> else

3> of up to Y1 slots + TSMTC_duration

,where, TSMTC_duration is

-	the longest SMTC duration among all above active serving cells in SCG and the SCell being activated when one SCell is activated. If SSB configuration (absoluteFrequencySSB) but no SMTC configuration is provided for the SCell being activated, the SSB transmission periodicity is assumed to be 5ms and TSMTC duration for the SCell being activated is x ms, where x = the number of consecutive subframes containing all SSBs in one SSB burst transmitted by the SCell being activated. If neither SSB configuration (absoluteFrequencySSB) nor SMTC configuration is provided for the SCell being activated, TSMTC duration for the SCell being activated is 0ms;

-	the longest SMTC duration among all above active serving cells in SCG when one SCell is deactivated.

-	of up to Y2 slots + TSMTC_duration if the active serving cells are in the same FR2 band as any of the SCells being activated or deactivated, provided the cell specific reference signals from the active serving cells and the SCells being activated or deactivated are available in the same slot, where, TSMTC_duration is

-	the longest SMTC duration among all above active serving cells in SCG and the SCell being activated when one SCell is activated. If SSB configuration (absoluteFrequencySSB) but no SMTC configuration is provided for the SCell being activated, the SSB transmission periodicity is assumed to be 5ms and TSMTC duration for the SCell being activated is x ms, where x = the number of consecutive subframes containing all SSBs in one SSB burst transmitted by the SCell being activated. If neither SSB configuration (absoluteFrequencySSB) nor SMTC configuration is provided for the SCell being activated, TSMTC duration for the SCell being activated is 0ms;

-	the longest SMTC duration among all above active serving cells in SCG when one SCell is deactivated.

Where X2 and Y2 are specified in table 8.2.1.2.4-2.

For UE supporting On-demand SSB operation, when one OD-SSB SCell in SCG is activated or deactivated, the requirements in this clause shall apply provided OD-SSB is activated on the SCell or OD-SSB activation/deactivation command and SCell activation/deactivation command are received in the same MAC command.

Table 8.2.1.2.4-1: Interruption length X2 and Y2 at E-UTRA SCell activation/deactivation

|  | NR Slot length | nterruption length X2 (slots) |  | Interruption length Y2 (slots) |  |
| --- | --- | --- | --- | --- | --- |
|  | (ms) | Sync | Async | Sync | Async |
| 0 | 1 | 1 | 2 | 1 | 2 |
| 1 | 0.5 | 1 | 2 | 1 | 2 |
| 2 | 0.25 | 3 |  | 2 | 3 |
| 3 | 0.125 | 5 |  | N/A | N/A |

Table 8.2.1.2.4-2: Interruption length X2 and Y2 at SCell activation/deactivation

|  | NR Slot length (ms) of victim cell | Interruption length X2 (slots) |  | Interruption length Y2 (slots) |
| --- | --- | --- | --- | --- |
| 0 | 1 | 1 |  | 1 |
| 1 | 0.5 | 1 |  | 1 |
| 2 | 0.25 | Both aggressor cell and victim cell are on FR2 | 2 | 2 |
|  |  | Either aggressor cell or victim cell is on FR1 | 3 |  |
| 3 | 0.125 | Aggressor cell is on FR2 | 4 | 4 |
|  |  | Aggressor cell is on FR1 | 5 |  |

-------------------- END OF CHANGES--------------------------

-------------------------- CHANGE #4 ------------------------------

##### 8.2.2.2.22	Interruptions at OD-SSB activation/deactivation

The requirements in this clause shall apply for the UE configured with at least one deactivated OD-SSB SCell. If more than one deactivated SCells are configured with OD-SSB transmission, the requirements in this clause shall apply provided OD-SSB is activated or deactivated on only one of the deactivated OD-SSB SCells.

When OD-SSB is activated or deactivated on a deactivated SCell, the UE is allowed an interruption on any active serving cell, provided that the OD-SSB activation/deactivation command and SCell activation/deactivation command are not received at the same time:

-	of up to X2 slots, if the active serving cell and the SCell on which the OD-SSB is being activated or deactivated are in a FR1 band pair or in a FR1+FR2 band pair.

-	of up to X2 slots, if the active serving cell and the SCell on which the OD-SSB is being activated or deactivated are in a FR2 band pair and UE is capable of independent beam management on this FR2 band pair.

-	of up to X2 slots, if the active serving cell is non-contiguous to the SCell on which the OD-SSB is being activated or deactivated in the same FR1 band and UE is capable of intraBandNR-CA-non-collocated-r18 and nonCollocatedTypeNR-CA-r18 is not provided.

Where X2 is specified in table 8.2.2.2.22-1.

or

-	of up to the duration shown in table 8.2.2.2.22-2, if the active serving cells are contiguous to the SCell on which the OD-SSB is being activated or deactivated in the same FR1 band, or if the active serving cells are non-contiguous to the SCell on which the OD-SSB is being activated or deactivated in the same FR1 band and UE is not capable of intraBandNR-CA-non-collocated-r18 or UE is capable of intraBandNR-CA-non-collocated-r18 and nonCollocatedTypeNR-CA-r18 is provided, provided the cell specific reference signals from the active serving cells and the SCells on which the OD-SSB is being activated or deactivated are available in the same slot.

-	of up to the duration shown in table 8.2.2.2.22-2, if the active serving cells are in the same FR2 band as the SCell on which the OD-SSB is being activated or deactivated provided the cell specific reference signals from the active serving cells and the SCells on which the OD-SSB is being activated or deactivated are available in the same slot.

Table 8.2.2.2.22-1: Interruption length X2 for OD-SSB activation/deactivation for inter-band CA

|  | NR Slot length (ms) of victim cell | Interruption length X2 (slots) |  |
| --- | --- | --- | --- |
| 0 | 1 |  | 1 |
| 1 | 0.5 |  | 1 |
| 2 | 0.25 | Both aggressor cell and victim cell are on FR2 | 2 |
|  |  | Either aggressor cell or victim cell is on FR1 | 3 |
| 3 | 0.125 | Aggressor cell is on FR2 | 4 |
|  |  | Aggressor cell is on FR1 | 5 |
| 5 | 0.03125 | Aggressor cell is on FR1 | 17 |
| 6 | 0.015625 | Aggressor cell is on FR1 | 33 |

Table 8.2.2.2.22-2: Interruption duration for OD-SSB activation/deactivation for intra-band CA

|  | NR Slot length (ms) | Interruption length (slots) |
| --- | --- | --- |
| 0 | 1 | 1 |
| 1 | 0.5 | 1 |
| 2 | 0.25 | 2 |
| 3 | 0.125 | 4 |
| 5 | 0.03125 | 16 |
| 6 | 0.015625 | 32 |

-------------------- END OF CHANGES--------------------------

-------------------------- CHANGE #4 ------------------------------

##### 8.2.2.2.1	Interruptions at SCell addition/release

When any number of SCells between one and 7 is added or released using the same RRCConnectionReconfiguration message as defined in TS 38.331 [2], the UE is allowed an interruption on any active serving cell during the RRC reconfiguration procedure as follows:

-	of up to X1 slots, if the active serving cell and the SCell being added or released are in a FR1 band pair or in a FR1+FR2 band pair.

-	of up to X1 slots, if the active serving cell and the SCell being added or released are in a FR2 band pair and UE is capable of independent beam management on this FR2 band pair.

-	of up to X1 slots, if the active serving cell is non-contiguous to the SCell being added or released in the same FR1 band and UE is capable of intraBandNR-CA-non-collocated-r18 and nonCollocatedTypeNR-CA-r18 is not provided.

Where X1 is specified in table 8.2.2.2.1-1.

or

-	of up to the duration shown in table 8.2.2.2.1-2, if the active serving cells are contiguous to any of the SCells being added or released in the same FR1 band, or if the active serving cells are non-contiguous to any of the SCells being added or released in the same FR1 band and UE is not capable of intraBandNR-CA-non-collocated-r18 or UE is capable of intraBandNR-CA-non-collocated-r18 and nonCollocatedTypeNR-CA-r18 is provided, provided the cell specific reference signals from the active serving cells and the SCells being added or released are available in the same slot.

-	of up to the duration shown in table 8.2.2.2.1-2, if the active serving cells are in the same FR2 band as any of the SCells being added or released, provided the cell specific reference signals from the active serving cells and the SCells being added or released are available in the same slot.

-	if the active serving cell is non-contiguous to the SCell being added or released in the same FR1 band, and UE is capable of intraBandNR-CA-non-collocated-r19,

1>	when UE is configured with maxMIMO-Layers with value equal to 4 for all corresponding serving cells, in case of TDD-TDD intra-band NR-CA

2> if nonCollocatedTypeMRDC-v19xy is presented with “type 4”

3>	of up to X1 slots

2>else if nonCollocatedTypeMRDC-v19xy is presented with “type 1”

3>	of up to the duration shown in table 8.2.2.2.1-2

2>else if nonCollocatedTypeMRDC-v19xy is not provided

3> of up to the duration shown in table 8.2.2.2.1-2

1>	when UE is configured with maxMIMO-Layers with value equal to 2 for the configured FR1 intra-band non-contiguous CA for all corresponding serving cells, in case of TDD-TDD intra-band NR-CA

2> if nonCollocatedTypeNR-CA-r18 is not provided

3>	of up to X1 slots

2> else

3> of up to the duration shown in table 8.2.2.2.1-2

For UE supporting On-demand SSB operation, when one OD-SSB SCell in SCG is added or released, no interruption is allowed on any active serving cell during the RRC reconfiguration procedure before OD-SSB is activated on the SCell wherein there is only OD-SSB transmission without the first SSB transmission.

Table 8.2.2.2.1-1: Interruption length X1 for SCell addition/release for inter-band CA

|  | NR Slot length (ms) of victim cell | Interruption length X1 (slots) |  |
| --- | --- | --- | --- |
| 0 | 1 | 1 |  |
| 1 | 0.5 | 2 |  |
| 2 | 0.25 | Both aggressor cell and victim cell are on FR2 | 4 |
|  |  | Either aggressor cell or victim cell is on FR1 | 5 |
| 3 | 0.125 | Aggressor cell is on FR2 | 8 |
|  |  | Aggressor cell is on FR1 | 9 |
| 5 | 0.03125 | Aggressor cell is on FR1 | 33 |
| 6 | 0.015625 | Aggressor cell is on FR1 | 65 |

Table 8.2.2.2.1-2: Interruption duration for SCell addition/release for intra-band CA

|  | NR Slot length (ms) | Interruption length (slots) |
| --- | --- | --- |
| 0 | 1 | 1 + TSMTC_duration * Nslotsubframe,μ |
| 1 | 0.5 | 2 + TSMTC_duration * Nslotsubframe,μ |
| 2 | 0.25 | 4 + TSMTC_duration * Nslotsubframe,μ |
| 3 | 0.125 | 8 + TSMTC_duration * Nslotsubframe,μ |
| 5 | 0.03125 | 32+ TSMTC_duration * Nslotsubframe,μ |
| 6 | 0.015625 | 64+ TSMTC_duration * Nslotsubframe,μ |
| NOTE 1: When UE is not supporting On-demand SSB operation or OD-SSB transmission is not indicated, TSMTC_duration measured in subframes is - the longest SMTC duration among all above active serving cells and the SCell being added when one SCell is added. If SSB configuration (absoluteFrequencySSB) but no SMTC configuration is provided for the SCell being added, the SSB transmission periodicity is assumed to be 5ms and TSMTC duration for the SCell being added is x ms, where x = the number of consecutive subframes containing all SSBs in one SSB burst transmitted by the SCell being added. If neither SSB configuration (absoluteFrequencySSB) nor SMTC configuration is provided for the SCell being added, TSMTC duration for the SCell being added is 0ms; - the longest SMTC duration among all active serving cells in the same band when one SCell is released. Otherwise, TSMTC duration for the SCell being activated with OD-SSB is the periodicity of OD-SSB NOTE 2: Nslotsubframe,μ is as defined in TS 38.211 [6]. |  |  |

-------------------- END OF CHANGES--------------------------

-------------------------- CHANGE #4 ------------------------------

##### 8.2.2.2.2	Interruptions at SCell activation/deactivation

When an SCell is activated or deactivated as defined in TS 37.340 [17], the UE is allowed an interruption on any active serving cell:

-	of up to X2 slots, if the active serving cell and the SCell being activated or deactivated are in a FR1 band pair or in a FR1+FR2 band pair.

-	of up to X2 slots, if the active serving cell and the SCell being activated or deactivated are in a FR2 band pair and UE is capable of independent beam management on this FR2 band pair.

-	of up to X2 slots, if the active serving cell is non-contiguous to the SCell being activated or deactivated in the same FR1 band and UE is capable of intraBandNR-CA-non-collocated-r18] and [nonCollocatedTypeNR-CA-r18 is not provided.

Where X2 is specified in table 8.2.2.2.2-1.

or

-	of up to the duration shown in table 8.2.2.2.2-2, if the active serving cells are contiguous to any of the SCells being activated or deactivated in the same FR1 band, or if the active serving cells are non-contiguous to any of the SCells being activated or deactivated in the same FR1 band and UE is not capable of intraBandNR-CA-non-collocated-r18 or UE is capable of intraBandNR-CA-non-collocated-r18 and nonCollocatedTypeNR-CA-r18 is provided, provided the cell specific reference signals from the active serving cells and the SCells being activated or deactivated are available in the same slot.

-	of up to the duration shown in table 8.2.2.2.2-2, if the active serving cells are in the same FR2 band as any of the SCells being activated or deactivated provided the cell specific reference signals from the active serving cells and the SCells being activated or deactivated are available in the same slot.

-	if the active serving cell is non-contiguous to the SCell being activated or deactivated in the same FR1 band, and UE is capable of intraBandNR-CA-non-collocated-r19,

1>	when UE is configured with maxMIMO-Layers with value equal to 4 for all corresponding serving cells, in case of TDD-TDD intra-band NR-CA

2> if nonCollocatedTypeMRDC-v19xy is presented with “type 4”

3>	of up to X2 slots

2>else if nonCollocatedTypeMRDC-v19xy is presented with “type 1”

3>	of up to the duration shown in table 8.2.2.2.2-2

2>else if nonCollocatedTypeMRDC-v19xy is not provided

3> of up to the duration shown in table 8.2.2.2.2-2

1>	when UE is configured with maxMIMO-Layers with value equal to 2 for the configured FR1 intra-band non-contiguous CA for all corresponding serving cells, in case of TDD-TDD intra-band NR-CA

2> if nonCollocatedTypeNR-CA-r18 is not provided

3>	of up to X2 slots

2> else

3> of up to the duration shown in table 8.2.2.2.2-2

For UE supporting On-demand SSB operation, when one OD-SSB SCell is activated or deactivated, the requirements in this clause shall apply provided OD-SSB is activated on the SCell or OD-SSB activation/deactivation command and SCell activation/deactivation command are received in the same MAC command.

Table 8.2.2.2.2-1: Interruption length X2 for SCell activation/deactivation for inter-band CA

|  | NR Slot length (ms) of victim cell | Interruption length X2 (slots) |  |
| --- | --- | --- | --- |
| 0 | 1 |  | 1 |
| 1 | 0.5 |  | 1 |
| 2 | 0.25 | Both aggressor cell and victim cell are on FR2 | 2 |
|  |  | Either aggressor cell or victim cell is on FR1 | 3 |
| 3 | 0.125 | Aggressor cell is on FR2 | 4 |
|  |  | Aggressor cell is on FR1 | 5 |
| 5 | 0.03125 | Aggressor cell is on FR1 | 17 |
| 6 | 0.015625 | Aggressor cell is on FR1 | 33 |

Table 8.2.2.2.2-2: Interruption duration for SCell activation/deactivation for intra-band CA

|  | NR Slot length (ms) | Interruption length (slots) |
| --- | --- | --- |
| 0 | 1 | 1 + TSMTC_duration * Nslotsubframe,μ |
| 1 | 0.5 | 1 + TSMTC_duration * Nslotsubframe,μ |
| 2 | 0.25 | 2 + TSMTC_duration * Nslotsubframe,μ |
| 3 | 0.125 | 4 + TSMTC_duration * Nslotsubframe,μ |
| 5 | 0.03125 | 16+ TSMTC_duration * Nslotsubframe,μ |
| 6 | 0.015625 | 32+ TSMTC_duration * Nslotsubframe,μ |
| NOTE 1: When UE is not supporting On-demand SSB operation or OD-SSB transmission is not indicated,TSMTC_duration measured in subframes is - the longest SMTC duration among all above active serving cells and the SCell being activated when one SCell is activated. If SSB configuration (absoluteFrequencySSB) but no SMTC configuration is provided for the SCell being activated, the SSB transmission periodicity is assumed to be 5ms and TSMTC duration for the SCell being activated is x ms, where x = the number of consecutive subframes containing all SSBs in one SSB burst transmitted by the SCell being activated. If no SSB configuration (absoluteFrequencySSB) nor SMTC configuration is provided for the SCell being activated, TSMTC duration for the SCell being activated is 0ms; - the longest SMTC duration among all active serving cells in the same band when one SCell is deactivated. Otherwise, TSMTC duration for the SCell being activated is 0ms. NOTE 2: Nslotsubframe,μ is as defined in TS 38.211 [6]. |  |  |

-------------------- END OF CHANGES--------------------------

-------------------------- CHANGE #4 ------------------------------

##### 8.2.2.2.3	Interruptions during measurements on deactivated SCC

Interruptions on PCell or activated SCell(s) due to measurements when an SCell is deactivated are allowed with up to 0.5% probability of missed ACK/NACK

when the configured measCycleSCell [2] is 640 ms or longer.

-	If the PCell or activated SCell(s) is not in the same band as the deactivated SCell, the UE is only allowed to cause interruptions on PCell or activated SCell(s) immediately before and immediately after an SMTC. Each interruption shall not exceed requirement in table 8.2.2.2.2-1.

-	If the PCell or activated SCell(s) is non-contiguous to the deactivated SCell in the same FR1 band and UE is capable of intraBandNR-CA-non-collocated-r18 on this FR1 band and nonCollocatedTypeNR-CA-r18 is not provided, the UE is only allowed to cause interruptions on PCell or activated SCell(s) immediately before and immediately after an SMTC. Each interruption shall not exceed requirement in table 8.2.2.2.2-1.

-	If the PCell or activated SCell(s) is contiguous to the deactivated SCell in the same FR1 band, or if the PCell or activated SCell(s) is non-contiguous to the deactivated SCell in the same FR1 band and UE is not capable of intraBandNR-CA-non-collocated-r18 or UE is capable of intraBandNR-CA-non-collocated-r18 and nonCollocatedTypeNR-CA-r18 is provided, the UE is only allowed to cause an interruption on PCell or activated SCell(s) no earlier than X slots before TSMTC_duration and no later than X slots after TSMTC_duration, provided the cell specific reference signals from the active serving cells and the deactivated SCell are available in the same slot, where X and TSMTC_duration are given by table 8.2.2.2.3-1. The interruption shall not exceed requirements in table 8.2.2.2.3-1.

-	If the PCell or activated SCell(s) is contiguous to the deactivated SCell in the same FR1 band, or if the PCell or activated SCell(s) is non-contiguous to the deactivated SCell in the same FR1 band and UE is not capable of intraBandNR-CA-non-collocated-r18 or UE is capable of intraBandNR-CA-non-collocated-r18 and nonCollocatedTypeNR-CA-r18 is provided, the UE is only allowed to cause an interruption on PCell or activated SCell(s) no earlier than X slots before TSMTC_duration and no later than X slots after TSMTC_duration, provided the cell specific reference signals from the active serving cells and the deactivated SCell are available in the same slot, where X and TSMTC_duration are given by table 8.2.2.2.3-1. The interruption shall not exceed requirements in table 8.2.2.2.3-1.

-	If the PCell or activated SCell(s) is non-contiguous to the deactivated SCell in the same FR1 band, and UE is capable of intraBandNR-CA-non-collocated-r19,

1>	when UE is configured with maxMIMO-Layers with value equal to 4 for all corresponding serving cells, in case of TDD-TDD intra-band NR-CA

2> if nonCollocatedTypeMRDC-v19xy is presented with “type 4”

3>	the UE is only allowed to cause interruptions on PCell or activated SCell(s) immediately before and immediately after an SMTC. Each interruption shall not exceed requirement in table 8.2.2.2.2-1.

2>else if nonCollocatedTypeMRDC-v19xy is presented with “type 1”

3>	the UE is only allowed to cause an interruption on PCell or activated SCell(s) no earlier than X slots before TSMTC_duration and no later than X slots after TSMTC_duration, provided the cell specific reference signals from the active serving cells and the deactivated SCell are available in the same slot, where X and TSMTC_duration are given by table 8.2.2.2.3-1. The interruption shall not exceed requirements in table 8.2.2.2.3-1.

1>	when UE is configured with maxMIMO-Layers with value equal to 2 for the configured FR1 intra-band non-contiguous CA for all corresponding serving cells, in case of TDD-TDD intra-band NR-CA

2> if nonCollocatedTypeNR-CA-r18 is not provided

3>	the UE is only allowed to cause interruptions on PCell or activated SCell(s) immediately before and immediately after an SMTC. Each interruption shall not exceed requirement in table 8.2.2.2.2-1.

2> else

3> the UE is only allowed to cause an interruption on PCell or activated SCell(s) no earlier than X slots before TSMTC_duration and no later than X slots after TSMTC_duration, provided the cell specific reference signals from the active serving cells and the deactivated SCell are available in the same slot, where X and TSMTC_duration are given by table 8.2.2.2.3-1. The interruption shall not exceed requirements in table 8.2.2.2.3-1.

The interruption requirements in table 8.2.2.2.3-1 are not applicable when a UE is configured with NCSG in the same frequency range as the deactivated SCell unless the SMTC on the deactivated SCC is fully non-overlapped with NCSG.

For UE supporting On-demand SSB operation,

-    No interruption is allowed on activated SCell(s) due to measurements on deactivated SCC before OD-SSB is activated on the deactivated OD-SSB SCell wherein there is only OD-SSB but not the first SSB transmission.

-    If UE does not receive SCell activation command before the end of FMW, the interruption requirement in Table 8.2.2.2.3-1 shall apply, provided that the measCycleSCell equal or longer than 640ms when

Table 8.2.2.2.3-1: Interruption duration for measurement on deactivated SCC for intra-band CA

|  | NR Slot length (ms) | X (slots) | Interruption length (slots) |
| --- | --- | --- | --- |
| 0 | 1 | 1 | 2 + TSMTC_duration * Nslotsubframe,μ |
| 1 | 0.5 | 1 | 2 + TSMTC_duration * Nslotsubframe,μ |
| 2 | 0.25 | 2 | 4 + TSMTC_duration * Nslotsubframe,μ |
| 3 | 0.125 | 4 | 8 + TSMTC_duration * Nslotsubframe,μ |
| 5 | 0.03125 | 16 | 32 + TSMTC_duration * Nslotsubframe,μ |
| 6 | 0.015625 | 32 | 64 + TSMTC_duration * Nslotsubframe,μ |
| NOTE 1: TSMTC_duration measured in subframes is the longest SMTC duration among all above active serving cells and the deactivated SCell to be measured i; NOTE 2: Nslotsubframe,μ is as defined in TS 38.211 [6]. |  |  |  |  |

For a UE supporting LB-CA via switching and configured with switch pattern, interruptions on PCell due to the measurement on deactivated SDL SCell are allowed with up to Y % probability of missed ACK/NACK. The value of Y is defined as 2*Length_intmeasCycleSCell ;where, Length_int is equal to  (Tuning time from PCell to SCell  + TSMTC_duation + Retuning time from SCell to PCell),

Tuning time from PCell to SCell is equal to the RRC configured switching gap from PCell to SCell, i.e., LBCA-SwitchingGap-Duration-PCelltoSCell-r19 in slots

Returning time from SCell to PCell is equal to the RRC configured switching gap from SCell to PCell, i.e., LBCA-SwitchingGap-SCelltoPCell-r19 in slots

-------------------- END OF CHANGES--------------------------

-------------------------- CHANGE #4 ------------------------------

### 8.3.19	OD-SSB based SCell Activation Delay Requirement for Deactivated SCell

The requirements in this clause shall apply for the UE supporting On-demand SSB operation and configured with at least one downlink OD-SSB SCell in EN-DC, or in standalone NR carrier aggregation or in NE-DC or in NR-DC and when one OD-SSB SCell is being activated.

The requirements in the clause 8.3.x1 apply, provided the following conditions are met:

-	an OD-SSB transmission is activated in a configured DL BWP where a first SSB transmission is not configured, or

-	an OD-SSB transmission is activated in a configured DL BWP where a first SSB transmission is configured and the OD-SSB cannot be used to obtain SIB1.

Where the first SSB transmission is specified as in TS 38.213 and have the following relations with the OD-SSB transmission:

-	are with the same physical cell identity, and

-	are in the same frequency location, and

-	are with same sub-carrier spacing, and

-	are with the same downlink transmit power, and

-	are quasi co-located with respect to Doppler shift, Doppler spread, average gain, average delay, delay spread and, when applicable, spatial RX parameters, when they have a same SSB index.

The delay within which the UE shall be able to activate the deactivated SCell depends upon the specified conditions.

Upon receiving SCell activation command in slot n, the UE shall perform SCell activation procedure based on the transmitted SSBs, where the transmitted SSBs are

-	OD-SSBs being transmitted in the configured DL BWP if no first SSB transmission is configured, or

-	the union of OD-SSBs and first SSBs if first SSBs are transmitted and the time instance when receiving OD-SSB activation indication is no later than the time instance when receiving the SCell activation command, or

-	first SSBs only if first SSBs are transmitted but no OD-SSB transmission during the SCell activation.

Upon receiving SCell activation command in slot n, the UE shall be capable to transmit valid CSI report and apply actions related to the activation command for the SCell being activated no later than in slot n+THARQ+Tactivation_time+TCSI_ReportingNR slot length , where:

THARQ (in ms) is the timing between DL data transmission and acknowledgement as specified in TS 38.213 [3]

Tactivation_time is the SCell activation delay in milliseconds.

If the SCell is known and belongs to FR1, Tactivation_time is:

-	TFirst, OD-SSB + T∆ + 5 ms, if the measurement period of the SCell being activated is equal to or smaller than 2400 ms.

-	TFirst, OD-SSB + Trs, OD-SSB + T∆ + 5 ms, if the measurement period of the SCell being activated is larger than 2400 ms.

If the SCell is unknown and belongs to FR1, and if one of the following conditions is met

-	only one SSB is indicated in ‘od-ssb-PositionInBurst’ if provided; otherwise, in ssb-PositionsInBurst, or

-	 multiple SSBs are indicated in ‘od-ssb-PositionInBurst’ if provided; otherwise, in ssb-PositionsInBurst and TCI indication of SSBs is provided in same MAC PDU with SCell activation,

provided that the side condition Ês/Iot ≥ -2 dB is fulfilled, Tactivation_time is:

-	TFirst, OD-SSB + 2* Trs,OD-SSB + 5 ms, if the following conditions are met,

-	the SCell is contiguous to an active serving cell in the same band, and

-	its od-ssb-PositionInBurst if provided; otherwise, ssb-PositionsInBurst provided is the same as the one of contiguous FR1 active serving cell, and

-	its SFN offset is same as the one of contiguous FR1 active serving cell, and

-	its RTD with contiguous FR1 active serving cell is smaller than or equal to 260 ns with respect to the to-be-activated SCell’s SSB numerology, and its reception power difference with contiguous FR1 active serving cell is smaller than or equal to 6 dB;

-	TFirst, OD-SSB + 3*Trs, OD-SSB + 5 ms, otherwise.

Otherwise, provided that the side condition Ês/Iot ≥ -2 dB is fulfilled, Tactivation_time is:

-	6 ms + TFirst, OD-SSB + 2*Trs, OD-SSB + TL1-RSRP, measure, OD-SSB + TL1-RSRP, report + THARQ + max(Tuncertainty_MAC + TFineTiming, OD-SSB + 2 ms, Tuncertainty_SP), if semi-persistent CSI-RS is used for CSI reporting,

-	3 ms + TFirst, OD-SSB + 2*Trs, OD-SSB + TL1-RSRP, measure, OD-SSB + TL1-RSRP ,report + max(THARQ + Tuncertainty_MAC + 5 ms + TFineTiming, OD-SSB, Tuncertainty_RRC + TRRC_delay), if periodic CSI-RS is used for CSI reporting.

-	However, when the following conditions are fulfilled, no activation requirement will be applied for this unknown SCell:

-	the SCell is contiguous to an active serving cell in the same band, and

-	A single SSB is used in the unknown SCell; or multiple SSBs are used in the SCell and TCI state indication for PDCCH is provided by the same MAC PDU used for SCell activation; and

-	its od-ssb-PositionInBurst if provided; otherwise, ssb-PositionsInBurst provided is same as the one of contiguous FR1 active serving cell, and

-	its SSB offset is same as the one of contiguous FR1 active serving cell, and

-	its RTD with contiguous FR1 active serving cell is larger than 260 ns, or its reception power difference with contiguous FR1 active serving cell is larger than 6 dB;

If the SCell being activated belongs to FR2 and if there is at least one active serving cell on that FR2 band, then Tactivation_time is TFirst OD_SSB + 5ms provided:

-	the SSBs in the serving cell(s) and the SSBs in the SCell fulfil the condition defined in clause 3.6.3, and

-	the parameter od-ssb-PositionsInBurst if provided; otherwise, ssb-PositionsInBurst provided if the SCell being activated is same for the serving cell(s), and

-	the SSB is in the same half-frame on the SCell and the contiguous FR2 active serving cell.

If the SCell being activated belongs to FR2 and if there is no active serving cell on that FR2 band provided that PCell or PSCell is in FR1 or in FR2:

If the target SCell is known to UE and semi-persistent CSI-RS is used for CSI reporting, then Tactivation_time is:

-	3 ms + max(Tuncertainty_MAC + TFineTiming, OD-SSB + 2 ms, Tuncertainty_SP), where Tuncertainty_MAC=0 and Tuncertainty_SP=0, if

-	UE receives the SCell activation command, semi-persistent CSI-RS activation command and TCI state activation command at the same time, or

-	Provided this is the initial activation after SCell addition, and

-	TCI-ActivatedConfig is configured for the SCell, and UE receives the SCell activation command and semi-persistent CSI-RS activation command at the same time.

If the target SCell is known to UE and periodic CSI-RS is used for CSI reporting, then Tactivation_time is:

-	max(Tuncertainty_MAC + 5 ms + TFineTiming, OD-SSB, Tuncertainty_RRC + TRRC_delay-THARQ), where Tuncertainty_MAC=0, if

-	UE receives the SCell activation command and TCI state activation commands at the same time, or

-	Provided this is the initial activation after SCell addition, and

-	UE receives the SCell activation command and TCI-ActivatedConfig is configured for the SCell.

If the PCell/PSCell and the target SCell are configured as FR1-FR2-1 CA or if the PCell/PSCell and the target SCell are in a FR2-1 band pair with independent beam management, and the target SCell is unknown to UE and semi-persistent CSI-RS is used for CSI reporting, provided that the side condition Ês/Iot ≥ -2 dB is fulfilled, then Tactivation_time is:

-	6 ms + TFirst, OD-SSB + 23*Trs, OD-SSB  + TL1-RSRP, measure, OD-SSB + TL1-RSRP, report  + THARQ + max(Tuncertainty_MAC + TFineTiming, OD-SSB  + 2 ms, Tuncertainty_SP), or

If the PCell/PSCell and the target SCell are configured as FR1-FR2-1 CA or if the PCell/PSCell and the target SCell are in a FR2-1 band pair with independent beam management, and the target SCell is unknown to UE and periodic CSI-RS is used for CSI reporting, provided that the side condition Ês/Iot ≥ -2 dB is fulfilled, then Tactivation_time is:

-	3 ms + TFirst OD-SSB + 23*Trs, OD-SSB + TL1-RSRP, measure, OD-SSB + TL1-RSRP, report + max {(THARQ + Tuncertainty_MAC + 5 ms + TFineTiming, OD-SSB), (Tuncertainty_RRC + TRRC_delay)}.

where,

Trs, OD-SSB is the periodicity of the transmitted SSB.

T∆ is the additional time for fine timing tracking and acquiring full timing information of the SCell. T∆ = 2*Trs, OD-SSB ms for SCell operating with 12 PRB SSB BW. Otherwise, T∆ = 0 ms.

TFirst, OD-SSB is the time to the end of the first complete transmitted SSB burst after slot n + THARQ+3msNR slot length, where the transmitted SSB is

-	OD-SSB in the configured DL BWP if no first SSB transmission is configured, or

-	the union of OD-SSB and first SSB if first SSB is transmitted and the time instance when receiving OD-SSB activation indication is no later than the time instance when receiving the SCell activation command, or

-	first SSB only if first SSB is transmitted but no OD-SSB transmission during the SCell activation.

TFineTiming, OD-SSB is the time period between UE finish processing the last activation command for PDCCH TCI, PDSCH TCI (when applicable) and the timing of first complete transmited SSB burst corresponding to the TCI state.

TL1-RSRP, measure, OD-SSB is the transmited SSB based L1-RSRP measurement delay TL1-RSRP_Measurement_Period_SSB based on applicability as defined in clause 9.5.4 assuming M=1 and TReport=0

TL1-RSRP, report is delay of acquiring CSI reporting resources.

Tuncertainty_MAC is the time period between reception of the last activation command for PDCCH TCI, PDSCH TCI (when applicable) relative to

-	SCell activation command for known case;

-	First valid L1-RSRP reporting for unknown case.

Tuncertainty_RRC is the time period between reception of the RRC configuration message for TCI of periodic CSI-RS for CQI reporting (when applicable) relative to

-	SCell activation command for known case;

-	First valid L1-RSRP reporting for unknown case.

Tuncertainty_SP is the time period between reception of the activation command for semi-persistent CSI-RS resource set for CQI reporting relative to

-	SCell activation command for known case;

-	First valid L1-RSRP reporting for unknown case.

TRRC_delay is the RRC procedure delay as specified in TS 38.331 [2].

TCSI_reporting is the delay (in ms) including uncertainty in acquiring the first available downlink CSI reference resource, UE processing time for CSI reporting and uncertainty in acquiring the first available CSI reporting resources as specified in TS 38.331 [2].

Longer delays for RRM measurement requirements, and in case of FR2 also SSB based RLM/BFD/CBD/L1-RSRP measurement requirements, can be expected during the cell detection time for unknown SCell activation.

SCell in FR1 is known if it has been meeting the following conditions:

-	During the period equal to max(5*measCycleSCell, 5*DRX cycles) for FR1 before the reception of the SCell activation command:

-	the UE has sent a valid measurement report for the SCell being activated and

-	the SSB measured remains detectable according to the cell identification conditions specified in clause 9.2 and 9.3.

-	the SSB measured during the period equal to max(5*measCycleSCell, 5*DRX cycles) or the SSB used for SCell activation which has the same SSB index of the SSB measured also remains detectable during the SCell activation delay according to the cell identification conditions specified in clause 9.2 and 9.3.

Otherwise SCell in FR1 is unknown.

For the first SCell activation in FR2 bands, the SCell is known if it has been meeting the following conditions:

-	During the period equal to 4 s for UE supporting power class 1/5 and 3 s for UE supporting power class 2/3/4 before UE receives the last activation command for PDCCH TCI, PDSCH TCI (when applicable) and semi-persistent CSI-RS for CQI reporting (when applicable):

-	the UE has sent a valid L3-RSRP measurement report with SSB index, and

-	SCell activation command is received after L3-RSRP reporting and no later than the time when UE receives MAC-CE command for TCI activation

-	During the period from L3-RSRP reporting to the valid CQI reporting, the reported SSBs with the indexes or the SSBs used for SCell activation with the same indexes remain detectable according to the cell identification conditions specified in clauses 9.2 and 9.3, and the TCI state is selected based on one of the latest reported SSB indexes.

Otherwise, the first SCell in FR2 band is unknown.

The requirement for unknown SCell applies provided that the activation commands for PDCCH TCI, PDSCH TCI (when applicable), semi-persistent CSI-RS for CQI reporting (when applicable), and configuration message for TCI of periodic CSI-RS for CQI reporting (when applicable) are based on the latest valid L1-RSRP reporting.

In addition to CSI reporting defined above, UE shall also apply other actions related to the activation command specified in TS 38.331 [2] for a SCell at the first opportunities for the corresponding actions once the SCell is activated.

The starting point of an interruption window on SpCell or any activated SCell, as specified in clause 8.2, shall not occur before slot n+1+THARQNR slot length  and not occur after slot slot n+1+THARQ+3ms+TXNR slot length, where NR slot length is with respect to the numerology used in the SCell being activated, and TX is:

-	0, if Tactivation_time is 3 ms;

-	TFirst, OD-SSB, for any scenario where Tactivation_time includes TFirst, OD-SSB;

-	Tuncertainty_MAC + TFineTiming, OD-SSB, for any scenario where Tactivation_time includes only TFineTiming, OD-SSB and no TFirst, OD-SSB.

The length of the interruption window may be different for different victim cells, and depends on the applicable scenario and on the frequency band relation between the aggressor cell and the victim cell.

The requirements in this clause and requirements on interruption due to SCell activation in clause 8.2 apply provided that the first SSB and the OD-SSB of the to-be-activated SCell is within the first active DL BWP of the SCell.

No OD-SSB based SCell activation requirement applies if OD-SSB is transmitting upon UE receiving the SCell activation command but NW deactivates the OD-SSB earlier than the slot n+THARQ+Tactivation_time+TCSI_ReportingNR slot length.

Starting from the slot specified in clause 4.3 of TS 38.213 [3] (timing for secondary Cell activation/deactivation) and until the UE has completed the SCell activation, the UE shall report out of range if the UE has available uplink resources to report CQI for the SCell.

Starting from the slot specified in clause 4.3 of TS 38.213 [3] (timing for secondary Cell activation/deactivation) and until the UE has completed a first L1-RSRP measurement, the UE shall report lowest valid L1 SS-RSRP range if the UE has available uplink resources to report L1-RSRP for the SCell.

### 8.3.20	OD-SSB based SCell Deactivation Delay Requirement for Activated SCell

The requirements in this clause shall apply for the UE supporting On-demand SSB operation configured with at least one downlink SCell, where OD-SSB transmission is activated, in EN-DC, or in standalone NR carrier aggregation or in NE-DC or in NR-DC and when one SCell is being deactivated.

Upon receiving SCell deactivation command in slot n, the UE shall accomplish the deactivation actions for the SCell being deactivated no later than in slot n + THARQ+3msNR slot length. The starting point of an interruption window on SpCell or any activated SCell, as specified in clause 8.2, shall not occur before slot n+1+THARQNR slot length and not occur after slot n+1+THARQ+3msNR slot length, where NR slot length is with respect to the numerology used in the SCell being deactivated.

Upon expiry of the sCellDeactivationTimer in slot n, the UE shall accomplish the deactivation actions for the SCell being deactivated no later than in slot n + 3msNR slot length. The starting point of an interruption window on SpCell or any activated SCell, as specified in clause 8.2, shall not occur before slot n+1 and not occur after slot n+1+ 3msNR slot length, where NR slot length is with respect to the numerology used in the SCell being deactivated.

The length of the interruption window may be different for different victim cells, and depends on the applicable scenario and on the frequency band relation between the aggressor cell and the victim cell.

-------------------- END OF CHANGES--------------------------

-------------------------- CHANGE #4 ------------------------------

-------------------- END OF CHANGES--------------------------

-------------------------- CHANGE #4 ------------------------------

#### 8.5.5.2	Minimum requirement

Upon request the UE shall be able to evaluate whether the L1-RSRP measured on the configured SSB resource in set q1 estimated over the last TEvaluate_CBD_SSB period becomes better than the threshold Qin_LR provided SSB_RP and SSB Ês/Iot are according to Annex table B.2.4.1 for a corresponding band.The UE shall monitor the configured SSB resources using the evaluation period in table 8.5.5.2-1 and 8.5.5.2-2 corresponding to the non-DRX mode, if the configured DRX cycle ≤ 320 ms.

The value of TEvaluate_CBD_SSB is defined in table 8.5.5.2-1 for FR1.

The value of TEvaluate_CBD_SSB is defined in table 8.5.5.2-2 for FR2 with scaling factor N, where

-	N = 2, 4 or 6 for serving cell in FR2-1 if the UE supports fastBeamSweepingMultiRx-r18 according to the conditions in clause 3.6.19,

-	N=8 for other cases in FR2-1, and

-	N=12 for FR2-2.

-	For a UE supporting LB-CA via switching, or for a UE supporting concurrentMeasGapsPreMG-r18 and when concurrent measurement gap(s) with Pre-MG(s) are configured, or a UE supporting concurrentMeasGapsNCSG-r18 and when concurrent measurement gap(s) with NCSG(s)  are configured, or a UE supporting concurrentMeasGap-r17 or musim-GapPreference-r17 or both concurrentMeasGap-r17 and musim-GapPreference-r17, and when concurrent gaps or periodic MUSIM gaps or both concurrent GAPs and periodic MUSIM gaps are configured,

-	an SSB resource occasion for candidate beam detection is not considered to be overlapped by a gap occasion if the gap occasion is dropped according to clauses 9.1.8 and 9.1.10,

-	P value for a CBD-RS resource to be measured is defined as

-	Ntotal / Noutside_MG in FR1

-	Psharing factor * Ntotal / Noutside_MG in FR2 with Navailable = 0

-	Ntotal / Navailable in FR2 with Navailable> 0

-	For a window W of duration max(TL1,  xRP_max, switching pattern periodicity), where xRP_max is the maximum xRP across all configured per-UE measurement gaps or periodic MUSIM gap(s) or NCSGs and per-FR measurement gaps or NCSGs, and, in case of Pre-MG, all activated per-UE measurement gaps and per-FR measurement gaps, within the same FR as serving cell, and starting at the beginning of any CBD-RS resource occasion:

-	Ntotal is the total number of CBD-RS resource occasions within the window W, including those overlapped with GAP occasions, MUSIM gap occasions or SMTC occasions within the window W,

For UEs supporting LB CA via switching and configured with low band CA via switching:

-	for the BFD monitoring on PCell, Ntotal also includes SSB resource occasions that overlap with the SDL ON duration within the window,

-	for the BFD monitoring on SCell, Ntotal also includes SSB resource occasions that overlap with the PCell ON duration within the window, as defined by the configured switching pattern, and

-	Noutside_MG is the number of CBD-RS resource occasions within the window W,

-	that are not overlapped with any non-dropped GAP occasion nor non-dropped MUSIM gap occasion, or

-	that are not overlapped with SCell ON duration corresponding to the LB CA switching pattern, for the BFD on PCell for a UE supporting LowBandSwitching-r19 and configured with low band CA via switching.

-	that are not overlapped with PCell ON duration corresponding to the LB CA switching pattern, for the BFD on SCell for a UE supporting LowBandSwitching-r19 and configured with low band CA via switching, and

-	Navailable is the number of CBD-RS resource occasions that are not overlapped with any non-dropped GAP occasion nor non-dropped MUSIM gap occasion nor any SMTC occasion within the window W, and

-	an SSB resource occasion for candidate beam detection is considered to be overlapped with the MUSIM gap if it overlaps a MUSIM gap occasion, and

-	TL1 is periodicity of the target CBD-RS.

-	xRP = MGRP when configured GAP is activated Pre-MG or MG, and xRP = VIRP when configured GAP is NCSG.

-	for UE not supporting LB CA via switching, switching pattern periodicity is not applicable.

-	CBD requirement in this clause is not applied when Noutside_MG = 0.

Otherwise, for a UE neither supporting concurrentMeasGap-r17 nor concurrentMeasGapsPreMG-r18 nor concurrentMeasGapsNCSG-r18 nor supporting musim-GapPreference-r17 or when neither of the above configurations applies, i.e. concurrent measurement gaps, concurrent measurement gap(s) with Pre-MG(s), concurrent measurement gap(s) with NCSG(s), and periodic MUSIM gaps,

For FR1,

-	P=11-TSSBxRP, when in the monitored cell there are GAPs configured for intra-frequency, inter-frequency or inter-RAT measurements, which are overlapping with some but not all occasions of the SSB,

-	P = 1 when in the monitored cell there are no GAPs overlapping with any occasion of the SSB.

For FR2,

-	P=11-TSSBTSMTCperiod, when candidate beam detection RS is not overlapped with GAP and candidate beam detection RS is partially overlapped with SMTC occasion (TSSB < TSMTCperiod).

-	P is Psharing factor, when candidate beam detection RS is not overlapped with GAP and candidate beam detection RS is fully overlapped with SMTC occasion (TSSB = TSMTCperiod).

-	P=11-TSSBxRP - TSSBTSMTCperiod, when candidate beam detection RS is partially overlapped with GAP and candidate beam detection RS is partially overlapped with SMTC occasion (TSSB < TSMTCperiod) and SMTC occasion is not overlapped with GAP and

-	TSMTCperiod ≠ xRP or

-	TSMTCperiod = xRP and TSSB < 0.5 × TSMTCperiod

-	P=Psharing factor1-TSSBxRP, when candidate beam detection RS is partially overlapped with GAP and candidate beam detection RS is partially overlapped with SMTC occasion (TSSB < TSMTCperiod) and SMTC occasion is not overlapped with GAP and TSMTCperiod = xRP and TSSB = 0.5 × TSMTCperiod

-	P=11-TSSBMin(xRP,TSMTCperiod), when candidate beam detection RS is partially overlapped with GAP and candidate beam detection RS is partially overlapped with SMTC occasion (TSSB < TSMTCperiod) and SMTC occasion is partially or fully overlapped with GAP

-	P=Psharing factor1-TSSBxRP, when candidate beam detection RS is partially overlapped with GAP and candidate beam detection RS is fully overlapped with SMTC occasion (TSSB = TSMTCperiod) and SMTC occasion is partially overlapped with GAP (TSMTCperiod < xRP)

where,

-	Psharing factor = 1, if the CBD-RS resource outside gap is

-	not overlapped with the SSB symbols indicated by SSB-ToMeasure and 1 data symbol before each consecutive SSB symbols indicated by SSB-ToMeasure and 1 data symbol after each consecutive SSB symbols indicated by SSB-ToMeasure, given that SSB-ToMeasure is configured, where the SSB-ToMeasure is the union set of SSB-ToMeasure from all the configured measurement objects merged on the same serving carrier, and,

-	not overlapped by the RSSI symbols indicated by ss-RSSI-Measurement and 1 data symbol before each RSSI symbol indicated by ss-RSSI-Measurement and 1 data symbol after each RSSI symbol indicated by ss-RSSI-Measurement, given that ss-RSSI-Measurement is configured.

-	Psharing factor = 3, otherwise.

-	If the higher layer in TS 38.331 [2] signalling of smtc2 is present, TSMTCperiod follows smtc2; Otherwise TSMTCperiod follows smtc1. TSMTCperiod is the shortest SMTC period among all CCs in the same FR2 band, provided the SMTC offset of all CCs in FR2 have the same offset.

-	When a measurement gap is configured and the measurement gap is not NCSG,

-	a CBD-RS resource or an SMTC occasion is considered to be overlapped with the GAP if it overlaps a measurement gap occasion, and

-	xRP = MGRP

-	Otherwise, when NCSG is measurement gap only is configured,

-	a CBD-RS resource or an SMTC occasion is considered to be overlapped with the GAP if

-	it overlaps the VIL1 or VIL2 of NCSG, or

-	it overlaps the ML of NCSG in FR2, and there exists a target carrier to be measured within NCSG that is intra-frequency carrier or inter-frequency carrier in the same band as the serving cell, or inter-frequency carrier in different band as the serving cell and UE does not support IBM between the target carrier and the serving cell,

-	and

-	xRP = VIRP

-	If the UE is configured with Pre-MG only, an CBD-RS resource or an SMTC occasion is only considered to be overlapped by the Pre-MG if the Pre-MG is activated.

-	When concurrent gaps or concurrent measurement gap(s) with Pre-MG(s) or concurrent measurement gap(s) with NCSG(s) are configured, a CBD-RS resource or an SMTC occasion is not considered to be overlapped by a gap occasion if the gap occasion is dropped according to clause 9.1.8, clause 9.1.12, clause 9.1.13, respectively.

Longer evaluation period would be expected if the combination of the CBD-RS resource, SMTC occasion and GAP configurations does not meet previous conditions.

For either an FR1 or FR2 serving cell, longer evaluation period would be expected during the period Tidentify_CGI when the UE is requested to decode an NR CGI.

When the configured aperiodic MUSIM gap is overlapping with SSB resource occasion for candidate beam detection, longer evaluation period would be expected.

When UE is configured with MUSIM gap(s), and if SSB resource occasions for candidate beam detection are fully overlapped with MUSIM gap(s), or the union of MUSIM gap(s) and GAPs, no requirement applies for SSB based candidate beam detection.

For either an FR1 or FR2 serving cell, longer CBD evaluation period would be expected during the period Tidentify_CGI,E-UTRAN when the UE is requested to decode an LTE CGI.

The values of PCBD used in table 8.5.5.2-1 and table 8.5.5.2-2 are defined as

For each SSB resource in the set q1 configured for PCell or PSCell in EN-DC or NE-DC or SA; or PCell in NR-DC

-	PCBD = 1.

For each SSB resource in the set q1 configured for PSCell in NR-DC

-	PCBD = 2 if UE is configured for candidate beam detection on SCell, 1 otherwise.

For each SSB resource in the set q1 configured for a SCell

-	PCBD = Z in EN-DC or NE-DC or SA.

-	PCBD = 2* Z in NR-DC.

-	Where Z is the number of band(s) on which UE is performing candidate beam detection only for SCell

-	PCBD is the number of band(s) on which UE is performing candidate beam detection only for SCell.

Table 8.5.5.2-1: Evaluation period TEvaluate_CBD_SSB for FR1

| Configuration | TEvaluate_CBD_SSB (ms) |
| --- | --- |
| non-DRX, DRX cycle ≤ 320 ms | Max(25, Ceil(3 P PCBD) TSSB) |
| DRX cycle > 320 ms | Ceil(3 P PCBD) TDRX |
| NOTE: TSSB is the periodicity of SSB in the set q1 for UE not supporting On-demand SSB operation or when OD-SSB transmission is not indicated, otherwise, TSSB = EMP of SSB in the set q1. The EMP is the periodicity of OD-SSBs in the configured DL BWP if no first SSB transmission is configured, or if UE supports timec1nc2 and OD-SSB transmission is indicated with first SSB transmission and the union of the first SSB transmission and OD-SSB transmission has a non-periodic time domain pattern, or the periodicity of the union of OD-SSBs and first SSBs if first SSBs are transmitted and the OD-SSB activation is indicated and the union of OD-SSBs and first SSBs has a periodic time domain pattern. TDRX is the DRX cycle length. |  |  |

Table 8.5.5.2-2: Evaluation period TEvaluate_CBD_SSB for FR2

| Configuration | TEvaluate_CBD_SSB (ms) |
| --- | --- |
| non-DRX, DRX cycle ≤ 320 ms | Max(25, Ceil(3 P N PCBD) TSSB) |
| DRX cycle > 320 ms | Ceil(3 P N PCBD) TDRX |
| NOTE: TSSB is the periodicity of SSB in the set if UE not supporting On-demand SSB operation or when OD-SSB transmission is not indicated, otherwise, TSSB = EMP of SSB in the set q1. The EMP is the periodicity of OD-SSBs in the configured DL BWP if no first SSB transmission is configured, or if UE supports timec1nc2 and OD-SSB transmission is indicated with first SSB transmission and the union of the first SSB transmission and OD-SSB transmission has a non-periodic time domain pattern, or the periodicity of the union of OD-SSBs and first SSBs if first SSBs are transmitted and the OD-SSB activation is indicated and the union of OD-SSBs and first SSBs has a periodic time domain pattern. TDRX is the DRX cycle length. |  |  |

-------------------- END OF CHANGES--------------------------

-------------------------- CHANGE #8 ------------------------------

### 8.5.12	Minimum requirement at transitions for candidate beam detection

For UE supporting On-demand SSB operation, when UE is indicated, by a MAC-CE, adaptation of periodicities for the activated transmission of OD-SSBs when the SCell is activated, the candidate beam detection requirements are based on the EMP of adapted OD-SSB burst periodicity which applies from the first OD-SSB burst after the time point as defined in TS 38.213 for adaptation of the second SS/PBCH blocks transmissionplus additional processing time as 2ms or 5ms depending on OD-SSB post processing time.

For UE supporting SSB adaptation, when UE receives DCI format 2_9 that indicates a change in SSB burst periodicity of the SSB transmission during one evaluation period, the evaluation period requirements for candidate beam detection based on the indicated SSB burst periodicity applies from the first SSB burst after the time point as defined in TS 38.213 plus 2 ms and 3 ms for FR1 and FR2 respectively.

-------------------- END OF CHANGES--------------------------

-------------------------- CHANGE #5 ------------------------------

#### 9.5.4.1	SSB based L1-RSRP Reporting

When groupBasedBeamReporting-r17 is not configured, the UE shall be capable of performing L1-RSRP measurements based on the configured SSB resource for L1-RSRP computation, and the UE physical layer shall be capable of reporting L1-RSRP measured over the measurement period of TL1-RSRP_Measurement_Period_SSB.

When groupBasedBeamReporting-r17 is configured, the UE shall be capable of performing L1-RSRP measurements based on the two configured SSB resource sets for L1-RSRP, and the UE physical layer shall be capable of reporting group-based L1-RSRP measured over the measurement period of TL1-RSRP_Measurement_Period_SSB.

When there is no L1-RSRP measurement on LTM neighbor cell(s) to measure within active BWP, the value of TL1-RSRP_Measurement_Period_SSB is defined in table 9.5.4.1-1 for FR1, the value of TL1-RSRP_Measurement_Period_SSB is defined in table 9.5.4.1-2 for FR2 when highSpeedMeasFlagFR2-r17 is not configured and if groupBasedBeamReporting-r17 is not configured, and defined in table 9.5.4.1-2A for FR2-1 when highSpeedMeasFlagFR2-r17 is not configured and if groupBasedBeamReporting-r17 is configured, and defined in table 9.5.4.1-3 for FR2 power class 6 UE when highSpeedMeasFlagFR2-r17 is configured, where

-	M=1 if higher layer parameter timeRestrictionForChannelMeasurement is configured, and M=3 otherwise

-	The value of N in table 9.5.4.1-2A is 2, 4 or 6 in FR2-1 for UE supporting fastBeamSweepingMultiRx-r18 according to the conditions described in clause 3.6.19.

-	The value of N in table 9.5.4.1-2 is 8.

-	The value of N in table 9.5.4.1-3 is 8.

For UE supporting On-demand SSB operation and when OD-SSB transmission is indicated, the EMP for L1-RSRP are

-	the periodicity of OD-SSBs in the configured DL BWP if no first SSB transmission is configured or if UE supports timec1nc2 and OD-SSB transmission is indicated with first SSB transmission and the union of the first SSB transmission and OD-SSB transmission has a non-periodic time domain pattern, or

-	the periodicity of the union of OD-SSBs and first SSBs if first SSBs are transmitted and the OD-SSB activation is indicated and the union of OD-SSBs and first SSBs has a periodic time domain pattern.

For a UE supporting LB CA via switching, or for a UE supporting concurrentMeasGapsPreMG-r18 and when concurrent measurement gap(s) with Pre-MG(s) are configured, or a UE supporting concurrentMeasGapsNCSG-r18 and when concurrent measurement gap(s) with NCSG measurement gap(s) are configured, or a UE supporting concurrentMeasGap-r17 or musim-GapPreference-r17 or both concurrent GAP and musim-GapPreference-r17 and when concurrent gaps or periodic MUSIM gaps or both concurrent gaps and periodic MUSIM gaps are configured,

-	an SSB or an SMTC occasion is not considered to be overlapped by a gap occasion if the gap occasion is dropped according to clause 9.1.8 and 9.1.10,

-	P value for SSB resource to be measured is defined as

-	Ntotal / Noutside_MG in FR1

-	Psharing factor * Ntotal / Noutside_MG in FR2 with Navailable = 0

-	Ntotal / Navailable in FR2 with Navailable > 0

-	For a window W of duration max(TL1,  xRP_max, switching pattern periodicity), where xRP_max is the maximum xRP across all configured per-UE GAPs , periodic MUSIM gap(s) and per-FR GAPs, and, in case of Pre-MG, all activated per-UE measurement gaps and per-FR measurement gaps, within the same FR as serving cell, and starting at the beginning of any SSB resource occasion:

Ntotal is the total number of SSB resource occasions within the window, including those overlapped with GAP occasions, MUSIM gap occasions or SMTC occasions within the window.

For UEs supporting LB CA via switching and configured with low NR inter-band carrier aggregation:

-	for the PCell L1-RSRP measurement, Ntotal also includes SSB resource occasions that overlap with the SDL SCell ON duration within the window,

-	for the SCell L1-RSRP measurement, Ntotal also includes SSB resource occasions that overlap with the PCell ON duration within the window, as defined by the configured switching pattern, and

-	Noutside_MG is the number of SSB resource occasions within the window W

-	that are not overlapped with any non-dropped GAP occasion nor non-dropped MUSIM gap occasion or

-	that are not overlapped with SDL SCell ON duration corresponding to the LB CA switching pattern, for the PCell L1-RSRP measurement for a UE supporting LB CA via switching with low NR inter-band carrier aggregation configured or

-	that are not overlapped with PCell ON duration corresponding to the LB CA switching pattern, for the SCell L1-RSRP measurement for a UE supporting LB CA via switching with low NR inter-band carrier aggregation configured, and

-	L1-RSRP requirement in this clause is not applied when Noutside_MG = 0.

-	Navailable is the number of SSB resource occasions that are not overlapped with any non-dropped GAP occasion, non-dropped MUSIM gap occasion nor any SMTC occasion within the window W.

-	for UEs supporting LB CA via switching, switching pattern periodicity is the periodicity of the RRC configured semi-static switching pattern; otherwise, it is not applicable.

-	an SSB or an SMTC occasion is considered to be overlapped with the MUSIM gap if it overlaps a MUSIM gap occasion.

-	TL1 is periodicity of the target SSB.

-	xRP = MGRP when configured GAP is activated Pre-MG or MG, and xRP = VIRP when configured GAP is NCSG.

Otherwise, for a UE neither supporting concurrentMeasGap-r17 nor concurrentMeasGapsPreMG-r18 nor concurrentMeasGapsNCSG-r18 or when neither of the above configurations applies, i.e. concurrent measurement gaps, concurrent measurement gap(s) with Pre-MG(s) and concurrent measurement gap(s) with NCSG(s) and UE does not support musim-GapPreference-r17 or when no MUSIM gaps are configured,

For FR1,

-	P=11-TSSBxRP, when in the monitored cell there are GAPs configured for intra-frequency, inter-frequency or inter-RAT measurements, which are overlapping with some but not all occasions of the SSB; and

-	P=1 when in the monitored cell there are no GAPs overlapping with any occasion of the SSB.

For FR2,

P1= 11-TSSBTSMTCperiod, when SSB is not overlapped with measurement gap and SSB is partially overlapped with SMTC occasion (TSSB < TSMTCperiod).

-	P1 is PL1_sharing*Psharing factor, when SSB is not overlapped with measurement gap and SSB is fully overlapped with SMTC occasion (TSSB = TSMTCperiod).

-	P1=11-TSSBxRP-TSSBTSMTCperiod, when SSB is partially overlapped with GAP and SSB is partially overlapped with SMTC occasion (TSSB < TSMTCperiod) and SMTC occasion is not overlapped with GAP and

-	TSMTCperiod ≠ xRP or

-	TSMTCperiod = xRP and TSSB < 0.5*TSMTCperiod

-	P is Psharing factor1-TSSBxRP, when SSB is partially overlapped with GAP and SSB is partially overlapped with SMTC occasion (TSSB < TSMTCperiod) and SMTC occasion is not overlapped with GAP and TSMTCperiod = xRP and TSSB = 0.5*TSMTCperiod

-	P1= 11-TSSBmin(TSMTCperiod,xRP), when SSB is partially overlapped with GAP (TSSB < xRP) and SSB is partially overlapped with SMTC occasion (TSSB < TSMTCperiod) and SMTC occasion is partially or fully overlapped with GAP.

-	P is  PL1_sharing*Psharing factor1-TSSBxRP, when SSB is partially overlapped with measurement gap and SSB is fully overlapped with SMTC occasion (TSSB = TSMTCperiod) and SMTC occasion is partially overlapped with GAP (TSMTCperiod < xRP)

-	If SSB resource from the cell with different PCI is configured for L1-RSRP measurement, and P2 is valid accoding to clause 9.13.4.1, and any symbol of the SSBs from serving cell and cell with different PCI are overlapping or adjacent (in time domain)

-	P = P11-P1*TSSBP2*TSSB_CDP ,   if P1*TSSB < P2*TSSB_CDP.

-	P = P1, if P1*TSSB > P2*TSSB_CDP.

-	P = 2*P1, if P1*TSSB = P2*TSSB_CDP.

-	Otherwise, P = P1

Where:

-	TSSB = ssb-periodicityServingCell of the serving cell

-	TSMTCperiod = the configured SMTC period

-	TSSB_CDP = SSB periodicity of the cell with PCI different from serving cell

-	Psharing factor = 1, if the SSB configured for L1-RSRP measurement outside gap is

-	not overlapped with the SSB symbols indicated by SSB-ToMeasure and 1 data symbol before each consecutive SSB symbols indicated by SSB-ToMeasure and 1 data symbol after each consecutive SSB symbols indicated by SSB-ToMeasure, given that SSB-ToMeasure is configured, where the SSB-ToMeasure is the union set of SSB-ToMeasure from all the configured measurement objects merged on the same serving carrier, and,

-	not overlapped by the RSSI symbols indicated by ss-RSSI-Measurement and 1 data symbol before each RSSI symbol indicated by ss-RSSI-Measurement and 1 data symbol after each RSSI symbol indicated by ss-RSSI-Measurement, given that ss-RSSI-Measurement is configured.

-	Psharing factor = 3, otherwise.

-	PL1_sharing = 2, if SSB resource from the cell with different PCI is configured for L1-RSRP measurement, and Psharing_factor,CDP is used in clause 9.13.4.1, and any symbol of the SSBs from serving cell and cell with different PCI are overlapping or adjacent (in time domain). PL1_sharing = 1, otherwise.

-	TSSB = ssb-periodicityServingCell

-	TSMTCperiod = the configured SMTC period

-	When a measurement gap is configured and the measurement gap is not NCSG,

-	an SSB or an SMTC occasion is considered to be overlapped with the GAP if it overlaps a measurement gap occasion, and

-	xRP = MGRP

-	If the UE is configured with Pre-MG, an SSB or an SMTC occasion is only considered to be overlapped by the Pre-MG if the Pre-MG is activated.

-	Otherwise, when NCSG measurement gap only is configured,

-	an SSB or an SMTC occasion is considered to be overlapped with the GAP if

-	it overlaps the VIL1 or VIL2 of NCSG, or

-	it overlaps the ML of NCSG in FR2, and there exists a target carrier to be measured within NCSG that is intra-frequency carrier or inter-frequency carrier in the same band as the serving cell, or inter-frequency carrier in different band as the serving cell and UE does not support IBM between the target carrier and the serving cell,

-	and

-	xRP = VIRP

The value of TL1-RSRP_Measurement_Period_SSB is defined in table 9.5.4.1-4 for UE incapable of multiCellL1-measRTD-greaterThan-CP-r18 in FR1, table 9.5.4.1-5 for UE capable of multiCellL1-measRTD-greaterThan-CP-r18 in FR1, table 9.5.4.1-6 for UE incapable of multiCellL1-measRTD-greaterThan-CP-r18 in FR2 and table 9.5.4.1-7 for UE capable of multiCellL1-measRTD-greaterThan-CP-r18 in FR2 when there is L1-RSRP measurement on LTM neighbor cell(s) to measure within active BWP, where

-	M=1 if higher layer parameter timeRestrictionForChannelMeasurement is configured, and M=3 otherwise

-	N= 8 in table 9.5.4.1-6 and table 9.5.4.1-7.

-	P value for SSB resource to be measured is defined as

-	Ntotal / Noutside_MG in FR1

-	Psharing factor * Ntotal / Noutside_MG in FR2 with Navailable = 0

-	Ntotal / Navailable in FR2 with Navailable > 0

For a window W of duration max (TL1,  MGRP_max), where MGRP max is the maximum MGRP across all configured per-UE measurement gaps and per-FR measurement gaps within the same FR as serving cell, and starting at the beginning of any SSB resource occasion:

-	Ntotal is the total number of SSB resource occasions within the window, including those overlapped with measurement gap occasions or SMTC occasions within the window, and

-	Noutside_MG is the number of SSB resource occasions that are not overlapped with any measurement gap occasion within the window W

-	Navailable is the number of SSB resource occasions that are not overlapped with any measurement gap occasion nor any SMTC occasion within the window W

-	TL1 is periodicity of the target SSB.

-	Psharing factor = 1, if the SSB configured for L1-RSRP measurement outside measurement gap is

-	not overlapped with the SSB symbols indicated by SSB-ToMeasure and 1 data symbol before each consecutive SSB symbols indicated by SSB-ToMeasure and 1 data symbol after each consecutive SSB symbols indicated by SSB-ToMeasure, given that SSB-ToMeasure is configured, where the SSB-ToMeasure is the union set of SSB-ToMeasure from all the configured measurement objects merged on the same serving carrier, and,

-	not overlapped with the RSSI symbols indicated by ss-RSSI-Measurement and 1data symbol before each RSSI symbol indicated by ss-RSSI-Measurement and 1 data symbol after each RSSI symbol indicated by ss-RSSI-Measurement, given that ss-RSSI-Measurement is configured,

-	Psharing factor = 3, otherwise.

-	PL1_sharing is defined as

- 	When number of neighboring cells configured with SSB based L1-RSRP measurement is 1

-	PL1_sharing = 2, if any symbol of the SSBs from serving cell and neighbor cell are overlapping or adjacent (in time domain)

-	PL1_sharing = 1, otherwise

-	When number of neighboring cells configured with SSB based L1-RSRP measurement is more than 1

-	PL1_sharing = 3.

Note:	RRM requirements are not applicable when L1 measurement on the cells with different PCI and LTM L1 measurement on neighbour cells are both configured.

If the high layer in TS 38.331 [2] signaling of smtc2 is configured, TSMTCperiod corresponds to the value of higher layer parameter smtc2; Otherwise TSMTCperiod corresponds to the value of higher layer parameter smtc1. TSMTCperiod is the shortest SMTC period among all CCs in the same FR2 band, provided the SMTC offset of all CCs in FR2 have the same offset.

For UE supporting measurement gap occasion cancellation, the UE is not required to perform SSB measurements during the cancelled gap occasions.

Longer evaluation period would be expected if the combination of SSB, SMTC occasion and GAP configurations does not meet previous conditions.

When UE is configured with aperiodic MUSIM gap and the aperiodic MUSIM gap is overlapping with SSB resource occasion for L1-RSRP, longer evaluation period would be expected.

When UE is configured with MUSIM gap(s), and SSB resource occasions for L1-RSRP are fully overlapped with MUSIM gap(s) or fully overlapped with the union of MUSIM gap(s) and GAPs, no requirement applies for the SSB based L1-RSRP measurement.

For either an FR1 or FR2 serving cell, longer evaluation period would be expected during the period Tidentify_CGI when the UE is requested to decode an NR CGI.

For either an FR1 or FR2 serving cell, longer L1 RSRP measurement period would be expected during the period Tidentify_CGI,E-UTRAN when the UE is requested to decode an LTE CGI.

Table 9.5.4.1-1: Measurement period TL1-RSRP_Measurement_Period_SSB for FR1

| Configuration | TL1-RSRP_Measurement_Period_SSB (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P)*TSSB) |
| DRX cycle ≤ 320 ms | max(TReport, ceil(K *M*P)*max(TDRX,TSSB)) |
| DRX cycle > 320 ms | ceil(M*P)*TDRX |
| Note 1: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-RSRP measurement if UE not supporting On-demand SSB operation or when OD-SSB transmission is not indicated, otherwise, TSSB = EMP of the SSB index configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. Note 2: K = 1 when TSSB ≤ 40 ms and highSpeedMeasFlag-r16 or highSpeedMeasCA-Scell-r17 are configured; otherwise, K = 1.5. Note 3: When highSpeedMeasFlag-r16 is configured, the requirements apply only to UE supporting either measurementEnhancement-r16 or intraNR-MeasurementEnhancement-r16. or measurementEnhancementCA-r17 |  |

Table 9.5.4.1-2: Measurement period TL1-RSRP_Measurement_Period_SSB for FR2

| Configuration | TL1-RSRP_Measurement_Period_SSB (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P*N)*TSSB) |
| DRX cycle ≤ 320 ms | max(TReport, ceil(1.5*M*P*N)*max(TDRX,TSSB)) |
| DRX cycle > 320 ms | ceil(1.5*M*P*N)*TDRX |
| Note: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-RSRP measurement if UE not supporting On-demand SSB operation or when OD-SSB transmission is not indicated, otherwise, TSSB = EMP of the SSB index configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. |  |

Table 9.5.4.1-2A: Measurement period TL1-RSRP_Measurement_Period_SSB configured with groupBasedBeamReporting-r17 for FR2-1

| Configuration | TL1-RSRP_Measurement_Period_SSB (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P*N)*TSSB) |
| DRX cycle ≤ 320 ms | max(TReport, ceil(1.5*M*P*N)*max(TDRX,TSSB)) |
| DRX cycle > 320 ms | ceil(1.5*M*P*N)*TDRX |
| Note1: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-RSRP measurement if UE not supporting On-demand SSB operation or when OD-SSB transmission is not indicated, otherwise, TSSB = EMP of the SSB index configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. |  |

Table 9.5.4.1-3: Measurement period TL1-RSRP_Measurement_Period_SSB configured with highSpeedMeasFlagFR2-r17 for FR2-1

| Configuration | TL1-RSRP_Measurement_Period_SSB (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P*N1Note2)*TSSB) |
| DRX cycle ≤ 80 ms | max(TReport, ceil(M*P*N1Note2*M2)*max(TDRX,TSSB)) |
| 80 ms< DRX ≤ 320 ms | max(TReport, ceil(1.5*M*P*N)*max(TDRX,TSSB)) |
| DRX cycle > 320 ms | ceil(1.5*M*P*N)*TDRX |
| Note1: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. Note 2: Scaling factor N1 = 2 when highSpeedMeasFlagFR2-r17 = set1 or scaling factor N1 = 6 when highSpeedMeasFlagFR2-r17 = set2, if UE does not support simultaneousReceptionTwoQCL-r18 or when highSpeedDeploymentTypeFR2-r17 is not configured as bidirectional. Scaling factor N1 = 1.5 when highSpeedMeasFlagFR2-r17 is configured to set1 or scaling factor N1 = 4 when highSpeedMeasFlagFR2-r17 is configured to set2, if UE supports simultaneousReceptionTwoQCL-r18 and when highSpeedDeploymentTypeFR2-r17 is configured as bidirectional. Note 3: M2 = 1.5 if SMTC periodicity > 40 ms; otherwise M2 = 1 |  |

Table 9.5.4.1-4: Measurement period TL1-RSRP_Measurement_Period_SSB in FR1 for UE incapable of multiCellL1-measRTD-greaterThan-CP-r18

| Configuration | TL1-RSRP_Measurement_Period_SSB_intra (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P)*TSSB) |
| DRX cycle ≤ 320 ms | max(TReport, ceil(K *M*P)*max(TDRX,TSSB)) |
| DRX cycle > 320 ms | ceil(M*P)*TDRX |
| Note 1: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. Note 2: K = 1.5. Note 3: The requirements apply if the actual RTD between serving cell and neighbor cell is not larger than CP. |  |

Table 9.5.4.1-5: Measurement period TL1-RSRP_Measurement_Period_SSB in FR1 for UE capable of multiCellL1-measRTD-greaterThan-CP-r18

| Configuration | TL1-RSRP_Measurement_Period_SSB_intra (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P)*TSSB) |
| DRX cycle ≤ 320 ms | max(TReport, ceil(K *M*P)*max(TDRX,TSSB)) |
| DRX cycle > 320 ms | ceil(M*P)*TDRX |
| Note 1: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. Note 2: K = 1.5. |  |

Table 9.5.4.1-6: Measurement period TL1-RSRP_Measurement_Period_SSB in FR2 for UE incapable of multiCellL1-measRTD-greaterThan-CP-r18

| Configuration | TL1-RSRP_Measurement_Period_SSB_intra (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P*PL1_sharing*N)*TSSB) |
| DRX cycle ≤ 320 ms | max(TReport, ceil(1.5*M*P*PL1_sharing *N)*max(TDRX,TSSB) ) |
| DRX cycle > 320 ms | ceil(1.5*M*P*PL1_sharing*N)*TDRX |
| Note 1: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. Note 2: The requirements apply if the actual RTD between serving cell and neighbor cell is not larger than CP. |  |

Table 9.5.4.1-7: Measurement period TL1-RSRP_Measurement_Period_SSB in FR2 for UE capable of multiCellL1-measRTD-greaterThan-CP-r18

| Configuration | TL1-RSRP_Measurement_Period_SSB_intra (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P*PL1_sharing*N)*TSSB) |
| DRX cycle ≤ 320 ms | max(TReport, ceil(1.5*M*P*PL1_sharing*N)*max(TDRX,TSSB)) |
| DRX cycle > 320 ms | ceil(1.5*M*P*PL1_sharing*N)*TDRX |
| NOTE 1: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. |  |

-------------------- END OF CHANGES--------------------------

-------------------------- CHANGE #6 ------------------------------

#### 9.8.4.2	L1-SINR reporting with SSB based CMR and dedicated IMR configured

The UE shall be capable of performing L1-SINR measurements with the SSB configured as CMR and dedicated resource configured as IMR for L1-SINR computation, in which the NZP-CSI-RS or CSI-IM resource configured as dedicated IMR shall be 1-to-1 mapped to SSB configured as CMR, with the same periodicity. The UE physical layer shall be capable of reporting L1-SINR measured over the measurement period of TL1-SINR_Measurement_Period_SSB_CMR_IMR.

The requirements in this clause are not applicable if NZP-CSI-RS or CSI-IM resource configured as dedicated IMR is scheduled with different periodicity as SSB configured as CMR.

For UE supporting On-demand SSB operation and when OD- SSB transmission is indicated, the EMP for L1-SINR are

-	the periodicity of OD-SSBs in the configured DL BWP if no first SSB transmission is configured or if UE supports timec1nc2 and OD-SSB transmission is indicated with first SSB transmission and the union of the first SSB transmission and OD-SSB transmission has a non-periodic time domain pattern, or

-	the periodicity of the union of OD-SSBs and first SSBs if first SSBs are transmitted and the OD-SSB activation is indicated and the union of OD-SSBs and first SSBs has a periodic time domain pattern.

The value of TL1-SINR_Measurement_Period_SSB_CMR_IMR is defined in table 9.8.4.2-1 for FR1 and in table 9.8.4.2-2 for FR2 when highSpeedMeasFlagFR2-r17 is not configured, and defined in table 9.8.4.2-3 for FR2 power class 6 UE when highSpeedMeasFlagFR2-r17 is configured, where

For the value of M

-	For periodic or semi-persistent NZP CSI-RS or CSI-IM resource as dedicated IMR, M=1 if the higher layer parameters timeRestrictionForChannelMeasurements and/or timeRestrictionForInterferenceMeasurements are configured, and M=3 otherwise;

For the value of N in FR2

-	N = 2, 4 or 6 in FR2-1 for UE supporting fastBeamSweepingMultiRx-r18, according to the conditions described in clause 3.6.19,

-	N = 8 otherwise.

For the value of L1,

1>	If UE does not support supportSBFD or SBFD is not configured by the network

2>	L1=0

1>	else (if UE supports supportSBFD and SBFD is configured by the network)

2>	if higher layer parameter timeRestrictionForChannelMeasurement is configured

3>	L1=0

2>	else (if higher layer parameter timeRestrictionForChannelMeasurement is not configured)

3>	if UE is configured to report L1-SINR for SBFD symbols

4>		When DRX is not configured, L1 is the number of occasions of the CSI-RS/CSI-IM resource as IMR that are overlapping with dynamic UL transmission or with non-SBFD symbols during TL1-SINR_Measurement_Period_SSB_CMR_IMR

4>		When DRX is configured, L1 is the number of DRX cycles in which at least one occasion of the CSI-RS/CSI-IM resource as IMR is overlapping with dynamic UL transmission or with non-SBFD symbols during TL1-SINR_Measurement_Period_SSB_CMR_IMR

3>	if UE is configured to report L1-SINR for non-SBFD symbols

4>	When DRX is not configured, L1 is the number of occasions of the CSI-RS/CSI-IM resource as IMR that are overlapping with non-SBFD symbols, during TL1-SINR_Measurement_Period_SSB_CMR_IMR

4>	When DRX is configured, L1 is the number of DRX cycles in which at least one occasion of the CSI-RS/CSI-IM resource as IMR is overlapping with non-SBFD symbols, during TL1-SINR_Measurement_Period_SSB_CMR_IMR

P is defined as the maximum value between PCMR and PIMR, i.e., P = max(PCMR, PIMR), where

-	the value of PCMR shall be derived in the same way as the value of P used for SSB based L1-RSRP measurement in clause 9.5.4.1, in which the occasions and period of the SSB for CMR shall be used instead.

-	the value of PIMR shall be derived in the same way as the value of P used for CSI-RS based L1-RSRP measurement in clause 9.5.4.2, in which the occasions and period of the NZP CSI-RS for NZP-IMR or CSI-IM for ZP-IMR shall be used instead.

For UE supporting measurement gap occasion cancellation, the UE is not required to perform SSB measurements during the cancelled gap occasions.

Longer evaluation period would be expected if the combination of SSB, SMTC occasion and measurement gap configurations does not meet previous conditions.

For L1-SINR measurement with SSB as CMR and CSI-RS or CSI-IM as IMR, the requirement shall apply if the CSI-RS is configured as IMR with repetition field as “repetition = OFF” or CSI-IM is configured as IMR.

For L1-SINR measurement with SSB as CMR and CSI-RS/CSI-IM as IMR, no requirement shall apply if SSB occasions for CMR or CSI-RS/CSI-IM occasions for IMR are fully overlapped with the configured measurement gap

Table 9.8.4.2-1: Measurement period TL1-SINR_Measurement_Period_SSB_CMR_IMR for FR1

| Configuration | TL1-SINR_Measurement_Period_SSB_CMR_IMR (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil((M+L1)*P)* max(TSSB, Tproc)) |
| DRX cycle ≤ 320 ms | max(TReport, ceil(1.5*(M+L1)*P)*max(TDRX,TSSB, Tproc)) |
| DRX cycle > 320 ms | ceil((M+L1)*P)*TDRX |
| NOTE 1: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-SINR channel measurement if UE not supporting On-demand SSB operation or when OD-SSB transmission is not indicated, otherwise, TSSB = EMP of the SSB index configured for L1-SINR channel measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. NOTE 2: The requirements are applicable provided that the CSI-RS resource configured for interference measurement shall be 1-to-1 mapped to SSB configured for channel measurement, with the same periodicity. NOTE 3: If UE indicates needForScaledCSIProcTimeDualDL and the CSI-RS resource for IMR is across 2 DL subbands, Tproc = 8ms; otherwise Tproc = 0. |  |

Table 9.8.4.2-2: Measurement period TL1-SINR_Measurement_Period_SSB_CMR_IMR for FR2

| Configuration | TL1-SINR_Measurement_Period_SSB_CMR_IMR (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil((M+L1)*P*N)* max(TSSB, Tproc)) |
| DRX cycle ≤ 320 ms | max(TReport, ceil(1.5*(M+L1)*P*N)*max(TDRX,TSSB, Tproc)) |
| DRX cycle > 320 ms | ceil(1.5*(M+L1)*P*N)*TDRX |
| NOTE 1: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-SINR measurement if UE not supporting On-demand SSB operation or when OD-SSB transmission is not indicated, otherwise, TSSB = EMP of the SSB index configured for L1-SINR channel measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. NOTE 2: The requirements are applicable provided that the CSI-RS resource configured for interference measurement shall be 1-to-1 mapped to SSB configured for channel measurement, with the same periodicity. NOTE 3: If UE indicates needForScaledCSIProcTimeDualDL and the CSI-RS resource for IMR is across 2 DL subbands, Tproc = 8ms; otherwise Tproc = 0. |  |

Table 9.8.4.2-3: Measurement period TL1-SINR_Measurement_Period_SSB_CMR_IMR configured with highSpeedMeasFlagFR2-r17 for FR2-1

| Configuration | TL1-SINR_Measurement_Period_SSB_CMR_IMR (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil((M+L1)*P*N1Note 3)* max(TSSB, Tproc)) |
| DRX cycle ≤ 80 ms | max(TReport, ceil((M+L1)*P*N1Note 3*M2)*max(TDRX,TSSB, Tproc)) |
| 80 ms< DRX cycle ≤ 320 ms | max(TReport, ceil(1.5*(M+L1)*P*N)*max(TDRX,TSSB)) |
| DRX cycle > 320 ms | ceil(1.5*(M+L1)*P*N)*TDRX |
| NOTE 1: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-SINR measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. NOTE 2: The requirements are applicable provided that the CSI-RS resource configured for interference measurement shall be 1-to-1 mapped to SSB configured for channel measurement, with the same periodicity. NOTE 3: N1 = 2 when highSpeedMeasFlagFR2-r17 = set1; N1 = 6 when highSpeedMeasFlagFR2-r17 = set2. NOTE 4: M2 = 1.5 if SMTC periodicity > 40 ms; otherwise M2 = 1 NOTE 5: If UE indicates needForScaledCSIProcTimeDualDL and the CSI-RS resource for IMR is across 2 DL subbands, Tproc = 8ms; otherwise Tproc = 0. |  |

-------------------- END OF CHANGES--------------------------

-------------------------- CHANGE #7 ------------------------------

### 9.8.7	Minimum requirement at transitions

For UE supporting ncd-SSB-BWP-Wor-r18, when the L1-SINR measurement transitions from measurements performed on CD-SSB to measurements performed on NCD-SSB or vice versa due to BWP switching during one measurement period, the UE shall use a L1-SINR measurement period that is the maximum of the L1-SINR measurement period corresponding to the first SSB type and the second SSB type after the BWP switching.

Subsequent to the above-mentioned measurement periods, the L1-SINR measurement period requirements are based on the SSB type after the BWP switch.

For UE supporting OD-SSB operation, when UE is indicated by an OD-SSB parameter update MAC-CE, adaptation of periodicities for the activated transmission of OD-SSBs when the SCell is activated, the L1-RSRP measurement period requirements are based on the EMP of adapted OD-SSB burst periodicity which applies from the first OD-SSB burst after the time point as defined in TS 38.213 for adaptation of the second SS/PBCH blocks transmission plus additional processing time of OD-SSB parameter update capability.

For UE supporting SSB adaptation, when UE receives DCI format 2_9 that indicates a change in SSB burst periodicity of the SSB transmission, the L1-SINR measurement period requirements based on the indicated SSB burst periodicity apply from the first SSB burst after the time point as defined in TS 38.213 plus 2 ms and 3 ms for FR1 and FR2, respectively.

-------------------- END OF CHANGES--------------------------

-------------------------- CHANGE #7 ------------------------------

### 9.17.1	Introduction

A UE can be indicated, by od-ssb-config TS 38.331[2] or by a MAC CE TS 38.321[7], activation of transmission for OD-SSB in a configured DL BWP of an SCell TS 38.300[10] if UE supports On-demand SSB operation.

The requirements in clause 9.17 apply, provided the following conditions are met:

-	an OD-SSB transmission is activated in a configured DL BWP where a first SSB transmission is not configured, or

-	an OD-SSB transmission is activated in a configured DL BWP where a first SSB transmission is configured which cannot be used to obtain SIB1.

Where the first SSB transmission is specified as in TS 38.213[3] have the following relations:

-	are with the same Cell ID, and

-	are in the same frequency location, and

-	are with same sub-carrier spacing, and

-	are with the same downlink transmit power, and

-	are quasi co-located with respect to Doppler shift, Doppler spread, average gain, average delay, delay spread and, when applicable, spatial RX parameters, when they have a same SSB index.

For UE supporting On-demand SSB operation and when OD- SSB transmission is indicated, the EMP for OD-SSB based intra-frequency measurement are

-	the periodicity of OD-SSBs in the configured DL BWP if no first SSB transmission is configured or if UE supports timec1nc2 and OD-SSB transmission is indicated with first SSB transmission and the union of the first SSB transmission and OD-SSB transmission has a non-periodic time domain pattern, or

-	the periodicity of the union of OD-SSBs and first SSBs if first SSBs are transmitted and the OD-SSB activation is indicated and the union of OD-SSBs and first SSBs has a periodic time domain pattern.

For UE supports OD-SSB operation capability and when OD-SSB activation is indicated,

-	the SMTC for OD-SSB based intra-frequency measurement for serving cell is OD-SMTC;

-	the SMTC for OD-SSB based intra-frequency measurement for neighbor cell is the first SMTC.

Otherwise, the SMTC for OD-SSB based intra-frequency measurement is the first SMTC.

-------------------- END OF CHANGES--------------------------

-------------------------- CHANGE #9 ------------------------------

#### 9.17.5.3	Intra-frequency cell identification for deactivated SCell

When UE is indicated the activation of OD-SSB transmission in slot n in a configured DL BWP of an SCell without a first SSB transmission before SCell activation command,

-	After slot n+THARQ+3ms+TprocessingNR slot length, UE shall perform the deactivated SCell measurement with TPSS/SSS_sync_intra_ODSSB, TSSB_time_index_intra_ODSSB, TSSB_measurement_period_intra_ODSSB given in table 9.17.5.3-1, 9.17.5.3-2 and 9.17.5.4-1 regardless of configured DRX and measCycleSCell till slot n+THARQ+3ms+Tprocessing+TFMWNR slot length, where:

-	TFMW equals min(Treport, TFirst, OD-SSB+ Tidentify_intra), which is the duration of the fast measurement window(FMW).

-	TFirst, OD-SSB is the time to the end of the first complete transmitted SSB burst after slot n + THARQ+3ms+TprocessingNR slot length.

-	Tidentify_intra equals Tidentify_intra_without_index_ODSSB or Tidentify_intra_with_index_ODSSB if the cell is a newly detectable cell or TSSB_measurement_period_intra_ODSSB if the cell has been detected for a period less than 5 seconds.

-	Tprocessing is the additional processing time as 2ms or 5ms depending on OD-SSB post processing time.

-	Treport is the duration after slot n+THARQ+3ms+TprocessingNR slot length to the earliest OD-SSB based L3 measurement reporting.

-	CSSFintra: it is a carrier specific scaling factor and is determined	according to CSSFoutside_gap,i in clause 9.1.5.1 for measurement conducted outside measurement gaps. When single OD-SSB carrier is configured with multiple SCCs without OD-SSB,  CSSFintra equals 1 for OD-SSB carrier during the FMW.

-	Kp_OD-SSB is the scaling factor for an OD-SSB based deactivated SCell to be measured. Kp_OD-SSB = Ntotal / Navailable, where Navailable and Ntotal are calculated as follows:

-	For a window W of duration max(SMTC period,  xRP_max), where xRP_max is the maximum xRP across all configured per-UE GAPs, periodic MUSIM gaps, and/or per-FR GAPs within the same FR as the OD-SSB frequency layer, and starting from the beginning of any OD-SSB occasion:

-	Ntotal is the total number of OD-SSB occasions within the window, including those overlapped with GAP and MUSIM gap occasions within the window, and

-	Navailable is the number of OD-SSB occasions that are not overlapped with any non-dropped GAP or non-dropped MUSIM gap occasions within the window W, after accounting for measurement GAP and MUSIM gap collisions by applying the collision rules for GAP and MUSIM gap in clauses 9.1.8.3, 9.1.10.4, 9.1.10.5, 9.1.12.3, and 9.1.13.3, respectively.

Kp_OD-SSB = 1 when Navailable = 0.

-	xRP = MGRP when configured GAP is activated Pre-MG or MG, and xRP = VIRP when configured GAP is NCSG, also xRP = MGRP for periodic MUSIM gap.

Requirements in this clause do not apply when Navailable = 0 due to fully overlapping between SMTC occasions and MUSIM gap occasions within the window W.

Otherwise, when the UE is not configured with or UE does not support concurrent GAPs and the UE is not configured with periodic MUSIM gaps or UE does not support MUSIM gaps:

When OD-SSB is fully non overlapping with measurement gaps or NCSG, Kp_OD-SSB =1;

When OD-SSB is partially overlapping with measurement gaps, Kp_OD-SSB = 1/(1- (OD-SSB period /MGRP)), where OD-SSB period < MGRP. When OD-SSB is partially overlapping with NCSG, Kp_OD-SSB = 1/(1- (OD-SSB period /VIRP)), where OD-SSB period < VIRP.

- When UE is indicated the activation of another OD-SSB transmission in a configured DL BWP of an SCell within the T2 after the previous OD-SSB deactivation and before the SCell activation command, the UE shall perform the deactivated SCell measurement with TPSS/SSS_sync_intra_ODSSB, TSSB_time_index_intra_ODSSB, TSSB_measurement_period_intra_ODSSB given in table 9.17.5.3-3, 9.17.5.3-4, 9.17.5.3-5, 9.17.5.4-2 and 9.17.5.4-3 if OD-SSB is transmitted, where T2 is 5*max (measCycleSCell, DRX cycles) in FR1 and 4s for power class 1/5 and 3s for power class 2/3/4 in FR2.

- When UE is indicated the activation of another OD-SSB transmission in a configured DL BWP of an SCell after previous OD-SSB deactivation plus T2 and before the SCell activation command, UE shall perform the deactivated SCell measurement with TPSS/SSS_sync_intra_ODSSB, TSSB_time_index_intra_ODSSB, TSSB_measurement_period_intra_ODSSB given in table 9.17.5.3-1, 9.17.5.3-2 and 9.17.5.4-1 regardless of configured DRX and measCycleSCell till slot n+THARQ+3ms+Tprocessing+TFMWNR slot length.

-	When UE is indicated the activation of another OD-SSB transmission with the same spatial reception parameter and same frequency as the previously deactivated OD-SSB in a configured DL BWP of an SCell, and the indication occurs within 5 seconds after the previous OD-SSB deactivation before the SCell activation command, and the SCell remains detectable during the time period Tidentify_intra_without_index_ODSSB or Tidentify_intra_with_index_ODSSB measured from previously transmitted OD-SSB, the UE shall skip the OD-SSB identification provided that the SCell is detectable and the timing that the cell has not been changed more than  3200/2µ Tc.

-	After the FMW, UE shall follow the legacy deactivated SCC measurement with TPSS/SSS_sync_intra_ODSSB, TSSB_time_index_intra_ODSSB, TSSB_measurement_period_intra_ODSSB given in table 9.17.5.3-3, 9.17.5.3-4, 9.17.5.3-5 9.17.5.4-2 and 9.17.5.4-3 if the OD-SSB transmission is not deactivated.

No OD-SSB based deactivated SCell requirement applies if NW deactivates the OD-SSB earlier than the time slot n+THARQ+3ms+Tprocessing+min(Treport, Tidentify_intra)NR slot length.

When UE is indicated the activation of OD-SSB transmission in slot n in a configured DL BWP of an SCell with a first SSB transmission before SCell activation command, after slot n+THARQ+3ms+TprocessingNR slot length,

- UE shall perform the deactivated SCC measurement with TPSS/SSS_sync_intra_ODSSB, TSSB_time_index_intra_ODSSB, TSSB_measurement_period_intra_ODSSB given in table 9.17.5.3-3, 9.17.5.3-4, 9.17.5.3-5, 9.17.5.4-2 and 9.17.5.4-3 if UE has sent a valid measurement report before receiving the activation of OD-SSB transmission during the period equal to max(5*measCycleSCell,  5*DRX cycles) in FR1 or 4s for UE supporting power class1 and 3s for UE supporting power class 2/3/4 in FR2.

- Otherwise, UE shall perform the deactivated SCell measurement with TPSS/SSS_sync_intra_ODSSB, TSSB_time_index_intra_ODSSB, TSSB_measurement_period_intra_ODSSB given in table 9.17.5.3-1, 9.17.5.3-2 and 9.17.5.4-1 regardless of configured DRX and measCycleSCell till slot n+THARQ+3ms+Tprocessing+TFMWNR slot length..

- Before UE receiving the activation of OD-SSB transmission or after OD-SSB deactivation in a deactivated SCell, UE shall perform deactivated SCC measurement with TPSS/SSS_sync_intra_ODSSB, TSSB_time_index_intra_ODSSB, TSSB_measurement_period_intra_ODSSB given in table 9.17.5.3-3, 9.17.5.3-4, 9.17.5.3-5, 9.17.5.4-2 and 9.17.5.4-3 if OD-SSB is transmitted.

- Before UE receiving the activation of OD-SSB transmission or after OD-SSB deactivation in a deactivated SCell, for UE indicating support3MHz-ChannelBW-Symmetric-r18 and configured to perform measurements on a deactivated SCell with 12PRB SSB in FR1, TSSB_time_index_intra_less_than_5MHz_ODSSB is given in table 9.17.5.3-6.

UE shall perform the deactivated SCC neighbour cell measurement with TPSS/SSS_sync_intra_ODSSB, TSSB_time_index_intra_ODSSB, TSSB_measurement_period_intra_ODSSB given in table 9.17.5.3-3, 9.17.5.3-4, 9.17.5.3-5, 9.17.5.3-6, 9.17.5.4-2 and 9.17.5.4-3 regardless of OD-SSB activation/deactivation.

If MCG DRX is in use, cell identification requirements for intra-frequency measurement in MCG specified in Table 9.17.5.3-2, Table 9.17.5.3-3, Table 9.17.5.3-4, 9.17.5.3-5 and Table 9.17.5.3-6 shall depend on the MCG DRX cycle. If SCG DRX is in use, cell identification requirements for intra-frequency measurement in SCG specified in Table 9.17.5.3-2, Table 9.17.5.3-3, Table 9.17.5.3-4, 9.17.5.3-5 and Table 9.17.5.3-6 shall depend on the SCG DRX cycle. Otherwise, the requirements for when DRX is not in use shall apply.

When NW indicates OD-SSB transmission in one of the deactivated SCell, the longer cell identification or measurement delay for other frequency layers being measured would be expected during the TFMW.

When more than one deactivated SCells are configured with OD-SSB transmission, the requirements in this clause shall be also applied provided that the time periods between OD-SSB activation and OD-SSB deactivation among multiple OD-SSBs’ frequency layers are non-overlapping.

Table 9.17.5.3-1: Time period for OD-SSB based PSS/SSS detection in FMW, deactivated SCell

| Frequency range | TPSS/SSS_sync_intra_ODSSB |
| --- | --- |
| FR1 | ceil(M Note 2 x Kp_OD-SSB) x max(Tlower Note 1, EMP) x CSSFintra |
| FR2 | ceil(M Note 2 x KFR x Kp_OD-SSB x Klayer1_measurement) x max(Tlower Note 1, EMP) x CSSFintra |
| NOTE 1: When UE reports not supporting lower bound capability, the Tlower equals 10ms; Otherwise, Tlower equals 0ms. NOTE 2: When multiple frequency layers are configured, M equals 6 in FR1 and Mpss/sss_sync_w/o_gaps+1 in FR2. Otherwise, M equals 5 in FR1 and Mpss/sss_sync_w/o_gaps in FR2. |  |

Table 9.17.5.3-2: Time period for OD-SSB based time index detection in FMW, deactivated SCell (FR1)

| FR | TSSB_time_index_intra_ODSSB |
| --- | --- |
| FR1 | ceil( 3 x Kp_OD-SSB ) x max(Tlower Note 1, EMP) x CSSFintra |
| NOTE 1: When UE reports not supporting lower bound capability, the Tlower equals 10ms; Otherwise, Tlower equals 0ms. |  |

Table 9.17.5.3-3: Time period for OD-SSB based PSS/SSS detection, deactivated SCC (FR1)

| DRX cycle | TPSS/SSS_sync_intra_ODSSB |
| --- | --- |
| No DRX | Ceil(5 x Kp_OD-SSB) x measCycleSCell x CSSFintra |
| DRX cycle≤ 320 ms | Ceil(5 x Kp_OD-SSB) x max(measCycleSCell, 1.5xDRX cycle) x CSSFintra |
| DRX cycle> 320 ms | Ceil(5 x Kp_OD-SSB) x max(measCycleSCell, DRX cycle) x CSSFintra |
| NOTE 1: The requirements also apply to CC with deactivated SCell in SCG. |  |

Table 9.17.5.3-4: Time period for OD-SSB based PSS/SSS detection, deactivated SCC (FR2)

| DRX cycle | TPSS/SSS_sync_intra_ODSSB |
| --- | --- |
| No DRX | Ceil(Mpss/sss_sync_w/o_gaps x Kp_OD-SSB) x measCycleSCell x CSSFintra |
| DRX cycle≤ 320 ms | Ceil(Mpss/sss_sync_w/o_gaps x Kp_OD-SSB) x max(measCycleSCell, 1.5xDRX cycle) x CSSFintra |
| DRX cycle> 320 ms | Ceil(Mpss/sss_sync_w/o_gaps x Kp_OD-SSB) x max(measCycleSCell, DRX cycle) x CSSFintra |
| NOTE 1: The requirements also apply to CC with deactivated SCell in SCG. |  |

Table 9.17.5.3-5: Time period for OD-SSB based time index detection, deactivated SCC (FR1)

| DRX cycle | TSSB_time_index_intra_ODSSB |
| --- | --- |
| No DRX | Ceil(3 x Kp_OD-SSB) x measCycleSCell x CSSFintra |
| DRX cycle≤ 320 ms | Ceil(3 x Kp_OD-SSB) x max(measCycleSCell, 1.5xDRX cycle) x CSSFintra |
| DRX cycle> 320 ms | Ceil(3 x Kp_OD-SSB) x max(measCycleSCell, DRX cycle) x CSSFintra |
| NOTE 1: The requirements also apply to CC with deactivated SCell in SCG. |  |

Table 9.17.5.3-6: Time period for time index detection for a UE operating on a target cell with 12PRB SSB, deactivated SCC (FR1)

| DRX cycle | TSSB_time_index_intra_less_than_5MHz |
| --- | --- |
| No DRX | Ceil(7 x Kp_OD-SSB) x measCycleSCell x CSSFintra |
| DRX cycle≤ 320ms | Ceil(7 x Kp_OD-SSB) x max(measCycleSCell, 1.5xDRX cycle) x CSSFintra |
| DRX cycle> 320ms | Ceil(7 x Kp_OD-SSB) x max(measCycleSCell, DRX cycle) x CSSFintra |
| NOTE 1: The requirements apply to CC with deactivated SCG SCell. |  |

-------------------- END OF CHANGES--------------------------