3GPP TSG-RAN4 Meeting #117	R4-2522796

# Dallas, United States, 17th - 21st November, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR | 0825 | rev | 1 | Current version: | 16.22.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (NR_HST-Perf) Clarification on HST-DPS test scenarios |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_HST-Perf |  |  |  |  |  | Date: |  |  | 2025-11-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Note 1 in HST-DPS test parameter table defines when and how MAC CE for TCI state switch command is sent by TE: TCI state switching command scheduled by MAC CE with PDSCH configuration - MCS 4, Layer 1, StartRB 24, NumOfRB 28 is transmitted in slot #i that satisfy modi,2n=n. Following formula mod(i,2n) = n, where n = 2520 for 15 kHz case and n = 5040 for 30 kHz case, only few slots in the whole hyperframe can meet with this criteria. Additionally, TE needs to wait to start fading HST-DPS profile as defined in Annex B.3.3 until formula mod(i,2n) = n is satisfied for ensuring TCI state swithing command is transmitted in the middle of two consecutive RRHs, this extra time to start measurement will increase testing time. However, it is possible to send TCI state switching command in the middle of two consecutive RRHs and meeting with Annex B.3.3 wihtout the need to wait for an specific start time. Hence, TCI state swithing command can be sent in any slot meeting with following criteria and Annex B: - Tranmission in the middle of two consecutive RRHs - Overlapping with slots with SS/PBCH blocks transmissions - Periodicity of 2n |  |  |  |  |  |  |  |  |
| Summary of change: |  | Udpated Note 1 for allowing any time meeting with above criteria |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TCI state swithing command will be stick to only few slots in a hyperframe |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.2.1.10, 5.2.2.2.10, 5.2.3.1.10, 5.2.3.2.10 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS/TR 38.521-4 CR 1035 |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Fina version of R4-2520712 correcting other spec affected in cover page. |  |  |  |  |  |  |  |  |

## <<< START OF CHANGES >>>

##### 5.2.2.1.10	Minimum requirements for HST-DPS

The performance requirements are specified in Table 5.2.2.1.10-3, with the addition of test parameters in Table 5.2.2.1.10-2 and the downlink physical channel setup according to Annex C.3.1.

The test purposes are specified in Table 5.2.2.1.10-1.

Table 5.2.2.1.10-1: Tests purpose

| Purpose | Test index |
| --- | --- |
| Verify UE performance in the HST-DPS scenario defined in B.3.3 | 1-1, 1-2 |

Table 5.2.2.1.10-2: Test parameters

| Parameter |  |  | Unit | Value |
| --- | --- | --- | --- | --- |
| Duplex mode |  |  |  | FDD |
| Active DL BWP index |  |  |  | 1 |
| PDCCH configuration | TCI state |  |  | Note 1 |
| PDSCH configuration | Mapping type |  |  | Type A |
|  | k0 |  |  | 0 |
|  | Starting symbol (S) |  |  | 2 |
|  | Length (L) |  |  | 12 |
|  | PDSCH aggregation factor |  |  | 1 |
|  | PRB bundling type |  |  | Static |
|  | PRB bundling size |  |  | 2 |
|  | Resource allocation type |  |  | Type 0 |
|  | RBG size |  |  | Config2 |
|  | VRB-to-PRB mapping type |  |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  |  | N/A |
|  | TCI state |  |  | Note 1 |
| PDSCH DMRS configuration | DMRS Type |  |  | Type 1 |
|  | Number of additional DMRS |  |  | 2 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  |  | 1 |
| CSI-RS for tracking | Resource set #1 | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 5 for CSI-RS resource 1 and 3 l0 = 9 for CSI-RS resource 2 and 4 |
|  |  | CSI-RS periodicity | Slots | 10 for CSI-RS resource 1,2,3,4. |
|  |  | CSI-RS offset | Slots | 1 for CSI-RS resource 1 and 2 2 for CSI-RS resource 3 and 4 |
|  |  | QCL info |  | TCI state #2 |
|  | Resource set #2 | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 6 for CSI-RS resource 5 and 7 l0 = 10 for CSI-RS resource 6 and 8 |
|  |  | CSI-RS periodicity | Slots | 10 for CSI-RS resource 5,6,7,8. |
|  |  | CSI-RS offset | Slots | 1 for CSI-RS resource 5 and 6 2 for CSI-RS resource 7 and 8 |
|  |  | QCL info |  | TCI state #3 |
| NZP CSI-RS for CSI acquisition | Resource set #3 | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 12 |
|  |  | CSI-RS periodicity | Slots | 20 |
|  |  | CSI-RS offset | Slots | 0 |
|  |  | QCL info |  | TCI state #0 |
|  | Resource set #4 | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 13 |
|  |  | CSI-RS periodicity | Slots | 20 |
|  |  | CSI-RS offset | Slots | 0 |
|  |  | QCL info |  | TCI state #1 |
| TCI state #0 | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 1 from 'CSI-RS for tracking Resource set #1' configuration |
|  |  | QCL Type |  | Type A |
|  | Type 2 QCL information | CSI-RS resource |  | N/A |
|  |  | QCL Type |  | N/A |
| TCI state #1 | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 5 from 'CSI-RS for tracking Resource set #2' configuration |
|  |  | QCL Type |  | Type A |
|  | Type 2 QCL information | CSI-RS resource |  | N/A |
|  |  | QCL Type |  | N/A |
| TCI state #2 | Type 1 QCL information | SSB index |  | SSB #0 |
|  |  | QCL Type |  | Type C |
|  | Type 2 QCL information | SSB index |  | N/A |
|  |  | QCL Type |  | N/A |
| TCI state #3 | Type 1 QCL information | SSB index |  | SSB #1 |
|  |  | QCL Type |  | Type C |
|  | Type 2 QCL information | SSB index |  | N/A |
|  |  | QCL Type |  | N/A |
| Number of HARQ Processes |  |  |  | 4 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  |  | 2 |
| Note 1: SSB # (k mod 2) , CSI-RS (for tracking) resource set # ((k mod 2) + 1) and CSI-RS (for CSI acquisition) resource set # ((k mod 2) + 3) are transmitted by kth RRH. For Test 1-1, TCI state switching command scheduled by MAC CE with PDSCH configuration - MCS 4, Layer 1, StartRB 24, NumOfRB 28 is transmitted in slot#i in the middle of two consecutive RRHs overlapping with slots with SS/PBCH blocks transmissions and a periodicity of 2n modi,2n=n. PDCCH and PDSCH associated with TCI # (k mod 2) is transmitted by kth RRH from slot# max⁡[2k-1n+1+THARQ+TMAC proc+TfirstTRS+TTRS proc, 0] to slot# 2k+1n+THARQ+TMAC proc, PDCCH and PDSCH are DTXed in other slots in which throughput statistics are not considered. For Test 1-2, TCI state switching command scheduled by MAC CE with PDSCH configuration - MCS 4, Layer 1, StartRB 24, NumOfRB 28 is transmitted in slot#i in the middle of two consecutive RRHs overlapping with slots with SS/PBCH blocks transmissions and a periodicity of 2n modi,2n=n. PDCCH and PDSCH associated with TCI # (k mod 2) is transmitted by kth RRH from: slot# max⁡[2k-1n+1+THARQ+TMAC proc, 0] to: slot# 2k+1n PDCCH and PDSCH are DTXed in other slots in which throughput statistics are not considered. Where k=0, 1, 2… is the RRH number, n = 2520 is half of the number of slots between two RRH, THARQ = 2 is the number of slots between PDSCH and corresponding HARQ-ACK information, TMAC proc = 3 is the number of slots for MAC CE processing, TfirstTRS = 6 is the number of slots to first TRS transmission occasion after MAC CE command is decoded by the UE, TTRS proc = 2 is the number of slots for TRS processing. |  |  |  |  |

Table 5.2.2.1.10-3: Minimum performance for HST-DPS

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Number of active PDSCH TCI states | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | R.PDSCH.1-8.4 FDD | 10 / 15 | 64QAM, 0.43 | HST-DPS | 1 | 2x2 | 70 | 13.4 |
| 1-2 | R.PDSCH.1-8.4 FDD | 10 / 15 | 64QAM, 0.43 | HST-DPS | 2 | 2x2 | 70 | 13.4 |

## <<< Skip unchanged sections >>>

##### 5.2.2.2.10	Minimum requirements for HST-DPS

The performance requirements are specified in Table 5.2.2.2.10-3, with the addition of test parameters in Table 5.2.2.2.10-2 and the downlink physical channel setup according to Annex C.3.1.

The test purposes are specified in Table 5.2.2.2.10-1.

Table 5.2.2.2.10-1: Tests purpose

| Purpose | Test index |
| --- | --- |
| Verify UE performance in the HST-DPS scenario defined in B.3.3 | 1-1, 1-2 |

Table 5.2.2.2.10-2: Test parameters

| Parameter |  |  | Unit | Value |
| --- | --- | --- | --- | --- |
| Duplex mode |  |  |  | TDD |
| Active DL BWP index |  |  |  | 1 |
| PDCCH configuration | TCI state |  |  | Note 1 |
| PDSCH configuration | Mapping type |  |  | Type A |
|  | k0 |  |  | 0 |
|  | Starting symbol (S) |  |  | 2 |
|  | Length (L) |  |  | Specific to each Reference channel |
|  | PDSCH aggregation factor |  |  | 1 |
|  | PRB bundling type |  |  | Static |
|  | PRB bundling size |  |  | 2 |
|  | Resource allocation type |  |  | Type 0 |
|  | RBG size |  |  | Config2 |
|  | VRB-to-PRB mapping type |  |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  |  | N/A |
|  | TCI state |  |  | Note 1 |
| PDSCH DMRS configuration | DMRS Type |  |  | Type 1 |
|  | Number of additional DMRS |  |  | 2 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  |  | 1 |
| CSI-RS for tracking | Resource set #1 | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 5 for CSI-RS resource 1 and 3 |
|  |  |  |  | l0 = 9 for CSI-RS resource 2 and 4 |
|  |  | CSI-RS periodicity | Slots | 20 for CSI-RS resource 1,2,3,4 |
|  |  | CSI-RS offset | Slots | 1 for CSI-RS resource 1 and 2 |
|  |  |  |  | 2 for CSI-RS resource 3 and 4 |
|  |  | QCL info |  | TCI state #2 |
|  |  | Frequency Occupation |  | Start PRB 0 |
|  |  |  |  | Number of PRB = 52 |
|  | Resource set #2 | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 6 for CSI-RS resource 5 and 7 |
|  |  |  |  | l0 = 10 for CSI-RS resource 6 and 8 |
|  |  | CSI-RS periodicity | Slots | 20 for CSI-RS resource 5,6,7,8. |
|  |  | CSI-RS offset | Slots | 1 for CSI-RS resource 5 and 6 |
|  |  |  |  | 2 for CSI-RS resource 7 and 8 |
|  |  | QCL info |  | TCI state #3 |
|  |  | Frequency Occupation |  | Start PRB 0 |
|  |  |  |  | Number of PRB = 52 |
| NZP CSI-RS for CSI acquisition | Resource set #3 | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 12 |
|  |  | CSI-RS periodicity | Slots | 40 |
|  |  | CSI-RS offset | Slots | 0 |
|  |  | QCL info |  | TCI state #0 |
|  | Resource set #4 | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 13 |
|  |  | CSI-RS periodicity | Slots | 40 |
|  |  | CSI-RS offset | Slots | 0 |
|  |  | QCL info |  | TCI state #1 |
| TCI state #0 | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 1 from 'CSI-RS for tracking Resource set #1' configuration |
|  |  | QCL Type |  | Type A |
|  | Type 2 QCL information | CSI-RS resource |  | N/A |
|  |  | QCL Type |  | N/A |
| TCI state #1 | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 5 from 'CSI-RS for tracking Resource set #2' configuration |
|  |  | QCL Type |  | Type A |
|  | Type 2 QCL information | CSI-RS resource |  | N/A |
|  |  | QCL Type |  | N/A |
| TCI state #2 | Type 1 QCL information | SSB index |  | SSB #0 |
|  |  | QCL Type |  | Type C |
|  | Type 2 QCL information | SSB index |  | N/A |
|  |  | QCL Type |  | N/A |
| TCI state #3 | Type 1 QCL information | SSB index |  | SSB #1 |
|  |  | QCL Type |  | Type C |
|  | Type 2 QCL information | SSB index |  | N/A |
|  |  | QCL Type |  | N/A |
| Number of HARQ Processes |  |  |  | 8 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  |  | Specific to each TDD UL-DL pattern and as defined in Annex A.1.2 |
| Note 1: SSB # (k mod 2), CSI-RS (for tracking) resource set # ((k mod 2) + 1) and CSI-RS (for CSI acquisition) resource set # ((k mod 2) + 3) are transmitted by kth RRH. For Test 1-1, TCI state switching command scheduled by MAC CE with PDSCH configuration - MCS 4, Layer 1, StartRB 32, NumOfRB 74 is transmitted in slot#i in the middle of two consecutive RRHs overlapping with slots with SS/PBCH blocks transmissions and a periodicity of 2n modi,2n=n. PDCCH and PDSCH associated with TCI # (k mod 2) is transmitted by kth RRH from: slot# max⁡[2k-1n+1+THARQ+TMAC proc+TfirstTRS+TTRS proc, 0] to: slot# 2k+1n, PDCCH and PDSCH are DTXed in other slots in which throughput statistics are not considered. For Test 1-2, TCI state switching command scheduled by MAC CE with PDSCH configuration - MCS 4, Layer 1, StartRB 32, NumOfRB 74 is transmitted in slot#i in the middle of two consecutive RRHs overlapping with slots with SS/PBCH blocks transmissions and a periodicity of 2n modi,2n=n. PDCCH and PDSCH associated with TCI # (k mod 2) is transmitted by kth RRH from: slot# max⁡[2k-1n+1+THARQ+TMAC proc, 0] to: slot# 2k+1n PDCCH and PDSCH are DTXed in other slots in which throughput statistics are not considered. Where k=0, 1, 2… is the RRH number, n = 5040 is half of the number of slots between two RRH, THARQ = 8 is the number of slots between PDSCH and corresponding HARQ-ACK information, TMAC proc = 6 is the number of slots for MAC CE processing, TfirstTRS = 7 is the number of slots to first TRS transmission occasion after MAC CE command is decoded by the UE, TTRS proc = 4 is the number of slots for TRS processing. |  |  |  |  |

Table 5.2.2.2.10-3: Minimum performance for HST-DPS

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition | Number of active PDSCH TCI states | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | R.PDSCH.2-10.5 TDD | 40 / 30 | 64QAM, 0.43 | FR1.30-1 | HST-DPS | 1 | 2x2 | 70 | 13.0 |
| 1-2 | R.PDSCH.2-10.5 TDD | 40 / 30 | 64QAM, 0.43 | FR1.30-1 | HST-DPS | 2 | 2x2 | 70 | 13.0 |

## <<< Skip unchanged sections >>>

##### 5.2.3.1.10	Minimum requirements for HST-DPS

The performance requirements are specified in Table 5.2.3.1.10-3, with the addition of test parameters in Table 5.2.3.1.10-2 and the downlink physical channel setup according to Annex C.3.1.

The test purposes are specified in Table 5.2.3.1.10-1.

Table 5.2.3.1.10-1: Tests purpose

| Purpose | Test index |
| --- | --- |
| Verify UE performance in the HST-DPS scenario defined in B.3.3 | 1-1, 1-2 |

Table 5.2.3.1.10-2: Test parameters

| Parameter |  |  | Unit | Value |
| --- | --- | --- | --- | --- |
| Duplex mode |  |  |  | FDD |
| Active DL BWP index |  |  |  | 1 |
| PDCCH configuration | TCI state |  |  | Note 1 |
| PDSCH configuration | Mapping type |  |  | Type A |
|  | k0 |  |  | 0 |
|  | Starting symbol (S) |  |  | 2 |
|  | Length (L) |  |  | 12 |
|  | PDSCH aggregation factor |  |  | 1 |
|  | PRB bundling type |  |  | Static |
|  | PRB bundling size |  |  | 2 |
|  | Resource allocation type |  |  | Type 0 |
|  | RBG size |  |  | Config2 |
|  | VRB-to-PRB mapping type |  |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  |  | N/A |
|  | TCI state |  |  | Note 1 |
| PDSCH DMRS configuration | DMRS Type |  |  | Type 1 |
|  | Number of additional DMRS |  |  | 2 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  |  | 1 |
| CSI-RS for tracking | Resource set #1 | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 5 for CSI-RS resource 1 and 3 l0 = 9 for CSI-RS resource 2 and 4 |
|  |  | CSI-RS periodicity | Slots | 10 for CSI-RS resource 1,2,3,4. |
|  |  | CSI-RS offset | Slots | 1 for CSI-RS resource 1 and 2 2 for CSI-RS resource 3 and 4 |
|  |  | QCL info |  | TCI state #2 |
|  | Resource set #2 | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 6 for CSI-RS resource 5 and 7 l0 = 10 for CSI-RS resource 6 and 8 |
|  |  | CSI-RS periodicity | Slots | 10 for CSI-RS resource 5,6,7,8. |
|  |  | CSI-RS offset | Slots | 1 for CSI-RS resource 5 and 6 2 for CSI-RS resource 7 and 8 |
|  |  | QCL info |  | TCI state #3 |
| NZP CSI-RS for CSI acquisition | Resource set #3 | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 12 |
|  |  | CSI-RS periodicity | Slots | 20 |
|  |  | CSI-RS offset | Slots | 0 |
|  |  | QCL info |  | TCI state #0 |
|  | Resource set #4 | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 13 |
|  |  | CSI-RS periodicity | Slots | 20 |
|  |  | CSI-RS offset | Slots | 0 |
|  |  | QCL info |  | TCI state #1 |
| TCI state #0 | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 1 from 'CSI-RS for tracking Resource set #1' configuration |
|  |  | QCL Type |  | Type A |
|  | Type 2 QCL information | CSI-RS resource |  | N/A |
|  |  | QCL Type |  | N/A |
| TCI state #1 | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 5 from 'CSI-RS for tracking Resource set #2' configuration |
|  |  | QCL Type |  | Type A |
|  | Type 2 QCL information | CSI-RS resource |  | N/A |
|  |  | QCL Type |  | N/A |
| TCI state #2 | Type 1 QCL information | SSB index |  | SSB #0 |
|  |  | QCL Type |  | Type C |
|  | Type 2 QCL information | SSB index |  | N/A |
|  |  | QCL Type |  | N/A |
| TCI state #3 | Type 1 QCL information | SSB index |  | SSB #1 |
|  |  | QCL Type |  | Type C |
|  | Type 2 QCL information | SSB index |  | N/A |
|  |  | QCL Type |  | N/A |
| Number of HARQ Processes |  |  |  | 4 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  |  | 2 |
| Note 1: SSB # (k mod 2), CSI-RS (for tracking) resource set # ((k mod 2) + 1) and CSI-RS (for CSI acquisition) resource set # ((k mod 2) + 3) are transmitted by kth RRH. For Test 1-1, TCI state switching command scheduled by MAC CE with PDSCH configuration - MCS 4, Layer 1, StartRB 24, NumOfRB 28 is transmitted in slot#i in the middle of two consecutive RRHs overlapping with slots with SS/PBCH blocks transmissions and a periodicity of 2n modi,2n=n. PDCCH and PDSCH associated with TCI # (k mod 2) is transmitted by kth RRH from: slot# max⁡[2k-1n+1+THARQ+TMAC proc+TfirstTRS+TTRS proc, 0] to: slot# 2k+1n, PDCCH and PDSCH are DTXed in other slots in which throughput statistics are not considered. For Test 1-2, TCI state switching command scheduled by MAC CE with PDSCH configuration - MCS 4, Layer 1, StartRB 24, NumOfRB 28 is transmitted in slot#i in the middle of two consecutive RRHs overlapping with slots with SS/PBCH blocks transmissions and a periodicity of 2n modi,2n=n. PDCCH and PDSCH associated with TCI # (k mod 2) is transmitted by kth RRH from: slot# max⁡[2k-1n+1+THARQ+TMAC proc, 0] to: slot# 2k+1n PDCCH and PDSCH are DTXed in other slots in which throughput statistics are not considered. Where k=0, 1, 2… is the RRH number, n = 2520 is half of the number of slots between two RRH, THARQ = 2 is the number of slots between PDSCH and corresponding HARQ-ACK information, TMAC proc = 3 is the number of slots for MAC CE processing, TfirstTRS = 6 is the number of slots to first TRS transmission occasion after MAC CE command is decoded by the UE, TTRS proc = 2 is the number of slots for TRS processing. |  |  |  |  |

Table 5.2.3.1.10-3: Minimum performance for HST-DPS

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Number of active PDSCH TCI states | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | R.PDSCH.1-8.4 FDD | 10 / 15 | 64QAM, 0.43 | HST-DPS | 1 | 2x4 | 70 | 10.6 |
| 1-2 | R.PDSCH.1-8.4 FDD | 10 / 15 | 64QAM, 0.43 | HST-DPS | 2 | 2x4 | 70 | 10.6 |

## <<< Skip unchanged sections >>>

##### 5.2.3.2.10	Minimum requirements for HST-DPS

The performance requirements are specified in Table 5.2.3.2.10-3, with the addition of test parameters in Table 5.2.3.2.10-2 and the downlink physical channel setup according to Annex C.3.1.

The test purposes are specified in Table 5.2.3.2.10-1.

Table 5.2.3.2.10-1: Tests purpose

| Purpose | Test index |
| --- | --- |
| Verify UE performance in the HST-DPS scenario defined in B.3.3 | 1-1, 1-2 |

Table 5.2.3.2.10-2: Test parameters

| Parameter |  |  | Unit | Value |
| --- | --- | --- | --- | --- |
| Duplex mode |  |  |  | TDD |
| Active DL BWP index |  |  |  | 1 |
| PDCCH configuration | TCI state |  |  | Note 1 |
| PDSCH configuration | Mapping type |  |  | Type A |
|  | k0 |  |  | 0 |
|  | Starting symbol (S) |  |  | 2 |
|  | Length (L) |  |  | Specific to each Reference channel |
|  | PDSCH aggregation factor |  |  | 1 |
|  | PRB bundling type |  |  | Static |
|  | PRB bundling size |  |  | 2 |
|  | Resource allocation type |  |  | Type 0 |
|  | RBG size |  |  | Config2 |
|  | VRB-to-PRB mapping type |  |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  |  | N/A |
|  | TCI state |  |  | Note 1 |
| PDSCH DMRS configuration | DMRS Type |  |  | Type 1 |
|  | Number of additional DMRS |  |  | 2 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  |  | 1 |
| CSI-RS for tracking | Resource set #1 | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 5 for CSI-RS resource 1 and 3 |
|  |  |  |  | l0 = 9 for CSI-RS resource 2 and 4 |
|  |  | CSI-RS periodicity | Slots | 20 for CSI-RS resource 1,2,3,4 |
|  |  | CSI-RS offset | Slots | 1 for CSI-RS resource 1 and 2 |
|  |  |  |  | 2 for CSI-RS resource 3 and 4 |
|  |  | QCL info |  | TCI state #2 |
|  |  | Frequency Occupation |  | Start PRB 0 |
|  |  |  |  | Number of PRB = 52 |
|  | Resource set #2 | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 6 for CSI-RS resource 5 and 7 |
|  |  |  |  | l0 = 10 for CSI-RS resource 6 and 8 |
|  |  | CSI-RS periodicity | Slots | 20 for CSI-RS resource 5,6,7,8. |
|  |  | CSI-RS offset | Slots | 1 for CSI-RS resource 5 and 6 |
|  |  |  |  | 2 for CSI-RS resource 7 and 8 |
|  |  | QCL info |  | TCI state #3 |
|  |  | Frequency Occupation |  | Start PRB 0 |
|  |  |  |  | Number of PRB = 52 |
| NZP CSI-RS for CSI acquisition | Resource set #3 | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 12 |
|  |  | CSI-RS periodicity | Slots | 40 |
|  |  | CSI-RS offset | Slots | 0 |
|  |  | QCL info |  | TCI state #0 |
|  | Resource set #4 | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 13 |
|  |  | CSI-RS periodicity | Slots | 40 |
|  |  | CSI-RS offset | Slots | 0 |
|  |  | QCL info |  | TCI state #1 |
| TCI state #0 | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 1 from 'CSI-RS for tracking Resource set #1' configuration |
|  |  | QCL Type |  | Type A |
|  | Type 2 QCL information | CSI-RS resource |  | N/A |
|  |  | QCL Type |  | N/A |
| TCI state #1 | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 5 from 'CSI-RS for tracking Resource set #2' configuration |
|  |  | QCL Type |  | Type A |
|  | Type 2 QCL information | CSI-RS resource |  | N/A |
|  |  | QCL Type |  | N/A |
| TCI state #2 | Type 1 QCL information | SSB index |  | SSB #0 |
|  |  | QCL Type |  | Type C |
|  | Type 2 QCL information | SSB index |  | N/A |
|  |  | QCL Type |  | N/A |
| TCI state #3 | Type 1 QCL information | SSB index |  | SSB #1 |
|  |  | QCL Type |  | Type C |
|  | Type 2 QCL information | SSB index |  | N/A |
|  |  | QCL Type |  | N/A |
| Number of HARQ Processes |  |  |  | 8 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  |  | Specific to each TDD UL-DL pattern and as defined in Annex A.1.2 |
| Note 1: SSB # (k mod 2) , CSI-RS (for tracking) resource set # ((k mod 2) + 1) and CSI-RS (for CSI acquisition) resource set # ((k mod 2) + 3) are transmitted by kth RRH. For Test 1-1, TCI state switching command scheduled by MAC CE with PDSCH configuration - MCS 4, Layer 1, StartRB 32, NumOfRB 74 is transmitted in slot#i in the middle of two consecutive RRHs overlapping with slots with SS/PBCH blocks transmissions and a periodicity of 2n modi,2n=n. PDCCH and PDSCH associated with TCI # (k mod 2) is transmitted by kth RRH from slot# max⁡[2k-1n+1+THARQ+TMAC proc+TfirstTRS+TTRS proc, 0] to slot# 2k+1n+THARQ+TMAC proc, PDCCH and PDSCH are DTXed in other slots in which throughput statistics are not considered. For Test 1-2, TCI state switching command scheduled by MAC CE with PDSCH configuration - MCS 4, Layer 1, StartRB 32, NumOfRB 74 is transmitted in slot#i in the middle of two consecutive RRHs overlapping with slots with SS/PBCH blocks transmissions and a periodicity of 2n modi,2n=n. PDCCH and PDSCH associated with TCI # (k mod 2) is transmitted by kth RRH from slot# max⁡[2k-1n+1+THARQ+TMAC proc, 0] to slot# 2k+1n+THARQ+TMAC proc Where k=0, 1, 2… is the RRH number, n = 5040 is half of the number of slots between two RRH, THARQ = 8 is the number of slots between PDSCH and corresponding HARQ-ACK information, TMAC proc = 6 is the number of slots for MAC CE processing, TfirstTRS = 7 is the number of slots to first TRS transmission occasion after MAC CE command is decoded by the UE, TTRS proc = 4 is the number of slots for TRS processing. |  |  |  |  |

Table 5.2.3.2.10-3: Minimum performance for HST-DPS

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition | Number of active PDSCH TCI states | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | R.PDSCH.2-10.5 TDD | 40 / 30 | 64QAM, 0.43 | FR1.30-1 | HST-DPS | 1 | 2x4 | 70 | 10.2 |
| 1-2 | R.PDSCH.2-10.5 TDD | 40 / 30 | 64QAM, 0.43 | FR1.30-1 | HST-DPS | 2 | 2x4 | 70 | 10.2 |

## <<< END OF CHANGES >>>