3GPP TSG-RAN4 Meeting #117	R4-2523220

# Dallas, United States, 17th Nov 2025 - 21st Nov 2025

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 6270 | rev | 1 | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (NR_HST) CR correction of EUTRA requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_HST-Core |  |  |  |  |  | Date: |  |  | 2025-11-07 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Wrong table numbering causing ambiguous requirements |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correction of table numbering which apears repeated for inter-RAT measurement requirements for HST. New number is included to avoid table number repetition. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Both HST and Rel-15 requirements are unclear, since the same table number applies for different releases. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.4.1, 9.4.2.3, 9.4.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

==============First change==============

## 9.4	Inter-RAT measurements

### 9.4.1	Introduction

The requirements in this clause are specified for NR−E-UTRAN FDD and NR−E-UTRAN TDD measurements and are applicable without an explicit E-UTRAN neighbour cell list containing physical layer cell identities, for a UE:

-	in RRC_CONNECTED state, and

-	configured

-	with SA or NR-DC operation mode or configured in NE-DC operation mode by PCell with NRE-UTRAN FDD or TDD measurement (RSRP, RSRQ, RS-SINR, RSTD, or E-CID RSRP and RSRQ) on E-UTRA non-serving frequency carrier, or

-	with SA operation mode on NR carrier frequencies with CCA by PCell with NRE-UTRAN FDD or TDD measurement (RSRP, RSRQ, RS-SINR) on E-UTRA non-serving frequency carrier, and

-	configured with an appropriate measurement gap pattern according to table 9.1.2-3.

The requirements in this clause for concurrent measurement gaps are only applied for UE in NR SA operation mode.

For UE supporting eutra-NeedForGapNCSG-reporting-r17 and indicating NeedForGapNCSG-InfoEUTRA for inter-RAT measurement,

-	An inter-RAT measurement is defined as measurement without gap if

-	the UE indicates ‘nogap-noncsg’ via NeedForGapNCSG-InfoEUTRA for the inter-RAT measurement

-	for UE support FG 32-4, the inter-RAT E-UTRAN measurements without gaps requirements are specified in clause 9.4.8.

-	An inter-RAT measurement is defined as measurement with NCSG if

-	the UE indicates ‘ncsg’ via NeedForGapNCSG-InfoEUTRA for the inter-RAT measurement

When network configures measurement gap or NCSG, the delay requirements are specified in clause 9.4.2 and 9.4.3.

-	An inter-RAT measurement is defined as measurement with gap if

-	the UE indicates ‘gap’ via NeedForGapNCSG-InfoEUTRA for the inter-RAT measurement

When network configures measurement gap, the delay requirements are specified in clauses 9.4.2 and 9.4.3.

-	For inter-RAT measurements with NCSG, UE may cause scheduling restriction as specified in clause 9.4.2.5 and 9.4.3.5.

For UE not supporting eutra-NeedForGapNCSG-reporting-r17,

-	An inter-RAT measurement is defined as measurement without gap if

-	the UE supports FG32-5 and the CRS is completely contained in the active BWP of the UE, and the inter-RAT EUTRAN measurements without gaps requirements are specified in clause 9.4.8.

For inter-RAT, when measurement is defined as measurement without gap the UE may cause scheduling restriction as specified in clause 9.4.8.3.5 and clause 9.4.8.4.5.

When the UE is in NE-DC operation mode and an NRE-UTRAN FDD or TDD measurement (RSRP, RSRQ, RS-SINR, or E-CID RSRP and RSRQ) configured by NR PCell is on an E-UTRA serving frequency carrier, then the corresponding E-UTRA intra-frequency measurements requirements specified in clause 8.19 of TS 36.133 [15] shall apply.

When highSpeedMeasFlag-r16 is configured but UE does not support either measurementEnhancement-r16 or interRAT-MeasurementEnhancement-r16, the UE is not required to meet the requirements specified in table 9.4.2.3-1b and table 9.4.3.3-1b.

Parameter TInter1 used in inter-RAT requirements in clause 9.4 is specified in table 9.4.1-1 when measurement gap is used, and in table 9.4.1-2 when NCSG is used.

The measurement reporting delay can be longer for the measurement reporting requirements in this clause when IDC autonomous denial is configured.

==============Next change==============

#### 9.4.2.3	Requirements when DRX is used

When DRX is in use and an appropriate measurement gap pattern or NCSG is configured, or when the UE is capable of concurrent measurement gap patterns and concurrent measurement gap patterns are configured, or an appropriate pre-MG is scheduled and activated, the UE shall be able to identify a new detectable E-UTRAN FDD cell within TIdentify, E-UTRAN FDD specified in table 9.4.2.3-1. When highSpeedMeasFlag-r16 is configured and UE supports the enhanced inter-RAT E-UTRAN measurement requirements, the UE shall be able to identify a new detectable E-UTRAN FDD cell within TIdentify, E-UTRAN FDD specified in table 9.4.2.3-1b. When the UE supports measurement gap occasion cancellation and provided that the configuration and conditions in clause 9.1.14.4 are met, the UE shall be able to identify a new detectable E-UTRAN FDD cell within TIdentify, E-UTRAN FDD specified in table 9.4.2.3-4.

For a UE supporting and configured with concurrent GAPs, or MUSIM gaps or both concurrent GAPs and MUSIM gaps, Kgap_EUTRA: it is the scaling factor for an E-UTRAN frequency layer to be measured within the associated GAP pattern. Kgap_EUTRA = 1 when the UE is not configured with concurrent GAPs nor MUSIM gaps. Otherwise, Kgap_EUTRA = Ntotal / Navailable for UE configured with concurrent GAPs.

For a window W of duration xRP_max, where xRP_max is the maximum xRP across all configured per-UE GAPs, periodic MUSIM gap(s), and per-FR GAPs for FR1, and starting from the beginning of any associated gap occasion:

Ntotal is the total number of associated GAP occasions within the window, including both dropped and non-dropped instances of the associated GAP within the window, and

Navailable is the number of non-dropped associated GAP occasions after accounting for measurement gap and MUSIM gap collisions by applying the collision rules for the GAP and MUSIM gap in clauses 9.1.8.3 and 9.1.10.5, respectively.

-	xRP = MGRP when configured GAP is activated Pre-MG or MG, and xRP = VIRP when configured GAP is NCSG, also xRP = MGRP for periodic MUSIM gap.

Requirements do not apply for UE configured with concurrent GAPs, or MUSIM gaps, if Navailable = 0

When UE supports musim-GapPreference-r17 and if the configured aperiodic MUSIM gap collides with the measurement gap associated with the target frequency layer, where MUSIM gap collision rule in clause 9.1.10.4 is applied, longer cell identification period for the target inter-RAT carrier is expected.

Table 9.4.2.3-1: Requirement to identify a newly detectable E-UTRAN FDD cell

| DRX cycle length (s) | TIdentify, E-UTRAN FDD (s) (DRX cycles) |  |
| --- | --- | --- |
|  | Gap/NCSG period = 40 ms, 20 ms | Gap/NCSG period = 80 ms |
| ≤0.16 | Non-DRX requirements in clause 9.4.2.2 apply | Non-DRX requirements in clause 9.4.2.2 apply |
| 0.256 | 5.12* CSSFinterRAT x Ceil(Kgap_EUTRA) (20*CSSFinterRAT x Ceil(Kgap_EUTRA)) | 7.68* CSSFinterRAT x Ceil(Kgap_EUTRA) (30*CSSFinterRAT x Ceil(Kgap_EUTRA)) |
| 0.32 | 6.4* CSSFinterRAT x Ceil(Kgap_EUTRA) (20*CSSFinterRAT x Ceil(Kgap_EUTRA)) | 7.68* CSSFinterRAT x Ceil(Kgap_EUTRA) (24*CSSFinterRAT x Ceil(Kgap_EUTRA)) |
| 0.32< DRX-cycle ≤10.24 | Note1 (20*CSSFinterRAT x Ceil(Kgap_EUTRA)) | Note1 (20*CSSFinterRAT x Ceil(Kgap_EUTRA)) |
| NOTE 1: The time depends on the DRX cycle length. NOTE 2: CSSFinterRAT is as defined in clause 9.4.2.2. NOTE 2: CSSFinterRAT is as defined in clause 9.4.2.2. NOTE 3: Kgap_EUTRA is only applicable for a UE supporting concurrent GAPs. Otherwise Kgap_EUTRA = 1 NOTE 4: If multiple concurrent gaps are configured, the gap period is the periodicity of the MG pattern associated to the E-UTRA inter-RAT frequency layer. |  |  |

Table 9.4.2.3-1b: Requirement to identify a newly detectable E-UTRAN FDD cell when highSpeedMeasFlag-r16 is configured

| DRX cycle length (s) | TIdentify, E-UTRAN FDD (s) (DRX cycles) |  |
| --- | --- | --- |
|  | Gap/NCSG period = 40 ms, 20 ms | Gap/NCSG period = 80 ms |
| ≤0.16 | Non-DRX requirements in clause 9.4.2.2 apply | Non-DRX requirements in clause 9.4.2.2 apply |
| 0.16<DRX cycle≤0.32 | Note 1(15*CSSFinterRAT x Ceil((Kgap_EUTRA)) |  |
| 0.32<DRX cycle ≤ 0.64 | Note 1(10*CSSFinterRAT x Ceil(Kgap_EUTRA)) |  |
| DRX cycle = 1.024 | Note 1(10*CSSFinterRAT x Ceil( Kgap_EUTRA)) | Note 1(10*CSSFinterRAT x Ceil(Kgap_EUTRA)) |
| DRX cycle = 1.28 | Note 1(8*CSSFinterRAT x Ceil( Kgap_EUTRA)) | Note 1(8*CSSFinterRAT x Ceil(Kgap_EUTRA)) |
| 1.28< DRX-cycle ≤10.24 | Note1 (20*CSSFinterRAT x Ceil( Kgap_EUTRA)) | Note1 (20*CSSFinterRAT x Ceil(Kgap_EUTRA)) |
| NOTE 1: The time depends on the DRX cycle length. NOTE 2: CSSFinterRAT is as defined in clause 9.4.2.2. NOTE 3: When highSpeedMeasFlag-r16 is configured, the requirements apply only to UE supporting either measurementEnhancement-r16 or interRAT-MeasurementEnhancement-r16. NOTE 4: Kgap_EUTRA is only applicable for a UE supporting concurrent GAPs. Otherwise Kgap_EUTRA = 1 NOTE 5: If multiple concurrent gaps are configured, the gap period is the periodicity of the MG pattern associated to the E-UTRA inter-RAT frequency layer. |  |  |

When DRX is in use, the UE shall be capable of performing NR – E-UTRAN FDD RSRP, RSRQ, and RS-SINR measurements of at least 4 identified E-UTRAN FDD cells per E-UTRA FDD frequency layer during each layer 1 measurement period, for up to 7 E-UTRA FDD carrier frequency layers, and the UE physical layer shall be capable of reporting NR – E-UTRAN FDD RSRP, RSRQ, and RS-SINR measurements to higher layers with the measurement period Tmeasure, E-UTRAN FDD specified in table 9.4.2.3-2 for the UE not supporting measurement gap occasion cancellation, or provided that the configuration and conditions in clause 9.1.14.4 are not met, and Table 9.4.2.3-5 for the UE supporting measurement gap occasion cancellation provided that the configuration and conditions in clause 9.1.14.4 are met..

Table 9.4.2.3-2: Requirement to measure E-UTRAN FDD cells

| DRX cycle length (s) | Tmeasure, E-UTRAN FDD (s) (DRX cycles) |
| --- | --- |
| ≤0.08 | Non-DRX requirements in clause 9.4.2.2 apply |
| 0.08< DRX-cycle ≤ 10.24 | Note1 (5* CSSFinterRAT x Ceil(Kgap_EUTRA)) |
| NOTE 1: The time depends on the DRX cycle length. NOTE 2: CSSFinterRAT is as defined in clause 9.4.2.2. NOTE 3: Kgap_EUTRA is only applicable for a UE supporting concurrent GAPs. Otherwise Kgap_EUTRA =1 |  |

Table 9.4.2.3-4: Requirement to identify a newly detectable E-UTRAN FDD cell with measurement gap cancellation

| DRX cycle length (s) | TIdentify, E-UTRAN FDD (s) (DRX cycles) |  |
| --- | --- | --- |
|  | Gap/NCSG period = 40 ms, 20 ms | Gap/NCSG period = 80 ms |
| ≤0.16 | Non-DRX requirements in clause 9.4.2.2 apply | Non-DRX requirements in clause 9.4.2.2 apply |
| 0.256 | (5.12+Lcancel *0.256)* CSSFinterRAT x Ceil(Kgap_EUTRA) ((20+Lcancel *0.256)*CSSFinterRAT x Ceil(Kgap_EUTRA)) | (7.68+Lcancel *0.256)* CSSFinterRAT x Ceil(Kgap_EUTRA) ((30+Lcancel *0.256)*CSSFinterRAT x Ceil(Kgap_EUTRA)) |
| 0.32 | (6.4+Lcancel *0.32)* CSSFinterRAT x Ceil(Kgap_EUTRA) ( (20+Lcancel *0.32)*CSSFinterRAT x Ceil(Kgap_EUTRA)) | (7.68+Lcancel *0.32)* CSSFinterRAT x Ceil(Kgap_EUTRA) ((24+Lcancel *0.32)*CSSFinterRAT x Ceil(Kgap_EUTRA)) |
| 0.32< DRX-cycle ≤10.24 | Note1 ((20+Lcancel *DRX-cycle)*CSSFinterRAT x Ceil(Kgap_EUTRA)) | Note1 ((20+Lcancel *DRX-cycle)*CSSFinterRAT x Ceil(Kgap_EUTRA)) |
| NOTE 1: The time depends on the DRX cycle length. NOTE 2: CSSFinterRAT is as defined in clause 9.4.2.2. NOTE 2: CSSFinterRAT is as defined in clause 9.4.2.2. NOTE 3: Kgap_EUTRA is only applicable for a UE supporting concurrent GAPs. Otherwise Kgap_EUTRA = 1 NOTE 4: If multiple concurrent gaps are configured, the gap period is the periodicity of the MG pattern associated to the E-UTRA inter-RAT frequency layer. NOTE 5: For the UE supporting measurement gap occasion cancellation provided that the configuration and conditions in clause 9.1.14.4 are met, Lcancel is the number of gap occasions cancelled during TIdentify, E-UTRAN FDD for PSS/SSS detection. |  |  |

Table 9.4.2.3-5: Requirement to measure E-UTRAN FDD cells with measurement gap cancellation

| DRX cycle length (s) | Tmeasure, E-UTRAN FDD (s) (DRX cycles) |
| --- | --- |
| ≤0.08 | Non-DRX requirements in clause 9.4.2.2 apply |
| 0.08< DRX-cycle ≤ 10.24 | Note1 ((5*+ Lcancel * DRX-cycle) CSSFinterRAT x Ceil(Kgap_EUTRA)) |
| NOTE 1: The time depends on the DRX cycle length. NOTE 2: CSSFinterRAT is as defined in clause 9.4.2.2. NOTE 3: Kgap_EUTRA is only applicable for a UE supporting concurrent GAPs. Otherwise Kgap_EUTRA =1 NOTE 4: For the UE supporting measurement gap occasion cancellation provided that the configuration and conditions in clause 9.1.14.4 are met, Lcancel is the number of gap occasions cancelled during Tmeasure, E-UTRAN FDD for measurement. |  |

If higher layer filtering is used, an additional cell identification delay can be expected.

The NR – E-UTRAN FDD RSRP measurement accuracy for all measured cells shall be as specified in clause 10.2.2. The NR – E-UTRAN FDD RSRQ measurement accuracy for all measured cells shall be as specified in clause 10.2.3. The NR – E-UTRAN FDD RS-SINR measurement accuracy for all measured cells shall be as specified in clause 10.2.5.

==============Next change==============

#### 9.4.3.3	Requirements when DRX is used

When DRX is in use and an appropriate measurement gap pattern or NCSG is configured, or when the UE is capable of concurrent measurement gap patterns and concurrent measurement gap patterns are configured, or an appropriate pre-MG is scheduled and activated, the UE shall be able to identify a new detectable E-UTRAN TDD cell within TIdentify, E-UTRAN TDD specified in table 9.4.3.3-1. When highSpeedMeasFlag-r16 is configured and UE supports the enhanced inter-RAT E-UTRAN measurement requirements, the UE shall be able to identify a new detectable E-UTRAN TDD cell within TIdentify, E-UTRAN TDD specified in table 9.4.3.3-1b. When the UE supports measurement gap occasion cancellation and provided that the configuration and conditions in clause 9.1.14.4 are met, the UE shall be able to identify a new detectable E-UTRAN TDD cell within TIdentify, E-UTRAN TDD specified in table 9.4.3.3-4.

For a UE supporting and configured with concurrent GAPs, or MUSIM gaps or both concurrent GAPs and MUSIM gaps, Kgap_EUTRA: it is the scaling factor for an E-UTRAN frequency layer to be measured within the associated GAP pattern. Kgap_EUTRA = 1 when the UE is not configured with concurrent GAPs nor MUSIM gaps.. Otherwise, Kgap_EUTRA = Ntotal / Navailable for UE configured with concurrent GAPs or MUSIM gaps.

For a window W of duration xRP_max, where xRP_max is the maximum xRP across all configured per-UE GAPs s, periodic MUSIM gap(s), and per-FR measurement GAPs for FR1, and starting from the beginning of any associated gap occasion: GAPs

Ntotal is the total number of associated GAP occasions within the window, including those overlapped with other measurement GAP and MUSIM gap instances within the window, and

Navailable is the number of non-dropped associated GAP occasions after accounting for measurement gap and MUSIM gap collisions by applying the collision rules for the GAP and MUSIM gap in clauses 9.1.8.3, 9.1.10.5, 9.1.12.3, and 9.1.13.3, respectively.

-	xRP = MGRP when configured GAP is activated Pre-MG or MG, and xRP = VIRP when configured GAP is NCSG, also xRP = MGRP for periodic MUSIM gap.

Requirements do not apply for UE configured with concurrent GAPs or MUSIM gaps, if Navailable = 0

When UE supports musim-GapPreference-r17 and if the configured aperiodic MUSIM gap collides with the measurement gap associated with the target frequency layer, where MUSIM gap collision rule in clause 9.1.10.4 is applied, longer cell identification period for the target inter-RAT carrier is expected.

Table 9.4.3.3-1: Requirement to identify a newly detectable E-UTRAN TDD cell

| DRX cycle length (s) | TIdentify, E-UTRAN TDD (s) (DRX cycles) |  |
| --- | --- | --- |
|  | Gap/NCSG period = 40 ms, 20 ms | Gap/NCSG period = 80 ms |
| ≤0.16 | Non-DRX requirements in clause 9.4.3.2 apply | Non-DRX requirements in clause 9.4.3.2 apply |
| 0.256 | 5.12* CSSFinterRAT x Ceil(Kgap_EUTRA) (20*CSSFinterRAT x Ceil(Kgap_EUTRA)) | 7.68* CSSFinterRAT x Ceil(Kgap_EUTRA) (30*CSSFinterRAT x Ceil(Kgap_EUTRA)) |
| 0.32 | 6.4* CSSFinterRAT x Ceil(Kgap_EUTRA) (20*CSSFinterRAT x Ceil(Kgap_EUTRA)) | 7.68* CSSFinterRAT x Ceil(Kgap_EUTRA) (24*CSSFinterRAT x Ceil(Kgap_EUTRA)) |
| 0.32< DRX-cycle ≤10.24 | Note1 (20*CSSFinterRAT x Ceil(Kgap_EUTRA)) | Note1 (20*CSSFinterRAT x Ceil(Kgap_EUTRA)) |
| NOTE 1: The time depends on the DRX cycle length. NOTE 2: CSSFinterRAT is as defined in clause 9.4.3.2. NOTE 3: Kgap_EUTRA is only applicable for a UE supporting concurrent GAPs and/or MUSIM gaps. Otherwise Kgap_EUTRA =1 NOTE 4: If multiple concurrent gaps are configured, the gap period is the periodicity of the MG pattern associated to the E-UTRA inter-RAT frequency layer. |  |  |

Table 9.4.3.3-1b: Requirement to identify a newly detectable E-UTRAN TDD cell when highSpeedMeasFlag-r16 is configured

| DRX cycle length (s) | TIdentify, E-UTRAN TDD (s) (DRX cycles) |  |
| --- | --- | --- |
|  | Gap/NCSG period = 40 ms, 20 ms | Gap/NCSG period = 80 ms |
| ≤0.16 | Non-DRX requirements in clause 9.4.3.2 apply | Non-DRX requirements in clause 9.4.3.2 apply |
| 0.16<DRX cycle≤0.32 | Note 1(15*CSSFinterRAT x Ceil(Kgap_EUTRA)) |  |
| 0.32<DRX cycle ≤ 0.64 | Note 1(10*CSSFinterRAT x Ceil(Kgap_EUTRA)) |  |
| DRX cycle = 1.024 | Note 1(10*CSSFinterRAT x Ceil(Kgap_EUTRA)) | Note 1(10*CSSFinterRAT x Ceil(Kgap_EUTRA)) |
| DRX cycle = 1.28 | Note 1(8*CSSFinterRAT x Ceil(Kgap_EUTRA)) | Note 1(8*CSSFinterRAT x Ceil(Kgap_EUTRA)) |
| 1.28< DRX-cycle ≤10.24 | Note1 (20*CSSFinterRAT x Ceil(Kgap_EUTRA)) | Note1 (20*CSSFinterRAT x Ceil(Kgap_EUTRA)) |
| NOTE 1: The time depends on the DRX cycle length. NOTE 2: CSSFinterRAT is as defined in clause 9.4.3.2. NOTE 3: When highSpeedMeasFlag-r16 is configured, the requirements apply only to UE supporting either measurementEnhancement-r16 or interRAT-MeasurementEnhancement-r16. NOTE 4: Kgap_EUTRA is only applicable for a UE supporting concurrent GAPs and/or MUSIM gaps. Otherwise Kgap_EUTRA = 1 NOTE 5: If multiple concurrent gaps are configured, the gap period is the periodicity of the MG pattern associated to the E-UTRA inter-RAT frequency layer. |  |  |

When DRX is in use, the UE shall be capable of performing NR – E-UTRAN TDD RSRP, RSRQ, and RS-SINR measurements of at least 4 identified E-UTRAN TDD cells per E-UTRA TDD frequency layer during each layer 1 measurement period, for up to 7 E-UTRA TDD carrier frequency layers, and the UE physical layer shall be capable of reporting NR – E-UTRAN TDD RSRP, RSRQ, and RS-SINR measurements to higher layers with the measurement period Tmeasure, E-UTRAN TDD specified in table 9.4.3.3-3 for the UE not supporting measurement gap occasion cancellation, or provided that the configuration and conditions in clause 9.1.14.4 are not met, and Table 9.4.3.3-5 for the UE supporting measurement gap occasion cancellation provided that the configuration and conditions in clause 9.1.14.4 are met.

Table 9.4.3.3-3: Requirement to measure E-UTRAN TDD cells

| DRX cycle length (s) | Tmeasure, E-UTRAN TDD (s) (DRX cycles) |
| --- | --- |
| ≤ 0.08 | Non-DRX Requirements in clause 9.4.3.2 apply |
| 0.128 | For configuration 2 Note3, non-DRX requirements in clause 9.4.3.2 apply, Otherwise: Note1 (5*CSSFinterRAT x Ceil(Kgap_EUTRA)) |
| 0.128 < DRX-cycle ≤ 10.24 | Note1 (5*CSSFinterRAT x Ceil(Kgap_EUTRA)) |
| NOTE 1: The time depends on the DRX cycle length. NOTE 2: CSSFinterRAT is as defined in clause 9.4.3.2. NOTE 3: See Table 9.4.3.2-1. NOTE 4: Kgap_EUTRA is only applicable for a UE supporting concurrent GAPs and/or MUSIM gaps. Otherwise Kgap_EUTRA = 1 |  |

Table 9.4.3.3-4: Requirement to identify a newly detectable E-UTRAN TDD cell with measurement gap cancellation

| DRX cycle length (s) | TIdentify, E-UTRAN TDD (s) (DRX cycles) |  |
| --- | --- | --- |
|  | Gap/NCSG period = 40 ms, 20 ms | Gap/NCSG period = 80 ms |
| ≤0.16 | Non-DRX requirements in clause 9.4.3.2 apply | Non-DRX requirements in clause 9.4.3.2 apply |
| 0.256 | (5.12 +Lcancel,PSS/SSS *0.256) * CSSFinterRAT x Ceil(Kgap_EUTRA) ((20 +Lcancel,PSS/SSS *0.256)*CSSFinterRAT x Ceil(Kgap_EUTRA)) | (7.68 +Lcancel,PSS/SSS *0.256)* CSSFinterRAT x Ceil(Kgap_EUTRA) ((30 +Lcancel,PSS/SSS *0.256))*CSSFinterRAT x Ceil(Kgap_EUTRA)) |
| 0.32 | (6.4 +Lcancel,PSS/SSS *0.32)* CSSFinterRAT x Ceil(Kgap_EUTRA) ((20 +Lcancel,PSS/SSS *0.32)*CSSFinterRAT x Ceil(Kgap_EUTRA)) | (7.68 +Lcancel,PSS/SSS *0.32)* CSSFinterRAT x Ceil(Kgap_EUTRA) ((24 +Lcancel,PSS/SSS *0.32))*CSSFinterRAT x Ceil(Kgap_EUTRA)) |
| 0.32< DRX-cycle ≤10.24 | Note1 ((20 +Lcancel,PSS/SSS * DRX-cycle)*CSSFinterRAT x Ceil(Kgap_EUTRA)) | Note1 ((20 +Lcancel,PSS/SSS * DRX-cycle))*CSSFinterRAT x Ceil(Kgap_EUTRA)) |
| NOTE 1: The time depends on the DRX cycle length. NOTE 2: CSSFinterRAT is as defined in clause 9.4.3.2. NOTE 3: Kgap_EUTRA is only applicable for a UE supporting concurrent GAPs and/or MUSIM gaps. Otherwise Kgap_EUTRA =1 NOTE 4: If multiple concurrent gaps are configured, the gap period is the periodicity of the MG pattern associated to the E-UTRA inter-RAT frequency layer. NOTE 5: For the UE supporting measurement gap occasion cancellation provided that the configuration and conditions in clause 9.1.14.4 are met, Lcancel,PSS/SSS is the number of gap occasions cancelled during TIdentify, E-UTRAN TDD for PSS/SSS detection. |  |  |

Table 9.4.3.3-5: Requirement to measure E-UTRAN TDD cells with measurement gap cancellation

| DRX cycle length (s) | Tmeasure, E-UTRAN TDD (s) (DRX cycles) |
| --- | --- |
| ≤ 0.08 | Non-DRX Requirements in clause 9.4.3.2 apply |
| 0.128 | For configuration 2 Note3, non-DRX requirements in clause 9.4.3.2 apply, Otherwise: Note1 ((5 +Lcancel,meas * 0.128)*CSSFinterRAT x Ceil(Kgap_EUTRA)) |
| 0.128 < DRX-cycle ≤ 10.24 | Note1 ((5 +Lcancel,meas * DRX-cycle)*CSSFinterRAT x Ceil(Kgap_EUTRA)) |
| NOTE 1: The time depends on the DRX cycle length. NOTE 2: CSSFinterRAT is as defined in clause 9.4.3.2. NOTE 3: See Table 9.4.3.2-1. NOTE 4: Kgap_EUTRA is only applicable for a UE supporting concurrent GAPs and/or MUSIM gaps. Otherwise Kgap_EUTRA = 1 NOTE 5: For the UE supporting measurement gap occasion cancellation provided that the configuration and conditions in clause 9.1.14.4 are met, Lcancel,meas is the number of gap occasions cancelled during Tmeasure, E-UTRAN TDD for measurement. |  |

If higher layer filtering is used, an additional cell identification delay can be expected.

The NR – E-UTRAN TDD RSRP measurement accuracy for all measured cells shall be as specified in clause 10.2.2. The NR – E-UTRAN TDD RSRQ measurement accuracy for all measured cells shall be as specified in clause 10.2.3. The NR – E-UTRAN TDD RS-SINR measurement accuracy for all measured cells shall be as specified in clause 10.2.5.

==============End of change==============