3GPP TSG RAN Meeting #110						RP-253785

Baltimore, USA, December 8-11, 2025

Agenda Item:	9.9.4.10

Source:	Qualcomm Incorporated

Title:	Release independence for 6L

Document for:	Approval

1	Introduction

Introduction of requirements for 6Rx with 4L/6L has been a part of Rel-19 package under the UE RF enhancements WI [1]. Introduction of some of the requirements in a release independent way has been discussed in RAN4 for several meetings but the discussion was not concluded. RAN2 confirmed the feasibility of introducing release independence for these requirements in [2].

In this paper we discuss the release independence handling of the requirements for 6L and propose to introduce the 6Rx/6L capability from Rel-18.

2	Discussion

2.1 Release Independent Introduction of Requirements and Features

3GPP has introduced the release independence concept such that certain features or functionalities introduced later can be implemented from a previous release. This mechanism enables faster time to market and simpler implementation since it is not necessary for a UE to support all other features/functionalities defined up to the current release. The mechanism has been extensively used for new frequency bands (usually made available from the 1st release of a certain RAT), band combinations, higher order MIMO, etc.

The release independence mechanism has in general two components: the signaling defined in RAN2 which is introduced such that a feature can be advertised and configured from a previous release and the RAN4 requirements referenced in TS 38.307 indicating the requirements which should be met by a UE implementing the feature from an earlier release.

For the RAN2 signaling part, the RAN2 CRs with “magic” sentence “implementable from an earlier release” are captured in Annex C of TS 38.331 to indicate that the feature can be implemented from an earlier release. For example, CRs with signalling changes for BCS4/5, NS value extension and FGB2 BW classes for FR2 are listed there.

Observation 1: Release independent handling of features is possible through RAN2 signaling and RAN4 requirement referencing in 38.307.

Observation 2: The release independence mechanism is important to enable faster time to market and simpler inter-dependence of features.

Currently there are multiple features which were introduced through the release independence mechanism and are usable from earlier releases. A list of features can be found in the Annex for reference, these are not spectrum related (even though the definition of spectrum related in this case is not clear).

2.2 Release independence handling of 6Rx/6L

RAN4 has discussed introducing the requirements for 6Rx/6L and/or 6Rx/4L for FWA/handheld devices in a release independent manner from Rel-18. This issue was discussed for several meetings, however consensus could not be reached. It should be noted that RAN2 has already confirmed the feasibility of introducing the capability signaling in a release independent way from Rel-18 [2] after it was asked by RAN4.

Observation 3: RAN2 has confirmed the feasibility of introducing the 6Rx/6L capability from Rel-18.

Furthermore, another important aspect is that no other signaling changes are needed to enable this feature from a previous release. The RAN1/2 components (signaling mechanism for MIMO configuration,etc) needed to configure the UE with this feature are already defined from Rel-18. As such, release independence for 6Rx/6L can be enable only through the “magic” sentence presented in Section 2.1.

Observation 4: No other changes besides the RAN2 “magic” sentence are needed to enable release independence for 6Rx/6L.

In RAN4#117 and RAN#110 several operators have expressed interest in having this feature early implementable to address commercial deployment needs [3], [4].

Proposal 1: Introduce 6Rx/6L (including 6L and 4L) requirements and corresponding capability signaling for FWA from Rel-18 to address commercial deployment needs.

2.3 6Rx and SRS Antenna Swtiching

In [5] it is claimed that 6Rx requirements should not be introduced from Rel-18 because the SRS antenna switching feature of 3T6R is only introduced from Rel-19 (srs-AntennaSwitching3T6R-r19 in TS 38.306). We would like to point out that this capability was introduced in Rel-19 because 3Tx was the new feature introduced in Rel-19, not 6Rx as the argument given in [5] implies. Antenna switching for 1T6R and 2T6R was already introduced in Rel-17 and the UE capability is defined as srs-AntennaSwitchingBeyond4RX-r17 (t1r6 and t2r6). The description of this capability from TS 38.306 is reproduced below for convenience.

srs-AntennaSwitchingBeyond4RX-r17 Indicates whether the UE supports SRS Antenna switching for more than 4 Rx. The capability signalling comprises the following parameters: - supportedSRS-TxPortSwitchBeyond4Rx-r17 indicates a combination of supported xTyRs. It includes 11-bit bitmap, where starting from the leading / leftmost bit (bit 0), each bit corresponds to {t1r1, t2r2, t1r2, t4r4, t2r4, t1r4, t2r6, t1r6, t4r8, t2r8, t1r8}. For any indicated value, x shall be equal to or smaller than the one associated with the largest y. - entryNumberAffectBeyond4Rx-r17 indicates the lowest band entry number of the UL group (see entryNumberSwitchBeyond4Rx-r17) that impacts the DL of this band entry; - entryNumberSwitchBeyond4Rx-r17 indicates the lowest band entry of the UL group, which is defined as band entries with UL (see NOTE 1) that impact each other's UL (i.e. SRS TX port switching on any of the cells in the group will impact UL on all the cells in the group). This parameter is absent if an UL group contains only one band entry. The UE indicating support of this shall indicate support of srs-TxSwitch. For entryNumberAffectBeyond4Rx-r17 and entryNumberSwitchBeyond4Rx-r17, value 1 means first entry, value 2 means second entry and so on. The UE may include entryNumberAffectBeyond4Rx-r17/entryNumberSwitchBeyond4Rx-r17 for a band entry even if all of the bits in the supportedSRS-TxPortSwitchBeyond4Rx-r17 are set to 0 for that band entry. All DL and UL that switch together indicate the same entry number. The entry number is the band entry number in a band combination. The UE is restricted not to include fallback band combinations for the purpose of indicating different SRS antenna switching capabilities. NOTE 1: The band with UL includes a band associated with FeatureSetUplinkId set to 0 corresponding to the support of SRS-SwitchingTimeNR. NOTE 2: If reported for the same values of xTyR in supportedSRS-TxPortSwitchBeyond4Rx-r17 as reported with supportedSRS-TxPortSwitch/supportedSRS-TxPortSwitch-v1610, the reported values for entryNumberAffectBeyond4Rx-r17 and entryNumberSwitchBeyond4Rx-r17 are not valid.

Observation 5: The introduction of 3T6R support in Rel-19 does not justify limiting 6Rx requirements to Rel-19. SRS antenna switching for 6Rx was already introduced in Rel-17 through srs-AntennaSwitchingBeyond4RX-r17 (t1r6 and t2r6).

2.4 RAN4 CR to TS 38.307

RAN4 can enable 6Rx/6L feature release-independent from Rel-18 by submitting CR to Rel-19 version of TS 38.307. An example of CR to Rel-19 TS 38.307 is drafted as below.

Table 5.4-1: Additional requirements of other release independent features

| Feature | Release independent from | Requirements to be fulfilled (see 38.307 of the REL when the feature was introduced) | Further information |
| --- | --- | --- | --- |
| -------- Skip the unchanged context -------- |  |  |  |
| RF requirements for 6Rx UE | Rel-18 | Table B.4.x-1 | Support 6Rx requirements for FWA UE from Rel-18 including 4 MIMO and 6 MIMO layer requirements |
| Demodulation requirements for 6Rx UE | Rel-18 | Table B.3.y-1 | Support 6Rx requirements for FWA UE from Rel-18 including 4 MIMO and 6 MIMO layer requirements |
| NOTE 1: Rel-15 UEs supporting the high speed train are assumed to read the Rel-16 high speed train scenario information, which is broadcast to all UEs. NOTE 2: Rel-16 UEs supporting the high speed train are assumed to read the Rel-17 high speed train scenario information, which is broadcast to all UEs. NOTE 3: Rel-16 UEs supporting the high speed train are assumed to read the Rel-17 high speed train scenario information, which is broadcast to all UEs. NOTE 4: Rel-15 UEs supporting the high speed train are assumed to read the Rel-17 high speed train scenario information, which is broadcast to all UEs. |  |  |  |

-------- Skip the unchanged context --------

B.3.y	Common PDSCH demodulation and CQI requirements with 6Rx

The requirements and test cases listed in Table B.3.y-1 are specified in Rel-P version of TS 38.101-4 [5].

Table B.3.y-1: UE PDSCH demodulation requirements with 6Rx

| Section / Clause | Description |
| --- | --- |
| 5.2.x | PDSCH demodulation requirements for 6Rx TDD |
| 6.2.x | CQI requirements with 6Rx |

-------- Skip the unchanged context --------

B.4.x-1	Common UE RF requirements for 6Rx

The requirements and test cases listed in Table B.4.x-1 are specified in REL-P version of TS 38.101-1 [2].

Table B.4.x-1: Common UE RF requirements for 6Rx for single band in FR1

| Clause | Description |
| --- | --- |
| 6.2.4 | Configured output power |
| 7.3 | Reference sensitivity |
| 7.4 | Maximum input level |
| 7.5 | Adjacent Channel Selectivity |
| 7.6 | Blocking characteristics |
| 7.7 | Spurious response |
| 7.8 | Intermodulation characteristics |
| 7.9 | Spurious emissions |

Observation 6: RAN4 can enable 6Rx/6L feature release independent from Rel-18 by submitting an CR to Rel-19 TS 38.307. The above CR draft can be used as a reference. The specific CR content could be further discussed at RAN4#116bis meeting.

3	Conclusion

In this paper we briefly discussed the mechanism of introducing new features or requirements in a release independent manner and this can be easily applied to the 6Rx/6L requirements. We made the following observations and proposal;

Observation 1: Release independent handling of features is possible through RAN2 signaling and RAN4 requirement referencing in 38.307.

Observation 2: The release independence mechanism is important to enable faster time to market and simpler inter-dependence of features.

Observation 3: RAN2 has confirmed the feasibility of introducing the 6Rx/6L capability from Rel-18.

Observation 4: No other changes besides the RAN2 “magic” sentence are needed to enable release independence for 6Rx/6L.

Observation 5: The introduction of 3T6R support in Rel-19 does not justify limiting 6Rx requirements to Rel-19. SRS antenna switching for 6Rx was already introduced in Rel-17 through srs-AntennaSwitchingBeyond4RX-r17 (t1r6 and t2r6).

Observation 6: RAN4 can enable 6Rx/6L feature release independent from Rel-18 by submitting an CR to Rel-19 TS 38.307. The above CR draft can be used as a reference. The specific CR content could be further discussed at RAN4#116bis meeting.

Proposal 1: Introduce 6Rx/6L (including 6L and 4L) requirements for FWA from Rel-18 to address commercial deployment needs.

References

[1]	RP-251816, “Revised WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4”, Huawei, Hisilicon, TSG RAN#108.

[2]	R4-2520009, “Reply LS on Release Independence of 6Rx”, TSG-RAN4#117,

[3]	R4-2509968, “On Release Independence of 6Rx”, T-Mobile USA, BT, SK Telecom. TELUS, Verizon, Deutsche Telekom, Bell Mobility, Boost Mobile, Southern Linc, Telecom Italia, Telstra, TSG-RAN4#116

[4] 	RP‑253701, “On Release Independence of 6Rx”, T-Mobile USA et al., TSG-RAN#110

[5]	RP‑253512, “Discussion on 6Rx release independence”, Huawei, HiSilicon, TSG-RAN#110

Annex

List of release independent features in TS 38.307 – Table 5.4-1

Table 5.4-1: Additional requirements of other release independent features

| Feature | Release independent from | Requirements to be fulfilled (see 38.307 of the REL when the feature was introduced) | Further information |
| --- | --- | --- | --- |
| RRM requirements for high speed train scenario | Rel-15 (NOTE 1) | Table C.1-1 | Rel-16 WI NR_HST introduced band independent RRM requirements: see Table C.1-1 |
| UE demodulation requirements for high speed train scenario | Rel-15 (NOTE 1) | Table C.2-1 | Rel-16 WI NR_HST introduced band independent UE demodulation requirements: see Table C.2-1 |
| Precoding matrix indicator (PMI) reporting requirements for TypeI-SinglePanel and TypeII Codebooks with more than 8TX and up to 32TX | Rel-15 | Table D.1-1, Table D.2-1 | Rel-16 NR_perf_enh-Perf WI introduced band independent PMI reporting requirements for 16TX and 32TX TypeI-SinglePanel Codebook, and 16TX TypeII Codebook: see Table D.1-1 and Table D.2-1 respectively. |
| PDSCH demoulation requirements with LTE CRS rate matching for TDD band | Rel-15 | Table E.1-1 | Rel-16 NR_perf_enh-Perf WI introduced band independent PDSCH demoulation requirements with LTE CRS rate mataching for TDD band: see Table E.1-1. |
| RF requirements for 4Rx UEs | Rel-15 | Table B.4.10-1, Table B.4.10-2 |  |
| Transparent Tx diversity | Rel-15 | Table B.4.11-1 | Rel-17 WI NR_RF_TxD introduced transparent Tx diversity requirements: see Table B.4.11-1 |
| UE demodulation and CSI requirements for MMSE-IRC receiver for scenarios with inter cell and intra cell inter user interference | Rel-15 | Table B.3.3-1 | Rel-17 WI NR_demod_enh2-Perf: see Table B.3.3-1. These requirements are optional for Rel-15 and Rel-16 UEs and can be executed based on UE declaration. |
| RRM enhancement for FR1 high speed train scenario enhancement | Rel-16 (NOTE 2) | Table C.3-1 | Rel-17 WI NR_HST_FR1_enh introduced band independent RRM enhancement: see Table C.3-1 |
| HST-SFN CA demodulation enhancement for FR1 high speed train scenario enhancement | Rel-16 (NOTE 3) | Table C.4-1 | Rel-17 WI NR_HST_FR1_enh introduced band independent HST-SFN CA demodulation enhancement: see Table C.4-1 |
| HST-DPS CA demodulation enhancement for FR1 high speed train scenario enhancement | Rel-15 (NOTE 4) | Table C.4-1 | Rel-17 WI NR_HST_FR1_enh introduced band independent HST-DPS CA demodulation enhancement: see Table C.4-1 |
| PDSCH absolute physical layer throughput requirements with link adaptation | Rel-17 | Table B.3.4-1 | Rel-18 WI NR_demod_enh3-Perf: see Table B.3.4-1. These requirements are optional for Rel-17 and can be executed based on UE declaration. |
| RF requirements for 8Rx UE | Rel-17 | Table B.4.17-1, Table B.4.17-2 |  |
| Demodulation requirements for 8Rx UE | Rel-17 | Table B.3.5-1 |  |
| NOTE 1: Rel-15 UEs supporting the high speed train are assumed to read the Rel-16 high speed train scenario information, which is broadcast to all UEs. NOTE 2: Rel-16 UEs supporting the high speed train are assumed to read the Rel-17 high speed train scenario information, which is broadcast to all UEs. NOTE 3: Rel-16 UEs supporting the high speed train are assumed to read the Rel-17 high speed train scenario information, which is broadcast to all UEs. NOTE 4: Rel-15 UEs supporting the high speed train are assumed to read the Rel-17 high speed train scenario information, which is broadcast to all UEs. |  |  |  |