3GPP TSG RAN Meeting #78	RP-172535

Lisbon, Portugal, 18 – 21 December 2017

Agenda item:	9.3.2

Source:	Huawei, HiSilicon

Title:	Consideration on IMT-2020 self evaluation: area traffic capacity

Document for:	Information

1	Introduction

At RAN#77 meeting, the preliminary work plan of self evaluation was agreed in [1], where a three-step plan is made. In [2], it is proposed that 3GPP starts Step 2 activity for self evaluation study, which includes the performance evaluation against the eMBB, mMTC and URLLC technical performance requirements as defined in Report ITU-R M.2410 (see [3]).

In this document, we focus on one of the technical performance requirement, i.e., area traffic capacity. The initial evaluation results and observations are provided.

2	Definition and requirements of area traffic capacity

In Report ITU-R M. 2410, the following definition and requirements of area traffic capacity are defined in subsection 4.6.

Area traffic capacity is the total traffic throughput served per geographic area (in Mbit/s/m2). The throughput is the number of correctly received bits, i.e. the number of bits contained in the SDUs delivered to Layer 3, over a certain period of time. This can be derived for a particular use case (or deployment scenario) of one frequency band and one TRxP layer, based on the achievable average spectral efficiency, network deployment (e.g., TRxP (site) density) and bandwidth. Let W denote the channel bandwidth and ρ the TRxP density (TRxP/m2). The area traffic capacity Carea is related to average spectral efficiency SEavg through equation (6). Carea = ρ × W × SEavg (6) In case bandwidth is aggregated across multiple bands, the area traffic capacity will be summed over the bands. This requirement is defined for the purpose of evaluation in the related eMBB test environment. The target value for Area traffic capacity in downlink is 10 Mbit/s/m2 in the Indoor Hotspot – eMBB test environment.

Therefore, area traffic capacity can be derived analytically through equation (6), where SEavg is the average spectral efficiency as defined in subsection 4.5 in Report ITU-R M.2410.

Observation 1: Area traffic capacity can be derived analytically through equation (6), where SEavg is the average spectral efficiency as defined in subsection 4.5 in Report ITU-R M.2410.

3	Evaluation methodology of area traffic capacity

From Report ITU-R M.2412 (see [4]), we can see the following evaluation methodology.

The evaluation is conducted in Indoor Hotspot-eMBB test environment where a single band is considered. Area traffic capacity is derived based on the achievable average spectral efficiency, TRxP density and bandwidth. Let W denote the channel bandwidth and ρ the TRxP density (TRxP/m2). The area traffic capacity Carea is related to average spectral efficiency SEavg as follows: Carea = ρ × W × SEavg

It should be noted that the TRxP density ρ needs to be calculated according to the network layout defined in subsection 8.3.1 in Report ITU-R M.2412. According to Figure 8-F1 illustrated in Report ITU-R M.2412 (see below), the simulation area of Indoor Hotspot – eMBB test environment is S=120×(15+20+15)=6000 m2. And there are 12 BSs/sites which are placed in 20 meter spacing in this simulation area. It is stated in Report ITU-R M.2412 that “The type of site deployed (e.g. one TRxP per site or 3 TRxPs per site) is not defined and should be reported by the proponent”. When one TRxP per site is assumed, there will be 12 TRxPs in the simulation area, and in this case, ρ=12 TRxP/6000 m2=0.002TRxP/m2. When 3 TRxPs per site is assumed, there will be 36 TRxPs in the simulation area, and in this case, ρ=12 TRxP/6000 m2=0.002TRxP/m2. Therefore, one observe that

Observation 2: According to Indoor Hotspot network layout as defined in Report ITU-R M.2412, the TRxP density is calculated to be:

-   For one TRxP per site (12 TRxP in simulation area), ρ=0.002TRxP/m2

-   For 3 TRxP per site (36 TRxP in simulation area), ρ=0.006TRxP/m2

Figure 8-F1

Indoor Hotspot sites layout

4	Initial evaluation results of area traffic capacity

Based on the definition and evaluation methodology, we derive the initial evaluation results of area traffic capacity in Indoor Hotspot-eMBB at 4GHz based on the average spectral efficiency evaluation provided in [5]. It is noted that more TRxPs can help to reach the target with smaller bandwidth. Detailed evaluation assupmtions are found in [5].

Table1 – Downlink area traffic capacity (Mbit/s/m2) in Indoor Hotspot-eMBB at 4GHz

| System bandwidth (MHz) | 12 TRxP (TRxP density ρ=0.002TRxP/m2) |
| --- | --- |
| W =300 | 6.9 |
| W = 350 | 8.1 |
| W = 400 | 9.2 |
| W = 450 | 10.4 |
| W = 500 | 11.5 |

Observation 3: IMT-2020 requirement on area traffic capacity can be met by NR at 4GHz.

5	Conclusion

In this document, we discussed the definition, requirement and evaluation methodology of area traffic capacity. After that we present the initial evaluation results of downlink area traffic capacity in Indoor-Hotspot eMBB test environment. Based on the discussions, the following observations are obtained.

Observation 1: Area traffic capacity can be derived analytically through equation (6), where SEavg is the average spectral efficiency as defined in subsection 4.5 in Report ITU-R M.2410.

Observation 2: According to Indoor Hotspot network layout as defined in Report ITU-R M.2412, the TRxP density is calculated to be:

-   For one TRxP per site (12 TRxP in simulation area), ρ=0.002TRxP/m2

-   For 3 TRxP per site (36 TRxP in simulation area), ρ=0.006TRxP/m2

Observation 3: IMT-2020 requirement on area traffic capacity can be met by NR at 4GHz.

Reference

- RP-172101, “WF on Work plan of Self Evaluation SI”, Huawei, Ericsson, Telecom Italia, September 2017.

- RT-172532, “Work plan for self evaluation towards 3GPP Update Submission to ITU-R”, Huawei, Ericsson, Telecom Italia, December 2017.

- Report ITU-R M.2410, “Minimum requirements related to technical performance for IMT-2020 radio interface(s)”, Nov. 2017.

- Report ITU-R M.2412, “Guidelines for evaluation of radio interface technologies for IMT-2020”, Nov. 2017.

- R1-1719418, “Consideration on self evaluation of eMBB spectral efficiency for IMT-2020”, Huawei, HiSilicon Nov. 2017.