3GPP TSG RAN meeting #83										RP-190240

Shenzhen, China, March 18-21, 2019

## Status Report to TSG

Agenda item:			9.4.6

| WI / SI Name | Study on NR-based Access to Unlicensed Spectrum |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: No |  | Testing part: No |
| Acronym | NR_unlic |  |  |  |  |
| Unique ID | 750045 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-182878 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: xx/xxxx | Core part: 12/2019 | Performance part: 06/2020 | Testing part: mm/yyyy |  |
| Overall Completion level | Study Item: Xx% | Core part: 20% | Performance Part: 5% | Testing part: xx% |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | TSG RAN1 |
| --- | --- | --- |
| Rapporteur | Name | Jing Sun |
|  | Company | Qualcomm |
|  | Email | jingsun@qti.qualcomm.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

#### 2.1.1	Agreements

Agreements in RAN1 Adhoc #1901

Agreement:

- UE assumes 30KHz SCS for SS/PBCH block for 5GHz band and 6GHz band if the SCS is not indicated by higher layers.

- Support configuration by higher layers of 15KHz or 30KHz SCS for SS/PBCH block

- Include this agreement in a LS to RAN4 (cc RAN2) for inclusion in specs managed by RAN4

Conclusion:

No changes are required to the time and frequency position of the PSS/SSS/PBCH relative to each other in one PSS/SSS/PBCH block.

Agreement:

The Type0-PDCCH monitoring configuration for NR-U should satisfy at least the following properties:

- TDM of Type0-PDCCH and SSB similar to existing pattern 1 (already agreed)

- Support the monitoring of Type0 PDCCH of the 2nd SSB position in a slot in the gap between 1st and 2nd SSB within the slot

  - FFS start at symbol #6 of #7 or both

- FFS: The Type0-PDCCH candidates associated with an SSB are confined within a slot carrying the associated SSB (with the same QCL assumptions)

Agreement:

Companies are encouraged to provide results comparing the different alternatives using the following simulation assumptions to select between alternative PRACH designs.

- The Rel-15 PRACH design should be simulated as a baseline

| Property | Value |
| --- | --- |
| Carrier frequency | 5 GHz |
| Channel model | TDL-C |
| Delay scaling | 10ns, 100 ns |
| Antenna configuration at BS(1) | (M,N,P) = (1,1,2) with omni-directional antenna element |
| Antenna configuration at UE | Single omni-directional antenna element |
| Antenna port virtualization | No beamforming and no beam selection |
| Frequency offset | 0.05ppm (fixed) at TRP, and 0.1 ppm (fixed) at UE |
| UE speed | 3 km/h |
| Initial timing offset | Uniformly distributed in [0, 1.2 µs (corresponding to 300 m ISD)] Optional: Uniformly distributed in [0, 2 µs (corresponding to 500 m ISD)] |
| PRACH format | A1 with other formats optional |
| Subcarrier spacing | 15/30 kHz. (with other SCS optional) |
| PRACH sequence and frequency resource allocation | For evaluation purpose, the Rel-15 PRACH ZC sequence (with possible length change) should be simulated. Additional/new sequences can be simulated. Each company should provide details on the sequence (type and length) and the resource allocation (e.g., Alt1~Alt4 and detailed mapping). |
| Total number of preambles per cell | 64, each company should provide details on how these 64 preambles are generated |
| Preamble detector | Each company should provide details on used algorithm |
| Interference assumption | No interference. Optional: -3/0/3dB interference power compared with target PRACH |
| Detection Criteria | 1% maximum mis-detection probability(2) |
|  | 0.1% maximum false alarm probability(3) |
|  | maximum timing estimation error being 50% of the normal CP length |
| Formatting of results (please also reference Section 8 of R1-1704144 for reporting formats) | Mis-detection probability vs. SNR |
|  | False alarm probability vs. SNR(4) |
|  | CDF of timing estimation error |
|  | PRACH capacity (maximum number of preambles) |
|  | Peak-to-average power ratio and cubic metric |
|  | MCL(5) |
| (1) See Table 7-1 of R1-1704144 (2) The missed detection probability is defined as the ratio between the total number of transmitted preambles that are either not detected, or detected as a different preamble, or detected but with timing error greater than the maximum value (i.e., 50% of normal CP length), and the total number of transmitted preambles within an observation interval. (3) Maximum false alarm probability refers to the case when input at receiver is noise only (considering 64 preamble detectors as in 3GPP TS 36.104, section 8.4.1). (4) False alarm probability is defined as the ratio of total number detected but not transmitted preambles, and the total number of possible detection occurrences, where each occurrence (occurrence refers to 64 detections, one for each of the 64 preambles in a cell) is one potential preamble transmission in a RO. (5) In the MCL calculation, needs to consider the maximum transmit power supported by the PRACH design under PSD limitation and PAPR/EVM characteristic of the design. Note: Assumptions on the following should be stated - use of a guard band (if any) - definition of SNR - signal bandwidth used |  |

Agreement:

- The UE may assume the presence of a signal, such as the DMRS in any [PDCCH or GC-PDCCH] transmission, to detect transmission bursts by the serving gNB, to enable power saving by not necessitating performing blind decodes to detect the transmission burst (Note: The power saving possibility by not necessitating blind decodes assumes performance relaxation for PDCCH decoding is not needed. Also, this does not mandate a two-step PDCCH decoding process for the UE with respect to DMRS detection).

- If a preamble transmitted at the start of a burst is agreed to be specified (this does not preclude usage of preambles by implementation), it may be used in addition to the DMRS of PDCCH/GC-PDCCH or any other signals in the gNB transmission to detect the start of transmission bursts by the serving gNB and potentially for power saving

  - Note: Whether a preamble, if defined, can be used for power saving in all cases depends on the details of the design.

- Note: Other signals present in the transmission burst may also be used for the purpose of detection of the transmission burst

- FFS: Potential enhancements to DMRS design to address issues with detection probability

- The payload of a PDCCH and/or GC-PDCCH transmission can contain information regarding COT structure that may be used by the UE for power saving

Agreement:

For interlace transmission of at least PUSCH and PUCCH, the following PRB-based interlace design is supported for the case of 20 MHz carrier bandwidth:

a.	15 kHz SCS: M = 10 interlaces with N = 10 or 11 PRBs / interlace

b.	30 kHz SCS: M = 5 interlaces with N = 10 or 11 PRBs / interlace

Note: PRACH design to be considered separately, including multiplexing aspects with PUSCH and PUCCH

Working assumption:

- For a given SCS, the following interlace design is supported at least for PUSCH:

  - Same spacing (M) between consecutive PRBs in an interlace for all interlaces regardless of carrier BW, i.e., the number of PRBs per interlace is dependent on the carrier bandwidth

  - Point A is the reference for the interlace definition

- For 15 kHz SCS, M = 10 interlaces and for 30 kHz SCS, M = 5 interlaces for all bandwidths

- FFS: Interlace design for PUCCH for bandwidths greater than 20 MHz

- FFS: Whether and how partial interlace allocation is supported

Agreement:

- A gap (DLUL, ULUL, or UL DL) of a specific duration is created using one or more of:

  - Timing Advance

  - CP extension

    - max value of not more than one OFDM symbol

  - Shortening of DL or UL transmission duration by one or more OFDM-symbol(s) by puncturing or rate matching

  - Note: the mechanisms applied in each case may be different for different SCSs

- FFS: how to signal the way of creating the gap to the UEs

Agreement:

- RRC parameter dl-DataToUL-ACK supports a value that can be signaled by PDSCH-to-HARQ-timing-indicator, which indicates that the UE needs to store the HARQ A/N feedback result for the corresponding PDSCH, and which does not provide any timing for the transmission of this HARQ A/N feedback result

Conclusion:

- No additional value of K1 signaled by PDSCH-to-HARQ-timing-indicator needs to be introduced for the purpose of requesting feedback outside the COT (i.e. Alt3 in TR38.889 section 7.2.1.3.3)

  - Further discuss the details for determining the LBT type for the UE transmission of HARQ A/N feedback for this case

  - Further discuss whether additional values need to be introduced for the purpose of requesting feedback inside the COT

    - If additional values are introduced these can also be used for requesting feedback outside the COT

  - Note: This does not necessitate that the UE needs to know whether the feedback is inside or outside the COT at the time when the feedback is requested

Agreement:

For enabling multiple opportunities for HARQ A/N transmission and for cross-COT HARQ-ACK feedback, at least the following is supported:

gNB requests/triggers feedback for PDSCH from earlier COT(s) or additional reporting of earlier HARQ feedback, where the exact HARQ feedback timing and resource is provided to the UE in another DCI (in the same or in another COT)

Agreement:

Cross-carrier HARQ re-transmissions will not be discussed for NR-U in Rel-16.

Agreement:

For configured grant resource configuration in time domain, the following alternatives are to be studied with more detailed proposal and analysis, strive to down-select in RAN1#96:

- Alt. 1: Bitmap based approach as baseline with potential enhancement

  - Companies are encouraged to provide detailed design in next meeting

- Alt. 2: NR Rel-15 based time domain resource allocation approach as baseline with potential enhancement

  - Companies are encouraged to provide detailed design in next meeting

Agreement:

- Support multiple UE starting time offsets with sub-symbol granularity with FeLAA AUL approach as the baseline

  - FFS: Enhancements specific to NRU

- Companies are encouraged to provide views and analysis on the following issues:

  - Whether to support allowing the UE to start transmission later than the starting symbol as indicated in configured grant based on LBT outcome

    - If yes, multiple starting positions within a slot for a configured grant configuration;

      - Alt. 1: subset of symbols

      - Alt. 2: any symbol

      - FFS: gNB knowledge of starting symbol, whether UE indicates to gNB

      - FFS signaling details

      - FFS: whether similar design for scheduled grant and configured grant

  - Whether the ending symbol can be punctured

  - Whether the position of the ending symbol can be shifted depending on the starting position due to LBT procedures

Conclusion:

The following aspects should be discussed further as part of the channel access discussions

- Contention window adjustment

- Details of COT sharing related to NRU configured grant including details and limitations on UE-initiated COT sharing with gNB and configured grant UL transmissions within gNB acquired COT

Agreement:

CG-UCI should at least include the following information:

- HARQ ID

- NDI

- RV

- COT sharing information, FFS details

- FFS: other information including UE ID

Agreement:

- For wideband operation in DL with a single serving cell operation within a carrier with bandwidth larger than 20 MHz

  - Multiple BWPs can be configured, single BWP activated, gNB may transmit PDSCH on parts or whole of single active BWP where CCA is successful at gNB (i.e., option 2 and 3 from previous agreement)

    - FFS: Restrictions on supportable gaps and combinations of gaps between discontiguous blocks where

      - each block spans contiguous (one or) multiple successful LBT sub-bands

      - each gap spans one or multiple contiguous unsuccessful LBT sub-bands

    - FFS: Transmission bandwidth adaptation delay, potentially different delay for e.g., different number of supported gaps, different transmission bandwidths and different positions of the LBT sub-bands where transmissions occur

    - FFS: Limit on the occupied LBT sub-bands due to regulation and coexistence considerations (not intended to imply that regulation and coexistence considerations will not be addressed)

    - FFS: Whether/how to indicate gNB’s transmitted LBT sub-bands

    - FFS: Enhancements to PDCCH/PDSCH configuration/transmission for the parts of BWP where gNB does not transmit due to CCA failure

- Send LS to RAN4 to inform above decision with the description that RAN1 requires RAN4’s feedback on the first three FFS parts in addition to what was requested in earlier LSs.

Agreement:

Operation with multiple active BWPs for a carrier on unlicensed bands is not supported for DL or UL at least in Rel-16 NR-U WI.

- Inform RAN2 of this decision

R1-1901446	[DRAFT] Reply to reply LS on wideband carrier operation for NR-U	LG Electronics

Final LS is agreed with the following changes in R1-1901460

- Include RAN WG2 in the “To” field

- Add an action to RAN WG2: “RAN1 respectfully asks RAN2 to take the above RAN1 agreements into account in their further work”

- Delete the corresponding statement requesting action from RAN2 above in the main body.

- Modify a statement in the main body as follows: “RAN1 respectfully asks RAN4 for a response to FFS parts in the above agreement, in addition to any pending input to questions as requested in [1] not yet answered.”

Conclusion:

The channel access aspects of wideband operation should be discussed further as part of the channel access discussions

Agreements in RAN1 #96

R1-1903576	[DRAFT] LS to ETSI BRAN on EN 301 893	Qualcomm Incorporated

Final LS is agreed in R1-1903757 with the following modifications:

- Replace the last sentence in the third paragraph with “Hence, it is RAN1’s understanding that for the case where more than one grant is provided to a responding device, 3GPP Rel-14/15 LTE-LAA specifications allow behaviour that is consistent with the provided clarifications and this is also expected to be the case for NR-U specifications currently being developed.”

- Replace the last two sentences of the second last paragraph with, “There is no consensus on the proposal to (1) ban the use of no LBT transmissions and (2) to restrict the use of short LBT transmissions so that it can only be used 1% of time rather than 5% as currently defined in the clause on Short Control Signalling Transmissions. There is also no consensus on limiting NR-U DRS transmissions to a 1% limit. Furthermore, reducing 5% transmission limit to 1% transmission limit would not be consistent with the behaviour of a device implementing LTE-LAA and is not preferred.”

- Modify the action as “RAN1 kindly requests ETSI TC BRAN to take the above information and requests for clarifications into account.”

Agreement:

- Down-select from the following options for SSB pattern (symbol index starts at 0)

  - Option 1: SSBs are at symbols (2,3,4,5) and (8,9,10,11) in the slot

  - Option 2: SSBs are at symbols (2,3,4,5) and (9,10,11,12) in the slot

- The down-selected pattern applies no matter if SSB SCS is indicated by higher layer or not, and no matter if RMSI is transmitted or not.

Agreement:

- The SCS for all SSBs and Coreset #0 on a carrier is always the same for operation of NR in unlicensed spectrum.

- CORESET #0 frequency domain resource configuration should be 48 RBs for 30KHz SCS and 96 RBs for 15KHz SCS.

Agreement:

For PRACH evaluations, following metrics should be provided:

| Parameter | Value | Notes |
| --- | --- | --- |
| Scheme |  | Eg. Alt4-ZC139x2 |
| SCS |  | 15KHz or 30KHz |
| PRACH sequence length (L_RA) |  | Eg. 139, |
| # of repetition (R) |  | If repetition of sequence is used in freq domain |
| N_cs |  | Eg. 11 |
| # of RBs used for one RO (N_RB) |  | # of RBs occupied by PRACH. Eg. 12 for ZC139 design |
| # of interlaces used by one RO (N_interlace) |  | # of uniform interlaces (M=5 for 30KHz and M=10 for 15KHz) with RBs used for one PRACH RO |
| RACH frequency occupancy (MHz) |  | The actually used bandwidth with one RO, SCS*L_RA*R |
| Noise level, Np (dBm) |  | Np= -174+10*log10(SCS*L_RA*R)+NF NF=-5dB |
| SNR (dB) |  | SNR needed at 1% misdetection, read from simulation curve |
| P_max (dBm) |  | Maximum allowed transmit power under PSD limit of 10dBm/MHz measured in any 1MHz chunk and considers the RBs used by the proposed scheme |
| Backoff (dB) |  | Backoff is computed as 95% percentile of CCDF of [cubic metric] over the preambles in the RO. Note: If cubic metric is not used, information on the backoff metric used should be provided. |
| P_TX (dBm) |  | P_TX=min(P_max, 23- Backoff) is maximum allowed transmit power for the waveform considering backoff |
| MCL (dB) |  | MCL = P_TX-SNR-Np |
| N_FDM |  | # of ROs in 20MHz |
| Capacity |  | Across all ROs in 20MHz. Should report any constraints on ISD for the scheme evaluated. |

Agreement:

- Support short and long PUCCH durations based on enhancements of at least Rel-15 PUCCH formats PF2 and PF3. The enhancements include at least the following aspects:

  - For a 20 MHz carrier bandwidth, support mapping to physical resources of at least one full interlace

  - Mechanism to support user multiplexing for both data and reference symbols of PUCCH

  - The following aspects are FFS:

    - Support for small payloads (1 and 2 bits)

      - Alt-1: Support both small payloads and larger payloads (> 2 bits) for enhanced PF2 and enhanced PF3

      - Alt-2: Small payloads are supported by enhanced PF0 and/or enhanced PF1

    - Whether or not to replace DFT-s-OFDM with CP-OFDM for the enhanced PF3

Agreement:

Suppport configuration of an SRS resource in additional OFDM symbol locations other than the last 6 symbols of a slot with PUSCH and SRS time division multiplexed as in Rel-15.

- FFS: which symbols locations.

Agreement:

Sub-PRB interlace design for PUSCH and PUCCH is not supported.

Conclusion:

A common preamble and a common energy detection threshold between NR-U and other technologies for 5 and 6 GHz was discussed. There is no consensus on these aspects at this stage.

Conclusion:

Cat 2 is not used for initiating a UE transmission outside of a gNB COT for the following channels/signals (or any combination of them):

- PUSCH (with or without UCI),

- SRS-only,

- PUCCH-only

Note:

- Cat 4 for these channels was already agreed during the study item

- This does not preclude the use of Cat 2 for transmission on a LBT bandwidth if it is allowed for the case of transmission on multiple LBT bandwidths

Agreement:

For initiation of a gNB transmission:

- LBT other than Cat 4 is not used for DRS multiplexed with unicast data

- LBT other than Cat 4 is not used for PDCCH and/or PDSCH transmission outside of DRS.

Note:

- This does not preclude the use of Cat 2 for transmission on a LBT bandwidth if it is allowed for the case of transmission on multiple LBT bandwidths

Agreement:

LBT other than Cat4 is not considered for UL transmissions that are part of a RACH procedure that initiate a channel occupancy

- Note: This does not preclude the use of Cat 2 for transmission on a LBT bandwidth if it is allowed for the case of transmission on multiple LBT bandwidths

Agreement:

- For a given cell, UE may assume SS/PBCH blocks in the same candidate position within the DRS transmission window are QCL across DRS transmission windows

  - Alt1: The PBCH DMRS sequence index is also the same

  - Alt2: The PBCH DMRS sequence index may be different

  - Note: The first candidate position of the DRS transmission window is located at the first half slot of a half frame

- FFS: QCL assumption for SSBs in different candidate positions within a DRS transmission window and across DRS transmission windows

Agreement:

- At least the functionalities of Rel-13 LTE-LAA RSSI and channel occupancy reporting as a baseline should be supported

- FFS:

  - Enhanced RSSI metrics, for e.g., sub-band-level interference measurements in a wideband operation scenario

  - Reporting of a new medium contention/load metric other than channel occupancy

  - Any modification of the parameters of the Rel-15 SMTC for operation in unlicensed spectrum

Agreement:

- An RLM measurement window for serving cell RLM measurements based on SSBs in the DRS is supported for in-sync and out-of-sync evaluations.

  - FFS: How RLM measurement window is indicated or determined and relation to DRS transmission window

  - FFS: Whether or not an SSB can fall outside the measurement window and, if so, whether it can be used for in-sync and out-of-sync evaluations.

  - FFS: Any relationship of RLM measurements based on CSI-RS to the measurement window.

- FFS: Mechanism to handle missing RLM-RS due to LBT failure

Agreement:

Scheduling PUSCH over multiple slots/mini-slots by single DCI supports at least multiple continuous PUSCHs with separate TBs

- Each TB is mapped to at most one slot or one mini-slot

Agreement:

For PUSCH transmitted using CG, CBG-based retransmission is supported at least by using dedicated scheduled resource allocated by an UL grant.

- FFS: CBG-based retransmission using a configured grant

- Note: Include this agreement in an LS to RAN2 informing them of relevant RAN1 agreements

Agreement:

For initial transmission on configured grant resource, HARQ retransmission on configured grant resource upon configured grant timer expiration (assume NACK if no ACK is received) is supported

- Note: Include this agreement in an LS to RAN2 informing them of relevant RAN1 agreements

Agreement:

When a UE initiates a channel occupancy with a transmission using a configured grant, it can signal at least the following

- The duration that the gNB is allowed to transmit in the channel occupancy initiated by the UE

- FFS:

  - How the duration is signalled

  - Whether the UE should signal continued use of the COT for its own transmissions

  - LBT priority class

#### 2.1.2	Remaining Open issues

The objectives from WID related to RAN1 are as follows:

-	Physical layer aspects including [RAN1]:

-	Frame structure including single and multiple DL to UL and UL to DL switching points within a shared COT with associated identified LBT requirements (TR Section 7.2.1.3.1).

-	NR-U supports a mode of operation where for a carrier and at least for intra-band CA on serving cells on unlicensed bands, all DL channels / signals can be operated with the same numerology, and all UL channels / signals can be operated with the same numerology.

-	Subcarrier spacing for control and data channels supporting 15kHz, 30kHz, and 60kHz (air-interface perspective; optionality to be discussed separately).

-	Wide band operation (in integer multiples of 20MHz) for DL and UL for NR-U supported with multiple serving cells, and wideband operation (in integer multiples of 20MHz) for DL and UL for NR-U supported with one serving cell with bandwidth > 20MHz with potential scheduling constraint subject to input from RAN2 and RAN4 on feasibility of operating the wideband carrier when LBT is unsuccessful in one or more LBT subbands within the wideband carrier. For all wide-band operation cases, CCA is performed in units of 20MHz (at least for 5GHz).

-	NR-U Discovery Reference Signal (DRS) containing at least SS/PBCH block burst set transmission and possibly CSI-RS, RMSI-CORESET(s)+PDSCH(s), OSI and paging with properties and extensions from NR Rel-15 in line with the agreements during the study phase (TR 38.889, Section 7.2.1.2). 60kHz based SSB/PBCH block is outside the scope of the WI.

-	PRACH including possible extension of PRACH format(s) in line with agreements during the SI phase (TR 38.889, Section 7.2.1.2) to support minimum bandwidth requirement given by regulation. Determine the applicability of Rel-15 NR formats to NR-U operation.RAN1 should decide whether 60 kHz subcarrier spacing for PRACH is supported, based on a unified design with 15 kHz and 30 kHz PRACH for meeting occupied channel bandwidth (OCB) requirements.

-	UL control including extension of PUCCH format(s) to support PRB-based frequency block-interlaced  transmission and use of Rel-15 NR PUCCH formats 2 and 3 for NR-U operation. Applicability of sub-PRB frequency block-interlaced transmission for 60kHz to be decided by RAN1.

-	UL data channel including extension of PUSCH to support PRB-based frequency block-interlaced transmission; support of multiple PUSCH(s) starting positions in one or multiple slot(s) depending on the LBT outcome with the understanding that the ending position is indicated by the UL grant; design not requiring the UE to change a granted TBS for a PUSCH transmission depending on the LBT outcome. The necessary PUSCH enhancements based on CP-OFDM. Applicability of sub-PRB frequency block-interlaced transmission for 60kHz to be decided by RAN1.

-	SRS including the introduction of additional flexibility in configuring/triggering SRS in line with agreements during the study phase.

-	For DL data channel, support of multiple PDSCH starting positions.

-	Mechanism to detect a gNB’s transmission burst in line with the TR 38.889, Section 7.2.1.2 related to UE power consumption.

-	DL control in line with the agreements during the study phase (TR 38.889, Section 7.2.1.2) including extensions allowing dynamic change of the time domain instances in which the UE is expected to receive PDCCH, modifications enabling DRS transmissions without gaps in the time-domain, and indication of time domain COT structure;

-	Physical layer procedure(s) including [RAN1, RAN2]:

-	For LBE, channel access mechanism in line with agreements from the NR-U study item (TR 38.889, Section 7.2.1.3.1). Specification work to be performed by RAN1.

-	For FBE, channel access mechanism in line with agreements from the NR-U study item (TR 38.889, Section 7.2.1.3.1). FBE is intended for environments where the absence of Wi-Fi is guaranteed (e.g., by level of regulations, private premises policies, etc. Further, the targeted scenario is limited to a single NR-U network (i.e. single operator) in the operating band and geographic area. Specification work to be performed by RAN1.

- 	Initial access: specify required NR modifications to increase the maximum number of candidate SS/PBCH block positions within the DRS transmission window; to handle reduced SS/PBCH block and RMSI transmission opportunities due to LBT failure; to determine frame timing and QCL assumptions from the detected SS/PBCH block; single SS/PBCH block numerology assumed per band for Pcells in unlicensed spectrum. (RAN1)

- 	Random access: specify required NR modifications to enhance RACH procedure in line with the agreements during the study phase, including 4-step RACH modifications to handle reduced Msg 1/2/3/4 transmission opportunities due to LBT failure (RAN1/RAN2); LBT for 2-step RACH and application of PRACH and PUSCH format improvements for NR-U to 2-step RACH. (RAN1

-	Scheduling request: specify required NR modifications due to LBT failure in line with agreements during the study phase. (RAN1/RAN2)

- 	RLM/RRM extensions for NR-U operation due to uncertain and reduced transmission opportunities for DL signals and channels due to LBT failure in line with agreements during the study phase (NR-U TR section 7.2.1.3.2), including configuring different DRS Measurement Time Configuration (DMTCs) for RRM and RLM respectively, identifying the set of RLM-RSs to measure, which set(s) are used for in-sync, out-of-sync evaluations, potential definition of a metric to accurately identify unsuccessful detection of RLM-RS. Support RSSI reporting. Define a metric to measure channel occupancy or medium contention and its corresponding reporting. (RAN1/RAN2)

- 	HARQ operation: NR HARQ feedback mechanisms are the baseline for NR-U operation with extensions in line with agreements during the study phase (NR-U TR section 7.2.1.3.3), including immediate transmission of HARQ A/N for the corresponding data in the same shared COT as well as transmission of HARQ A/N in a subsequent COT. Potentially support mechanisms to provide multiple and/or supplemental time and/or frequency domain transmission opportunities. (RAN1)

-	Scheduling multiple TTIs for PUSCH in-line with agreements from the study phase (TR 38.889, Section 7.2.1.3.3). (RAN1)

- 	Configured Grant operation: NR Type-1 and Type-2 configured grant mechanisms are the baseline for NR-U operation with modifications in line with agreements during the study phase (NR-U TR section 7.2.1.3.4). (RAN1)

-	CSI: NR Rel-15 CSI feedback mechanism is the baseline for NR-U operation. Enhancements can be considered in line with agreements from the NR-U study item (e.g., TR 38.889, Section 7.2.1.3.1). (RAN1)

- 	Data multiplexing aspects (for both UL and DL) considering LBT and channel access priorities. (RAN1/RAN2)

## 2.2	RAN2

#### 2.2.1	Agreements

On RLM:

Consistent LBT failures can lead to RLF, at least for UL transmissions, for which consistent failures can currently eventually lead to RLF

On DRX:

In NR-U, DRX On-duration starts as in Rel-15 NR (except for potentially have a new switch trigger to go to short DRX).

One DRX configuration for one MAC entity (no change)

FFS if DRX active time somehow be extended, or go to short DRX, by a non-data DL transmission (not WUS)

On Paging:

A UE can be configured for an additional number of monitoring occasions at or after or before (FFS) its calculated PO (when the paging message can be transmitted)

FFS dynamic extension

FFS dynamic termination

There will be four email discussions until RAN2#105bis to make progress on the following topics:

- Connected mode measurements (Qualcomm)

- LBT impact in MAC (except RACH and SR) (Huawei)

- RACH 4-step and SR (Oppo)

- Configured Grants (Ericsson)

#### 2.2.2	Remaining Open issues

The objectives from WID related to RAN2 are as follows:

-	Physical layer procedure(s) including [RAN1, RAN2]:

-	For LBE, channel access mechanism in line with agreements from the NR-U study item (TR 38.889, Section 7.2.1.3.1). Specification work to be performed by RAN1.

-	For FBE, channel access mechanism in line with agreements from the NR-U study item (TR 38.889, Section 7.2.1.3.1). FBE is intended for environments where the absence of Wi-Fi is guaranteed (e.g., by level of regulations, private premises policies, etc. Further, the targeted scenario is limited to a single NR-U network (i.e. single operator) in the operating band and geographic area. Specification work to be performed by RAN1.

- 	Initial access: specify required NR modifications to increase the maximum number of candidate SS/PBCH block positions within the DRS transmission window; to handle reduced SS/PBCH block and RMSI transmission opportunities due to LBT failure; to determine frame timing and QCL assumptions from the detected SS/PBCH block; single SS/PBCH block numerology assumed per band for Pcells in unlicensed spectrum. (RAN1)

- 	Random access: specify required NR modifications to enhance RACH procedure in line with the agreements during the study phase, including 4-step RACH modifications to handle reduced Msg 1/2/3/4 transmission opportunities due to LBT failure (RAN1/RAN2); LBT for 2-step RACH and application of PRACH and PUSCH format improvements for NR-U to 2-step RACH. (RAN1

-	Scheduling request: specify required NR modifications due to LBT failure in line with agreements during the study phase. (RAN1/RAN2)

- 	RLM/RRM extensions for NR-U operation due to uncertain and reduced transmission opportunities for DL signals and channels due to LBT failure in line with agreements during the study phase (NR-U TR section 7.2.1.3.2), including configuring different DRS Measurement Time Configuration (DMTCs) for RRM and RLM respectively, identifying the set of RLM-RSs to measure, which set(s) are used for in-sync, out-of-sync evaluations, potential definition of a metric to accurately identify unsuccessful detection of RLM-RS. Support RSSI reporting. Define a metric to measure channel occupancy or medium contention and its corresponding reporting. (RAN1/RAN2)

- 	HARQ operation: NR HARQ feedback mechanisms are the baseline for NR-U operation with extensions in line with agreements during the study phase (NR-U TR section 7.2.1.3.3), including immediate transmission of HARQ A/N for the corresponding data in the same shared COT as well as transmission of HARQ A/N in a subsequent COT. Potentially support mechanisms to provide multiple and/or supplemental time and/or frequency domain transmission opportunities. (RAN1)

-	Scheduling multiple TTIs for PUSCH in-line with agreements from the study phase (TR 38.889, Section 7.2.1.3.3). (RAN1)

- 	Configured Grant operation: NR Type-1 and Type-2 configured grant mechanisms are the baseline for NR-U operation with modifications in line with agreements during the study phase (NR-U TR section 7.2.1.3.4). (RAN1)

-	CSI: NR Rel-15 CSI feedback mechanism is the baseline for NR-U operation. Enhancements can be considered in line with agreements from the NR-U study item (e.g., TR 38.889, Section 7.2.1.3.1). (RAN1)

- 	Data multiplexing aspects (for both UL and DL) considering LBT and channel access priorities. (RAN1/RAN2)

-	Radio interface protocol architecture and procedures (ref. TR 38.889 section 7.2.2) [RAN2]:

-	Inactive and Idle mode procedures: Rel-15 NR design is the baseline, including NR licensed measurement framework (cell and beam quality derivation for RSRP, RSRQ, and SINR, filtering and combining multiple beams). Specify necessary extensions for NR-U operation for cell selection/reselection, including enabling the possibility for the UE to camp on a non-best cell if the best cell does not belong to the registered PLMN (or E-PLMN), where the non-best cell would still be the best cell of the registered PLMN; increased paging opportunities per DRX cycle for the UE to receive the page; increased opportunities for SI delivery

-	Introduction of access priority for control signalling (transmissions over SRBs) over unlicensed carriers (highest access priority).

-	Mobility: For non-standalone NR-U deployments, connected mode mobility is supported on licensed spectrum using the baseline mobility procedure specified for the concerned licensed radio access technology (LTE or NR). For standalone NR-U deployments, specify mechanisms for inter-cell handover between NR-U and NR-U, inter-cell handover between NR-U and NR, and Inter-RAT handover between NR-U and LTE (connected to either EPC of 5GC). Extensions of mobility-related measurement reporting for unlicensed operation including channel occupancy indication and RSSI measurements.

-	Potential enhancements to DRX considering channel access uncertainty.

-	Paging: specify required NR modifications to enhance paging opportunities by increasing time-domain paging occasions or paging monitoring occasions while taking UE power consumption into account.

-	System information: specify required NR modifications to enhance acquisition of system information in NR-U including increased transmission opportunities in time domain for SI transmission.

## 2.3	RAN3

#### 2.3.1	Agreements

#### 2.3.2	Remaining Open issues

## 2.4	RAN4

#### 2.4.1	Agreements

Agreements in RAN4 #90

Outcome: Reply LS to RAN1 on wideband carrier operation for NR-U (R4-1902503)

- It is a common understanding that, we will not use RF filtering for adaption for both the UE and gNB side.

- RAN4 to study the feasibility of changing the baseband filtering between subsequent gNB transmissions

  - Feasibility will depend on RF leakage and blocking requirements

- RAN4 to study the feasible durations of changing baseband filtering at UE receiver

  - Feasibility will depend on RF leakage and blocking requirements

- RAN4 to study the need for guard-bands between LBT sub-bands for wideband carrier operation between the adjacent LBT sub-bands where CCA passes on all adjacent LBT sub-bands once LBT outcome is known to the UE

Outcome: WF for NR-U workplan in Rel-16 (R4-1902504)

- RAN4 to agree on the RF part of workplan as presented in Section 3 of R4-1901646 contribution.

- The responsibility for providing CR is shared between companies for different specs for NR-U work in Rel-16.

- Following companies are responsible for the big CRs for corresponding spec:

  - TS 38.101-1: Qualcomm

  - TS 38.101-3: Huawei

  - TS 38.104: Nokia

  - TS 38.133: Ericsson

  - TS 36.133: Ericsson

  - TS 37.106: Oppo

  - TS 37.107: Nokia

  - TS 38.141: Huawei

- Responsible companies are requested to provide proposal on specification structures for NR-U in RAN4#90bis

- In this WI, the responsible company will provide one big CR covering all change requests for the spec.

Outcome: WF on NR-U BS RF requirements (R4-1902505)

- NR FR1 BS RF core specification requirements in TS 38.104 can be used as a baseline for NR-U BS RF requirement.

- Following NR FR1 transmitter requirements could be reused by NR-U:

  - BS output power,

  - RE power control dynamic range,

  - Transmitter transient period,

  - Frequency error, Modulation quality,

  - Time alignment error,

  - Transmitter spurious emission,

  - Transmitter intermodulation.

- 35dB ACLR requirements can be reused for NR-U single carrier <=20MHz and CA

- LAA Operating band unwanted emission mask can be reused by NR-U single carrier <=20Mhz and CA

- For NR-U for channel bandwidths above 20 MHz OBUE masks should be considered

- Following NR FR1 receiver requirements can be reused for 5GHz NR-U:

  - In-band blocking,

  - Out-of-band blocking,

  - Receiver spurious emission,

  - Receiver general intermodulation

- There will be no narrowband blocking and narrowband intermodulation requirement for NR-U.

- For NR-U the need for new reference measurement channels (FRCs) should be considered.

- For NR unlicensed band(s) above 6 GHz, extension of BS Tx and RX requirements from below 6 GHz shall be verified.

Agreement: It is proposed to re-farm E-UTRA band 46 for NR unlicensed usage as band n46 as below:

- FFS for sub-band plan for band n46.

- FFS for 6GHz band plan for NR-U

  - Regulatory requirements are not yet defined for 6GHz band

Agreement: Band n46 specific requirements can be discussed using 20MHz as starting point. FFS for 10MHz and other larger BW.

Agreement: It is a common understanding that we will not use additional sub-band RF filtering for Rx requirements for UE

Agreement: spectrum utilization for NR-U for 20MHz with 15KHz SCS and 30KHz SCS is agreed to be as in licensed NR:

Agreement: Ad-hoc meeting minutes for NR-U (R4-1902502)

- RAN4 to agree on the RF part of workplan as presented in Section 3 of R4-1901646 contribution.

- The responsibility for providing CR is shared between companies for different specs for NR-U work in Rel-16.

- It is a common understanding that, we will not use RF filtering for adaption for both the UE and gNB side.

- RAN4 to study the feasibility of changing the baseband filtering between subsequent gNB transmissions

  - Feasibility will depend on RF leakage and blocking requirements

- RAN4 to study the feasible durations of changing baseband filtering at UE receiver

  - Feasibility will depend on RF leakage and blocking requirements

- RAN4 to study the need for guardbands between LBT sub-bands for wideband carrier operation between the adjacent LBT sub-bands where CCA passes on all adjacent LBT sub-bands once LBT outcome is known to the UE

#### 2.4.2	Remaining Open issues

The objectives from WID related to RAN4 are as follows:

-	Core specifications for UE, gNB and RRM requirements [RAN4]:

-	Specify new unlicensed band(s) for the 5 GHz and 6 GHz frequency ranges (see Note 1). The band(s) definition should include DL only and UL/DL operation.

-	Specify gNB and UE RF core requirements for the band(s) in the above frequency range,  including a limited set of example band combinations (see Note 2).

-	Investigate the feasibility of increasing the number of usable PRBs for different numerologies and channel bandwidths in unlicensed bands, and if found feasible, specify such support.

- 	Specify RRM/RLM core requirements.

Note 1: The actual frequency range for specification can be further discussed based on regulatory updates in the US and Europe.

Note 2: The WI can be completed provided requirements for at least one band combination involving a new NR-U band is specified as long as it is in line with country-specific regulatory directives.

## 2.5	RAN5

#### 2.5.1	Agreements

#### 2.5.2	Remaining Open issues

#### 2.5.3	Remaining Open issues with cross-WG dependencies

## 2.6	RAN6

#### 2.6.1	Agreements

#### 2.6.2	Remaining Open issues

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

## 3.1	SAx/CTs

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

## 4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template