3GPP TSG RAN meeting #83										RP-190247

Shenzhen, China, March 18-21, 2019

## Status Report to TSG

Agenda item:			10.3.5

| WI / SI Name | Even further Mobility enhancement in E-UTRAN |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | LTE_feMob |  |  |  |  |
| Unique ID | 800189 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-181544 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: | Core part: 12/2019 | Performance part: 12/2019 | Testing part: |  |
| Overall Completion level | Study Item: | Core part: 20% | Performance Part: 0% | Testing part: |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN2 |
| --- | --- | --- |
| Rapporteur | Name | Wang Da |
|  | Company | China Telecom |
|  | Email | wangda.bri@chinatelecom.cn |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | Yes |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table: Even though it is agreed to specify the ”non-split bearer” solution for minimizing the interruption time during mobility at the end of study phase, more RAN2 online time is needed to make further selection between single and dual active protocol stack options during the work phase. And also the number of topics and contributions are too large to finalise within the allocated TUs for RAN2. Considering Rel-16 completion in RAN2 will be March 2020, additional TUs are requested for RAN2 and other impacted WGs.

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

#### 2.1.1	Agreements

RAN1 #95:

During the email discussion, the conclusion part in R1-1814410 is agreed:

Inter-frequency inter-band

For UEs with dual Tx/Rx chains, it may be feasible to support simultaneous transmission/reception for both synchronous and asynchronous scenarios. Not all combinations of source/target bands may be supported.

Inter-frequency intra-band synchronous

Simultaneous reception: Feasible at least for UEs supporting intra-band DL CA on frequencies which would be supported for CA serving cells.

Simultaneous transmission: Feasible at least for UEs supporting intra-band UL CA on frequencies which would be supported for CA serving cells, under some conditions (e.g. TAG of source/target CC, power control, contiguous/non-contiguous).

Inter-frequency intra-band asynchronous

RAN1 has not concluded whether simultaneous Tx and/or Rx can be supported for this case.

Intra-frequency synchronous

Reception: Feasible for some UEs at least if source/target bandwidth are the sameand under some conditions (e.g. received signal power are comparable).

Transmission: Feasible for UEs with single RF chain, and source/target eNB in same PC group and under some conditions (e.g. MTTD is zero or near zero). UEs that have multiple RF chains that can be enabled for this bandmay be able to relax this condition.

Intra-frequency asynchronous

Reception: Feasible for UEs with single RF chain under some conditions (e.g. small received power difference). Requires separate FFT/loops with source and target cells.

Transmission:  RAN1 has not concluded whether simultaneous Tx and/or Rx can be supported for this case.

For all scenarios, the above statements are to be confirmed by RAN4 taking into account aspects such as required degrees of synchronization, AGC, power difference, etc. For the cases where simultaneous transmission/reception are not possible, some techniques (e.g. TDM at the physical layer) may be applied to allow for simultaneous connection to source and target cells.

Some UE resources may need to be released in the source eNB (e.g. number of CC/bands/MIMO layers) to support simultaneous tx/rx with target eNB.

The reply LS for interruption time during mobility in LTE is endorsed in R1-1814411.

#### 2.1.2	Remaining Open issues

## 2.2	RAN2

#### 2.2.1	Agreements

RAN2 #105:

For user data interruption during handover, the following agreements are made based on the discussion on report from [104#61][LTE/feMOB]:

Agreements

1	Specify the ”non-split bearer” solution candidate for the Rel-16 E-UTRA enhancements minimizing the interruption time during mobility.

2	Decide during the work item phase whether a single active protocol stack or two active protocol stacks are used in enhanced Rel-16 E-UTRAN mobility solution.

3	Agree the following common aspects for “non-split bearer” solution candidate:

a.	PDCP SN assignment (for DL) is done at source eNB. PDCP SDUs and the SN assigned to each SDU are then forwarded to target eNB. Details of how SN information is transferred is FFS.

b.	RoHC and remaining PDCP functions (e.g. ciphering, PDCP PDU creation) are executed separately at each network node

c.	The UE procedure when UE detaches from the source cell is explicitly defined in the specifications (e.g. via procedural text and/or via dedicated message/indication.).

d.	In case of two active protocol stacks, a separate security key is used for each of the protocol stacks.

4	RAN2 is asked to work further on the details of the following open issues:

a.	When detaching from the source shall occur and whether it has to be separately considered from the UE’s and NW’s side

b.	Whether data forwarding is done “late” or “early”. Consider potential combination with CHO and how SN Status transfer is done and how HFN is handled.

c.	LS to RAN3 on data forwarding enhancements to enable reduced interruption time during HO

5	The detailed assumptions of simultaneous transmission/reception for the solutions depend on the feedback from RAN1 and RAN4 (i.e. response to R2-1815706). RAN2 shall continue working based on the received LS replies.

Definitions on single and dual protocol stacks as captured in R2-1902520 are agreed. And there is an email discussion [105#57] to discuss UE and network side impacts of single/dual protocol stacks.

For mobility robustness enhancement, some baseline agreements for conditional handover is made:

Agreements

1: The baseline operation for E-UTRAN Conditional HO procedure assumes HO command type of message contains HO triggering condition(s) and dedicated RRC configuration(s). UE accesses the prepared target when the relevant condition is met.

3: The baseline operation for E-UTRAN Conditional HO assumes the source eNB remains responsible for RRC until UE successfully sends RRC Connection Reconfiguration Complete message to target eNB.

4: RAN2 assumes late packet forwarding (i.e. not done immediately when the CHO target cells become prepared) could be more suitable for E-UTRAN CHO when there are multiple candidate target cells. In case of single prepared candidate target cell, early packet forwarding could be considered as an option. Detailed decisions require RAN3 study.

5: RAN2 will inform the Conditional HO assumptions (including the baseline operation) to RAN3 via LS at RAN#105bis, requesting RAN3 to kindly work on the CHO scheme aspects matching their expertise (e.g. data forwarding).

#### 2.2.2	Remaining Open issues

- Specify further enhancements to achieve following targets,

  - reduce user data interruption during handover, which targets as close as possible to 0ms, i.e. relaxed requirements could be considered.

  - improve the robustness during handover,

## 2.3	RAN3

#### 2.3.1	Agreements

#### 2.3.2	Remaining Open issues

- Specify further enhancements to achieve following targets,

  - reduce user data interruption during handover, which targets as close as possible to 0ms, i.e. relaxed requirements could be considered.

  - improve the robustness during handover,

## 2.4	RAN4

#### 2.4.1	Agreements

RAN4 #90:

The WF on Reply LS for the interruption time during mobility in LTE is approved in R4-1902011, with the following additional agreements captured in RAN4 meeting report.

Agreement: Add the following bullets to slide #3 in WF

- Single FFT solution works on condition that the time difference between received signals from targeting and serving cells is within the CP.

- RAN4 should consider both single FFT and dual FFT when discussing the requirements in the future.

Agreement:

- Change “The TA difference between source and target cells shall be in a certain limit” to “The actual transmit timing difference between source and target cells shall be in a certain limit” on Slide #6.

The Reply LS on the interruption time during mobility in LTE is approved in R4-1902030.

#### 2.4.2	Remaining Open issues

- Specify necessary core requirements for the identified solutions

## 2.5	RAN5

#### 2.5.1	Agreements

#### 2.5.2	Remaining Open issues

#### 2.5.3	Remaining Open issues with cross-WG dependencies

## 2.6	RAN6

#### 2.6.1	Agreements

#### 2.6.2	Remaining Open issues

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

## 3.1	SAx/CTs

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

## 4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

RAN1 #95

- R1-1814410  Email discussion on reply LS for interruption time during mobility Qualcomm Incorporated

- R1-1814411	Reply LS on the Interruption time during mobility in LTE RAN1

RAN2 #105:

- R2-1900020	Reply LS on the Interruption time during mobility in LTE (R1-1814411; contact: Qualcomm)	RAN1	LS in

- R2-1902601	Reply LS on the interruption time during mobility in LTE  (R1-1902030; contact: Huawei)	RAN4	LS in

- R2-1900619	Report from [104#61][LTE/feMOB]  Solution directions for minimizing user data interruption for UL/DL	Nokia, Nokia Shanghai Bell

- R2-1900806	Discussion and comparison of the interruption reduction solutions	ZTE Corporation, Sanechips

- R2-1900443	Single active protocol stack and dual active protocol stacks to reduce HO interruption	Mediatek Inc.

- R2-1900807	Discussion on the non-split bearer solution with single active protocol stack	ZTE Corporation, Sanechips

- R2-1900273	Non-split Bearer Option on Reduction in User Data Interruption	CATT

- R2-1900508	Clarification on the uplink interruption and the uplink data forwarding for the non-split bearer option	vivo

- R2-1900792	Key metrics analysis for LTE HO architecture solution discussion 	Qualcomm Incorporated

- R2-1900967	RAN2 impacts due to RAN4 and RAN1 feedbacks	Huawei, HiSilicon

- R2-1901376	Way forward on the progress of Rel-16 LTE feMob	China Telecom

- R2-1901088	Scenarios for mobility interruption enhancements	Ericsson	discussion	Rel-16	LTE_feMob-Core

- R2-1900787	LTE Mobility Interruption time definitions 	Qualcomm Incorporated

- R2-1901494	Reducing User Data Interruption during Handover in LTE	InterDigital

- R2-1900958	Details of eMBB solution	Huawei, HiSilicon

- R2-1900617	Alternative eMBB handover procedure with close-to-zero ms interruption	Nokia, Nokia Shanghai Bell

- R2-1901085	Enhancements to Make-before-break	Ericsson

- R2-1900791	LTE Mobility Enhancements for MBB handover	Qualcomm India Pvt Ltd

- R2-1900858	Control plane consideration for eMBB-based handover	OPPO

- R2-1900871	Detail for non-split bearer option for simultaneous connectivity	Intel Corporation

- R2-1900274	Further Consideration on Simultaneous Connectivity Handover	CATT

- R2-1902109	Security related Considerations to Non-split based HO	LG Electronics Inc.

- R2-1900957	Performance evaluations of eMBB solution	Huawei, HiSilicon

- R2-1900959	Performance evaluations of DC-based handover solution	Huawei, HiSilicon

- R2-1900960	Details of DC-based handover solution	Huawei, HiSilicon

- R2-1900505	Discussion on the RLF and HOF during the DC handover	vivo

- R2-1900861	Comparison of DC-based and eMBB-based handover	OPPO

- R2-1900509	Consideration on the SRB of the DC handover	vivo

- R2-1901086	DC based handover	Ericsson

- R2-1900506	Discussion on the ROHC procedure	vivo

- R2-1900183	User plane consideration for eMBB based handover	OPPO

- R2-1901090	User plane aspects of make-before-break handover	Ericsson

- R2-1901377	PDCP impact for non-split dual active protocal stack solution	China Telecom

- R2-1900922	Data Forwarding in Enhanced Mobility Solutions	ETRI

- R2-1901969	Discussion of ‘0ms’ interruption handover in context of E-UTRAN	CMCC

- R2-1900444	Single/Dual RRC for simultaneous connectivity during HO	MediaTek Inc.

- R2-1901993	Discussion on single RRC or dual RRC for non split-bearer	Beijing Xiaomi Software Tech

- R2-1900789	RLM and RLF handling during LTE enhanced MBB HO	Qualcomm Incorporated

- R2-1900618	Simultaneous connectivity handover with single uplink operation	Nokia, Nokia Shanghai Bell

- R2-1900968	single uplink feedback for dual downlink data transmission	Huawei, HiSilicon

- R2-1900970	Preamble gap design for one uplink path	Huawei, HiSilicon

- R2-1900507	Capability coordination between the source and target node	vivo

- R2-1901073	Capability coordination in mobility enhancement	ZTE Corporation, Sanechips

- R2-1902254	Basic details of Conditional Handover in E-UTRAN	Nokia, Nokia Shanghai Bell, ZTE, Qualcomm Incorporated, OPPO, Xiaomi, Charter Communications

- R2-1900275	Open issues of Conditional Handover	CATT

- R2-1901958	Open Issues for Conditional Handover

- R2-1900143	Discussions on Conditional Handover Procedures	MediaTek Inc.

- R2-1900870	Discussion of conditional handover	Intel Corporation

- R2-1900808	Discussion on the conditioinal handover procedure	ZTE Corporation, Sanechips

- R2-1901082	Further details of conditional handover in LTE	Ericsson

- R2-1900614	Further details of Conditional Handover in E-UTRAN	Nokia, Nokia Shanghai Bell

- R2-1901003	Conditional handover procedure in LTE	NEC

- R2-1901378	Configuration of CHO	China Telecom

- R2-1901495	Details of Conditional Handover procedure for LTE	InterDigital

- R2-1901840	Details of conditional handover	Apple

- R2-1900144	Signalling Overhead Reduction for Conditional Handover	MediaTek Inc.

- R2-1900510	Signaling procedures of conditional handover	vivo

- R2-1900615	Optimizations for Conditional Handover in E-UTRAN	Nokia, Nokia Shanghai Bell

- R2-1900859	Discussion on conditional handover	OPPO

- R2-1900923	Considerations on Conditional Handover	ETRI

- R2-1900949	Signalling Procedure of Conditional Handover	CATT

- R2-1902075	Configuration management aspects in conditional handover	Samsung R&D Institute UK

- R2-1902110	Consideration to Support Conditional HO in LTE	LG Electronics Inc.

- R2-1900963	Comparison of CHO and two-step handover	Huawei, HiSilicon

- R2-1900961	Considerations about conditional handover	Huawei, HiSilicon

- R2-1900962	Details of conditional handover	Huawei, HiSilicon

- R2-1901496	Triggers for Conditional Handover in LTE	InterDigital

- R2-1901091	Triggering of conditional handover	Ericsson

- R2-1900279	Discussion for UE Behaviors in CHO Case	CATT

- R2-1902223	Discussion for UE Behaviors in CHO Case	CATT

- R2-1901071	CP flow for LTE conditional handover	Lenovo, Motorola Mobility

- R2-1900971	Dual L3 filters for Conditional handover	Huawei, HiSilicon

- R2-1900872	Performance evaluation on exit condition for conditional handover	Intel Corporation

- R2-1900974	Simulation of key parameters impact for conditional handover	China Telecom

- R2-1901089	User plane aspects of conditional handover in LTE	Ericsson

- R2-1900277	Data Forwarding with Conditional HO	CATT

- R2-1901379	Data forwarding for CHO	China Telecom

- R2-1900511	Discussion on the number of prepared cells for CHO	vivo

- R2-1900924	Number of Prepared Cells in Conditional Handover	ETRI

- R2-1900276	Support of One or More Candidate Cells for Conditional HO	CATT

- R2-1900988	On Supporting Conditional HO	Samsung

- R2-1900278	Handling of Conditional Handover Failure	CATT

- R2-1900512	Discussion on the RLF and HOF for CHO	vivo

- R2-1900616	Considerations on failure recovery in LTE	Nokia, Nokia Shanghai Bell

- R2-1900860	Discussion on failure handling of handover	OPPO

- R2-1900788	LTE Mobility Robustness Enhancements	Qualcomm Incorporated

- R2-1900969	Cell selection criteria for the combination of eMBB and CHO	Huawei, HiSilicon

- R2-1901163	Fast RLF triggering based on timer T312	Ericsson	discussion

- R2-1901358	Introduction of T312 for IRAT NR measurement in EUTRA	Samsung R&D Institute India	draftCR	Rel-16	36.331	15.4.0

- R2-1901359	Introduction of early RLF declaration for PSCell in (NG)EN-DC	Samsung R&D Institute India	draftCR	Rel-16	36.331	15.4.0

- R2-1900790	Lossless Make-Before-Break (MBB) HO support for low latency, high reliability services	Qualcomm Incorporated

- R2-1901084	Measurement reporting overhead reduction based on enhanced event triggering	Ericsson

- R2-1901087	RACH-less handover robustness	Ericsson

- R2-1900964	Enhancements to RACH-less solution	Huawei, HiSilicon	discussion

- R2-1901165	RRC connection re-establishment for handover failure recovery	Ericsson

- R2-1900966	Considerations on failure handlings	Huawei, HiSilicon	discussion

- R2-1901083	Repetition of RRC messages	Ericsson

- R2-1900965	PDCP duplication for improving robustness in handover	Huawei, HiSilicon

- R2-1902520	Outcome of offline discussion #800: Definition on single and dual protocol stacks	Intel

- R2-1902521	Outcome of offline discussion #801: Baseline agreements for conditional handover	Nokia

RAN4 #90:

- R4-1900333	Reply to LS for interruption during mobility	Qualcomm Incorporated

- R4-1900773	WF on Reply LS for the interruption time during mobility in LTE	China Telecom, Ericsson, Nokia, Huawei

- R4-1900938	Considerations on further enhancements for mobility	Ericsson

- R4-1901240	Discussion on LS on the interruption time during mobility in LTE	Huawei, HiSilicon

- R4-1901241	[DRAFT] Reply LS on the interruption time during mobility in LTE	Huawei, HiSilicon

- R4-1902011	WF on Reply LS for the interruption time during mobility in LTE	China Telecom, Ericsson, Nokia, Huawei

- R4-1902030	Reply LS on the interruption time during mobility in LTE	Huawei, HiSilicon

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template