3GPP TSG-TSG RAN WG1 Meeting # 96	R1-1903272

# Athens, Greece,  25th Feb - 1st March, 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.213 | CR | 1220 | rev |  | Current version: | 14.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Power control for PUSCH enhancements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_VoLTE_ViLTE_enh-Core |  |  |  |  |  | Date: |  |  | 2019-02-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - Power control section does not include references to DCI format 0C - Behavior for the case when δmsg2 is not included in RAR is not specified. - Implement the following agreement correctly (contention-free random access part is missing) Agreements: - For a UE configured with the new PUSCH enhancement mode, the TPC field in RAR corresponding to a non-contention PRACH indicated by PDCCH order indicates the repetition number of msg3 of RACH procedure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add DCI format 0C to all relevant sections - Clarify that δmsg2=0 for the case of field not being present in RAR grant. - Clarify that the TPC command is changed to “number of repetitions” only for the case of contention-free random access |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Specification is incomplete and inconsistent with agreements |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.1.1, 6.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

#### 5.1.1.1	UE behaviour

The setting of the UE Transmit power for a Physical Uplink Shared Channel (PUSCH) transmission is defined as follows.

If the UE transmits PUSCH without a simultaneous PUCCH for the serving cell , then the UE transmit power  for PUSCH transmission in subframe i for the serving cell is given by

[dBm]

If the UE transmits PUSCH simultaneous with PUCCH for the serving cell , then the UE transmit power  for the PUSCH transmission in subframe i for the serving cell  is given by

[dBm]

If the UE is not transmitting PUSCH for the serving cell c, for the accumulation of TPC command received with DCI format 3/3A for PUSCH, the UE shall assume that the UE transmit power  for the PUSCH transmission in subframe i for the serving cell  is computed by

[dBm]

where,

-	is the configured UE transmit power defined in [6] in subframe i for serving cell  and  is the linear value of . If the UE transmits PUCCH without PUSCH in subframe for the serving cell c, for the accumulation of TPC command received with DCI format 3/3A for PUSCH, the UE shall assume  as given by Subclause 5.1.2.1. If the UE does not transmit PUCCH and PUSCH in subframe  for the serving cell c, for the accumulation of TPC command received with DCI format 3/3A for PUSCH, the UE shall compute  assuming MPR=0dB, A-MPR=0dB, P-MPR=0dB and TC =0dB, where MPR, A-MPR, P-MPR and TC are defined in [6].

-	 is the linear value of defined in Subclause 5.1.2.1

-	is the bandwidth of the PUSCH resource assignment expressed in number of resource blocks valid for subframe i and serving cell .

-	If the UE is configured with higher layer parameter UplinkPowerControlDedicated-v12x0 for serving cell  and if subframe  belongs to uplink power control subframe set 2 as indicated by the higher layer parameter tpc-SubframeSet-r12,

-	when j=0, , where j=0 is used for PUSCH (re)transmissions corresponding to a semi-persistent grant. and  are the parameters p0-UE-PUSCH-Persistent-SubframeSet2-r12 and p0-NominalPUSCH-Persistent -SubframeSet2-r12 respectively provided by higher layers, for each serving cell .

-	when j=1,, where j=1 is used for PUSCH (re)transmissions corresponding to a dynamic scheduled grant. and are the parameters p0-UE-PUSCH-SubframeSet2-r12 and p0-NominalPUSCH-SubframeSet2-r12 respectively, provided by higher layers for serving cell .

-	when j=2, where  and , where the parameter preambleInitialReceivedTargetPower [8] () and  are signalled from higher layers for serving cell , where j=2 is used for PUSCH (re)transmissions corresponding to the random access response grant.

Otherwise

-	is a parameter composed of the sum of a component  provided from higher layers for j=0 and 1 and a component  provided by higher layers for j=0 and 1 for serving cell . For PUSCH (re)transmissions corresponding to a semi-persistent grant then j=0 , for PUSCH (re)transmissions corresponding to a dynamic scheduled grant then j=1 and for PUSCH (re)transmissions corresponding to the random access response grant then j=2.  and , where the parameter preambleInitialReceivedTargetPower [8] () and  are signalled from higher layers for serving cell .

-	If the UE is configured with higher layer parameter UplinkPowerControlDedicated-v12x0 for serving cell  and if subframe  belongs to uplink power control subframe set 2 as indicated by the higher layer parameter tpc-SubframeSet-r12,

-	For j=0 or 1, . is the parameter alpha-SubframeSet2-r12 provided by higher layers for each serving cell .

-	For j=2, .

Otherwise

-	For j =0 or 1,  is a 3-bit parameter provided by higher layers for serving cell . For j=2,

-	 is the downlink path loss estimate calculated in the UE for serving cell  in dB and  = referenceSignalPower – higher layer filtered RSRP, where referenceSignalPower is provided by higher layers and RSRP is defined in [5] for the reference serving cell and the higher layer filter configuration is defined in [11] for the reference serving cell.

-	If serving cell  belongs to a TAG containing the primary cell then, for the uplink of the primary cell, the primary cell is used as the reference serving cell for determining referenceSignalPower and higher layer filtered RSRP. For the uplink of the secondary cell, the serving cell configured by the higher layer parameter pathlossReferenceLinking defined in [11] is used as the reference serving cell for determining referenceSignalPower and higher layer filtered RSRP.

-	If serving cell  belongs to a TAG containing the PSCell then, for the uplink of the PSCell, the PSCell is used as the reference serving cell for determining referenceSignalPower and higher layer filtered RSRP; for the uplink of the secondary cell other than PSCell, the serving cell configured by the higher layer parameter pathlossReferenceLinking defined in [11] is used as the reference serving cell for determining referenceSignalPower and higher layer filtered RSRP.

-	If serving cell  belongs to a TAG not containing the primary cell or PSCell then serving cell  is used as the reference serving cell for determining referenceSignalPower and higher layer filtered RSRP.

-	for and 0 for where  is given by the parameter deltaMCS-Enabled provided by higher layers for each serving cell .  and , for each serving cell , are computed as below.  for transmission mode 2.

-	for control data sent via PUSCH without UL-SCH data and for other cases.

-	where  is the number of code blocks,  is the size for code block ,  is the number of CQI/PMI bits including CRC bits and  is the number of resource elements determined as , where , ,  and  are defined in [4].

-	 for control data sent via PUSCH without UL-SCH data and  for other cases.

-	 is a correction value, also referred to as a TPC command and is included in PDCCH/EPDCCH with DCI format 0/0A/0B/0C/4/4A/4B or in MPDCCH with DCI format 6-0A for serving cell or jointly coded with other TPC commands in PDCCH/MPDCCH with DCI format 3/3A whose CRC parity bits are scrambled with TPC-PUSCH-RNTI. If the UE is configured with higher layer parameter UplinkPowerControlDedicated-v12x0 for serving cell  and if subframe  belongs to uplink power control subframe set 2 as indicated by the higher layer parameter tpc-SubframeSet-r12, the current PUSCH power control adjustment state for serving cell is given by, and the UE shall use  instead of to determine . Otherwise, the current PUSCH power control adjustment state for serving cell is given by.  and  are defined by:

-	 and  if accumulation is enabled based on the parameter Accumulation-enabled provided by higher layers or if the TPC command  is included in a PDCCH/EPDCCH with DCI format 0 or in a MPDCCH with DCI format 6-0A for serving cell  where the CRC is scrambled by the Temporary C-RNTI

-	where was signalled on PDCCH/EPDCCH with DCI format 0/0A/0B/0C/4/4A/4B or MPDCCH with DCI format 6-0A or PDCCH/MPDCCH with DCI format 3/3A on subframe , and where  is the first value after reset of accumulation. For a BL/CE UE configured with CEModeA, subframe  is the last subframe in which the MPDCCH with DCI format 6-0A or MPDCCH with DCI format 3/3A is transmitted.

-	The value of  is

-	For FDD or FDD-TDD and serving cell frame structure type 1,  = 4

-	For TDD, if the UE is configured with more than one serving cell and the TDD UL/DL configuration of at least two configured serving cells is not the same, or if the UE is configured with the parameter EIMTA-MainConfigServCell-r12 for at least one serving cell, or for FDD-TDD and serving cell frame structure type 2, the "TDD UL/DL configuration" refers to the UL-reference UL/DL configuration (defined in Subclause 8.0) for serving cell .

-	For TDD UL/DL configurations 1-6 and UE not configured with higher layer parameter symPUSCH-UpPts-r14 for the serving cell ,  is given in Table 5.1.1.1-1.

-	For TDD UL/DL configuration 0 and UE not configured with higher layer parameter symPUSCH-UpPts-r14 for the serving cell .

-	If the PUSCH transmission in subframe 2 or 7 is scheduled with a PDCCH/EPDCCH of DCI format 0/4 or a MPDCCH of DCI format 6-0A in which the LSB of the UL index is set to 1, = 7

-	For all other PUSCH transmissions,  is given in Table 5.1.1.1-1.

-	For TDD UL/DL configurations 0-5 and UE configured with higher layer parameter symPUSCH-UpPts-r14 for the serving cell ,  is given in Table 5.1.1.1-4.

-	For TDD UL/DL configuration 6 and UE configured with higher layer parameter symPUSCH-UpPts-r14 for the serving cell

-	If the PUSCH transmission in subframe 2 or 7 is scheduled with a PDCCH/EPDCCH of DCI format 0/4 in which the LSB of the UL index is set to 1, = 6

-	For all other PUSCH transmissions,  is given in Table 5.1.1.1-4.

-	For a serving cell with frame structure type 3,

-	For an uplink DCI format 0A/0B/0C/4A/4B with PUSCH trigger A set to 0, is equal to k+l, where k and l are defined in in Subclause 8.0.

-	For an uplink DCI format 0A/0B/0C/4A/4B with PUSCH trigger A set to 1 and upon the detection of PDCCH with DCI CRC scrambled by CC-RNTI and with 'PUSCH trigger B' field set to '1' described in Subclause 8.0, is equal to p+k+l, where p, k and l are defined in Subclause 8.0.

- If a UE detected multiple TPC commands in subframe , the UE shall use the TPC command in the PDCCH/EPDCCH with DCI format 0A/0B/0C/4A/4B which schedules PUSCH transmission in subframe i.

-	For serving cell  and a non-BL/CE UE, the UE attempts to decode a PDCCH/EPDCCH of DCI format 0/0A/0B/0C/4/4A/4B with the UE's C-RNTI or DCI format 0 for SPS C-RNTI or DCI format 0 for UL-SPS-V-RNTI and a PDCCH of DCI format 3/3A with this UE's TPC-PUSCH-RNTI in every subframe except when in DRX or where serving cell  is deactivated.

- For serving cell  and a BL/CE UE configured with CEModeA, the UE attempts to decode a MPDCCH of DCI format 6-0A with the UE's C-RNTI or SPS C-RNTI and a MPDCCH of DCI format 3/3A with this UE's TPC-PUSCH-RNTI in every BL/CE downlink subframe except when in DRX

-	For a non-BL/CE UE, if DCI format 0/0A/0B/0C/4/4A/4B for serving cell and DCI format 3/3A are both detected in the same subframe, then the UE shall use the  provided in DCI format 0/0A/0B/0C/4/4A/4B.

- For a BL/CE UE configured with CEModeA, if DCI format 6-0A for serving cell and DCI format 3/3A are both detected in the same subframe, then the UE shall use the  provided in DCI format 6-0A.

-	dB for a subframe where no TPC command is decoded for serving cell or where DRX occurs or i is not an uplink subframe in TDD or FDD-TDD and serving cell c frame structure type 2.

-	dB if the subframe i is not the first subframe scheduled by a PDCCH/EPDCCH of DCI format 0B/4B.

-	The  dB accumulated values signalled on PDCCH/EPDCCH with DCI format 0/0A/0B/0C/4/4A/4B or MPDCCH with DCI format 6-0A are given in Table 5.1.1.1-2. If the PDCCH/EPDCCH with DCI format 0 or MPDCCH with DCI format 6-0A is validated as a SPS activation or release PDCCH/EPDCCH/MPDCCH, then  is 0dB.

-	The  dB accumulated values signalled on PDCCH/MPDCCH with DCI format 3/3A are one of SET1 given in Table 5.1.1.1-2 or SET2 given in Table 5.1.1.1-3 as determined by the parameter TPC-Index provided by higher layers.

-	If UE has reached  for serving cell , positive TPC commands for serving cell  shall not be accumulated

-	If UE has reached minimum power, negative TPC commands shall not be accumulated

-	If the UE is not configured with higher layer parameter UplinkPowerControlDedicated-v12x0 for serving cell , the UE shall reset accumulation

-	For serving cell , when  value is changed by higher layers

-	For serving cell , when the UE receives random access response message for serving cell

-	If the UE is configured with higher layer parameter UplinkPowerControlDedicated-v12x0 for serving cell ,

-	the UE shall reset accumulation corresponding to  for serving cell

-	when  value is changed by higher layers

-	when the UE receives random access response message for serving cell

-	the UE shall reset accumulation corresponding to  for serving cell

-	when  value is changed by higher layers

-	If the UE is configured with higher layer parameter UplinkPowerControlDedicated-v12x0 for serving cell  and

-	if subframe  belongs to uplink power control subframe set 2 as indicated by the higher layer parameter tpc-SubframeSet-r12

-	if subframe  does not belong to uplink power control subframe set 2 as indicated by the higher layer parameter tpc-SubframeSet-r12

-	 and  if accumulation is not enabled for serving cell  based on the parameter Accumulation-enabled provided by higher layers

-	where was signalled on PDCCH/EPDCCH with DCI format 0/0A/0B/0C/4/4A/4B or MPDCCH with DCI format 6-0A for serving cell  on subframe  . For a BL/CE UE configured with CEModeA, subframe  is the last subframe in which the MPDCCH with DCI format 6-0A or MPDCCH with DCI format 3/3A is transmitted.

-	The value of  is

-	For FDD or FDD-TDD and serving cell frame structure type 1, = 4

-	For TDD, if the UE is configured with more than one serving cell and the TDD UL/DL configuration of at least two configured serving cells is not the same, or if the UE is configured with the parameter EIMTA-MainConfigServCell-r12 for at least one serving cell, or FDD-TDD and serving cell frame structure type 2, the "TDD UL/DL configuration" refers to the UL-reference UL/DL configuration (defined in Subclause 8.0) for serving cell .

-	For TDD UL/DL configurations 1-6 and UE not configured with higher layer parameter symPUSCH-UpPts-r14 for the serving cell ,  is given in Table 5.1.1.1-1.

-	For TDD UL/DL configuration 0 and UE not configured with higher layer parameter symPUSCH-UpPts-r14 for the serving cell .

-	If the PUSCH transmission in subframe 2 or 7 is scheduled with a PDCCH/EPDCCH of DCI format 0/4 or a MPDCCH with DCI format 6-0A in which the LSB of the UL index is set to 1, = 7

-	For all other PUSCH transmissions,  is given in Table 5.1.1.1-1.

-	For TDD UL/DL configurations 0-5 and UE configured with higher layer parameter symPUSCH-UpPts-r14 for the serving cell ,  is given in Table 5.1.1.1-4.

-	For TDD UL/DL configuration 6 and UE configured with higher layer parameter symPUSCH-UpPts-r14 for the serving cell

-	If the PUSCH transmission in subframe 2 or 7 is scheduled with a PDCCH/EPDCCH of DCI format 0/4 in which the LSB of the UL index is set to 1, = 6

-	For all other PUSCH transmissions,  is given in Table 5.1.1.1-4.

-	For a serving cell with frame structure type 3,

-	For an uplink DCI format 0A/4A with PUSCH trigger A set to 0, is equal to k+l, where k and l are defined in in Subclause 8.0.

-	For an uplink DCI format 0B/4B with PUSCH trigger A set to 0, is equal to k+l+i' with , where niHARQ_ID is HARQ process number in subframe i, and k, l, nHARQ_ID and NHARQ are defined in Subclause 8.0.

-	For an uplink DCI format 0A/4A with PUSCH trigger A set to 1 and upon the detection of PDCCH with DCI CRC scrambled by CC-RNTI and with 'PUSCH trigger B' field set to '1' described in Subclause 8.0, is equal to p+k+l, where p, k and l are defined in Subclause 8.0.

-	For an uplink DCI format 0B/4B with PUSCH trigger A set to 1 and upon the detection of PDCCH with DCI CRC scrambled by CC-RNTI and with 'PUSCH trigger B' field set to '1' described in Subclause 8.0, is equal to p+k+l+i' with , where niHARQ_ID is HARQ process number in subframe i, and p, k, l, nHARQ_ID and NHARQ are defined in Subclause 8.0.

- If a UE detected multiple TPC commands in subframe , the UE shall use the TPC command in the PDCCH/EPDCCH with DCI format 0A/0B/0C/4A/4B which schedules PUSCH transmission in subframe i.

-	The  dB absolute values signalled on PDCCH/EPDCCH with DCI format 0/0A/0B/0C/4/4A/4B or a MPDCCH with DCI format 6-0A are given in Table 5.1.1.1-2. If the PDCCH/EPDCCH with DCI format 0 or a MPDCCH with DCI format 6-0A is validated as a SPS activation or release PDCCH/EPDCCH/MPDCCH, then  is 0dB.

-	for a non-BL/CE UE,  and  for a subframe where no PDCCH/EPDCCH with DCI format 0/0A/0B/0C/4/4A/4B is decoded for serving cell  or where DRX occurs or i is not an uplink subframe in TDD or FDD-TDD and serving cell c frame structure type 2.

- for a BL/CE UE configured with CEModeA, and  for a subframe where no MPDCCH with DCI format 6-0A is decoded for serving cell  or where DRX occurs or i is not an uplink subframe in TDD.

-	If the UE is configured with higher layer parameter UplinkPowerControlDedicated-v12x0 for serving cell  and

-	if subframe  belongs to uplink power control subframe set 2 as indicated by the higher layer parameter tpc-SubframeSet-r12

-	if subframe  does not belong to uplink power control subframe set 2 as indicated by the higher layer parameter tpc-SubframeSet-r12

-	For both types of  (accumulation or current absolute) the first value is set as follows:

-	If  value is changed by higher layers and serving cell  is the primary cell or, if  value is received by higher layers and serving cell  is a Secondary cell

-

-	Else

-	If the UE receives the random access response message for a serving cell

-	, where

-	 is the TPC command indicated in the random access response corresponding to the random access preamble transmitted in the serving cell , see Subclause 6.2, and

and  is provided by higher layers and corresponds to the total power ramp-up requested by higher layers from the first to the last preamble in the serving cell , is the bandwidth of the PUSCH resource assignment expressed in number of resource blocks valid for the subframe of first PUSCH transmission in the serving cell , and TF,c(0) is the power adjustment of first PUSCH transmission in the serving cell c. If a UE is performing non-contention based random access procedure and is configured with higher layer parameter pusch-EnhancementsConfig, δmsg2=0.

-	If value is received by higher layers for a serving cell c.

-

Table 5.1.1.1-1:  for TDD configuration 0-6

| TDD UL/DL Configuration | subframe number i |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| 0 | - | - | 6 | 7 | 4 | - | - | 6 | 7 | 4 |
| 1 | - | - | 6 | 4 | - | - | - | 6 | 4 | - |
| 2 | - | - | 4 | - | - | - | - | 4 | - | - |
| 3 | - | - | 4 | 4 | 4 | - | - | - | - | - |
| 4 | - | - | 4 | 4 | - | - | - | - | - | - |
| 5 | - | - | 4 | - | - | - | - | - | - | - |
| 6 | - | - | 7 | 7 | 5 | - | - | 7 | 7 | - |

Table 5.1.1.1-2: Mapping of TPC Command Field in DCI format 0/0A/0B/0C/3/4/4A/4B/6-0A/3B to absolute and accumulated  values

| TPC Command Field in DCI format 0/0A/0B/0C/3/4/4A/4B/6-0A/3B | Accumulated [dB] | Absolute [dB] only DCI format 0/0A/0B/0C/4/4A/4B/6-0A |
| --- | --- | --- |
| 0 | -1 | -4 |
| 1 | 0 | -1 |
| 2 | 1 | 1 |
| 3 | 3 | 4 |

<Unchanged parts are omitted>

## 6.2	Random Access Response Grant

<Unchanged parts are omitted>

else,

-	Nr=20, and the content of these 20 bits starting with the MSB and ending with the LSB are as follows:

-	Hopping flag – 1 bit

-	Fixed size resource block assignment – 10 bits

-	Truncated modulation and coding scheme – 4 bits

If a UE is performing non-contention based random access procedure and is configured with higher layer parameter pusch-EnhancementsConfig, then

-	Repetition number of Msg3 – 3 bits

else

-	TPC command for scheduled PUSCH – 3 bits

-	UL delay – 1 bit

-	CSI request – 1 bit

-	The UE shall use the single-antenna port uplink transmission scheme for the PUSCH transmission corresponding to the Random Access Response Grant and the PUSCH retransmission for the same transport block.

<Unchanged parts are omitted>