| Radiocommunication Study Groups |  |
| --- | --- |
| Received: 18 September 2017 | Document 5D/702-E |
|  | 19 September 2017 |
|  | English only SPECTRUM ASPECTS |
| Director, Radiocommunication Bureau |  |
| liaison statement on Characteristics of IMT-2020 system for coexistence study in the frequency band 4 800-4 990 MHz |  |

Response to:	LS (RP-171542) on Characteristics of IMT-2020 system for coexistence study in the frequency band 4 800-4 990 MHz

Release:		Rel-15

Work item:	NR_newRAT

Source:		3GPP TSG RAN

To:		ITU-R WP 5D

# 1	Overall description

3GPP TSG RAN received the LS from ITU-R WP 5D on “Characteristics of IMT-2020 system for coexistence study in the frequency band 4 800-4 990 MHz” (RP-171542).

Currently the 5G RF parameters for BS and UE for sub6GHz are still under development in RAN4. However, as an initial estimation, majority of the RF parameters for IMT-advanced will be reused for the same channel bandwidth for sub6GHz. So it would be feasible to reuse the IMT-advanced parameters for 4 800-4 990 MHz from sharing study point of view.

Further, the beamforming antenna characteristics in Table A2-1 can also be used for sub6 GHz sharing study although they are originally replied for 24.25-33.4 GHz frequency range.

One thing to be pointed out is that the combination of maximum output power in Table A1-1 and the beamforming antenna characteristic in Table A2-1 should be well considered from a link budget point of view. E.g. it may not need as much power as for the cases where beamforming is not considered since beamforming can improve the coverage.

# 2	Actions

To TSG RAN

Action:

3GPP TSG RAN asks ITU-R WP 5D to take the above information into consideration.

# 3	Date of next TSG-RAN meeting

TSG-RAN Meeting #78:	18-21 December 2017 	Lisbon, Portugal

ANNEX 1

IMT-Advanced technology-related and deployment-related parameters 
for bands between 3 and 6 GHz

TABLE A1-1

IMT-Advanced specification related parameters

|  |  | IMT-Advanced |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Duplex mode | FDD |  | TDD |  |
| No. | Parameter | Base station | Mobile station | Base station | Mobile station |
| 1 | Access technique | OFDM | SC-FDMA | OFDM | SC-FDMA |
| 2 | Modulation parameters | QPSK 16-QAM 64-QAM |  |  |  |
| 3 | Channel spacing | Nominal channel spacing = (BWChannel(1) + BWChannel((2))/2 |  |  |  |
| 4 | Channel bandwidth (MHz) | (1) | (1), (19) | (1) | (1), (19) |
| 5 | Signal bandwidth (MHZ) | (1) | (1), (19) | (1) | (1), (19) |
| 6 | Transmitter characteristics |  |  |  |  |
| 6.1 | Power dynamic range (dB) | (2) | 63(18) | (2) | 63(18) |
| 6.2 | Spectral mask | (3), (17) | (4), (17) | (3), (17) | (4), (17) |
| 6.3 | ACLR | (5), (17) | (6), (17) | (5), (17) | (6), (17) |
| 6.4 | Maximum output power | (7) | (8) | (7) | (8) |
| 6.5 | Spurious emissions | (15), (17) | (16), (17) | (15), (17) | (16), (17) |

TABLE A1-1 (end)

|  |  | IMT-Advanced |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Duplex mode | FDD |  | TDD |  |
| No. | Parameter | Base station | Mobile station | Base station | Mobile station |
| 7 | Receiver characteristics |  |  |  |  |
| 7.1 | Noise figure | 5 dB (macro) 10 dB (micro) 13 dB (pico/femto) | 9 dB | 5 dB (macro) 10 dB (micro) 13 dB (pico/femto) | 9 dB |
| 7.2 | Sensitivity | (9) | (10) | (9) | (10) |
| 7.3 | Blocking response | (11) | (12) | (11) | (12) |
| 7.4 | ACS | (13) | (14) | (13) | (14) |
| Notes to the Table: (1) See 3GPP Document TS 36.101 v.11.2.0, § 5.6. Signal bandwidth in MHz corresponds to “Transmission bandwidth configuration*0.180”. (2) See 3GPP Document TS 36.104 v.11.2.0, § 6.3.2.1. (3) See 3GPP Document TS 36 104 v.11.2.0, § 6.6.3. (4) See 3GPP Document TS 36 101 v.11.2.0, §§ 6.6.2.1, 6.6.2.1A, 6.6.2.2 and 6.6.2.2A describe UE spectrum emissions masks for different channel bandwidths. In case multiple UEs are transmitting simultaneously on the same channel they will share the available radio resource blocks. As the actual transmission bandwidth is thus decreased the unwanted emissions performance might be improved. This may be taken into account during sharing analysis when measurements or detailed models are available. (5) See 3GPP Document TS 36.104 v.11.2.0, § 6.6.2. (6) See 3GPP Document TS 36.101 v.11.2.0, § 6.6.2.3. (7) See 3GPP Document TS 36.104 v.11.2.0, § 6.2. (8) See 3GPP Document TS 36.101 v.11.2.0, § 6.2. (9) See 3GPP Document TS 36.104 v.11.2.0, § 7.2. (10) See 3GPP Document TS 36.101 v.11.2.0, § 7.3. (11) See 3GPP Document TS 36.104 v.11.2.0, § 7.6. (12) See 3GPP Document TS 36.101 v.11.2.0, § 7.6 and § 7.7. (13) See 3GPP Document TS 36.104 v.11.2.0, § 7.5. (14) See 3GPP Document TS 36.101 v.11.2.0, § 7.5. (15) See 3GPP Document TS 36.104 v.11.2.0, § 6.6.4. (16) See 3GPP Document TS 36.101 v.11.2.0, § 6.6.3. (17) These unwanted emission limits are the upper limits from SDO specifications for laboratory testing with maximum transmitting power. It is assumed that when the in-band transmitting power is reduced by x dB through power control, the unwanted emission levels would be reduced by x dB in consequence in the coexistence simulations. (18) See 3GPP Document TS 36.101 v.11.2.0, § 6.3. (19) In case multiple UEs are transmitting simultaneously on the same channel they will share the available radio resource blocks. As the actual transmission bandwidth is thus decreased the unwanted emissions performance might be improved. This may be taken into account during sharing analysis when measurements or detailed models are available. |  |  |  |  |  |

TABLE A1-2

IMT-Advanced Deployment-related parameters for bands between 3 and 6 GHz

|  | Macro suburban | Macro urban | Small cell outdoor/ Micro urban | Small cell indoor/ Indoor urban |
| --- | --- | --- | --- | --- |
| Base station characteristics/Cell structure |  |  |  |  |
| Cell radius/Deployment density | 0.3-2 km (typical figure to be used in sharing studies 0.6 km) | 0.15-0.6 km (typical figure to be used in sharing studies 0.3 km) | 1-3 per urban macro cell <1 per suburban macro site | Depending on indoor coverage/capacity demand |
| Antenna height | 25 m | 20 m | 6 m | 3 m |
| Sectorization | 3 sectors | 3 sectors | Single sector | Single sector |
| Downtilt | 6 degrees | 10 degrees | n.a. | n.a. |
| Frequency reuse | 1 | 1 | 1 | 1 |
| Antenna pattern | Recommendation ITU-R F.1336 (recommends 3.1) ka = 0.7 kp = 0.7 kh = 0.7 kv = 0.3 Horizontal 3 dB beamwidth: 65 degrees Vertical 3 dB beamwidth: determined from the horizontal beamwidth by equations in Recommendation ITU-R F.1336. Vertical beamwidths of actual antennas may also be used when available. |  | Recommendation ITU-R F.1336 omni |  |

TABLE A1-2 (continued)

|  | Macro suburban | Macro urban |  | Small cell outdoor/ Micro urban | Small cell indoor/ Indoor urban |  |
| --- | --- | --- | --- | --- | --- | --- |
| Base station characteristics/Cell structure |  |  |  |  |  |  |
| Antenna polarization | Linear/±45 degrees |  | Linear/±45 degrees | Linear | Linear |  |
| Indoor base station deployment | n.a. |  | n.a. | n.a. | 100% |  |
| Indoor base station penetration loss | n.a. |  | n.a. | n.a. | 20 dB (3-5 GHz) 25 dB (5-6 GHz) (horizontal direction) Rec. ITU-R P.1238, Table 3 (vertical direction) |  |
| Below rooftop base station antenna deployment | 0% |  | 50% | 100% | n.a. |  |
| Feeder loss | 3 dB |  | 3 dB | n.a | n.a |  |
| Maximum base station output power (5/10/20 MHz) | 43/46/46 dBm |  | 43/46/46 dBm | 24 dBm | 24 dBm |  |
| Maximum base station antenna gain | 18 dBi |  | 18 dBi | 5 dBi | 0 dBi |  |
| Maximum base station output power/sector (EIRP) | 58/61/61 dBm |  | 58/61/61 dBm | 29 dBm | 24 dBm |  |
| Average base station activity | 50% |  | 50% | 50% | 50% |  |
| Average base station power/sector taking into account activity factor | 55/58/58 dBm |  | 55/58/58 dBm | 26 dBm | 21 dBm |  |
| User terminal characteristics |  |  |  |  |  |  |
| Indoor user terminal usage | 70% |  | 7% | 70% |  | 100% |
| Indoor user terminal penetration loss | 20 dB |  | 20 dB | 20 dB |  | 20 dB (3-5 GHz) 25 dB (5-6 GHz) (horizontal direction) Rec. ITU-R P.1238, Table 3 (vertical direction) |
| User terminal density in active mode to be used in sharing studies | 2.16/5 MHz/km2 |  | 3/5 MHz/km2 | 3/5 MHz/km2 |  | Depending on indoor coverage/capacity demand |

TABLE A1-2 (end)

|  | Macro suburban | Macro urban |  | Small cell outdoor/Micro urban | Small cell indoor/Indoor urban |  |
| --- | --- | --- | --- | --- | --- | --- |
| User terminal characteristics |  |  |  |  |  |  |
| Maximum user terminal output power | 23 dBm |  | 23 dBm | 23 dBm |  | 23 dBm |
| Average user terminal output power | –9 dBm |  | –9 dBm | –9 dBm |  | –9 dBm |
| Typical antenna gain for user terminals | –4 dBi |  | –4 dBi | –4 dBi |  | –4 dBi |
| Body loss | 4 dB |  | 4 dB | 4 dB |  | 4 dB |

ANNEX 2

Antenna characteristics for IMT-2020 for bands 
between 24.25 and 33.4GHz

TABLE A2-1

Antenna Characteristics for IMT-2020 for bands between 24.25 and 33.4 GHz

|  |  | Outdoor Suburban hotspot |  | Outdoor Urban hotspot | Indoor |
| --- | --- | --- | --- | --- | --- |
| 1 | Base station Antenna Characteristics |  |  |  |  |
| 1.1 | Antenna pattern | Refer to Recommendation ITU-R M.2101 |  |  |  |
| 1.2 | Element gain (dBi) | 5 | 5 |  | 5 |
| 1.3 | Horizontal/vertical 3 dB beamwidth of single element (degree) | 65º for both H/V | 65º for both H/V |  | 90º for both H/V |
| 1.4 | Horizontal/vertical front-to-back ratio (dB) | 30 for both H/V | 30 for both H/V |  | 25 for both H/V |
| 1.5 | Antenna polarization | Linear ±45º | Linear ±45º |  | Linear ±45º |
| 1.6 | Antenna array configuration (Row × Column) NOTE 2 | 8×8 elements | 8×8 elements |  | 8×8 elements |
| 1.7 | Horizontal/Vertical radiating element spacing | 0.5 of wavelength for both H/V | 0.5 of wavelength for both H/V |  | 0.5 of wavelength for both H/V |
| 1.8 | Array Ohmic loss (dB) | 3 | 3 |  | 3 |
| 1.9 | Conducted power (before Ohmic loss) per antenna element (dBm/200 MHz) | 10 | 10 |  | 5 |
| 1.10 | Base station maximum coverage angle in the horizontal plane (degrees) | 120 | 120 |  | 120 |
| 2 | User terminal Antenna Characteristics |  |  |  |  |
| 2.1 | Antenna pattern | Refer to Recommendation ITU-R M.2101 |  |  |  |
| 2.2 | Element gain (dBi) | 5 | 5 |  | 5 |
| 2.3 | Horizontal/vertical 3 dB beamwidth of single element (degree) | 90º for both H/V | 90º for both H/V |  | 90º for both H/V |
| 2.4 | Horizontal/vertical front-to-back ratio (dB) | 25 for both H/V | 25 for both H/V |  | 25 for both H/V |
| 2.5 | Antenna polarization | Linear ±45º | Linear ±45º |  | Linear ±45º |
| 2.6 | Antenna array configuration (Row × Column) NOTE 2 | 4×4 elements | 4×4 elements |  | 4×4 elements |
| 2.7 | Horizontal/Vertical radiating element spacing | 0.5 of wavelength for both H/V | 0.5 of wavelength for both H/V |  | 0.5 of wavelength for both H/V |
| 2.8 | Array Ohmic loss (dB) | 3 | 3 |  | 3 |
| 2.9 | Conducted power (before Ohmic loss) per antenna element (dBm / 200 MHz) | 10 | 10 |  | 10 |

| Contacts: Yuexia Song | E-mail: songyuexia@catt.cn |
| --- | --- |
| Liu Ye | E-mail: leo.liuye@huawei.com |

______________

____________________

Submitted on behalf of 3GPP TSG RAN.