3GPP TSG RAN WG1 Meeting #100-e	R1-2001336

# February 24- March 6, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.213 | CR | 1307 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on Msg3 NPUSCH for NB-IoT EDT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NB_IOTenh2-Core |  |  |  |  |  | Date: |  |  | 2020-03-01 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In Rel-15 EDT for NB-IoT, UE is allowed to select a suitable TBS from a configured TBS set. The repetition number of the corresponding Msg3 NPUSCH is scaled to the nearest higher integer multiple of L. In order to reduce eNB complexity of blind decoding of different TBSs, the following agreement was made in RAN1#93. RAN1#93 agreements: - The parameter L for cyclic repetition is identical for all 4 actual TBS candidates. - When the repetition number for the maximum TBS is equal or larger than 8, L = 4. - When the repetition number for the maximum TBS is smaller than 8, L=1. Since Rel-13, multi-tone NPUSCH supports cyclic repetition but single-tone NPUSCH does not. The intention of the agreement above is for multi-tone only. The current specification for narrowband RAR interpretation does not state that restriction, thus L=4 could appear to be a valid network selection for single-tone Msg3 NPUSCH procedures even though there is no corresponding specification of the transmission procedure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Specify “ if ” is applicable only for multi-tone NPUSCH, and L equals 1 for single-tone NPUSCH. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Specification contains ambiguous statements which may result in different interpretations for eNB and UE for single-tone EDT Msg3 NPUSCH transmission. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 16.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### < Unchanged parts are omitted >

### 16.3.3	Narrowband random access response grant

The higher layers indicate the Nr-bit UL Grant to the physical layer, as defined in 3GPP TS 36.321 [8]. 
This is referred to as the Narrowband Random Access Response Grant in the physical layer.

Nr-bit =15, and the content of these 15 bits starting with the MSB and ending with the LSB are as follows:

-	Uplink subcarrier spacing  is '0'=3.75 kHz or '1'=15 kHz – 1 bit

-	Subcarrier indication field  as determined in Subclause 16.5.1.1 – 6 bits

-	Scheduling delay field () as determined in Subclause 16.5.1 with k0 = 12 for IDelay = 0 , where NB-IoT DL subframe n is the last subframe in which the NPDSCH associated with the Narrowband Random Access Response Grant is transmitted – 2 bits

-	Msg3 repetition number  as determined in Subclause 16.5.1.1 – 3 bits

-	MCS index indicating TBS, modulation, and number of RUs for Msg3 – 3 bits

The redundancy version for the first transmission of Msg3 is 0.

If the UE is not using higher layer parameter edt-Parameters, or the UE is using higher layer parameter edt-parameters and ,

-	the TBS, modulation, and number of RUs for Msg3 are determined according to Table 16.3.3-1

otherwise,

-	if the UE is configured with higher layer parameter edt-SmallTBS-Enabled set to ‘false’,

-	the TBS is given by higher layer parameter edt-TBS

-	otherwise,

- the UE selects a TBS from the allowed TBS values according to Table 16.3.3-2

- the repetition number for Msg3 is the smallest integer multiple of L value that is equal to or larger than where  is the selected TBS for Msg3, and  is given by higher layer parameter edt-TBS

-	if  and  and , then is used in subclause 16.5.1.2, otherwise is used

-	the number of RUs for Msg3 are determined according to Table 16.3.3-3

- π/4 QPSK modulation is used for  and for  with ; QPSK modulation is used for with

#### < Unchanged parts are omitted >