3GPP TSG-RAN4 Meeting #96-e	R4-2009657

# Online, 17th Aug - 28th Aug 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-2 | CR | 0217 | rev | - | Current version: | 15.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on Minimum output power and Off power MBW definition in FR2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2020-8-7 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Excluding ACLR, measurement bandwidth definitions in the Minimum output power and Off power have inconsistencies between FR1 and FR2. - Current specification in FR2 uses a MBW equal to Transmission Bandwidth Configuration but centered on the wanted and adjacent channels - MBW does not account for the half SCS Shift and thus results in the transmission signal not being fully captured for all cases. - ACLR MBW was corrected at R4-2006150. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Measurement bandwidth is changed following the same policy with ACLR. Change from 47.52 MHz to 47.58 MHz for 50 MHz Channel bandwidth. Change from 95.04 MHz to 95.16 MHz for 100 MHz Channel bandwidth. Change from 190.08 MHz to 190.20 MHz for 200 MHz Channel bandwidth. Change from 380.16 MHz to 380.28 MHz for 400 MHz Channel bandwidth. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Transmission signal can not be fully captured in the Min Out and Off power measurement bandwidth in FR2. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.1, 6.3.2, 6.3A.1, 6.3A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.521-2 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## <<Unchaged sections skipped>>

<<Start of change>>

## 6.3	Output power dynamics

### 6.3.1	Minimum output power

#### 6.3.1.0	General

The minimum controlled output power of the UE is defined as the EIRP in the channel bandwidth for all transmit bandwidth configurations (resource blocks) when the power is set to a minimum value.

#### 6.3.1.1	Minimum output power for power class 1

For power class 1 UE, the minimum output power shall not exceed the values specified in Table 6.3.1.1-1 for each operating band supported. The minimum power is verified in beam locked mode with the test metric of EIRP (Link=TX beam peak direction, Meas=Link angle).

Table 6.3.1.1-1: Minimum output power for power class 1

| Operating band | Channel bandwidth (MHz) | Minimum output power (dBm) | Measurement bandwidth (MHz) |
| --- | --- | --- | --- |
| n257, n258, n260, n261 | 50 | 4 | 47.58 |
|  | 100 | 4 | 95.16 |
|  | 200 | 4 | 190.20 |
|  | 400 | 4 | 380.28 |

#### 6.3.1.2	Minimum output power for power class 2, 3, and 4

The minimum output power shall not exceed the values specified in Table 6.3.1.2-1 for each operating band supported. The minimum power is verified in beam locked mode with the test metric of EIRP (Link=TX beam peak direction, Meas=Link angle).

Table 6.3.1.2-1: Minimum output power for power class 2, 3, and 4

| Operating band | Channel bandwidth (MHz) | Minimum output power (dBm) | Measurement bandwidth (MHz) |
| --- | --- | --- | --- |
| n257, n258, n260, n261 | 50 | -13 | 47.58 |
|  | 100 | -13 | 95.16 |
|  | 200 | -13 | 190.20 |
|  | 400 | -13 | 380.28 |
| NOTE 1: n260 is not applied for power class 2. |  |  |  |

### 6.3.2	Transmit OFF power

The transmit OFF power is defined as the TRP in the channel bandwidth when the transmitter is OFF. The transmitter is considered OFF when the UE is not allowed to transmit on any of its ports.

The transmit OFF power shall not exceed the values specified in Table 6.3.2-1 for each operating band supported. The requirement is verified with the test metric of TRP (Link=TX beam peak direction, Meas=TRP grid).

Table 6.3.2-1: Transmit OFF power

| Operating band | Channel bandwidth / Transmit OFF power (dBm) / measurement bandwidth |  |  |  |
| --- | --- | --- | --- | --- |
|  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| n257, n258, n260, n261 | -35 | -35 | -35 | -35 |
|  | 47.58 MHz | 95.16 MHz | 190.20 MHz | 380.28 MHz |

<<End of change>>

<<Start of change>>

## 6.3A	Output power dynamics for CA

### 6.3A.1	Minimum output power for CA

Table 6.3A.1-1: Void

#### 6.3A.1.0	General

For intra-band contiguous carrier aggregation, the minimum controlled output power of the UE is defined as the transmit power of the UE per component carrier, i.e., EIRP in the channel bandwidth of each component carrier for all transmit bandwidth configurations (resource blocks), when the power on both component carriers are set to a minimum value.

#### 6.3A.1.1	Minimum output power for power class 1

The minimum output power shall not exceed the values specified in Table 6.3A.1.1-1 for each operating band supported. The minimum power is verified in beam locked mode with the test metric of EIRP (Link=TX beam peak direction, Meas=Link angle).

Table 6.3A.1.1-1: Minimum output power for power class 1

| Operating band | Channel bandwidth (MHz) | Minimum output power (dBm) | Measurement bandwidth (MHz) |
| --- | --- | --- | --- |
| n257, n258, n260, n261 | 50 | 4 | 47.58 |
|  | 100 | 4 | 95.16 |
|  | 200 | 4 | 190.20 |
|  | 400 | 4 | 380.28 |

#### 6.3A.1.2	Minimum output power for power class 2, 3, and 4

The minimum output power shall not exceed the values specified in Table 6.3A.1.2-1 for each operating band supported. The minimum power is verified in beam locked mode with the test metric of EIRP (Link=TX beam peak direction, Meas=Link angle).

Table 6.3A.1.2-1: Minimum output power for CA for power class 2, 3, and 4

| Operating band | Channel bandwidth (MHz) | Minimum output power (dBm) | Measurement bandwidth (MHz) |
| --- | --- | --- | --- |
| n257, n258, n260, n261 | 50 | -13 | 47.58 |
|  | 100 | -13 | 95.16 |
|  | 200 | -13 | 190.20 |
|  | 400 | -13 | 380.28 |
| NOTE 1: n260 is not applied for power class 2. |  |  |  |

### 6.3A.2	Transmit OFF power for CA

For intra-band contiguous carrier aggregation, the transmit OFF power is defined as the TRP in the channel bandwidth per component carrier when the transmitter is OFF. The transmitter is considered OFF when the UE is not allowed to transmit on any of it sports.

The transmit OFF power shall not exceed the values specified in Table 6.3A.2-1 for each operating band supported.

Table 6.3A.2-1: Transmit OFF power for CA

| Operating band | Channel bandwidth / Transmit OFF power (dBm) / measurement bandwidth |  |  |  |
| --- | --- | --- | --- | --- |
|  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| n257, n258, n260, n261 | -35 | -35 | -35 | -35 |
|  | 47.58 MHz | 95.16 MHz | 190.20 MHz | 380.28 MHz |

## <<End of change>>