3GPP TSG-RAN WG4 Meeting # 95-e 	R4-2010600

# Electronic Meeting, 17-28 Aug., 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.124 | CR | 0025 | rev | - | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to TS 38.124 combined correction R15 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Xiaomi |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2020-08-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Couple of changes have been made because of: 1, Some editorial correction. 2, CISPR 16-1 reference is not needed. 3, Some abbreviations are missing. 4, Note 4 is missing which is not aligned to RF spec. 5, Max hold detector and corresponding annex A should not be used. |  |  |  |  |  |  |  |  |
| Summary of change: |  | With the reason for change, following changes have been made: 1, Editorial corrections. 2, CISPR 16-1 is deleted in reference and table in subclause 7. 3, Abbreviations are added. 4, Note 4 is added to align with RF spec. 5, Max hold detector methods are deleted. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Specification not wtriten correctly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 1, 2, 3, 7.1, 8.2, 9.3, 9.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

--------------Start of text change-------------

# 1	Scope

The present document establishes the essential EMC requirements for "3rd generation" digital cellular mobile terminal equipment and ancillary accessories in combination with a 3GPP NR user equipment (UE).

The equipment conforming to the requirements laid out in the present document and used in its intended electromagnetic environment in accordance with the manufacturer's instructions

-	shall not generate electromagnetic disturbances at a level which may interfere with the intended operation of other equipment;

-	has an adequate level of intrinsic immunity to electromagnetic disturbances to operate as intended;

The present document specifies the applicable EMC tests, methods of measurement, frequency ranges, applicable limits and minimum performance criteria for all types of NR UE(s) and their accessories. NR base station equipment operating within network infrastructure is outside the scope of the present document. However, the present document does cover mobile and portable equipment that is intended to be operated in a fixed location while connected to the AC mains. NR base station equipment operating within network infrastructure is covered by the technical specification TS 38.113 [2].

Requirements for the radiated emission from the enclosure port of integral antenna equipment and ancillaries are included in the present document. Technical specifications for conducted emissions from the antenna connector are found in the 3GPP specifications for the radio interface of NR UE, e.g. TS 38.101-1 [3], for the effective use of the radio spectrum.

Requirements for the radiated emissions from the enclosure port and ancillaries cover the following case:

-	UE equipment supporting operations in a frequency range for which antenna connectors are available (i.e. for operations in FR1 as defined in e.g. TS 38.101-1 [3] for the radio interface).

-The immunity requirements are selected to ensure an adequate level of compatibility for apparatus in residential, commercial, light industrial and vehicular environments. The levels however, do not cover extreme cases, which may occur in any location but with low probability of occurrence.

Compliance of radio equipment to the requirements of the present document does not signify compliance to any requirement related to the use of the equipment (i.e. licensing requirements).

Compliance to the requirements of the present document does not signify compliance to any safety requirement. However, any temporary or permanent unsafe condition caused by EMC is considered as non-compliance.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 38.113: "NR; Base Station (BS) ElectroMagnetic Compatibility (EMC)".

[3]	3GPP TS 38.101-1: "NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone".

[4]	3GPP TS 38.101-2: " NR; User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone".

[5]	ITU-R Recommendation SM.329: "Unwanted emissions in the spurious domain".

[6]	Void

[7]	Void.

[8]	IEC 61000-3-2: "Electromagnetic compatibility (EMC) - Part 3-2: Limits - Limits for harmonic current emissions (equipment input current ≤ 16 A per phase)".

[9]	IEC 61000-3-3: "Electromagnetic compatibility (EMC) - Part 3-3: Limits - Limitation of voltage changes, voltage fluctuations and flicker in low-voltage supply systems, for equipment with rated current ≤ 16 A per phase and not subject to conditional connection".

[10]	IEC 61000-4-3: "Electromagnetic compatibility (EMC) - Part 4-3: Testing and measurement techniques - Radiated, radio-frequency electromagnetic field immunity test".

[11]	IEC 61000-4-2: "Electromagnetic compatibility (EMC) - Part 4-2: Testing and measurement techniques - Electrostatic discharge immunity test".

[12]	IEC 61000-4-4: "Electromagnetic compatibility (EMC) - Part 4-4: Testing and measurement techniques - Electrical fast transient/burst immunity test - Basic EMC publication".

[13]	IEC 61000-4-6: "Electromagnetic compatibility (EMC) - Part 4-6: Testing and measurement techniques - Immunity to conducted disturbances induced by radio frequency fields".

[14]	Void

[15]	ISO 7637-2: "Road vehicles -- Electrical disturbances from conduction and coupling -- Part 2: Electrical transient conduction along supply lines only".

[16]	IEC 61000-4-11: "Electromagnetic compatibility (EMC) - Part 4-11: Testing and measurement techniques - Voltage dips, short interruptions, and voltage variations immunity test".

[17]	IEC 61000-4-5: "Electromagnetic compatibility (EMC) - Part 4-5: Testing and measurement techniques - Surge immunity test".

[18]	ITU-R Recommendation SM.1539 (2001): "Variation of the boundary between the out-of-band and spurious domains required for the application of Recommendations ITU-R SM.1541 and ITU-R SM.329".

[19]	IEC 60050-161: "International Electrotechnical Vocabulary - Chapter 161: Electromagnetic compatibility".

[20]	IEC CISPR 32: "Electromagnetic compatibility of multimedia equipment - Emission requirements".

[21]	3GPP TS 38.508-1: "User Equipment (UE) conformance specification; Part 1: Common test environment".

[22]	3GPP 38.509: "Special conformance testing functions for User Equipment (UE)".

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Ancillary equipment: Equipment (apparatus), used in connection with a user equipment (UE) is considered as an ancillary equipment (apparatus) if:

-	the equipment is intended for use in conjunction with a UE to provide additional operational and/or control features to the UE, (e.g. to extend control to another position or location); and

-	the equipment cannot be used on a stand-alone basis to provide user functions independently of a UE; and

-	the UE to which it is connected, is capable of providing some intended operation such as transmitting and/or receiving without the ancillary equipment (i.e. it is not a sub-unit of the main equipment essential to the main equipment basic functions).

Channel bandwidth: The RF bandwidth supporting a single NR RF carrier with the transmission bandwidth configured in the uplink or downlink of a cell. The channel bandwidth is measured in MHz and is used as a reference for transmitter and receiver RF requirements.

Enclosure port: Physical boundary of the apparatus through which electromagnetic fields may radiate or impinge. In the case of integral antenna equipment, this port is inseparable from the antenna port.

Idle mode: Idle mode is the state of User Equipment (UE) when switched ON but with no Radio Resource Control (RRC) connection.

Integral antenna: Antenna designed to be connected directly to the equipment with or without the use of an external connector and considered to be part of the equipment. An integral antenna may be fitted internally or externally to the equipment.

Necessary bandwidth: For a given class of emission, the width of the frequency band which is just sufficient to ensure the transmission of information at the rate and with the quality required under specified conditions.

Out of band emissions: Emission on a frequency or frequencies immediately outside the necessary bandwidth, which results from, the modulation process, but excluding spurious emissions.

Spurious emission: Emission on a frequency, or frequencies, which are outside the necessary bandwidth and the level of which may be reduced without affecting the corresponding transmission of information. Spurious emissions include harmonic emissions, parasitic emissions, intermodulation products and frequency conversion products but exclude out-of-band emissions.

Transient phenomena: Pertaining to or designating a phenomenon or a quantity which varies between two consecutive steady states during a time interval short compared with the time-scale of interest (IEC 60050-161 [19])

User equipment: is a "Mobile Station" (MS) which is an entity capable of accessing a set of NR services via one or more radio interfaces. This entity may be stationary or in motion within the NR service area while accessing the NR services, and may simultaneously serve one or more users.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

BWChannel	Channel bandwidth

FOOB	The boundary between the NR out of band emission and spurious emission domains

NRB	Transmission bandwidth configuration, expressed in units of resource blocks

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

AC	Alternating Current

CA	Carrier Aggregation

DC	Direct Current

ESD	Electrostatic Discharge

EUT	Equipment Under Test

FAC	Fully Anechoic Chamber

FR	Frequency Range

LISN	Line Impedance Stabilising Networks

NR	New Radio

RF	Radio Frequency

RMS	Root Mean Square (value)

UE	User Equipement

-------------End of first change-------------

-------------Start of next change-------------

# 7	Applicability overview tables

## 7.1	EMC emissions

Table 7.1-1: Emission applicability

|  |  | Equipment test requirement |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Phenomenon | Application | Equipment connected to fixed AC or DC power installations | Equipment connected to vehicular DC supplies | Equipment powered by integral battery | Reference clause in the present document | Reference standard |
| Radiated emission | Enclosure | applicable | applicable | applicable | 8.2 | ITU-R SM.329 [5] TS 38.101-1 [3] |
| Conducted emission | DC power input/output port | applicable | applicable | not applicable | 8.3 | CISPR 32 [20], - |
| Conducted emission | AC mains input/output port | applicable | not applicable | not applicable | 8.4 | CISPR 32 [20] |
| Harmonic current emissions | AC mains input port | applicable | not applicable | not applicable | 8.5 | IEC 61000-3-2 [8] |
| Voltage fluctuations and flicker | AC mains input port | applicable | not applicable | not applicable | 8.6 | IEC 61000-3-3 [9] |

For UE equipment operating in FR1 (table 4.1-1), the radiated emission applies to the enclosure port with antenna ports available.

For UE equipment supporting operations in FR2 (table 4.1-1) the enclosure port is inseparable from the antenna port.

-------------End of 2nd  change-------------

-------------Start of next change-------------

## 8.2	Radiated emission

### 8.2.1	General

This test is applicable to radio communications equipment and ancillary equipment.

This test shall be performed on the radio equipment and/or a representative configuration of the ancillary equipment.

### 8.2.2	Definition

This test assesses the ability of radio equipment and ancillary equipment to limit unwanted emissions from the enclosure port.

For UE equipment supporting operations in FR2 (table 4.1-1) with integral antennas only (no antenna connectors available), the EMC radiated emissions cannot be distinguished between the intended emissions nor to any spurious emissions related to these intentional transmissions:

-	This test is applicable to UE equipment without integral antennas, i.e. FR1.

-	For UE equipment with integral antennas only, i.e. FR2, the radiated emission is covered by radiated spurious emission requirement in TS 38.101-2 [4].

### 8.2.3	Test method

Whenever possible the site shall be a fully anechoic chamber (FAC) simulating the free-space conditions. EUT shall be placed on a non-conducting support. Mean power of any spurious components shall be detected by the test antenna and measuring receiver (e.g. a spectrum analyser).

At each frequency at which a component is detected, the EUT shall be rotated to obtain maximum response, and the effective radiated power (e.r.p.) of that component determined by a substitution measurement, which shall be the reference method. The measurement shall be repeated with the test antenna in the orthogonal polarization plane.

NOTE:	Effective radiated power e.r.p. refers to the radiation of a half wave tuned dipole instead of an isotropic antenna. There is a constant difference of 2.15 dB between e.i.r.p. and e.r.p.

e.r.p. (dBm)  e.i.r.p. (dBm)  2.15	, Ref. ITU-R SM. 329 annex 1 [5]

Measurements are made with a tuned dipole antenna or a reference antenna with a known gain referenced to an isotropic antenna. Unless otherwise stated, all measurements are done as mean power (RMS).

### 8.2.4	Limits

Unless otherwise stated, the radiated spurious emission limits apply for the frequency ranges that are more than FOOB (MHz) in table 8.2.4-0 from the edge of the channel bandwidth. The radiated spurious emission limits in table 8.2.4-1 apply for all transmitter band configurations (NRB) and channel bandwidths.

The references for these requirements are ITU-R SM 329 [5], SM.1539 [18] and TS 38.101-1 [3] for FR1.

The frequency boundary and reference bandwidths for the detailed transitions of the limits between the requirements for out of band emissions and spurious emissions are based on ITU-R SM 329 [5].

These requirements are only applicable for frequencies in the spurious domain. The limits are specified in table 8.2.4-1 for UE equipment supporting operations in FR1 only.

Table 8.2.4-0: Boundary between NR out of band and general spurious emission domain

| Channel bandwidth | OOB boundary FOOB (MHz) |
| --- | --- |
| BWChannel | BWChannel + 5 |

Table 8.2.4-1: Radiated spurious emissions requirements for UE equipment supporting operations in FR1 only

| Frequency range | Maximum level (dBm) | Measurement bandwidth | Notes |
| --- | --- | --- | --- |
| 9 kHz ≤ f < 150 kHz | -36 | 1 kHz |  |
| 150 kHz ≤ f < 30 MHz | -36 | 10 kHz |  |
| 30 MHz ≤ f < 1000 MHz | -36 | 100 kHz |  |
| 1 GHz ≤ f < 12.75 GHz | -30 | 1 MHz | 4 |
|  | -25 | 1 MHz | 3 |
| 12.75 GHz ≤ f < 5th harmonic of the upper frequency edge of the UL operating band in GHz | -30 | 1 MHz | 1 |
| 12.75 GHz < f < 26 GHz | -30 | 1 MHz | 2 |
| NOTE 1: Applies for Band that the upper frequency edge of the UL Band more than 2.69 GHz. NOTE 2: Applies for Band that the upper frequency edge of the UL Band more than 5.2 GHz. NOTE 3: Applies for Band n41, CA configurations including Band n41, and EN-DC configurations that include n41 specified in clause 5.2B of TS 38.101-3 [3] when NS_04 is signalled. NOTE 4: Does not apply for Band n41, CA configurations including Band n41, and EN-DC configurations that include n41 specified in subclause 5.2B of TS 38.101-3 [3] when NS_04 is signalled. |  |  |  |

NOTE:	This requirement is aligned with the NR UE spurious emissions requirement in TS 38.101-1 [3].

-------------End of 3rd   change-------------

-------------Start of next change-------------

## RF electromagnetic field (80 MHz to 6000 MHz)

The test shall be performed on a representative configuration of the equipment or a representative configuration of the combination of UE and ancillary equipment.

### 9.2.1	Definition

This test assesses the ability of UE and ancillary equipment to operate as intended in the presence of a radio frequency electromagnetic field disturbance at the enclosure.

### 9.2.2	Test method and level

The test method shall be in accordance with IEC 61000-4-3 [10]

-	For UE and ancillary equipment, the following requirements shall apply;

-	The test level shall be 3 V/m amplitude modulated to a depth of 80 % by a sinusoidal audio signal of 1 kHz;

-	The stepped frequency increments shall be 1 % of the momentary frequency;

-	The test shall be performed over the frequency range 80 MHz to 6000 MHz;

-	Responses in stand-alone receivers or receivers which are part of transceivers occurring at discrete frequencies which are narrow band responses, shall be disregarded, see clause 4.3;

-	The frequencies selected during the test shall be recorded in the test report.

### 9.2.3	Performance criteria

The performance criteria of clause 6.1 shall apply.

## 9.3	Electrostatic discharge

### 9.3.0	General

The test shall be performed on a representative configuration of the equipment or a representative configuration of the combination of UE and ancillary equipment.

### 9.3.1	Definition

This test assesses the ability of UE and ancillary equipment to operate as intended in the event of an electrostatic discharge.

### 9.3.2	Test method and level

The test method shall be in accordance with IEC 61000-4-2 [11]:

-	for contact discharge, the equipment shall pass at ±2 kV and ±4 kV;

-	for air discharge the equipment shall pass at ±2 kV, ±4 kV and ±8 kV (only for non-conducting surfaces, see IEC 61000-4-2 [11]).

NOTE:	The EUT shall be fully discharged between each ESD exposure by connecting its ground point (where applicable) to the HCP by a resistive wire with a 470 k. resistor in either end.

### 9.3.3	Performance criteria

The performance criteria of clause 6.2 shall apply.

## 9.4	Fast transients common mode

### 9.4.0	General

The test shall be performed on AC mains power input ports.

This test shall be performed on signal ports, control ports and DC power input/output ports if the cables may be longer than 3 m.

Where this test is not carried out on a port or any other ports because the manufacturer declares that it is not intended to be used with cables longer than 3 m, a list of ports which were not tested for this reason shall be included in the test report.

This test shall be performed on a representative configuration of the equipment or a representative configuration of the combination of UE and ancillary equipment.

### 9.4.1	Definition

This test assesses the ability of UE and ancillary equipment to operate as intended in the event of fast transients present on one of the input/output ports.

### 9.4.2	Test method and level

The test method shall be in accordance with IEC 61000-4-4 [12]

-	the test level for signal and control ports shall be 0,5 kV open circuit voltage as given in IEC 61000-4-4 [12];

-	the test level for DC power input/output ports shall be 0,5 kV open circuit voltage as given in IEC 61000-4-4 [12];

-	the test level for AC mains power input ports shall be 1 kV open circuit voltage as given in IEC 61000-4-4 [12].

### 9.4.3	Performance criteria

The performance criteria of clause 6.2 shall apply.

## 9.5	RF common mode (0.15 MHz to 80 MHz)

### 9.5.0	General

This test is applicable for UE for fixed, mobile, and portable use and for ancillary equipment.

This test shall be performed on signal, control and DC power input/output ports, which may have cables longer than 3 m.

This test shall be performed on AC mains power input/output ports of UE for fixed use and for fixed ancillary equipment. Where this test is not carried out on a port or any other ports because the manufacturer declares that it is not intended to be used with cables longer than stated above, a list of ports which were not tested shall be included in the test report.

This test shall be performed on a representative configuration of the UE or a representative configuration of the combination of UE and ancillary equipment.

### 9.5.1	Definition

This test assesses the ability of equipment and ancillary equipment to operate as intended in the presence of a radio frequency electromagnetic disturbance.

### 9.5.2	Test method and level

The test method shall be in accordance with IEC 61000-4-6 [13].

-	the test signal shall be amplitude modulated to a depth of 80 % by a sinusoidal audio signal of 1 kHz;

-	the stepped frequency increments shall be either 50 kHz or 1 % frequency increment of the momentary frequency in the frequency range 150 kHz - 5 MHz and 1 % frequency increment of the momentary frequency in the frequency range 5 MHz - 80 MHz;

-	the test level shall be severity level 2 as given in IEC 61000-4-6 [13] corresponding to 3 V RMS, at a transfer impedance of 150 W;

-	the test shall be performed over the frequency range 150 kHz - 80 MHz;

-	responses of stand alone receivers or receivers which are part of transceivers occurring at discrete frequencies which are narrow band responses, shall be disregarded, see subclause 4.3;

-	the frequencies selected during the test and the test method used shall be recorded in the test report.

### 9.5.3	Performance criteria

The performance criteria of clause 6.1 shall apply.

-------------End of change-------------