3GPP TSG RAN Meeting #89-e								RP-201607

Electronic Meeting, Sept 14th - 18th, 2020

## Status Report to TSG

Agenda item:			9.2.13

| WI / SI Name | NR RF requirement enhancements for frequency range 2 (FR2) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NR_RF_FR2_req_enh |  |  |  |  |
| Unique ID | 830089 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-200668 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: NA | Core part: 09/2020 | Performance part: 09/2020 | Testing part: NA |  |
| Overall Completion level | Study Item: NA | Core part: 100% | Performance Part: 100% | Testing part: NA |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN WG4 |
| --- | --- | --- |
| Rapporteur | Name | Petri Vasenkari |
|  | Company | Nokia |
|  | Email | petri.j.vasenkari@nokia.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

#### 2.1.1	Agreements

#### 2.1.2	Remaining Open issues

## 2.2	RAN2

#### 2.2.1	Agreements

FR2 MPE

TSG-RAN2 Meeting 111-e

Agreements on MPE during meeting:

R2 assumes to reuse / extend PHR MAC CE, and continue the MPE work accordingly (main remaining FFS is the required number of bits).

We use Indep prohibit timer instance (for MPE and PHR)

The UE cap is already in the mega CR, if we cannot converge on MAC and RRC CRs, also the UE cap should be removed for now (to stick to normal practice)

Include MAC and RRC CR in the email discussion.

Conclusions on MPE CR details in post-meeting email discussion (see R2-2008241)

Question 1: Do you agree to use the P-bit to indicate whether power backoff due to MPE was applied or not, when MPE reporting is configured, e.g. per v7 of the MAC CR?

Conclusion 1: P-bit can be used to indicate whether power backoff due to MPE was applied.

Question 2: Which option is preferred for implementing the relative threshold in R4’s LS?

Conclusion 2: Agree to implement the relative threshold by reusing “phr-Tx-PowerFactorChange” with no changes (i.e. no MPE specific relative threshold is introduced) and consequently the specification implementation for 38.321 (R2-2008570 CR0883_rev1 Introduction of MPE reporting) and for 38.331 (R2-2008595 CR1873_rev1 Introduction of MPE reporting)

Question 3: Which option is preferred to stop using the relative threshold to report MPE-MPR?

Conclusion 3: Nothing to implement for this particular question.

Agreed CRs (via above email discussion):

- R2-2008595	Configuration for uplink power boosting via suspended IBE requirements	Interdigital, Nokia, Nokia Shanghai Bell	CR	Rel-16	38.331	16.1.0		1873	1	B	NR_RF_FR2_req_enh

- R2-2008570	Configuration for uplink power boosting via suspended IBE requirements	Interdigital, Nokia, Nokia Shanghai Bell	CR	Rel-16	38.321	16.1.0		0883	1	B	NR_RF_FR2_req_enh

Output power boost when IBE is suspended:

TSG-RAN2 Meeting 111-e

Following CRs endorsed (with UE capability parts merged to the UE capability mega-CR):

- R2-2008621  Uplink power boosting via suspended IBE requirements       Nokia, Nokia Shanghai Bell         CR       Rel-16  38.331  16.1.0   1840     2          B   NR_RF_FR2_req_enh

- R2-2008622  Uplink power boosting via suspended IBE requirements       Nokia, Nokia Shanghai Bell      CR       Rel-16  38.306  16.1.0   0379     -           B   NR_RF_FR2_req_enh

CR for the signalling agreed:

- R2-2008621	Configuration for uplink power boosting via suspended IBE requirements	Nokia, Nokia Shanghai Bell	CR	Rel-16	38.331	16.1.0	2008	-	B	NR_RF_FR2_req_enh

DL-only spectrum

TSG-RAN2 Meeting 111-e

Following CRs endorsed (with UE capability parts merged to the UE capability mega-CR):

- R2-2008462	Introduction on frequency separation class for DL-only FR2 spectrum	Apple, Ericsson	CR	Rel-16	38.306	16.1.0	0371	-	F	TEI16, NR_RF_FR2_req_enh

- R2-2008463	Introduction on frequency separation class for DL-only FR2 spectrum	Apple, Ericsson	CR	Rel-16	38.331	16.1.0	1784	-	F	TEI16, NR_RF_FR2_req_enh

#### 2.2.2	Remaining Open issues

None

## 2.3	RAN3

#### 2.3.1	Agreements

#### 2.3.2	Remaining Open issues

## 2.4	RAN4

TSG-RAN4 Meeting #96-e approved/endorsed contributions

R4-2010226	TR v0.2.0	Nokia, Nokia Shanghai Bell

R4-2011734      WF on MPE enhancements, OPPO

R4-2011733	LS on MPE enhancements, Nokia, Nokia Shanghai Bell

R4-2011735	CR Addressing the P-MPR related updates in 38.101-2, Interdigital Inc

R4-2011678      WF on FR2 Beam Correspondence, Apple

R4-2011737      TP to TR38.831: beam correspondence enhancement, Apple

R4-2011928	Beam correspondence enhancement, Nokia, Nokia Shanghai Bell

R4-2011741      Draft LS on UE capability for FR2 inter-band CA, Nokia

R4-2011740	CR to 38.101-2: DL CA BW Enhancement and CA REFSENS, Qualcomm Incorporated

R4-2011739	CR to 38.101-2 (Rel-16) intra-band non-cont. DL CA, Intel Corporation

R4-2011744	CR to 38.101-2: FR2 NC ULCA feature, Qualcomm Incorporated

R4-2011914	Introduction of FR2 inter-band DL CA, Nokia, Nokia Shanghai Bell

R4-2011745	LS on UL power boost mode and IBE relaxation, Qualcomm Incorporated

R4-2011905	CR to 38.101-2: FR2 UE EIRP increase with IBE relaxation, Qualcomm Incorporated

R4-2011736  	Introduction of the P-MPR 2 bits report mapping in 38.133, InterDigital, Inc.

R4-2012151	CR on MRTD for inter-band CA, Apple

#### 2.4.1	Agreements

MPE:

- RAN4#96e: work on FR2 MPE was completed. WF from OPPO and LS to RAN2 from Nokia was approved. CR was agreed.

Beam correspondence:

- RAN4#96e: work for beam correspondence enhancement was completed. WF and TP from Apple was approved and CR from Nokia was agreed. For CSI-RS based BC The downlink reference signals including both SSB and CSI-RS are provided. Conditions for ensuring CSI-RS based beam correspondence is TBD. Furthermore, additional applicability rules are TBD.

Intra-band non-contiguous UL CA:

- RAN4#96e: work for Intra-band non-contiguous UL CA was completed. CR from Qualcomm was agreed.

Inter-band DL CA:

- RAN4#96e: work for Inter-band DL CA was completed. LS from Nokia was approved and CR also from Nokia was agreed.

Output power boost when IBE is suspended:

- RAN4#96e: work for Output power boost when IBE is suspended was completed. LS from Qualcomm was approved and CR also from Qualcomm was agreed.

Intra-band non-cont. DL CA:

- RAN4#96e: CR from Intel to add scheduling restriction to prevent DL CC from being configured across the boundary of DL only spectrum and bi-directional spectrum was agreed.

RRM DL interband CA:

- RAN4#96e: Agreement: No CBM-specific RRM requirements are specified in Rel-16 and that current MRTD requirement applies to the UE capable of independent beam management for FR2 inter-band CA. MTTD with IBM is 8.5us.

RRM MPE:

- RAN4#96e: FR2 P-MPR report mapping table was introduced to 38.133.

#### 2.4.2	Remaining Open issues

None

## 2.5	RAN5

#### 2.5.1	Agreements

#### 2.5.2	Remaining Open issues

#### 2.5.3	Remaining Open issues with cross-WG dependencies

## 2.6	RAN6

#### 2.6.1	Agreements

#### 2.6.2	Remaining Open issues

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

## 3.1	SAx/CTs

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

## 4.	References

TSG-RAN2 Meeting #111-e (36 contributions)

R2-2008571	LS on MPE enhancements		RAN4	Lsin	NR_RF_FR2_req_enh	Rel-16

R2-2008511	Addition of MPE reporting to TS 38.331	InterDigital	CR	Rel-16	38.331	16.1.0	1996	-	B	NR_RF_FR2_req_enh

R2-2007650	Summary of MPE mitigation in FR2	InterDigital	discussion	Rel-16	NR_RF_FR2_req_enh

R2-2007649	L2/3 aspects of MPE mitigation	InterDigital	discussion	Rel-16	NR_RF_FR2_req_enh

R2-2008093	Discussion on MPE enhancements	Ericsson	discussion	Rel-16	NR_RF_FR2_req_enh	R2-2004932

R2-2007375	UE FR2 MPE enhancements and solutions	Nokia, Nokia Shanghai Bell	discussion	Rel-16	NR_RF_FR2_req_enh	R2-2004906

R2-2006808	Discussion on UE FR2 P-MPR reporting	OPPO	discussion	Rel-16	NR_RF_FR2_req_enh

R2-2007123	P-MPR Reporting	Apple	discussion	Rel-16	NR_RF_FR2_req_enh

R2-2007152	Discussion on the MPE enhancements	vivo	discussion

R2-2006809	Draft Response LS on UE FR2 P-MPR reporting	OPPO	LS out	Rel-16	NR_RF_FR2_req_enh	To:RAN4

R2-2007153	Draft CR on supporting the MPE enhancements	vivo	CR	Rel-16	38.321	16.1.0	0806	-	B	NR_RF_FR2_req_enh

R2-2007154	Draft CR on supporting the MPE enhancements	vivo	CR	Rel-16	38.331	16.1.0	1792	-	B	NR_RF_FR2_req_enh

R2-2007376	Introduction of MPE reporting for FR2	Nokia, Nokia Shanghai Bell	CR	Rel-16	38.331	16.1.0	1515	2	B	NR_RF_FR2_req_enh	R2-2004907

R2-2007377	Introduction of MPE reporting for FR2	Nokia, Nokia Shanghai Bell	CR	Rel-16	38.321	16.1.0	0707	2	B	NR_RF_FR2_req_enh	R2-2004908

R2-2007378	Introduction of MPE reporting for FR2	Nokia, Nokia Shanghai Bell	CR	Rel-16	38.321	16.1.0	0819	-	B	NR_RF_FR2_req_enh

R2-2007379	Introduction of MPE reporting for FR2	Nokia, Nokia Shanghai Bell	CR	Rel-16	38.300	16.2.0	0210	2	B	NR_RF_FR2_req_enh	R2-2004910

R2-2007533	Enhancement on FR2 MPE Mitigation	ZTE Corporation, Sanechips	discussion	Rel-16	NR_RF_FR2_req_enh

R2-2007651	Addition of MPE reporting to TS 38.321	InterDigital	CR	Rel-16	38.321	16.1.0	0833	-	B	NR_RF_FR2_req_enh

R2-2007652	Addition of MPE reporting to TS 38.331	InterDigital	CR	Rel-16	38.331	16.1.0	1873	-	B	NR_RF_FR2_req_enh

R2-2008094	Implementing MPE enhancements	Ericsson	CR	Rel-16	38.321	16.1.0	0748	1	B	NR_RF_FR2_req_enh	R2-2004936 	Late

R2-2008095	Implementing MPE enhancements	Ericsson	CR	Rel-16	38.331	16.1.0	1640	1	B	NR_RF_FR2_req_enh	R2-2004938 	Late

R2-2008096	Implementing MPE enhancements	Ericsson	CR	Rel-16	38.306	16.1.0	0322	1	B	NR_RF_FR2_req_enh	R2-2004939 	Late

R2-2007380	Uplink power boosting via suspended IBE requirements	Nokia, Nokia Shanghai Bell	CR	Rel-16	38.331	16.1.0	1840	-	B	NR_RF_FR2_req_enh

R2-2008621	Uplink power boosting via suspended IBE requirements	Nokia, Nokia Shanghai Bell	CR	Rel-16	38.331	16.1.0	1840	2	B	NR_RF_FR2_req_enh

R2-2007381	Uplink power boosting via suspended IBE requirements	Nokia, Nokia Shanghai Bell	CR	Rel-16	38.306	16.1.0	0379	-	B	NR_RF_FR2_req_enh

R2-2008622	Uplink power boosting via suspended IBE requirements	Nokia, Nokia Shanghai Bell	CR	Rel-16	38.306	16.1.0	0379	-	B	NR_RF_FR2_req_enh

R2-2008621	Configuration for uplink power boosting via suspended IBE requirements	Nokia, Nokia Shanghai Bell	CR	Rel-16	38.331	16.1.0	2008	-	B	NR_RF_FR2_req_enh

R2-2008407	Summary of [AT111-e] [020] [NR16] UE cap RF FR2 (Nokia)		Nokia

R2-2008580	Summary of [AT111-e] [020] [NR16] UE cap RF FR2 (Nokia)		Nokia

R2-2007403	DL-only spectrum	Ericsson, Apple	discussion	Rel-16	NR_RF_FR2_req_enh-Core

R2-2007082	Introduction on frequency separation class for DL-only FR2 spectrum	Apple, Ericsson	CR	Rel-16	38.306

R2-2008462	Introduction on frequency separation class for DL-only FR2 spectrum	Apple, Ericsson	CR	Rel-16	38.306	16.1.0	0371	-	F	TEI16, NR_RF_FR2_req_enh

R2-2007083	Introduction on frequency separation class for DL-only FR2 spectrum	Apple, Ericsson	CR	Rel-16	38.331	16.1.0	1784	-	F	TEI16, NR_RF_FR2_req_enh

R2-2008463	Introduction on frequency separation class for DL-only FR2 spectrum	Apple, Ericsson	CR	Rel-16	38.331	16.1.0	1784	-	F	TEI16, NR_RF_FR2_req_enh

R2-2008214	Summary of MPE mitigation in FR2 – Phase 2	InterDigital	discussion	Rel-16	NR_RF_FR2_req_enh

R2-2008595	Configuration for uplink power boosting via suspended IBE requirements	Interdigital, Nokia, Nokia Shanghai Bell	CR	Rel-16	38.331	16.1.0		1873	1	B	NR_RF_FR2_req_enh

R2-2008570	Configuration for uplink power boosting via suspended IBE requirements	Interdigital, Nokia, Nokia Shanghai Bell	CR	Rel-16	38.321	16.1.0		0883	1	B	NR_RF_FR2_req_enh

TSG-RAN4 Meeting #96-e (64 contributions)

R4-2010226	TR v0.2.0	Nokia, Nokia Shanghai Bell	draft TR

R4-2012051	Email discussion summary for [96e][220] NR_RF_FR2_req_enh_RRM	Moderator (Apple)	other

R4-2009600	Pcmax defined reporting ranges accuracy issue in 38.133 for FR2	InterDigital, Inc.	other

R4-2009555	Remaining issues on P-MPR reporting	Sony, Ericsson	other

R4-2009597	Options for P-MPR reporting	InterDigital, Inc.	other

R4-2009598	Introduction of the P-MPR 3 bits report mapping in 38.133	InterDigital, Inc.	CR

R4-2009599	CR Addressing the P-MPR related updates in 38.101-2	InterDigital, Inc.	CR

R4-2009932	Further considerations on the uplink duty cycle enhancements for the MPE scenario	Apple Inc.	discussion

R4-2010019	Views on P-MPR reporting for FR2 MPE.	Xiaomi	other

R4-2010237	LS on MPE enhancements	Nokia, Nokia Shanghai Bell	LS out

R4-2010238	UE FR2 MPE enhancements and solutions	Nokia, Nokia Shanghai Bell	other

R4-2010619	Enhancement on FR2 MPE mitigation	ZTE Corporation	other

R4-2010770	Discussion on MPE remaining issues	OPPO	discussion

R4-2010854	Discussion on remaining FR2 MPE issues	vivo	discussion

R4-2011322	MRTD requirements for CBM in FR2 inter-band CA	Intel Corporation	discussion

R4-2011441	P-MPR consideration	FUTUREWEI	discussion

R4-2011548	Email discussion summary for [96e][115] NR_RF_FR2_req_enh_Part_1	Moderator (OPPO)	other

R4-2009556	Remaining issues in beam correspondence	Sony, Ericsson	other

R4-2009956	Remaining issues with beam correspondence enhncements	Apple Inc.	discussion

R4-2010119	FR2 Beam Correspondence	Qualcomm Incorporated	discussion

R4-2010134	Discussion on enhancement of BC in Rel-16 at FR2	LG Electronics France	other

R4-2010198	Discussion on beam correspondence remaining issues	Samsung	discussion

R4-2010199	CR to TS38.101-2 on beam correspondence enhancement	Samsung	CR

R4-2010239	Beam correspondence enhancement	Nokia, Nokia Shanghai Bell	CR

R4-2010240	FR2 eBC requirements	Nokia, Nokia Shanghai Bell	other

R4-2010771	Discussion on SSB based BC	OPPO	discussion

R4-2011479	On beam correspondence enhancement	Huawei, HiSilicon	other

R4-2011549	Email discussion summary for [96e][116] NR_RF_FR2_req_enh_Part_2	Moderator (Apple)	other

R4-2009753	CC allocation in intra-band non-cont. DL CA	Intel Corporation	discussion

R4-2009754	CR to 38.101-2 (Rel-16) intra-band non-cont. DL CA	Intel Corporation	CR

R4-2011451	CR to 38.101-2: DL CA BW Enhancement and CA REFSENS	Qualcomm Incorporated	CR

R4-2011550	Email discussion summary for [96e][117] NR_RF_FR2_req_enh_Part_3	Moderator (Qualcomm)	other

R4-2011454	CR to 38.101-2: FR2 NC ULCA feature	Qualcomm Incorporated	CR

R4-2011478	On intra-band NC UL CA_FR2	Huawei, HiSilicon	other

R4-2011511	Discussion on FR2 NC UL CA requirements	Qualcomm India Pvt Ltd	other

R4-2011551	Email discussion summary for [96e][118] NR_RF_FR2_req_enh_Part_4	Moderator (Nokia)	other

R4-2009557	Inter-band DL CA in FR2: CBM/IBM capability and associated spherical coverage EIS tests	Sony, Ericsson	other

R4-2009755	FR2 inter-band DL CA	Intel Corporation	discussion

R4-2009963	Beam Management assumptions for inter-band CA	Apple Inc., Qualcomm Incorporated	discussion

R4-2010200	Discussion on FR2 inter-band DL CA	Samsung	discussion

R4-2010537	Scope and TP of FR2 Inter-band DL CA in Rel-16	Nokia, Nokia Shanghai Bell	other

R4-2010538	Introduction of FR2 inter-band DL CA	Nokia, Nokia Shanghai Bell	CR

R4-2011390	Inter-band CA UE requirement fragmentation 	Qualcomm Incorporated	other

R4-2011420	Inter-band DL CA in FR2: CBM/IBM capability and associated spherical coverage EIS tests	Sony, Ericsson	other

R4-2011448	Remaining Details on FR2 Inter-band DL CA	Futurewei Technologies	discussion

R4-2011483	On inter band DL CA_FR2	Huawei, HiSilicon	other

R4-2011510	PSD for FR2 inter-band CA with IBM	NTT DOCOMO INC.	other

R4-2011452	LS on UL power boost mode and IBE relaxation	Qualcomm Incorporated	LS out

R4-2011453	CR to 38.101-2: FR2 UE EIRP increase with IBE relaxation	Qualcomm Incorporated	CR

R4-2011442	Beam squint considerations	FUTUREWEI	discussion

R4-2009766	Discussion on the remaining issues on MRTD requirement for FR2 inter-band CA	Xiaomi	discussion

R4-2009767	CR on MRTD requirement for FR2 inter-band CA	Xiaomi	CR

R4-2009984	MRTD/MTTD requirements in FR2 inter-band CA	Qualcomm	discussion

R4-2010051	On MRTD for inter-band CA	Apple	discussion

R4-2010056	CR on MRTD for inter-band CA	Apple	CR

R4-2010311	MRTD requirements for FR2 inter-band DL CA	MediaTek inc.	discussion

R4-2010616	Further study feasibility to support up to 3 us MRTD	Ericsson, KDDI Corporation, NTT DOCOMO INC	other

R4-2010617	Updates on MRTD and MTTD requirements for FR2 inter-band DL CA	Ericsson, KDDI Corporation, NTT DOCOMO INC	CR

R4-2010710	On MRTD requirement for FR2 DL inter-band CA	OPPO	discussion

R4-2010757	Discussion on MRTD requirement for FR2 inter-band DL CA	NEC	discussion

R4-2010758	CR to TS 38.133 on MRTD values for FR2 inter-band CA	NEC	CR

R4-2011062	Discussion on MRTD requirements for FR2 inter-band DL CA	Huawei, HiSilicon	discussion

R4-2011429	MRTD for FR2 inter-band DL CA	Nokia, Nokia Shanghai Bell	discussion

R4-2011430	CR on 38133 MRTD for FR2 inter-band DL CA	Nokia, Nokia Shanghai Bell	CR

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template