3GPP TSG-RAN WG1 Meeting #102-e 	R1-2007441

# eMeeting,  Aug 17 – 28, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | 0135 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to 38.213 on HARQ-ACK processing timeline for DCI format 1_1 with Scell dormancy indication without scheduling PDSCH |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Moderator (Ericsson) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NR_DC_CA_enh-Core |  |  |  |  |  | Date: |  |  | 2020-08-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The CR implements the following agreement made in [102-e-NR-MRDC-CA-Dormancy-01] email discussion Agreement: HARQ-ACK timeline of the Case 2 SCell dormancy indication DCI is defined by adding X symbols to the SPS release PDCCH HARQ-ACK timeline (i.e., X+N symbols) - X = 4, 4, 5, 6 for capability #1 for μ =0,1,2,3 - X = 2, 2, 4 for capability #2 for μ =0,1,2 |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Update the values on N (HARQ-ACK processing time) in sub-clause 10.3 to match the agreed values - Clarify in sub-clause 9.2.5 that N for DCI format 1_1 indicating SCell dormancy without scheduling PDSCH is given in sub-clause 10.3 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect HARQ-ACK processing time values used in the specification for DCI format 1_1 indicating SCell dormancy without scheduling PDSCH |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.5, 10.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Isolated Impact Analysis:. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 9.2.5	UE procedure for reporting multiple UCI types

<unchanged text omitted >

If a UE would transmit multiple overlapping PUCCHs in a slot or overlapping PUCCH(s) and PUSCH(s) in a slot and, when applicable as described in Clauses 9.2.5.1 and 9.2.5.2, the UE is configured to multiplex different UCI types in one PUCCH, and at least one of the multiple overlapping PUCCHs or PUSCHs is in response to a DCI format detection by the UE, the UE multiplexes all corresponding UCI types if the following conditions are met. If one of the PUCCH transmissions or PUSCH transmissions is in response to a DCI format detection by the UE, the UE expects that the first symbol  of the earliest PUCCH or PUSCH, among a group overlapping PUCCHs and PUSCHs in the slot, satisfies the following timeline conditions

-	 is not before a symbol with CP starting after Tproc,1mux after a last symbol of any corresponding PDSCH, Tproc,1mux is given by maximum of Tproc,1mux,1,⋯,Tproc,1mux,i,⋯ where for the i-th PDSCH with corresponding HARQ-ACK transmission on a PUCCH which is in the group of overlapping PUCCHs and PUSCHs, Tproc,1mux,i=N1+d1,1+1⋅2048+144⋅κ⋅2-μ⋅TC, d1,1 is selected for the i-th PDSCH following [6, TS 38.214], N1 is selected based on the UE PDSCH processing capability of the i-th PDSCH and SCS configuration μ, where μ corresponds to the smallest SCS configuration among the SCS configurations used for the PDCCH scheduling the i-th PDSCH (if any), the i-th PDSCH, the PUCCH with corresponding HARQ-ACK transmission for i-th PDSCH, and all PUSCHs in the group of overlapping PUCCHs and PUSCHs.

-	 is not before a symbol with CP starting after Tproc,releasemux after a last symbol of any corresponding SPS PDSCH release or of a DCI format 1_1 indicating SCell dormancy as described in Clause 10.3. Tproc,releasemux is given by maximum of Tproc,releasemux,1,⋯,Tproc,releasemux,i,⋯ where for the i-th PDCCH providing the SPS PDSCH release or the DCI format 1_1 with corresponding HARQ-ACK transmission on a PUCCH which is in the group of overlapping PUCCHs and PUSCHs, Tproc,releasemux,i=N+1⋅2048+144⋅κ⋅2-μ⋅TC, N is selected based on the UE PDSCH processing capability of the i-th SPS PDSCH release as described in Clause 10.2, or DCI format 1_1 that requests Type-3 HARQ-ACK codebook report as described in Clause 10.2, or the DCI format 1_1 indicating SCell dormancy as described in Clause 10.3 and SCS configuration μ, where μ corresponds to the smallest SCS configuration among the SCS configurations used for the PDCCH providing the i-th SPS PDSCH release, the PUCCH with corresponding HARQ-ACK transmission for i-th SPS PDSCH release or the DCI format 1_1, and all PUSCHs in the group of overlapping PUCCHs and PUSCHs.

-	if there is no aperiodic CSI report multiplexed in a PUSCH in the group of overlapping PUCCHs and PUSCHs,  is not before a symbol with CP starting after Tproc,2mux after a last symbol of

-	any PDCCH with the DCI format scheduling an overlapping PUSCH, and

-	any PDCCH scheduling a PDSCH or SPS PDSCH release with corresponding HARQ-ACK information in an overlapping PUCCH in the slot

If there is at least one PUSCH in the group of overlapping PUCCHs and PUSCHs, Tproc,2mux is given by maximum of Tproc,2mux,1,⋯,Tproc,2mux,i,⋯ where for the i-th PUSCH which is in the group of overlapping PUCCHs and PUSCHs, Tproc,2mux,i=maxN2+d2,1+1⋅2048+144⋅κ⋅2-μ⋅TC+Tswitch,d2,2, d2,1, d2,2 and Tswitch are selected for the i-th PUSCH following [6, TS 38.214], N2 is selected based on the UE PUSCH processing capability of the i-th PUSCH and SCS configuration μ, where μ corresponds to the smallest SCS configuration among the SCS configurations used for the PDCCH scheduling the i-th PUSCH (if any), the PDCCHs scheduling the PDSCHs with corresponding HARQ-ACK transmission on a PUCCH which is in the group of overlapping PUCCHs/PUSCHs, and all PUSCHs in the group of overlapping PUCCHs and PUSCHs.

If there is no PUSCH in the group of overlapping PUCCHs and PUSCHs, Tproc,2muxis given by maximum of Tproc,2mux,1,⋯,Tproc,2mux,i,⋯ where for the i-th PDSCH with corresponding HARQ-ACK transmission on a PUCCH which is in the group of overlapping PUCCHs, Tproc,2mux,i=N2+1⋅2048+144⋅κ⋅2-μ⋅TC, N2is selected based on the UE PUSCH processing capability of the PUCCH serving cell if configured.  N2 is selected based on the UE PUSCH processing capability 1, if PUSCH processing capability is not configured for the PUCCH serving cell. μ is selected based on the smallest SCS configuration between the SCS configuration used for the PDCCH scheduling the i-th PDSCH (if any) with corresponding HARQ-ACK transmission on a PUCCH which is in the group of overlapping PUCCHs, and the SCS configuration for the PUCCH serving cell.

-	if there is an aperiodic CSI report multiplexed in a PUSCH in the group of overlapping PUCCHs and PUSCHs,  is not before a symbol with CP starting after Tproc,CSImux=maxZ+d⋅2048+144⋅κ⋅2-μ⋅TC+Tswitch,d2,2 after a last symbol of

-	any PDCCH with the DCI format scheduling an overlapping PUSCH, and

-	any PDCCH scheduling a PDSCH, or SPS PDSCH release, or providing a DCI format 1_1 indicating SCell dormancy with corresponding HARQ-ACK information in an overlapping PUCCH in the slot

where μ corresponds to the smallest SCS configuration among the SCS configuration of the PDCCHs, the smallest SCS configuration for the group of the overlapping PUSCHs, and the smallest SCS configuration of CSI-RS associated with the DCI format scheduling the PUSCH with the multiplexed aperiodic CSI report, and d=2 for μ=0,1 , d=3 for μ=2 and d=4 for μ=3

-	N1, N2, d1,1, d2,1, d2,2, and Z are defined in [6, TS 38.214], and κ and TC are defined in [4, TS 38.211].

<unchanged text omitted >

## 10.3	PDCCH monitoring indication and dormancy/non-dormancy behaviour for SCells

<unchanged text omitted >

A UE is expected to provide HARQ-ACK information in response to a detection of a DCI format 1_1 indicating SCell dormancy after N symbols from the last symbol of a PDCCH providing the DCI format 1_1. If processingType2Enabled of PDSCH-ServingCellConfig is set to enable for the serving cell with the PDCCH providing the DCI format 1_1, N=57 for μ=0, N=57.5 for μ=1, and N=115 for μ=2; otherwise, N=104 for μ=0, N=126 for μ=1, N=227 for μ=2, and N=2531 for μ=3, where μ is the smallest SCS configuration between the SCS configuration of the PDCCH providing the DCI format 1_1 and the SCS configuration of a PUCCH with the HARQ-ACK information in response to the detection of the DCI format 1_1.