3GPP TSG-RAN WG1 Meeting #105-e	R1-2106354

# e-Meeting, May 10th – 27th, 2021

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR | 0187 | rev | 1 | Current version: | 16.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-16 editorial corrections for TS 38.214 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16, 5G_V2X_NRSL-Core, NR_eMIMO-Core, NR_pos-Core, NR_RF_FR1-Core |  |  |  |  |  | Date: |  |  | 2021-05-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In section 5.1.4.2, clarified the wording of common RS and also fixed a small error on the higher layer parameter RateMatchPatternLTE-CRS. In section 5.1.4.2, based on email discussion [104b-e-Prep-NR-R16-TEI], the higher layer parameter lte-CRS-PatternList-r16 should be replaced with lte-CRS-PatternList1-r16 to align with the RRC parameter naming in 38.331. In section 5.1.6.5, based on email discussion [104b-e-NR-Pos-01], performed an editorial change on the text DL-PRS-Resource. In section 5.5, based on email discussion [105-e-NR-MRDC-CA-01], a correction of the subscript when referencing the SCS of a PDSCH is needed as now ot refers to PDCCH, which is wrong. In sections 5.2.1.6 and 6.1, based on email discussions [104bis-e-NR-eMIMO-02], and as agreed in R1-2103991, two editorial corrections on parameter naming have been done. In section 6.1.2.1, based on the email discusion [104b-e-NR-L1enh-URLLC-03] and as summarized in R1-2104106, it was not clear whether or not the same TB is transmitted across actual PUSCH repetitions for PUSCH repetition Type B. In section 6.1.2.1, based on the email discusion [105-e-NR-R16-TxSwitching-01], some RRC parameters are not up to date. In section 6.1.6, based on email discussions [104b-e-NR-Rel16-TxSwitching-01], the higher layer parameter uplinkTxSwitchRequest is wrongly used. In section 6.2.1, based on email discussion [104b-e-NR-Pos-01], clarified the wording on what the higher layer parameter resourceMapping-r16 indicates. In section 6.3.1, based on email discussions [104b-e-NR-7.1CRs-09], and as agreed in R1-2103904, the following reason for change has been considered: in Rel-16, the frequency offset for PUSCH frequency hopping is described as follows For a PUSCH scheduled by RAR UL grant, fallbackRAR UL grant, or by DCI format 0_0 with CRC scrambled by TC-RNTI, frequency offsets are obtained as described in clause 8.3 of [6, TS 38.213]. For a PUSCH scheduled by DCI format 0_0/0_1 or a PUSCH based on a Type2 configured UL grant activated by DCI format 0_0/0_1 and for resource allocation type 1, frequency offsets are configured by higher layer parameter frequencyHoppingOffsetLists in pusch-Config. The case of “PUSCH scheduled by DCI format 0_0” as described in the second sentence also covers the case of “PUSCH scheduled by DCI format 0_0 scambled by TC-RNTI” in the first sentence. Therefore, it is not clear which frequency offsets are used for PUSCH scheduled by DCI format 0_0 with CRC scrambled by TC-RNTI. Note that in Rel-15, there is an “Otherwise” at the beginning of the second sentence. For a PUSCH scheduled by RAR UL grant or by DCI format 0_0 with CRC scrambled by TC-RNTI, frequency offsets are obtained as described in clause 8.3 of [6, TS 38.213]. Otherwise, for a PUSCH scheduled by DCI format 0_0/0_1 or a PUSCH based on a Type2 configured UL grant and for resource allocation type 1, frequency offsets are configured by higher layer parameter frequencyHoppingOffsetLists in pusch-Config: Besides the above, a square bracket is missing in the reference for TS 38.213 when the freqeuncy offset for MsgA PUSCH is descibed. In section 8.1.2, based on email discussions [104b-e-NR-5G_V2X-02], a reference to 38.213 specification is incorrect. In section 8.1.2.1, some small editorial changes improving the clarity of the text. In section 8.1.2.1, based on email discussions [105-e-NR-5G_V2X-04, the configuration index field in DCI format 3_0 corresponds to value 0~7 which is not aligned with SL-ConfigIndexCG-r16 which refer to 1~8. In section 8.1.4, based on email discussion [104b-e-NR-5G_V2X-03] and as summarized in R1-2104084, the higher layer parameter sl-ThresPSSCH-RSRP-List was not according to the higher layer specification. One subscript index is missing for S_A. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In section 5.1.4.2, replaced the term common RS by Cell-specfic RS and also fixed a small error on the higher layer parameter RateMatchPatternLTE-CRS. In section 5.1.4.2, based on email discussion [104b-e-Prep-NR-R16-TEI], the higher layer parameter lte-CRS-PatternList-r16 has been replaced with lte-CRS-PatternList1-r16 to align with the RRC parameter naming in 38.331. In section 5.1.6.5, based on email discussion [104b-e-NR-Pos-01], changed the text “DL-PRS-Resource” into “DL PRS resource”. In sections 5.2.1.6 and 6.1, based on email discussions [104bis-e-NR-eMIMO-02], and as agreed in R1-2103991, two editorial corrections on parameter naming have been done: word “is” was removed in section 5.2.1.6 and “-0” was added to higher layer parameter name in section 6.1. In section 5.5, based on email discussion [105-e-NR-MRDC-CA-01], corrected the subscript when referencing the SCS of a PDSCH. In section 6.1.2.1, based on the email discusion [104b-e-NR-L1enh-URLLC-03] and as summarized in R1-2104106, improved the quality of the specification. In section 6.1.2.1, based on the email discusion [105-e-NR-R16-TxSwitching-01], performed an update on RRC parematers. In section 6.1.6, based on email discussions [104b-e-NR-Rel16-TxSwitching-01], replaced the wrongly used parameter uplinkTxSwitchRequest with BandCombination-UplinkTxSwitch. In section 6.2.1, based on email discussion [104b-e-NR-Pos-01], clarified the wording on what the higher layer parameter resourceMapping-r16 indicates, previous text being unclear. In section 6.3.1, based on email discussions [104b-e-NR-7.1CRs-09], and as agreed in R1-2103904, clarified the frequency offset for PUSCH scheduled by DCI format 0_0 with CRC scrambled by TC-RNTI. Added a square bracket to the reference for TS 38.213. In section 8.1.2, based on email discussions [104b-e-NR-5G_V2X-02], corrected a reference to 38.213 specification. In section 8.1.2.1, performed some small editorial changes improving the clarity of the text. In section 8.1.2.1, based on email discussions [105-e-NR-5G_V2X-04, added one clarification with respect to the configuration index field in DCI format 3_0. Also corrected a higher layer parameter name alignment for sl-DCI-ToSL-Trans. In section 8.1.4, based on email discussion [104b-e-NR-5G_V2X-03] and as summarized in R1-2104084, updated the higher layer parameter sl-ThresPSSCH-RSRP-List into sl-Thres-RSRP-List. Subscript A has been added to S_A. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear specification. For the change in section 6.3.1, it is not clear which frequency offsets are used for PUSCH scheduled by DCI format 0_0 with CRC scrambled by TC-RNTI when PUSCH frequency hopping is enabled. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.4.2, 5.1.6.5, 5.2.1.6, 5.5, 6.1, 6.1.2.1, 6.1.6, 6.1.6.1, 6.2.1, 6.3.1, 8.1.2, 8.1.2.1, 8.1.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<omitted text>

#### 5.1.4.2	PDSCH resource mapping with RE level granularity

The procedures for PDSCH scheduled by PDCCH with DCI format 1_1 described in this clause equally apply to PDSCH scheduled by PDCCH with DCI format 1_2, by applying the parameters of aperiodicZP-CSI-RS-ResourceSetsToAddModListDCI-1-2 instead of aperiodic-ZP-CSI-RS-ResourceSetsToAddModList.

A UE may be configured with any of the following higher layer parameters:

-	REs indicated by the 'RateMatchPatternLTE-CRS' in lte-CRS-ToMatchAround in ServingCellConfig or ServingCellConfigCommon configuring cell-specific RS, in 15 kHz subcarrier spacing applicable only to 15 kHz subcarrier spacing PDSCH, of one LTE carrier in a serving cell are declared as not available for PDSCH.

-	REs indicated by RateMatchPatternLTE-CRS in lte-CRS-PatternList1-r16 in ServingCellConfig configuring cell-specific RS, in 15 kHz subcarrier spacing applicable only to 15 kHz subcarrier spacing PDSCH, of one LTE carrier in a serving cell are declared as not available for PDSCH.

-	Each RateMatchPatternLTE-CRS configuration contains v-Shift consisting of LTE-CRS-vshift(s), nrofCRS-Ports consisting of LTE-CRS antenna ports 1, 2 or 4 ports, carrierFreqDL representing the offset in units of 15 kHz subcarriers from (reference) point A to the LTE carrier centre subcarrier location, carrierBandwidthDL representing the LTE carrier bandwidth, and may also configure mbsfn-SubframeConfigList representing MBSFN subframe configuration. A UE determines the CRS position within the slot according to Clause 6.10.1.2 in [15, TS 36.211], where slot corresponds to LTE subframe.

-	If the UE is configured by higher layer parameter PDCCH-Config with two different values of coresetPoolIndex in ControlResourceSet and is also configured by the higher layer parameter lte-CRS-PatternList1-r16 and lte-CRS-PatternList2-r16 in ServingCellConfig, the following REs are declared as not available for PDSCH:

-	if the UE is configured with crs-RateMatch-PerCoresetPoolIndex, REs indicated by the CRS pattern(s) in lte-CRS-PatternList1-r16 if the PDSCH is associated with coresetPoolIndex set to '0', or the CRS pattern(s) in lte-CRS-PatternList2-r16 if PDSCH is associated with coresetPoolIndex set to '1';

-	otherwise, REs indicated by lte-CRS-PatternList1-r16 and lte-CRS-PatternList2-r16, in ServingCellConfig.

<omitted text>

#### 5.1.6.5	PRS reception procedure

<omitted text>

A DL PRS resource is defined by:

-	nr-DL-PRS-ResourceID determines the DL PRS resource configuration identity. All DL PRS resource IDs are locally defined within a DL PRS resource set.

-	dl-PRS-SequenceID is used to initialize cinit value used in pseudo random generator as described in Clause 7.4.1.7.2  of [4, TS 38.211] for generation of DL PRS sequence for a given DL PRS resource.

-	dl-PRS-CombSizeN-AndReOffset defines the starting RE offset of the first symbol within a DL PRS resource in frequency. The relative RE offsets of the remaining symbols within a DL PRS resource are defined based on the initial offset and the rule described in Clause 7.4.1.7.3 of [4, TS 38.211].

-	dl-PRS-ResourceSlotOffset determines the starting slot of the DL PRS resource with respect to corresponding DL PRS resource set slot offset.

-	dl-PRS-ResourceSymbolOffset determines the starting symbol of a slot configured with the DL PRS resource.

-	dl-PRS-QCL-Info defines any quasi co-location information of the DL PRS resource with other reference signals. The DL PRS may be configured with QCL 'typeD' with a DL PRS from a serving cell or a non-serving cell, or with rs-Type set to 'typeC', 'typeD', or 'typeC-plus-typeD' with a SS/PBCH Block from a serving or non-serving cell.

The UE assumes constant EPRE is used for all REs of a given DL PRS resource.

<omitted text>

If the UE is configured with DL-PRS-QCL-Info and the QCL relation is between two DL PRS resources, then the UE assumes those DL PRS resources are associated with the same dl-PRS-ID. If DL-PRS-QCL-Info is configured to the UE with qcl-Type set to 'type-D' with a source DL PRS resource then the nr-DL-PRS-ResourceSetId and the nr-DL-PRS-ResourceId of the source DL PRS resource are expected to be indicated to the UE.

<omitted text>

#### 5.2.1.6	CSI processing criteria

The UE indicates the number of supported simultaneous CSI calculations NCPU with parameter simultaneousCSI-ReportsPerCC in a component carrier, and simultaneousCSI-ReportsAllCC across all component carriers. If a UE supports NCPU simultaneous CSI calculations it is said to have NCPU CSI processing units for processing CSI reports. If L CPUs are occupied for calculation of CSI reports in a given OFDM symbol, the UE has NCPU-L unoccupied CPUs. If N CSI reports start occupying their respective CPUs on the same OFDM symbol on which NCPU-L CPUs are unoccupied, where each CSI report n=0, …, N-1 corresponds to OCPU(n), the UE is not required to update the N-M requested CSI reports with lowest priority (according to Clause 5.2.5), where 0≤M≤N is the largest value such that n=0M-1OCPU(n)≤NCPU-L  holds.

A UE is not expected to be configured with an aperiodic CSI trigger state containing more than NCPU Reporting Settings. Processing of a CSI report occupies a number of CPUs for a number of symbols as follows:

-	OCPU=0  for a CSI report with CSI-ReportConfig with higher layer parameter reportQuantity set to 'none' and CSI-RS-ResourceSet with higher layer parameter trs-Info configured

-	OCPU=1  for a CSI report with CSI-ReportConfig with higher layer parameter reportQuantity set to 'cri-RSRP', 'ssb-Index-RSRP', 'cri-SINR', 'ssb-Index-SINR' or 'none' (and CSI-RS-ResourceSet with higher layer parameter trs-Info not configured)

-	for a CSI report with CSI-ReportConfig with higher layer parameter reportQuantity set to 'cri-RI-PMI-CQI', 'cri-RI-i1', 'cri-RI-i1-CQI', 'cri-RI-CQI', or 'cri-RI-LI-PMI-CQI',

-	if a CSI report is aperiodically triggered without transmitting a PUSCH with either transport block or HARQ-ACK or both when L = 0 CPUs are occupied, where the CSI corresponds to a single CSI with wideband frequency-granularity and to at most 4 CSI-RS ports in a single resource without CRI report and where codebookType is set to 'typeI-SinglePanel' or where reportQuantity is set to 'cri-RI-CQI', OCPU=NCPUOCPU=NCPU,

-	otherwise, OCPU=KsOCPU=KS, where Ks Ks is the number of CSI-RS resources in the CSI-RS resource set for channel measurement.

For a CSI report with CSI-ReportConfig with higher layer parameter reportQuantity not set to 'none', the CPU(s) are occupied for a number of OFDM symbols as follows:

-	A periodic or semi-persistent CSI report (excluding an initial semi-persistent CSI report on PUSCH after the PDCCH triggering the report) occupies CPU(s) from the first symbol of the earliest one of each CSI-RS/CSI-IM/SSB resource for channel or interference measurement, respective latest CSI-RS/CSI-IM/SSB occasion no later than the corresponding CSI reference resource, until the last symbol of the configured PUSCH/PUCCH carrying the report.

-	An aperiodic CSI report occupies CPU(s) from the first symbol after the PDCCH triggering the CSI report until the last symbol of the scheduled PUSCH carrying the report.

-	An initial semi-persistent CSI report on PUSCH after the PDCCH trigger occupies CPU(s) from the first symbol after the PDCCH until the last symbol of the scheduled PUSCH carrying the report.

For a CSI report with CSI-ReportConfig with higher layer parameter reportQuantity set to 'none' and CSI-RS-ResourceSet with higher layer parameter trs-Info not configured, the CPU(s) are occupied for a number of OFDM symbols as follows:

-	A semi-persistent CSI report (excluding an initial semi-persistent CSI report on PUSCH after the PDCCH triggering the report) occupies CPU(s) from the first symbol of the earliest one of each transmission occasion of periodic or semi-persistent CSI-RS/SSB resource for channel measurement for L1-RSRP computation, until Z3' symbols after the last symbol of the latest one of the CSI-RS/SSB resource for channel measurement for L1-RSRP computation in each transmission occasion.

-	An aperiodic CSI report occupies CPU(s) from the first symbol after the PDCCH triggering the CSI report until the last symbol between Z3 symbols after the first symbol after the PDCCH triggering the CSI report and Z3' symbols after the last symbol of the latest one of each CSI-RS/SSB resource for channel measurement for L1-RSRP computation.

where (Z3,Z3') are defined in the table 5.4-2.

<omitted text>

## 5.5	UE PDSCH reception preparation time with cross carrier scheduling with different subcarrier spacings for PDCCH and PDSCH

This clause applies only if the PDCCH carrying the scheduling DCI is received on one carrier with one OFDM subcarrier spacing (µPDCCH), and the PDSCH scheduled to be received by the DCI is on another carrier with another OFDM subcarrier spacing (µPDSCH).

<omitted text>

## 6.1	UE procedure for transmitting the physical uplink shared channel

<omitted text>

For PUSCH scheduled by DCI format 0_0 on a cell, the UE shall transmit PUSCH according to the spatial relation, if applicable, corresponding to the dedicated PUCCH resource with the lowest ID within the active UL BWP of the cell, as described in Clause 9.2.1 of [6, TS 38.213].

For PUSCH scheduled by DCI format 0_0 on a cell and if the higher layer parameter enableDefaultBeamPL-ForPUSCH0-0 is set 'enabled', the UE is not configured with PUCCH resources on the active UL BWP and the UE is in RRC connected mode, the UE shall transmit PUSCH according to the spatial relation, if applicable, with a reference to the RS configured with qcl-Type set to 'typeD' corresponding to the QCL assumption of the CORESET with the lowest ID on the active DL BWP of the cell.

For PUSCH scheduled by DCI format 0_0 on a cell and if the higher layer parameter enableDefaultBeamPL-ForPUSCH0-0 is set 'enabled', the UE is configured with PUCCH resources on the active UL BWP where all the PUCCH resource(s) are not configured with any spatial relation and the UE is in RRC connected mode, the UE shall transmit PUSCH according to the spatial relation, if applicable, with a reference to the RS configured with qcl-Type set to 'typeD' corresponding to the QCL assumption of the CORESET with the lowest ID on the active DL BWP of the cell in case CORESET(s) are configured on the cell.

For uplink, 16 HARQ processes per cell is supported by the UE.

<omitted text>

#### 6.1.2.1	Resource allocation in time domain

<omitted text>

For PUSCH repetition Type B, after determining the invalid symbol(s) for PUSCH repetition type B transmission for each of the K nominal repetitions, the remaining symbols are considered as potentially valid symbols for PUSCH repetition Type B transmission. If the number of potentially valid symbols for PUSCH repetition type B transmission is greater than zero for a nominal repetition, the nominal repetition consists of one or more actual repetitions, where each actual repetition consists of a consecutive set of all potentially valid symbols that can be used for PUSCH repetition Type B transmission within a slot. An actual repetition with a single symbol is omitted except for the case of L=1. An actual repetition is omitted according to the conditions in Clause 9, Clause 11.1 and Clause 11.2A of [6, TS38.213].  The UE shall repeat the TB across actual repetitions. The redundancy version to be applied on the nth actual repetition (with the counting including the actual repetitions that are omitted) is determined according to table 6.1.2.1-2.

<omitted text>

### 6.1.6	Uplink switching

The UE may omit uplink transmission during the uplink switching gap NTx1-Tx2 if the conditions defined in this clause are met and the UE is configured with uplinkTxSwitching. The switching gap NTx1-Tx2 is indicated by UE capability uplinkTxSwitchingPeriod:

-	If a UE indicated a capability for uplink switching with BandCombination-UplinkTxSwitch for a band combination, and if it is for that band combination

-	Configured with a MCG using E-UTRA radio access and with a SCG using NR radio access (EN-DC), or

-	Configured with uplink carrier aggregation, or

-	Configured in a serving cell with two uplink carriers with higher layer parameter supplementaryUplink.

the conditions under which the switching gap may be present and the location of the switching gap are defined for each of the cases in clauses 6.1.6.1, 6.1.6.2, and 6.1.6.3 respectively.

If an uplink switching is triggered for an uplink transmission starting at T0, after T0-Toffset, the UE is not expected to cancel the uplink switching, or to trigger any other new uplink switching occurring before T0 for any other uplink transmission that is scheduled after T0-Toffset, where Toffset is the UE processing procedure time defined for the uplink transmission triggering the switch given in clause 5.3, clause 5.4, clause 6.2.1, clause 6.4 and in clause 9 of [6, TS 38.213].

The UE does not expect to perform more than one uplink switching in a slot with µUL = max(µUL, 1, µUL, 2), where the µUL, 1 corresponds to the subcarrier spacing of the active UL BWP of one uplink carrier before the switching gap and the µUL, 2 corresponds to the subcarrier spacing of the active UL BWP of the other uplink carrier after the switching gap.

<omitted text>

#### 6.1.6.1	Uplink switching for EN-DC

For a UE indicating a capability for uplink switching with BandCombination-UplinkTxSwitch for a band combination, and if it is for that band combination configured with a MCG using E-UTRA radio access and with a SCG using NR radio access (EN-DC), if the UE is configured with uplink switching with parameter uplinkTxSwitching,

-	for the UE configured with switchedUL by the parameter uplinkTxSwitchingOption, when the UE is to transmit in the uplink based on DCI(s) received before T0-Toffset or based on a higher layer configuration(s):

-	when the UE is to transmit an NR uplink that takes place after an E-UTRA uplink on another uplink carrier then the UE is not expected to transmit for the duration of NTx1-Tx2 on any of the two carriers.

-	when the UE is to transmit an E-UTRA uplink that takes place after an NR uplink on another uplink carrier then the UE is not expected to transmit for the duration of NTx1-Tx2 on any of the two carriers.

-	the UE is not expected to transmit simultaneously on the NR uplink and the E-UTRA uplink. If the UE is scheduled or configured to transmit any NR uplink transmission overlapping with an E-UTRA uplink transmission, the NR uplink transmission is dropped,

-	for the UE configured with uplinkTxSwitchingOption set to 'dualUL', when the UE is to transmit in the uplink based on DCI(s) received before T0-Toffset or based on a higher layer configuration(s):

-	when the UE is to transmit an NR two-port uplink that takes place after an E-UTRA uplink on another uplink carrier then the UE is not expected to transmit for the duration of NTx1-Tx2 on any of the two carriers.

-	when the UE is to transmit an E-UTRA uplink that takes place after an NR two-port uplink on another uplink carrier then the UE is not expected to transmit for the duration of NTx1-Tx2 on any of the two carriers.

-	the UE is not expected to transmit simultaneously a two- port transmission on the NR uplink and the E-UTRA uplink.

-	in all other cases the UE is expected to transmit normally all uplink transmissions without interruptions.

-	when the UE is configured with tdm-PatternConfig or by tdm-PatternConfig2

-	for the E-UTRA subframes designated as uplink by the configuration, the UE assumes the operation state in which one-port E-UTRA uplink can be transmitted.

-	for the E-UTRA subframes other than the ones designated as uplink by the configuration, the UE assumes the operation state in which two-port NR uplink can be transmitted.

<omitted text>

### 6.2.1	UE sounding procedure

<omitted text>

The UE may be configured by the higher layer parameter resourceMapping in SRS-Resource with an SRS resource occupying  adjacent OFDM symbols within the last 6 symbols of the slot, or at any symbol location within the slot if resourceMapping-r16 is provided subject to UE capability, where all antenna ports of the SRS resources are mapped to each symbol of the resource. When the SRS is configured with the higher layer parameter SRS-PosResourceSet the higher layer parameter resourceMapping-r16 in SRS-PosResource indicates an SRS resource occupying NS∈1,2,4,8,12 adjacent symbols anywhere within the slot.

<omitted text>

### 6.3.1	Frequency hopping for PUSCH repetition Type A

For PUSCH repetition Type A (as determined according to procedures defined in Clause 6.1.2.1 for scheduled PUSCH, or Clause 6.1.2.3 for configured PUSCH), a UE is configured for frequency hopping by the higher layer parameter frequencyHoppingDCI-0-2 in pusch-Config for PUSCH transmission scheduled by DCI format 0_2, and by frequencyHopping provided in pusch-Config for PUSCH transmission scheduled by a DCI format other than 0_2, and by frequencyHopping provided in configuredGrantConfig for configured PUSCH transmission. One of two frequency hopping modes can be configured:

-	Intra-slot frequency hopping, applicable to single slot and multi-slot PUSCH transmission.

-	Inter-slot frequency hopping, applicable to multi-slot PUSCH transmission.

In case of resource allocation type 2, the UE transmits PUSCH without frequency hopping.

In case of resource allocation type 1, whether or not transform precoding is enabled for PUSCH transmission, the UE may perform PUSCH frequency hopping, if the frequency hopping field in a corresponding detected DCI format or in a random access response UL grant is set to 1, or if for a Type 1 PUSCH transmission with a configured grant the higher layer parameter frequencyHoppingOffset is provided, otherwise no PUSCH frequency hopping is performed. When frequency hopping is enabled for PUSCH, the RE mapping is defined in clause 6.3.1.6 of [4, TS 38.211].

For a PUSCH scheduled by RAR UL grant, fallbackRAR UL grant, or by DCI format 0_0 with CRC scrambled by TC-RNTI, frequency offsets are obtained as described in clause 8.3 of [6, TS 38.213]. Otherwise, for a PUSCH scheduled by DCI format 0_0/0_1 or a PUSCH based on a Type2 configured UL grant activated by DCI format 0_0/0_1 and for resource allocation type 1, frequency offsets are configured by higher layer parameter frequencyHoppingOffsetLists in pusch-Config. For a PUSCH scheduled by DCI format 0_2 or a PUSCH based on a Type2 configured UL grant activated by DCI format 0_2 and for resource allocation type 1, frequency offsets are configured by higher layer parameter frequencyHoppingOffsetListsDCI-0-2 in pusch-Config.

-	When the size of the active BWP is less than 50 PRBs, one of two higher layer configured offsets is indicated in the UL grant.

-	When the size of the active BWP is equal to or greater than 50 PRBs, one of four higher layer configured offsets is indicated in the UL grant.

For PUSCH based on a Type1 configured UL grant the frequency offset is provided by the higher layer parameter frequencyHoppingOffset in rrc-ConfiguredUplinkGrant.

For a MsgA PUSCH the frequency offset is provided by the higher layer parameter as described in [6, TS 38.213].

<omitted text>

### 8.1.2	Resource allocation

In sidelink resource allocation mode 1:

-	for PSSCH and PSCCH transmission, dynamic grant, configured grant type 1 and configured grant type 2 are supported. The configured grant Type 2 sidelink transmission is semi-persistently scheduled by a SL grant in a valid activation DCI according to Clause 10.2A of [6, TS 38.213].

#### 8.1.2.1	Resource allocation in time domain

The UE shall transmit the PSSCH in the same slot as the associated PSCCH.

The minimum resource allocation unit in the time domain is a slot.

The UE shall transmit the PSSCH in consecutive symbols within the slot, subject to the following restrictions:

-	The UE shall not transmit PSSCH in symbols which are not configured for sidelink. A symbol is configured for sidelink, according to higher layer parameters startSLsymbols and lengthSLsymbols, where startSLsymbols is the symbol index of the first symbol of lengthSLsymbols consecutive symbols configured for sidelink.

-	Within the slot, PSSCH resource allocation starts at symbol startSLsymbols+1.

-	The UE shall not transmit PSSCH in symbols which are configured for use by PSFCH, if PSFCH is configured in this slot.

-	The UE shall not transmit PSSCH in the last symbol configured for sidelink.

-	The UE shall not transmit PSSCH in the symbol immediately preceding the symbols which are configured for use by PSFCH, if PSFCH is configured in this slot.

In sidelink resource allocation mode 1:

-	For sidelink dynamic grant, the PSSCH transmission is scheduled by a DCI format 3_0.

-	For sidelink configured grant type 2, the configured grant is activated by a DCI format 3_0.

-	For sidelink dynamic grant and sidelink configured grant type 2:

-	The "Time gap" field value m of the DCI format 3_0 provides an index m + 1 into a slot offset table. That table is given by higher layer parameter sl-DCI-ToSL-Trans and the table value at index m + 1 will be referred to as slot offset KSL.

-	The slot of the first sidelink transmission scheduled by the DCI is the first SL slot of the corresponding resource pool that starts not earlier than  TDL-TTA2+KSL×Tslot , where TDL is the starting time of the downlink slot carrying the corresponding DCI, TTA is the timing advance value corresponding to the TAG of the serving cell on which the DCI is received and KSL is the slot offset between the slot of the DCI and the first sidelink transmission scheduled by DCI and Tslott Tslot is the SL slot duration.

-	The “Configuration index” field of the DCI format 3_0, if provided and not reserved, indicates the index of the sidelink configured type 2.

-	For sidelink configured grant type 1:

-	The slot of the first sidelink transmissions follows the higher layer configuration according to [10, TS 38.321].

<omitted text>

### 8.1.4	UE procedure for determining the subset of resources to be reported to higher layers in PSSCH resource selection in sidelink resource allocation mode 2

In resource allocation mode 2, the higher layer can request the UE to determine a subset of resources from which the higher layer will select resources for PSSCH/PSCCH transmission. To trigger this procedure, in slot n, the higher layer provides the following parameters for this PSSCH/PSCCH transmission:

-	the resource pool from which the resources are to be reported;

-	L1 priority, prioTX;

-	the remaining packet delay budget;

-	the number of sub-channels to be used for the PSSCH/PSCCH transmission in a slot, LsubCH;

-	optionally, the resource reservation interval, Prsvp_TX, in units of msec.

-	if the higher layer requests the UE to determine a subset of resources from which the higher layer will select resources for PSSCH/PSCCH transmission as part of re-evaluation or pre-emption procedure, the higher layer provides a set of resources (r0,r1,r2,…) which may be subject to re-evaluation and a set of resources (r0',r1',r2',…) which may be subject to pre-emption.

-	it is up to UE implementation to determine the subset of resources as requested by higher layers before or after the slot ri'' - T3, where ri'' is the slot with the smallest slot index among (r0,r1,r2,…) and (r0',r1',r2',…) , and T3 is equal to Tproc,1SL, where Tproc,1SL  is defined in slots in Table 8.1.4-2 where μSL is the SCS configuration of the SL BWP.

The following higher layer parameters affect this procedure:

-	sl-SelectionWindowList: internal parameter T2min is set to the corresponding value from higher layer parameter sl-SelectionWindowList for the given value of prioTX.

-	sl-Thres-RSRP-List: this higher layer parameter provides an RSRP threshold for each combination pi, pj, where pi is the value of the priority field in a received SCI format 1-A and pj is the priority of the transmission of the UE selecting resources; for a given invocation of this procedure, pj = prioTX.

-	sl-RS-ForSensing selects if the UE uses the PSSCH-RSRP or PSCCH-RSRP measurement, as defined in clause 8.4.2.1.

-	sl-ResourceReservePeriodList

-	sl-SensingWindow: internal parameter T0 is defined as the number of slots corresponding to sl-SensingWindow msec

-	sl-TxPercentageList: internal parameter X for a given prioTX is defined as sl-TxPercentageList (prioTX) converted from percentage to ratio

-	sl-PreemptionEnable: if sl-PreemptionEnable is provided, and if it is not equal to 'enabled', internal parameter priopre is set to the higher layer provided parameter sl-PreemptionEnable

The resource reservation interval, Prsvp_TX, if provided, is converted from units of msec to units of logical slots, resulting in Prsvp_TX' according to clause 8.1.7.

Notation:

t'0SL, t'1SL, t'2SL,… denotes the set of slots which belongs to the sidelink resource pool and is defined in Clause 8.

The following steps are used:

1)	A candidate single-slot resource for transmission Rx,y is defined as a set of LsubCH contiguous sub-channels with sub-channel x+j in slot t'ySL where j=0,...,LsubCH-1. The UE shall assume that any set of LsubCH contiguous sub-channels included in the corresponding resource pool within the time interval [n+T1,n+T2] correspond to one candidate single-slot resource, where

-	selection of T1 is up to UE implementation under 0 ≤ T1≤ Tproc,1SL  , where Tproc,1SL  is defined in slots in Table 8.1.4-2 where μSL is the SCS configuration of the SL BWP;

-	if  T2min is shorter than the remaining packet delay budget (in slots) then T2 is up to UE implementation subject to  T2min ≤ T2 ≤ remaining packet delay budget (in slots); otherwise T2 is set to the remaining packet delay budget (in slots).

The total number of candidate single-slot resources is denoted by Mtotal.

2)	The sensing window is defined by the range of slots [n –T0,n–Tproc,0SL) where T0 is defined above and Tproc,0SL is defined in slots in Table 8.1.4-1 where μSL is the SCS configuration of the SL BWP. The UE shall monitor slots which belongs to a sidelink resource pool within the sensing window except for those in which its own transmissions occur. The UE shall perform the behaviour in the following steps based on PSCCH decoded and RSRP measured in these slots.

3)	The internal parameter Th(pi,pj)  is set to the corresponding value of RSRP threshold indicated by the i-th field in sl-Thres-RSRP-List, where i=pi+pj-1*8.

4)	The set SA is initialized to the set of all the candidate single-slot resources.

5)	The UE shall exclude any candidate single-slot resource Rx,y from the set SA if it meets all the following conditions:

-	the UE has not monitored slot t'mSL in Step 2.

-	for any periodicity value allowed by the higher layer parameter sl-ResourceReservePeriodList and a hypothetical SCI format 1-A received in slot t'mSL with 'Resource reservation period' field set to that periodicity value and indicating all subchannels of the resource pool in this slot, condition c in step 6 would be met.

6)	The UE shall exclude any candidate single-slot resource Rx,y from the set SA if it meets all the following conditions:

a)	the UE receives an SCI format 1-A in slot t'mSL, and 'Resource reservation period' field, if present, and 'Priority' field in the received SCI format 1-A indicate the values Prsvp_RX and prioRX, respectively according to Clause 16.4 in [6, TS 38.213];

b)	the RSRP measurement performed, according to clause 8.4.2.1 for the received SCI format 1-A, is higher than ThprioRX,prioTX;

c)	the SCI format received in slot t'mSL or the same SCI format which, if and only if the 'Resource reservation period' field is present in the received SCI format 1-A, is assumed to be received in slot(s) t'm+q×Prsvp_RX'SL determines according to clause 8.1.5 the set of resource blocks and slots which overlaps with Rx,y+j×Prsvp_TX' for q=1, 2, …, Q and j=0, 1, …, Cresel-1. Here, Prsvp_RX' is Prsvp_RX converted to units of logical slots according to clause 8.1.7, Q=TscalPrsvp_RX  if Prsvp_RX< Tscal and  n'-m≤Prsvp_RX', where t'n'SL = n if slot n belongs to the set t'0SL,t'1SL,...,t'T'max-1SL, otherwise slot t'n'SL is the first slot after slot n belonging to the set t'0SL,t'1SL,...,t'T'max-1SL; otherwise Q=1. Tscal is set to selection window size T2 converted to units of msec.

7)	If the number of candidate single-slot resources remaining in the set SA is smaller than X⋅Mtotal, then Th(pi,pj) is increased by 3 dB for each priority value Th(pi,pj) and the procedure continues with step 4.

The UE shall report set SA to higher layers.

If a resource ri from the set (r0,r1,r2,…) is not a member of SA, then the UE shall report re-evaluation of the resource ri to higher layers.

If a resource ri' from the set (r0',r1',r2',…) meets the conditions below then the UE shall report pre-emption of the resource ri' to higher layers

-	ri' is not a member of SAS, and

-	ri' meets the conditions for exclusion in step 6, with ThprioRX,prioTX set to the final threshold after executing steps 1)-7), i.e. including all necessary increments for reaching X⋅Mtotal, and

-	the associated priority prioRX, satisfies one of the following conditions:

-	sl-PreemptionEnable is provided and is equal to 'enabled' and prioTX>prioRX

-	sl-PreemptionEnable is provided and is not equal to 'enabled', and prioRX<priopre and prioTX>prioRX

<omitted text>