3GPP TSG-RAN Meeting #92-e 											RP-211578 (Rev of RP-211531)

Electronic Meeting, 19th – 27th May 2021

Agenda item:			9.1.4, 9.13

Source:	Moderator (RAN4 Chair)

Title:	Email discussion summary for [92-e-05-Spectrum-WIs]

Document for:	Information

Introduction

In this document, we capture comments and responses in the email thread of [92-e-05-Spectrum-WIs] for new RAN4-led spectrum related WI proposals. Based on the discussions the recommendations will be provided. The following documents under the agenda 9.1.4 and 9.13 will be covered in this email thread.

| TDoc | Title | Source | Type | AI |
| --- | --- | --- | --- | --- |
| RP-211202 | New WID: High-power UE operation for fixed-wireless/vehicle-mounted use cases in LTE bands 5 and 12 and NR band n71 | Nokia, Nokia Shanghai Bell | WID new | 9.1.4 |
| RP-211283 | New WID on LTE/NR spectrum sharing in Band 34/n34 | CMCC | WID new | 9.1.4 |
| RP-211284 | New WID on LTE/NR spectrum sharing in Band 39/n39 | CMCC | WID new | 9.1.4 |
| RP-211393 | New WID on DC of x bands (x=1,2,3) LTE inter-band CA (xDL/1UL) and 4 bands NR inter-band CA (4DL/1UL) | Huawei, HiSilicon | WID new | 9.1.4 |
| RP-211445 | Motivation for Introduction of the 6GHz unlicensed band in other countries/regions | Apple Inc. | discussion | 9.1.4 |
| RP-211446 | New WID for Introduction of the 6GHz unlicensed band in other countries/regions | Apple Inc. | WID new | 9.1.4 |
| RP-211305 | Improved MSD for CA and DC | Qualcomm Incorporated | other | 9.13 |

Topic #1: New WI for HPUE on B5, B12 and n71

Companies’ contributions summary

| T-doc number | Title | Sourcing company |
| --- | --- | --- |
| RP-211202 | New WID: High-power UE operation for fixed-wireless/vehicle-mounted use cases in LTE bands 5 and 12 and NR band n71 | Nokia, Nokia Shanghai Bell |

Initial round

Comments & responses

In this section, we collect the comments and responses for the proposed work item. Based on the comments, we will decide how to move forward in the next step.

Sub-topic 1-1: Any question or comment on the justification or any other general comment?

Companies are invited to provide the general comments, including comments on justification part, whether the WI is needed, how to handle the work, in the follow table.

| Company | Comments |
| --- | --- |
| Qualcomm | It is unclear how the requirments for PC1 in Band n71 should be derived. While the SI included example “product brief” (not a datasheet) for Band 5 and Band 12 filters, there was none provided for Band n71. Band n71 is challenging due to the small duplex offset. Even for PC3 significant refsens degradation was specified. We suggest that Band n71 is removed from the scope of this WI. |
| T-Mobile USA | We disagree with Qualcomm. As far as we are aware, there has never been a requirement in RAN4 that a datasheet must be provided prior to the start of a Work Item. The Study Item Technical Report concluded that there were no hardware problems, but that for any of the bands UE REFSENS exceptions may be needed and should be studied during the WI phase. From the TR [RP-210985]: “HPUE operation impacts on Band 12, Band 5, and Band n71 BS and UE performance have been studied in terms of UE transmitter harmonics and UE self-dense, as well as BS receiver blocking. No need has been identified for any additional UE requirements or relaxations due to uplink harmonics, nor any change in the BS receiver blocking requirements. On the other hand, UE REFSENS exception due to self-dense may be needed, and this should be further discussed and agreed during the WI phase.” “HPUE hardware and software requirements in Band 12, Band 5, and Band n71 as well as implementation feasibility have been studied, and no major implementation issue has been found, given the target form factors for fixed-wireless/vehicle-mounted use cases.” The proposal to remove n71 is not supported by the conclusion of the SI TR. T-Mobile cannot accept approval of the Work Item if n71 is removed. |
| LGE | In our understanding, the new WI is for PC1 UE operation with NR Uu interface for FWA and Vehicular UE in the candidate bands. For the n71, we have same view with Qualcomm on that small duplexer gap can have impact on the desense. RAN4 can further discuss on this impacts if current objectives are kept. |
| Nokia | Band n71 filter can be further discussed when relating requirements like MRP are decided in the WI phase. |
| AT&T | Concerning the PC1 requirements for n71, the existing PC1 requirements (MPR, ACLR, etc.) for band n14 should be leveraged where possible. We think that this was the intent when RAN4 decided to add n14 PC1 in Rel-16 with the goal that the general PC1 requirements would align. |
| Skyworks | Regarding PC1 for n71, PC1 has been specified for other bands that have small duplex gaps like 400MHz bands and band 12 also has a small duplex gap. The FWA form factor enables different duplexer technologies to be used and also the PA has much higher linearity. We do see the benefit to use n71 for FWA and RAN4 can study how to mitigate potential MSD based on duplexer isolation and PA linearity assumptions. Note that UL BW is limited to 20MHz and UL allocation can be optimized for good link performance even if the worst case allocation results in MSD. We support to keep n71 in the scope. |
| ZTE | Clarification questions: For release independence in the performance part, is it from Rel-15 or Rel-16 intended? |
| Verizon | We support and cosign this work item! We also want to include following bands in scope of this work item, - band n5 - band 13, and - band n13 |

Sub-topic 1-2: Comments and responses on proposed objectives

The following objectives are proposed.

Core part:

As stated in the conclusion of TR 37.880, the following topics should be further discussed and agreed during the WI phase:

1.	Coexistence studies between HPUE in Band 5 and adjacent channel public safety operation in the same geographical area.

2.	UE REFSENS exception due to self-dense.

3.	UE MOP, MPR, A-MPR, and ACLR, considering the interactions between them (e.g. MRP and ACLR) and other related parameters (e.g. larger NR spectrum utilization for band n71).

The corresponding HPUE requirements for each band can be included in the RAN4 specifications independently when the work on this band is complete, i.e. no need to wait for the completion of other bands.

Performance part:

Specify, if necessary, the performance requirements such as release independence in TS 36.307 and TS 38.307.

Companies are invited to provide comments and responses in the following table.

| Company | Comments |
| --- | --- |
| Qualcomm | In bullet 3, MPR is misspelled as MRP. In the last sentence for the core part, the general requirements i.e., MPR, ACLR, need to be completed first before the HPUE requirements for any band can be included in the specifications. |
| Nokia | Will correct the typo in the revision, thanks. The intention here is that HPUE requirements for a band can be included in the specifications once all requirements for this band are agreed, there is no need to e.g. wait for MRP for other bands to be agreed on first. |
| AT&T | For bullet 3, see our comment on sub-topic 1-1. |
| Skyworks | For band n71 and PC1, MPR needs to be re-assessed for BW > 10MHz due to larger SU of NR and for the ACLR requirement based on the coexistence studies. |
| ZTE | Clarification question: What is the exact meaning of “larger NR spectrum utilization for band n71” in Objective 3. Do you mean a higher SU dedicated for n71 is targeted? |

Sub-topic 1-3: Comments and responses on impacted/new specifications and target completion date

The proposed impacted specifications as well as target completion date are as follows:

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| 36.101 | Introduce core requirements for high-power UE operation for fixed-wireless/vehicle-mounted use cases in Band 12 and Band 5. | RAN#95 | Core part |
| 38.101-1 | Introduce core requirements for high-power UE operation for fixed-wireless/vehicle-mounted use cases in Band n71. | RAN#95 | Core part |
| 36.307 | Define high-power UE operation for fixed-wireless/vehicle-mounted use cases in Band 12 and Band 5 as release independent feature, if necessary. | RAN#96 | Perf. Part |
| 38.307 | Define high-power UE operation for fixed-wireless/vehicle-mounted use cases in Band n71 as release independent feature, if necessary. | RAN#96 | Perf. Part |

Companies are invited to provide comments and responses in the following table.

| Company | Comments |
| --- | --- |
| XXX |  |

Summary

Moderator summarizes discussion status for this round, list all the identified open issues and tentative agreements or candidate options and suggestion for next round.

|  | Status summary |
| --- | --- |
| Sub-topic #1-1 General part | 7 companies commented. The major issue is whether power class 1 UE for n71 should be included in WI or not. One company proposed to remove n71 from WI scope. Three companies including two operators insisted keeping n71 power class 1. Two companies thought that the filter issue and its impact on requirements like MPR can be further discussed in WI. No comments on the LTE band B5 and B12. One operator requested to add additional band, i.e., NR band n5 and n13, LTE band B13 into the WID, which seems not be included in SI. The other question is from which release the requirement should be applied. Tentative agreements: It is agreeable to include power class 1 on LTE band B12 and B5 in WID. Candidate options: - Include power class 1 for n71 into WI - Further discuss the impact of small duplex offset on the RF requirements. Recommendations for intermediate round: Further discuss the following issues: - Whether the power class 1 for n71 can be included in WID on the condition that the issue of small duplex will be addressed and its impact on RF requirements will be investigated in the WI. - Whether NR band n5 and n13 and LTE band B13 will be included in WI. - Provide the answer to the question on release independent and discuss from which release the requirements will be applied or decide whether the release independency will be decided in WI. |
| Sub-topic #1-2 Objectives | 5 companies made comment. Some issues were raised and companies seemed have different views. Further discussion is needed. Tentative agreements: None. Candidate options: None. Recommendations for intermediate round: Further discuss and answer the following questions related to bullet 3 - Whether the general issues including MPR, ACLR (which seems band agnostic) should be finalized before the band specific requirements are specified. - Option 1: yes, general requirements need be completed first - Option 2: no, RAN4 will include HPUE requirements for a band once all the band specific requirements are agreed. There is no need to wait for other bands. - How to introduce the power class 1 requirements for n71 - Option 1: for n71, the existing PC1 requirements (MPR, ACLR, etc.) for band n14 should be leveraged where possible. - Option 2: For band n71 and PC1, MPR needs to be re-assessed for BW > 10MHz due to larger SU of NR and for the ACLR requirement based on the coexistence studies. - Whether the higher SU dedicated for n71 will be introduced |
| Sub-topic #1-3 Specs & timeline | No comments. The table can be checked in the final round when approving the WID |

Intermediate round

Comments & responses

In this round, the following issues need be further discussed and addressed.

Sub-topic 1-1: Any question or comment on the justification or any other general comment?

Further discuss the following issues:

- Q1: Whether the power class 1 for n71 can be included in WID on the condition that the issue of small duplex will be addressed and its impact on RF requirements will be investigated in the WI.

- Q2: Whether NR band n5 and n13 and LTE band B13 will be included in WI.

- Q3: From which release the requirements will be applied, or whether the release independency should be decided in WI.

Companies are invited to provide comments and responses in the following table.

| Company | Comments |
| --- | --- |
| T-Mobile USA | Q1: Technical discussion belong in RAN4. RAN4 concluded that PC1 for n71 was feasible, so it should be included in the WI. If we don’t follow the conclusions of the SI TR why do we even do Study Items? Q2: We are OK with adding n5, n13 and B13 to the WI as long as n71 is included, also. Q3: Power Class 1 is already release independent from Rel-15 in 38.307. |
| Qualcomm | Q1: We think it is essential to understand the performance and feasibility of the filter for Band n71 because of the small duplex. In fact, we don’t see how the work can proceed without such understanding. For Band 5 and Band 12, at least some filter feedback was provided in the study item but nothing was provided for Band n71. We suspect that performance will be significantly degraded. Perhaps one possible WF is to generate two work items – one for B5 and B12 and a second one for n71. This second WI could have a “study phase” first before the requirements definition phase. Q2: We have the same concern regarding B13/n13 as we do n71 in that the band is challenging already for PC3 (especially NS_07 and self desense) and we would need to understand the filter performance and feasbility for PC1. Q3: Release independence can be decided in the WI when we learn what the requirements will be. |
| Verizon | We agree with T-Mobile For Qualcomm concern on Q2, the regulation limits for band 13 is up to 3 Watts. There should be no technical restriction to the PC1. We prefer to include this band into the WI now and update the band requirement later. |
| LGE | A1: We can compromise to add n71 for PC1, and still need to study the desense problem by cross band isolation issue. A2: Same as above answer of Q1. So we can added NR Band n5, n13 and LTE Band 13. But need to study the desense problem. A3: This is up to RAN4 decision in WI phase. |
| ZTE | Q1: we think n71 can be included in the WI with a study phase, similar view as Qualcomm Q2: NR band n5 would be straightforward to be included in the WI. For n13, besides the technical concerns, since this WI proposal does not act as a “HPUE basket”, and if it is not for the same use, i.e., fixed-wireless/vehicule-mounted use cases, then it may not be included in the WI. |
| Nokia | Q1: Can be included Q2: Can be included but n13 maynot be feasible due to A-MPR. Anyways as bands can be completed independently this is ok. Q3: REL15 |
| Skyworks | Q1: we do not understand why n71 is seen as particular case here, the main RAN4 work is to study and derive REFSENS for any of the bands for this new power class. FWA form factor will enable to use larger duplexers (and different technologies are anyhow needed for any of those bands (LTE or NR) to handle the power) and mitigate the issue. Also note that PC1 PA linearity is better than PC3 or PC2 resulting in lower noise levels. All these aspects can be worked within the WI in RAN4 and derive MSD values that would only apply to the worst case allocation knowing that many allocations will not result in MSD even for the maximum 20MHz UL BW. Q2: The additional work needed to cover NR when LTE is already covered is small enough that it would be inefficient to push this to another WI or release. So we are fine to add those as long as n71 is treated the same Q3: We can decide this depending on the RAN4 work. For example new MPR may apply for NR PC1 for BW>10MHz. |

Sub-topic 1-2: Comments and responses on proposed objectives

Further discuss and answer the following questions related to bullet 3

- Whether the general issues including MPR, ACLR (which seems band agnostic) should be finalized before the band specific requirements are specified.

  - Option 1: yes, general requirements need be completed first

  - Option 2: no, RAN4 will include HPUE requirements for a band once all the band specific requirements are agreed. There is no need to wait for other bands.

Companies are invited to provide comments and responses in the following table.

| Company | Comments |
| --- | --- |
| T-Mobile USA | Option 1: RAN4 should complete the general requirements first. The sentence in the WID “The corresponding HPUE requirements for each band can be included in the RAN4 specifications independently when the work on this band is complete, i.e. no need to wait for the completion of other bands.” means that not all of the bands have to be completed at the same time. But the general requirements need to be completed first. For instance, currently PC1 MPR only applies to n14 in 38.101-1 RAN4 needs to decide if this MPR can be a general requirement, or if a different general MPR table is needed |
| Verizon | Option 1 |
| LGE | Option 1 |
| ZTE | Option 1 |
| Nokia | Option 1 for NR if something is missing, LTE has general requirements already. |
| Skyworks | Option 1: ACLR first then MPR. Especially for NR. Note that PC1 MPR is currently limited to n14 because n14 is 5 and 10MHz only and the larger NR SU for 10MHz only reduces marginaly the gurad band. This is not the case for 15MHz (79 vs 75RB) or 20MHz (106 vs 100RB) ao MPR for edge allocations should be verified for those. |

- How to introduce the power class 1 requirements for n71

  - Option 1: for n71, the existing PC1 requirements (MPR, ACLR, etc.) for band n14 should be leveraged where possible.

  - Option 2: For band n71 and PC1, MPR needs to be re-assessed for BW > 10MHz due to larger SU of NR and for the ACLR requirement based on the coexistence studies.

Companies are invited to provide comments and responses in the following table.

| Company | Comments |
| --- | --- |
| T-Mobile USA | If n5 is added to the WID, then n5 will also have channel BWs > 10 MHz. We prefer: Option 3: For all of the NR bands in the WID the existing PC1 requirements (MPR, ACLR, etc.) for band n14 should be leveraged where possible. For band n71 and n5 and PC1, MPR needs to be re-assessed for BW > 10MHz due to larger SU of NR and for the ACLR requirement based on the coexistence studies. |
| Qualcomm | We think that existing PC1 requirements should not be copied from Band n14. The B14/n14 requirements were derived under the assumptions suitable for public safety application that may not necessarily apply for commercial FWA that is considered here. |
| AT&T | As the existing PC1 MPR requirements for band n14 and the MPR requirements for all power classes apply for all bands with the appropriate conditions: - Channel bandwidth ≤ 100 MHz. - Relative channel bandwidth ≤ 4 % for TDD bands and ≤ 3 % for FDD bands. Unless otherwise stated, the ∆MPR is set to zero. we think that the PC1 requirements for n14 should be able to be fully leveraged. We agree with TMUS that Option 1 should be considered in either case for the evaluation of the PC1 requirements (MPR, ACLR, etc.). |
| LGE | Both option are possible as mentioned from TM-US. RAN4 need further discuss in WI phase how to specify the n71 RF requirements for PC1. |
| ZTE | Option 3 proposed by TM-US seems more reasonable. |
| Nokia | Option 3 proposed by TM-US |
| Skyworks | We are fine with TMO-US option 3 proposal as LB (n5/n71) are up to 20MHz UL. We need to decide then if MPR is studied only up to 20MHz BW and if so that the PC1 MPR table clearly states that limitation. |

- Whether the higher SU dedicated for n71 will be introduced

Companies are invited to provide comments and responses in the following table.

| Company | Comments |
| --- | --- |
| T-Mobile USA | The SU for n71 is the same as for other bands. Our understanding is that the reason higher SU for n71 is mentioned in the TR and the WID is because the n14 only has channel BWs up to 10 MHz, but n71 also has 15 and 20 MHz bandwidths defined and 15 and 20 MHz have higher SU than 5 and 10 MHz. . |
| Qualcomm | Higher SU can be introduced but MPR and other requirements may need to be adjusted accordingly. |
| LGE | Need to study the different point between n14 and n71. |
| ZTE | As raised in the first round, we need clarification on what “higher SU” here exactly means: either TM-US understanding, or a higher spectrum utilization for the same channel bandwidth compared with other bands. |
| Skyworks | MPR is a general requirement and should cover all the BW that are candidate for PC1 bands. We suggest that NR PC1 MPR table covers 5/10/15/20MHz UL. SU for n71 is as per the general requirement so n5 and n71 needs 15/20MHz MPR verified for the larger NR SU compared to LTE. |

Summary

Moderator summarizes discussion status for this round, list all the identified open issues and tentative agreements or candidate options and suggestion for next round.

|  | Status summary |
| --- | --- |
| Sub-topic #1-1 | Most companies are OK to add n71 and additional bands into WI. 2 companies had concern on introduction of n13. Tentative agreements: - Include power class 1 UE for band [B13], [n13] and n71 in the WID with a study phase - Phase I: Investigate the feasibility of filter with small duplex - Phase II: if feasible, specify the requirements of power class 1 for band B13, n13 and n71. - Include an objective for release independence in WID and decide it in WI Recommendations for final round: Further check if there is still concern on B13 and n13 Please proponent to provide the revised WID capturing the agreement for review in the final round. |
| Sub-topic #1-2 | Tentative agreements: - General issues including MPR, ACLR (which seems band agnostic) should be finalized before the band specific requirements are specified. Recommendations for final round: Further discussion on revised WID for other issues. |

Final round

Comments & responses

In the final round companies are requested to check the following proposed agreements. And the proponent please provide the revised WID as soon as possible, then companies can review it. We are supposed to approve WID in the final round.

Sub-topic #1-1

The proposed agreements are

- Include power class 1 UE for band B12, B5 in the WID

- Include power class 1 UE for band B13, n13 and n71 in the WID with a study phase

  - Phase I: Investigate the feasibility of filter with small duplex

  - Phase II: if feasible, specify the requirements of power class 1 for band B13, n13 and n71.

- Include an objective for release independence in WID and decide it in WI

Sub-topic #1-2

The proposed agreement is

- General issues including MPR, ACLR (which seems band agnostic) should be finalized before the band specific requirements are specified.

Companies are invited to provide comments and responses in the following table.

| Company | Comments |
| --- | --- |
| XXX |  |
| Nokia | Release independence objective is already included in the Performance part of proposed WI. |
| T-Mobile USA | PC1 is already release independent to Rel-15. Phase I also needs to include a coexistence study for B13 and n13 PC1 since B13 and n13 were not included in the SI. |
| Qualcomm | In addition to filter feasibility, the PA also needs to be studied. If not, how will A-MPR and MPR be derived? Based on which PA model? For Band 13 and n13, these were not include in the preceding SI, so are there aspects that need to be studied first for these bands? For example, coexistence was studied in the SI, but not for B13/n13. |
| LGE | We support the moderator proposal in section 1.4.1. |
| Verizon | We agree a coexistence study, duplexer and others would be assessed for B13/n13, but that would be required for any FDD band. And, it's an area of concern that needs to be addressed in the WI. We don't think it should preclude inclusion of B13/n13 in the WI. |
| ZTE | Since the original WI proposal includes several bands for the same usage (Fixed wireless/Vehicular-mounted), it would be great to confirm the same use case if adding other band the WID, e.g., B13/n13. |
| Skyworks | Regarding the PA study, there are already LB PC1 PAs. In any case, the PA linearity and power capability may have to be revisited based on ACLR target. |

Summary

Moderator summarizes discussion status and provide the recommendation.

|  | Status summary |
| --- | --- |
| Sub-topic #1-X | Based on comments from companies, the moderator would like to propose Proposal#1: New WI for power class 1 UE on a number of bands is agreeable with the following objectives: - Include power class 1 UE for band B12, B5 in the WID for fixed wireless/vehicular-mounted use cases - Include power class 1 UE for band B13, n13 and n71 in the WID with study phase for fixed wireless/vehicular-mounted use cases - Phase I: - Investigate the feasibility of filter with small duplex for B13, n13 and n71 - Investigate the PA for B13, n13 and n71 in order to specify A-MPR and MPR - Coexistence study for B13 and n13 - Phase II: if feasible, specify the requirements of power class 1 for band B13, n13 and n71. - Include an objective for release independence in WID and decide it in WI Since in TS38.307 the release independence for PC1.5 has been specified, moderator suggests to remove release independence objective. |

Topic #2: LTE/NR spectrum sharing for B34/n34, B39/n39

Companies’ contributions summary

| T-doc number | Title | Sourcing company |
| --- | --- | --- |
| RP-211283 | New WID on LTE/NR spectrum sharing in Band 34/n34 | CMCC |
| RP-211284 | New WID on LTE/NR spectrum sharing in Band 39/n39 | CMCC |

Initial round

Comments & responses

In this section, we collect the comments and responses for the proposed work item. Based on the comments, we will decide how to move forward in the next step.

Sub-topic 2-1: Any question or comment on the justification or any other general comment for two WIDs?

Companies are invited to provide the general comments, including comments on justification part, whether the WI is needed, how to handle the work, in the follow table.

| Company | Comments |
| --- | --- |
| Huawei, HiSilicon | Support the WI proposals. |
| Apple | We have no concerns with the justification for these two WIs. As a relatively minor remark, since the work for introducing requirements to n34 and n39 is minimal and the request for both bands comes from the same operator, we suggest contemplating an approach of having just one WID instead of two to minimize the procedural overhead. |
| Skyworks | We also think that it is simpler to consolidate similar spectrum related work in a single WI |
| ZTE | We are supportive to the two WIDs. |

Sub-topic 2-2: Comments and responses on objectives for WI on LTE/NR spectrum sharing in B34/n34

The following objectives are proposed.

Core part:

The work item aims to specify spectrum sharing requirements for n34.

- Introduce UL 7.5kHz frequency shift for 15kHz SCS operation from Rel-15 [RAN4]

    - Study for backward compatibility

    - Study for which release UL shift can be made mandatory

Performance part:

None

Companies are invited to provide comments and responses in the following table.

| Company | Comments |
| --- | --- |
| XXX |  |
| Apple | We agree with the objectives of the WIDs. As commented earlier, since objectives are exactly the same for both bands, we suggest instantiating one WID instead of two separate WIDs. |

Sub-topic 2-3: Comments and responses on objectives for WI on LTE/NR spectrum sharing in B39/n39

The following objectives are proposed.

Core part:

The work item aims to specify spectrum sharing requirements for n39.

- Introduce UL 7.5kHz frequency shift for 15kHz SCS operation from Rel-15 [RAN4]

    - Study for backward compatibility

    - Study for which release UL shift can be made mandatory

Performance part:

None

Companies are invited to provide comments and responses in the following table.

| Company | Comments |
| --- | --- |
| Apple | We agree with the objectives of the WIDs. As commented earlier, since objectives are exactly the same for both bands, we suggest instantiating one WID instead of two separate WIDs. |

Sub-topic 2-4: Comments and responses on impacted/new specifications and target completion date

The proposed impacted specifications as well as target completion date are as follows:

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| 38.101-1 | NR; User Equipment (UE) radio transmission and reception | RAN#92e | Core Part |
| 38.104 | NR; Base Station (BS) radio transmission and reception | RAN#92e | Core Part |
| 38.307 | NR; Requirements on User Equipments (UEs) supporting a release-independent frequency band | RAN#92e | Core Part |

Companies are invited to provide comments and responses in the following table.

| Company | Comments |
| --- | --- |
| Apple | Is it possible to have the target completion in the same meeting (RAN #92e) for WID approval? Should not it be RAN#93 or RAN#94? |
| ZTE | As commented offline and similar question to Apple, the target completion date of RAN#92e, i.e., this week, seems not feasible. A new date should be indicated. |

Summary

Moderator summarizes discussion status for this round, list all the identified open issues and tentative agreements or candidate options and suggestion for next round.

|  | Status summary |
| --- | --- |
| Sub-topic #2-1 General part | 4 companies made comment. There is no objection to approve the WID. But two companies suggested to merge them into one WI. Tentative agreements: It is agreeable to introduce the requirements for LTE/NR spectrum sharing in Band 34/n34 and Band 39/n39. Candidate options: None. Recommendations for intermediate round: Further discuss - Whether to merge two draft WID into one. |
| Sub-topic #2-2 Objectives for B34/n34 | 1 company commented that one WID is preferable. Tentative agreements: None. Candidate options: None. Recommendations for intermediate round: First address the issue under Sub-topic #2-1. Moderator: there is a typo in the objective of WID for 34/n34. The band number should be 34. |
| Sub-topic #2-3 Objectives for B39/n39 | 1 company commented that one WID is preferable. Tentative agreements: None. Candidate options: None. Recommendations for intermediate round: First address the issue under Sub-topic #2-1. |
| Sub-topic #2-4 Specs & timeline | 2 companies commented the target completion date. It seems target completion date needs corrected. Tentative agreements: None. Candidate options: None. Recommendations for intermediate round: Further clarification and revision on target completion date is needed. |

Intermediate round

Comments & responses

Based on the initial round discussion, the following issue needs be discussed in the intermediate round.

Sub-topic 2-1: Any question or comment on the justification or any other general comment for two WIDs?

Further discuss

- Whether to merge two draft WID into one.

Companies are invited to provide comments and responses in the following table.

| Company | Comments |
| --- | --- |
| CMCC | We are OK to merge the two proposals into one WID. |
| ZTE | Merging into one WID would reduce routinary “paperwork”, and we would like to co-sign the WID. |
| Apple | We appreciate the flexibility of the companies and would like to thank CMCC for merging two WIs into one WID, this will reduce "paperwork" allowing us to focus on critical technical aspects. |
| Skyworks | We think it is better to merge and treat in a single thread, thanks to the proponent to accept this approach. |

Sub-topic 2-4: Comments and responses on impacted/new specifications and target completion date

- Further clarification and revision on target completion date is needed.

Companies are invited to provide comments and responses in the following table.

| Company | Comments |
| --- | --- |
| CMCC | The target completion date will be revised to RAN#94e. |
| Apple | RAN#94e is good. |

Summary

Moderator summarizes discussion status for this round, list all the identified open issues and tentative agreements or candidate options and suggestion for next round.

|  | Status summary |
| --- | --- |
| Sub-topic #2-1 and 2-4 | Tentative agreements: - Merge two proposals into one WID - Target completion date is RAN#94e. Recommendations for final round: Please proponent provide the revised WID for review in the final round. |

Final round

Comments & responses

The tentative agreements in the intermediate round are acceptable to the group. The proponent has sent out the revised WID. Please review it.

Sub-topic #2-1 and #2-4

The proposed agreement is

- It is proposed to approve the WID capturing both 34/n34 and 39/n39 with the target completion date at RAN#94e.

Companies are invited to provide comments and responses in the following table.

| Company | Comments |
| --- | --- |
| Apple | The title in the revised WID (RP-211509) content needs to be updated to add 39/n39 |
| CMCC | Thanks Apple for the careful check. Please see the updated version below. Thank you. https://www.3gpp.org/ftp/TSG_RAN/TSG_RAN/TSGR_92e/Inbox/Drafts/%5B92-e-05-Spectrum-WIs%5D/Intermediate%20round/revised%20WID%20on%20spectrum%20sharing/revised%20RP-211283_v2.doc |

Summary

Moderator summarizes discussion status and provide the recommendation.

|  | Status summary |
| --- | --- |
| Sub-topic #2-1 and 2-4 | Proposal #2: the revised WID RP-211283 is agreeable. |

Topic #3: DC of x-band LTE CA + 4 bands NR CA

Companies’ contributions summary

| T-doc number | Title | Sourcing company |
| --- | --- | --- |
| RP-211393 | New WID on DC of x bands (x=1,2,3) LTE inter-band CA (xDL/1UL) and 4 bands NR inter-band CA (4DL/1UL) | Huawei, HiSilicon |

Initial round

Comments & responses

In this section, we collect the comments and responses for the proposed work item. Based on the comments, we will decide how to move forward in the next step.

Sub-topic 3-1: Any question or comment on the justification or any other general comment?

Companies are invited to provide the general comments, including comments on justification part, whether the WI is needed, how to handle the work, in the follow table.

| Company | Comments |
| --- | --- |
| Huawei, HiSilicon | Support the WI proposal to consider the operator requested band combinations. |
| Apple | We would like to have more clarifications on a demand for a total of up to 7 aggregated bands (3 LTE bands + 4 NR bands) DC combination as currently only up to 6 aggregated bands combinations have been proposed. Should we consider x = 1, 2 only? |
| Skyworks | The example band combinations all include an FR2 band. is this 7band combination targeting FR1+FR2 combination only or 7 FR1 band only is also targeted? |

Sub-topic 3-2: Comments and responses on objectives

The following objectives are proposed.

Core part:

- . Specify the DC configurations listed in the tables below,  including at least

  - Applicable frequencies if necessary

  - Applicable bandwidths and bandwidth sets if necessary

- Specify the band-combination specific RF requirements for these DC configurations, i.e.

  - Analyse combinations that have self-desensitization due to following reasons:

    - TX Harmonic and/or intermodulation overlap of receive band

    - TX signal overlap of receiver harmonic frequency

    - TX frequency being in close proximity of one of the receive bands

    - Any other identified reasons such that insufficient cross band isolation, harmonic mixing

  - For the combination where self-desensitization exists, specify at least needed

    - ∆TIB, c and ∆RIB, c

    - Reference sensitivity exceptions including MSD test cases

of all REL-17 DC including EN-DC and NE-DC of LTE CA for up to 3 different bands DL with 1 band UL + NR CA for 4 different bands DL with 1 band UL that fall into the category is defined by the WI title.

An overview table of these EN-DC and NE-DC configurations are provided in the EXCEL file zipped together with WORD file WID.

Unless stated otherwise, the release independent for EN-DC and NE-DC configurations are from Rel-15.

Performance part:

This Perf. Part WI has to standardize the Perf. Part requirements:

- Required changes to be added to release independence TS 38.307.

of all REL-17 DC combinations including EN-DC and NE-DC that fall into the category is defined by the WI title.

The band combinations are included:

Refer to attached excel file of RP-211393.

Companies are invited to provide comments and responses in the following table.

| Company | Comments |
| --- | --- |
| Huawei, HiSilicon | Support the objectives, and they are similar to other basket WIs in terms of band combination specific requirements. |
| Apple | We would like to have further clarifications on whether the NR CA part is always an FR1+FR2 combination or can be a CA within FR1? |
| CHTTL | There are few proposed combinations with uplink configuration “DC_n3A”, maybe they are typos? |

Sub-topic 3-3: Comments and responses on impacted/new specifications and target completion date

The proposed impacted specifications as well as target completion date are as follows:

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Remarks |
| Internal TR | 37.717-11-41 | DC of x bands (x=1,2,3) LTE inter-band CA (xDL/1UL) and 4 bands NR inter-band CA (4DL/1UL) | TSG#94 | TSG#95 | WANG, Zhou, Huawei, research.wangzhou@huawei.com |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| 38.101-3 | Add new DC band combinations and related RF core requirements | TSG#95 | Core part |
| 38.307 | Release independent manner will be applied to all new DC band combinations according to each DC band combination | TSG#95 | Perf. part |

Companies are invited to provide comments and responses in the following table.

| Company | Comments |
| --- | --- |
| XXX |  |

Summary

Moderator summarizes discussion status for this round, list all the identified open issues and tentative agreements or candidate options and suggestion for next round.

|  | Status summary |
| --- | --- |
| Sub-topic #3-1 General part | 3 companies commented. There are questions for clarifications on up to 7 aggregated bands. Further discussion is needed. Tentative agreements: None. Candidate options: None. Recommendations for intermediate round: Further clarifications on the following questions on the scope are needed: - Should we consider x=1, 2 only? - Is this 7band combination targeting FR1+FR2 combination only or 7 FR1 band only is also targeted? (whether the NR CA part is always an FR1+FR2 combination or can be a CA within FR1) |
| Sub-topic #3-2 Objectives | 3 companies commented. Except for the similar questions as above for sub-topic #3-1, the question on uplink configuration”DC_3A” was raised. Tentative agreements: None. Candidate options: None. Recommendations for intermediate round: Further discuss the following questions - Are the uplink configurations “DC_n3A” for proposed band combinations typo? |
| Sub-topic #3-3 Specs & timeline | No comment was received. Further review can be done in the final round. |

Intermediate round

Comments & responses

Based on the initial round discussion, the following issues/questions need be addressed/answered.

Sub-topic 3-1: Any question or comment on the justification or any other general comment?

Further clarifications on the following questions on the scope are needed:

- Should we consider x=1, 2 only?

- Is this 7band combination targeting FR1+FR2 combination only or 7 FR1 band only is also targeted? (whether the NR CA part is always an FR1+FR2 combination or can be a CA within FR1)

Companies are invited to provide comments and responses in the following table.

| Company | Comments |
| --- | --- |
| MTK | Sorry for late comment. We have similar question for clarification as raised in the initial round from some companies. We would like to understand the demand of 7 bands deployment and also the FR allocation. |
| Apple | Our preference is to keep the total number of (LTE+FR1) bands to be within 6 bands. That means for a 7-band combination, one of the NR band must be in FR2. |
| Skyworks | We would be OK if this applies only to FR1+FR2 cases, but first we need clarification from the proponent on the cases that are targeted. |
| Huawei, HiSilicon | X=1, 2, 3 is the max LTE bands. Currently the request from operator is only for 2 LTE bands. We have no strong view whether X should be 1, 2 only according to the available request. Three bands just for possible future extension purpose. Regarding whether NR CA is limited to FR1+FR2, we think that the basket WI should be generic, i.e. it depends on the requests from operator. In the attached band combo table, it just lists inputs from one operator. But we think that most likely the possible scenario for NR CA includes FR2 band. If needed, we can add a note to clarify the scenario. |

Sub-topic 3-2: Comments and responses on objectives

Further discuss the following questions

- Are the uplink configurations “DC_n3A” for proposed band combinations typo?

Companies are invited to provide comments and responses in the following table.

| Company | Comments |
| --- | --- |
| Huawei, HiSilicon | ”DC_3A” is a typo, it should be "DC_n3A_1A". |

Summary

Moderator summarizes discussion status for this round, list all the identified open issues and tentative agreements or candidate options and suggestion for next round.

|  | Status summary |
| --- | --- |
| Sub-topic #3-1 | Tentative agreements: The proposed WID is agreeable with the following modifications - Limit the total number of bands to 6, i.e., X =1,2 - Add the note to clarify the WID applies to NR CA including FR2 bands Recommendations for final round: Please proponent provide the revised WID for review in the final round. |
| Sub-topic #3-2 | Conclusion: ”DC_3A” is a typo, it should be "DC_n3A_1A". |

Final round

Comments & responses

It seems that the tentative agreements in the intermediate round are agreeable. Based on them the proponent should provide the revised WID as early as possible for review. We are supposed to approve the WID in final round.

Sub-topic #3-1 and #3-2

It is propose to approve the revised WID of RP-211393, where

- Limit the total number of bands to 6, i.e., X =1,2

- Add the note to clarify the WID applies to NR CA including FR2 bands

Companies are invited to provide comments and responses in the following table.

| Company | Comments |
| --- | --- |
| XXX |  |

Summary

Moderator summarizes discussion status and provide the recommendation.

|  | Status summary |
| --- | --- |
| Sub-topic #3-1 and 3-2 | Proposal #3: The revised WD of RP-211393 is agreeable, with the following changes - Limit the total number of bands to 6, i.e., X =1,2 - Add the note to clarify the WID applies to NR CA including FR2 bands https://www.3gpp.org/ftp/tsg_ran/TSG_RAN/TSGR_92e/Inbox/Drafts/%5B92-e-05-Spectrum-WIs%5D/Final%20round/Revised%20WID/REV%20RP-211393%20Band%20combinations%20for%20DC_LTE%20xDL_2UL(x%3D1%2C2)%2BNR%204DL_1UL.zip |

Topic #4: 6GHz unlicensed band in other countries/regions

Companies’ contributions summary

| T-doc number | Title | Sourcing company |
| --- | --- | --- |
| RP-211445 | Motivation for Introduction of the 6GHz unlicensed band in other countries/regions | Apple Inc. |
| RP-211446 | New WID for Introduction of the 6GHz unlicensed band in other countries/regions | Apple Inc. |

More issue is added according to RAN chair guidance, which is related to the flagged SR for European lower 6GHz bands. The information is provided as below.

| Tdoc | Title | Reason for discussion |
| --- | --- | --- |
| RP-211320 | SR for introduction of lower 6GHz NR unlicensed operation for Europe | The SR contains an action to RAN: “REQUESTED ACTION: RAN plenary shall decide if unlicensed operation in the frequency range 5945 MHz to 6425 MHz in Europe shall be based on available ECC Decision (20)01 or RAN4 shall consider other aspects as well. Further, RAN plenary is requested to decide which of the two listed options RAN4 shall follow to introduce unlicensed operation in the frequency range 5945 MHz to 6425 MHz in Europe.” Also note that the TR 38.849 is not ready for official presentation to RAN for information, so RP-211321 is withdrawn. |

Initial round

Comments & responses

In this section, we collect the comments and responses for the proposed work item. Based on the comments, we will decide how to move forward in the next step.

Sub-topic 4-1: Can we agree to introduce the support of the 6GHz band in countries/regions that are not covered by the scope of the existing WIs?

Proposals in RP-211445:

“…there are a number that either have recently announced support for the 6GHz band or are in the process of public consultations to open the 6GHz band for the license-exempt operation…. As can be seen, quite many countries have somewhat different regulatory requirements for the maximum mean EIRP density and out of band emissions. And since those parameters have direct impact on e.g. A-MPR values, 3GPP RAN WG4 will have to perform analysis on how much A-MPR each country/region will have and how it can be supported with existing or new NS values. Furthermore, since other Administrations have been working on the 6GHz band, even more countries will start publishing the corresponding regulatory requirements. Based on that we anticipate that non-trivial technical discussions will be needed to understand how to support 6GHz in all countries/regions.”

Proposal:	Introduce support for the 6GHz band in countries/regions that are not covered by the scope of the existing WIs.

Companies are invited to provide the general comments on the above proposal.

| Company | Comments |
| --- | --- |
| Qualcomm | We are generally supportive of the proposal; however, we need to agree on an approach to introduce the specialized requirements for each country. It is not practical for 3GPP to define new emission and A-MPR requirements, for example, on a country-specific basis if each country will have a slightly different regulatory rules. |
| LGE | LGE support to study the regulatory requirements for 6GHz band in each countries/regions that are not covered in the existing WIs. The current specification only complied with U.S. requirements. EU regulation is ongoing in the existing WI, So need to new WID to support of the 6GHz band in other countries/regions. |
| Deutsche Telekom | We are a bit surprised that we talk about “countries” here … (I know we a talking here about a particular regulation in a particular country) Can Apple please explain why there is a need to talk about “countries” ? Could we find a more 3GPP appropriate language, please. |
| Intel | Overall, we are supportive to extend the work on 6GHz band definition to cover different regions. The work shall be conducted for the regions, where the regulatory work is finalized. Suggest adding an explicit list of regions covered by the work item. |
| Apple | @Qualcomm: Yes, we do have the same view that introducing specialized requirements for every new country is not a good approach. Our current understanding is that some countries anyway “share” similar regulatory parameters for some modes. Referring to Region 2 as an example, Canada and US share same parameters for the SP and LPI operation, but Canada has also VLP. Brazilian LPI is same as US/Canada LPI, but the VLP mode in Brazil is a different when compared to the Canadian VLP. Peru and Chile have only LPI, which is identical to other Region 2 countries. Referring to Region 3, South Korea VLP mode is identical to EU/CEPT VLP, but LPI has slightly different parameters. @DT: The wording is not perfect and can be changed later. In fact, in the objective part we make it more explicit be referring to TR 37.890. |
| Skyworks | We support this WI as 3GPP cannot ignore some regulation in some countries/regions especially for an unlicensed band. We have done the work for n46 and we should now work on the 6GHz band. So far we have used different NS for different regions and UE types/usages (ie indoor/outdoor or LPI/VLP). We agree that the goal should be to minimize the number of NS used and as far as possible reuse existing band definition. RAN4 should first try to consolidate the different requirements into a manageable set of cases. Note that the available NS mechanism will allow NR-U to be more agile than WiFi in supporting the different regulations as they develop. |

Sub-topic 4-2: Any question or comment on justification or any other general comment on the proposed WI RP-211446?

Companies are invited to provide the general comments, including comments on justification part, how to handle the work, in the follow table.

| Company | Comments |
| --- | --- |
| Qualcomm | We are generally supportive of the proposal; however, we would like to understand whether new NS values will be perpetually added to Band n96 as countries make the spectrum available with lightly different rules than already covered by existing NS’s. In this case, legacy devices may not be able to support the new NS. While there has been discussion in the past about new NS values and incompatibility with legacy devices, those previous discussions were somewhat limited to a single new NS or relative few new NS’s. In the case of Band n96, it seems that a much larger number of NS’s could be added over a much longer period of time. |
| Charter Communications Inc. | With regards to this WID, we understand that we need to address regulatory requirements in other regions for 6 GHz. We also understand that such requirements can possibly be handled with existing NS values or new NS values. We have concerns that we could potentially run out of NS values to address other countries and regions while we might still need ns values for the US in n96 to address vlp mode. Therefore, we suggest to add an additional objective to read, “Depending on the regulatory requirements and if these requirements cannot be handled by existing NS values or whether new NS values are not possible then an introduction to a new band will be needed.” This objective will not exclude the possibility of introducing a new band in 6 GHz if handling through NS values is not possible. |
| Huawei, HiSilicon | We also think that to address regulatory requirements in different regions may end up with large number of NS values. And it is expected that the NS related discussion according to update of regional regulatory requirements could last a long period, in that sense, also considering the backward compatibility issues, new NR band(s) could be considered for other regions with different regulatory requirements. |
| CMCC | If the motivation is to study an approach on how to support 6GHz in countries/regions, then this should not belong to a spectrum WI in our view. If there is other requests from countries or regions to support 6GHz that not covered by the existing Wis, they can have their own proposal. This is the business as usual. Don’t understand why we need this WI. |
| LGE | Generally, LGE prefer to define new NS for 6GHz band (n96). If there is some issue to define new NS for the n96, then RAN4 can send LS to RAN2 to increase the number of NS for n96. It is up to RAN4 study outcome. We prefer this WI is to start in Rel-17 and RAN can make additional WI according to new regulatory requirements in countries in Rel-18. |
| Telecom Italia | We agree with CMCC, this is not a spectrum activity, and at the moment there is no room in RAN4 for new non spectrum activities. If specific regulations are defined for countries/Regions, a spectrum WI could be approved, with the assumption that a new band is defined |
| Deutsche Telekom | We agree with CMCC and Telecom Italia, but our comment above also applies here … |
| BT | What do we mean by “other countries/regions”? We believe that this wording is far too vague for a WI. If it is considered that there is sufficient support for this WI and it is decided that it should proceed then it will need to clearly specify those countries / regions to which it applies |
| Intel | 1) It is not very clear whether the WI aims to reuse n96 or additional bands can be considered depending on the specific regulatory status. We think that the WID description can be kept generic and both options can be considered in the WI stage. 2) The NS questions raised by companies seem valid and detailed solutions to address this problem can be discussed in the WG-level. |
| Apple | @Qualcomm: We agree with the general observation that since we cannot predict the outcome of the regulatory decision in a particular country, we indeed might face a situation when new NS values will be “perpetually” added. However, as we also exemplified in the previous point, the local Administrations usually try to harmonize their requirements with other countries in the same region and thus we do not anticipate a plethora of new NS values. One of the WI goals will be to take a consistent look at all potential NS values and see whether we can minimize them. @Charter: We will be Ok with adding the following statement that “Depending on the regulatory requirements and if these requirements cannot be handled by existing NS values or whether new NS values are not possible then an introduction to a new band will be needed.” @CMCC, TIM, DT: This is “business-as-usual” spectrum related WI. As agreed at the September 2020 RAN meeting, once we have the regulatory requirements released by the corresponding countries (refer to TR 37.890), 3GPP can proceed with the normative work on how these requirements can be supported. @BT, DT: The wording in “other countries/regions” is indeed somewhat vague, we are sure that 3GPP can end up with a better name for the WI. Referring to the objective part, there is an explicit reference to TR 37.890 so we are limited by the countries captured there and by the target completion date of this WI. @Intel: Similar to comment from Charter, we can clarify that “the WID description can be kept generic and both options can be considered in the WI stage.” |
| CHTTL | share the view as CMCC. |
| Skyworks | When compared to n46 where we use 4 NS, for the 6GHz cases, we have to treat indoor/outdoor or VLP/LPI cases on top. Part of the work will be to group the different cases in a minimum set of NS, we could develop criterias like how much difference in back-off justifies a different NS (this needs to be discussed amongst RAN4 experts within the WI). Regarding the WI being justified, so far for unlicensed band we have carried the work as soon as regulation was available. This is the case already for n46 and then we did n96 in the scope of NR-U without specific request as the regulation was then available. To avoid the confusion on the term countries/regions the WI could directly refer to the regulatory document. |
| Orange | Same view as CMCC and others, Orange is not supportive of this WI. Regarding EU, a WI is already approved to cover EU specific requirements. By the way, we are supportive of creating a new band corresponding to the targeted European band plan for unlicensed 6 GHz (5925-6425 MHz) once regulatory decisions are finalized. We do not think re-using n96 is the best way forward. |

Sub-topic 4-2a: Reuse the existing n96 or define a new band for European frequency range of 5945 to 6425 MHz

In the SR RP-211320 there is one outstanding issue for which the proponent want to further discuss in RAN plenary.

- How to introduce the frequency range 5945 to 6425 MHz for unlicensed operation in Europe to the specification by RAN4. As agreed in WF R4-2103229 unlicensed operation in the range 5945-6425 MHz can be introduced by:

  - Option 1: Re-using already defined band n96

    - FFS if additional notes and/or clarifications are needed. Regional specific requirements to be included in relevant specifications.

  - Option 2: Defining a new band n[xx]

    - On top of specific requirements provided by ECC, the new band shall reuse requirements already defined for n96, where possible.

Companies are invited to provide comments on which option should be agreed.

| Company | Comments |
| --- | --- |
| Telecom Italia | We cannot accept re-use of band n96 in Europe. A new band must be defined to take into account the European regulation |
| Charter Communications Inc. | We can compromise with option 1 if Skyworks comment above prevails, “When compared to n46 where we use 4 NS, for the 6GHz cases, we have to treat indoor/outdoor or VLP/LPI cases on top.” If this cannot be assured then a new band can be defined. In general, we should attempt not defining a new band unless is absolutely necessary. We would had prefer the sub-bullet in option 2 to reflect a statement like this. |
| Nokia | Support option 1. From ecosystem point of view, RAN4 shall always strive to use existing bands unless it is not possible. n96 with NS values will take European regulation into account in exactly the same way as a new band. |

Sub-topic 4-3: Comments and responses on objectives of the proposed work item in RP-211446

The following objectives are proposed.

Core part:

The objectives of the core part work item are:

-	Analyse regulatory requirements for new countries/regions that have adopted the 6GHz band for the license-exempt operation:

NOTE: The list of new countries/regions with the corresponding parameters can be taken from TR 37.890;

-	Depending on the regulatory requirements, determine whether they can be handled by existing NS values or whether new NS values are needed.

-	Define or update (if needed) system parameters such as channel bandwidths and channel arrangements.

-	Define or update (if needed) transmitter and receiver characteristics requirements for the UE.

-	Define or update (if needed) transmitter and receiver characteristics requirements for the BS.

Performance part:

The objective of the performance part work item is to define or update (if needed) conformance requirements for BS testing.

Changes are to be made in a release-independent manner.

Companies are invited to provide comments and responses in the following table.

| Company | Comments |
| --- | --- |
| XXX |  |

Sub-topic 4-4: Comments and responses on impacted/new specifications and target completion date

The proposed impacted specifications as well as target completion date are as follows:

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Remarks |
| Internal TR | TR 38.xxx | Introduction of the 6GHz unlicensed band in other countries/regions |  | RAN#95 | Core part |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| 38.101-1 | NR; UE Radio transmission and reception | RAN#95 | Core UE part |
| 38.104 | NR; BS Radio transmission and reception | RAN#95 | Core BS part |
| 37.104 | E-UTRA, UTRA and GSM/EDGE; Multi-Standard Radio (MSR) Base Station (BS) radio transmission and reception | RAN#95 | Core BS part |
| 37.105 | Active Antenna System (AAS) Base Station (BS) transmission and reception | RAN#95 | Core BS part |
| 38.141-1 | NR; Base Station (BS) conformance testing Part 1: Conducted conformance testing | RAN#96 | Perf. BS part |
| 38.141-2 | NR; Base Station (BS) conformance testing Part 2: Radiated conformance testing | RAN#96 | Perf. BS part |
| 37.141 | E-UTRA, UTRA and GSM/EDGE; Multi-Standard Radio (MSR) Base Station (BS) conformance testing | RAN#96 | Perf. BS part |
| 37.145-1 | Active Antenna System (AAS) Base Station (BS) conformance testing; Part 1: conducted conformance testing | RAN#96 | Perf. BS part |
| 37.145-2 | Active Antenna System (AAS) Base Station (BS) conformance testing; Part 2: radiated conformance testing | RAN#96 | Perf. BS part |

Companies are invited to provide comments and responses in the following table.

| Company | Comments |
| --- | --- |
| XXX |  |

Summary

Moderator summarizes discussion status for this round, list all the identified open issues and tentative agreements or candidate options and suggestion for next round.

|  | Status summary |
| --- | --- |
| Sub-topic #4-1 Proposal in RP-211445 | 6 companies commented. 5 companies are supportive to study and/or design a solution to meet the regulation requirements in different regions. One company commented that here we should consider a particular regulation rather a “country”. It seems unclear how to introduce the support of more 6GHz spectrum, which needs further discussion. Tentative agreements: None. Candidate options: None. Recommendations for intermediate round: Further discuss and answer the questions: - Whether it is agreeable to introduce the support of 6GHz unlicensed band to fulfill the regional regulations which are not covered by the scope of the existing WIs. - If the answer to above question is yes, then further discuss how to introduce the support of 6GHz unlicensed spectrum to fulfil the regional regulations which are not covered by the scope of the existing WIs. |
| Sub-topic #4-2 General part | 13 companies commented. Companies’ comments are on different level in this sub-topic. 5 operators questioned whether the proposed WID belongs to spectrum related topic and questioned the need of the work. 2 operators thought the wording “country” is vague. 6 companies discussed the detailed approach of using NS based on n96 to fulfil the regional regulation requirements. At least 2 companies expressed the view that the new band needs be specified to fulfil the regional regulation requirements. From moderator perspective, we need a consensus on whether such WI is agreeable or not before discussing the detailed general solution. Tentative agreements: None. Candidate options: None. Recommendations for intermediate round: Further discuss and answer the following questions: - How to introduce the support of 6GHz unlicensed spectrum to fulfil the regional regulations which are not covered by the scope of the existing WIs, considering the following options - Option 1: Depending on the regulatory requirements, determine whether they can be handled by existing NS values or whether new NS values are needed (based on existing NR band n96) - It will need to clearly specify those countries / regions to which it applies - Option 1a: - Depending on the regulatory requirements, determine whether they can be handled by existing NS values or whether new NS values are needed (based on existing NR band n96). - Depending on the regulatory requirements and if these requirements cannot be handled by existing NS values or whether new NS values are not possible, then an introduction to a new band will be needed. - It will need to clearly specify those countries / regions to which it applies - Option 2: New NR band(s) could be considered for other regions with different regulatory requirements. - Whether the proposed WI RP=211446 is spectrum related or non-spectrum related |
| Sub-topic #4-2a Objectives | 3 companies commented. 2 companies supported option 1. 1 company disagreed to reuse n96. But this sub-topic was added at the late stage of initial round. More comments are welcome. Tentative agreements: None. Candidate options: None. Recommendations for intermediate round: Further discuss this topics in the next round. According to comments from Apple, we split the discussion into two bullets. Companies can comment further in the intermediate round. |
| Sub-topic #4-3 Objectives | No comment |
| Sub-topic #4-4 Specs & timeline | No comment. |

Intermediate round

Comments & responses

Based on the comments in the initial round, we can merge the discussion. In the intermediate round, we can focus on the following questions. In moderator view, the basic issue under sub-topic 4-2 and sub-topic 4-2a are the same. So we can merge them for the discussion in the intermediate round.

According to feedback from Apple we list two main issues for sub-topic 4-2a, and copied the companies’ previous comments. Please continue discussion in the intermediate round.

Sub-topic 4-1, sub-topic 4-2

Further discuss

- How to introduce the support of 6GHz unlicensed spectrum to fulfil the regional regulations which are not covered by the scope of the existing WIs, considering the following options

  - Option 1: Depending on the regulatory requirements, determine whether they can be handled by existing NS values or whether new NS values are needed (based on existing NR band n96)

    - It will need to clearly specify those countries / regions to which it applies

  - Option 1a:

    - Depending on the regulatory requirements, determine whether they can be handled by existing NS values or whether new NS values are needed (based on existing NR band n96).

    - Depending on the regulatory requirements and if these requirements cannot be handled by existing NS values or whether new NS values are not possible, then an introduction to a new band will be needed.

    - It will need to clearly specify those countries / regions to which it applies

  - Option 2: New NR band(s) could be considered for other regions with different regulatory requirements.

Companies are invited to provide comments and responses in the following table.

| Company | Comments |
| --- | --- |
| Qualcomm | Option 1 should be the baseline. If it is found to be unworkable, then other options can be considered. However, we disagree that “It will need to clearly specify those countries / regions to which it applies.” In general, 3GPP bands do not list the countries to which they apply (there are some exceptions). The list could be very long and some countries will almost certainly be overlooked. And it will be an endless maintenance exercise to add countries. Instead, a general statement could be added something like “this band is applicable in countries for which the regional regulations are met”. |
| Facebook | Option 1. We also share the QC view about the sub-bullet of Option 1. The countries regulations in the 6GHz band are evolving so to clearly specify each country is not practical approach. |
| CMCC | We support option 2. New bands can of course be considered if the regulatory requirements are different. Option1a also mentions that an introduction to a new band is needed in some cases. |
| LGE | Prefer option 1. In LTE-U, RAN4 defined just 1 LTE band as band 46 in TS36.101. In BS specification, RAN4 defined 4 sub-band such as Band 46a/b/c/d. This approach can be used in NR. |
| Orange | We support Option 2. A new band should be created when there is a different band plan with different regulatory requirements. As such, a new band should be created in Europe, instead of re-using n96. |
| Intel | Prefer option 1a. This seem to be a compromise approach between Options 1/2 and opens the door for further discussion in RAN4. For the specific countries/regions – it would be good to clarify in WID at least a set of regions which is in the scope. Agree that we may not need to capture in the specs the list of countries/regions. |
| BT | We support Option 2 |
| Apple | Echoing detailed explanations from Qualcomm, Option 1 is the general RAN4 baseline because we strive for re-using existing bands instead of adding news ones. Having said that, other options are of course not excluded, because it is also a common practice in RAN4 to add new bands when other options are not feasible. As a proponent of the WI proposal, we added a new a objective into the WI description that explicitly says that depending on whether new NS values are needed and if so, how many new NS values we need, we can consider adding a new band. @Orange: Referring to the list of candidate countries mentioned in the WI justification part, Canada, Brazil, Peru, Chilie, South Korea, please note that their band plan is identical to the US band and even the regulatory parameters are close, if not identical in many cases, to US. This WI is not about the EU/CEPT 6GHz band. |
| CHTTL | We think this issue should be decided case-by-case, agree with CMCC that new bands can of course be considered. |
| Telecom Italia | Option 2 |
| Skyworks | Option 2 is totally unpractical on the UE side. WiFi6E has no notion of bands and is targeting all these regulation with the same hardware in the UE. Also n46 is a single band covering multiple regulatory requirement via NS. We certainly support the fact that high regulatory requirement is studied to determine if it can be covered by an existing NS. We support option 1. For which cases should be considered the WI can directly point at the regulatory requirement document like it has been done for FCC and ECC cases. If for obvious reason it a new band is needed, this is not precluded but need to be highly justified. Note that n78 can be supported with n77 HW or n38 with n41…this is commant practice in TDD bands and in the meantime some countries had their spectrum extended from n78 to n77 and enjoyed an early support by UEs due to the larger ecosystem. |
| Huawei, HiSilicon | We think that option 2 would be easier to address the NS signaling and legacy UE issues discussed in round 1. And the study should be based on specific requests for curtained regions. |
| Charter Communications Inc. | I agree with Intel and we prefer option 1a. In 3GPP, we should avoid adding more bands if it can be avoided but in the case of 6 GHz, there is a potential to need, in some cases for certain regions, new bands, especially if we get into a situation where NS values are running out. In summary the wid objective should not exclude the potential of having to add a new band. I also agree with Intel that 1a seems to be a compromise between option 1 and 2 |

Sub-topic 4-2

Discuss and answer the following questions:

- Whether the proposed WI RP-211446 is spectrum related or non-spectrum related

Companies are invited to provide comments and responses in the following table.

| Company | Comments |
| --- | --- |
| Telecom Italia | We cannot accept re-use of band n96 in Europe. A new band must be defined to take into account the European regulation |
| Charter Communications Inc. | We can compromise with option 1 if Skyworks comment above prevails, “When compared to n46 where we use 4 NS, for the 6GHz cases, we have to treat indoor/outdoor or VLP/LPI cases on top.” If this cannot be assured then a new band can be defined. In general, we should attempt not defining a new band unless is absolutely necessary. We would had prefer the sub-bullet in option 2 to reflect a statement like this. |
| Nokia | Support option 1. From ecosystem point of view, RAN4 shall always strive to use existing bands unless it is not possible. n96 with NS values will take European regulation into account in exactly the same way as a new band. |
| Qualcomm | The above comments in this table don’t seem to correspond to the question of whether RP-211446 is spectrum or non-spectrum. They seem to belong to sub-topic 4-2a. For sub-topic 4-2, we believe RP-211446 is spectrum related. |
| Facebook | Our view it is spectrum related |
| CMCC | As we commented in the initial round. If the motivation is to find a generic approach for other regions/countries, then this is a non-spectrum WI. If the motivation is to analyze the requirements for each region and check whether new band or n96 should be used, then we are afraid the WI scope will be too vague and become an endless WI. |
| LGE | The WI is spectrum related issue |
| Intel | Although there is no very clear formal boundary, in our assessment this is more spectrum-related item. |
| BT | We believe that the WI is spectrum related. BT also supports Telecom Italia; the re-use of band n96 in Europe is not acceptable |
| Apple | @TIM: This WI is not for EU/CEPT, but rather for those countries (e.g. Canada and Brazil) which are not covered by the existing work scope. So we do not aim at discoursing which band should be used for Europe, this is out of scope of this WI. @CMCC: Maybe it was not crystal clear from the WID justification, but the intention is to leverage technical information contained in TR 37.890. Procedurally we have two tracks: one track is to analyze and update the 6GHz regulations which are captured in TR 37.890, this is the RAN level exercise. And another track is to implement available regulations into the RAN4 specifications. In other words, the WI scope is very focused. Our understanding is that this is the spectrum related WI because it aims at enabling the support of a particular frequency range in a set of countries based on the regulatory requirements, which have been recently published by the corresponding countries. On the technical side, our general preference is Option 1. And as we mentioned earlier, it is up to further RAN4 discussions whether we can survive with one band and NS values or whether we will have to introduce new bands. |
| CHTTL | share the similar view as CMCC that the scope seems not very clear on the boundary. |
| Telecom Italia | I must say the discussion is confusing. But in any case the specific regulations have to be taken into account and if they differ from the US regulation (the basis for n96) new bands should be defined |
| Skyworks | Some of the comments and statements that a new band must be defined because of a different regulation is not actually the general case in 3GPP. Band 41 has different regulation and spectrum range in different regions of the world (and can support n38). Same for band 28, 78,77,79,46….I am missing plenty. And this is even if the spectrum overlaps with other bands Band 20 vs 28, 38 vs 41…. |

Sub-topic 4-2a: Reuse the existing n96 or define a new band for European frequency range of 5945 to 6425 MHz

Discuss the following two issues:

Issue#1: Regulatory framework for the EU/CEPT 6GHz band

(RAN plenary shall decide if unlicensed operation in the frequency range 5945 MHz to 6425 MHz in Europe shall be based on available ECC Decision (20)01 or RAN4 shall consider other aspects as well.)

- Option 1: Unlicensed operation in the frequency range 5945 MHz to 6425 MHz in Europe shall be based on available ECC Decision (20)01

- Option 2: For unlicensed operation in the frequency range 5945 MHz to 6425 MHz in Europe, RAN4 consider other aspects as well, besides available ECC Decision (20)01.

Companies are invited to provide comments and responses in the following table.

| Company | Comments |
| --- | --- |
| Ericsson | Option 1. |
| HPE | We support Option 1. In fact, we do not understand how Option 2 is being considered as a viable option or what other aspects would be considered. |
| Qualcomm | Option 1 |
| Facebook | Option 1. There is no basis to consider in Europe other aspects other than the ECC decision. |
| LGE | Option 1 |
| Intel | Option 1 shall be baseline For option 2 further clarifications on “RAN4 consider other aspects as well” are encouraged. What are the other aspects? |
| BT | Option 1 |
| Apple | Option 1. Option 2 does not even exist because from the EU/CEPT regulatory perspective we have to rely upon the voluntary ECC Decision and the mandatory EC Decision. |
| Skyworks | Not clear to us that we have other regulatory aspects to consider. And the current regulation does not mandate or justify a new band and can be supported by the UEs with a band n96 implementation. |
| Charter Communications Inc | Option 1 |

Issue#2: Band plan for the EU/CEPT 6GHz band

How to introduce the frequency range 5945 to 6425 MHz for unlicensed operation in Europe to the specification by RAN4. As agreed in WF R4-2103229 unlicensed operation in the range 5945-6425 MHz can be introduced by:

- Option 1: Re-using already defined band n96

  - FFS if additional notes and/or clarifications are needed. Regional specific requirements to be included in relevant specifications.

- Option 2: Defining a new band n[xx]

  - On top of specific requirements provided by ECC, the new band shall reuse requirements already defined for n96, where possible.

Companies are invited to provide comments and responses in the following table.

NOTE: companies’ comments under sub-topic #4-2a are copied here.

| Company | Comments |
| --- | --- |
| Telecom Italia | We cannot accept re-use of band n96 in Europe. A new band must be defined to take into account the European regulation |
| Charter Communications Inc. | We can compromise with option 1 if Skyworks comment above prevails, “When compared to n46 where we use 4 NS, for the 6GHz cases, we have to treat indoor/outdoor or VLP/LPI cases on top.” If this cannot be assured then a new band can be defined. In general, we should attempt not defining a new band unless is absolutely necessary. We would had prefer the sub-bullet in option 2 to reflect a statement like this. |
| Nokia | Support option 1. From ecosystem point of view, RAN4 shall always strive to use existing bands unless it is not possible. n96 with NS values will take European regulation into account in exactly the same way as a new band. |
| Ericsson | Option 2. The band for Europe should be based on European regulation. This band is different than other bands in that in Europe there is: unlicensed operation over 5945-6425 MHz and possible licensed operation over upper part (6425-7125 MHz). Therefore, we do not think it is feasible to reuse n96 in Europe. |
| HPE | HPE concurs with Nokia in supporting Option 1 and sees no compelling reason why the already-defined n96 band should not be used or how any existing requirements of the ECC would preclude this approach. |
| Qualcomm | Option 1 |
| Facebook | Option 1 |
| Orange | Same view as Telecom Italia. A new band needs to be defined for Europe taken into account Europe specific band plan and regulatory requirements. |
| Intel | Option 1. Agree with Nokia’s observations. |
| BT | Option 2 We believe that it is necessary to define a new band for 6 GHz NR-U in Europe (option 2); so that equipment can show compliance to the Radio Equipment Directive 2014/53/EU article 3.2, “Radio equipment shall be so constructed that it both effectively uses and supports the efficient use of radio spectrum in order to avoid harmful interference”. At present: - the ECC decision (20)01 doesn’t define out-of-band emission levels above 6425MHz Considering f) states, “there is no out-of-band emissions (OOBE) limit included in this ECC Decision, to protect adjacent incumbents operating above 6425 MHz, as the same incumbents will be operating co-channel with WAS/RLAN below 6425 MHz” - the ECC decision has only considered adjacent satellite and FWA systems operating above 6425MHz. No consideration has been given to future IMT systems - additionally, the draft ETSI standard for 6GHz RLANs (EN 303 687) doesn’t define receiver blocking tests above 6425MHz We believe that having a dedicated European 6GHz NR-U band would: - enable compliance with the Radio Equipment Directive 2014/53/EU - improve receiver selectivity and reduce the need for AMPR (i.e. avoid receiver blocking and support higher transmit powers without causing interference) - give the 3GPP specifications greater flexibility to accommodate future regulatory changes; likely to follow WRC2023 |
| Apple | Option 1. Referring to the existing and available regulatory documents, we cannot see reasons why the existing band n96 cannot be re-used. |
| Skyworks | Option 1: band n96 can be reused and we have a hard time understanding the arguments for option 2: On one end it is agreed that this WI shall use the available regulation that does not require any specific OOB protection and thus can be supported with n96 and NS; and on the other end speculates that a different band is needed because of some unknown future requirements. Even if future requirements did appear, it can be handled with NS and A-MPR and it is exactly how it is done in n46 (some regulation allows operation in UNII4 for example and some others don’t). |
| Huawei, HiSilicon | Option 2. The regulatory requirements are different from those defined for n96. |

Summary

Moderator summarizes discussion status for this round, list all the identified open issues and tentative agreements or candidate options and suggestion for next round.

|  | Status summary |
| --- | --- |
| Sub-topic #4-1, #4-2 | 12 companies commented. 4 companies supported Option 1. 5 companies supported Option 2. 1 company supported Option 1a. Tentative agreements: None. Recommendations for final round: According to the initial round discussion, there is no consensus among companies to start the WI. In the intermediate round, companies had different views on the optional solutions to introduce the support of 6GHz unlicensed spectrum. It seems premature to try to agree on the WI. As moderator, I would like to close the discussion in this meeting. But we saw the proponent revised the WID and want to have a try. So during the GTW we can further check if companies can agree to have a WI for introduction of support of 6GHz unlicensed spectrum to fulfill the regional regulations taking into account all three options. - Can we agree to have a WI for introduction of support of 6GHz unlicensed spectrum to fulfill the regional regulations taking into account all three optional solutions? - Option 1: Depending on the regulatory requirements, determine whether they can be handled by existing NS values or whether new NS values are needed (based on existing NR band n96) - Option 1a: - Depending on the regulatory requirements, determine whether they can be handled by existing NS values or whether new NS values are needed (based on existing NR band n96). - Depending on the regulatory requirements and if these requirements cannot be handled by existing NS values or whether new NS values are not possible, then an introduction to a new band will be needed. - It will need to clearly specify those countries / regions to which it applies - Option 2: New NR band(s) could be considered for other regions with different regulatory requirements. |
| Sub-topic 4-2 | 4 companies expressed the view. 3 agreed that the proposed work should be spectrum related. 1 company think it should be non-spectrum related. Tentative agreements: None. |
| Sub-topic 4-2a Issue#1 | 9 companies provided the comments. All supports Option 1. Tentative agreements: - Unlicensed operation in the frequency range 5945 MHz to 6425 MHz in Europe shall be based on available ECC Decision (20)01 |
| Sub-topic 4-2a Issue#2 | 13 companies commented. 8 companies supported Option 1. 5 supported Option 2. There is no concensus. The follow issues need be discussed in GTW: |
|  | How to introduce the frequency range 5945 to 6425 MHz for unlicensed operation in Europe to the specification by RAN4. As agreed in WF R4-2103229 unlicensed operation in the range 5945-6425 MHz can be introduced by: - Option 1: Re-using already defined band n96 (Charter, Nokia, HPE, Qualcomm, Facebook, Intel, Apple, Skyworks) - FFS if additional notes and/or clarifications are needed. Regional specific requirements to be included in relevant specifications. - Option 2: Defining a new band n[xx] (Telecom Italia, Ericsson, Orange, BT, Huawei) - On top of specific requirements provided by ECC, the new band shall reuse requirements already defined for n96, where possible. |

Final round

Comments & responses

In the final round, according to guidance from GTW on June 16, companies are invited to provide the views on the following discussion points.

The following proposal is aligned with revised WID from proponent.

Sub-topic #4-1, #4-2 Introduction of the 6GHz unlicensed band in other countries/regions

- Have a WI for introduction of support of 6GHz unlicensed spectrum to fulfill the regional regulations taking into account all three optional solutions?

  - Option 1: Depending on the regulatory requirements, determine whether they can be handled by existing NS values or whether new NS values are needed (based on existing NR band n96)

  - Option 1a:

    - Depending on the regulatory requirements, determine whether they can be handled by existing NS values or whether new NS values are needed (based on existing NR band n96).

    - Depending on the regulatory requirements and if these requirements cannot be handled by existing NS values or whether new NS values are not possible, then an introduction to a new band will be needed.

Companies are invited to provide comments and responses in the following table.

| Company | Comments |
| --- | --- |
| Charter Communications Inc | Our preference is Option 1a. We feel this option is a compromise between Option 1 and Option 2. Let’s try to minimize or avoid band proliferation and CA configuration explosions but if it is justifiable then a new band can be introduced |
| Apple | Referring to the latest revised WID we already included the following objective: “Depending on whether existing NS values cannot be re-used and/or whether new NS values are not possible, determine whether an existing band n96 can be re-used or introduction of a new band will be needed”. This is effective what Option 1a is, i.e. whether a new band is needed or not will be decided on further analysis of the regulatory requirements and the number of new NS values. The revised WID can be found here: ftp://ftp.3gpp.org//tsg_ran/TSG_RAN/TSGR_92e/Inbox/Drafts/%5B92-e-05-Spectrum-WIs%5D/Intermediate%20round/revised%20WID%20for%206GHz/draft_RP-21xxxx%206GHz%20WID.docx |
| Qualcomm | Option 1. If the concern is the limited number of NS values available per band, then we can ask whether RAN2 can extend the number of NS’s by using the last NS as a pointer to an extension field. |
| LGE | We can compromise with option 1a for the 6GHz unlicensed band in other countries/regions. |
| Facebook | Option 1. We would like to find a compromise but the second bullet in 1a is too vague – what is the meaning that dependency on the regulatory requirement warrants introduction of a new band and how to determine it. It does not seem as a compromise, rather it extends the arguments. We share QC view on the ask of RAN2 to extend the NS’s |
| CMCC | The original option 2 is already precluded? We support option2 and propose to add the option 2 back. |
| Intel | Option 1 or 1A are fine. 1A is a compromise between 1 and 2 and seem a reasonable way forward. |
| Skyworks | Option 1a is OK but as far as number of NS, if we are close to run out we will anyhow need to extend the range for upcoming bands. Number of NS available should not be the reason for introducing a new band but if some requirement cannot be accommodated by NS a new band can be introduced. |
| Apple | @all: Somewhat re-iterating what we wrote earlier, out current technical understanding is that Option 1 can work. As explained by Qualcomm, we can always extend the number of NS fields. We checked the ASN.1 encoding and can confirm that the existing NS signaling is in the extendable container thus allowing us to add as many values as needed. However, to accommodate all the views and to avoid rushing with the final conclusions, we suggest the following generic objective. It will be the WI phase conclusion which direction we go. “Depending on whether existing NS values cannot be re-used and/or whether new NS values are not possible, determine whether an existing band n96 can be re-used or introduction of a new band will be needed”. Based on that we would like to ask companies whether they are Ok with the proposed objective and/or whether we need to change anything in the final wording. |
| Huawei, HiSilicon | Seems the original option 2 is missing in the summary, we support option 2, i.e. - Option 2: New NR band(s) could be considered for other regions with different regulatory requirements. |

Sub-topic 4-2a:

It is proposed to agree that

- Unlicensed operation in the frequency range 5945 MHz to 6425 MHz in Europe shall be based on available ECC Decision (20)01

Companies are invited to provide comments and responses in the following table.

| Company | Comments |
| --- | --- |
| Charter communications Inc. | We agree |
| Apple | We agree. As an additional clarification, it is not only the existing voluntary ECC Decision, but also the mandatory EC Decision (to be published soon), that unlicensed operation in 5945-6425MHz shall be based on. |
| HPE | We agree |
| Ericsson | Agree |
| Qualcomm | Agree |
| LGE | Agree |
| Facebook | Agree |
| Broadcom | Agree |
| Intel | Agree |
| Skyworks | Agree, and as suggested by Apple any formal update of that regulation should be considered. |

The following discussion point needs further discussion:

In order to move forward, moderator suggests Option 3 for consideration

- The unlicensed operation in the range 5945-6425 MHz can be introduced in RAN4 by:

    - Option 1: Re-using already defined band n96

      - FFS if additional notes and/or clarifications are needed. Regional specific requirements to be included in relevant specifications.

    - Option 2: Defining a new band n[xx]

      - On top of specific requirements provided by ECC, the new band shall reuse requirements already defined for n96, where possible.

    - Option 3: Duplicate

      - Defining a new band n[xx] for BS

        - On top of specific requirements provided by ECC, the new band shall reuse requirements already defined for n96, where possible.

      - Re-using already define band n96 for UE

        - FFS if additional notes and/or clarifications are needed. Regional specific requirements to be included in relevant specifications.

Companies are invited to provide comments and responses in the following table.

| Company | Comments |
| --- | --- |
| Charter Communications Inc. | We believe option 1makes the best sense. Option 3 might be a compromise if there is a majority agreement with this option. |
| Nokia | Option 1. We do not understand what would be the technical justification for a new band for BS? In contrast to UE, BS manufacturer can declare supported frequency range (e.g. 5945-6425 MHz) of frequency band and still use n96 in this case. Introduction of new band can be considered only if well justified. |
| Apple | Option 1. We do not understand how we ended up with Option 3 because it is not clear which band a UE should support for EU/CEPT, not mentioning the fact that it will increase further the number of potential band combinations. |
| HPE | Option 1. We see no compelling reason and no current or probable future requirements, from the ECC or elsewhere, that would warrant any other approach. |
| Ericsson | We support option 2. There are technical reasons for defining new band (5945-6425 MHz) for Europe as explained below: The two band plans (n96 with NS values and European specific band) are not identical. The dedicated unlicensed is better from the point of view of the European regulatory conformance, the frequency range indicated by a dedicated "EU" FBI matches European regulation, operation beyond this as allowed by n96 is illegal. This is particularly important for general authorization, which cannot be guaranteed for n96 usage in Europe, to prevent unauthorize operation in 6425-7125 MHz. Moreover, NS values may be subject to user access restriction since the equipment may not always be professionally installed. The risk of unprofessional installation is higher especially for standalone unlicensed operation. There is therefore some risk that NS values are not properly implemented/signaled. Furthermore, as commented earlier in Europe 6425-7125 MHz is not unlicensed. Due to these reasons we strongly prefer European specific band plan. Requirements for the European band can account for use of a wider RF filter (5925-7125 MHz) e.g. most RX requirements (except OOBB) can be reused from n96. This will allow reuse of n96 hardware. |
| Qualcomm | Option 1 is our preference but we are open to finding a compromise. We don’t understand what option 3 means. Which band number would the basestation advertise? Is the UE expected to map different band numbers advertised by the basestation into n96? Would the basestation signal different band identifier in a secondary field? For option 2, we would like to better understand exactly which requirements would differ in the new band compared to Band n96. Is it just the ARFCN range, ranges of in-band vs. out-of-band, and corresonding general Tx and Rx out-of-band requirements? Or are there additional requirements beyond these? |
| LGE | The baseline is option 1 to define RF requirements for 5945-6425 MHz. RAN4 can further discuss the different point between ECC and FCC regulation requirements. |
| Facebook | Option 1. Our views are similar to QC to understand better what would differ between new band and n96. We would not be supportive to drive new band requirements by anticipation of possibility of IMT to be authorized in the upper part of the band unless there is regulatory certainty. We do not understand Option 3 and it seems to further complicate the matters |
| Broadcom | Option 1 |
| Intel | Option 1 is preferable. We are open to discuss potential compromise approaches, but do not think that Option 3 is a valid compromise. We do not understand the motivation to use new band specifically for BS. We think additional RAN4 discussion on the technical aspect can still help to achieve further progress (e.g. discuss a new option proposed by E/// for use of a wider RF filter). |
| Skyworks | Option 1, in any case Option 3 does not really require a new band for the BS as the BS can anyhow restrict its operation to the European frequency range and thus UEs could not operate above 6425MHz either. |
| Telecom Italia | Option 2. Option 3 is not clear, but if it means having two options for the same problem, it is not acceptable Option 1 is not acceptable. We will make sustained objection against option 1. And this issue must be solved in RAN, not sent back to RAN4. |
| Deutsche Telekom | Option 2 is the only acceptable way forward for us. Option 3 does not make any sense. Ericsson also explained some further technical and practical implications if n96 is reused. |
| BT | Option 2: We agree with Telecom Italia, the reuse of NR band n96 in Europe is unacceptable. BT’s priority is to provide the best possible quality of service to our customers. They are no technical justifications, why our customers should accept poor NR-U/RLAN performance? We see a dedicated 6GHz NR-U band in Europe as essential. This issue needs to be resolved within RAN plenary, as this is not a technical issue. It should be noted; that all European operators (which have expressed an opinion), are in favour of option 2. |
| Huawei, HiSilicon | We support Option 2. Option 1 is not acceptable since in Europe 6425-7125 MHz is not unlicensed. And the regulatory requirements are different from those defined for n96. Define a new band can still reuse n96 hardware where possible. |
| Vodafone | We have some concerns similar to those expressed by Ericsson when it comes to protection of 6425-7125 MHz. At this stage, defining a European band that leverages the existing n96 Rx requirements (allowing reuse of hardware) would appear to be the safest option. As such, we also prefer option 2. |
| Orange | Same view as Telecom Italia, DR and BT. Option 2 is the only solution acceptable. We also support Ericsson’s analysis on the technical merit of a new band. 3GPP should strive to make the best choice for many years to come. It should also be highlighted that the decision relates to the European market, and therefore the preference of European operators should be strongly considered. |

Summary

Moderator summarizes discussion status and provide the recommendation.

|  | Status summary |
| --- | --- |
| Sub-topic #4-1, #4-2 | According the comments in the last round, we observed compromise. Given one company still proposed to add the Option 2 back. So moderator propose to go back to proposal in the intermediate round to some extent: Proposal #4-1: It is proposed to approve the WI of introduction of the 6GHz unlicensed band in other countries/regions, where the following optional solutions need be taken into account: - Option 1a: - Depending on the regulatory requirements, determine whether they can be handled by existing NS values or whether new NS values are needed. - Depending on whether existing NS values cannot be re-used and/or whether new NS values are not possible, determine whether an existing band n96 can be re-used or introduction of a new band will be needed. - Option 2: New NR band(s) could be considered for other regions with different regulatory requirements. |
| Sub-topic 4-2a: | Proposal #4-2: it is proposed to agree on the following statement. - Unlicensed operation in the frequency range 5945 MHz to 6425 MHz in Europe shall be based on available ECC Decision (20)01 For Sub-topic #4-2a Issue #2, introduction of European lower 6GHz unlicensed band, there is no consensus. Some companies proposed to go back to RAN4, while some company propose to address this in RAN plenary. This topic needs further discussion in GTW. |

Topic #5: Improving MSD for CA and DC

Companies’ contributions summary

| T-doc number | Title | Sourcing company |
| --- | --- | --- |
| RP-211305 | Improved MSD for CA and DC | Qualcomm Incorporated, T-Mobile USA, Verizon, AT&T, DISH Network, TELUS, Deutsche Telekom, CMCC, CHTTL, KT Corp, Vodafone, BT plc., Telecom Italia, Bell Mobility |

Initial round

Comments & responses

In this section, we collect the comments and responses for the proposed work item. Based on the comments, we will decide how to move forward in the next step.

Background:

The issue was triggered by the proposal to design a solution to improve MSD for power class 2 EN-DC in RAN4. And then the proponents proposed to apply it for all the power classes. The latest discussions were captured in RAN4 email summaries of R4-2107937 and R4-2107926.

Because there is no clear objective for such generic solution to improve MSD in any existing WID, and it seems not proper to treat it under basket work items for band combinations in a band specific manner, it was suggested to discuss how to handle it in RAN.

Sub-topic 5-1: Comments and questions on proposal#1, i.e., should we introduce a generic UE capability for improving MSD?

Proposal 1: RAN4 to introduce a new UE capability bit to allow a UE indicate support for improved MSD

Companies are invited to provide comments on Proposal 1 in the follow table.

| Company | Comments |
| --- | --- |
| Ericsson | We support the proposal to improve MSD. However, we have few comments on the scope of the work: - We prefer not to define improved MSD as UE capability. Instead new MSD values identified during the study should replace the existing MSD values in Rel-17. In the past we have reduced margins/improve requirements which replaced the old ones e.g. RF margins in RSRP accuracy. - The above proposal is to reduce the MSD when the IMD overlaps with the wanted channel. However, there is also an ongoing discussion in RAN4 on MSD improvement triggered by RAN5 LS (R4-2104470/R5-211609). This concerns requirements for IMD when the IMD falls just outside the wanted DL channel i.e. to verify the UE performance when the channel is assigned to avoid IMD. Both mechanisms should be considered. In summary the scope of “MSD improvement” should include: - Verification of the MSD when the IMD misses the wanted channel (MSD = 0 dB or a small value) as triggered by RAN5 LS. - Reduction of the MSD when the IMD overlaps with the wanted channel. |
| T-Mobile USA | We support the proposal for an improved MSD capability. We don’t think the right approach is simply new MSD values. The 3GPP minimum requirements allow for a diversity of implementation options, and the current requirements allow for the worst-case assumptions. In certain markets a large percentage of Ues have implementations, for instance integrated rather than discrete RF Front end, that allow the MSD requirements to be greatly exceeded. The problem the operators have is that we cannot distinguish between Ues that only meet the minimum requirements and require close to the allowed MSD and Ues that require very little MSD. Revising the current MSD values as suggested by Ericsson will not rectify the situation, because there will still be a need for architectural flexibility. We support architectural flexibility, but we need to be able to distinguish between Ues that need to allowed MSD, and Ues that require very little MSD. |
| Qualcomm | We (obviously) support this proposal as the specified MSD values are too large to enable operator deployment and we recognize the benefit for the network to be able to distinguish higher performing devices by signaling. We agree with T-Mobile that a greatly improved MSD should be specified with capability signaling in addition to, not instead of, the existing minimum requirement MSD value. |
| SoftBank | We support this proposal. |
| Huawei, HiSilicon | Generally we are open to have further discussion on the issue of MSD improvement, but we are not ready to accept the capability without clear understanding and consensus in the group which assumptions should be considered for the evaluation of improved MSD. There are still quite a lot issues to be addressed for the MSD topic. In the spec, there are different types of MSD, e.g. harmonic MSD, harmonic mixing MSD, cross band isolation MSD, intermodulation MSD, etc. For a specific band combination, it may have one of them or several MSDs. For example, if the combo has both harmonic MSD and intermodulation MSD, It is possible that harmonic could be improved but IMD cannot, and even for the harmonic, as the current requirement is defined on band basis, which means for a specific frequency configuration, there could be no MSD issue at all for the operator. All the cases mixed together, it would be difficult for the network to distinguish the Ues just by the reported signaling. Replacing the existing MSD values is one way we could consider. On the other hand, there are many new band combinations proposed in Rel-17, how to define the MSD for some of the band combinations are still undergoing, the alternative way is to define better MSD requirements for combos, certainly it also depends on appropriate component capability assumptions. But without the MSD values specified for the combos, in which place we say the possible improvement? Should we focus on defining the MSD values for the proposed band combinations introduced in Rel-17 firstly? The workload in RAN4 is extremely heavy, and the discussion of MSD improvement already occupied lots of RAN4 TU, which actually is out of the scope of the WID. We don’t think RAN4 could have additional TU to study MSD improvement in Rel-17, and since there are so many issues related to MSD need to be addressed, a feasibility study in Rel-18 is more practical to proceed. |
| NTT DOCOMO, INC. | We support this proposal to make NW deployment more flexible. Also agree with T-Mobile comment. From operator perspective, the UE who can perform better performance and only meet the minimum requirement should be distinguished to provide good service for all Ues. |
| CMCC | We support improving MSD requirements. Coverage is very important for operators. And it seems common understanding that existing MSD values are too large, and some UE can achieve much better performance. We support to distinguish UE with different capabilities. |
| KDDI | We support this proposal. |
| LGE | RAN4 already discussed why UE vendor had concern to improve MSD value. And LGE really would like to know how we can improve MSD values under the condition that UE should support variable MR-DC, NR CA combinations and other features (e.g. UL-MIMO, TxD). |
| Telecom Italia | As cosigning company we support the proposal |
| DISH Network | We support the proposal. The problem for operators is real; it is very challenging to deploy some combination e.g 30dB 3GPP defined MSD without knowing each UE MSD performance more in detail, as some devices require only few dB and some require significantly more, up to close 30dB. From RF technology perspective, it has been demonstrated by devices in the field that it is fully possible to optimize MSD performance really a lot in typical smartphone type of devices. We don’t understand LGE comment, does that comment mean LGE thinks that 30dB MSD cannot be improved in real device? This would be per band combination anyways, so agreeing the proposal does not mean ALL combinations in device must be optimized. |
| Deutsche Telekom | We support the proposal |
| BT | We support this proposal. |
| Telstra | We support the proposal |
| Intel | We are open to discuss it. For the exact solution, we think that further analysis on the exact capability is required (i.e. whether it should be capability or we simply tighten the MSD values in the specs; what should be the granularity of uplexing; whether uplexing shall be provided per BC or per UE). Overall, we prefer not to have a single MSD values to ensure flexibility of implementation. The exact details can be left up to the WG-level discussion. Also, RAN4 may need to assess the actual system benefits from the improved MSD. In particular, it is helpful to understand the existing network implementations to handle the MSD and how the network may use the information on the actual MSD values (e.g. whether legacy networks perform or not perform scheduling of Ues in problematic resource allocations and how it is planned to be improved in case improved MSD capabilities are defined). So, we suggest having a discussion in the WG-level to identify the exact impacts on the system performance for the legacy systems and for the new improved MSD scenarios. This work affects the generic requirements rather than BC-specific and in our view, it may not qualify as purely spectrum item. So, it is preferable to keep the discussion under non-spectrum items. Finally, further discussion whether it can fit to Rel-17 is required taking into account the TU assessment and RAN4 workload. |
| Apple | Our preference is also not to define improved MSD as UE capability as it would introduce more specifications development works and signaling loading. We would also like to understand how much improvement is expected especially for IMD2 and IMD3 and what are the key mechanism(s) for improvement? |
| CHTTL | We support the proposal and agree with T-mobile’s comment. |
| MTK | One uplexingion question. How could a single bit uplexi different UE implementations. As we know, MSD is also UE FE architecture dependent. Will there be new UE capability bit indicating FE architecture also? (for example: MSD for separate antenna would be different from UE using diplexer) |
| Nokia | We support proposal 1 |
| AT&T | We support this proposal. The existing MSD requirements based on conservative assumptions result in the inability for operators to confidently allocate the very CA/DC combinations intended to improve customer experience and network capacity. In the end, the work done to specify very large MSD values might be wasted if the combinations are never deployed/utilized and results in UE conformance test cases that add very little value. In addition, we also support the Ericsson comment related to concerns about requirements for IMD when the IMD falls just outside the wanted DL channel and we would support adding this analysis to the effort. This issue results in the same situation where certain combinations are limited in deployment. |
| Skyworks | Since it is impossible for RAN4 to revisit every single MSD values we first need to understand which are the MSD that are in scope: are they MSD above a given value? A given MSD type? Only inter-band or also intra-band? What is then signaled? -a better value? A new value in a different table? -if a better MSD value is signaled,is it valid for all MSDs of a given combinations (all harmonics, IMD, cross band related) or one by one? -if a better MSD value is signaled for a low order combination, is it still realizable for a higher order combination? For example band A and B use a simple uplexing but the band C and D are added in the same range than B and requires an hexaplexer: is the improved MSD still feasible? To the same value? Unless we have a clear view of how to tackle the above questions the scope is too vague, too large to be handled and may result in lower MSD only for the simple cases but not when higher order combinations needs to be supported. Also it should be considered that RAN4 only look at band combinations one by one where in reality a UE supports a large set of overlapping band combinations. If we agree that higher integration and state of the art technology have better performance than that of the minimum requirement it shall also be acknowledged that margin is required as real implementations cover a large set of overlapping higher order combinations covering multiple regions and thus must have trade offs between all combinations and not just one as specified in RAN4. |
| ZTE | We appreciate the efforts to improve MSD and consider proposal 1 if TU permitted for the required work. For a specific UL/DL combination, MSD improvement could be different for different configurations, e.g., channel bandwidth, in this case, is the least MSD improvement selected among all possible channel bandwidths if a single value is specified? |
| Vivo | We agree that for certain combinations there could be improvements for MSD value. However, this was also depending on many factors and quite implementation specific, thus may not that easily to be specified. It is probable that only in quite limited cases that noticeable effect can be seen and worthy to be done. Considering the Rel-17 time frame and the non-urgent nature of this topic, may be Rel-18 is more appropriate, and release independency can also be considered if deemed necessary and applicable. For Rel17 this work is proposed to be put on hold. |
| Orange | We are supportive of this proposal |
| OPPO | This has been discussed in RAN4 before, and no conclusion there, and is not clear of how much commercial UE can improve the MSD and also in which level this can be improved. Many open questions are not been answered in RAN4. Therefore it is premature to discuss introducing new singaling for MSD improvement. |
| Xiaomi | MSD improvement for specific band combination is possible. However, in our view, it is very hard to get a common rule on assumptions to derive the improved MSD for those band combinations having MSD, since the MSD improvement in one band combination may led to MSD deterioration for other band combinations. Thus, the feasibility should be discussed first. |
| Verizon | We support the proposal and do believe this work can enhance the network performance. |
| Bell Mobility | We support the proposal |

Sub-topic 5-2: If we can agree to do the work for improve MSD, then how can we treat it, i.e., in new WI, adding objectives in the existing WI, or TEI?

Question: How can we handle the work?

  - Option 1: new work item

  - Option 2: adding the new objectives in the existing work item

  - Option 3: TEI17

  - Other option

Companies are invited to provide answers on the above questions in the follow table.

| Company | Comments |
| --- | --- |
| Ericsson | Option 2. Prefer to add it in one of the existing WIs. It might be difficult to complete in one WG meeting so TEI17 may be unrealistic. |
| T-Mobile USA | Option 2: Prefer to add it in one of the existing WIs. |
| Qualcomm | Also prefer option 2 |
| Huawei, HiSilicon | Other option. The focus of Rel-17 is to specify the MSD requirements for the proposed band combinations rather than to discuss the MSD improvement. As the heavy workload in RAN4, we prefer to have feasibility study of improvement in Rel-18. |
| CMCC | Prefer option2. |
| LGE | Other option. This is not feasible as UE vendor perspective. |
| Telecom Italia | Option 2 or TEI 17 |
| DISH Network | Prefer option 2. Option 1 could also be considered as option 2 might have the challenge to address both PC2 and PC3 on one go (no existing single WI which has both PC2 and PC3). |
| Deutsche Telekom | Preference for Option 2 |
| Telstra | Option 2 preferred |
| Intel | Option 2. The scope is quite big for TEI17. Prefer to add to the non-spectrum WI. |
| Apple | Companies can follow the same exercise for every MSD calculation with increment improvement in RF components performance to develop the MSD requirements for the newly proposed combinations. It may not be necessary to have a new handing on the improved MSD requirements. |
| CHTTL | Prefer option 2. |
| MTK | Give the current RAN4 remaining RF TU is already a negative value, we wonder whether we still have the margin to start a new work? |
| Nokia | Work under TEI is not preferred |
| AT&T | We prefer Option 2. |
| Skyworks | It is unclear which WI it should be attached to since MSD is discussed in basket WI but also in the 35/45MHz, new BW, PC2 TDD, PC2 FDD……we believe this work if pursued cannot be under the block approval process and the signaling aspects have to be discussed specifically. |
| ZTE | Option 2 seems more feasible. |
| vivo | Considering the time frame and workload, if really considered, R18 as part of a WI is preferred. |
| Orange | Option 2 is preferred |
| OPPO | Rel-17 is not a good choice, can be further discussed in Rel-18 maybe. |
| Xiaomi | We prefer to have feasibility study of improvement in Rel-18. |
| Verizon | Option 2 |

Sub-topic 5-3: Comments and responses on potential objectives

The following objectives are proposed.

  - How to determine the improved MSD value [RAN4]

  - The details of the signaling need to be decided [RAN2/RAN4?]

  - Should this be limited to new combinations being specified, i.e., PC2 CA, or should this also apply (optionally) to existing combinations even PC3? [RAN4?]

  - Under what work item and release should this be carried out? [RAN]

For the last bullet, please focus on “release” here.

Companies are invited to provide comments and responses in the following table.

| Company | Comments |
| --- | --- |
| Ericsson | As commented in 5-1, we prefer not to define improved MSD as UE capability. The scope can also be applied to existing combinations and also PC3. It can be done in Rel-17. |
| T-Mobile USA | We support the proposed objectives. We think the new MSD capability can apply to Rel-17. |
| Qualcomm | Our preference is Rel-17 with release independence to earlier releases if possible |
| Huawei, HiSilicon | Our preference is not to define new UE capability for improved MSD. Feasibility study is needed for MSD improvement, and the work should be done in Rel-18 rather than in Rel-17 due to the heavy workload in RAN4. |
| NTT DOCOMO, INC. | As stated in sub-topic 5-1, we prefer to introduce new MSD capability in Rel-17 and it should also be applied to existing combinations even PC3. |
| CMCC | Support the objetvies and also support to apply to existing combinations including PC3. |
| LGE | RAN4 had many discussion on this issues. So, do not need to define additional capability. |
| DISH Network | This should apply to both PC3 and PC2. Release 17 should be fine, recognizing that not all combinations have to be analyzed during Rel17; this should be release independent. For instance, the highest priority combinations could be chosen based on real spectrum holdings, deployments, and the amount of existing MSD. |
| Deutsche Telekom | Rel-17 is fine |
| Telstra | Rel-17 |
| Apple | Our preference is to limit to new combinations only. Companies should provide justifications on how the improvement is achieved with detailed link budget. If more than one companies provide the analysis and proposed MSDs, average or some middle ground would be considered as has been commonly practiced in RAN4. |
| CHTTL | Share the view as T-mobile, NTT Docomo and CMCC. |
| Nokia | We don’t think the 2nd bullet is necessary if the idea is just for UE to tell NW MSD value with a certain granularity such that 0, 10, 20 or 30dB… Regarding the 3rd bullet, we haven’t seen the reason not to include PC3. With respect to the release, the discussion on this improvement is conducted in Rel-17 but from which release this capability is available as release independent should be discussed in parallel. |
| AT&T | This effort should apply to PC3 and PC2. Concerning which release, we prefer Rel-17 with release independence if possible. |
| ZTE | This could be carried out in Rel-17. |
| Vivo | If considered, R18 is preferred. |
| Orange | Rel17 should be targeted |
| Xiaomi | Too early to discuss this. |
| Verizon | It can be done in Rel-17 with release independence to earlier releases if possible |
| Bell Mobility | Rel-17 |

Sub-topic 5-4: Comments and responses on signalling design, i.e., Proposal #2 and #3 in RP-211445.

Proposal 2: RAN4 will choose a (preferably low single digit) improved MSD value that a UE could declare support for a given combination with a new capability bit

- RAN4 could either define a single improved MSD value for all the types of MSD, or it could choose one level for harmonics, one level for harmonic mixing, one level for cross-band isolation and one value for Intermods.

Proposal 3: The new capability would be ignaled per UL/DL band combination

Companies are invited to provide general comments on the above two proposals in the follow table.

| Company | Comments |
| --- | --- |
| Ericsson | The details should be left for RAN4. As commented in previous sub-topics, new requirements should apply to all Rel-17 Ues. |
| T-Mobile USA | We think the details should be left to RAN4. |
| Huawei, HiSilicon | We would like define MSD requirements for the proposed band combinations firstly according to the existing WID objectives in Rel-17. Whether and how to improve the MSD as well as the details can be left for future release. |
| CMCC | The proposal can be used as a starting point for RAN4 discussion. |
| LGE | Based on our comments in sub-topic 5-1, 5-2 and 5-3, it should be discussed based on RAN4 consensus which factor are possible improvement point compare to current MSD study. |
| DISH Network | This is a good starting point |
| Deutsche Telekom | This is RAN4 work |
| Telstra | RAN 4 to decide |
| Apple | Our preference is not to introduce capability for MSD. |
| CHTTL | Though the details can be discussed in RAN4, the proposal can be the starting point, and some guidance can be helpful. |
| MTK | The proposals are too detail. Note sure if this is what should be discussed in plenary. For P2, there are different cases for MSD due to cross band isolation (case 1~case 3), different orders of intermods, how can a single value represent all these complicated cases. For P3, MSD due to IMD for 3-bands combination as well as due to triple-bit shall be considered also for the new signaling if new UE capability bit is agreed. In another words, all MSD mechanisms being discussed in RAN4. |
| Nokia | This kind of details should be discussed in RAN4. |
| AT&T | RAN4 can decide. |
| Skyworks | Since it is impossible given the current load to spend a large effort on optimization, we suggest that the scope should be limited or have clear criterias to avoid a flood of contributions with poor quality. Please note that close to 50% of block approval contributions for ½and 3 bands in FR1 have a flag as the proposed MSD has errors (missing case, underestimated issues….). The careful check of block approval contributions for MSD have effectively added an extra week of work in front of the meeting, we simply cannot afford the same for an optimized performance. |
| ZTE | As commented before, more discussions are needed on whether or not to go for a single MSD improvement value if the work is agreed. |
| Vivo | Proposed to not discussed in Rel17 in current condition. |
| Orange | To be left to RAN4 decision |
| OPPO | Too early to discuss signaling. |
| Xiaomi | It shall depend on RAN4 |
| Verizon | Let RAN4 make decision |
| Bell Mobility | Can be left to RAN4 to make the decision |

Summary

Moderator summarizes discussion status for this round, list all the identified open issues and tentative agreements or candidate options and suggestion for next round.

|  | Status summary |
| --- | --- |
| Sub-topic #5-1 | 25 companies commented. Among them, 16 companies including 13 operators supported the work. 4 companies are open to further discussion. 5 companies still had concerns. Companies’ views are quite diverse. It seems difficult to agree on introduction of a new UE capability signaling. The technique issues are discussed extensively in RAN4 but most vendors still thought that it is not easy to improve MSD and too many factors need be considered. Since there are widely demands on improving MSD from operators, I wonder if companies can reach a consensus to initialize a work targeting at improving MSD rather than agreeing on introduction of signaling. Tentative agreements: None. Candidate options: None. Recommendations for intermediate round: Further discuss - Whether 3GPP can initialize the work targeting at improving MSD. - FFS whether the capability signaling is feasible - FFS how to improve MSD and which factors should be taken into account |
| Sub-topic #5-2 | 22 companies commented. 13 companies supported option 2. It seems that the scope is big if the further evaluation on how to improve MSD is needed. Thus Option 3 is not feasible. From moderator perspective, I would like to suggest companies to consider to include it in the existing non-spectrum related WID. Based on current situation, it is difficult to directly agree on introduction of capability signaling. The improved MSD requirements need be studied. So I would like to suggest considering adding an objective with a study phase. But we should also consider that the workload for RF room is very high. Tentative agreements: None. Candidate options: None. Recommendations for intermediate round: Down-select to Option 2 and other options, further discuss how to organize the work: - Option 2: adding the new objectives in the existing work item - Option 2a: adding the new objectives in the existing non-spectrum related work item - Have a study phase to evaluate - How to improve MSD - Feasibility to introduce a capability signaling - Option 3:Defer the work improving MSD to Rel-18 WI |
| Sub-topic #5-3 potential objectives | 18 companies commented. 12 companies supported to introduce the capability from Rel-17. 2 companies prefer Rel-18. 3 companies referred not to specify the capability signaling. Tentative agreements: None. Candidate options: None. Recommendations for intermediate round: There is no consensus on the capability signaling. We need first address the sub-topic #5-1 and #5-2. Before addressing #5-1 and #5-2, we do not need discuss the detailed objectives in the intermediate round. |
| Sub-topic #5-4 signaling design | Most companies agreed that the signaling design should be left to working group. Then we may not need further discuss this topic in the intermediate round. Tentative agreements: None. Candidate options: None. Recommendations for intermediate round: No further discussion on it in the intermediate round. |

Intermediate round

Comments & responses

Based on the comments in the initial round, we would like to encourage companies to have further discussion on sub-topic #5-1 and sub-topic #5-2.

Sub-topic 5-1: Comments and questions on proposal#1, i.e., should we introduce a generic UE capability for improving MSD?

Further discuss

- Whether 3GPP can initialize the work targeting at improving MSD

  - FFS whether the capability signaling is feasible

  - FFS how to improve MSD and which factors should be taken into account

Companies are invited to provide comments and responses in the following table.

| Company | Comments |
| --- | --- |
| Ericsson | We support work on MSD improvement. However, lot of details need to be discussed in RAN4. Without knowing the detail it is difficult to conclude whether the generic signaling is feasible. The need for signaling can be decided by RAN4 when MSD work has progressed. |
| T-Mobile USA | We support capability signalling for UEs with better MSD. We think that improving the existing MSD may only result in a small improvement. The problem that we have is some of the MSD in the spec is very high, but mid to high tier UEs have little to no sensitivity degradation, so our network colleagues choose to ignore the MSD. As an alternative to improved MSD capability signalling, RAN4 should explore the use of In Device Coexistence (IDC) to allow the UE to send a list of frequencies affected by self-interference to indicate the problem to the network. TS 38.300 includes some wording on IDC, ref paragraph 7.9 on UE Assistance Information (which in turn points to TS 36.300 where IDC indication was specified for LTE). |
| Qualcomm | We believe that capability signaling should be introduced. It is optional anyways so there should be no concerns from vendors. Then we can further discuss the MSD values associated with that capability. Introduction of this capability is supported by 15 operators, 100% of the operators who responded. |
| AT&T | We continue to support the need for MSD improvement. We agree with TMUS that for certain classes of UEs that there is little sensitivity degradation. We should discuss the details concerning UE assumptions in RAN4 to derive achievable MSD performance levels for mid to high tier UEs. We can decide on the necessary signalling to differentiate UEs after work has progressed and we see what options are more suitable. |
| Verizon | If the capability signaling becomes a condition for better MSD, we are ok. Otherwise, the detailed requirement should let RAN4 make decision |
| CMCC | We support MSD improvement in general. The details on how to improve MSD and UE capabilities can be further discussed in RAN4. |
| Samsung | We agree with Moderator that further discussion shall focus on how or whether to improve MSD instead of discussing the UE capability signaling. Different UE capability if introduced not only have impact to UE but also network has to deal with different capability UE in the same network. Therefore, we think we shall focus on how to improve the MSD in intermediate round discussion. On improving the MSD, we think the feasibility study shall be carried out firstly before we agreed on working target on improving MSD. The feasibility study shall including - which CA/DC band combinations MSD needs improvement. It seems the scope includes CA and DC band combinations with large MSD. It is better to clarify how to decide the large MSD. To improve the MSD for every band combination shall be certainly avoided given RAN4 is only supposed to define the minimum requirements in implementation agnostic manner. To have different set of requirements for UE with different implementations (or capability) has to be restricted for essential enhancements. Also, the spectrum holding information from operators are also required for RAN4 to further determine whether the root cause of large MSD exists or not in the deployed network based on operator spectrum holding situation. - How to improve the MSD considering different implementations |
| DISH Network | We support MSD improvement, and believe that capability signaling is the best way to achieve significantly better MSD numbers for challenging combinations. Without capability signaling it is very likely that improvement, if any is just few dB out of 30 or so which is not solving the problems. |
| LGE | The scope is unclear and still we do not find the necessity of the proposed WI. So RAN4 need SI to make WI for MSD improvement whether RF factors will be enhanced or not. |
| MTK | RAN4 discussion is needed. It is too early to decide this one. |
| Orange | We support MSD improvements. We believe that introducing UE capability signaling or not for specific combinations can be treated on a case by case basis depending on the MSD improvement envisaged. |
| KDDI | Same view as Orange. |
| Intel | We are supportive of overall direction. Detailed technical discussion on capability signalling and achievable MSD in RAN4 is required before making final conclusions. In general, RAN4 shall discuss: 1) Applicable BCs; 2) Achievable MSD improvement; 3) Performance impact from network perspective; 4) capability signalling |
| ZTE | Without capability signaling, not sure how to achieve the potential gain from the improved MSD from network perspective. Another point is the potential value set associated with the capability, perhaps it is more feasible to indicate the delta values on top of the current MSD values for a specific band combo and configuration, instead of absolute improved MSD values. |
| Apple | The MSD improvement can be further discussed by RAN4. However, the implication on introducing the capability signaling for MSD should be carefully considered. It would mean that at least two sets of MSD requirements would need to be evaluated for each band combination, which could result in at least doubling the workload, increasing the block approval TP review time, doubling the MSD table size and complicating the technical specifications, and not to mention the signaling overhead and the potential compliance test burden. Improvement in MSD may also come with better filter isolation which could be at the expense of higher filter insertion loss. Then we may have to create another set of Tib and Rib, etc. It is also not clear how network would handle UE differently simply based on the MSD difference for one particular test configuration. The way the current MSD is defined is based on the worst-case configurations. It does not mean the UE would always be subject to such MSD influence. On the other hand, MSD is typically caused by higher order ( ≥ 2) IMD or harmonic. When the UL power is reduced, the MSD would drop rather quickly. So even for worst-case MSD in the 30ish dB range, it does not mean the combination is at all unusable. On the other hand, high MSD is not the issue only to CA or DC combinations. For certain FDD bands with narrow duplex distance, we could also have rather high MSD, possibly in the 20ish dB or higher range. However, the REFSENS requirements definition had been intentionally hiding such high MSD value by having the UL configurations constrained to a rather slim RB number. If we would define REFSENS with full UL RB allocation for those FDD bands, shouldn’t we have concern in supporting wider UL channel BW for certain FDD bands? Lastly, if we would continue introducing more UE requirement dependent capability, wouldn’t the UEs become so fragmented in the field which could further burden the network management? |
| CHTTL | We share the same view as T-Mobile USA and Qualcomm. We think the capability signalling is feasible, and we think overall we need to agree on the having this capability first, and RAN4 can further discuss on the remaining details. As in previous RAN4 meetings, we also tried to discuss on which CA/DC band combinations MSD needs improvement ex: starting from 2nd, 3rd order of harmonics/IMD, anyway this can be further discussed in RAN4. |
| Skyworks | As discussed in initial round, we do not see how one can sate that capability signaling is feasible if none can answer the few basic questions we raised. At least how many bits, per UE (then what value, for all band all combinations, if one fails all the other improved bands /combinations would not benefit?), per combinations, then this improved MSD should work up to the highest order the UE supports just not the lowest fall back. Unless there are clear answers about what the capability means we do not see how we can agree on it. In our view the signaling design if any should come after we have an agreement on if/how/which MSD can be improved. Furthermore this is an optimization effort in a time of overload in RAN4 so we need clear criteria on what is worth speding effort on. And this certainly cannot be a block approval method. |
| Nokia | We don’t see the reason to discuss feasibility of signaling. It is difficult to imagine the case that signaling is not feasible. And also we don’t see the reason to discuss the factors related to MSD. If our understanding is correct, proponents of this improved MSD are just saying that there are UEs with even smaller MSD and it is good for NW to distinguish these UEs and UEs with large MSD. The root cause of the MSD does not matter. |
| vivo | It is proposed to discuss this in later stage as part of a WI in Rel-18, since the benefit is still not that clear. MSD is anyway a threshold that only have limited guidance to network planning, and the enhancement is highly implementation specific, thus not that easily to be specified efficiently. |
| NTT DOCOMO, INC. | We support MSD improvement. Although the intention of “generic UE capability” is unclear, at least the capability signaling to distinguish the UE who can perform better performance and only meet the minimum requirement is needed considering various UE implementation. |
| Huawei, HiSilicon | It’s too early to decide whether the UE capability is needed or not and whether the UE capability can well solve the MSD improvement issue, it should depends on the further study of the issue. But we are open to have improvement of the MSD values. |
| Vodafone | We support the work on MSD improvement. With regards to deriving the achievable MSD performance levels and any signaling requirements, we share the same view as Orange and AT&T. |

Sub-topic 5-2: If we can agree to do the work for improve MSD, then how can we treat it, i.e., in new WI, adding objectives in the existing WI, or TEI?

Further discuss how to organize the work:

- Option 2: adding the new objectives in the existing work item

- Option 2a: adding the new objectives in the existing non-spectrum related work item

  - Have a study phase to evaluate

    - How to improve MSD

    - Feasibility to introduce a capability signaling

- Option 3: Defer the work improving MSD to Rel-18 WI

Companies are invited to provide comments and responses in the following table.

| Company | Comments |
| --- | --- |
| Ericsson | Options 2 and 2a. We agreed there is need for study phase to identify mechanisms to improve MSD and also whether signaling is needed, and also type/details of signaling (if needed). |
| T-Mobile USA | We prefer Option 2 or 2a. But if the issue has not been resolved by the time Rel-18 Wis are approved, we would support a Rel-18 WI. |
| Qualcomm | Option 2 |
| AT&T | We agree with the comments from Ericsson and TMUS. |
| Facebook | Option 2 |
| Verizon | Option 2 |
| CMCC | Option 2 |
| Samsung | Both option 2 and option 2a require overall RAN4 workload impact discussion with other up-scoping discussion in other e-mail threads. Also, the impact to RAN4 workload for MSD improving work depends on the working scope, i.e., as commented in Sub topic 5-1, which band combinations requires MSD improvement. If MSD for every band combination has to be discussed in Rel-17 timeframe, it certainly cannot be handled at all in Rel-17 and shall be deferred to REl-18 as a WI. |
| DISH Network | Option 2. With Option 2, and release independent signaling we don’t need to discuss _all_ combinations during Rel-17. |
| LGE | Other Option. We prefer to study item firstly in Rel-18. RAN4 will study whether improve the MSD for PC2 UE considering with detail RF factor for enhancement compare to legacy MSD study. |
| MTK | We should consider current RAN4 remaining TUs which is negative. In this case, Option 3 seems more feasible choice. |
| Orange | Option 2 or 2a |
| Intel | Prefer Option 3. There is no room for new RF work in Rel-17 with such big scope. Options 2/2a will require substantial downscoping of other RF relevant Rel-17 objectives first in order to keep the reasonable Rel-17 scope. |
| ZTE | Option 2 or 2a would be fine. |
| CHTTL | Option 2 |
| Skyworks | Option 2 is not feasible as many WI are involved for MSD: block approval WI, 35/45MHz, new BW, PC2 vs PC3… If agreed and whatever which release this requires a dedicated WI to properly decide if/how/which MSD are improved and design the subsequent signaling. |
| Nokia | Option 2 |
| vivo | Option 3 |
| Huawei, HiSilicon | Given the current workload, we prefer option 3. SI firstly or WI with study phase in Rel-18 are both acceptable for us. Whether release independent can be used for MSD improvement can be further discussed. |
| Vodafone | Option 2 or 2a |

Summary

Moderator summarizes discussion status for this round, list all the identified open issues and tentative agreements or candidate options and suggestion for next round.

|  | Status summary |
| --- | --- |
| Sub-topic #5-1 | 22 companies commented. 14 companies supported improving MSD. 6 companies had concern on feasibility of signaling and would like to first study how and whether to improve MSD first. But it seems that no company is against this direction to improve MSD. Tentative agreements: - It is agreeable to improve the MSD for CA and DC. - adding the new objectives in the existing work item (non-spectrum) - Have a study phase to evaluate - How to improve MSD - Feasibility to introduce a capability signaling - Take one CA and on DC combination as example for evaluation Recommendations for final round: Check if the above agreement is agreeable. |

Final round

Comments & responses

After GTW session, we see it is difficult to reach consensus. But according to guidance, we would like to further check companies’ view or suggestion of modifications on the tentative agreements to make them more agreeable.

From moderator perspective, on one hand I observed the strong support and requirement from operators and on the other hand I observed the wide scope for study on the approach to improve MSD and feasibility of capability signalling, especially considering such many bands. Anyway the requirement for a band combinations will be specified in a release independent manner. So I would like to further suggest to consider the following proposal.

Sub-topic #5-1: Improving MSD for CA and DC

  - Improve the MSD for CA and DC in Rel-18 with a systematically study

  - How to improve MSD

  - Feasibility to introduce a capability signaling.

  - The improved MSD requirements will be introduced in a release independent manner.

Companies are invited to provide comments and responses in the following table.

| Company | Comments |
| --- | --- |
| XXX |  |
| Nokia | First of all, at least we do NOT to propose a way to improve MSD itself, but rather we are just proposing a way to distinguish UEs with small MSD and those with big MSD which just meet R4 requirements without large margin. Since MSDs for each of the UEs are affected by UE implementations. We don’t say that all the UEs shall have to have small MSD. But if UEs have smaller MSD than specified one, it is better to tell that information. We don’t have motivation to discuss mechanism to improve MSD. Hence, the discussion is quite simple. The bullets indicating “improve the MSD” or “how to improve MSD” is NOT what we proposed. Regarding signalling, again, what feasibility study do proponents have in mind? It is ok for us to say to discuss how signalling look, but studying feasibility of signaling does not reflect the reality. |
| T-Mobile USA | We agree with Nokia. RAN4 spent a lot of time developing MSD, and since MSD allows for architectural flexibility we don’t think that it would be productive to try to improve on the existing MSD. What the operators have asked for is a way to for the network to identify which UEs require little or no MSD. We propose the following: - Provide a means to identify UEs with low MSD for CA and DC in Rel-18 with a systematically study - How to define “low MSD” - Discuss if the “low MSD” capability will be per UE, or per combination - The improved MSD requirements will be introduced in a release independent manner. |
| Qualcomm | We agree with the comments from Nokia. Also, it’s not clear how RAN can agree to the moderator’s proposal since RAN is not yet discussing Rel-18 package for RAN4. Is the moderator suggesting that RAN can now make a decision on this? If not, we should focus on what RAN can make agreements on. |
| Apple | In general we are fine with moderator’s proposal. Procedure wise, we propose to study how to improve MSD first. Without knowing how much MSD can be improved on what combinations, it would be meaningless to talk about the feasibility to introduce a capability signaling. Also how network would handle UE differently simply based on the MSD difference for one particular test configuration needs to be clarified. And the concern on UE fragmentation in the field due to the potential complicated MSD capability matrices needs to be carefully considered. |
| LGE | We are fine to the moderator proposal. The Improving MSD for CA and DC will be started in Rel-18. RAN4 need to understand how to define the MSD value. Basically, RAN4 consider candidate RF architecture and need to collect the detail RF component parameters such as IIP2/IIP3/IIP4/IIP5 according to the impacted IMD order for Switch, Duplexer, Diplexer, triplexer, Quadplexer and PA. Also need to consider isolation level between Tx-Tx and Tx-Rx for antenna, filters and others. This is quite complicated to derive MSD level in small UE form factor. |
| Verizon | We also agree with Nokia! RAN4 needs to define a way for the network to identify different UE which have different MSD applied. |
| Samsung | It seems we have some different understanding. In our understanding, the outcome of study is the feasibility of defining different set of minimum requirements of MSD for certain band combination. UE is allow to follow either of these set of requirements by indicating to the network using capability singling. Based on this, I have to clarify with proponents who supports to distinguish the UEs with different de-sense capability, how to quantify the “small” and “big” MSD without study the mechanism/implementation to achieve better sensitivity performance. Even the question has been raised in the 1st round, intermediate round and also GTW session, we still did not get any answer on which band combination(s) are we talking about. Is RAN4 going to check every band combination with MSD, or (as some companies comments), is RAN4 going to check band combination with “large” MSD only. If only check large MSD, what is the definition of “large”， 20dB, 30dB? RAN can only discuss whether such task is feasible or not for RAN4 in Rel-17 timeframe once the above scope is clear, i.e., it may be feasible to task RAN4 to study only one example band combinations but certainly NOT feasible at all to task RAN4 to check every band combination with MSD value. |
| OPPO | From the comments above it seems the proponents of defining MSD signaling is just want to differentiate UEs with different MSD capabilities, or in other words the UEs that can do better than the MSD defined in RAN4. However, there is no explanation on how much MSD is considered to be good UE and how much MSD is bad UE. We agree with Samsung that the signaling of good MSD to NW should base on good MSD defined in RAN4 spec and it should have clear meaning of good MSD and perhaps testing of that. Therefore, it has to study how much MSD can be achieved at least for the good UE based on technical analysis rather than based on hope. With clear definition of band combinations, and improved requirements, NW could have clear meaning of the signaled MSD. Therefore, we are ok to further consider the MSD improvement in Rel-18 if contents is agreeable and following RAN Rel-18 discussion procedures, but for Rel-17 we see no reason to further press RAN4 without good justification and consensus. |
| DISH Network | We agree with Nokia comments. We also think that companies may have slight interpretation differencies in the topics, i.e for some “improving MSD” means simply lowering the existing MSD value and for some (including us) it means defining lower MSD value in addition to existing MSD value, and adding signaling to distinguigh these two. Hence, the working could perhaps be along the lines of T-Mo proposal, with one addition: - Provide a means to identify UEs with low MSD for CA and DC in Rel-18 with a systematically study - How to define “low MSD” - Discuss if the “low MSD” capability will be per UE, or per combination - The improved MSD requirements will be introduced in a release independent manner. - “low MSD” is defined in addition to currently defined MSD Addressing Samsung questions above: Our initial view is that at least firstly the combos with H2/IM2/IM3 MSD should be prioritized, i.e. those which have ~>15dB MSD defined. After that, more discussions could be carried out. We are fine to discuss this further and finetune. Our view and understanding is that at the end it is everyone’s benefit within the industry to have the possibility to distinguish “baseline” and “optimized” MSD value: -Operators may deploy/utilize spectrum in more efficient manner -Infra vendors have more information on each UE performance, improving scheduling possibilities -UE/chipset vendors are able to show optimized performance, which means devices work better which should be a positive marketing asset -RF FE vendors are able to benefit from their ability to optimize the performance of at least key combinations |
| ZTE | In our understanding, the signaling to distinguish different UE MSD capability is not that complex, as long as RAN4 can provide a set of delta MSD values on top of the current general MSD requirements. So the main focus here would be how many delta MSD values are needed and achievable for different UE implementation from RAN4 perspective. |
| CMCC | We support the proposal in general. Our understanding of this work is that the existing MSD requirements will be kept. And RAN4 will work on improved MSD requirements. UE can report which MSD requirements it supports to the network. More details related to how to improve the MSD, and how to define big MSD/ small MSD, and which band combination should be chosen as the example band indeed need to be further discussed. |
| Intel | We are supportive to have a discussion and high-level proposals on objectives of the work seem fine. For release independent manner – just a question how it is going to work if we define new UE capability signalling in the new Release? Same time, we do not think it is appropriate to decide on Rel-18 topic right away and we need to follow the previously agreed timelines for Rel-18 package approval. So, we cannot confirm that it should be approved as a Rel-18 objective now. |
| Skyworks | We think the goals are still too vague to even be able to gage the amount of work and depending on the mechanism (ie all MSD improved, a set of MSD…) and what is captured in the specification, the signaling may look different. Especially we do not believe that a UE level signaling is reasonable as it will unfairly penalize the UEs that supports the highest number of bands and band combinations with the highest order because it would mean that failure to meet one improved MSD out of 100s of combination case. we maintain that it would be fairly easy to “Show off” very good numbers in the spec as they only apply to 1/2/3band cases one by one but real implementations have to get every combination right which requires trade-off. In our view only some specific high values MSDs are worth revisiting and one should note that this is not only a RF front end aspect since for example transceiver impairments play a big role in some cases. |
| CHTTL | We fully support the Nokia’s comment, Nokia’s comment is the correct interpretation of the original proposal in our view, the discussion seems expended and diverse after multiple rounds. We also think T-mobile’s proposal is much aligned as the WF for the discussion, except we hope it can be start from Rel.17. Regarding Samsung’s question, I think we had provide some of the answer in the intermediate round that 2nd, 3rd order of harmonics/IMD can be the starting point, we also share the Dish’s comment that H2/IM2/IM3 MSD should be prioritized, i.e. those which have ~>15dB MSD (but note that IMD4 for PC2 might exceed 15dB MSD), we also tried to discuss this during last RAN4 meeting. |
| AT&T | We agree with the comments from Nokia. We also agree with QC that we likely cannot agree to any Rel-18 package for RAN4 at this time and support CHTTL comment about starting in Rel-17 timeframe. Perhaps, it could be feasible to perform a limited study in the Rel-17 timeframe in order to allow for the normative work in Rel-18 based on RAN approval. This would allow for early adoption of release independent requirements in the Rel-18 timeframe. Please find the proposed feasibility study based on this proposal. • Study a means to identify UEs with low MSD for CA and DC in Rel-17 with a systematically study • How to define “low MSD” • Discuss if the “low MSD” capability will be per UE, or per combination • Discuss ways to introduce the improved MSD requirements in a release independent manner. • “low MSD” would defined in addition to currently defined MSD as part of a future WI The feasibility study in the Rel-17 timeframe would focus on the combos with H2/IM2/IM3 MSD per DISH Network’s recommendation. |
| NTT DOCOMO, INC. | We agree with Nokia, DISH Network, and ZTE. The important thing is not how to improve MSD but whether a UE achieves “low MSD” or not, and “low MSD” can be defined relatively, i.e. based on delta MSD value. |
| Samsung | AT&T proposal of wording is a good direction we can go. To have more efficient study considering the limited remaining time in Rel-17, we can consider to pick out one example band combination among H2/IM2/IM3 MSD for further study (as proposed by RAN4 chair in the intermediate round). The scope in future WI phase (if approved) will not only consider H2/IM2/IM3 but also the spectrum holding situation has to be considered for RAN4 to specific the requirements. Operator demand/request with proper justifications could be one way we can consider. With above clarifications and modification, we are fine with a study item in Rel-17 (To CHTTL, I miss your comments in the intermediated round. Sorry about that) |
| Huawei, HiSilicon | Generally we support the proposal by moderator. As commented by some companies, it’s not clear even how to differentiate “small MSD” and “large MSD”. With the capability to indicate the UE have better MSD than those defined in the spec, wouldn’t the UE also need to comply with the improved MSD values? To fulfill that purpose, it cannot avoid the discussion of how to improve the MSD. As there are thousands of band combinations in the spec, the change would have huge impact. Given the current situation, we don’t believe RAN4 could have handle this resource consuming topic in Rel-17. |
| vivo | Generally, we support the moderator’s proposals. Without a reasonable analysis on the MSD improvement analysis, it is difficult to have a number for that. For the per-UE or per-band combination argument, we share skyworks’ comments that the per-band combination is only reasonable option. Regarding AT&T’s proposal to conduct certain study in Rel-17, it can also be considered, but the scope need to be carefully studied, and not sure whether we can reach consensus in such short notice. |
| Xiaomi | We don't think we can talk about MSD improvement without considering RF architecture. How much MSD improvements can be as lower MSD capability needs to be based on the reasonable assumption and analysis of RF architecture. Currently in RAN4, MSD is derived by case by case manner together with some common assumptions i.e. PCB isolation, Antenna isolation and IP2/IP3/IP4/IP5, which are based on a long time discussion. We are not clear what is the main factor /assumption to make better MSD. In addition, as commented by other companies, there are lots of work by considering the huge band combination. Thus we are fine with moderator’s proposal to have a feasibility study starting from R18. |
| Vodafone | Agree with the views expressed by Nokia and the approach proposed by T-Mobile USA. |

Summary

Moderator summarizes discussion status and provide the recommendation.

|  | Status summary |
| --- | --- |
| Sub-topic #5-1 | Companies’ view are diverse. One camp made it clear that they proposed to define a signaling to distinguish UEs with small MSD and those with big MSD. And one company clarified not to have motivation to discuss the mechanism to improve MSD. The other camp questioned how 3GPP can design the signaling without knowing how to improve and to what extent to improve MSD. And there are questions on the feasibility to improve MSD, e.g., on which band combination, all MSD or set of MSD improved, impacts of IIP2/IIP3/… PA…. And how can network use the signaling? In moderator view, it is difficult to reach a consensus but we can have further consideration and come back in next RAN. Proposal #5: Decide which alternatives to go with for improving MSD, and after make decision on one alternative RAN can task RAN4 to do study in Q3 and come back to RAN in September to decide how to handle the topic - Alternative #1: Study a means to identify UEs with low MSD for CA and DC in Rel-17 - How to define “low MSD” - Discuss if the “low MSD” capability will be per UE, or per combination - Discuss ways to introduce the improved MSD requirements in a release independent manner. - “low MSD” would defined in addition to currently defined MSD as part of a future WI - Alternative #2: Study method to improve the MSD for CA and DC - How to improve MSD - Feasibility to introduce a capability signaling. - The improved MSD requirements will be introduced in a release independent manner. |

Summary of Recommendations

Topic #1: New WI for HPUE on B5, B12 and n71

Proposal#1: New WI for power class 1 UE on a number of bands is agreeable with the following objectives: - Include power class 1 UE for band B12, B5 in the WID for fixed wireless/vehicular-mounted use cases - Include power class 1 UE for band B13, n13 and n71 in the WID with study phase for fixed wireless/vehicular-mounted use cases - Phase I: - Investigate the feasibility of filter with small duplex for B13, n13 and n71 - Investigate the PA for B13, n13 and n71 in order to specify A-MPR and MPR - Coexistence study for B13 and n13 - Phase II: if feasible, specify the requirements of power class 1 for band B13, n13 and n71. - Include an objective for release independence in WID and decide it in WI

Topic #2: LTE/NR spectrum sharing for B34/n34, B39/n39

Proposal #2: the revised WID RP-211283 is agreeable.

Topic #3: DC of x-band LTE CA + 4 bands NR CA

Proposal #3: The revised WD of RP-211393 is agreeable, with the following changes - Limit the total number of bands to 6, i.e., X =1,2 - Add the note to clarify the WID applies to NR CA including FR2 bands https://www.3gpp.org/ftp/tsg_ran/TSG_RAN/TSGR_92e/Inbox/Drafts/%5B92-e-05-Spectrum-WIs%5D/Final%20round/Revised%20WID/REV%20RP-211393%20Band%20combinations%20for%20DC_LTE%20xDL_2UL(x%3D1%2C2)%2BNR%204DL_1UL.zip

Topic #4: 6GHz unlicensed band in other countries/regions

According the comments in the last round, we observed compromise. Given one company still proposed to add the Option 2 back. So moderator propose to go back to proposal in the intermediate round to some extent.

Proposal #4-1: It is proposed to approve the WI of introduction of the 6GHz unlicensed band in other countries/regions, where the following optional solutions need be taken into account: - Option 1a: - Depending on the regulatory requirements, determine whether they can be handled by existing NS values or whether new NS values are needed. - Depending on whether existing NS values cannot be re-used and/or whether new NS values are not possible, determine whether an existing band n96 can be re-used or introduction of a new band will be needed. - Option 2: New NR band(s) could be considered for other regions with different regulatory requirements.

For European lower 6GHz unlicensed band

Proposal #4-2: it is proposed to agree on the following statement. - Unlicensed operation in the frequency range 5945 MHz to 6425 MHz in Europe shall be based on available ECC Decision (20)01 For Sub-topic #4-2a Issue #2, introduction of European lower 6GHz unlicensed band, there is no consensus. Some companies proposed to go back to RAN4, while some company propose to address this in RAN plenary. This topic needs further discussion in GTW.

Topic #5: Improving MSD for CA and DC

One camp made it clear that they proposed to define a signaling to distinguish UEs with small MSD and those with big MSD. And one company clarified not to have motivation to discuss the mechanism to improve MSD.

The other camp questioned how 3GPP can design the signaling without knowing how to improve and to what extent to improve MSD. And there are questions on the feasibility to improve MSD, e.g., on which band combination, all MSD or set of MSD improved, impacts of IIP2/IIP3/… PA…. And how can network use the signaling?

Moderator try to provide two alternative approaches for moving forward.

Proposal #5: Decide which alternatives to go with for improving MSD, and after make decision on one alternative RAN can task RAN4 to do study in Q3 and come back to RAN in September to decide how to handle the topic - Alternative #1: Study a means to identify UEs with low MSD for CA and DC in Rel-17 - How to define “low MSD” - Discuss if the “low MSD” capability will be per UE, or per combination - Discuss ways to introduce the improved MSD requirements in a release independent manner. - “low MSD” would defined in addition to currently defined MSD as part of a future WI - Alternative #2: Study method to improve the MSD for CA and DC - How to improve MSD - Feasibility to introduce a capability signaling. - The improved MSD requirements will be introduced in a release independent manner.