3GPP TSG-RAN #95-e	RP-220180

E-meeting, March 2022

Agenda Item:	9.3.2.4

Source:	OPPO, LG Electronics, Huawei, HiSilicon, Apple, ZTE, MediaTek

Title:	Discussion on SL-DRX for L2 U2N Relay

Document for:	Discussion, Decision

Introduction

In RAN#94-E, the following left issue was identified in the approved WID RP-213585.[1]

- Support of sidelink DRX for Layer-2 UE-to-Network sidelink relay operation if not done in Rel-17 [RAN2]

Note 4A: This objective is to be checked in RAN#95e.

This paper is to discuss the left issue above.

Discussion

In order to conclude on the SL-DRX applicability on ProSe traffic (which includes SL Relay as a part), during RAN2#116, the following conclusion was reached [2]

Agreements on SL-DRX for ProSe:

1:	RAN2 confirm R17 SL-DRX design can support non-relay-related ProSe communication directly without additional specific solution discussion / specification effort.

2:	RAN2 confirm the R17 SL-DRX design can support non-relay-related ProSe discovery by reusing SL default-DRX configuration used for communication without further additional specific solution discussion / specification effort.

Agreements on SL-DRX for ProSe:

1:	RAN2 confirms Rel-17 SL-DRX design can be reused for relay-related ProSe communication in layer-3 relay without additional specific solution discussion/specification effort.

2:	Keep RAN2 previous agreement (prioritize the non-relay case without consideration of relay specific optimization in Rel-17) but we’re not going to make any conclusion if L2 relay-related ProSe communication is supported or not in Rel-17 now.

3:	RAN2 confirms Rel-17 SL-DRX design can be reused for L3 relay-related ProSe discovery without additional specific solution discussion/specification effort (by applying SL default-DRX configuration). No conclusion if L2 relay-related ProSe discovery is supported or not in Rel-17 now. RAN2 does not specify any restriction now.

To summarize, the conclusion confirmed the applicability of R17 SL-DRX scheme for each case as follows

Table 1 Applicability of R17 SL-DRX scheme to each scenario of ProSe 
(discovery/communication, L2/L3 relay/non-relay)

|  | Discovery | Communication |
| --- | --- | --- |
| Non-Relay Scenario | Support | Support |
| L2 UE-to-Network Relay Scenario | No conclusion | No conclusion |
| L3 UE-to-Network Relay Scenario | Support | Support |

And therefore, the left issue is on L2 U2N Relay case only, for which

- RAN2 failed to reach consensus

- Yet would not specify any restriction either

As discussed during the meeting and supported by majority companies [3], the R17 SL-DRX scheme can be applied to L2 and L3 Relay without need to further differentiate, which is also aligned with RAN2 conclusion that no specification on restriction. In more details,

- For L2 U2N relay discovery, it can use the default DRX configuration as in non-relay-related and L3 relay-related discovery, i.e., requiring no dedicated DRX configuration. Additional DRX work on the discovery aspect can be regarded as optimization.

- For L2 U2N relay communication, except for the first DCR message which relies on the default DRX configuration as in non-relay-related and L3 relay-related communication, the other messages over sidelink between Remote-UE and Relay-UE will use sidelink unicast DRX mechanism for which Rel-17 SL DRX can be applied as well.

I.e., the SL-DRX scheme designed in R17 is sufficient to support the core functionality for sidelink power efficiency in both L2 and L3 relay architecture.

- R17 SL-DRX design is applicable to L2 UE-to-Network Relay scenario.

On the other hand, given the other objectives for R18 SL Relay enhancement (besides objective-5 which is RAN4-centric)

- Objective-1 on U2U Relay, including both L2 and L3;

- Objective-2 on service-continuity, including both intra and inter-gNB

- Objective-3 on multi-path Relay

It would be very helpful to focus on the objectives with more support and of more critical need, aiming at a manageable scope within the allocated TU capacity.

- A manageable scope for R18 SL Relay enhancement WI is important to meet the allocated TU capacity.

So it is suggested to remove the objective-4 from R18 SL relay enhancement WI.

Confirm R17 SL-DRX design can support L2 relay scenario, and thus remove objective-4 from R18 NR sidelink Relay enhancement WI.

Conclusion

We have the following observations:

Observation 1	R17 SL-DRX design is applicable to L2 UE-to-Network Relay scenario.

Observation 2	A manageable scope for R18 SL Relay enhancement WI is important to meet the allocated TU capacity.

We have the following proposals:

Proposal 1	Confirm R17 SL-DRX design can support L2 relay scenario, and thus remove objective-4 from R18 NR sidelink Relay enhancement WI.

Reference

- RP-213585, New WID on NR sidelink relay enhancements

- R2-2201970, Report of 3GPP TSG RAN WG2 meeting #116-e

- R2-2111420	[Draft] [AT116-e][703][V2X/SL] SL-DRX for ProSe (LG)	LG	Rel-17	NR_SL_enh-Core