3GPP TSG RAN meeting #97-e								RP-222151

Electronic Meeting, September 12-16, 2022

Agenda Item:	9.3.2.7

Source:	vivo

Title:	Discussions on NR NTN coverage enhancements

Document for:	Discussion, Decision

# 1	Introduction

A work item for enhancements of NR NTN was approved at RAN#94-e. The latest WID [1] states that RAN should determine in this meeting the set of RAN1 related work item objectives for NTN coverage enhancement.

RAN to determine by RAN#97 (for RAN1 items) and RAN#98 (for RAN2 items) whether the study phase has identified any need for NTN-specific coverage enhancements in Rel-18. If needed, the set of NTN-specific work item objectives will be updated.

In this contribution, we provide our views on the detailed objectives of NR NTN coverage enhancement expected to be captured in the WID according to the observations and conclusions made in RAN1 #110 meeting.

# 2	Discussion

In RAN1 #110 meeting, no conclusion has been made on whether any coverage enhancement is needed for downlink channels since different companies have different understandings on whether any power reduction from satellites is needed to solve the PFD issue. However, it’s common understanding that no enhancement for downlnk is needed at least for the case when there is no power reduction at satellite side.

No enhancement for downlink is needed when power reduction is not required for satellites.

How to resolve the PFD issue should be concluded in this meeting so that whether any downlink coverage enhancement is needed can be concluded in time from RAN1 perspective.

RAN should conclude in this meeting whether/how to solve the PFD issue and determine whether any downlink coverage enhancement is needed in NTN.

As for uplink channels, following agreements/conclusions/observations have been made.

Conclusion For Rel-18 coverage enhancement in NTN, NLOS environment is deprioritized. Agreement For NR-NTN coverage enhancement, RAN1 concludes that coverage enhancements specifically for GEO and MEO are de-prioritized in Rel-18. - Potential enhancements for LEO can also apply to GEO and MEO Agreement For NR-NTN coverage enhancement in Rel-18, link budget of parameter set-1 for LEO-1200 operating at LOS is considered as the target to evaluate whether each channel/signal with the existing specification needs to be enhanced or not. The targeted performances are used to evaluate the following services: - VoIP using AMR 4.75 kbps. - Low data rate of 3 kbps. - Potential enhancements for deployments with parameter set-1 can also apply for deployments for parameter set-2 Observation For PUCCH format 1 with parameter set-1 for LEO-1200 operating at LOS, - Five sources observed that the existing specification can meet the performance requirement Conclusion RAN1 concluded that enhancement is unnecessary for PUCCH format 1 with parameter set-1 for LEO-1200 operating at LOS, assuming -5dBi UE antenna gain. Observation For PUCCH format 3 with parameter set-1 for LEO-1200 operating at LOS, - Six sources observed that the existing specification can meet the performance requirement - One source observed that the existing specification cannot meet the performance requirement with at least 0.6 dB gap Conclusion RAN1 concluded that enhancement is unnecessary for PUCCH format 3 with parameter set-1 for LEO-1200 operating at LOS, assuming -5dBi UE antenna gain. Observation For PUCCH for Msg4 HARQ-ACK with parameter set-1 for LEO-1200 operating at LOS, - One source observed that the existing specification can meet the performance requirement - Three sources observed that the existing specification cannot meet the performance requirement with a gap of 1.8 to 6 dB. Conclusion RAN1 concluded that PUCCH for Msg4 HARQ-ACK should be enhanced to meet the coverage requirements for parameter set-1 for LEO-1200 operating at LOS, assuming -5dBi UE antenna gain. Observation For PUSCH for low data rate of 3 kbps with parameter set-1 for LEO-1200 operating at LOS, - Eight sources observed that the existing specification can meet the performance requirement Conclusion RAN1 concluded that enhancement is unnecessary for PUSCH for low data rate of 3 kbps with parameter set-1 for LEO-1200 operating at LOS, assuming -5dBi UE antenna gain. Observation For PRACH format 0 with parameter set-1 for LEO-1200 operating at LOS, - One source observed that the existing specification can meet the performance requirement - Eight sources observed that the existing specification cannot meet the performance requirement with a gap of 0.3 to 5.3 dB For PRACH format 2 with parameter set-1 for LEO-1200 operating at LOS, - Ten sources observed that the existing specification can meet the performance requirement - Two sources observed that the existing specification cannot meet the performance requirement with a gap of 1.9 to 8.8 dB For PRACH format B4 with parameter set-1 for LEO-1200 operating at LOS, - Ten sources observed that the existing specification cannot meet the performance requirement with a gap of 1.2 to 11.9 dB Note: for the observations above, some sources used 1 Rx antenna and some sources used 2 Rx antennas at the satellite. Observation For PUSCH for VoIP with parameter set-1 for LEO-1200 operating at LOS, - Six sources observed that the existing specification can meet the performance requirement with a margin of 0 to 1.7 dB - One company simulated by using 20 repetitions without DMRS bundling - Four companies simulated by using 20 repetitions with DMRS bundling - One company simulated by using 32 repetitions with DMRS bundling - Note: this is the only result using frame combining by application layer - Nine sources observed that the existing specification cannot meet the performance requirement with a gap of 0.3 to 8.6 dB - Eight companies simulated by using 20 repetitions without DMRS bundling - Seven companies simulated without frequency hopping - One company simulated by using 16 repetitions with DMRS bundling Note: for the observations above, some sources used 1 Rx antenna and some sources used 2 Rx antennas at the satellite. Observation RAN1 concluded that enhancement for PUSCH for VoIP may be needed to meet the coverage requirements for parameter set-1 for LEO-1200 operating at LOS, assuming -5dBi UE antenna gain, when DMRS bundling is not applied. Observation For Msg3 PUSCH with parameter set-1 for LEO-1200 operating at LOS, - Eight sources observed that the existing specification can meet the performance requirement - One source observed that the existing specification cannot meet the performance requirement with a gap of 1.5 dB. Conclusion RAN1 concluded that enhancement is unnecessary for Msg3 PUSCH with parameter set-1 for LEO-1200 operating at LOS, assuming -5dBi UE antenna gain.

From above agreements, conclusions and observations, it can be observed that potential enhancements for some of the uplink channels are needed. For PUSCH, joint channel estimation (JCE) specified in NR Rel-17 is needed so that the studied 4.75kbps VoNR service on PUSCH for LEO-1200 satellites with set-2 parameters can be supported. However, to support JCE of PUSCH in NTN, RAN1 should discuss whether any NTN specific updates are needed. For PRACH, it can be up to Rel-18 coverage enhancement work item to discuss how to support PRACH repetition which can be reused in NTN. For PUCCH, no enhancement is needed for normal dedicated PUCCH transmission, however, the enhancement of PUCCH for Msg4 HARQ feedback is needed.

With JCE of PUSCH, 4.75kbps VoNR in uplink can be supported even for set-2 LEO satellites.

PRACH repetition can be supported for NTN, which can be up to the discussions in Rel-18 coverage enhancement work item.

No enhancement is needed for dedicated PUCCH transmission. The enhancement of PUCCH for Msg4 HARQ feedback is needed.

According to the observations, it can be concluded that some NTN specific issues if justified can be studied to support JCE and PRACH repetition. Furthermore, PUCCH repetition for Msg4 HARQ feedback can be supported and specified, and this can be extended to any PUCCH transmissions when common PUCCH resource are used. Thereby section 4.1.1 of the WID can be updated as illustrated in table 1.

Table 1. Text proposal to section 4.1.1 of NTN WID[1]

4.1.1 Coverage enhancement The detailed objectives of coverage enhancement in this work item are as follows: - Specify following enhancement of PUCCH transmissions on common PUCCH resource - Repetition of PUCCH for msg4 HARQ-ACK feedback [RAN1, RAN2] - Specify NTN specific changes if justified for supporting following features in NTN - JCE of PUSCH transmissions in RRC connected state [RAN1] - PRACH repetition for which the NTN specific changes if needed should be based on the PRACH repetition mechnism specified in Rel-18 coverage enhancement work item. [RAN1, RAN2] Note: RAN should determine by RAN#98 whether the study phase has identified any other RAN2 items need for NTN-specific coverage enhancements in Rel-18. If needed, the set of NTN-specific work item objectives will be updated.

According to above, we have following proposal

For NR NTN uplink coverage enhancement in Rel-18, specify PUCCH repetition on common PUCCH resources, study and specify any NTN specific enhancements if justified for supporting JCE of dedicated PUSCH and PRACH repetition, and revise the WID as proposed in table 1.

# 3 Conclusion

In the previous sections we made the following observation:

Observation 1	No enhancement for downlink is needed when power reduction is not required for satellites.

Observation 2	With JCE of PUSCH, 4.75kbps VoNR in uplink can be supported even for set-2 LEO satellites.

Observation 3	PRACH repetition can be supported for NTN, which can be up to the discussions in Rel-18 coverage enhancement work item.

Observation 4	No enhancement is needed for dedicated PUCCH transmission. The enhancement of PUCCH for Msg4 HARQ feedback is needed.

Based on the observations and discussions, we propose the following:

Proposal 1	RAN should conclude in this meeting whether/how to solve the PFD issue and determine whether any downlink coverage enhancement is needed in NTN.

Proposal 2	For NR NTN uplink coverage enhancement in Rel-18, specify PUCCH repetition on common PUCCH resources, study and specify any NTN specific enhancements if justified for supporting JCE of dedicated PUSCH and PRACH repetition, and revise the WID as proposed in table 1.

# References

RP-221819, " Revised WID: NR NTN (Non-Terrestrial Networks) enhancements", Thales, RAN#96, June 6-9, 2022.