3GPP TSG RAN#97e	RP-22xxxx

Electronic Meeting, September 12 – 16, 2022

- Source:	Huawei, HiSilicon, China Mobile, China Unicom, China Telecom, Novamint, Orange, Spreadtrum, vivo, Vodafone

- Title:	New SID: Study on Ambient Power Enabled IoT

- Document for:	Information

Agenda Item:	9.1.1

3GPP™ Study Item Description

For guidance, see 3GPP Working Procedures, article 39; and 3GPP TR 21.900.
Comprehensive instructions can be found at http://www.3gpp.org/Work-Items

# Title: 	Study on Ambient Power Enabled IoT

## Acronym:

## Unique identifier:

NOTE:	For new WIs/SIs leave the Unique identifier empty but you may make a proposal for an Acronym.

If this is a RAN WID including Core and Perf. part, then Title, Acronym and Unique identifier refer to the feature WI.

Please tick (X) the applicable box(es) in the table below:

Either:

| This WID includes a Core part |  |
| --- | --- |
| This WID includes a Performance part |  |

or:

| This WID includes a Testing part |  |  |
| --- | --- | --- |
| and it addresses the following 3GPP work area: | Radio Access |  |
|  | Core Network |  |
|  | Services |  |

## 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X | X |  |  |
| No | X |  |  |  | X |
| Don't know |  |  |  | X |  |

## 2	Classification of the Work Item and linked work items

### 2.1	Primary classification

This work item is a

|  | Feature |
| --- | --- |
|  | Building Block |
|  | Work Task |
| X | Study Item |

NOTE:	Normally, Core/Perf./Testing parts in RAN WIDs are Building Blocks. Only if they are under an SA or CT umbrella, we define them as work tasks. If you are in doubt, please contact MCC.

### 2.2	Parent and child Work Items

| Parent and child Work Items |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |

NOTE:	RAN agreed some time ago, that it describes the feature WI + Core/Perf. part WI or Testing part WI in one WID. Therefore the table above should just include the feature WI Unique ID and title and Nature of relationship is "parent WID".

### 2.3	Other related Work Items and dependencies

| Other related Work Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 950004 | Study on Ambient power-enabled Internet of Things | Related SA1 study item |

NOTE:	Also related or dependent WIs in other TSGs should be indicated.

## 3	Justification

In recent years, IoT has attracted much attention in the wireless communication world. More ‘things’ are expected to be interconnected for improving productivity efficiency and increasing comforts of life. Further reduction of size, complexity, and power consumption of IoT devices can enable the deployment of tens or even hundreds of billion IoT devices for various applications and provide added value across the entire value chain. It is impossible to power all the IoT devices by battery that needs to be replaced or recharged manually, which leads to high maintenance cost, serious environmental issues, and even safety hazards for some use cases (e.g., wireless sensor in electric power and petroleum industry).

Most of the existing wireless communication devices are powered by battery that needs to be replaced or recharged manually. The automation and digitalization of various industries open numbers of new markets requiring new IoT technologies of supporting batteryless devices with no energy storage capability or devices with energy storage that do not need to be replaced or recharged manually. The form factor of such devices must be reasonably small to convey the validity of target use cases.

TR 22.840 is being developed by SA1 to capture use cases, traffic scenarios, device constraints of ambient power-enabled Internet of Things and identify new potential service requirements as well as new KPIs. SA1 are considering devices being either battery-less or with limited energy storage capability (i.e., using a capacitor) and the energy is provided through the harvesting of radio waves, light, motion, heat, or any other power source that could be seen suitable.

Considering the limited size and complexity required by practical applications for batteryless devices with no energy storage capability or devices with limited energy storage that do not need to be replaced or recharged manually, the output power of energy harvester is typically from 1µW to a few hundreds of µW. Existing cellular devices may not work well with energy harvesting due to their peak power consumption of higher than 10mW.

An example type of application in TR 22.840 is asset identification, which presently has to resort mainly to barcode and RFID in most industries. The main advantage of these two technologies is the ultra-low complexity and small form factor of the tags. However, the limited reading range of a few meters usually requires handheld scanning which leads to labor intensive and time-consuming operations, or RFID portals/gates which leads to costly deployments. Moreover, the lack of interference management scheme results in severe interference between RFID readers and capacity problems, especially in case of dense deployment. It is hard to support large-scale network with seamless coverage for RFID.

Since existing technologies cannot meet all the requirements of target use cases, a new IoT technology is recommended to open new markets within 3GPP systems, whose number of connections and/or device density can be orders of magnitude higher than existing 3GPP IoT technologies. The new IoT technology shall provide complexity and power consumption orders of magnitude lower than the existing 3GPP LPWA technologies (e.g. NB-IoT and eMTC), and shall address use cases and scenarios that cannot otherwise be fulfilled based on existing 3GPP LPWA IoT technologies.

## 4	Objective

### 4.1	Objective of SI

This study targets at a new 3GPP IoT technology, suitable for cellular deployment, which relies on ultra-low complexity devices with ultra-low power consumption for the very-low end IoT applications. The study shall provide clear differentiation, i.e. addressing use cases and scenarios that cannot otherwise be fulfilled based on existing 3GPP LPWA IoT technology e.g. NB-IoT including with reduced peak Tx power.

In terms of energy storage, the study will consider the following device characteristics:

•	Pure batteryless devices with no energy storage capability at all, and completely dependent on the availability of an external source of energy

•	Devices with limited energy storage capability  that do not need to be replaced or recharged manually, and which can manage short periods of external energy source outage.

Device categorization based on corresponding characteristics (e.g. energy source, energy storage capability, passive/active transmission, etc.) may be discussed during the study, in relation with the relevant use cases. The device’s peak power consumption shall be limited by its practical form factor for the intended use cases, and shall consider its energy source.

  - Identify the suitable deployment scenarios and their characteristics, at least for the use cases/services agreed in SA1’s “Study on Ambient power-enabled internet of Things”, comprising among at least the following aspects

    - Indoor/outdoor environment

    - Basestation characteristics, e.g. macro/micro/pico cells-based deployments

    - Connectivity topologies, including which node(s) can be activator and/or reader, e.g. basestation, UE, relay, repeater, etc.

    - TDD/FDD, and frequency bands in licensed or unlicensed spectrum

    - Coexistence with UEs and infrastructure in frequency bands for existing 3GPP technologies

    - Device originated and/or device terminated traffic assumption

NOTE: There can be more than one deployment scenario identified for a use case, and a deployment scenario may be common to more than one use case.

NOTE: Where more than one deployment scenario is identified for a use case, the trade-offs between them should also be studied.

NOTE: The study shall not prioritize deployment aspects that should be coordinated with SA, e.g. public or private network, with or without CN connection.

NOTE: A representative use case can be studied for a group of use cases that have similar requirements.

  - Formulate a set of RAN design targets based on the identified deployment scenarios and their characteristics for the relevant use cases, at least including

      - Power consumption

      - Complexity

      - Coverage

      - Data rate

      - Positioning accuracy

NOTE: The requirements from SA1 on the relevant use cases shall be taken into consideration.

NOTE: The study shall aim to provide better coverage compared to existing non-3GPP technologies for the relevant use cases.

NOTE: Other RAN design targets in relation to connection density, mobility, security, latency, reliability etc. may be discussed, if necessary for the relevant use cases.

NOTE: Detailed definitions of the RAN design targets should be discussed during the study.

  - Compare and assess the feasibility of meeting the design targets for relevant use case on the basis of the deployment scenario(s) appropriate to it, and identify assumptions on required functionality to be supported.

NOTE: This is not to require a detailed WG-level of analysis.

Note: This study shall target for an IoT segment well below the existing 3GPP IoT technologies, e.g. NB-IoT, eMTC, RedCap, etc. The study shall not aim to replace existing 3GPP LPWA technologies.

### 4.2	Objective of Performance part WI

NOTE:	Leave empty if the WI proposal does not contain a RAN performance part.

### 4.3	RAN time budget request (not applicable to RAN5 WIs/SIs)

NOTE:	For all new RAN related WIs/SIs which are not led by RAN WG5 the WI/SI rapporteur has to fill out the attached Excel table to request time budgets for corresponding RAN WG meetings.
The Excel table has to be filled out for all affected RAN WGs and up to the target date of the WI/SI.
One time unit (TU) corresponds to ~ 2 hours in the meeting.
If no TU is needed leave the field empty otherwise enter a number >0 in the field.

For revisions of already approved WI/SI descriptions: Please remove the Excel table from the WID/SID's zip file. The time budgets are already recorded. If you want to modify them, then this has to be done via the status report and not via a revised WID/SID.

If this WID is covering Core and Performance part, then please fill out one line for each part in the attached Excel table.

additional comments to the time budget request in the attached Excel table:

## 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Proposed Spec no. or series | Type (see note 1) | Title | For info at TSG# | For approval at TSG# | Remarks |
| TR 38.XXX |  | Study on Ambient Power Enabled IoT | RAN #99 | RAN #100 |  |

NOTE:	If this is a RAN WID including Core and Perf. part, then all new Core part specs have to be listed first and then all new Perf. part specs. Indicate "Core part" or "Perf. part" under Remarks for each spec.
By default a new specs can only be new for one of both parts.

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |

NOTE:	If this is a RAN WID including Core and Perf. part, then all new Core part specs have to be listed first and then all new Perf. part specs. Indicate "Core part" or "Perf. part" under Remarks for each spec.
If an existing spec is affected by both (Core part and Perf. part), then it has to be listed twice with appropriate approval dates.

## 6	Work item Rapporteur(s)

## 7	Work item leadership

Primary:	RAN

Secondary:

## 8	Aspects that involve other WGs

NOTE:	For RAN WIDs: Section 8 applies only to WGs outside of TSG RAN because RAN WG aspects have to be covered in section 4.

## 9	Supporting Individual Members

Supporting IM name

Huawei

HiSilicon

China Mobile

China Telecom

China Unicom

Novamint

Orange

Spreadtrum

vivo

Vodafone