3GPP TSG-RAN Meeting #97-e	REV RP-222513

Electronic meeting, 12 – 16 September, 2022

Source:					Huawei, Hisilicon

Title:	Discussion on intra-band EN-DC combination

Agenda item:			9.12

Document for:	Information

# Introduction

The discussion of intraBandENDC-Support can be traced back to RAN4#98e, over one year and a half ago, and the issue is still under discussion in RAN4 without a final solution.

Two cases were discussed.

- Case 3: All CCs are contiguous in DL but neither carrier is contiguous to each other in UL:

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_(n)41AB DC_(n)41CA DC_(n)41DA | DC_41A_n41A |
| DC_(n)48CA | DC_48A_n48A |
| DC_(n)48DA | DC_48A_n48A |

- Case 4: LTE and NR adjacent carriers are contiguous but carriers in LTE or NR are non-contiguous, it will has two kinds of UL ENDC configurations:

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_48A_(n)48AA | DC_(n)48AA DC_48A_n48A |

RAN4 also discussed the possible solutions to address the issue in [1][2][3], but no consensus yet.

# Discussion

The intra-band EN-DC contiguous capability is indicated by IE intraBandENDC-support in the current TS 38.331 as follows:

The IE intraBandENDC-Support in UE capability can indicate three states {contiguous, non-contiguous, both} per band combination, it can’t indicate per band combination per UL or Per DL:

| intraBandENDC-Support Indicates whether the UE supports intra-band (NG)EN-DC with only non-contiguous spectrum, or with both contiguous and non-contiguous spectrum for the (NG)EN-DC combination as specified in TS 38.101-3 [4]. If the UE does not include this field for an intra-band (NG)EN-DC combination the UE only supports the contiguous spectrum for the intra-band (NG)EN-DC combination. | BC | No | N/A | N/A |
| --- | --- | --- | --- | --- |

For IE intraBandENDC-support, default value is contiguous, and it is indicated per band combination without UL/DL distinction. If UE do not report anything, it means that UE support only contiguous ENDC for both UL and DL. If UE report ‘non-contiguous’, it supports only non-contiguous ENDC for both UL and DL. UE could not indicate contiguous or non-contiguous support in UL or DL separately.

Observation 1: A UE is not allowed to indicate intra-band EN-DC contiguous/non-contiguous capability in UL or DL separately.

In Rel-16, RAN4 introduce more intra-band contiguous and non-contiguous ENDC configurations with more than 2 carriers, as illustrated in TS38.101-3:

Table 5.3B.0-1: Intra-band contiguous EN-DC bandwidth classes

| Intra-band contiguous EN-DC bandwidth class | Number of contiguous CC |  |
| --- | --- | --- |
|  | E-UTRA | NR |
| AA | 1 | 1 |
| AB | 1 | 2 |
| CA | 2 | 1 |
| DA | 3 | 1 |

Table 5.5B.2-1: Intra-band contiguous EN-DC configurations

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | Single UL allowed |
| --- | --- | --- |
| DC_(n)5AA | DC_(n)5AA6 | Yes6 |
| DC_(n)12AA | DC_(n)12AA6 | Yes6 |
| DC_(n)38AA5 | DC_(n)38AA6 | Yes6 |
| DC_(n)41AA5 DC_(n)41AB5 DC_(n)41CA5 DC_(n)41DA5 | DC_(n)41AA | Yes3 |
| DC_(n)41AB5 DC_(n)41CA5 DC_(n)41DA5 | DC_41A_n41A | Yes3 |
| DC_(n)48AA5 | DC_(n)48AA6 | Yes6 |
| DC_(n)48CA5 | DC_(n)48AA6 DC_48A_n48A6 | Yes6 |
| DC_(n)48DA5 | DC_(n)48AA6 DC_48A_n48A6 | Yes6 |
| DC_(n)71AA2 | DC_(n)71AA | No4 |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. NOTE 2: Requirements in this specification apply for NR SCS of 15 kHz only. NOTE 3: Single UL allowed due to potential emission issues, not self-interference. NOTE 4: For UE(s) supporting dynamic power sharing it is mandatory to do dual simultaneous UL. For UE(s) not supporting dynamic power sharing single UL is allowed. Uplink DC_(n)71AA is not applicable to BCS2. NOTE 5: The minimum requirements only apply for non-simultaneous Tx/Rx between all carriers. NOTE 6: Only single switched UL is supported |  |  |

Table 5.5B.3-1: Intra-band non-contiguous EN-DC configurations

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | Single UL allowed |
| --- | --- | --- |
| DC_2A_n2A | DC_2A_n2A5 | Yes5 |
| DC_3A_n3A | DC_3A_n3A2 | Yes2 |
| DC_5A_n5A | DC_5A_n5A5 | Yes5 |
| DC_7A_n7A6 | DC_7A_n7A5 | Yes5 |
| DC_41A_n41A3 DC_41C_n41A3 DC_41D_n41A3 | DC_41A_n41A | Yes4 |
| DC_48A_n48A3 | DC_48A_n48A5 | Yes5 |
| DC_48A_(n)48AA3 | DC_(n)48AA5 DC_48A_n48A5 | Yes5 |
| DC_48A-48A_n48A3 | DC_48A_n48A5 | Yes5 |
| DC_48C_n48A3 | DC_48A_n48A5 | Yes5 |
| DC_48D_n48A3 | DC_48A_n48A5 | Yes5 |
| DC_66A_n66A | DC_66A_n66A5 | Yes5 |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. NOTE 2: Void. NOTE 3: The minimum requirements only apply for non-simultaneous Tx/Rx between all carriers. NOTE 4: Single UL allowed due to potential emission issues, not self-interference. NOTE 5: Only single switched UL is supported. NOTE 6: Requirements in this specification apply for NR SCS of 15 kHz only. NOTE 7: Single UL allowed due to potential emission issues and self-interference. |  |  |

The first we need to justify whether Case 3: All CCs are contiguous in DL but neither carrier is contiguous to each other in UL, is a valid scenario. In RAN2 TS38.306, the definition of fallback is as follows.

Fallback band combination: A Uu band combination that would result from another Uu band combination (parent band combination) by releasing at least one SCell or uplink configuration of SCell, or SCG, or SUL. A PC5 band combination that would result from another PC5 band combination (parent band combination) by releasing at least one sidelink carrier. An intra-band non-contiguous band combination is not considered to be a fallback band combination of an intra-band contiguous band combination. A fallback band combination supports the same channel bandwidth(s) for each carrier as its parent band combination(s).

For the EN-DC configuration DC_(n)41CA in downlink, the fallback combination could be DL DC_(n)41AA. While DL DC_41A_n41A breaks the rule of fallback band combination above. The divergence is that whether uplink configuration is viewed as the the fallback of downlink configuration. If the intra-band contiguity of uplink and downlink are separately configured, the uplink EN-DC configuration of DC_41A_n41A is not the fallback of DL DC_(n)41CA. Besides, RF architecture for intra-band contiguous and non-contiguous EN-DC in UL and DL can be different, and UE may support DL contiguous but UL non-contiguous. Whether or not the intra-band EN-DC contiguous configurations related to Case 3 follow the fallback rule is up to RAN2’s decision considering ‘Fallback band combination’ is defined in RAN2 spec TS38.306. From the perspective of RAN4 spec TS38.101-3, the uplink EN-DC configuration of DC_41A_n41A is included in the Table 5.5B.2-1: Intra-band contiguous EN-DC configurations. Thus, RAN4 views the uplink configuration as part of DC_(n)41CA and whether contiguous or non-contiguous EN-DC is defined only based on DL configuration.

Observation 2: Whether contiguous or non-contiguous EN-DC is defined only based on DL configuration in current RAN4 spec.

Secondly, the related configurations of Case 3 and Case 4 were raised by operators considering the employment scenarios. The problem found in RAN4 is that it’s possible that the UE capability may not aligned between UE and network, which leads to configuration failure. For example, if the UE reports ‘contiguous’ of Band combination DC_(n)41CA, it may not support DC_41A_n41A(non-contiguous) in uplink. But considering the table above defined in TS 38.101-3, the BS would get confused that whether the UE supports DC_41A_n41A(non-contiguous) in uplink or not.

The UE capability of intraBandENDC-Support with ‘both’ is not enough to indicate Case 3 and Case 4. For the problems of ambiguity on some intra-band ENDC band combinations with more than 2 carriers, justification and possible solution are necessary in RAN2 and RAN4.

- Conclusion

In this contribution we discussed on the open issues on intrabandENDC-support feature, according to the analysis, we have the following observations:

Observation 1: A UE is not allowed to indicate intra-band EN-DC contiguous/non-contiguous capability in UL or DL separately.

Observation 2: Whether contiguous or non-contiguous EN-DC is defined only based on DL configuration in current RAN4 spec.

Moderator’s summary for discussion in RAN#97e on the topic is also listed for reference [4].

# References

[1] R4-2212852, “Draft CR for 38.101-3 Rel-16 intra-band contiguous EN-DC band combination”, Google Inc., Comcast, CableLabs, Federated Wireless, RAN4#104e, August 15-26, 2022

[2] R4-2212854, “Draft CR for 38.101-3 Rel-17 intra-band contiguous EN-DC band combination”, Google Inc., Comcast, CableLabs, Federated Wireless, RAN4#104e, August 15-26, 2022

[3] R4-2212582, “Draft CR for 38.101-3 Rel-16 to correct band combination for intra-band ENDC”, Xiaomi, RAN4#104e, August 15-26, 2022

[4] RP-222594, Moderator's summary for discussion [97e-34-IntraBand-ENDC-Comb]

# : Intra-band contiguous EN-DC configurations

n RAN2 TS38.306, the definition of fallback is as follows.