3GPP TSG RAN meeting #99	RP-230075

Rotterdam, Netherlands, March 20 – 23, 2023

## Status Report to TSG

Agenda item:	9.3.1.5

| WI / SI Name | NR sidelink revolution |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NR_SL_enh2 |  |  |  |  |
| Unique ID | 940097 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-222806 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: mm/yyyy | Core part: 12/2023 | Performance part: 06/2024 | Testing part: mm/yyyy |  |
| Overall Completion level | Study Item: xx % | Core part: 55% (RAN to decide on the CA objective) | Performance Part: 0% | Testing part: xx % |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN1 |
| --- | --- | --- |
| Rapporteur | Name | Kevin Lin |
|  | Company | OPPO |
|  | Email | Kevin.Lin@oppo.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

#### 2.1.1	Agreements

NR sidelink CA operation

- On hold until further checking in RAN#99

NR Sidelink operation in FR1 unlicensed spectrum

- Progress made in RAN1#112

Conclusion

When a SL LBT failure is notified by PHY, RAN1 considers that indicating the granularity of SL LBT failure indication at BWP level, RB set level, or SL resource pool level, are all feasible. RAN1 leaves it to RAN2 to determine the granularity of SL LBT failure indication.

Agreement

The CAPC level that should be used for S-SSB transmissions:

- Option 1: CAPC value (p) should be set to 1 when UE performs Type 1 channel access procedure for S-SSB transmission

Agreement

The CAPC level that should be used for PSFCH transmission, CAPC value (p) should be set to 1 when UE performs Type 1 channel access procedure for PSFCH transmission

Agreement

The end timing for the definition of reference duration in the contention window adjustment procedure for SL-U is defined as follows:

- Option 1a

  - the end of the first slot where at least one PSSCH with ACK/NACK HARQ-ACK enabled is transmitted

  - Note, SL reference duration is not used if PSSCH with ACK/NACK HARQ-ACK enabled cannot be found in the latest COT

  - FFS: Whether to support another ending timing is FFS, e.g. for MCSt if needed

  - Whether/how to adjust CWS for groupcast option 1 NACK-only case and whether/how to define reference duration for groupcast option 1 NACK-only case can still be discussed

Agreement

A CPE can be transmitted from a CPE starting position before SL transmission for the following two options:

- Option 1: within the symbol just before the next AGC symbol

- Option 2:

  - within the symbol just before the next AGC symbol for 15 kHz SCS

  - within at most 2 symbols just before the next AGC symbol for 30 or 60 kHz SCS

- FFS applicable scenario(s), condition(s) and channel type(s) to apply Option 1 or Option 2

Agreement

- A responding UE over a shared COT can be:

  - a receiving UE, which is the target of a PSCCH/PSSCH transmission of a COT initiator

    - In the case of unicast from the COT initiator, within the same COT when the source and destination IDs contained in the COT initiator’s SCI match to the corresponding destination and source IDs relating to the same unicast at the receiving UE

    - In the case of groupcast and broadcast, when the destination ID contained in the COT initiator’s SCI match to a destination ID known at the receiving UE

  - a UE identified by ID(s), if additional IDs are supported in the COT sharing information (in addition to the source and destination IDs of the PSCCH/PSSCH transmission), when additional IDs are included in the COT sharing information from the COT initiator

    - FFS Limitations on what additional IDs may be included and how they may be indicated

Agreement

A responding UE’s SL transmission(s) within RB set(s) corresponding to a shared COT can be transmitted when the CAPC value(s) of the SL transmission(s) have an equal or smaller CAPC value than the CAPC value indicated in the COT sharing information.

Agreement

A responding UE’s PSSCH/PSCCH transmission(s) within RB set(s) corresponding to a shared COT is intended for the COT initiating UE when,

- In the case of unicast from the responding UE, when the source and destination IDs contained in the responding UE’s PSCCH/PSSCH match to the destination and source IDs from a COT initiator’s unicast transmission that included COT sharing information, or match to the additional ID(s) included in the COT sharing information (if supported)

- In the case of groupcast or broadcast from the responding UE, when the destination ID contained in the responding UE’s PSCCH/PSSCH matches to the destination ID from a COT initiator’s groupcast or broadcast transmission that included COT sharing information, or matches to the additional ID(s) included in the COT sharing information (if supported)

- FFS: all other details and additional restrictions

Agreement

For slots with 2 candidate starting symbols for a PSCCH/PSSCH transmission:

- The location of 1st starting symbol can be (pre)configured from {#0,#1,#2,#3,#4,#5,#6} per BWP

  - By default (if no (pre)configuration), the location of the 1st starting symbol is symbol#0

- The location of 2nd starting symbol is (pre-)configured from {#3,#4,#5,#6,#7} per BWP

  - It shall be configured such that within a slot, the number of symbols used for PSCCH/PSSCH transmission from 2nd starting symbol is not smaller than 6

  - It shall be configured such that within a slot, the 2nd starting symbol is later than the 1st starting symbol

- PSCCH/PSSCH transmission starting from 1st or 2nd starting symbol shall have the same ending symbol within a slot

- Note: assume symbol index in a slot starts from #0

Agreement

For interlace RB-based PSCCH/PSSCH transmission in SL-U:

- Regarding mapping between sub-channel and interlace, 1 sub-channel is defined and indexed within 1 RB set, and is periodically indexed across different RB sets within the resource pool

Agreement

Regarding PSFCH transmission with 15 kHz and 30 kHz SCS, RAN1 down-select one of followings, or support the combination of followings:

- Alt 1-1a: each PSFCH transmission occupies 1 common interlace and K3 dedicated PRB(s)

  - FFS: value of K3

- Alt 2-2a: each PSFCH transmission occupies 1 interlace, and further apply PRB-level cyclic shift

  - A UE transmits dedicated cyclic shift on K1 dedicated PRB(s) within this interlace, and transmits common cyclic shift on other PRBs of this interlace

  - FFS: value of K1

- Alt 2-3a: each PSFCH transmission occupies 1 dedicated interlace

- Alt 2-4a: each PSFCH transmission occupies 1 dedicated interlace and adopt PRB-level cyclic shift hopping as in NR-U

- Alt 3-2a: each PSFCH transmission occupies K4 dedicated PRB(s) and K2 common PRBs, where K2 common PRBs locate at the two edges of a RB set

  - FFS: value of K2, K4

- FFS: the impact of PSD limit, e.g., whether/how to handle the case when common PRB and dedicated PRB locate within the same 1 MHz bandwidth, e.g., drop common PRB or reduce power on common PRB in such case

- FFS: whether/how to reduce PAPR of PSFCH transmission

Agreement

To address PSFCH transmission dropping due to LBT failure:

- Support more than 1 PSFCH occasion per PSCCH/PSSCH transmission

  - Down-select one or support both of the followings

    - Option 1: Such PSFCH occasion(s) are (pre-)configured

    - Option 2: Such PSFCH occasion(s) are (pre-)configured and dynamically indicated

    - FFS applicable scenarios, e.g., considering the applicability of COT sharing, MCSt, etc.

  - FFS other details

Agreement

For contiguous RB-based PSCCH/PSSCH transmission in SL-U:

- Regarding mapping between sub-channel and PRBs, down-select one of the followings during RAN1#112:

  - Option 1 (sub-channel aligns with resource pool boundary): Same as in legacy NR SL, i.e., the mapping of sub-channel starts from the first PRB of the resource pool and mapped sequentially within the resource pool according to the sub-channel size

    - FFS: how to deal with the remaining PRBs, e.g. for meeting OCB requirements

Agreement

For S-SSB transmission within 1 RB set, down-select to one or more of the following for 15 kHz and 30 kHz SCS:

- Option 1-1: Using interlaced RB transmission for all of S-PSS/S-SSS/PSBCH

  - FFS: whether/how to handle the case when each interlace has only 10 PRBs in a RB set, e.g. whether 1 or 2 interlaces will be used for S-SSB

- Option 3-1: Transmit S-PSS/S-SSS/PSBCH N times by repetition in frequency domain, and there is a gap between the repetition(s) to meet OCB requirement

  - FFS details, e.g., the length of gap between repetitions is (pre-)configured or pre-defined, value of N (e.g., N=2), how to reduce PAPR, etc.

  - FFS gap of 0

- Option A: Apply OCB exemption to all of S-PSS/S-SSS/PSBCH

  - Continue studying how to meet the minimum 2 MHz requirements under 15 kHz SCS.

Working assumption

If a resource pool includes slots with 2 candidate starting symbols for a PSCCH/PSSCH transmission:

- At least for COT initiation, TBS is determined based on a reference number of symbols as follows:

  - Option 4: The reference number of symbols is determined by (pre-)configuration

  - FFS: value range

  - FFS: whether a different reference number of symbols is needed for transmission in a shared COT

Agreement

Down-select one or support both of the followings:

- Option 1: Additional candidate S-SSB occasions are excluded from resource pool

- Option 2: Additional candidate S-SSB occasions belong to resource pool

- Note: Companies are encouraged to consider aspects including: S-SSB resource overhead, Tx/Rx UE behavior (e.g., whether any blind detection in Option 2), applicable scenarios, etc.

Agreement

RAN1 further study the followings:

- Whether/how to maintain a COT when the COT contains multiple RB sets and includes S-SSB slot(s), e.g., whether to transmit S-SSB repetitions in more than one RB set, etc.

Agreement

RAN1 further study the followings:

- Whether any updates on power control are necessary considering PSD limit in unlicensed spectrum regulation.

Co-channel coexistence for LTE sidelink and NR sidelink

- Progress made in RAN1#112

Agreement

In NR SL resource (re)selection procedure, option 1 is adopted for how to determine candidate resource set for NR SL considering the LTE SL reserved resources by other LTE SL UE

- Option 1: The PHY layer of NR SL module excludes NR SL candidate resources overlapping with LTE SL reserved resources by other LTE SL UE when the SL RSRP value associated with the LTE SL reserved resources is higher than a SL RSRP threshold, where the SL RSRP threshold is derived based on LTE SL priority of other LTE SL UE and NR SL priority for NR SL transmission

  - For the list of the above initial SL RSRP threshold, down-select one of followings:

    - Alt 1: NR SL RSRP threshold list (pre)configured in a NR SL resource pool

    - Alt 2: LTE SL RSRP threshold list (pre)configured in a LTE SL resource pool

    - Alt 3: SL RSRP threshold list separately (pre)configured for dynamic resource pool sharing

    - Alt 4: SL RSRP threshold list separately (pre)configured for dynamic resource pool sharing

      - A different SL RSRP threshold list may be (pre)configured for selecting single slot resources in NR SL slots with NR PSFCH

  - For the LTE SL periodic reserved resources by other LTE SL UE,

    - For determining the above LTE SL reserved resources, the time-and-frequency resources of LTE SL reserved resources by other LTE SL UE are repeated Q times according to the LTE SL resource reservation period

      - FFS details including at least whether the formula of Q in Section 14.1.1.6 in TS 36.213 or the formula of Q in Section 8.1.4 in TS 38.214 is used

  - The PHY layer of NR module applies the above procedure in Step 6 in Section 8.1.4 of TS 38.214

    - Note: For periodic resource reservation of NR SL transmission, the PHY layer of NR SL module further excludes NR SL candidate resources of which NR SL periodic resources are overlapping with LTE SL reserved resources by other LTE SL UE when the SL RSRP value associated with the LTE SL reserved resources is higher than a SL RSRP threshold according to condition c in Step 6 in Section 8.1.4 of TS 38.214

  - Note: It is assumed that the information relevant to LTE SL reserved resources by other LTE SL UE used in the above procedure is shared from LTE SL module to NR SL module

  - FFS whether/how LTE SL RSRP is applied

Working assumption

- For dynamic resource pool sharing, select one option between 1-1 and 1-2, and select one option between 3-1 and 4-1-a, and define how the two selected options can be used in the operation:

  - Option 1-1: Avoiding PSFCH transmissions overlapping with LTE SL resources reserved for LTE SL transmissions in the time domain is performed by the UE transmitting PSFCH

    - FFS whether/how to define condition(s) under which the UE transmitting PSFCH drops its PSFCH transmission overlapping with the LTE SL resources reserved for LTE SL transmission

  - Option 1-2: Avoiding PSFCH transmissions overlapping with LTE SL resources reserved for LTE SL transmissions in the time domain is ensured by the UE transmitting PSSCH

    - FFS whether/how to define condition(s) of selecting NR SL candidate resources for NR SL PSCCH/PSSCH

  - Option 3-1: Additional PSFCH periodicity(ies) is introduced

    - Alt 3-1-a: 10 logical slots

    - Alt 3-1-b: 8 logical slots

    - Alt 3-1-c: 5 logical slots

    - FFS: Alignment between PSFCH periodicity and LTE logical subframes should be ensured by proper configuration

  - Option 4-1: PSFCH resources and LTE SL resources are TDMed

    - Alt 4-1-a: PSFCH resources is confined within the guard symbol of the LTE SL subframe in the time domain

- Note: selecting a single option is not precluded

Agreement

In NR SL resource (re)selection procedure, down-select one of followings for how to determine candidate resource set for NR SL considering the LTE SL resources selected to be used for LTE SL module’s own LTE SL transmission

- Option 1: The PHY layer of NR SL module excludes NR SL candidate resources in a NR SL slot overlapping with LTE SL resources selected to be used for LTE SL module’s own LTE SL transmission

    - For the LTE SL periodic resources selected to be used for LTE SL module’s own LTE SL transmission,

      - For determining the above LTE SL selected resources, the time-and-frequency resources of LTE SL resources selected to be used for LTE SL module’s own LTE SL transmission are repeated according to the LTE SL resource reservation period and LTE SL resource reselection count

    - The PHY layer of NR module applies the above procedure in Step 5 in Section 8.1.4 of TS 38.214

      - Note: For periodic resource reservation of NR SL transmission, the PHY layer of NR SL module further excludes NR SL candidate resources of which NR SL periodic resources are in a NR SL slot overlapping with LTE SL resources selected to be used for LTE SL module’s own LTE SL transmission according to Step 5 in Section 8.1.4 of TS 38.214

      - Note: When the PHY layer of NR module cancels the above procedure according to Step 5a in Section 8.1.4 of TS 38.214, UE selects either LTE SL transmission or NR SL transmission according to Rel-16 NR SL in-device coexistence rule

    - FFS: whether the above procedure is applied based on the priority of LTE SL transmission and the priority of NR SL transmission

    - Note: It is assumed that the information relevant to LTE SL resources selected to be used for LTE SL module’s own LTE SL transmission used in the above procedure is shared from LTE SL module to NR SL module

- Option 2: The PHY layer of NR SL module determines NR SL candidate resources regardless of whether the NR SL candidate resources are in a NR SL slot overlapping with LTE SL resources selected to be used for LTE SL module’s own LTE SL transmission

    - UE selects either LTE SL transmission or NR SL transmission according to Rel-16 NR SL in-device coexistence rule

NR sidelink enhancement in FR2 licensed spectrum

- Progress made in RAN1#112

Agreement

For sidelink beam management, RAN1 is to study

- how transmit beam(s) training and/or receive beam(s) training is performed

- whether and how spatial related information (e.g., TCI, QCL, beam ID, etc) information could be identified

- the relationship between PC5 unicast link establishment and sidelink initial beam pairing (e.g., whether initial beam pairing procedure starts before, during or after sidelink unicast link establishment procedure.)

Agreement

RAN1 is to study sidelink beam measurement and reporting schemes (e.g., periodicity, contents, container, timing, procedure, etc.) for sidelink beam maintenance.

Agreement

RAN1 is to study sidelink beam indication and switching schemes (e.g., framework, general procedure, contents, signaling, timing, etc.) for sidelink beam maintenance.

Agreement

RAN1 is to study the information related to a sidelink beam failure instance that the PHY layer provides to the MAC layer.

#### 2.1.2	Remaining Open issues

- Specify support of sidelink on unlicensed spectrum for both mode 1 and mode 2 where Uu operation for mode 1 is limited to licensed spectrum only.

  - Channel access mechanisms from NR-U shall be reused for sidelink unlicensed operation

    - Remaining details of Type 1 channel access procedure for SL

    - Remaining details of Type 2 channel access procedures for SL

    - Remaining details of contention window adjustment

    - Remaining details of cyclic prefix extension

    - Remaining details of shared channel occupancy

    - Remaining details of multi-channel access procedures

    - Remaining details of multi consecutive slots transmission (MCSt)

    - Remaining details of SCI for SL-U channel access and resource allocation

    - Whether any update necessary on SL mode 1 and mode 2 RA

  - Physical channel design framework: Required changes to NR sidelink physical channel structures and procedures to operate on unlicensed spectrum

    - Remaining details of 2 candidate starting symbols for PSCCH/PSSCH transmission

    - Remaining details of mapping between sub-channel and interlaced RBs/contiguous RBs

    - Remaining details of PSFCH transmission to meet OCB and PSD requirement

    - Remaining details of PSFCH occasion(s) design and SL-HARQ procedure

    - Remaining details of S-SSB transmission to meet OCB and PSD requirement

    - Remaining details of additional S-SSB occasions design and synchronization procedure

    - Whether any other update necessary on the existing SL procedure

- Dynamic resource pool sharing for LTE/NR V2X co-channel coexistence

  - Remaining details of NR SL module excluding candidate resources based on the shared information from LTE SL module

  - Remaining details of avoiding collisions between NR PSFCH transmission and LTE SL transmission

  - Whether or not to support NR SL resource pool configured with higher SCS, and if supported, details of reducing AGC impact to LTE SL

- Study support of beam management for NR sidelink in FR2 licensed spectrum

  - Identify signal(s)/channel(s) for NR sidelink beam management

  - Initial beam pairing solution(s) based on identified beam management signal(s)/channel(s)

  - Beam maintenance solution(s) based on identified beam management signal(s)/channel(s)

  - Beam failure detection and recovery solution(s) based on identified beam management signal(s)/channel(s)

## 2.2	RAN2

#### 2.2.1	Agreements

NR sidelink CA operation

- On hold until further checking in RAN#99

NR Sidelink operation in FR1 unlicensed spectrum

- Progress made in RAN2#121

Agreements on SL CAPC mapping table:

1: 	Mapping PQI 90/91/92/93/21/22/23/55/56/57/58 to CAPC priority class 1.

2: 	Mapping PQI 59/61 to CAPC priority class 3.

3: 	Mapping PQI 25 to CAPC priority class 2.

4: 	Mapping PQI 24/26/60 to CAPC priority class 1.

Agreement on SL CAPC mapping rule:

1: 	As in NR-U, the lowest priority CAPC of the logical channel(s) with MAC SDU multiplexed in the TB is used regardless of whether the TB also contains SL MAC CEs in addition to MAC SDUs.

2: 	For an IDLE/INACTIVE/OOC UE, if a QoS flow cannot be mapped to a non-default SLRB: 1) if the per-bearer CAPC is configured in SIB/Pre-configuration, the UE use the configured CAPC; 2) else, select CAPC of the standardized PQI which best matches the QoS characteristics of the non-standardized QoS flow based on one or more QoS characteristics. For a standardized QoS flow, CAPC is directly derived from CAPC table.

Agreement on SL consistent LBT failure detection

1: 	SL-specific consistent LBT failure detection is not performed per cast type/DST/unicast link.

Agreement on SL LCP and COT

1: 	UE can select 1/ either to do a changed-LCP, in order to satisfy the COT requirement, and to do the type-2 LBT (How to do the LCP can be decided after RAN1 agreement) 2/ or to do a legacy-LCP, e.g. using type-1, type-2 LBT. FFS on the need of assistance INFO to initiating UE. FFS on spec impact, e.g., conditions for UE to choose either solution.

Agreements on SL DRX

1: 	RAN2 deprioritizes the SL DRX enhancement on active time extension for SL LBT failure.

2:	Working assumption: Not define shared COT as SL DRX active time.

3a:	Working assumption: If multiple PSFCH occasion per PSCCH/PSSCH is supported in RAN1, if HARQ A/N is successfully transmitted in one PSFCH occasion, Rx UE starts the sl-drx-HARQ-RTT-Timer for the corresponding Sidelink process in the first slot after the end of the corresponding PSFCH transmission carrying the SL HARQ feedback.

3b: If multiple PSFCH occasion per PSCCH/PSSCH is supported in RAN1, if LBT failure happens in all PSFCH occasions, Rx UE starts the sl-drx-HARQ-RTT-Timer for the corresponding Sidelink process in the first slot after the end of the last PSFCH occasion for the SL HARQ feedback.

Agreements on SL CG

1: 	Working assumption: Not to support CG retransmission timer in SL-U.

Agreements on SL consistent LBT failure

1: 	Consistent LBT failure does not trigger the UE in RRC idle/inactive to enter RRC connected.

2:	Working assumption:

- If SL LBT failure granularity is resource pool/RB set, UE uses the MAC CE to report consistent LBT failure to the gNB.

- If SL LBT failure granularity is resource pool/RB set, the MAC CE indicates SL pool/RB set where SL consistent LBT failure was declared.

- If SL LBT failure granularity is SL BWP (and the UE declares SL consistent LBT failure, the UE declares SL RLF and the existing RRC message is used for SL RLF indication for all UC connections. FFS on the need of new cause value.

- If SL LBT failure granularity is resource pool/RB set, UE triggers SL RLF for all UC connections when UE has triggered consistent SL LBT failure in all resource pools/RB sets.

3:	Working assumption: If SL LBT failure granularity is resource pool/RB set, support the change of resource pool/RB set of which consistent SL LBT failure has not been triggered from SL consistent LBT failure by TX UE upon consistent LBT failure detection. FFS whether/how the triggered consistent SL LBT failure is cancelled.

Agreements on SL resource (re)selection

1: 	RAN2 understands UE triggers a resource (re)selection when PSSCH transmission was not performed due to an LBT failure indication from L1. FFS on MCST case. Send LS to RAN1 to check if there is any concern.

2a:	RAN2 understands L1 handles LBT impact to/from other UEs’ reserved resources in SL candidate resource selection (inter-UE case).

2b:	RAN2 will study how MAC performs resource (re)selection with the consideration of LBT impact to its own candidate resource (intra-UE case).

3:	Will send LS to RAN1 to check if there is any concern.

Agreements on reserved resource and COT

1: 	For type-1 LBT, if UE observes buffer status change after LBT initiation (i.e., before MAC PDU generation), which leads to a higher CAPC priority than the value used for type-1 LBT, it’s left to UE implementation how to handle this case (like NR-U). No spec impact.

Agreements on SL LBT failure indication granularity

1: 	SL LBT failure indication granularity is per SL RB set.

#### 2.2.2	Remaining Open issues

- Mechanism to support NR sidelink CA operation based on LTE sidelink CA operation (This part of the work is put on hold until further checking in RAN#99)

- SL-U objective:

  - Further details of CAPC determination, e.g., CAPC determination for non-standardized QoS flow

  - Consistent LBT failure detection, report and recovery mechanism;

  - Whether/how to enhance resource allocation procedure (e.g., mode-1 CG retransmission and mode-2 resource re-selection) in SL-U;

  - Study if enhancements to the SL RLF procedure are needed due to LBT failure;

  - Study if any impact is needed due to the COT sharing mechanism based on RAN1 conclusion.

- Study enhanced sidelink operation on FR2 licensed spectrum

## 2.3	RAN3

#### 2.3.1	Agreements

#### 2.3.2	Remaining Open issues

## 2.4	RAN4

#### 2.4.1	Agreements

Progress made in RAN4#106

Below UE RF agreements have been reached in WF R4-2303568.

- Issue 1-1: Support for NR sidelink CA operation

- Agreement: Put work linked with NR sidelink CA operation on hold until further checking in RAN#99.

- Issue 1-2: Support for NR sidelink operation in FR2 licensed spectrum

- Agreement: Based on WID, there is no RAN4 objectives for FR2 sidelink operation

- Issue 1-4-1: Concurrent operation on Uu@Licensed and SL@Un-licensed

- Agreement: Inter-band combinations for concurrent operation for Uu @Licensed and SL @Un-licensed are to be studied based on company inputs. In order to meet the WI objectives and schedule it is requested that combinations are decided in the next RAN4 meeting.

    - One example band combination is needed. FFS on which band combination is chosen.

- Issue 2-2: Supported un-licensed bands for NR SL-U operation

- Agreement: Focus on n46, n96, n102.

- Issue 2-3: Supported UE channel bandwidth of 20, 40, 60 and 80MHz for SL-U

- Agreement: Support channel bandwidth of 20, 40, 60 and 80 MHz i.e. same maximum bandwidth as is mandatory for NR-U.

    - Maximum channel bandwidth of 40MHz for V2X is not impacted

- Issue 2-4: Support for 10MHz channel bandwidth for SL-U

- Agreement: To put 10MHz on hold to wait for sidelink CA discussion.

- Issue 2-5: Support for 100MHz channel bandwidth for SL-U

- Agreement: Support for 100MHz is optional for SL-U

- Issue 2-10-1: Maximum output power

- Agreement: Power class 5 should be considered as the starting point for the sidelink operation in unlicensed spectrum. FFS on other power classes

- Issue 2-10-2: Default power class

- Agreement: Use power class 5 as the default power class for SL-U.

Below RRM agreements have been reached in WF R4-2303265.

- Scope of sidelink evolution

- <Agreement>:

- Identify and specify RRM requirements

  - SL unlicensed operation

  - Co-channel coexistence for LTE SL and NR SL

- Postpone the discussion on RRM impacts due to NR sidelink CA until further checking in RAN-P #99 meeting

- Note: It is a common understanding that according to WID the RRM requirements for enhanced SL operation in FR2 are out of scope of Rel-18 work

- Co-channel coexistence between LTE sidelink and NR sidelink

- <Way forward >: RAN4 to continue discussion based on RAN1 progress

#### 2.4.2	Remaining Open issues

Below UE RF open issues have been captured in WF R4-2303568.

- Issue 1-3: Single carrier SL-U operation and then add combinations for concurrent operation based on company inputs.

- Issue 1-4-2: Concurrent operation on Uu@Un-licensed and SL@Un-licensed

- Issue 1-5: Adjacent channel coexistence evaluation for NR SL-U

- Issue 1-6: Co-channel coexistence for LTE sidelink and NR sidelink

- Issue 2-1: SL-U requirements location in 38.101-1

- Issue 2-6: Channel spacing

- Issue 2-7-1: Channel raster

- Issue 2-7-2: Applicability of V2X channel raster shift

- Issue 2-7-3: Channel raster to resource element mapping

- Issue 2-7-4: Channel raster entries for each operating band

- Issue 2-8: Synchronization raster

- Issue 2-9-1: Requirements for wideband operation

- Issue 2-9-2: Intra-cell guard bands for wideband operation of SL-U

- Issue 2-11: Transmit power density requirements

Below RRM open issues have been captured in WF R4-2303265.

- FFS how to introduce identified requirements in the specification.

- RAN4 to continue discussion whether and how to define the following requirements considering SL-U operation

- UE transmit timing

- Initiation/Cease of SLSS transmission

- Selection/Reselection of V2X synchronization reference source

- L1 SL-RSRP measurement

- Congestion control measurements

- Interruption

- Scheduling availability

- Other issues due to LBT

- FFS: Conditions when SLSS occasion can be considered as available or unavailable

- FFS: UE behavior when experiencing large number of missing SLSS due to LBT failures

UE demodulation performance requirements

UE RRM performance requirements

## 2.5	RAN5

#### 2.5.1	Agreements

#### 2.5.2	Remaining Open issues

#### 2.5.3	Remaining Open issues with cross-WG dependencies

## 2.6	RAN6

#### 2.6.1	Agreements

#### 2.6.2	Remaining Open issues

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

## 3.1	SAx/CTs

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

## 4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

RAN1#112

- R1-2300296	Discussion on SL LBT failure indication and consistent SL LBT failure	OPPO

- R1-2300353	Draft reply LS on SL LBT failure indication and consistent SL LBT failure	ZTE, Sanechips

- R1-2300406	Draft reply LS on SL LBT failure indication and consistent SL LBT failure	vivo

- R1-2300407	Discussion on SL LBT failure indication and consistent SL LBT failure	vivo

- R1-2300508	Discussion of RAN2 LS on SL LBT failure indication and consistent SL LBT failure	Nokia, Nokia Shanghai Bell

- R1-2300509	[Draft] Reply LS on SL LBT failure indication and consistent SL LBT failure	Nokia, Nokia Shanghai Bell

- R1-2300612	Discussion on LS on SL LBT failure indication and consistent SL LBT failure	CATT, GOHIGH

- R1-2301122	[Draft] Reply LS on SL LBT failure indication and consistent SL LBT failure	Ericsson

- R1-2301123	Discussion on SL LBT failure indication and consistent SL LBT failure	Ericsson

- R1-2301326	Discussion on RAN2 LS on SL LBT failure indication and consistent SL LBT	Apple

- R1-2301327	Draft reply LS to RAN2 on SL LBT failure indication and consistent SL LBT	Apple

- R1-2301530	Discussion on LS on SL LBT failure indication and consistent SL LBT failure	LG Electronics

- R1-2301565	Discussion on LS on SL LBT failure indication and consistent SL LBT failure	Sharp

- R1-2301709	Discussion on RAN2 LS on SL LBT failure indication and consistent SL LBT failure	Huawei, HiSilicon

- R1-2300036	On Channel Access Mechanism for SL-U	Nokia, Nokia Shanghai Bell

- R1-2300060	Channel access mechanism for sidelink on unlicensed spectrum	FUTUREWEI

- R1-2300124	Channel access mechanism and resource allocation for sidelink operation over unlicensed spectrum	Huawei, HiSilicon

- R1-2300219	Discussion on channel access mechanism for sidelink on unlicensed spectrum	Spreadtrum Communications

- R1-2300297	On channel access mechanism and resource allocation for SL-U	OPPO

- R1-2300313	Discussion on Channel Access Mechanisms	Johns Hopkins University APL

- R1-2300354	Discussion on channel access mechanism for SL-U	ZTE, Sanechips

- R1-2300386	Sidelink channel access mechanisms	National Spectrum Consortium

- R1-2300453	Channel access mechanism for sidelink on unlicensed spectrum	vivo

- R1-2300576	Discussion on channel access mechanism for sidelink-unlicensed	xiaomi

- R1-2300680	Discussion on channel access mechanism for sidelink on unlicensed spectrum	CATT, GOHIGH

- R1-2300732	Channel access mechanism for sidelink on FR1 unlicensed spectrum	Lenovo

- R1-2300751	Discussion on channel access mechanism for SL-U	Fujitsu

- R1-2300771	Clarifications on SL-U channel access mechanism	CableLabs

- R1-2300816	Channel Access of Sidelink on Unlicensed Spectrum	NEC

- R1-2300875	Discussion on channel access mechanism for SL-unlicensed	Sony

- R1-2300948	On the Channel Access Mechanisms for SL Operating in Unlicensed Spectrum	Intel Corporation

- R1-2301002	Discussion on channel access mechanism for sidelink on unlicensed spectrum	CMCC

- R1-2301037	Considerations on channel access mechanism of SL-U	CAICT

- R1-2301045	Discussion on channel access mechanism for sidelink on unlicensed spectrum	ETRI

- R1-2301130	Channel access mechanism for SL-U	Ericsson

- R1-2301173	Sidelink channel access on unlicensed spectrum	InterDigital, Inc.

- R1-2301189	NR Sidelink Unlicensed Channel Access Mechanisms	Fraunhofer HHI, Fraunhofer IIS

- R1-2301264	On channel access mehanism for sidelink on FR1 unlicensed spectrum	Samsung

- R1-2301297	Sidelink channel access on unlicensed spectrum	Panasonic

- R1-2301306	Discussion of channel access mechanism for sidelink in unlicensed spectrum	Transsion Holdings

- R1-2301346	Discussion on channel access mechanism for sidelink on FR1 unlicensed spectrum	Apple

- R1-2301413	Channel Access Mechanism for Sidelink on Unlicensed Spectrum	Qualcomm Incorporated

- R1-2301493	Discussion on channel access mechanism in SL-U	NTT DOCOMO, INC.

- R1-2301532	Discussion on channel access mechanism for sidelink on unlicensed spectrum	LG Electronics

- R1-2301543	Discussion on channel access mechanism for NR sidelink evolution	Sharp

- R1-2301584	Discussion on channel access mechanism	MediaTek Inc.

- R1-2301627	Channel Access Mechanism for SL-U	ITL

- R1-2301734	Discussion on channel access mechanism for SL-U	WILUS Inc.

- R1-2300037	On Physical Channel Design Framework for SL-U	Nokia, Nokia Shanghai Bell

- R1-2300061	Physical channel design for sidelink on unlicensed spectrum	FUTUREWEI

- R1-2300125	Physical channel design for sidelink operation over unlicensed spectrum	Huawei, HiSilicon

- R1-2300220	Discussion on Physical channel design for sidelink on unlicensed spectrum	Spreadtrum Communications

- R1-2300298	On PHY channel designs and procedures for SL-U	OPPO

- R1-2300355	Discussion on physical layer structures and procedures for SL-U	ZTE, Sanechips

- R1-2300387	Sidelink Physical Channel Design Considerations	National Spectrum Consortium

- R1-2300454	Physical channel design framework for sidelink on unlicensed spectrum	vivo

- R1-2300577	Discussion on physical channel design for sidelink-unlicensed	xiaomi

- R1-2300681	Discussion on physical channel design framework for sidelink on unlicensed spectrum	CATT, GOHIGH

- R1-2300733	Physical layer design framework for sidelink on FR1 unlicensed spectrum	Lenovo

- R1-2300826	Discussion on physical channel design framework	NEC

- R1-2300876	Discussion on physical channel design framework for SL-unlicensed	Sony

- R1-2300901	Discussion on Physical channel design framework for sidelink on unlicensed spectrum	Hyundai Motor Company

- R1-2300949	On the Physical Layer Enhancements for SL Operating in Unlicensed Spectrum	Intel Corporation

- R1-2301003	Discussion on physical channel design framework for sidelink on unlicensed spectrum	CMCC

- R1-2301131	PHY channel design framework for SL-U	Ericsson

- Revised to R1-2301839

- R1-2301174	SL U physical layer design framework	InterDigital, Inc.

- R1-2301190	NR Sidelink Unlicensed Physical Channel Design	Fraunhofer HHI, Fraunhofer IIS

- R1-2301265	On physical channel design framework for sidelink on FR1 unlicensed spectrum	Samsung

- R1-2301298	Physical channel design for sidelink on unlicensed spectrum	Panasonic

- R1-2301307	Discussion of physical channel design for sidelink in unlicensed spectrum	Transsion Holdings

- R1-2301347	Discussion on Sidelink Physical Channel Design Framework for Unlicensed Spectrum	Apple

- R1-2301414	Physical Channel Design for Sidelink on Unlicensed Spectrum	Qualcomm Incorporated

- R1-2301459	Discussion on channel structure for SL-U	ASUSTeK

- R1-2301494	Discussion on channel design framework in SL-U	NTT DOCOMO, INC.

- R1-2301533	Discussion on physical channel design framework for sidelink on unlicensed spectrum	LG Electronics

- R1-2301544	Discussion on physical channel design framework for NR sidelink evolution on unlicensed spectrum	Sharp

- R1-2301585	Discussion on physical channel design framework	MediaTek Inc.

- R1-2301628	Physical Channel Design framework for SL-U	ITL

- R1-2301735	Discussion on PHY channel design framework for SL-U	WILUS Inc.

- R1-2300038	On Co-channel Coexistence for LTE Sidelink and NR Sidelink	Nokia, Nokia Shanghai Bell

- R1-2300062	Dynamic coexistence between NR and LTE sidelink	FUTUREWEI

- R1-2300126	Co-channel coexistence for LTE sidelink and NR sidelink	Huawei, HiSilicon

- R1-2300221	Discussion on Co-channel coexistence for LTE sidelink and NR sidelink	Spreadtrum Communications

- R1-2300299	Discussion on dynamic resource sharing in co-channel coexistence of LTE and NR SL	OPPO

- R1-2300356	Discussion on co-channel coexistence for LTE sidelink and NR sidelink	ZTE, Sanechips

- R1-2300380	Dynamic co-channel coexistence for LTE sidelink and NR sidelink	TOYOTA Info Technology Center

- R1-2300455	Co-channel coexistence for LTE sidelink and NR sidelink	vivo

- R1-2300578	Discussion on co-channel coexistence for LTE and NR sidelink	xiaomi

- R1-2300682	Discussion on co-channel coexistence for LTE sidelink and NR sidelink	CATT, GOHIGH

- R1-2300734	Discussion on co-channel coexistence for LTE sidelink and NR sidelink	Lenovo

- R1-2300817	Co-existence between LTE and NR sidelink	NEC

- R1-2300877	Discussion on co-channel coexistence for LTE sidelink and NR sidelink	Sony

- R1-2300897	Discussion on Sidelink Co-channel Coexistence	Panasonic

- R1-2300950	On the Co-channel Coexistence between LTE and NR Sidelink	Intel Corporation

- R1-2301004	Discussion on co-channel coexistence for LTE sidelink and NR sidelink	CMCC

- R1-2301046	Discussion on co-channel coexistence for LTE sidelink and NR sidelink	ETRI

- R1-2301129	Co-channel coexistence between LTE sidelink and NR sidelink	Ericsson

- R1-2301150	On sidelink co-channel coexistence issues	Mitsubishi Electric RCE

- R1-2301175		Co-channel coexistence for LTE sidelink and NR sidelink	InterDigital, Inc.

- R1-2301191		Discussion on Co-Channel Coexistence for LTE and NR Sidelink	Fraunhofer HHI, Fraunhofer IIS

- R1-2301266	On co-channel coexistence for LTE sidelink and NR sidelink	Samsung

- R1-2301304	Discussion on co-channel coexistence for LTE sidelink and NR sidelink	ASUSTeK

- R1-2301308	Discussion of co-channel coexistence for LTE sidelink and NR sidelink	Transsion Holdings

- R1-2301348	Discussion on Co-channel Coexistence for LTE Sidelink and NR Sidelink	Apple

- R1-2301415	Co-channel Coexistence Between LTE SL and NR SL	Qualcomm Incorporated

- R1-2301495	Discussion on co-channel coexistence of LTE-SL and NR-SL	NTT DOCOMO, INC.

- R1-2301534	Discussion on co-channel coexistence for LTE sidelink and NR sidelink	LG Electronics

- R1-2301545	Discussion on co-channel coexistence for LTE sidelink and NR sidelink	Sharp

- R1-2301736	Discussion on co-channel coexistence for LTE sidelink and NR sidelink	WILUS Inc.

- R1-2300039	On Beam Management for Sidelink in FR2	Nokia, Nokia Shanghai Bell

- R1-2300127	Enhanced sidelink operation on FR2 licensed spectrum	Huawei, HiSilicon

- R1-2300300	Sidelink beam management on FR2 licensed spectrum	OPPO

- R1-2300314	Discussion on Enhanced Sidelink Operation on FR2 Licensed Spectrum	Johns Hopkins University APL

- R1-2300357	Discussion on enhanced sidelink operation on FR2 licensed spectrum	ZTE, Sanechips

- R1-2300381	Discussion on enhanced sidelink operation on FR2 licensed spectrum	TOYOTA Info Technology Center

- R1-2300456	Enhanced sidelink operation on FR2 licensed spectrum	vivo

- R1-2300579	Discussion on SL beam management in FR2 licensed spectrum	xiaomi

- R1-2300683	Discussion on enhanced sidelink operation on FR2 licensed spectrum	CATT

- R1-2300735	Discussion on enhanced sidelink operation on FR2 licensed spectrum	Lenovo

- R1-2300827	Discussion on sidelink operation on FR2 licensed spectrum	NEC

- R1-2300878	Discussion on sidelink beam management on FR2 licensed spectrum	Sony

- R1-2300951	On Sidelink Operation in FR2 Licensed Spectrum	Intel Corporation

- R1-2301005	Discussion on sidelink enhancements on FR2 licensed spectrum	CMCC

- R1-2301132	Study aspects for sidelink in FR2	Ericsson

- R1-2301176	On enhanced SL FR2 operation	InterDigital, Inc.

- R1-2301192	NR Sidelink Operation in FR2	Fraunhofer HHI, Fraunhofer IIS

- R1-2301267	On enhanced SL Operation in FR2	Samsung

- R1-2301349	Discussion on Sidelink Operation on FR2	Apple

- R1-2301416	Enhanced sidelink operation on FR2 licensed spectrum	Qualcomm Incorporated

- R1-2301496	Discussion on enhanced sidelink operation on FR2 licensed spectrum	NTT DOCOMO, INC.

- R1-2301535	Discussion on enhanced sidelink operation on FR2 licensed spectrum	LG Electronics

- R1-2301546	Discussion on enhanced sidelink operation on FR2 licensed spectrum	Sharp

- R1-2301616	Discussion on SL operation on FR2	MediaTek Inc.

- R1-2301694	Discussion on evaluation methodology and inital results for beam enhancement on sidelink FR2 operation	CEWiT

- R1-2301737	Discussion on enhanced sidelink operation on FR2 licensed spectrum	WILUS Inc.

RAN2#121

- R2-2300342	Discussion on remaining issues for CAPC in SL-U	vivo

- R2-2300119	Remaining issues on CAPC for SL-U	Huawei, HiSilicon

- R2-2300120	Further discussin on SL-U	Huawei, HiSilicon

- R2-2300126	Discussion on remaining issues in SL-U	OPPO

- R2-2300136	Discussion on LBT impact in SL-U	OPPO, MediaTek Inc., Intel

- R2-2300339	Discussion on SL LBT failure for UE in RRC idle/inactive/OOC state	SHARP Corporation

- R2-2300342	Remaining issues on RAN2 aspects for SL-U	vivo

- R2-2300499	Discussion on other MAC impacts for NR SL-U	Lenovo

- R2-2300519	Aspects of channel access mechanisms	Ericsson

- R2-2300520	CAPC table and MAC multiplex rules	Ericsson

- R2-2300615	Further discussion on CAPC for SL-U	Intel Corporation

- R2-2300616	On SL-LBT aspects	Intel Corporation

- R2-2300622	CAPC and COT sharing for SL Unlicensed	InterDigital

- R2-2300623	LBT Failure for SL Unlicensed	InterDigital

- R2-2300624	Configured Grants for SL Unlicensed	InterDigital

- R2-2300645	Remaining issues on channel access priority in SL-U	Spreadtrum Communications

- R2-2300646	Consistent LBT failure handling for SL-U	Spreadtrum Communications

- R2-2300669	Consideration on CAPC for SL-U	CATT

- R2-2300670	Further Discussion on LBT	CATT

- R2-2300705	Discussion on remaining issues of CAPC in SL-U	Apple

- R2-2300706	Further discussion on MAC impacts due to LBT and COT sharing in SL-U	Apple

- R2-2300840	Discussion on channel access for sidelink operation on unlicensed spectrum	Xiaomi

- R2-2300841	Discussion on LBT for sidelink operation on unlicensed spectrum	Xiaomi

- R2-2300915	Discussion on MAC related aspects for SL-U	ZTE Corporation, Sanechips

- R2-2300916	Discussion on CAPC in SL-U	ZTE Corporation, Sanechips

- R2-2300970	Remaining issue of channel access priority for NR SL-U	Lenovo

- R2-2300971	Discussion on LBT impact to MAC for NR SL-U	Lenovo

- R2-2300989	LBT failure detection and recovery procedure for SL-U	NEC

- R2-2300994	Discussion on sidelink un-licensed	ITL

- R2-2301356	Considerations on consistent LBT failure and HARQ procedure	Nokia, Nokia Shanghai Bell

- R2-2301357	On Sidelink DRX and remaining CPAC issues	Nokia, Nokia Shanghai Bell

- R2-2301462	Considerations on resource allocation for SL-U	Nokia, Nokia Shanghai Bell

- R2-2301474	Remaining SL CAPC issues	Samsung Research America

- R2-2301475	SL resource allocation	Samsung Research America

- R2-2301542	Discussion on SL-U	ASUSTeK

- R2-2301700	Discussion on sidelink unlicensed 	Qualcomm India Pvt Ltd

- R2-2301705	Discussion on sidelink CAPC	Qualcomm India Pvt Ltd

- R2-2301719	Discussion on RAN2 aspects on SL-U	LG Electronics France

- R2-2301722	LBT impact to SL-U	MediaTek Inc.

- R2-2301723	Channel Access Priority Classes for SL-U	MediaTek Inc.

- R2-2300127	Discussion on SL-FR2 impact	OPPO

- R2-2301720	Discussion on RAN2 aspects on SL-FR2	LG Electronics France

- R2-2300394	Discussion on SL-FR2 impact to RAN2	Xiaomi

- R2-2300489	Discussion on SL-FR2	Huawei, HiSilicon

- R2-2300521	SL in FR2		Ericsson

- R2-2300617	On FR-2 aspects for SL-U	Intel Corporation

- R2-2300671	Discussion on Sidelink Operation on FR2	CATT

- R2-2300707	Discussion on RAN2 work of SL FR2	Apple

- R2-2300917	Initial consideration on sidelink FR2	ZTE Corporation, Sanechips

- R2-2301374	Discussion on RAN2 aspects for FR2 licensed spectrum	vivo

- R2-2301701	Discussion on Sidelink FR2	Qualcomm India Pvt Ltd

- R2-2301887	RAN2 Aspects of NR Sidelink Operation in FR2	Fraunhofer IIS, Fraunhofer HHI

RAN4#106

- R4-2300207	Scope and priority of NR SL evolution in Rel-18; Meta

- R4-2300974	Discussion on support of NR sidelink on unlicensed spectrum; LG Electronics

- R4-2301182		R18 workplan for NR SL evolution WI; OPPO, LGE

- R4-2301183		R18 TR38.786 v0.0.1 skeleton for SL evolution; OPPO

- R4-2301418 	RRM and RF Requirements for SL LTE-NR co-channel coexistence; Nokia

- R4-2301535 	Initial views on NR sidelink evolution; vivo

- R4-2301919 	on the SL-e; Xiaomi

- R4-2301184 	R18 NR SL evolution impacts; OPPO

- R4-2301677 	Discussion on SL-U impact on RF requirements; Nokia

- R4-2301918 	on the SL-e RF requirement; Xiaomi

- R4-2302045 	Regulations for SL on unlicensed spectrum; Huawei

- R4-2302046 	On Rel-18 NR SL evolution; Huawei

- R4-2302047 	draft LS on co-channel coexistence; Huawei

- R4-2302480 	UE RF requirements in sidelink; Qualcomm

- R4-2302839 	Topic summary for [106][146] NR_SL_enh2_UERF; LGE

- R4-2303568 	WF on NR sidelink evolution; LGE

- R4-2301418 	RRM and RF Requirements for SL LTE-NR co-channel coexistence; Nokia

- R4-2300947 	Work plan for RRM part on Rel-18 NR Sidelink Evolution; LG Electronics Inc., OPPO

- R4-2300485	Discussion on the scope of RRM requirement specifications in sidelink evolution; Intel

- R4-2300836	On SL further enhancement RRM scope; Qualcomm

- R4-2300906	Initial discussion on RRM core requirements for NR sidelink envolution; Xiaomi

- R4-2300934	Discussion on RRM impacts for R18 NR Sidelink evolution; MediaTek

- R4-2300948	Discussion on general RRM on Rel-18 NR Sidelink Evolution; LG E

- R4-2301578	Discussion on SL-U impact on RRM requirements; Nokia

- R4-2301648	Discussion on RRM core requirements for NR sidelink evolution; OPPO

- R4-2302019	Discussion on RRM impacts for R18 sidelink evolution; Huawei

- R4-2302398	Discussions on RRM requirements for sidelink evolution; Ericsson

- R4-2302785	Topic summary for [106][227] NR_SL_enh2; LGE

- R4-2303265	WF on R18 NR SL RRM requirements; LGE

10.01.2022		minor adaptations for RAN #95e

04.10.2021		minor adaptations for RAN #94e

08.08.2021		minor adaptations for RAN #93e

17.05.2021		minor adaptations for RAN #92e

28.01.2021		minor adaptations for RAN #91e

09.11.2020		minor adaptations for RAN #90e

31.08.2020		minor adaptations for RAN #89e

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template