3GPP RAN Plenary Meeting #99	                                       RP-230163

Rotterdam, Netherlands, March 20-23, 2023

Agenda item:	9.3.4.1

Source:	Rakuten Mobile, Inc., KDDI Corporation, NTT DOCOMO, INC., Softbank Corp.

Title:	Discussion on scope of the NR less than 5 MHz

Document for:	Discussion

- Background

In RP-230033 (R1-2302186), LS to RAN Plenary to ask clarification of the target transmission bandwidths for performance evaluation was agreed. Based on the LS, information about appropriate assumption based on planned deployments is requested as the input to the meeting.

In Japan, Ministry of Internal Affairs and Communications (MIC) has started a feasibility study for narrow-band LTE-Advanced system, where system bandwidth is limited to 3MHz [1]. The study is targeting 700MHz, 800MHz, 900MHz, and 1.7GHz, where 3MHz bandwidth is specified in the current version of TS 36.101 specification. Within the study, new B28 3MHz frequency allocation is discussed as in Figure 1. Although the discussion assumes allocation as LTE system, we see necessity of the refarming of the frequency range into NR systems in future.

In this contribution, we inform of the current discussion situation of the new frequency allocation and propose one of the options to be included in the reply LS.

Figure 1 Discussed Frequency range in Japan [1]

- Discussion

Domestic discussion status

As explained in the background section, Japan MIC has started feasibility study of the 3MHz BW LTE-A systems, and there is possibility of new frequency allocation of 3MHz bandwidth in band 28. The frequency allocation is intended for further utilization of low band as a component of mobile broadband communication systems. The government plans to finalize the study including co-existence study with other systems this spring. Actual radio protection requirement has not been concluded yet.

Observation 1 New frequency allocation in band 28 with 3MHz bandwidth is discussed in Japan.

Discussion plan

In case that the domestic discussion is finalized with the shortest period, detail of the requirements as LTE system on the frequency range will become clear by RAN#100 meeting.

Currently, there is no discussion about NR migration of the frequency range in the study as the frequency range is still under feasibility study as LTE systems. Discussion of the NR migration can be brought to domestic meetings based on input from a mobile operator to whom the frequency range is allocated.

Possibility of NR migration

3GPP has agreed to start WI: NR support for dedicated spectrum less than 5MHz for FR1 in Rel-18 [2], with consideration of the requirements of narrow band NR from multiple communities. n28 is included in the scope with consideration of PPDR deployment in EU in the WID.

Currently, Japanese mobile operators provide services based on both LTE and NR systems. With consideration of migration of LTE systems, a part of LTE bands is redefined as NR bands. Since use case of the new 3MHz B28 allocation is assumed to be same as other B28 frequency allocations in Japan, there is high possibility to migrate the frequency range to NR system in future.

Observation 2 The new frequency allocation in Japan leads to demand of 3MHz NR frequency band.

Hence, regarding the discussion about transmission bandwidth in RAN1, it is desirable for Japanese operators that evaluation option compatible with LTE 3 MHz bandwidth is kept as the evaluation assumption. In RAN4 #106 meeting, RAN4 has already agreed that the maximum transmission bandwidth for 3 MHz channel bandwidth is 15 PRB with 15 kHz SCS. It is reasonable to keep 15 PRB as one of the options for the evaluation assumption.

Proposal As one of the transmission bandwidths for evaluation of the 3 MHz channel bandwidth, 15 PRB should be included.

- Conclusion

In this contribution, information about new frequency allocation activity in Japan was provided. Based on that, it is requested to consider transmission bandwidth compatible to LTE 3MHz as an evaluation option.

Observations and a proposal are the followings:

Observation 1 New frequency allocation in band 28 with 3MHz bandwidth is discussed in Japan.

Observation 2 The new frequency allocation in Japan leads to demand of 3MHz NR frequency band.

Proposal As one of the transmission bandwidths for evaluation of the 3 MHz channel bandwidth, 15 PRB should be included.

References

[1] Initiation of the study on technical condition for narrow-band LTE-Advanced systems in technical condition of next generation mobile communication systems (in Japanese), available at https://www.soumu.go.jp/main_content/000851247.pdf

[2] Work Item Description, RP-222645