3GPP TSG RAN WG5#98	R5-230717

# Athens, Greece, 27 February - 3 March 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-4 | CR | 0638 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Test tolerances for newly introduced RedCap test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap_plus_ARCH-UEConTest |  |  |  |  |  | Date: |  |  | 2023-02-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Test tolerances for demod RedCap test cases needs to be specified |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added test case 5.2.1.2.1, 5.2.2.1.17, 5.2.2.2.18 to table F.1.3.2-1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The RedCap WI cannot be finalized |  |  |  |  |  |  |  |  |
| Clauses affected: |  | F.1.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### << Start of changes >>

### F.1.3.2	Measurement of Demod Performance requirements

The derivation of the test requirements for the test cases in section 5 is defined in Table F.1.3.2-1.

Table F.1.3.2-1: Derivation of Test Requirements (FR1 demodulation performance tests)

| Test |  |  | Minimum Requirement in TS 38.101-4 |  | Test Tolerance (TT) |  | Test Requirement in TS 38.521-4 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5.2.1.1.1 1 Rx FDD FR1 PDSCH performance for RedCap |  |  | SNRs as specified |  | 0.9 dB for > 10 Hz doppler 1.0 dB for 10Hz doppler |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.1.2.1 1Rx TDD FR1 PDSCH performance for RedCap |  |  | SNRs as specified |  | 0.9 dB for > 10 Hz doppler 1.0 dB for 10Hz doppler |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.2.1.1_1 2Rx FDD FR1 PDSCH mapping Type A performance - 2x2 MIMO with baseline receiver for both SA and NSA |  |  | SNRs as specified |  | 0.9 dB for > 10 Hz doppler 1.0 dB for 10Hz doppler 0.6 dB for test 1-6 0.9 dB for test 1-7 |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.2.1.1_2 2Rx FDD FR1 PDSCH Mapping Type A performance - 2x2 MIMO with enhanced receiver type X for both SA and NSA |  |  | SNRs as specified |  | 0.9 dB for > 10 Hz doppler 1.0 dB for 10Hz doppler |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.2.1.2_1 2Rx FDD FR1 PDSCH mapping Type A and CSI-RS overlapped with PDSCH performance - 2x2 MIMO with baseline receiver for both SA and NSA |  |  | SNRs as specified |  | 0.9 dB for > 10 Hz doppler 1.0 dB for 10Hz doppler |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.2.1.3_1 2Rx FDD FR1 PDSCH mapping Type B performance - 2x2 MIMO with baseline receiver for both SA and NSA |  |  | SNRs as specified |  | 0.9 dB for > 10 Hz doppler 1.0 dB for 10Hz doppler |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.2.1.4_1 2Rx FDD FR1 PDSCH Mapping Type A and LTE-NR coexistence performance - 4x2 MIMO with baseline receiver for both SA and NSA |  |  | SNRs as specified |  | 0.9 dB for > 10 Hz doppler 1.0 dB for 10Hz doppler |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.2.1.5_1 2Rx FDD FR1 PDSCH 0.001% BLER performance - 1x2 MIMO with baseline receiver for both SA and NSA |  |  | SNRs as specified |  | 0.6 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.2.1.6_1 2Rx FDD FR1 PDSCH repetitions over multiple slots performance - 2x2 MIMO with baseline receiver for both SA and NSA | SNRs as specified |  | [0.7] |  | Formula: SNR + TT |  |
| 5.2.2.1.7 2Rx FDD FR1 PDSCH Mapping Type B and UE processing capability 2 performance - 2x2 MIMO with baseline receiver for both SA and NSA |  |  | SNRs as specified |  | 1.0 dB for 10Hz doppler |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.2.1.8_1 2Rx FDD FR1 PDSCH pre-emption performance - 2x2 MIMO with baseline receiver for both SA and NSA |  |  | SNRs as specified |  | 1.0 dB for 10Hz doppler |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.2.1.9_1 2Rx FDD FR1 HST-SFN performance - 2x2 MIMO with baseline receiver for both SA and NSA |  |  | SNRs as specified |  | 0.6 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.2.1.10_1 2Rx FDD FR1 HST-DPS performance - 2x2 MIMO with baseline receiver for both SA and NSA |  |  | SNRs as specified |  | 0.6 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.2.1.11_1 2Rx FDD FR1 PDSCH Single-DCI based SDM scheme performance - 2x2 MIMO for both SA and NSA |  |  | SNRs as specified |  | 1.0 dB for 10Hz doppler |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.2.1.12_1 2Rx FDD FR1 PDSCH Multiple-DCI based transmission scheme performance - 2x2 MIMO for both SA and NSA |  |  | SNRs as specified |  | 1.0 dB for 10Hz doppler |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.2.1.13_1 2Rx FDD FR1 PDSCH Single-DCI based FDM scheme A performance - 2x2 MIMO for both SA and NSA |  |  | SNRs as specified |  | 1.0 dB for 10Hz doppler |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.2.1.14_1 2Rx FDD FR1 PDSCH Single-DCI based Inter-slot TDM scheme performance - 2x2 MIMO for both SA and NSA |  |  | SNRs as specified |  | [0.7] |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.2.1.17 2Rx FDD FR1 PDSCH performance for RedCap |  |  | SNRs as specified |  | 0.9 dB for > 10 Hz doppler 1.0 dB for 10Hz doppler |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.2.2.1_1 2Rx TDD FR1 PDSCH mapping Type A performance - 2x2 MIMO with baseline receiver for both SA and NSA |  |  | SNRs as specified |  | 0.9 dB for > 10 Hz doppler 1.0 dB for 10Hz doppler 0.9 dB for test 1-10 0.6 dB for test 1-11 |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.2.2.1_2 2Rx TDD FR1 PDSCH Mapping Type A performance - 2x2 MIMO with enhanced receiver type X for both SA and NSA |  |  | SNRs as specified |  | 0.9 dB for > 10 Hz doppler 1.0 dB for 10Hz doppler |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.2.2.2_1 2Rx TDD FR1 PDSCH mapping Type A and CSI-RS overlapped with PDSCH performance - 2x2 MIMO with baseline receiver for both SA and NSA |  |  | SNRs as specified |  | 0.9 dB for > 10 Hz doppler 1.0 dB for 10Hz doppler |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.2.2.3_1 2Rx TDD FR1 PDSCH mapping Type B performance - 2x2 MIMO with baseline receiver for both SA and NSA |  |  | SNRs as specified |  | 0.9 dB for > 10 Hz doppler 1.0 dB for 10Hz doppler |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.2.2.4_1 2Rx TDD FR1 PDSCH Mapping Type A and LTE-NR coexistence performance - 4x2 MIMO with baseline receiver for both SA and NSA |  |  | SNRs as specified |  | 0.9 dB for > 10 Hz doppler 1.0 dB for 10Hz doppler |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.2.2.5_1 2Rx FDD FR1 PDSCH 0.001% BLER performance - 1x2 MIMO with baseline receiver for both SA and NSA |  |  | SNRs as specified |  | [0.6 dB] |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.2.2.6_1 2Rx TDD FR1 PDSCH repetitions over multiple slots performance - 2x2 MIMO with baseline receiver for both SA and NSA |  |  | SNRs as specified |  | [0.6 dB] |  | Formula: SNR + TT |  |
| 5.2.2.2.7_1 2Rx TDD FR1 PDSCH Mapping Type B and UE processing capability 2 performance - 2x2 MIMO with baseline receiver for both SA and NSA |  |  | SNRs as specified |  | 1.0 dB for 10Hz doppler |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.2.2.8_1 2Rx TDD FR1 PDSCH pre-emption performance - 2x2 MIMO with baseline receiver for both SA and NSA |  |  | SNRs as specified |  | 1.0 dB for 10Hz doppler |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.2.2.9_1 2Rx TDD FR1 HST-SFN performance - 2x2 MIMO with baseline receiver for both SA and NSA |  |  | SNRs as specified |  | 0.6 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.2.2.10_1 2Rx TDD FR1 HST-DPS performance - 2x2 MIMO with baseline receiver for both SA and NSA |  |  | SNRs as specified |  | 0.6 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.2.2.11_1 2Rx TDD FR1 PDSCH Single-DCI based SDM scheme performance - 2x2 MIMO for both SA and NSA |  |  | SNRs as specified |  | 1.0 dB for 10Hz doppler |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.2.2.12_1 2Rx TDD FR1 PDSCH Multiple-DCI based transmission scheme performance - 2x2 MIMO for both SA and NSA |  |  | SNRs as specified |  | 1.0 dB for 10Hz doppler |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.2.2.13_1 2Rx TDD FR1 PDSCH Single-DCI based FDM scheme A performance - 2x2 MIMO for both SA and NSA |  |  | SNRs as specified |  | 1.0 dB for 10Hz doppler |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.2.2.14_1 2Rx TDD FR1 PDSCH Single-DCI based Inter-slot TDM scheme performance - 2x2 MIMO for both SA and NSA |  |  | SNRs as specified |  | [0.7dB] |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.2.2.18 2Rx TDD FR1 PDSCH performance for RedCap |  |  | SNRs as specified |  | 0.9 dB for > 10Hz doppler 1.0 dB for 10Hz doppler |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.3.1.1_1 4Rx FDD FR1 PDSCH mapping Type A performance - 2x4 MIMO baseline receiver for both SA and NSA |  |  | SNRs as specified |  | 0.9 dB for > 10Hz doppler 1.0 dB for 10Hz doppler |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.3.1.1_2 4Rx FDD FR1 PDSCH mapping Type A performance - 4x4 MIMO baseline receiver for both SA and NSA |  |  | SNRs as specified |  | 0.9 dB for > 10Hz doppler 1.0 dB for 10Hz doppler |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.3.1.1_4 4Rx FDD FR1 PDSCH mapping Type A performance - 4x4 MIMO with enhanced receiver type 1 for both SA and NSA |  |  | SNRs as specified |  | 0.9 dB for > 10Hz doppler 1.0 dB for 10Hz doppler |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.3.1.2_1 4Rx FDD FR1 PDSCH mapping Type A and CSI-RS overlapped with PDSCH performance - 4x4 MIMO with baseline receiver for both SA and NSA |  |  | SNRs as specified |  | 0.9 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.3.1.3_1 4Rx FDD FR1 PDSCH mapping Type B performance - 2x4 MIMO with baseline receiver for both SA and NSA |  |  | SNRs as specified |  | 1.0 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.3.1.4_1 4Rx FDD FR1 PDSCH Mapping Type A and LTE-NR coexistence performance - 4x4 MIMO with baseline receiver for both SA and NSA |  |  | SNRs as specified |  | 0.9 dB for > 10 Hz doppler 1.0 dB for 10Hz doppler |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.3.1.5_1 4Rx FDD FR1 PDSCH 0.001% BLER performance - 1x4 MIMO with baseline receiver for both SA and NSA |  |  | SNRs as specified |  | [0.6 dB] |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.3.1.6_1 4Rx FDD FR1 PDSCH repetitions over multiple slots performance - 2x4 MIMO with baseline receiver for both SA and NSA |  |  | SNRs as specified |  | [0.7dB] |  | Formula: SNR + TT |  |
| 5.2.3.1.7_1 4Rx FDD FR1 PDSCH Mapping Type B and UE processing capability 2 performance - 2x4 MIMO with baseline receiver for both SA and NSA |  |  | SNRs as specified |  | 1.0 dB for 10Hz doppler |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.3.1.8_1 4Rx FDD FR1 PDSCH pre-emption performance - 2x4 MIMO with baseline receiver for both SA and NSA |  |  | SNRs as specified |  | 1.0 dB for 10Hz doppler |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.3.1.9_1 4Rx FDD FR1 HST-SFN performance - 2x4 MIMO with baseline receiver for both SA and NSA |  |  | SNRs as specified |  | 0.6 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.3.1.10_1 4Rx FDD FR1 HST-DPS performance - 2x4 MIMO with baseline receiver for both SA and NSA |  |  | SNRs as specified |  | 0.6 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.3.1.11_1 4Rx FDD FR1 PDSCH Single-DCI based SDM scheme performance - 2x4 MIMO for both SA and NSA |  |  | SNRs as specified |  | 1.0 dB for 10Hz doppler |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.3.1.12_1 4Rx FDD FR1 PDSCH Multiple-DCI based transmission scheme performance - 2x4 MIMO for both SA and NSA |  |  | SNRs as specified |  | 1.0 dB for 10Hz doppler |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.3.1.13_1 4Rx FDD FR1 PDSCH Single-DCI based FDM scheme A performance - 2x4 MIMO for both SA and NSA |  |  | SNRs as specified |  | 1.0 dB for 10Hz doppler |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.3.1.14_1 4Rx FDD FR1 PDSCH Single-DCI based Inter-slot TDM scheme performance - 2x4 MIMO for both SA and NSA |  |  | SNRs as specified |  | [0.7dB] |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.3.2.2_1 4Rx TDD FR1 PDSCH mapping Type A and CSI-RS overlapped with PDSCH performance - 2x4 MIMO with baseline receiver for both SA and NSA |  |  | SNRs as specified |  | 0.9 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.3.2.3_1 4Rx TDD FR1 PDSCH mapping Type B performance - 2x4 MIMO with baseline receiver for both SA and NSA |  |  | SNRs as specified |  | 1.0 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.3.2.4_1 4Rx TDD FR1 PDSCH Mapping Type A and LTE-NR coexistence performance - 4x4 MIMO with baseline receiver for both SA and NSA |  |  | SNRs as specified |  | 1.0 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.3.2.5_1 4Rx TDD FR1 PDSCH 0.001% BLER performance - 1x4 MIMO with baseline receiver for both SA and NSA |  |  | SNRs as specified |  | [0.6 dB] |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.3.2.6_1 4Rx TDD FR1 PDSCH repetitions over multiple slots performance - 2x4 MIMO with baseline receiver for both SA and NSA |  |  | SNRs as specified |  | [0.7dB] |  | Formula: SNR + TT |  |
| 5.2.3.2.7_1 4Rx TDD FR1 PDSCH Mapping Type B and UE processing capability 2 performance - 2x4 MIMO with baseline receiver for both SA and NSA |  |  | SNRs as specified |  | 1.0 dB for 10Hz doppler |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.3.2.8_1 4Rx TDD FR1 PDSCH pre-emption performance - 2x4 MIMO with baseline receiver for both SA and NSA |  |  | SNRs as specified |  | 1.0 dB for 10Hz doppler |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.3.2.9_1 4Rx TDD FR1 HST-SFN performance - 2x4 MIMO with baseline receiver for both SA and NSA |  |  | SNRs as specified |  | 0.6 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.3.2.10_1, 4Rx TDD FR1 HST DPS performance - 2x4 MIMO with baseline receiver for both SA and NSA |  |  | SNRs as specified |  | 0.6 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.3.2.11_1 4Rx TDD FR1 PDSCH Single-DCI based SDM scheme performance - 2x2 MIMO for both SA and NSA |  |  | SNRs as specified |  | 1.0 dB for 10Hz doppler |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.3.2.12_1 4Rx TDD FR1 PDSCH Multiple-DCI based transmission scheme performance - 2x2 MIMO for both SA and NSA |  |  | SNRs as specified |  | 1.0 dB for 10Hz doppler |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.3.2.13_1 4Rx TDD FR1 PDSCH Single-DCI based FDM scheme A performance - 2x2 MIMO for both SA and NSA |  |  | SNRs as specified |  | 1.0 dB for 10Hz doppler |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.3.2.14_1 4Rx TDD FR1 PDSCH Single-DCI based Inter-slot TDM scheme performance - 2x2 MIMO for both SA and NSA |  |  | SNRs as specified |  | [0.7dB] |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.2A.2.1.1 2Rx Normal PDSCH Demodulation Performance for CA (2DL CA) |  |  | SNRs as specified |  | 1.0 dB for 10Hz doppler |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.2A.2.1.2 2Rx Normal PDSCH Demodulation Performance for CA (3DL CA) |  |  | SNRs as specified |  | 1.0 dB for 10Hz doppler |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.2A.2.1.3 2Rx Normal PDSCH Demodulation Performance for CA (4DL CA) |  |  | SNRs as specified |  | 1.0 dB for 10Hz doppler |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.2A.2.2.1 2Rx PDSCH Demodulation Performance for CA with power imbalance (2DL CA) |  |  | Power level as specified |  | No TT added |  | T-put limit unchanged |  |
| 5.2A.2.4.1 2RX PDSCH Demodulation Performance for HST-SFN CA |  |  | SNR as specified |  | 0.6 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.2A.2.5.1 2RX PDSCH Demodulation Performance for HST-DPS CA |  |  | SNR as specified |  | 0.6 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.2A.3.1.1 4Rx Normal PDSCH Demodulation Performance for CA (2DL CA) |  |  | SNRs as specified |  | 0.9 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.2A.3.1.2 4Rx Normal PDSCH Demodulation Performance for CA (3DL CA) |  |  | SNRs as specified |  | 0.9 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.2A.3.1.3 4Rx Normal PDSCH Demodulation Performance for CA (4DL CA) |  |  | SNRs as specified |  | 0.9 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.2A.3.2.1 4Rx PDSCH Demodulation Performance for CA with power imbalance (2DL CA) |  |  | SNRs as specified |  | 0.9 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.2A.3.4.1 4Rx PDSCH Demodulation Performance for HST-SFN CA |  |  | SNRs as specified |  | 0.6 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.2A.3.5.1 4Rx PDSCH Demodulation Performance for HST-DPS CA |  |  | SNRs as specified |  | 0.6 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.2A.3A.1.1 2Rx-4Rx Normal PDSCH Demodulation Performance for CA (2DL CA) |  |  | SNRs as specified |  | 2Rx CC: 1.0 dB for 10Hz doppler 4Rx CC: 0.9dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.2A.3A.1.2 2Rx-4Rx Normal PDSCH Demodulation Performance for CA (3DL CA) |  |  | SNRs as specified |  | 2Rx CC: 1.0 dB for 10Hz doppler 4Rx CC: 0.9dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.2A.3A.1.3 2Rx-4Rx Normal PDSCH Demodulation Performance for CA (3DL CA) |  |  | SNRs as specified |  | 2Rx CC: 1.0 dB for 10Hz doppler 4Rx CC: 0.9dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.3.2.1.1 2Rx FDD FR1 PDCCH 1 Tx antenna performance for both SA and NSA |  |  | SNRs as specified |  | 0.9 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.3.2.1.2 2Rx FDD FR1 PDCCH 2 Tx antenna performance for both SA and NSA |  |  | SNRs as specified |  | 1.0 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.3.2.1.3 2Rx FDD FR1 PDCCH 1 Tx antenna performance for power saving |  |  | SNRs as specified |  | 0.9 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.3.2.2.1 2Rx TDD FR1 PDCCH 1 Tx antenna performance for both SA and NSA |  |  | SNRs as specified |  | 0.9 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.3.2.2.2 2Rx TDD FR1 PDCCH 2 Tx antenna performance for both SA and NSA |  |  | SNRs as specified |  | 1.0 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.3.2.2.3 2Rx TDD FR1 PDCCH 1 Tx antenna performance for power saving |  |  | SNRs as specified |  | 0.9 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.3.3.1.1 4Rx FDD FR1 PDCCH 1 Tx antenna performance for both SA and NSA |  |  | SNRs as specified |  | 0.9 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.3.3.1.2 4Rx FDD FR1 PDCCH 2 Tx antenna performance for both SA and NSA |  |  | SNRs as specified |  | 1.0 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.3.3.1.3 4Rx FDD FR1 PDCCH 1 Tx antenna performance power saving |  |  | SNRs as specified |  | 0.9 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.3.3.2.1 4Rx TDD FR1 PDCCH 1 Tx antenna performance for both SA and NSA |  |  | SNRs as specified |  | 0.9 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.3.3.2.2 4Rx TDD FR1 PDCCH 2 Tx antenna performance for both SA and NSA |  |  | SNRs as specified |  | 1.0 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 11.1.2.1.1_1 2Rx FR1 PSSCH performance - single active PSSCH link |  |  | SNR as specified |  | 0.8 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 11.1.3.1.1_1 2Rx FR1 PSCCH performance - single active PSSCH link |  |  | SNR as specified |  | 0.8 dB |  | Formula: SNR + TT missing detection probability limit unchanged |  |
| 11.1.5.1.1_1 2Rx FR1 PSCCH performance - single active PSSCH link |  |  | SNR as specified |  | 0.8 dB |  | Formula: SNR + TT missing detection probability limit unchanged |  |
| 11.1.6.1.1_1 2Rx FR1 Power imbalance performance - two active PSSCH link |  |  | SNR as specified |  | 0.6 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 11.1.7.1.1_1 2Rx FR1 HARQ buffer soft combining performance - maximum number of HARQ processes |  |  | SNR as specified |  | 0.6 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 11.1.8.1.1_1 2Rx FR1 PSCCH decoding capability - maximum number of received PSCCHs |  |  | sidelink power |  | 0 dB |  | sidelink power unchanged missing detection probability limit unchanged |  |
| 11.1.9.1.1_1 2Rx FR1 PSFCH decoding capability - maximum number of received PSFCHs |  |  | sidelink power |  | 0 dB |  | sidelink power unchanged missing detection probability limit unchanged |  |
| 5.5.1 FR1 Sustained downlink data rate performance for single carrier |  |  | Downlink power -112 dBm/Hz |  | 0 dB |  | Formula: Downlink power + TT T-put limit unchanged |  |
| 5.5A.1.1 FR1 SDR performance for CA (2DL CA) | Same as 5.5.1 |  | Same as 5.5.1 |  | Same as 5.5.1 |  |
| 5.5A.1.2 FR1 SDR performance for CA (3DL CA) | Same as 5.5.1 |  | Same as 5.5.1 |  | Same as 5.5.1 |  |
| 5.5A.1.3 FR1 SDR performance for CA (4DL CA) | Same as 5.5.1 |  | Same as 5.5.1 |  | Same as 5.5.1 |  |
| 5.5A.1.4 FR1 SDR performance for CA (5DL CA) | Same as 5.5.1 |  | Same as 5.5.1 |  | Same as 5.5.1 |  |

#### << End of changes >>