3GPP TSG RAN Meeting #99									RP-230634

Rotterdam, Netherlands, March 20-23, 2023

## Status Report to TSG

Agenda item:			10.2.2

| WI / SI Name | Perf. part: NB-IoT (Narrowband IoT)/eMTC (enhanced Machine Type Communication) core & performance requirements for Non-Terrestrial Networks (NTN) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: No | Performance part: Yes |  | Testing part: No |
| Acronym | LTE_NBIoT_eMTC_NTN_req-Perf |  |  |  |  |
| Unique ID | 950274 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-223437 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: | Core part: 12/2022 | Performance part: 06/2023 | Testing part: |  |
| Overall Completion level | Study Item: | Core part: Overall: 100% | Performance Part: Overall: 45% | Testing part: |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | TSG RAN WG4 |
| --- | --- | --- |
| Rapporteur | Name | Hsuanli Lin |
|  | Company | MediaTek Inc. |
|  | Email | hsuanli.lin@mediatek.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

#### N/A

## 2.2	RAN2

#### N/A

## 2.3	RAN3

#### N/A

## 2.4	RAN4

#### 2.4.1	Agreements

RAN4 #106 (Feb. 2023)

General

- Work Plan for performance part is agreed in [1], including the following aspects

  - SAN RF conformance testing

  - RRM performance requirements

  - UE demodulation requirements

  - SAN demodulation requirements

SAN RF conformance testing

- The proposed TS 36.181 skeleton in R4-2302451 agreed.

- For SAN RF conformance, agreements were captured in the following documents:

  - “WF for IoT over NTN SAN RF conformance requirements” in [2].

  - “WF for MU assessment for IoT over NTN SAN RF conformance” in [3].

// Agreements in “WF for IoT over NTN SAN RF conformance requirements [3]”

Issue 2-1 IoT level for NB-IoT over NTN

- Agreement: to agree the IoT level as 31dBc for LEO dynamic rang requirements.

Issue 2-2-1 test configuration

- Proposals

  - Proposal 1: to use ETC1 and ETC6 for IoT over NTN SAN test configuration and FFS for ETC7. [ZTE, R4-2301738]

- Agreement: Proposal 1 agreed

Issue 2-2-2 test case

- Agreement: Using Rel-17 NTN SAN approach as starting point.

Issue 2-2-3 test model for for eMTC over NTN

- Proposals

  - Proposal 1: for eMTC over NTN, to agree with test models as above table 3 and reuse data content in clause 6.1.2 in TS36.141 for IoT NTN SAN conformance testing for both type 1-H and type 1-O.  [ZTE, R4-2301738]

- Agreement: Proposal 1 agreed.

Issue 2-2-4 test model for for NB-IoT over NTN

- Proposals

  - Proposal 1: for NB-IoT over NTN, to reuse the test model and data content in clause 6.1..3 and 6.1.4 for IoT over NTN SAN conformance testing. [ZTE, R4-2301738]

- Agreement:

  - Proposal 1 agreed.

Issue 2-3 measurement setup

- Agreement:

  - For measurement setup for SAN type 1-H and 1-O, the existing generic setup in TS 38.181 could be reused for IoT over NTN SAN

  - TS 36.181 shall capture same DUT size clarification for the OTA conformance testing requirements applicability, same as captured in TS 38.181.

// Agreements in “WF for MU assessment for IoT over NTN SAN RF conformance [4]”

All the MU and TT values agreed below may be further adjusted in case of any technical are identified.

  - TX MU

The following is MU values are agreed for TX:

Table 1: Maximum Test System uncertainty for conducted transmitter tests

| Clause | Maximum Test System Uncertainty |
| --- | --- |
| 6.2 SAN output power | ±0.7 dB, f ≤ 3 GHz ±1 dB for standalone NB-IoT |
| 6.3 Output power dynamics | ±0.4 dB |
| 6.5.2 Frequency error | ±12 Hz |
| 6.5.3 EVM | ±1% |
| 6.5.5 DL RS power | ±0.8 dB, f ≤ 3 GHz |
| 6.6.2 Occupied bandwidth | 1.4 MHz channel BW: 30 kHz |
| 6.6.3 Adjacent Channel Leakage power Ratio (ACLR) | ±0.8 dB |
| 6.6.4 Operating band unwanted emissions | ±1.5 dB, f ≤ 3 GHz |
| [6.6.5.5.1.1] TX spurious emissions: General requirements | 9 kHz < f ≤ 4 GHz: ±2 dB 4 GHz < f ≤ 15 GHz: ±4 dB |
| [6.6.5.5.1.2] TX spurious emissions: Protection of SAN receiver | ±3 dB |
| NOTE: Test system uncertainty values are applicable for normal condition unless otherwise stated. |  |

Table 2: Maximum OTA Test System uncertainty for OTA transmitter tests

| Clause | Maximum OTA Test System uncertainty |
| --- | --- |
| 9.2 Radiated transmit power | [±1.1] dB, f ≤ 3 GHz ±[1 ÷ 1.4] dB for standalone NB-IoT |
| 9.3 OTA SAN output power | [±1.4] dB, f ≤ 3 GHz |
| 9.4.3 OTA total power dynamic range | [±0.4] dB |
| 9.6.2 OTA frequency error | [±12] Hz |
| 9.6.3 OTA EVM | [±1] % |
| 9.6.5 OTA DL RS power | [±1.1] dB |
| 9.7.2 OTA occupied bandwidth | 1.4 MHz channel BW: 30 kHz |
| 9.7.3 OTA ACLR | [±1] dB, f ≤ 3 GHz |
| 9.7.4 OTA operating band unwanted emissions | Absolute power: [±1.8] dB, f ≤ 3 GHz |
| [9.7.5.5.1] OTA TX spurious emissions: General requirements | [±2.3] dB, 30 MHz < f ≤ 6 GHz [±4.2] dB, 6 GHz < f ≤ 15 GHz |
| NOTE: Test system uncertainty values are applicable for normal condition unless otherwise stated. |  |

  - RX MU

The following is MU values are agreed for RX:

Table 3: Maximum Test System Uncertainty for conducted receiver tests

| Clause | Maximum Test System Uncertainty | Derivation of Test System Uncertainty |
| --- | --- | --- |
| 7.2 Reference sensitivity level | ±0.7 dB, f ≤ 3 GHz For NB-IoT: ±1 dB, f ≤ 3 GHz |  |
| 7.3 Dynamic range | ±0.3 dB | Overall system uncertainty for static conditions is equal to signal-to-noise ratio uncertainty. Signal-to-noise ratio uncertainty ±0.3 dB Definitions of signal-to-noise ratio, AWGN and related constraints are given in Table 4.1.2-3. |
| 7.4.1 Adjacent channel selectivity | ±1.4 dB, f ≤ 3 GHz | Overall system uncertainty comprises three quantities: 1. Wanted signal level error 2. Interferer signal level error 3. Additional impact of interferer ACLR Items 1 and 2 are assumed to be uncorrelated so can be root sum squared to provide the ratio error of the two signals. The interferer ACLR effect is systematic, and is added aritmetically. Test System uncertainty = [SQRT (wanted_level_error2 + interferer_level_error2)] + ACLR effect. f ≤ 3 GHz Wanted signal level ± 0.7dB Interferer signal level ± 0.7dB |
| 7.5 Out-of-band blocking | fwanted ≤ 3GHz 1 MHz < finterferer ≤ 3 GHz: ±1.3 dB 3 GHz < finterferer ≤ 4.2 GHz: ±1.5 dB 4.2GHz < finterferer ≤ 12.75 GHz: ±3.2 dB | Overall system uncertainty can have these contributions: 1. Wanted signal level error 2. Interferer signal level error 3. Interferer ACLR 4. Interferer broadband noise Items 1 and 2 are assumed to be uncorrelated so can be root sum squared to provide the ratio error of the two signals. The Interferer ACLR or Broadband noise effect is systematic, and is added aritmetically. Test System uncertainty = [SQRT (wanted_level_error2 + interferer_level_error2)] + ACLR effect + Broadband noise effect. Out of band blocking, using CW interferer: Wanted signal level: ± 0.7 dB f ≤ 3 GHz Interferer signal level:± 1.0 dB up to 3 GHz Interferer ACLR not applicable. Impact of interferer Broadband noise 0.1 dB. |
| 7.8 In-channel selectivity | ±1.4 dB, f ≤ 3 GHz | Overall system uncertainty comprises three quantities: 1. Wanted signal level error 2. Interferer signal level error 3. Additional impact of interferer leakage Items 1 and 2 are assumed to be uncorrelated so can be root sum squared to provide the ratio error of the two signals. The interferer leakage effect is systematic, and is added aritmetically. Test System uncertainty = [SQRT (wanted_level_error2 + interferer_level_error2)] + leakage effect. f ≤ 3 GHz: Wanted signal level ± 0.7dB Interferer signal level ± 0.7dB |
| NOTE 1: Unless otherwise noted, only the Test System stimulus error is considered here. The effect of errors in the throughput measurements due to finite test duration is not considered. NOTE 2: The Test equipment ACLR requirement for a specified uncertainty contribution is calculated as below: a) The wanted signal to noise ratio for Reference sensitivity is calculated based on a 5 dB noise figure b) The same wanted signal to (noise + interference) ratio is then assumed at the desensitisation level according to the ACS test conditions c) The noise is subtracted from the total (noise + interference) to compute the allowable BS adjacent channel interference. From this an equivalent BS ACS figure can be obtained d) The contribution from the Test equipment ACLR is calculated to give a 0.4 dB additional rise in interference. This corresponds to a Test equipment ACLR which is 10.2 dB better than the BS ACS e) This leads to the following Test equipment ACLR requirements for the interfering signal: Adjacent channel Selectivity E-UTRA 1.4 MHz channel bandwidth: 56 dB Stand-alone NB-IoT 200 kHz channel bandwidth: 56 dB |  |  |

Table 4: Maximum OTA Test System uncertainty for FR1 OTA receiver tests

| Clause | Maximum OTA Test System uncertainty |
| --- | --- |
| 10.2 OTA sensitivity | [±1.3] dB, f ≤ 3 GHz |
| 10.3 OTA reference sensitivity level | [±1.3] dB, f ≤ 3 GHz |
| 10.4 OTA dynamic range | [±0.3] dB |
| 10.5.1 OTA adjacent channel selectivity | [±1.7] dB, f ≤ 3 GHz |
| 10.6 OTA out-of-band blocking | fwanted ≤ 3 GHz: [±2] dB, finterferer ≤ 3 GHz [±2.1] dB, 3 GHz < finterferer ≤ 6 GHz [±3.5] dB, 6 GHz < finterferer ≤ 12.75 GHz |
| 10.9 OTA in-channel selectivity | [±1.7] dB, f ≤ 3 GHz |
| NOTE: Test system uncertainty values are applicable for normal condition unless otherwise stated. |  |

  - TX TT

The following is TT values are agreed for TX:

Table 5: Derivation of test requirements for conducted transmitter tests

| Test | Minimum requirement in TS 36.108 [2] | Test Tolerance (TT) | Test requirement in the present document |
| --- | --- | --- | --- |
| 6.2 SAN output power | clause 6.2 | 0.7 dB 1 dB for standalone NB-IoT | Formula: Upper limit + TT, Lower limit - TT |
| 6.3.3 Total power dynamic range | clause 6.3.3 | 0.4 dB | Formula: Total power dynamic range – TT |
| 6.5.2 Frequency error | clause 6.5.1 | 12 Hz | Formula: Frequency Error limit + TT |
| 6.5.3 EVM | clause 6.5.2 | 1% | Formula: EVM limit + TT |
| 6.5.5 DL RS power | clause 6.5.4 | 0.8 dB, f ≤ 3 GHz | Formula: Upper limit + TT Lower limit - TT DL RS power shall be within 2.9 dB, f ≤ 3 GHz |
| 6.6.2 Occupied bandwidth | clause 6.6.2 | 0 Hz | Formula: Minimum Requirement + TT |
| 6.6.3 Adjacent Channel Leakage power Ratio (ACLR) | clause 6.6.3 | BW ≤ 20 MHz: 0.8 dB | Formula: ACLR Minimum Requirement - TT |
| 6.6.4 Operating band unwanted emissions | clause 6.6.4 | Offsets < 10 MHz: 1.5 dB, f ≤ 3 GHz Offsets ≥ 10 MHz: 0 dB | Formula: Minimum Requirement + TT |
| [6.6.5.5.1.1] TX spurious emissions: General requirements | clause 6.6.5.2.1 | 0 dB | Formula: Minimum Requirement + TT |
| [6.6.5.5.1.2] TX spurious emissions: Protection of SAN receiver | clause 6.6.5.2.2 | 0 dB | Formula: Minimum Requirement + TT |
| NOTE: TT values are applicable for normal condition unless otherwise stated. |  |  |  |

Table 6: Derivation of test requirements for OTA transmitter tests

| Test | Minimum requirement in TS 36.108 [2] | Test Tolerance (TTOTA) | Test requirement in the present document |
| --- | --- | --- | --- |
| 9.2 Radiated transmit power | clause 9.2 | E-UTRA: 1.1 dB NB-IoT: [1.1] dB | Formula: Upper limit + TT, Lower limit – TT |
| 9.3 OTA SAN output power | clause 9.3 | 1.4 dB | Formula: Upper limit + TT, Lower limit – TT |
| 9.4.3 OTA total power dynamic range | clause 9.4.3 | 0.4 dB | Formula: Total power dynamic range – TT |
| 9.6.2 OTA frequency error | clause 9.6.1 | 12 Hz | Formula: Frequency Error limit + TT |
| 9.6.3 OTA EVM | clause 9.6.2 | 1% | Formula: EVM limit + TT |
| 9.6.5 OTA DL RS power | clause 9.6.4 | 3.2 dB | Formula: limit + TT |
| 9.7.2 OTA occupied bandwidth | clause 9.7.2 | 0 Hz | Formula: Minimum Requirement + TT |
| 9.7.3 OTA ACLR | clause 9.7.3 | 1 dB, f ≤ 3 GHz | Formula: Relative limit - TT |
| 9.7.4 OTA operating band unwanted emissions | clause 9.7.4 | Offsets < 10 MHz: 1.8 dB Offsets ≥ 10MHz: 0 dB | Formula: Minimum Requirement + TT |
| [9.7.5.5.1] OTA TX spurious emissions: General requirements | clause 9.7.5.2.2 | 0 dB | Formula: Minimum Requirement + TT |
| NOTE: TT values are applicable for normal condition unless otherwise stated. |  |  |  |

  - RX TT

The following is TT values are agreed for RX:

Table 7: Derivation of test requirements for conducted receiver tests

| Test | Minimum requirement in TS 36.108 [2] | Test Tolerance (TT) | Test requirement in the present document |
| --- | --- | --- | --- |
| 7.2 Reference sensitivity level | clause 7.2 | E-UTRA: 0.7 dB, f ≤ 3 GHz NB-IoT: 1 dB | Formula: Reference sensitivity power level + TT |
| 7.3 Dynamic range | clause 7.3 | 0.3 dB | Formula: Wanted signal power + TT |
| 7.4.1 Adjacent Channel Selectivity (ACS) | clause 7.4 | 0 dB | Formula: Wanted signal power + TT |
| 7.5 Out-of-band blocking | clause 7.5 | 0 dB | Formula: Wanted signal power + TT |
| 7.8 In-channel selectivity | clause 7.8 | 1.4 dB | Formula: Wanted signal power + TT |
| NOTE: TT values are applicable for normal condition unless otherwise stated. |  |  |  |

Table 8: Derivation of test requirements for OTA receiver tests

| Test | Minimum requirement in TS 36.108 [2] | Test Tolerance (TTOTA) | Test requirement in the present document |
| --- | --- | --- | --- |
| 10.2 OTA sensitivity | clause 10.2 | E-UTRA: 1.3 dB NB-IoT: [1.3] dB | Formula: Declared Minimum EIS + TT |
| 10.3 OTA reference sensitivity level | clause 10.3 | E-UTRA: 1.3 dB NB-IoT: [1.3] dB | Formula: EISREFSENS + TT |
| 10.4 OTA dynamic range | clause 10.4 | 0.3 dB | Formula: Wanted signal power + TT Interferer signal power unchanged. |
| 10.5.1 OTA adjacent channel selectivity | clause 10.5.1 | 0 dB | Formula: Wanted signal power + TT Interferer signal power unchanged. |
| 10.6 OTA out-of-band blocking | clause 10.6 | 0 dB | Formula: Wanted signal power + TT Interferer signal power unchanged. |
| 10.9 OTA in-channel selectivity | clause 10.9 | 1.7 dB | Formula: Wanted signal power + TT Interferer signal power unchanged |
| NOTE: TT values are applicable for normal condition unless otherwise stated. |  |  |  |

RRM performance requirements

- RRM test cases list and work split were agreed and captured in [4].

Issue 1: NB-IoT operation mode

- Agreement

  - Introduce test cases for NB-IoT over NTN operating in standalone mode only

Issue 3: Tests involving neighbor cell measurement

- Agreements

  - Do not introduce the following test cases in R17 IOT-NTN for NGSO case

    - Cell-reselection

      - Intra-frequency measurement

      - Inter-frequency measurement

    - RRC Re-establishment

    - RRC Connection Release with Redirection

    - E-UTRAN inter frequency and intra frequency measurements

    - Measurement accuracy

      - Intra-frequency measurement accuracy requirements

      - Inter-frequency measurement accuracy requirements

    - HO requirements

  - FFS on test cases for GSO case

Issue 5-1: Gap between Segmented UL Transmissions - applicability rules for timing advance is decreasing

- RAN4 will not re-define UE behaviour during the gap between segment transmissions

- There is no need to define additional gap applicability rules in RAN4

Issue 5-2: Gap between Segmented UL Transmissions – others

- Tx samples within gap between segment transmissions should not be taken into account in performance measurement at TE.

- FFS: whether to define UE initial transmission timing test for UL segmented transmission

Issue 6: GNSS Position Acquisition Method

- FFS whether to follow the same approach as NR NTN for GNSS Position Acquisition Method for UE transmit timing test cases.

Issue 7: PHR reporting

Companies are encouraged to provide analysis on the following options:

- Option 1: For NB-IoT and eMTC, legacy PHR table can be reused for IoT NTN.

- Option 2: For NB-IoT operation with NGSO, the RAN4 to discuss the need to implement a different table in the specification for NGSO.

Issue 8-1: Test setup and Satellite assistance information

- RAN4 to reuse the Rel-17 NR NTN test framework for IoT NTN tests which includes predefining configurations for serving cell’s satellite.

- Create reference SIB31 configuration

- Not to consider the configuration of the common delay parameters, as discussed in NR NTN in R4-2220711

Issue 8-3: Satellite trajectory and TE reference propagator model

- Follow the same reference model as NR NTN for satellite trajectory and TE reference propagator model

For RRM performance requirements, further agreements were captured in [4], as follows:

UE demodulation requirements

- Work split on CRs for UE demodulation requirements were captured in [5].

- TS36.102 skeleton for IoT-NTN UE demodulation is agreed and captured in [5].

For UE demodulation requirements, agreements on simulation assumptions were captured in [5], as follows:

Topic #3: Simulation assumptions for Cat-M1 and NB-IoT over NTN

Issue 3-1: Whether the UE is expected to adjust DL frequency compensation on the receiver end within a block of repetitions

- Agreement:

  - From RAN4 performance requirements assumption perspective, UE assumed to be capable of DL frequency compensation on the receiver end within a block of repetitions.

Issue 3-2: Maximum Doppler spread for Cat-M1 PDSCH Test 2

- Agreement:

  - 200Hz

Issue 3-3: FRCs for Cat-M1 PDSCH demodulation requirements for NTN

- Agreement:

  - Test 1: TM1, 3RB, 16QAM, Channel bits: 1656 bits, TBS: 744 bits (Effective CR: 0.46)

  - Test 2: TM1, 6RB, QPSK, Channel bits: 1656 bits, TBS: 504 bits (Effective CR: 0.32)

  - Test 3: TM1, 6RB, QPSK, Channel bits: 1656 bits, TBS: 152 bits (Effective CR: 0.10)

Issue 3-4: DL scheduling for Cat-M1 PDSCH demodulation requirements

- Agreement:

  - dropping/puncturing MPDCCH/PDSCH when MPDCCH/PDSCH subframe collides with PSS/SSS/PBCH/SI subframe

  - The following DL scheduling pattern are agreed to be the baseline and companies are encouraged to bring simulation results based on the following DL scheduling pattern.

  - Other scheduling patterns are not precluded.

    - For Cat-M1 PDSCH test 1:

    - For Cat-M1 PDSCH test 2:

    - For Cat-M1 PDSCH test 3:

Issue 3-5: Assumption for channel estimation length in NPDSCH simulation

- Agreement:

  - 1ms

  - Note: 1ms is used in simulation assumption to define minimum requirements, not to restrict UE implementation

Issue 3-6: Maximum Doppler spread for NB-IoT NPDSCH Test 1

- Agreement:

  - 200Hz

Issue 3-7: Repetition factor for NB-IoT NPDSCH Test 2

- Agreement:

  - 128

Issue 3-8: PDSCH demodulation requirements for Cat-M1 over NTN

- Agreement

| Test number | CBW | MCS | Transmission mode | PDSCH repetition | Propagation condition | Antenna configuration | K_offset | SNR requirements |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | FDD 1.4MHz | 16QAM 1/2 | TM1 | 1 | NTN-TDLC5- 30 | 1x1 | 8ms | TBD |
| 2 | FDD 1.4MHz | QPSK 1/3 | TM1 | 8 | NTN-TDLA100-200 | 1x1 | 8ms | TBD |
| 3 | FDD 1.4MHz | QPSK 1/10 | TM1 | 64 | NTN-TDLA100-10 | 1x1 | 8ms | TBD |

Issue 1-4-1: PDSCH demodulation requirements for NB-IoT over NTN

- Agreement:

| Test number | CBW | Carrier type | MCS | NPDSCH repetition | Propagation condition | Antenna configuration | K_offset | SNR requirements |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | FDD 200kHz | Anchor | QPSK 1/2 (Reuse R.NB6.FDD) | 32 | NTN-TDLC5-200 | 1x1 | 8ms | TBD |
| 2 | FDD 200kHz | Non-anchor | QPSK 1/3 (Reuse R.NB6-1.FDD) | 128 | NTN-TDLA100-10 | 1x1 | 8ms | TBD |

SAN demodulation requirements

- Work split on CR for SAN demodulation requirements were captured in [6].

- For SAN demodulation requirements, agreements were captured in [6], as follows:

# Topic #2: General

  - Sub-topic 2-1: Scenario

Agreement: Deployment mode for NB-IoT

  - Only standalone deployment considered for NB-IoT demodulation requirement. The requirement can be applicable for other mode for BS demodulation requirements

Agreement: Duplex mode for eMTC/NB-IoT

  - Only consider FDD

    - For Cat-M1 UE, define SAN demodulation requirements only for FDD

    - For Cat-NB1/NB2 UEs, define SAN demodulation requirements only for FDD.

Agreement: UL segmented transmission with multiple segments

  - No PUSCH/NPUSCH/PRACH/NPRACH requirement for Uplink segmented transmission with multiple segments. Only define requirements with one segment.

Agreement: Assumption of pre-compensation gap for UL segmented transmission

  - Not to consider pre-compensation gap for simulation.

    - Interested companies can bring simulation results for pre-compensation gap enabled case.

    - See the detailed simulation assumption in Topic #3 and #4.

  - Sub-topic 2-2: Channel model

Agreement: Additional Doppler shift offset and transmit timing offset

  - When using segments size (Nsegmentprecompensation) greater than 1, for NPRACH, NPUSCH and NPUCCH, create additional frequency doppler shift offset for every subsequent repetition after the first in a segment.

  - Evaluate if an offset in transmit timing needs to be applied to the subsequent repetitions in a segment as well

  - For additional Doppler shift offset modeling

    - Option 1: gradual offset accumulation is considered for all Tx duration period in one segment.

    - Option 2: Only consider the largest accumulated offset for all Tx duration period in one segment.

  - For additional Doppler offset applied physical channel

    - Option 1: only for NPUSCH format 1 and PUSCH

    - Option 2: for NPUSCH, PUSCH, PUCCH, NPRACH, and PRACH

  - For additional transmit timing offset modeling

    - Option 1: gradual offset accumulation is considered for all Tx duration period in one segment.

    - Option 2: Only consider the largest accumulated offset for all Tx duration period in one segment.

  - For additional transmit timing offset applied physical channel

    - Option 1: only for NPRACH and PRACH

    - Option 2: for NPUSCH, PUSCH, PUCCH, NPRACH, and PRACH

Figure 1	Example of additional Doppler shift offset modeling (For information)

Agreement: Channel model for eMTC SAN demodulation requirements

  - For initial simulation purpose, below channel model considered

    - NTN-TDLA100-X,		X = [5, 200] Hz

    - NTN-TDLC5-Y,		Y = [5, 30] Hz, Rice factor=8.05dB

Agreement: Channel model for NB-IoT SAN demodulation requirements

  - For initial simulation purpose, below channel model considered

    - NTN-TDLA100-X,		X = [1, 200] Hz

    - NTN-TDLC5-Y,		Y = [1, 30] Hz, Rice factor=8.05dB

  - Sub-topic 2-3: Demodulation requirement scope

Agreement: Antenna branch for SAN demodulation

  - Define both 1Rx and 2Rx requirements for all SAN demodulation requirements for NB-IoT/eMTC.

Agreement: Scope of eMTC SAN demodulation requirements

  - PUSCH CE mode A

  - PUSCH CE mode B

  - PUCCH DTX to ACK performance requirement

  - PUCCH format 1a

  - PRACH false alarm probability

  - PRACH preamble detection with frequency hopping off

Agreement: Channel bandwidth for eMTC SAN demodulation requirements

  - 1.4MHz only

Agreement: Scope of NB-IoT SAN demodulation requirements

  - NPUSCH format 1

  - NPUSCH format 2 DTX to ACK performance requirement

  - NPUSCH format 2

  - NPRACH false alarm

  - NPRACH miss detection

Agreement: Channel bandwidth for NB-IoT SAN demodulation requirements

  - 200kHz only

# Topic #3: SAN demodulation requirement for eMTC

  - Sub-topic 3-1: PUSCH requirements

Way forward: Simulation assumptions for PUSCH CE mode A (for initial simulation purpose)

  - Assume TxDuration is same as the number of repetitions

| Parameters | CE Mode A |
| --- | --- |
| Max number of HARQ transmissions | 4 |
| RV sequences | 0, 2, 3, 1, 0, 2, 3, 1 |
| Number of PUSCH repetitions | 8 |
|  | FFS: 1 |
| Frequency hopping | OFF |
| Duplex mode | FDD only |
| Number of Tx antennas | 1 |
| Number of Rx antennas | 1 and 2 |
| Propagation channel | Option 1: NTN-TDLA100-X, X = [5, 200] Hz Option 2: NTN-TDLC5-Y, Y = [5, 30] Hz |
| FRC | TS36.104 A3-2 |
| System BW | 1.4MHz only |
| Doppler shift offset | FFS |
| Transmit timing offset | FFS |
| Fraction of maximum throughput | 70% |

Way forward: Simulation assumption for PUSCH CE mode B (for initial simulation purpose)

  - Assume TxDuration is same as the number of repetitions

| Parameters | CE Mode B |
| --- | --- |
| Max number of HARQ transmissions | 2 |
| RV sequences | 0, 0, 0, 0, 2, 2, 2, 2, 3, 3, 3, 3, 1, 1, 1, 1 |
| Number of PUSCH repetitions | 256 |
| Frequency hopping | OFF |
| Duplex mode | FDD only |
| Number of Tx antennas | 1 |
| Number of Rx antennas | 1 and 2 |
| Propagation channel | NTN-TDLA100-X, X = [5, 200] Hz |
| FRC | TS36.104 A3-1 |
| System BW | 1.4MHz only |
| Doppler shift offset | FFS |
| Transmit timing offset | FFS |
| Fraction of maximum throughput | 70% |

  - Sub-topic 3-2: PUCCH requirements

Way forward: Simulation assumption for PUCCH format 1a (for initial simulation purpose)

  - The ACK missed detection probability shall not exceed 1%

  - Assume TxDuration is same as the number of repetitions

  - RAN4 will set the same propagation channel for PUCCH format 1a and PRACH fading scenario.

| Parameters | Values |
| --- | --- |
| Number of PUCCH repetitions | 8 |
| Frequency hopping | OFF |
| Duplex mode | FDD only |
| Number of Tx antennas | 1 |
| Number of Rx antennas | 1 and 2 |
| Propagation channel | Option 1: NTN-TDLA100-X, X = [5, 200] Hz Option 2: NTN-TDLC5-Y, Y = [5, 30] Hz |
| System BW | 1.4MHz |
| Fraction of maximum throughput | 70% |

  - Sub-topic 3-3: PRACH requirements

Way forward: Simulation assumptions for PRACH requirements (for initial simulation purpose)

  - The false alarm probability shall be less than or equal to 0.1%.

  - The probability of detection shall be equal to or exceed 99%.

  - Frequency hopping: OFF

  - Assume TxDuration is same as the number of repetitions

  - RAN4 will set the same propagation channel for PUCCH format 1a and PRACH fading scenario.

| Number of Tx antennas | Number of Rx antennas | Propagation channel | Frequency offset | Number of repetitions | Burst format 0 | Burst format 1 | Burst format 2 | Burst format 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 and 2 | Option 1: NTN-TDLA100-X, X = [5, 200] Hz Option 2: NTN-TDLC5-Y, Y = [5, 30] Hz | 200Hz + X (X: See Sub-topic 2-2 Additional Doppler shift offset) | 8 | YES | YES |  |  |
|  |  |  |  | 16 |  |  | YES | YES |

# Topic #4: SAN demodulation requirement for NB-IoT

  - Sub-topic 4-1: NPUSCH format 1 requirements

Way forward: Simulation assumptions for NPUSCH format 1 requirements (for initial simulation purpose)

  - FFS for Doppler shift offset and Transmit timing offset

| Number of TX antennas | Number of RX antennas | Subcarrier spacing | Number of allocated subcarriers | Propagation conditions | FRC | Repetition number | Fraction of maximum throughput |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 and 2 | 3.75KHz | 1 | Option 1: NTN- TDLA100-X, X = [1, 200] Hz Option 2: NTN-TDLC5-Y, Y = [1, 30] Hz | TS36.104 A16-1 | 16 | 70% |
| 1 | 1 and 2 | 15KHz | 12 | Option 1: NTN- TDLA100-X, X = [1, 200] Hz Option 2: NTN-TDLC5-Y, Y = [1, 30] Hz | TS36.104 A16-5 | 16 | 70% |

  - Sub-topic 4-2: NPUSCH format 2 requirements

Way forward: Simulation assumptions for NPUSCH format 2 requirements (for initial simulation purpose)

  - The DTX to ACK probability, i.e., the probability that ACK is detected when nothing is sent per NPUSCH format 2 transmission, shall not exceed 1% per NPUSCH format 2 transmission.

  - The ACK missed detection probability shall not exceed 1%.

  - Assume TxDuration is same as the number of repetitions.

  - RAN4 will set the same propagation channel for NPUCCH format 2 and NPRACH fading scenario.

| Number of TX antennas | Number of RX antennas | Propagation conditions | Number of allocated subcarriers | Subcarrier spacing | Repetition number |
| --- | --- | --- | --- | --- | --- |
| 1 | 1 and 2 | Option 1: NTN- TDLA100-X, X = [1, 200] Hz Option 2: NTN-TDLC5-Y, Y = [1, 30] Hz | 1 | 3.75KHz | 16 |
| 1 | 1 and 2 | Option 1: NTN- TDLA100-X, X = [1, 200] Hz Option 2: NTN-TDLC5-Y, Y = [1, 30] Hz | 1 | 15KHz | 16 |

  - Sub-topic 4-3: NPRACH requirements

Way forward: Simulation assumptions for NPRACH demodulation requirements (for initial simulation purpose)

  - The false alarm probability shall be less than or equal to 0.1%.

  - The probability of detection shall be equal to or exceed 99%.

  - Assume TxDuration is same as the number of repetitions

  - RAN4 will set the same propagation channel for NPUCCH format 2 and NPRACH fading scenario.

| Number of Tx | Number of Rx | Repetition number | Propagation channel and correlation | Frequency offset | PF0 | PF1 |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 and 2 | 8 | Option 1: NTN- TDLA100-X, X = [1, 200] Hz Option 2: NTN-TDLC5-Y, Y = [1, 30] Hz | 200Hz + X (X: See sub-topic 2-2 Additional Doppler shift offset) | YES | YES |
| 1 | 1 and 2 | 32 | Option 1: NTN- TDLA100-X, X = [1, 200] Hz Option 2: NTN-TDLC5-Y, Y = [1, 30] Hz | 200Hz + X (X: See sub-topic 2-2 Additional Doppler shift offset) | YES | YES |

#### 2.4.2	Remaining Open issues

Performance part

- For SAN conformance testing requirements, the details of tests remain open.

- For UE RRM performance requirements, the details of RRM tests remain open. Whether or not to introduce the test cases which involving neighbor cell measurement in GEO/GSO remains open.

- For UE demodulation requirements, the details of performance requirements remain open.

- For SAN demodulation requirements, the details of performance requirements and test configurations remain open.

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

## 3.1	SAx/CTs

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

## 4.	References

- R4-2302934, Work Plan for performance part for NB-IoT/eMTC for NTN requirements, MediaTek inc., Huawei, HiSilicon.

- R4-2302906, WF for IoT over NTN SAN RF conformance requirements, ZTE.

- R4-2302907, WF for MU assessment for IoT over NTN SAN RF conformance, Huawei.

- R4-2303311, WF on NB-IoT/eMTC RRM requirements for NTN, MediaTek.

- R4-2302932, WF for UE demodulation requirements of IoT over NTN, MediaTek.

- R4-2302933, WF for SAN demodulation requirements for IoT over NTN, Ericsson.

01.08.2022		minor adaptations for RAN #97e

21.05.2022		minor adaptations for RAN #96

10.01.2022		minor adaptations for RAN #95e

04.10.2021		minor adaptations for RAN #94e

08.08.2021		minor adaptations for RAN #93e

17.05.2021		minor adaptations for RAN #92e

28.01.2021		minor adaptations for RAN #91e

09.11.2020		minor adaptations for RAN #90e

31.08.2020		minor adaptations for RAN #89e

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template