3GPP TSG RAN Meeting #99	RP-230803

Rotterdam, Netherlands, 20-23 March, 2023

Source:	Huawei

Title:	pCR for TR 38.848 “Study on Ambient IoT (Internet of Things) in RAN” – Use case mapping

Agenda item:	9.2.3

Document for:	Agreement

- Introduction

This pCR implements a mapping from SA1 use cases to RAN representative use cases for FS_Ambient_IoT_RAN.

The baseline text is that resulting after the inclusion into TR 38.848 v0.0.2 of the pCR implementing the decisions from RAN#98-e, in RP-230571.

- Proposed changes

# <Unchanged text is omitted>

# 4	Deployment scenarios, use cases, services

### 4.1	Use cases/services

The use cases and services considered in this TR are those described in [2, TR 22.840]. In TSG RAN, they were formed into several groups of representative use cases, as defined in the next section, to help identifying commonalities or similar requirements among the members of a representative use case which could help the analysis conducted during the study.

### 4.1	Representative use cases

Two sets or levels of grouping were defined. The first, Grouping A, is on the basis of the deployment environment(s) described for a use case in [2, TR 22.840], and the second, Grouping B, is on the basis of functionality/application described in [2, TR 22.840].

Grouping A:

- Indoor

- Outdoor

- Indoor/outdoor

Grouping B:

- Inventory

- Sensors

- Positioning

- Command

These two groupings are then used to form representative use cases (rUCs) as follows, which are used in Clause 4.2 – Deployment scenarios.

- rUC1: Indoor inventory

- rUC2: Indoor sensors

- rUC3: Indoor positioning

- rUC4: Indoor command

- rUC5: Outdoor inventory

- rUC6: Outdoor sensors

- rUC7: Outdoor positioning

- rUC8: Outdoor command

This resulted in the following mapping from SA1 use cases and traffic scenarios onto RAN rUCs:

Table 4.1-1: Mapping between RAN representative use cases and SA1 use cases

| rUC | Applicable SA1 UCs / traffic scenarios |
| --- | --- |
| rUC1: Indoor inventory | 5.1 Automated warehousing 5.2 Medical instruments inventory management and positioning 5.4 Non-Public Network for logistics 5.5 Automobile manufacturing 5.7 Airport terminal / shipping port 5.15 Smart laundry 5.16 Automated supply chain distribution 5.18 Fresh food supply chain 5.27 End-to-end logistics 6.1 Flower auction 6.3 Electronic shelf label |
| rUC2: Indoor sensor | 5.6 Smart homes 5.13 Base station machine room environmental supervision 5.15 Smart laundry 5.20 Smart agriculture 5.23 Smart pig farm 6.2 Cow stable |
| rUC3: Indoor positioning | 5.8 Finding Remote Lost Item 5.9 Location service 5.10 Ranging in a home 5.12 Personal belongings finding 5.14 Positioning in shopping centre 5.21 Museum Guide |
| rUC4: Indoor command | 5.11 Online modification of medical instruments status 5.17 Device activation and deactivation 5.26 Elderly Health Care 5.29 Device Permanent Deactivation 6.3 Electronic shelf label |
| rUC5: Outdoor inventory | 5.2 Medical instruments inventory management and positioning 5.4 Non-public network for logistics 5.7 Airport terminal / shipping port 5.16 Automated supply chain distribution |
| rUC6: Outdoor sensor | 5.3 Smart grids 5.19 Forest Fire Monitoring 5.22 Dairy farming 5.24 Smart manhole cover safety monitoring 5.25 Smart bridge health monitoring |
| rUC7: Outdoor positioning | 5.8 Finding remote lost item 5.9 Location service 5.12 Personal belongings finding |
| rUC8: Outdoor command | 5.11 Online modification of medical instruments status 5.17 Device activation and deactivation 5.26 Elderly Health Care 5.30 Controller in smart agriculture |