3GPP TSG RAN WG1 Meeting #100	R1-2000300

e-Meeting, February 24th – March 6th, 2020

Title:	Draft reply LS on default codebook parameters

Response to:	R2-1916592

Release:	Rel-15

Work Item:	NR_newRAT-Core

Source:	vivo [RAN WG1]

To:	RAN WG2

Cc:

# Contact Person:

# Name:

# E-mail Address:

# 1. Overall Description:

RAN1 thanks RAN2 for the LS R2-1916592 on default codebook parameters. On the default parameters for mandatory type1 singlePanel codebook, RAN1 would like to provide the values as below.

Parameters for type I single panel codebook (type1 singlePanel) supported by the UE, which is mandatory to report:

-	supportedCSI-RS-ResourceList; -> default value is 1

-	modes indicates supported codebook modes (mode 1, both mode 1 and mode 2); -> default mode is mode1

-	maxNumberCSI-RS-PerResourceSet indicates the maximum number of CSI-RS resource in a resource set. -> default value is 1

# 2. Actions:

# To RAN WG2

# RAN1 respectfully requests RAN2 to take the above into account.

# 4. Date of Next TSG-RAN WG1 Meetings:

TSG RAN WG1 Meeting #100b         20 – 24 Apr 2019, Busan, Korea

TSG RAN WG1 Meeting #101          24 – 28 Feb 2020, Athens, Greece