3GPP TSG RAN WG1 Meeting #100                                           	                    R1-2000523

e-Meeting, February 24th – March 6th, 2020

Source:	CATT

Title:	Remaining issues on sidelink synchronization mechanism in NR V2X

Agenda Item:	7.2.4.3

Document for:	Discussion and Decision

Introduction

In RAN1#99 meeting, the following agreements were achieved on NR V2X Sidelink synchronization mechanism [1]:

Agreements:

Confirm the working assumptions that the same sequence is used for both symbols of S-PSS and the same sequence is used for both symbols of S-SSS.

Agreements:

- Proposal 4 in R1-1912157 is agreed

Agreements:

The in-coverage indicator as included in PSBCH payload is 1-bit.

R1-1913392

Agreements:

For PSBCH DM-RS RE position:

- No shift in frequency domain is adopted.

Agreements:

For PSBCH DM-RS RE position:

- Every symbol of PSBCH contains the DM-RS.

Agreements:

- The S-SSB resources in time domain is (pre-)configured.

Agreements:

S-PSS uses the same polynomial (i.e., x7+x4+1) and the same initial value, but different cyclic shifts {22, 65} to that of NR DL-PSS.

Agreeements:

S-SSS reuses, the same polynomials, initial values and cyclic shifts as that of the Gold sequences of NR DL-SSS.

Agreements:

- The triggering of S-SSB transmission in NR V2X reuses the same mechanism that in LTE V2X.

Working assumption:

- PSBCH payload size is 56 bits including 24 bits of CRC.

Agreements:

- Note: “green” already earlier; “blue” new agreements, “brown” working assumption, “change marks” for updates

| PSBCH contents | Number of bits | Notes |
| --- | --- | --- |
| DFN | 10 |  |
| Indication of TDD configuration | 12 | System-wide information, e.g. TDD-UL-DL common configuration and/or potential SL slots |
| Slot index | 7 | Note: Up to 3 bits can be carried in DM-RS or in PBCH payload. |
| In-coverage indicator | 1 |  |
| Reserve bits |  |  |
| CRC | 24 |  |
| Total bits | 56 |  |

In RAN#86 meeting, the following remaining issues were noted on R16 V2X Sidelink synchronization mechanism in task list for 5G V2X [2]:

| Category | Remaining issue in RAN1 |  | RAN2 impact |
| --- | --- | --- | --- |
| Synchronization | E1 | Details of transmission power of S-SSB | Depends on the outcome of RAN1 discussion |
|  | E2 | Details and clarification of PSBCH contents including TDD configuration and slot number in PSBCH | YES |
|  | E3 | DM RS sequence initialization for PSBCH | No |
|  | E4 | Whether/how to define QCL for S-SSB transmission | No |

This contribution discusses remaining issues on sidelink synchronization mechanism in NR V2X, and proposes related solutions. Section 2 discusses PSBCH DM-RS sequence initialization, PSBCH contents, and payload size of PSBCH. Section 3 summarizes the proposals with conclusions.

Discussion

This section discusses PSBCH DM-RS sequence initialization, PSBCH contents, and payload size of PSBCH.

PSBCH DM-RS sequence initialization

In the email discussion [98b-NR-17] after RAN1#98bis, RAN1 reached the agreements about PSBCH DM-RS that PSBCH uses the same DM-RS sequence generation method as that by NR Uu PBCH. In this section, we will discuss the details on PSBCH DM-RS sequence initialization.

- PSBCH DM-RS design uses NR Uu PBCH DM-RS design as the starting point

      - DM-RS RE mapping

  - Equal DM-RS density across NR-PBCH with 3 REs/PRB/symbol (i.e., comb-4 type)

      - Same DM-RS sequence generation with NR Uu PBCH

  - length-31 Gold sequence with QPSK modulation

      - FFS DM-RS RE position shift in frequency domain

      - FFS the number of PSBCH symbols that contain DM-RS

In NR Uu, the PBCH DM-RS sequence  is defined by [3]:

where  is a length-31 Gold sequence. The scrambling sequence generator shall be initialized at the start of each SSB occasion with:

where

-	for L=4,  where  is the number of the half-frame index, and  is the two least significant bits of the SSB index.

-	for L=8 or L=64,  where  is the three least significant bits of the SSB index.

with L being the maximum number of SSB beams in an SSB period for a particular band.

In NR PBCH DM-RS sequence initialization, the parameters of cell ID (NIDcell) and half-frame index (nhf) are used to initialize the scrambling sequence generator. For NR V2X, S-SSB is related to SL-SSID instead of cell ID, therefore, the cell ID should be replaced with the SL-SSID. For the half frame index, it can be removed from the formula. In RAN1#99 meeting [1], it is agreed that for PSBCH DM-RS RE position, no shift in frequency domain is adopted, therefore, the operation of “NIDcell mod 4” in the formula is not needed. The NIDcell4 and NIDcell mod 4 in the formula of NR PBCH DM-RS sequence initialization can be directly replaced by SL-SSID.

Based on the above discussion, the PSBCH DM-RS sequence initialization should use the following formula:

cinit=211iS-SSB+1NIDSLSS+1+26iS-SSB+1+ NIDSLSS

where

-	for FR1, iS-SSB is the S-SSB index.

-	for FR2, iS-SSB is the three least significant bits of the S-SSB index.

Proposal 1: The PSBCH DM-RS sequence initialization should use the following formula:

cinit=211iS-SSB+1NIDSLSS+1+26iS-SSB+1+ NIDSLSS

where

-	for FR1, iS-SSB is the S-SSB index.

-	for FR2, iS-SSB is the three least significant bits of the S-SSB index.

-------------------------------------------------Start of Text Proposal for 38.211-----------------------------------------------

- 8.4.1.4.	Demodulation reference signals for PSBCH

- 8.4.1.4.1	Sequence generation

The reference-signal sequence rm for an S-SS/PSBCH block is defined by

rm=121-2cm+j121-2cm+1

where cn is given by clause 5.2. The scrambling sequence generator shall be initialized at the start of each S-SS/PSBCH block occasion with

cinit=211iS-SSB+1NIDSLSS+1+26iS-SSB+1+ NIDSLSS

where

-	for FR1, iS-SSB is the S-SSB index.

-	for FR2, iS-SSB is the three least significant bits of the S-SSB index.

--------------------------------------------------------End of Text Proposal -----------------------------------------------------

Proposal 2: Adopt the above text proposal for PSBCH DM-RS sequence initialization in 38.211.

PSBCH contents

In RAN1#99 meeting [1], the following fields in PSBCH contents with “green” background were agreed.

- Note: “green” already earlier; “blue” new agreements, “brown” working assumption, “change marks” for updates

| PSBCH contents | Number of bits | Notes |
| --- | --- | --- |
| DFN | 10 |  |
| Indication of TDD configuration | 12 | System-wide information, e.g. TDD-UL-DL common configuration and/or potential SL slots |
| Slot index | 7 | Note: Up to 3 bits can be carried in DM-RS or in PBCH payload. |
| In-coverage indicator | 1 |  |
| Reserve bits |  |  |
| CRC | 24 |  |
| Total bits | 56 |  |

From above table, DFN (10 bits), indication of TDD configuration, In-coverage indicator (1 bit) and reserve bits had been agreed as the contents of PSBCH. For indication of TDD configuration, it had been agreed to be system-wide information, e.g. TDD-UL-DL common configuration and/or potential SL slots, the details of how to inform PSBCH contents to the UE is FFS. S-SSB index and indication of synchronization resource should also be included into the PSBCH contents

Proposal 3: PSBCH for NR V2X should include DFN, S-SSB index, Indication of synchronization resource, Indication of TDD configuration and In-coverage indicator.

    - S-SSB index

In RAN1#98bis [5], DFN is agreed to be carried by PSBCH as the frame-level timing information. In order to obtain accurate timing, only DFN is not enough, slot-level timing information is also necessary. There are two alternatives to inform the slot-level timing information:

- Alt-1: Slot-level timing is indicated by: S-SSB index

- Alt-2: Slot-level timing is indicated by: slot number within a frame.

For the Alt-1 scheme, if the S-SSB index is indicated by PSBCH, UE will know the index of current received S-SSB in all S-SSBs sent in the synchronization period, so that the UE receiving S-SSB can obtain timing information and the location of the remaining S-SSBs in the period. It is suggested that reuse the indication mechanism of SSB index in NR Uu, i.e., for FR1, the 2 or 3 bits indicating S-SSB index can be obtained from blind detection of sequences of PSBCH DMRS, while for FR2, the 3 MSBs of 6 bits indicating S-SSB index come from PSBCH payload and the 3 LSBs of the S-SSB index from blind detection of sequences of PSBCH DMRS. Using this method, the content of PSBCH payload for 4/8 S-SSBs is the same, thus avoiding multiple hypotheses when combining multiple S-SSBs for PSBCH decoding.

In addition, when two sets of synchronization resources are configured, UE doesn’t know which synchronization resource the received S-SSB belongs to, if UE only knows S-SSB index. As a result, there is timing ambiguity between two sets of synchronization resources. To solve this issue, indication of synchronization resource can be introduced. Detailed discussions can be found in section 2.2.2.

On the other hand, as proposed in previous meetings by some companies, Alt-2 scheme, i.e., slot number within a frame can also be used to fulfill the timing function of S-SSB index, but the slot number method has the following drawbacks:

- Drawback-1: slot number of each S-SSB is not consecutive as S-SSB index, which will hinder the combining of multiple S-SSBs for PSBCH decoding. Even the 4 MSBs of 7 bits indicating slot number come from PSBCH payload and the left 3 LSBs of the slot number from blind detection of sequences of PSBCH DMRS, the content of PSBCH payload for 4 or 8 combining S-SSBs is still different, thus multiple hypotheses have to be evolved when combining multiple S-SSBs, which will introduce considerable complexity.

As shown in Table 1, considering the UE received 64 S-SSBs and try to combines the 8 consecutive S-SSBs (e.g., 17th ~ 24th S-SSB), if 3 MSBs of 6 bits indicating S-SSB index come from PSBCH payload and the left 3LSBs of S-SSB index from blind detection of sequences of PSBCH DMRS, it can be observed that 3 MSBs in PSBCH payload is the same “010” as shown in Table 1,  so the PSBCH payload is the same for the 8 received consecutive S-SSBs (17th ~ 24th S-SSB), therefore, the multiple hypotheses can be avoided when combining multiple S-SSBs.

However, if 4 MSBs of 7 bits indicating slot number come from PSBCH payload and the left 3 LSBs of slot number from blind detection of sequences of PSBCH DMRS, as slot number is not consecutive for consecutive S-SSBs(17th ~ 24th S-SSB), the 4 MSBs in PSBCH payload of 7 bits indicating slot number are various as showed in Table 1, the content of PSBCH payload for the 8 consecutive S-SSBs is still different, thus multiple hypotheses have to be evolved when combining multiple S-SSBs, which will introduce considerable complexity.

- Drawback-2: introduce more standardization efforts, as a lot of procedures are related to S-SSB index, such as PSBCH DM-RS sequence initialization and PSBCH scrambling.

- Drawback-3: slot number cannot carry the information of beam index when beam sweeping was introduced in future release.

Table 1: Comparison between S-SSB index and Slot number

| Received S-SSB | S-SSB index | Slot number |
| --- | --- | --- |
| 17th S-SSB | 16(010000) | 19(0010011) |
| 18th S-SSB | 17(010001) | 22(0010110) |
| 19th S-SSB | 18(010010) | 25(0011001) |
| 20th S-SSB | 19(010011) | 28(0011100) |
| 21st S-SSB | 20(010100) | 31(0011111) |
| 22nd S-SSB | 21(010101) | 34(0100010) |
| 23rd S-SSB | 22(010110) | 37(0100101) |
| 24th S-SSB | 23(010111) | 40(0101000) |

Proposal 4: Don’t confirm the working assumption that slot number is included into PSBCH payload. S-SSB index should be included into PSBCH payload instead.

Proposal 5: The indication mechanism of SSB index in NR Uu can be reused for the indication of S-SSB index in NR V2X.

- For FR1, the 2 bits indicating S-SSB index are carried by DMRS sequence of PSBCH

- For FR2, the 3 MSBs of 6 bits indicating S-SSB index are carried by PSBCH payload and the 3 LSBs of the S-SSB index by DMRS sequence of PSBCH.

-------------------------------------------------Start of Text Proposal for 38.212-----------------------------------------------

8.1.1	PSBCH payload generation

Denote the bits in a transport block delivered to layer 1 by, where  is the payload size generated by higher layers. The lowest order information bit  is mapped to the most significant bit of the transport block.

Generate the following additional timing related PSBCH payload bits , where:

-	if the frequency range of S-SSB is FR2,

are the 6th, 5th, and 4th bits of S-SSB index, respectively.

-	else if the frequency range of S-SSB is FR1

are reserved.

-	end if

<Irrelevant part omitted>

--------------------------------------------------------End of Text Proposal -----------------------------------------------------

Proposal 6: Adopt the above text proposal for PSBCH payload generation in 38.212.

    - Indication of synchronization resource

As mentioned in section 2.2.1, when multiple sets of synchronization resources are configured, UE doesn’t know which synchronization resource indicated by the detected S-SSB belongs to among synchronization resources. As a result, there is timing ambiguity between synchronization resources.

As shown in Figure 1, the schematic diagram shows the time-domain location of multiple S-SSB transmission in one synchronization period (160ms) under 120kHz SCS. There are 8 slots in one subframe, and each small rectangle represents a slot. The small colorful rectangles represent the synchronization resources which contain S-SSBs. The small white rectangles represent the slots which do not contain S-SSB. Each set of synchronization resource includes 8 S-SSBs. In Figure 1, two sets of synchronization resource are configured, which are identified by different colors. The first set of synchronization resource (R1) is colored by yellow, and the second set of synchronization resource (R2) is colored by green. The current received S-SSB is represented by a yellow shaded rectangle in Figure 1, which belongs to the second set of synchronization resource (yellow).

If there is no indication of synchronization resource information, the UE does not know the yellow shaded S-SSB belongs to the first set of synchronization resource (R1, yellow) or the second set of synchronization resource (R2, green) after the UE receives the yellow shaded S-SSB in Figure 1. UE cannot derive the frame boundary and the starting position of synchronization period.  Indication of synchronization resource should be included in PSBCH payload. The receiver UE will know the yellow shaded S-SSB belongs to the first set of synchronization resource (R1, yellow) based on the indication bit in the PSBCH. UE can further use the timeOffsetSSB-SL-1 corresponding to the first set of synchronization resource (R1, yellow) to calculate the starting position of synchronization period, based on DFN, S-SSB index and the interval between the neighboring S-SSBs (timeIntervalSSB-SL).

Figure 1: Two sets of synchronization resources configured in one synchronization period

Proposal 7: Indication of synchronization resource should be carried by PSBCH to indicate the set of synchronization resource which the S-SSB belonging to.

    - Indication of TDD configuration

In order to protect the data transmission of NR Uu users, the TDD UL-DL configuration needs to be transmitted to out-of-coverage UEs in PSBCH. In LTE V2X, TDD Configuration occupies 3 bits in PSBCH, which corresponding to 8 TDD configurations in one radio frame. In NR Uu, in order to support more flexible transmission, the mechanism of TDD configuration becomes more complicated and needs more bits to indicate the slot/symbol format to be downlink, flexible or uplink.

In NR Uu, the slot format can be configured by semi-static signaling or dynamic signaling. The TDD configuration of semi-static signaling includes SIB1, cell-specific RRC configuration and UE-specific RRC configuration. Dynamic signaling refers to SFI via DCI format 2_0. Table 2 summarizes the signaling for TDD configuration in NR Uu.

Table 2: Summary of signaling for TDD configuration in NR Uu

| No. | Signaling | Container | Semi-static or Dynamic | Cell or UE specific |
| --- | --- | --- | --- | --- |
| 1 | tdd-UL-DL-ConfigurationCommon | SIB1 | Semi-static signaling | Cell-specific |
| 2 | tdd-UL-DL-ConfigurationCommon | ServingCellConfigCommon | Semi-static signaling | Cell-specific |
| 3 | tdd-UL-DL-ConfigurationDedicated | ServingCellConfig | Semi-static signaling | UE-specific |
| 4 | Slot Format Indicator | DCI format 2_0 | Dynamic signaling | UE-group-specific |

The DL and UL slots/symbols configured by cell-specific configuration signaling cannot be changed to other directions by UE-specific or dynamic signaling. Only flexible symbols can be changed. Cell-specific configuration signaling can be chosen as the TDD UL-DL configuration signaling indicated by PSBCH.

Cell-specific “tdd-UL-DL-ConfigurationCommon” in SIB1 and servingCellConfigCommon defined in 38.331[4] includes reference SCS and two TDD-UL-DL-Patterns. Each pattern includes periodicity, number of DL Slots, number of DL Symbols, number of UL Slots, and number of UL Symbols, as shown in below. Figure 2 gives an example for the indication of one Cell-specific TDD-UL-DL-Pattern for the case of 120KHz SCS and 10ms period of TDD configuration.

TDD-UL-DL-ConfigCommon ::=          SEQUENCE {

referenceSubcarrierSpacing          SubcarrierSpacing,

pattern1                            TDD-UL-DL-Pattern,

pattern2                            TDD-UL-DL-Pattern                                                       OPTIONAL, -- Need R

...

}

TDD-UL-DL-Pattern ::=               SEQUENCE {

dl-UL-TransmissionPeriodicity       ENUMERATED {ms0p5, ms0p625, ms1, ms1p25, ms2, ms2p5, ms5, ms10},

nrofDownlinkSlots                   INTEGER (0..maxNrofSlots),

nrofDownlinkSymbols                 INTEGER (0..maxNrofSymbols-1),

nrofUplinkSlots                     INTEGER (0..maxNrofSlots),

nrofUplinkSymbols                   INTEGER (0..maxNrofSymbols-1),

...,

[[

dl-UL-TransmissionPeriodicity-v1530     ENUMERATED {ms3, ms4}                                               OPTIONAL -- Need R

]]

}

The indication of only one TDD-UL-DL-Pattern requires more than 30 bits, and the number of bits for NR Uu TDD UL-DL configuration is so large that NR V2X PSBCH cannot accommodate these bits. Therefore, how to reduce the number of bits for TDD configuration and include them into NR PSBCH is critical.

As only uplink slots/symbols can be used for NR sidelink communication, one solution is to only inform the number of consecutive uplink slots/symbols, instead of full downlink/uplink pattern. 8 periods of TDD configurations can be informed by 3 bits. At most 80 numbers of UL slots and 13 numbers of UL symbols can be informed by 7 bits and 4 bits, respectively. Therefore, totally 14bits (14bits = 3bits (Period) + 7bits (UL Slot) + 4bits (UL Symbol)) should be included in PSBCH payload for indication of TDD configuration.

Figure 2: Indication of one Cell-specific TDD-UL-DL-Pattern

In RAN1#99 meeting [1], the agreed working assumption is that indication of TDD configuration occupies 12 bits in PSBCH payload. The 12 bits come from the number of available bits remaining, after the necessary bits have been allocated to other information in 56 bits PSBCH payload. However, slot number (7 bits) in PSBCH payload is not necessary, and it can be replaced with S-SSB index (3 bits) plus indication of synchronization resource (2 bits), as described in section 2.2.1 and 2.2.2. Thus, the extra 2 bits can be saved for indication of TDD configuration. Therefore, indication of TDD configuration can occupies 14 bits while the total PSBCH payload size is still 56 bits.

However, according to the agreements in sidelink physical layer structure, one sidelink data transmission needs at least 7 OFDM symbols (including AGC and GP symbols), so indication of UL symbols in TDD configuration field only needs 3 bits to indicate the 7 cases of {7, 8, 9, 10, 11, 12, 13}. In addition, in order to further reduce the overhead of indication of TDD configuration in PSBCH payload, configuration of number of UL symbols can be limited to 4 cases {e.g., 7, 9, 11, 13}, so that the number of bits occupied by indication of UL symbols can be reduced from 3 bits to 2 bits. In this way, totally 12bits (12bits = 3bits (Period) + 7bits (UL Slot) + 2bits (UL Symbol)) should be included in PSBCH payload for indication of TDD configuration, which is shown in section 2.3.

Proposal 8: NR Uu Cell-specific TDD-UL-DL-Pattern should be used as indication of TDD configuration in PSBCH with overhead reduction.

Proposal 9: Only inform the number of consecutive uplink slots/symbols and periodicity of NR Uu TDD UL-DL configuration information in order to reduce the overhead.

Proposal 10: Confirm the working assumption that indication of TDD configuration occupies 12 bits in PSBCH payload.

- 12 bits = 3bits (Period) + 7bits (UL Slot) + 2bits (UL Symbol)

Payload size of PSBCH

In RAN1#99 meeting [1], the agreed working assumption is that PSBCH payload size is 56 bits including 24 bits of CRC. The contents of PSBCH discussed in section 2.2 along with DFN, S-SSB index, Indication of synchronization resource, Indication of TDD configuration and In-coverage indicator should be included into PSBCH payload. Whether such information can be included by PSBCH with the payload size of 56 bits need to be analyzed.

Table 3 in below gives the composition of PSBCH content, and its size is equal to current working assumption of 56 bits payload size. PSBCH payload size of 56 bits can accommodate all the necessary PSBCH contents without capacity extension.

Table 3: Proposed composition of PSBCH content with payload size of 56 bits

| PSBCH contents | bits | Notes |
| --- | --- | --- |
| Direct Frame Number | 10 | [0,1023] |
| Indication of TDD configuration | 12 | 14bits = 3bits (Period) + 7bits (UL Slot) + 2bits (UL Symbol) |
| S-SSB index | 3 | 3 MSBs of 6 bits indicating S-SSB index come from PSBCH payload for FR2 |
| Indication of synchronization resource | 2 | 2bits indicate 3 candidate sets of synchronization resource |
| In-coverage Indicator | 1 | Carried by PSBCH Payload explicitly |
| Reserved | 4 | Reserved bits |
| CRC | 24 |  |
| Total | 56 |  |

Proposal 11: Confirm the working assumption that PSBCH payload size is 56 bits including 24 bits of CRC.

| PSBCH contents | bits | Notes |
| --- | --- | --- |
| Direct Frame Number | 10 | [0,1023] |
| Indication of TDD configuration | 12 | 14bits = 3bits (Period) + 7bits (UL Slot) + 2bits (UL Symbol) |
| S-SSB index | 3 | 3 MSBs of 6 bits indicating S-SSB index come from PSBCH payload for FR2 |
| Indication of synchronization resource | 2 | 2bits indicate 3 candidate sets of synchronization resource |
| In-coverage Indicator | 1 | Carried by PSBCH Payload explicitly |
| Reserved | 4 | Reserved bits |
| CRC | 24 |  |
| Total | 56 |  |

Conclusion

In this contribution, we discuss PSBCH DM-RS sequence initialization, PSBCH contents, payload size of PSBCH, and give the following proposals:

Proposal 1: The PSBCH DM-RS sequence initialization should use the following formula:

cinit=211iS-SSB+1NIDSLSS+1+26iS-SSB+1+ NIDSLSS

where

-	for FR1, iS-SSB is the S-SSB index.

-	for FR2, iS-SSB is the three least significant bits of the S-SSB index.

Proposal 2: Adopt the above text proposal for PSBCH DM-RS sequence initialization in 38.211.

Proposal 3: PSBCH for NR V2X should include DFN, S-SSB index, Indication of synchronization resource, Indication of TDD configuration and In-coverage indicator.

Proposal 4: Don’t confirm the working assumption that slot number is included into PSBCH payload. S-SSB index should be included into PSBCH payload instead.

Proposal 5: The indication mechanism of SSB index in NR Uu can be reused for the indication of S-SSB index in NR V2X.

- For FR1, the 2 bits indicating S-SSB index are carried by DMRS sequence of PSBCH

- For FR2, the 3 MSBs of 6 bits indicating S-SSB index are carried by PSBCH payload and the 3 LSBs of the S-SSB index by DMRS sequence of PSBCH.

Proposal 6: Adopt the above text proposal for PSBCH payload generation in 38.212.

Proposal 7: Indication of synchronization resource should be carried by PSBCH to indicate the set of synchronization resource which the S-SSB belonging to.

Proposal 8: NR Uu Cell-specific TDD-UL-DL-Pattern should be used as indication of TDD configuration in PSBCH with overhead reduction.

Proposal 9: Only inform the number of consecutive uplink slots/symbols and periodicity of NR Uu TDD UL-DL configuration information in order to reduce the overhead.

Proposal 10: Confirm the working assumption that indication of TDD configuration occupies 12 bits in PSBCH payload.

- 12 bits = 3bits (Period) + 7bits (UL Slot) + 2bits (UL Symbol)

Proposal 11: Confirm the working assumption that PSBCH payload size is 56 bits including 24 bits of CRC.

| PSBCH contents | bits | Notes |
| --- | --- | --- |
| Direct Frame Number | 10 | [0,1023] |
| Indication of TDD configuration | 12 | 14bits = 3bits (Period) + 7bits (UL Slot) + 2bits (UL Symbol) |
| S-SSB index | 3 | 3 MSBs of 6 bits indicating S-SSB index come from PSBCH payload for FR2 |
| Indication of synchronization resource | 2 | 2bits indicate 3 candidate sets of synchronization resource |
| In-coverage Indicator | 1 | Carried by PSBCH Payload explicitly |
| Reserved | 4 | Reserved bits |
| CRC | 24 |  |
| Total | 56 |  |

References

  - RAN1 Chairman’s Notes, 3GPP TSG RAN WG1 Meeting #99.

  - RP-193198, “Task list for 5G V2X in RAN1#100”, LG Electronics, Sitges, ES, 3GPP TSG RAN Meeting #86.

  - 3GPP TS 38.211, “NR; Physical channels and modulation”.

  - 3GPP TS 38.331, “NR; Radio Resource Control (RRC)”.

  - RAN1 Chairman’s Notes, 3GPP TSG RAN WG1 Meeting #98bis.