3GPP TSG-RAN WG1 Meeting#100-e	R1-2000579

# e-Meeting, 24th February- 6th March, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR | xxx | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on UE behaviour with PUSCH and no transmission of UL-SCH |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Fujitsu |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2020-02-24 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | For the purpose of CSI transmission on PUSCH a UE can receive DCI format 0_1 with an indication that no UL-SCH should be transmitted. In such a case we think that the expected UE behaviour with respect to handling NDI is not so clear in the RAN1 specifications. Conclusion: - For Format 0_1 the UE should ignore NDI if there is no UL-SCH transmission. - For Rel-16 this would also apply to DCI format 0_2 |  |  |  |  |  |  |  |  |
| F |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add “Upon detection of a DCI format 0_1 or 0_2 with “UL-SCH indicator” set to “0” the UE shall ignore the NDI field”. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If not approved, the UE and gNB may have different understandings on the expected UE behaviour with respect to NDI. Isolated impact statement: This CR has isolated impact only on the case of PUSCH scheduled with no UL-SCH. The changes in this CR are in-line with the common understanding in RAN1 and does not change existing functionalities. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# 6	Physical uplink shared channel related procedure

## 6.1	UE procedure for transmitting the physical uplink shared channel

PUSCH transmission(s) can be dynamically scheduled by an UL grant in a DCI, or the transmission can correspond to a configured grant Type 1 or Type 2. The configured grant Type 1 PUSCH transmission is semi-statically configured to operate upon the reception of higher layer parameter of configuredGrantConfig including rrc-ConfiguredUplinkGrant without the detection of an UL grant in a DCI. The configured grant Type 2 PUSCH transmission is semi-persistently scheduled by an UL grant in a valid activation DCI according to Clause 10.2 of [6, TS 38.213] after the reception of higher layer parameter configuredGrantConfig not including rrc-ConfiguredUplinkGrant. If Configuredgrantconfig-ToAddModList-r16 is configured, more than one configured grant configuration of configured grant Type 1 and/or configured grant Type 2 may be active at the same time on an active BWP of a serving cell.

For the PUSCH transmission corresponding to a Type 1 configured grant or a Type 2 configured grant activated by DCI format 0_0 or 0_1, the parameters applied for the transmission are provided by configuredGrantConfig except for dataScramblingIdentityPUSCH, txConfig, codebookSubset, maxRank, scaling of UCI-OnPUSCH, which are provided by pusch-Config. For the PUSCH transmission corresponding to a Type 2 configured grant activated by DCI format 0_2, the parameters applied for the transmission are provided by configuredGrantConfig except for dataScramblingIdentityPUSCH, txConfig, codebookSubset-ForDCIFormat0_2, maxRank-ForDCIFormat0_2, scaling of UCI-OnPUSCH provided by pusch-Config [as well as DCI format 0_2 specific handling with respect resource allocation of clause 6.1.2.2, TBA]. If the UE is provided with transformPrecoder in configuredGrantConfig, the UE applies the higher layer parameter tp-pi2BPSK, if provided in pusch-Config, according to the procedure described in Clause 6.1.4 for the PUSCH transmission corresponding to a configured grant.

For the PUSCH retransmission scheduled by a PDCCH with CRC scrambled by CS-RNTI with NDI=1, the parameters in pusch-Config are applied for the PUSCH transmission except for p0-NominalWithoutGrant, p0-PUSCH-Alpha, powerControlLoopToUse, pathlossReferenceIndex described in Clause 7.1 of [6, TS 38.213], mcs-Table, mcs-TableTransformPrecoder described in Clause 6.1.4.1 and transformPrecoder described in Clause 6.1.3.

For a UE configured with two uplinks in a serving cell, PUSCH retransmission for a TB on the serving cell is not expected to be on a different uplink than the uplink used for the PUSCH initial transmission of that TB.

Upon detection of a DCI format 0_1 or 0_2 with “UL-SCH indicator” set to “0” the UE shall ignore the NDI field.

A UE shall upon detection of a PDCCH with a configured DCI format 0_0, 0_1 or 0_2 transmit the corresponding PUSCH as indicated by that DCI. Upon detection of a DCI format 0_1 or 0_2  with "UL-SCH indicator" set to "0" and with a non-zero "CSI request" where the associated "reportQuantity" in CSI-ReportConfig set to "none" for all CSI report(s) triggered by "CSI request" in this DCI format 0_1 or 0_2, the UE ignores all fields in this DCI except the "CSI request" and the UE shall not transmit the corresponding PUSCH as indicated by this DCI format 0_1 or 0_2. For any HARQ process ID(s) in a given scheduled cell, the UE is not expected to transmit a PUSCH that overlaps in time with another PUSCH. For any two HARQ process IDs in a given scheduled cell, if the UE is scheduled to start a first PUSCH transmission starting in symbol j by a PDCCH ending in symbol i, the UE is not expected to be scheduled to transmit a PUSCH starting earlier than the end of the first PUSCH by a PDCCH that ends later than symbol i. The UE is not expected to be scheduled to transmit another PUSCH by DCI format 0_0, 0_1 or 0_2 scrambled by C-RNTI or MCS-C-RNTI for a given HARQ process until after the end of the expected transmission of the last PUSCH for that HARQ process.

If a UE is configured by higher layer parameter PDCCH-Config that contains two different values of CORESETPoolIndex in ControlResourceSet for the active BWP of a serving cell and PDCCHs that schedule two non-overlapping in time domain PUSCHs are associated to different ControlResourceSets having different values of CORESETPoolIndex, for any two HARQ process IDs  in a given scheduled cell, if the UE is scheduled to start a first PUSCH transmission starting in symbol j by a PDCCH associated with a value of CORESETpoolIndex ending in symbol i, the UE can be scheduled to transmit a PUSCH starting earlier than the end of the first PUSCH by a PDCCH associated with a different value of CORESETpoolIndex  that ends later than symbol i.

A UE is not expected to be scheduled by a PDCCH ending in symbol i to transmit a PUSCH on a given serving cell overlapping in time with a transmission occasion, where the UE is allowed to transmit a PUSCH with configured grant according to [10, TS38.321], starting in a symbol j on the same serving cell if the end of symbol i is not at least N2 symbols before the beginning of symbol j. The value N2 in symbols is determined according to the UE processing capability defined in Clause 6.4, and N2 and the symbol duration are based on the minimum of the subcarrier spacing corresponding to the PUSCH with configured grant and the subcarrier spacing of the PDCCH scheduling the PUSCH.

[If [a UE reports the capability of intra-UE prioritization], and if a PUSCH corresponding to a configured grant and a PUSCH scheduled by a PDCCH on a serving cell are partially or fully overlapping in time,

-	If the PUSCH corresponding to the configured grant has priority in configuredGrantConfig set to 1 (i.e., high priority), and the PUSCH scheduled by the PDCCH is indicated as low priority by having the [priority indicator] field in the scheduling DCI set to 0 or by not having the [priority indicator] field present in the scheduling DCI, the UE is expected to transmit the PUSCH corresponding to the configured grant, and cancel the PUSCH transmission scheduled by the PDCCH at latest starting at the first symbol of the PUSCH corresponding to the configured grant.

-	Otherwise, the UE shall cancel the PUSCH transmission corresponding to the configured grant at latest starting M symbols after the end of the last symbol of the PDCCH carrying the DCI scheduling the PUSCH, and transmit the PUSCH scheduled by the PDCCH, where

-	M = Tproc,2 +d1, where Tproc,2 is given by clause 6.4 for the corresponding PUSCH timing capability assuming d2,1 = 0 and d1 is determined by the reported UE capability [XXXXX],

-	In this case, the UE is not expected to be scheduled for the PUSCH by the PDCCH where the PUSCH starts earlier than N symbols after the end of the last symbol of the PDCCH, where

-	N = Tproc,2 + d2, where Tproc,2 is the PUSCH preparation time of the PUSCH scheduled by the PDCCH using the associated PUSCH timing capability according to clause 6.4 and d2 is determined by the reported UE capability [YYYYY].

-	In case of PUSCH repetitions, the overlapping handling is performed for each PUSCH repetition separately.

-	The UE is not expected to be scheduled for another PUSCH by a PDCCH where this PUSCH starts no earlier than the end of the prioritized transmitted PUSCH and before the end of the time domain allocation of the cancelled PUSCH.]

A UE is not expected to be scheduled by a PDCCH ending in symbol i to transmit a PUSCH on a given serving cell for a given HARQ process, if there is a transmission occasion where the UE is allowed to transmit a PUSCH with configured grant according to [10, TS38.321] with the same HARQ process on the same serving cell starting in a symbol j after symbol i, and if the gap between the end of PDCCH and the beginning of symbol j is less than N2 symbols. The value N2 in symbols is determined according to the UE processing capability defined in Clause 6.4, and N2 and the symbol duration are based on the minimum of the subcarrier spacing corresponding to the PUSCH with configured grant and the subcarrier spacing of the PDCCH scheduling the PUSCH.

For PUSCH scheduled by DCI format 0_0 on a cell, the UE shall transmit PUSCH according to the spatial relation, if applicable, corresponding to the dedicated PUCCH resource with the lowest ID within the active UL BWP of the cell, as described in Clause 9.2.1 of [6, TS 38.213].

For PUSCH scheduled by DCI format 0_0 on a cell and if the higher layer parameter enableDefaultBeamPlForPUSCH0_0 is set ‘enabled’, the UE is not configured with PUCCH resources on the active UL BWP and the UE is in RRC connected mode, the UE shall transmit PUSCH according to the spatial relation, if applicable, with a reference to the RS with ‘QCL-Type-D’ corresponding to the QCL assumption of the CORESET with the lowest ID.

For PUSCH scheduled by DCI format 0_0 on a cell and if the higher layer parameter enableDefaultBeamPlForPUSCH0_0 is set ‘enabled’, the UE is configured with PUCCH resources on the active UL BWP where all the PUCCH resource(s) are not configured with any spatial relation and the UE is in RRC connected mode, the UE shall transmit PUSCH according to the spatial relation, if applicable, with a reference to the RS with ‘QCL-Type-D’ corresponding to the QCL assumption of the CORESET with the lowest ID in case CORESET(s) are configured on the CC.

For uplink, 16 HARQ processes per cell is supported by the UE.