3GPP TSG RAN WG1 #100-e Meeting	    			             R1-2000620

e-Meeting, February 24th – March 6th, 2020

Agenda item:	7.2.4.4

Source: 	Samsung

Title: 				    On In-device Coexistence between LTE and NR Sidelink

Document for:	Discussion and Decision

Introduction

It was declared that the WI for 5G V2X with NR sidelink was completed in RAN#86. However, there are still many issues to be resolved in current version of Rel-16 NR specifications [3]-[6]. In this contribution, we provide views on the remaining issues on sidelink physical layer structure. In this contribution, we provide our views on the remaining issues on sidelink coexistence between LTE and NR sidelinks.

Discussion

During RAN1#98bis and RAN1#99, the following agreements were made.

- Agreements on TX/RX overlap handling in short-term TDM solution - For Tx/Rx overlap, - If packet priorities of both LTE and NR sidelinks are known to both RATs prior to time of transmission/reception (subject to processing time restrictions), then the packet with a higher relative priority is transmitted/received - In case the priorities of LTE and NR sidelink packets are the same, then it is up to UE implementation as to which packet is transmitted/received - Agreements on priority of SLSS/PSBCH and PSFCH - For sidelink synchronization signal/channel (including S-SSB and LTE SLSS/PSBCH) priority for a UE is (pre)-configured per UE - The (pre)-configured priority is used in the same way as the priority for other channel/signals w.r.t. prioritization for handling in-device co-existence - Note: it is understood that the same priority (pre)-configuration is intended for all the related UEs - The priority of PSFCH is set as the priority of the corresponding PSSCH. - Agreements on short-term time scale TDM solution (in-device coexistence) - UE reports its capability to the network of whether it supports short-term time scale TDM solutions. - Resource allocation related information is not reported to other RAT. - Agreements on handling of overlap between multiple NR SL TXs and LTE SL TX/RX - When NR multiple transmissions (if supported) are overlapped with LTE SL TX/RX and if these NR multiple transmissions have different priorities (which are known in advance to the UE), the highest priority value of NR multiple transmissions is used for comparing that of LTE SL TX/RX and then SL operation with a higher relative priority is performed. - Agreements on handling of conflict with network-controlled mode - In-device coexistence conflicts for network-controlled modes are addressed in the same way as for UE-autonomous modes - No addition spec is expected

For short-term time scale TDM solution, it was decided that resource allocation related information is not reported to other RAT. This enables to avoid dynamic information report about resource allocation. Also, as long term time scale TDM solution for in-device coexistence between LTE and NR sidelinks in case of 오류! 참조 원본을 찾을 수 없습니다., it was agreed that no information is exchanged between LTE and NR sidelinks within the UE and no additional modifications to LTE specifications are needed. One thing to discuss is whether there is no way for the gNB/eNB to do for coexistence.

For operations of NR sidelink and LTE sidelink, a vehicle will be (pre-)configured with information of resource pools, which includes frequency-domain and time-domain resources for transmission and reception. The gNB/eNB may configure non-overlapping resource pools in time domain for NR sidelink and LTE sidelink. If it is possible, the vehicle can naturally avoid in-device coexistence issues between NR sidelink and LTE sidelink because it does not need to transmit and receive NR sidelink and LTE sidelink signals and channels simultaneously.

Figure 1. Example of resource pool (RP) of NR and LTE sidelinks with non-overlapping RPs in time domain

As shown in Figure 1, the gNB or eNB can configure non-overlapping resource pools for NR and LTE sidelink. For this, it is necessary that the UE reports its configured resource pool information to the gNB/eNB. For example, if the UE already performs LTE sidelink operations and it becomes RRC_connected to the gNB, then for performing NR sidelink operations as well, the gNB can configure resource pool for NR sidelinks. Before configuring the resource pool for NR sidelink, the gNB may request the UE report information on its resource pool for LTE sidelink. Then, after receiving that UE assistance information as shown in Figure 2, the gNB may configure resource pool for NR sidelink in order to avoid in-device coexistence. Most of UE behaviours on transmission and reception cases between LTE and NR sidelinks may be left up to UE implementation. Then it would be the best way for the gNB to configure non-overlapping resource pool for NR sidelink.

Figure 2. Illustration of UE assistance information on resource pool

Proposal 1. As UE assistant information, UE reports information on its configured resource pool of LTE sidelink and/or NR sidelink to the eNB and gNB.

Conclusions

This contribution considered aspects for in-device coexistence between LTE and NR sidelink, and the followings are proposed.

Proposal 1. As UE assistant information, UE reports information on its configured resource pool of LTE sidelink and/or NR sidelink to the eNB and gNB.

- References:

[1] RP‑191723, “Revised WID: 5G V2X with NR sidelink,” Sept. 2019.

[2] 3GPP TR 38.885 V2.0.0, “Study on Vehicle-to-Everything,” Mar. 2019.

[3] 3GPP TS 38.211 V16.0.0, “NR; Physical channels and modulation,” Dec. 2019.

[4] 3GPP TS 38.212 V16.0.0, “NR; Multiplexing and channel coding,” Dec. 2019.

[5] 3GPP TS 38.213 V16.0.0, “NR; Physical layer procedures for control,” Dec. 2019.

[6] 3GPP TS 38.214 V16.0.0, “NR; Physical layer procedures for data,” Dec. 2019.