3GPP TSG RAN WG1 #100-e	R1-2000804

e-Meeting, February 24th – March 6th, 2020

Agenda item:		7.2.8.1

Source:	Nokia, Nokia Shanghai Bell

Title:	Correction of DL PRS sequence generation

Document for:		Discussion and Decision

# Introduction

After RAN1#99 the CRs for inclusion of NR positioning into Rel-16 RAN1 specs were agreed, including to TS 38.211 [1]. These were then approved at RAN#86. This contribution discusses some corrections we see that are needed for that specification.

# Discussion

During the Rel-16 Work Item on NR Positioning [2] the following agreements were made regarding the DL PRS sequence:

Agreement:

- DL PRS sequence is generated using a Gold sequence generator as defined in TS 38.211 Section 5.2.1

    - FFS: cinit value for DL PRS

- QPSK modulation is used for the DL PRS signal transmitted using CP-OFDM

- FFS: Whether a DL PRS sequence generated using a different mechanism is also specified

Agreement:

cinit is a function of PRS sequence ID, slot index and symbol index

  - FFS: Other parameters and initialization details until RAN1#98bis

It was agreed that the Gold sequence would be reused similar to both NR CSI-RS and the LTE PRS. In TS 36.211 the sequence generation function for the LTE PRS is given by:

From [3] we see that the NR CSI-RS sequence generation function is given by:

Observation 1: NR CSI-RS and LTE PRS have the same sequence generation function.

However, in the NR DL PRS sequence generation equation there appears to be a factor of 2 missing. We think this should be a simple fix.

Additionally, a factor of 2μ seems to be missing from the list of values of the periodicity of the DL PRS and [3] is currently misaligned with both the agreement and TS 38.214. See the agreement below from RAN1#99:

Agreement:

The following periodicity values of DL PRS resource allocation are supported depending on SCS

- 2^μ {4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 160, 320, 640, 1280, 2560, 5120, 10240} slots, µ = 0, 1, 2, 3 for SCS 15, 30, 60 and 120kHz respectively

Proposal 1: Adopt the TP in the Appendix to fix the DL PRS sequence generation equation and periodicities.

# Conclusion

Observation 1: NR CSI-RS and LTE PRS have the same sequence generation function.

Proposal 1: Adopt the TP in the Appendix to fix the DL PRS sequence generation equation.

# References

- R1-1913661, Introduction of NR Posiitoning Support, Ericsson.

- RP-190723, “New WID: NR Positioning Support”, Intel Corporation, Ericsson, RAN#83, Shenzhen, China, March 18-21, 2019.

- TS 38.211.

# Appendix: TP for TS 38.211

#### 7.4.1.7	Positioning reference signals

##### 7.4.1.7.1	General

A positioning frequency layer consists of one or more downlink PRS resource sets, each of which consists of one or more downlink PRS resources as described in [6, TS 38.214].

##### 7.4.1.7.2	Sequence generation

The UE shall assume the reference-signal sequence rm is defined by

rm=121-2cm+j121-2cm+1rm=121-2c2m+j121-2c2m+1

where the pseudo-random sequence ci is defined in clause 5.2.1. The pseudo-random sequence generator shall be initialised with

cinit=222nID,seqPRS1024+210Nsymbslotns,fμ+l+12nID,seqPRS mod 1024+1+nID,seqPRS mod 1024 mod 231

where ns,fμ is the slot number, the downlink PRS sequence ID nID,seqPRS∈0,1,…,4095 is given by the higher-layer parameter DL-PRS-SequenceId, and l is the OFDM symbol within the slot to which the sequence is mapped.

-----------------------------------------------------Omited text----------------------------------------------------------------------

##### 7.4.1.7.4	Mapping to slots in a downlink PRS resource set

For a downlink PRS resource in a downlink PRS resource set, the UE shall assume the downlink PRS resource being transmitted when the slot and frame numbers fulfil

Nslotframe,μnf+ns,fμ-ToffsetPRS-Toffset,resPRS mod 2μTperPRS∈iTgapPRSi=0TrepPRS-1

and one of the following conditions are fulfilled:

-	the higher-layer parameter DL-PRS-MutingPattern is not provided;

-	the higher-layer parameter DL-PRS-MutingPattern is provided and bitmap b1 but not bitmap b2 is provided, and bit bi1 is set;

-	the higher-layer parameter DL-PRS-MutingPattern is provided and bitmap b2 but not bitmap b1 is provided, and bit bi2 is set;

-	the higher-layer parameter DL-PRS-MutingPattern is provided and both bitmaps b1 and b2 are provided, and both bit bi1 and bi2 are set.

where

-	bi1 is bit i=Nslotframe,μnf+ns,fμ-ToffsetPRS-Toffset,resPRS 2μTmutingPRSTperPRSmod L in the bitmap given by the higher-layer parameter DL-PRS-MutingPattern where L∈2, 4, 8, 16, 32 is the size of the bitmap;

-	bi2 is bit i=Nslotframe,μnf+ns,fμ-ToffsetPRS-Toffset,resPRS mod 2μTperPRS TgapPRS mod TrepPRS in the bitmap given by the higher-layer parameter DL-PRS-MutingPattern;

-	the slot offset ToffsetPRS∈0,1,…,TperPRS-1 is given by the higher-layer parameter DL-PRS-ResourceSetSlotOffset;

-	the downlink PRS resource slot offset Toffset,resPRS is given by the higher-layer parameter DL-PRS-ResourceSlotOffset;

-	the periodicity TperPRS∈2μ 4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 160, 320, 640, 1280, 2560, 5120, 10240 is given by the higher-layer parameter DL-PRS-Periodicity;

-	the repetition factor TrepPRS∈1,2,4,6,8,16,32 is given by the higher-layer parameter DL-PRS-ResourceRepetitionFactor;

-	the muting repetition factor TmutingPRS is given by the higher-layer parameter DL-PRS-MutingBitRepetitionFactor;

-	the time gap TgapPRS∈1,2,4,8,16,32 is given by the higher-layer parameter DL-PRS-ResourceTimeGap;