3GPP TSG RAN WG1 Meeting #100bis  			    			            R1-2002248

e-Meeting, April 20th – 30th, 2020

# Source:	ETRI

# Title:		Remaining issues on initial access procedure for NR-U

# Agenda item:	7.2.2.2.2

# Document for:	Discussion/Decision

Introduction

In the last meeting, many issues on initial access procedures for NR-U WI have been resolved and captured in the specifications. However, there are couple of remaining issues that still need to be clarified and resolved. In this contribution, we introduce our views on the remaining issues.

NR-U initial access procedures

Clarification on allowance of repetition of SSBs

The following is the description for cell search in [1].

TS 38.213 Subclause 4.1, Current Specification----------------------------------------------

For operation with shared spectrum channel access, a UE assumes that transmission of SS/PBCH blocks in a half frame is within a discovery burst transmission window that starts from the first symbol of the first slot in a half-frame. The UE can be provided per serving cell by DiscoveryBurst-WindowLength-r16 a duration of the discovery burst transmission window. If DiscoveryBurst-WindowLength-r16 is not provided, the UE assumes that the duration of the discovery burst transmission window is a half frame. For a serving cell, the UE assumes that a periodicity of the discovery burst transmission window is same as a periodicity of half frames for receptions of SS/PBCH blocks in the serving cell. The UE assumes that one or more SS/PBCH blocks indicated by ssb-PositionsInBurst may be transmitted within the discovery burst transmission window and have candidate SS/PBCH blocks indexes corresponding to SS/PBCH block indexes provided by ssb-PositionsInBurst. If MSB k, k≥1, of ssb-PositionsInBurst is set to 1, the UE assumes that one or more SS/PBCH blocks within the discovery burst transmission window with candidate SS/PBCH block indexes corresponding to SS/PBCH block index equal to k-1 may be transmitted; if MSB k is set to 0, the UE assumes that the SS/PBCH block(s) are not transmitted.

For operation with shared spectrum channel access, a UE assumes that SS/PBCH blocks in a serving cell that are within a same discovery burst transmission window or across discovery burst transmission windows are quasi co-located with respect to average gain, QCL-TypeA, and QCL-TypeD properties, when applicable [6, TS 38.214], if a value of NDM-RSPBCHmodNSSBQCL is same among the SS/PBCH blocks. NDM-RSPBCH is an index of a DM-RS sequence transmitted in a PBCH of a corresponding SS/PBCH block, and NSSBQCL is either provided by ssbPositionQCL-Relationship-r16 or, if ssbPositionQCL-Relationship-r16 is not provided, obtained from a MIB provided by a SS/PBCH block according to Table 4-1. ssbSubcarrierSpacingCommon indicates SCS of RMSI only for the case of "operation without shared spectrum". The UE assumes that within a discovery burst transmission window, a number of transmitted SS/PBCH blocks on a serving cell is not larger than NSSBQCL. The UE can determine an SS/PBCH block index according to NDM-RSPBCHmodNSSBQCL, or according to i modNSSBQCL where i is the candidate SS/PBCH block index.

----------------------------------------------------------------------------------------------------------

According to the above description of the specification, if NSSBQCL=4, ssb-PositionsInBurst = ‘10100000’ (gray number is not valid), and DiscoveryBurst-WindowLength-r16 = 2 ms with 30 kHz SCS are configured, candidate SSB positions for actual SSB transmission will be set as ‘10101010000000000000’ (gray number is not valid). In this case, the SSBs in 1st and 3rd candidate positions are QCLed, and the SSBs in 2nd and 4th candidate positions are also QCLed. From the above highlighted sentence saying “The UE assumes that within a discovery burst transmission window, a number of transmitted SS/PBCH blocks on a serving cell is not larger than NSSBQCL”, the UE can assume that up to 4 SSBs can be transmitted. Therefore, it can be assumed that multiple SSBs with the same beam can be transmitted within the DRS transmission window, for example 1st & 3rd SSBs with the beam #1, and 2nd & 4th SSBs with the beam #2. Even though there is no strong preference on this issue, it is needed to discuss whether the transmission of repetition of SSB with the same beam within DRS transmission window is allowed. If the transmission of repeated SSBs with the same beam is allowed, it is up to gNB and UE implementation issues, and the specification can be kept as it is. However, if it is NOT allowed, the related description of the specification should be modified properly.

Proposal 1: Discuss whether the transmission of repetition of SSBs with the same beam within DRS transmission window is allowed or not.

PDSCH rate-matching around candidate SSBs

Regarding the relationship between SSB index and candidate SSB index was discussed and agreed, and the impacts of specifications were summarized in [2]. Most of those have been captured in the related specification. However, the index #22 is still missed as shown in Appendix, even though couple of contributions mentioned on the issue in the last meeting [3][4]. Therefore, it is proposed to adopt the following TP based on the TP in [3] in order to address the remaining issue for indexing SSB for NR-U.

Proposal 2: The following TP should be adopted to address the remaining issue for indexing SSBs.

TS 38.214 Subclause 5.1.4, Start of Text Proposal ---------------------------------------------

### 5.1.4	PDSCH resource mapping

When receiving the PDSCH scheduled with SI-RNTI and the system information indicator in DCI is set to 0, the UE shall assume that no SS/PBCH block is transmitted in REs used by the UE for a reception of the PDSCH.

When receiving the PDSCH scheduled with SI-RNTI and the system information indicator in DCI is set to 1, RA-RNTI, MsgB-RNTI, P-RNTI or TC-RNTI, the UE assumes SS/PBCH block transmission according to ssb-PositionsInBurst, and if the PDSCH resource allocation overlaps with PRBs containing SS/PBCH block transmission resources the UE shall assume that the PRBs containing SS/PBCH block transmission resources are not available for PDSCH in the OFDM symbols where SS/PBCH block is transmitted.

A UE expects a configuration provided by ssb-PositionsInBurst in ServingCellConfigCommon to be same as a configuration provided by ssb-PositionsInBurst in SIB1.

When receiving PDSCH scheduled by PDCCH with CRC scrambled by C-RNTI, MCS-C-RNTI, CS-RNTI, or PDSCHs with SPS, the REs corresponding to the configured or dynamically indicated resources in Clauses 5.1.4.1, 5.1.4.2 are not available for PDSCH. Furthermore, the UE assumes SS/PBCH block transmission according to ssb-PositionsInBurst if the PDSCH resource allocation overlaps with PRBs containing SS/PBCH block transmission resources, the UE shall assume that the PRBs containing SS/PBCH block transmission resources are not available for PDSCH in the OFDM symbols where SS/PBCH block is transmitted.

A UE is not expected to handle the case where PDSCH DM-RS REs are overlapping, even partially, with any RE(s) not available for PDSCH.

For operation with shared spectrum channel access, SS/PBCH block transmission according to ssb-PositionsInBurst represents candidate SS/PBCH blocks corresponding to SS/PBCH block indexes provided by ssb-PositionsInBurst as described in Clause 4.1 of [6, TS 38.213].

TS 38.214 Subclause 5.1.4, End of Text Proposal ---------------------------------------------

Summary

In this contribution, we made the following proposals for the remaining issues on initial access procedure for NR-U.

Proposal 1: Discuss whether the transmission of repetition of SSBs with the same beam within DRS transmission window is allowed.

Proposal 2: The following TP should be adopted to address the remaining issue for indexing SSBs.

References

- 3GPP, TS38.213 v16.1.0, Mar. 2020.

- R1-1913538, “Summary of discussion on SS/PBCH block indexing,” Samsung

- R1-2000664, “Remaining issues of initial access and mobility for NR-U”, LG Electronics

- R1-2000771, “Remaining issues on initial access procedure for NR-U”, ETRI

- Appendix

| Index | Spec | Session | Topic | Detailed description in Rel-15 | RAN1 consensus | Other comments |
| --- | --- | --- | --- | --- | --- | --- |
| 22 | 38.214 | 5.1.4 | PDSCH resource mapping | When receiving the PDSCH scheduled with SI-RNTI and the system information indicator in DCI is set to 0, the UE shall assume that no SS/PBCH block is transmitted in REs used by the UE for a reception of the PDSCH. When receiving the PDSCH scheduled with SI-RNTI and the system information indicator in DCI is set to 1, RA-RNTI, P-RNTI or TC-RNTI, the UE assumes SS/PBCH block transmission according to ssb-PositionsInBurst, and if the PDSCH resource allocation overlaps with PRBs containing SS/PBCH block transmission resources the UE shall assume that the PRBs containing SS/PBCH block transmission resources are not available for PDSCH in the OFDM symbols where SS/PBCH block is transmitted. A UE expects a configuration provided by ssb-PositionsInBurst in ServingCellConfigCommon to be same as a configuration provided by ssb-PositionsInBurst in SIB1. When receiving PDSCH scheduled by PDCCH with CRC scrambled by C-RNTI, MCS-C-RNTI, CS-RNTI, or PDSCHs with SPS, the REs corresponding to the configured or dynamically indicated resources in Subclauses 5.1.4.1, 5.1.4.2 are not available for PDSCH. Furthermore, the UE assumes SS/PBCH block transmission according to ssb-PositionsInBurst if the PDSCH resource allocation overlaps with PRBs containing SS/PBCH block transmission resources, the UE shall assume that the PRBs containing SS/PBCH block transmission resources are not available for PDSCH in the OFDM symbols where SS/PBCH block is transmitted. A UE is not expected to handle the case where PDSCH DM-RS REs are overlapping, even partially, with any RE(s) not available for PDSCH. | RAN1’s intention is, the UE assumes SS/PBCH block transmission according to candidate SS/PBCH block index(es) corresponding to SS/PBCH block index(es) provided by ssb-PositionsInBurst. |  |