3GPP TSG-RAN WG1 Meeting #100bis-e	R1-20xxxxx

e-Meeting, April 20th – 30th, 2020

Agenda Item:	6.2.1.4

Source:	Moderator (Ericsson)

Title:	Feature lead summary for NR coexistence performance improvements for LTE-MTC

Document for:	Discussion, Decision

# Introduction

In the Rel-16 work item on “Additional MTC enhancements for LTE” [1], one of the objectives is to specify performance improvements for LTE-MTC coexistence with NR.

The objective is to specify the following set of improvements for machine-type communications for BL/CE UEs. [...] Coexistence with NR: - Specify the following performance improvements for LTE-MTC coexistence with NR [RAN1, RAN2, RAN4] - LTE-MTC resource reservation in the DL frequency domain and the DL/UL time domain with slot-level and symbol-level granularity to avoid resource overlap between NR and LTE-MTC when LTE-MTC is deployed within an NR carrier - LTE-MTC subcarrier puncturing for 1 or 2 LTE-MTC DL subcarriers (excluding CRS) to reduce the number of NR resource blocks that need to be reserved for LTE-MTC when LTE-MTC is deployed within an NR carrier

RAN1 agreements made until RAN1#99 are summarized in [2] and RAN1 agreements made in RAN1#100e are listed below. RAN2 agreements are summarized in [3]. The endorsed L1 configuration parameter list can be found in [4], the initial RAN1 UE feature list in [5], and the endorsed RAN1 CRs in [6] – [16].

R1-2001058 Feature lead summary for NR coexistence performance improvements for LTE-MTC Ericsson R1-2001186 Feature lead summary#2 for NR coexistence performance improvements for LTE-MTC Ericsson R1-2001221 Feature lead summary#3 for NR coexistence performance improvements for LTE-MTC Ericsson [100e-LTE-eMTC5-NR-coexistence-01] – Johan (Ericsson) Email discussion/approval focusing on the following issues (refering to section 2 of R1-2001186): - UE-specific resource reservation (including configuration aspects and Type0-CSS aspects) - Clarification of handling of fully and partially reserved subframes - Resource reservation in special subframes in TDD - Definition of subcarrier puncturing by 2/27; if there is a spec impact, followed by endorsing the corresponding TP by 3/2 Decision: As per email decision posted on Mar. 4th, Agreement Issue #1: UE-specific resource reservation (including configuration aspects and Type0-CSS aspects) - The 36.211 TPs in Section 2.1 in FL summary in R1-2001186 are endorsed. TP to be included in 36.211 editor’s CR. - The 36.212 TPs in Section 2.1 in FL summary in R1-2001186 are endorsed with the following change: the words “given by C-RNTI or SPS C-RNTI” are removed in all four places. TP to be included in 36.212 editor’s CR. - It should be possible to enable the resource reservation feature using UE-specific signaling, separately for DL and UL. Indicate this in the updated L1 parameter list. The details are up to RAN2. Issue #2: Clarification of handling of fully and partially reserved subframes - The 36.213 TP in Section 2 in FL summary in R1-2001221 is endorsed. TP to be included in 36.213 editor’s CR. - The 36.211 TP in Section 3 in FL summary in R1-2001221 is endorsed. TP to be included in 36.211 editor’s CR. Issue #4: Definition of subcarrier puncturing - The 36.211 TP on definition of subcarrier puncturing in Section 2.4 in FL summary in R1-2001186 is endorsed. TP to be included in 36.211 editor’s CR.

This document provides a prioritized list of issues and proposals based on the contributions in [17] – [25].

# Issue #1: LS response to RAN2

RAN2 has sent questions regarding configuration of LTE-MTC/NB-IoT resource reservation to RAN1 in the LS in [18], and several companies have provided discussion and proposed answers in [18][19][20][21][23].

Discuss LS response to RAN2 in a joint email discussion for LTE-MTC and NB-IoT.

# Issue #2: Special subframes

RAN1#100e discussed whether and how to support resource reservation in special subframes in TDD without reaching a conclusion. Based on the input in contributions [21][22][23][24][25], the following can be considered.

Symbol-level granularity resource reservation is not applied in special subframes.

If above proposal can be agreed, the following 36.211 TP from Huawei’s contribution [21] can be considered.

Consider the following 36.211 TP on transmission in special subframes.

## 4.2 Frame structure type 2 <Unchanged parts are omitted> For frame structure type 2, if higher layer parameter ce-reserved-resource-DL-time is configured, ce-reserved-resource-DL-time-symbol-bitmap1 and ce-reserved-resource-DL-time-symbol-bitmap2 is not applied to special subframes. ## 4.3 Frame structure type 3 <Unchanged parts are omitted>

# Issue #3: DL DMRS

Qualcomm’s contribution [23] has the following 36.211 TP on DL DMRS transmission (see contribution for further discussion).

Consider the following 36.211 TP on DL DMRS transmission.

#### 6.10.3.2 Mapping to resource elements <Unchanged parts are omitted> For BL/CE UEs, if higher layer parameter ce-reserved-resource-DL-freq or ce-reserved-resource-DL-time is configured, and the Resource reservation field in the DCI is set to 1, then in case of PDSCH transmission associated with C-RNTI or SPS C-RNTI using UE-specific MPDCCH search space, - If and only if all OFDM symbols in a PRB are reserved, the demodulation reference signal transmission in that PRB is dropped. <Unchanged parts are omitted> #### 6.10.3A.2 Mapping to resource elements <Unchanged parts are omitted> For BL/CE UEs, if higher layer parameter ce-reserved-resource-DL-freq or ce-reserved-resource-DL-time is configured, then in case of MPDCCH transmission associated with C-RNTI or SPS C-RNTI using UE-specific MPDCCH search space, - If and only if all OFDM symbols in a PRB are reserved, the demodulation reference signal transmission in that PRB is dropped. <Unchanged parts are omitted>

# Issue #4: UL DMRS

Huawei’s contribution [21] has the following 36.211 TP on UL DMRS transmission (see contribution for further discussion).

Consider the following 36.211 TP on UL DMRS transmission.

##### 5.5.2.1.2 Mapping to physical resources <Unchanged parts are omitted> For BL/CE UEs, if higher layer parameter ce-reserved-resource-UL-time is configured, and the Resource reservation field in the DCI is set to 1, then in case of PUSCH transmission associated with C-RNTI or SPS C-RNTI, - In a subframe that is partially reserved, the demodulation reference signal transmission in a SC-FDMA symbol that is reserved is dropped. <Unchanged parts are omitted>

# Issue #5: SRS

ZTE’s contribution [22] proposes that SRS transmission that would fall into a reserved UL resource should be dropped (see contribution for further discussion).

If a 36.211 TP on SRS transmission can be provided in time, it can be considered in this meeting, otherwise it can be treated in the next meeting.

# Issue #6: SPS

Ericsson’s contribution [24] proposes to discuss whether and how to update the formulation “associated with C-RNTI or SPS C-RNTI using UE-specific MPDCCH search space” in 36.211 and 36.213 to cover MPDCCH-less SPS PUSCH transmission in the intended way (see contribution for further discussion).

If 36.211/36.213 TPs on SPS transmission can be provided in time, they can be considered in this meeting, otherwise they can be treated in the next meeting.

# References

- RP-192875, “Revised WID: Additional MTC enhancements for LTE”

- R1-1913594, “RAN1 agreements for Rel-16 Additional MTC Enhancements for LTE”

- R2-2001886, “RAN2 agreements for Rel-16 additional enhancements for NB-IoT and MTC”

- R1-2001477, “Updated consolidated parameter list for Rel-16 LTE”

- R1-2001485, “RAN1 UE features list for Rel-16 LTE after RAN1#100-E”

- R1-1913610, Addition of feature for 36.211

- R1-1913611, Addition of feature for 36.212

- R1-1913612, Addition of feature for 36.213 (s00-s05)

- R1-1913613, Addition of feature for 36.213 (s06-s07)

- R1-1913614, Addition of feature for 36.213 (s08-s09)

- R1-1913684, Addition of feature for 36.213 (s10-s13)

- R1-1913615, Addition of feature for 36.213 (s14-sxx)

- R1-1913683, Addition of feature for 36.214

- R1-2001427, Corrections for 36.211

- R1-2001431, Corrections for 36.212

- R1-2001433, Corrections for 36.213

- R1-2001518, “LS on NR coexistence”, RAN2

- R1-2001848, “Discussion on RAN2 LS on NR coexistence”, ZTE

- R1-2002502, “On the LS on NR coexistence for NB-IoT/eMTC”, Ericsson

- R1-2002602, “Draft reply LS on NR coexistence”, Huawei, HiSilicon

- R1-2001569, “Corrections on eMTC co-existence with NR”, Huawei, HiSilicon

- R1-2001855, “Remaining issues on LTE-MTC resource reservation”, ZTE

- R1-2002175, “Coexistence of LTE-MTC with NR”, Qualcomm Incorporated

- R1-2002505, “Corrections for NR coexistence performance improvements for LTE-MTC”, Ericsson

- R1-2002643, “Remaining issues for co-existence of eMTC with NR”, Nokia, Nokia Shanghai Bell