3GPP TSG RAN WG1 Meeting #100bis-e	R1-200xxxx

E-Meeting, April 20 – 30, 2020

Agenda Item:	6.2.2.4

Source:	Moderator (Huawei)

Title:	TP on resource reservation of TDD special subframes in TS 36.211

Document for:	Discussion and Decision

Introduction

This document provides the text proposal as outcomes of the following email discussion [1]:

[100b-e-LTE-NB_IoTenh3-Coex-NR-01] Resource reservation for TDD NB-IoT by 4/24 and corresponding TP (if any) by 4/30 – Yubo (Huawei)

- Issues #1, #4 in R1-2002700

Discussion

Reason for changes:

It was agreed in RAN1#100bis-e that symbol-level resource reservation is not applied to special subframes.

Summary of changes:

Specify that symbol-level resource reservation is not applied to special subframes.

Specs/sections impacted:

36.211 sections 10.0.1.2

Consequences if not approved:

Resource reservation may be applied to special subframes incorrectly.

-----------------------------------------------------Start of Text Proposal-----------------------------------

#### 10.0.1.2	Frame structure type 2

Frame structure type 2 is applicable to TDD operation only.

The following restrictions apply:

-	Uplink-downlink configuration 0 and 6 are not supported.

-	UpPTS is not used for NPUSCH or NPRACH.

-	DwPTS and UpPTS in special subframe configuration 10 is not used for transmissions.

-	On an NB-IoT carrier for which higher-layer parameter operationModeInfo indicates inband-SamePCI or inband-DifferentPCI, or higher-layer parameter inbandCarrierInfo is present, or on an NB-IoT carrier for SystemInformationBlockType1-NB for which sib1-carrierInfo indicates non-anchor and the value of the higher layer parameter sib-GuardbandInfo is set to sib-GuardbandInbandSamePCI or sib-GuardbandinbandDiffPCI, DwPTS in special subframe configuration 0 and 5 for normal cyclic prefix is not used for NPDCCH and NPDSCH transmission.

-	Higher-layer parameter symbolBitmap-r16 does not apply to special subframes.

------------------------------------------------------End of Text Proposal------------------------------------

- References

- R1-200x	Feature lead summary #1 on 100b-e-LTE-NB_IoTenh3-Coex-NR-01	Moderator(Huawei)