3GPP TSG RAN WG1 Meeting #100bis-E					     R1-200xxxx

e-Meeting, April 20th – 30th, 2020

Source:	Moderator (Intel Corporation)

Title:	TP Email Discussion #5 [100b-e-NR-5G_V2X_NRSL-Mode-2-05]

Agenda item:	7.2.4.2.2

Document for:	Discussion and Decision

Introduction

This document provides discussion on TP as per the fifth email discussion on V2X Mode-2 during RAN1#100bis-e.

[100b-e-NR-5G_V2X_NRSL-Mode-2-05] Email approval of TP capturing previous meeting’s agreement regarding Time and frequency resource indication based on the submitted contributions till 4/22 (Intel, Sergey)

The intention of the TP is to implement the following agreements made in RAN1#100e:

Agreements: - Time resource assignment in SCI uses an extended time domain RIV mechanism as follows: if N=1 TRIV=0 elseif N=2 TRIV=T1 else if T2-T1-1≤15 TRIV=30T2-T1-1+T1+31 else TRIV=3031-T2+T1+62-T1 end if end if where - N denotes the actual number of resources indicated - Ti denotes i-th resource time offset - for N=2, 1≤T1≤31 - for N=3, 1≤T1≤30, T1<T2≤31 Agreements: - For frequency resource indication, the following resource index calculation is used - For Nmax = 2, - r=f2+i=1m-1N subchannel SL+1-i - For Nmax = 3, - r=f2+f3⋅N subchannel SL+1-m+i=1m-1N subchannel SL+1-i2 - where - f2 denotes lowest sub-channel index for the second resource, if any - f3 denotes lowest sub-channel index for the third resource, if any - m denotes number of sub-channels in a frequency resource allocation - If time domain allocation indicates N < Nmax, the decoded lowest sub-channel indexes corresponding to Nmax minus N last resources are not used

The draft TPs were identified in contributions [1] and [13]. In the next section, the TP based on these contributions is provided.

Proposed draft TP to TS 38.214

--------------------------------------------- TP to 38.214, section 8.1.5 starts ---------------------------------------------------

### 8.1.5	UE procedure for determining slots and resource blocks for PSSCH transmission associated with an SCI format 0-1

The set of slots and resource blocks for PSSCH transmission is determined by the resource used for the PSCCH transmission containing the associated SCI format 0-1, and fields "Frequency resource assignment", "Time resource assignment" of the associated SCI format 1 as described below.

"Time resource assignment" carries logical slot index indication of N = 1 or 2 or 3 actual resources in a form of time RIV (TRIV) field which is determined as follows:

if N=1

TRIV=0

elseif N=2

TRIV=T1

else

if T2-T1-1≤15

TRIV=30T2-T1-1+T1+31

else

TRIV=3031-T2+T1+62-T1

end if

end if

where Ti denotes i-th resource time offset in logical slots of a resource pool with respect to the first resource where N = 2, 1≤T1≤31, for N = 3, 1≤T1≤30, T1<T2≤31.

The lowest sub-channel index of the first PSSCH associated with the received SCI format 0-1 is determined from the sub-channel index where PSCCH carrying the SCI format 0-1 is received. The number of allocated sub-channels LsubCH and the lowest sub-channel indexes of other PSSCH reserved by the received SCI format 0-1 are determined from "Frequency resource assignment" which is equal to a frequency resource index r where.

If sl-MaxNumPerReserve is 2 then r=f2+i=1LsubCH-1N subchannel SL+1-i,

If sl-MaxNumPerReserve is 3 then r=f2+f3⋅N subchannel SL+1-LsubCH+i=1LsubCH-1N subchannel SL+1-i2,

where

-	f2 denotes the lowest sub-channel index for the second resource, if any

-	f3 denotes the lowest sub-channel index for the third resource, if any

If TRIV indicates N < sl-MaxNumPerReserve, the decoded lowest sub-channel indexes corresponding to sl-MaxNumPerReserve minus N last resources are not used.

--------------------------------------------- TP to 38.214, section 8.1.5 ends ---------------------------------------------------

| Source | Comments |
| --- | --- |
| Qualcomm | To clarify that N is bounded by N_max: - "Time resource assignment" carries logical slot index indication of N = 1 or 2 or, when sl-MaxNumPerPreserve is 3, 3 actual resources in a form of time RIV (TRIV) field which is determined as follows Editorial comments: - where Ti denotes i-th resource time offset in logical slots of a resource pool with respect to the first resource where for N = 2, 1≤T1≤31,; and for N = 3, 1≤T1≤30, T1<T2≤31. - Could you replace T1,T2 with different variable names to avoid confusion with the selection window limits, which are also used in 38.214? For example, S1,S2 (or X1,X2), … |
| Apple | We prefer to denote f2 and f3 as “the starting sub-channel index….”, as defined in LTE V2X (TS36.213). Similar to TRIV, we could use the term FRIV, rather than “r”, for the frequency resource index. |

- References

- R1-2001552	Remaining details of sidelink resource allocation mode 2	Huawei, HiSilicon

- R1-2001661	Remaining issues on mode 2 resource allocation mechanism	vivo

- R1-2001749	Discussion on remaining open issue for mode 2	OPPO

- R1-2001793	Remaining Issues on Sidelink Mode 2 Resource Allocation	Panasonic Corporation

- R1-2001805	Remaining details of Resource allocation for sidelink - Mode 2	Nokia, Nokia Shanghai Bell

- R1-2001877	Remaining details on mode 2 resource allocation for NR V2X	Fujitsu

- R1-2001886	Discussion on resource allocation for Mode 2	LG Electronics

- R1-2001896	Remaining issues of mode 2 operation on sidelink	ZTE, Sanechips

- R1-2001907	Sidelink mode-2 resource allocation	MediaTek Inc.

- R1-2001964	Resource allocation for NR sidelink Mode 2	TCL Communication Ltd.

- R1-2001969	Discussion on resource allocation for NR sidelink Mode 2	Lenovo, Motorola Mobility

- R1-2001978	Remaining Issues in Resource Allocation for Mode 2 NR V2X	Fraunhofer HHI, Fraunhofer IIS

- R1-2001994	Solutions to remaining opens of resource allocation mode-2 for NR V2X sidelink design	Intel Corporation

- R1-2002041	Remianing details on mode-2 resource allocation	Futurewei

- R1-2002078	Remaining issues on Mode 2 resource allocation in NR V2X	CATT

- R1-2002126	On Mode 2 for NR Sidelink	Samsung

- R1-2002234	Resource allocation Mode 2 for NR SL	Ericsson

- R1-2002267	Remaining issues in NR sidelink mode 2 resource allocation	Spreadtrum Communications

- R1-2002301	Remaining Issues on NR Sidelink Mode 2 Resource Allocation	InterDigital, Inc.

- R1-2002325	On Remaining Details of Mode 2 Resource Allocation	Apple

- R1-2002362	Remaining issues on resource allocation Mode 2	NEC

- R1-2002388	Remaining issues on resource allocation mode 2 for NR sidelink	Sharp

- R1-2002402	On resource reservation in Mode 2 resource allocation	Xiaomi Communications

- R1-2002439	Remaining issues on resource allocation mechanism mode 2	NTT DOCOMO, INC.

- R1-2002487	Remain details on mode-2 resource allocation for NR V2X	ITL

- R1-2002489	Remaining issue for Mode 2 resource allocation in NR V2X	ASUSTeK

- R1-2002539	Sidelink Resource Allocation Mechanism for NR V2X	Qualcomm Incorporated